The Printer Working Group

Standard for PDF Image-Streamable Format – “PDF/is”

(Formerly “PDFax”)

Proposed Standard - Working Draft

510n.y-P0.54

28 July 2026

The Printer Working Group Standard for

PDF Image-Streamable Format (PDF/is)

Proposed Standard - Working Draft

510n.y-P0.54

Abstract: This standard specifies a subset of PDF (Portable Document Format) 1.4 known as the PDF Image-Streamable Format (PDF/is) by formally defining a series of PDF/is “profiles” distinguished primarily by the method of image compression employed and color space used.

In summary PDF/is is an image document format intended for use by, but not limited to, the IPPFAX protocol, which is used to provide a synchronous, reliable exchange of image Documents between Senders and Receivers. PDF/is makes reference to the PDF 1.4 Reference [pdf], which describes the PDF representation of image data specified by the ITU-T Recommendations for black-and-white facsimile (see [t.4], [t.6]), the ISO/IEC Specifications for Digital Compression and Coding of Continuous-Tone Still Images (see [jpeg]), and Lossy/Lossless Coding of Bi-Level Images (see [jbig2]), and the general purpose Flate compression methods (see [rfc1950] and [rfc1951]).

This document is available electronically at:

ftp://pwg.org/pub/pwg/QUALDOCS/pwg-ifx-pdfis-P05-021219.pdf, .doc

A version showing the changes from the previous version is available at:

ftp://pwg.org/pub/pwg/QUALDOCS/pwg-ifx-pdfis-P05-021219-rev.pdf

The latest version of this specification is available at:

ftp://pwg.org/pub/pwg/QUALDOCS/pwg-ifx-pdfis-latest.pdf, .doc

Copyright (C) 2002, IEEE ISTO. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  The Printer Working Group Standard for PDF Image-Streamable Format

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

About the IEEE-ISTO

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit http://www.ieee-isto.org.

About the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:  http://www.pwg.org

Contact information:

IFX Web Page:  http://www.pwg.org/qualdocs

IFX Mailing List:  ifx@pwg.org

To subscribe to the ipp mailing list, send the following email:

1) send it to majordomo@pwg.org

2) leave the subject line blank

3) put the following two lines in the message body:

subscribe ifx

end

Implementers of this specification are encouraged to join the IFX Mailing List in order to participate in any discussions of clarifications or review of registration proposals for additional names.  Requests for additional media names, for inclusion in this specification, should be sent to the IFX Mailing list for consideration.

Contents

1	Introduction	8

2	Terminology	8

2.1	Conformance Terminology	8

2.2	Other Terminology	9

3	PDF/is Support	10

3.1	Profiles	10

3.1.1	Image Profiles	10

3.1.2	Security Profiles	10

3.1.3	Color Profiles	10

3.2	PDF Object Requirements	10

3.3	PDF Field Specification	12

3.3.1	’PDF/is’ object	12

3.3.2	‘FlateDecode’ Filter	14

3.3.3	‘CCITTFaxDecode’ Filter	14

3.3.4	‘JBIG2Decode’ Filter	15

3.3.5	‘DCTDecode’ Filter	15

3.3.6	File Trailer	15

3.3.7	Encryption Dictionary	16

3.3.8	Document Catalog	16

3.3.9	Page Tree Nodes	17

3.3.10	Page Objects	17

3.3.11	Content Stream Operators	18

3.3.12	Resource Dictionaries	21

3.3.13	Color Spaces	22

3.3.14	Image XObjects	23

3.3.15	Masked Images	23

3.3.16	Interactive Form Dictionary	23

3.3.17	Annotation Field Dictionary	24

3.3.18	Signature Dictionary	24

3.3.19	Document Information Dictionary	25

3.4	Object Lifetime	25

3.5	Cached Objects	26

3.5.1	Cache Hold	26

3.5.2	Cache Release	26

4	Conformance Requirements	27

4.1	Producer conformance requirements	27

4.2	Consumer conformance requirements	28

4.3	File Layout	28

5	Issues	29

6	Sample PDF/is PDFs	29

7	Normative References	30

8	Informative References	31

9	Revision History (to be removed when standard is approved)	31

10	Contributors	31

11	Acknowledgments	32

12	Author’s Address	32

13	Appendix A	32

13.1	Intellectual Property Statement – Adobe Systems Incorporated	32

Table of Tables

Table 3-1:  Image Profiles	10

Table 3-2:  Security Profiles	10

Table 3-3:  Color Profiles	10

Table 3-4:  PDF Object Requirements	11

Table 3-5:  PDF/is Object	12

Table 3-6:  PDF/is Object ‘IMAGES’ Element	13

Table 3-7:  PDF/is Object ‘SECURITY’ Element	13

Table 3-8:  FlateDecode Filter	14

Table 3-9:  CCITTFaxDecode Filter	14

Table 3-10:  JBIG2Decode Filter	15

Table 3-11:  DCTDecode Filter	15

Table 3-12:  File Trailer	15

Table 3-13:  Encryption Dictionary	16

Table 3-14:  Document Catalog	16

Table 3-15:  Page Tree Nodes	17

Table 3-16:  Page Objects	17

Table 3-17:  Content Stream Operators	18

Table 3-18:  Resource Dictionaries	21

Table 3-19:  Color Spaces	22

Table 3-20:  ICCBased Color Space	22

Table 3-21:  Image XObjects	23

Table 3-22:  Masked Images	23

Table 3-23:  Interactive Form Dictionary	24

Table 3-24:  Annotation Field Dictionary	24

Table 3-25:  Signature Dictionary	25

Table 3-26:  Document Information Dictionary	25

Table 4-1:  File Layout	28

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

# Introduction

In summary, PDF/is is a raster image data format intended for use by, but not limited to, the IPPFAX protocol.  IPPFAX is used to provide a synchronous, reliable exchange of image Documents between Senders and Receivers. PDF/is makes reference to the PDF 1.4 specification [pdf], which describes the PDF (Portable Document Format) representation of image data specified by the ITU-T Recommendations for black-and-white facsimile (see [t.4], [t.6]), the ISO/IEC Specifications for Digital Compression and Coding of Continuous-Tone Still Images (see [jpeg]), and Lossy/Lossless Coding of Bi-Level Images (see [jbig2]), and the general purpose Flate compression methods (see [rfc1950] and [rfc1951]).

PDF/is is an image-only, streamable, subset specification of PDF 1.4 [pdf] and, as such, follows all of the specification requirements of PDF.

As a streamable version of PDF, it is not required that a Consumer of a PDF/is document be able to randomly access the PDF.  The format has been adopted in such a way as to allow a Consumer the ability to read the PDF/is document from the beginning to end without the necessity to cache more data than is necessary to print the current page with some exceptions, as noted.

If a Document adhering to this specification is not encrypted (does not Implement Profiles ‘STD-ENC’ nor ‘PPK-ENC’) it will Implement a conforming subset of the “PDF/X-3” specification (See [pdf-x3]) for use in digital prepress data exchange.

# Terminology

This section defines terminology used throughout this document.

## Conformance Terminology

Capitalized terms, such as MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, MAY, NEED NOT, OPTIONAL, and PROHIBITED, have special meaning relating to conformance as defined in RFC 2119 [rfc2119] and [rfc2911] section 12.1.  If an implementation supports the extension defined in this document, then these terms apply; otherwise, they do not.  These terms define conformance to this document (and [rfc2911]) only; they do not affect conformance to other documents, unless explicitly stated otherwise.  To be more specific:

REQUIRED (REQ) - an adjective used to indicate that a conforming PDF/is Producer or Consumer’s implementation MUST support the indicated operation, object, attribute, or attribute value.  See [rfc2911] “Appendix A - Terminology for a definition of “support”.

RECOMMENDED (REC) - an adjective used to indicate that a conforming PDF/is Producer or Consumer’s implementation SHOULD support the indicated operation, object, attribute, or attribute value.

OPTIONAL (OPT) - an adjective used to indicate that a conforming PDF/is Producer or Consumer’s implementation MAY support the indicated operation, object, attribute, or attribute value.

PROHIBITED (PROH) - an adjective used to indicate that a conforming PDF/is Producer or Consumer’s implementation MUST NOT support the indicated operation, object, attribute, or attribute value.

IGNORED – an adjective used to indicate that a conforming PDF/is Producer or Consumer implementation NEED NOT support the indicated operation, object, attribute, or attribute value; but this feature MAY be added to a future version of this specification.

AS SPECIFIED – is used to indicate that a conforming PDF/is Producer or Render implementation MUST, MAY, or MUST NOT support the indicated operation, object, attribute, or attribute value as is defined in the indicated specification.

OR – a conjunction that specifies a logical ‘or’, implying that a choice of one or more of the choices specified.

XOR – a conjunction that specifies a logical ‘exclusive or’, implying that a choice of one and only one of the choices specified.

## Other Terminology

The following terms are introduced and capitalized in order to indicate their specific meaning:

Implement – The specified feature is present in the Document.

Support – A Producer has the capability of Implementing the feature specified, or the Consumer has the capability of understanding and acting on the Implementation.

Document – The PDF/is-formatted electronic representation of a set of one or more pages that the Sender sends to the Receiver.

Consumer – This is the agent (software, hardware or some combination) that converts the Document into a displayed or printed form.

Producer -- This is the agent (software, hardware or some combination) that creates the Document.

Interpolation – See ‘Interpolation’ in [pdf] pg. 273.

Forward-Reference – In indirect object reference (See [pdf] Section 3.2.9) to an object that appears later in the Document.

Cache – Consumer’s storage, either memory, disk, or the like, to hold Document data as it’s received from the Producer.

Page-Relative Objects – Objects that are indirectly referenced (See [pdf] Section 3.2.9) by either a ‘Page’ object or through a chain of object references that start with a reference from a ‘Page’ object.

Discarded – An adjective that describes a PDF object.  An object is ‘Discarded’ when the Consumer no longer has access to the data within the object in question.

# PDF/is Support

## Profiles

### Image Profiles

The following table defines the Profile names used to describe various image compression filters and techniques.

Table 3-1-:  Image Profiles

| Profile | Image Implementation | Reference |
| --- | --- | --- |
| <G4> | ‘CCITTFaxDecode’ Filter | [pdf] Section 3.3.5 |
| <FLATE> | ‘FlateDecode’ Filter | [pdf] Section 3.3.3 |
| <JBIG2> | ‘JBIG2Decode’ Filter | [pdf] Section 3.3.6 |
| <MASK> | Masked Images | [pdf] Section 4.8.5 |
| <JPEG> | ‘DCTDecode’ Filter | [pdf] Section 3.3.7 |
| <JP2K> | JPEG2000 Filter | Not Currently Supported |

Security Profiles

There are several options that MAY be Supported by a Producer or Consumer with regard to security:

Table 3-2-:  Security Profiles

| Profile | Security Implementation | Reference |
| --- | --- | --- |
| <STD-ENC> | ‘Standard’ Encryption | [pdf] Section 3.5.2 |
| <PPK-ENC> | PPK Encryption | [pdf-ppk] Section 3 |
| <DIG-SIG> | Digital Signature | [pdf-ppk] Section 2.2 |

### Color Profiles

Table 3-3-:  Color Profiles

| Profile | Color Space Implementation |  | Reference |  |  |
| --- | --- | --- | --- | --- | --- |
| <GRAY> | ‘DeviceGray’ |  | [pdf] Page 182 |  |  |
| <RGB> | ‘DeviceRGB’ |  | [pdf] Page 184 |  |  |
| <LAB> | ‘Lab’ |  | [pdf] Page 187 |  |  |
| <ICC> | ‘ICCBased’ |  | [pdf] Page 189 |  |  |
| <IDX> | ‘Indexed’ |  | [pdf] Page 199 |  |  |

-

### PDF Object Requirements

For the table shown below, if an Object/Filter is not Implemented then its associated Profile is not Implemented.

Key:

Producer:  Producer Requirement.

Consumer:  Consumer Requirement.

Profile:  If the indicated ‘PDF Object/Filter’ is Implemented then the Document Implements the indicated Profile.

Dependencies:  In order to Implement the ‘PDF Object/Filter’ the Profiles indicated in the Dependencies column MUST also be implemented.  Note that a comma ‘,’ in this column indicates an ‘and’.

Table 3-4-:  PDF Object Requirements

| PDF Object/Filter | Producer | Consumer | Reference |
| --- | --- | --- | --- |
| ‘ASCIIHexDecode’ Filter | PROH | PROH | [pdf] Section (3.3.1) |
| ‘ASCII85Decode’ Filter | PROH | PROH | [pdf] Section (3.3.2) |
| ‘LZWDecode’ Filter | PROH | PROH | [pdf] Section (3.3.3) |
| ‘RunLengthDecode’ Filter | PROH | PROH | [pdf] Section (3.3.4) |
| Incremental Updates | PROH | PROH | [pdf] Section (3.4.5) |
| Functions | PROH | PROH | [pdf] Section (3.9) |
| Files | PROH | PROH | [pdf] Section (3.10) |
| Graphics State | PROH | PROH | [pdf] Section (4.3) |
| Path objects | PROH | PROH | [pdf] Section (4.4) |
| ‘DeviceGray’ Color Space | PROH | PROH | [pdf] Section (4.5.3) |
| ‘DeviceRGB’ Color Space | PROH | PROH | [pdf] Section (4.5.3) |
| ‘DeviceCMYK’ Color Space | PROH | PROH | [pdf] Section (4.5.3) |
| Pattern Color Space | PROH | PROH | [pdf] Section (4.5.5) |
| Separation Color Space | PROH | PROH | [pdf] Section (4.5.5) |
| DeviceN Color Space | PROH | PROH | [pdf] Section (4.5.5) |
| Pattern Objects | PROH | PROH | [pdf] Section (4.6) |
| Inline Image Objects | PROH | PROH | [pdf] Section (4.8.6) |
| Form Xobjects | PROH | PROH | [pdf] Section (4.9) |
| Postscript Xobjects | PROH | PROH | [pdf] Section (4.10) |
| Text Objects | PROH | PROH | [pdf] Section (5) |
| Transparency | PROH | PROH | [pdf] Section (7) |
| ‘CCITTFaxDecode’ Filter (Image Profile <G4>) | REQ | REQ | [pdf] Section (3.3.5) |
| File Header | REQ | REQ | [pdf] Section (3.4.1) |
| Cross-Reference Table | REQ | REQ | [pdf] Section (3.4.3) |
| File Trailer | REQ | REQ | [pdf] Section (3.4.4) |
| Document Catalog | REQ | REQ | [pdf] Section (3.6.1) |
| Page Tree Nodes | REQ | REQ | [pdf] Section (3.6.2) |
| Page Objects | REQ | REQ | [pdf] Section (3.6.2) |
| Content Streams | REQ | REQ | [pdf] Section (3.7.1) |
| Resource Dictionaries | REQ | REQ | [pdf] Section (3.7.2) |
| Image XObjects | REQ | REQ | [pdf] Section (4.8) |
| ‘FlateDecode’ Filter (Image Profile <FLATE>) | OPT | REQ | [pdf] Section (3.3.3) |
| ‘JBIG2Decode’ Filter (Image Profile <JBIG2>) | OPT | OPT | [pdf] Section (3.3.6) |
| ‘DCTDecode’ Filter (Image Profile <JPEG>) | OPT | REQ | [pdf] Section (3.3.7) |
| Encryption Dictionary ‘Standard’ Encryption (Security Profile <STD-ENC>) | OPT | OPT | [pdf] Section (3.5) |
| Encryption Dictionary PPK Encryption (Security Profile <PPK-ENC>) | OPT | OPT | [pdf-ppk] Section (3) |
| ‘DeviceGray’ Color Space (Color Profile <GRAY>) | OPT | REQ | [pdf] pg. 182 |
| ‘DeviceRGB’ Color Space (Color Profile <RGB>) | OPT | REQ | [pdf] pg. 184 |
| ‘Lab’ Color Space (Color Profile <LAB>) | OPT | REQ | [pdf] pg. 187 |
| ‘ICCBased’ Color Space (Color Profile <ICC>) | OPT | OPT | [pdf] pg. 189 |
| ‘Indexed’ Color Space (Color Profile <IDX>) | OPT | REQ | [pdf] pg. 199 |
| Masked Images (Image Profile <MASK>) | OPT | REQ | [pdf] Section (4.8.5) |
| Interactive Form Dictionary and Annotation Field Dictionary and Signature Dictionary (Security Profile <DIG-SIG>) | OPT | OPT | [pdf] Section (8.6.1-3) [pdf-ppk] Section (2) |
| Cached Objects | OPT | REQ | Section 3.4 |
| Tiling | REQ | REQ | Section 3.3.11.3 |

## PDF Field Specification

The following list describes the object field values of the REQUIRED and OPTIONAL PDF objects in PDF/is.  The numbers in ‘( )’s refer to section numbers in the PDF Specifications [pdf], unless otherwise noted.  ‘AS SPECIFIED’ refers to [pdf] unless otherwise noted.

### ’PDF/is’ object

A new ‘PDF Name Registry’ (See [pdf] – Appendix E) object that is REQUIRED for a PDF/is document.  The existence of this dictionary object is the one and only way to determine if the PDF in question is a PDF/is.  Spec:

Table 3-5-:  PDF/is Object

| Field | Type | Specification |
| --- | --- | --- |
| ‘Fis_Profiles’ | Array of Numeric Objects | REQUIRED: An array consisting of [MAJ_VER MIN_VER IMAGES SECURITY MEMORY] |
| ‘Encrypt’ | Dictionary | MUST have same value as ‘Encrypt’ field in the ‘Document Trailer’. See [pdf] table 3.12 for specification. |
| ‘Root’ | Dictionary | MUST have same value as ‘Root’ field in the ‘Document Trailer’. See [pdf] Table 3.12 for specification. |
| ‘Info’ | Dictionary | MUST have same value as ‘Info’ field in the ‘Document Trailer’. See [pdf] Table 3.12 for specification. |
| ‘ID’ | Array | MUST have same value as ‘ID’ field in the ‘Document Trailer’. See [pdf] Table 3.12 for specification. |
| ‘Fis_NextPage’ | Dictionary | REQUIRED: An Indirect Object Reference to the first ‘Page’ object. |

See [pdf] Section 3.2.5 for definition of an ‘Array Object’.  See [pdf] Section 3.2.2 for definition of a ‘Numeric Object’.

#### Fis_Profiles Key

##### MAJ_VER:

The ‘major’ version number of this PDF/is specification to which the Producer conforms to at the time the Document was created.  The ‘major’ version of this specification is currently ‘0’.

##### MIN_VER:

The ‘minor’ version number of this PDF/is specification to which the Producer conforms to at the time the Document was created.  The ‘minor’ version of this specification is currently ‘54’.

##### IMAGES, SECURITY:

Each value in the array MUST be a ‘Numeric Integer Object’ (See [pdf] Section 3.2.2) that is the sum of all of the Integer equivalents of the binary ‘Bit Positions’ for the Profiles that are Implemented in the Document, as indicated under the appropriate section below.  The ‘Bit Positions’ are numbered from 1 (low-order) to 32 (high-order).  A ‘1’ in a ‘Bit Position’ indicates the Profile in indicated.  All other Bit Positions for each element MUST be 0.  Note that PDF Numeric Integer Objects in fact are represented in signed twos-complement form.

For example, to indicate that the IMAGES Profiles ‘FLATE’ (bit position 3 or 100 binary) and ‘MASK’ (bit position 5, or 10000 binary), the value of ‘20’ (10100 binary) should be used as the value for the ‘IMAGES’ field.

The Producer of the Document MUST NOT Implement a Profile that is not indicated in this field.  The Producer of the Document MAY Implement all Profiles indicated in this field, but is NOT REQUIRED.

Rationale:  Since this object must be Implemented at the beginning of the Document, it may not be known for certain which Profiles will be Implemented.  This field is an advisory indicator to a Consumer as to which Profiles they MUST Support in order to be able to render the Document for certain.  If all Profiles indicated are not Supported, the Document may still be rendered if a non-Supported Profile is indicated but is not actually Implemented in the Document.

Note that even though a Profile is higher in the Image Profile tree it SHOULD NOT be indicated in this object unless that feature is Implemented in the document.  For example, if the document contained ‘FLATE’ (FlateDecode) images but no ‘JPEG’ (DCTDecode) images, only Profile ‘FLATE’ should be indicated.

Table 3-6-:  PDF/is Object ‘IMAGES’ Element

| Profile | Bit Position |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| <JBIG2> | 1 |  |  |  |  |
| <JP2K> | 2 |  |  |  |  |

Table 3-7-:  PDF/is Object ‘SECURITY’ Element

| Profile | Bit Position |
| --- | --- |
| <STD-ENC> | 1 |
| <PPK-ENC> | 2 |
| <DIG-SIG> | 3 |

-

-

##### MEMORY:

A ‘Numeric Object’ that is the decimal value of the minimum amount of cache memory the Consumer will need to cache all objects necessary to render any particular page.  This memory MUST be available for PDF/is data file caching and MUST not be part of any image processing or page buffer memory.

The value specified for ‘MEMORY’ is in Megabytes and is in addition to a base memory requirement of 2 Megabytes (2^21 bytes).

The value of the memory requirement MUST be agreed upon between the Producer and the Consumer before the Document is generated.  This value is usually the minimum of the cache memory available to either the Producer or the Consumer.  The usage of this memory is to cache objects as specified in the “Object Lifetime” section of this specification.  It should be noted that an ‘Image XObjects’ data stream typically won’t be ‘cached’ into this memory since these streams can often be rendered into a page buffer as they are received, even if masked.  This is true since all image masks and color profile data MUST occur in the Document before the ‘Image XObject’s that reference them.

##### Example

An example of the PDF/is object for a Document containing a CalRGB color space (Profile <RGB>), masked (Profile <MASK>), JPEG image (Profile <JPEG>) that’s Standard encrypted (Profile <STD-ENC>) would look like this:

1 0 obj

<<

/Fis_Profiles [0 54 24 0]

/Encrypt 2 0 R

/Root 3 0 R

/Info 4 0 R

/Fis_NextPage 5 0 R

>>

endobj

### ‘FlateDecode’ Filter

See [pdf] Section 3.3.3, [rfc1950], and [rfc1951].

Table 3-8-:  FlateDecode Filter

| Field | Specification |
| --- | --- |
| <All Fields> | AS SPECIFIED |

### ‘CCITTFaxDecode’ Filter

See [pdf] Section 3.3.5, [t.4], and [t.6].  Note that only Group 4 images are Supported by PDF/is, see ‘K’, below.

Table 3-9-:  CCITTFaxDecode Filter

| Field | Specification |
| --- | --- |
| ‘K’ | MUST have a value of -1. |
| ‘EndOfLine’ | AS SPECIFIED |
| ‘EncodedByteAlign’ | AS SPECIFIED |
| ‘Columns’ | AS SPECIFIED |
| ‘Rows’ | AS SPECIFIED |
| ‘EndOfBlock’ | AS SPECIFIED |
| ‘BlackIs1’ | AS SPECIFIED |
| ‘DamagedRowsBeforeError’ | AS SPECIFIED |

### ‘JBIG2Decode’ Filter

See [pdf] Section 3.3.6, [jbig2], and [t.89].

Table 3-10-:  JBIG2Decode Filter

| Field | Specification |
| --- | --- |
| <All Details> | AS SPECIFIED, except as noted below. |

- The Producer MUST Implement ONLY JBIG2 Profile 1 (0x00000101 BASE) OR Profile 4 (0x00000104 Medium lossy/lossless arithmetic) of [t.89].

- All Consumers MUST support at least “Level 2” Memory (See [t.89], Table 1, Item 18).

- The Producer MUST adhere to the Function and Memory constraints as specified in [t.89].

### ‘DCTDecode’ Filter

See http://partners.adobe.com/asn/developer/acrosdk/docs/filefmtspecs/PDFReference.pdf[pdf] Section 3.3.7, [ps-jpeg], [ps], and [jpeg].  PDF/is supports both the JPEG Baseline DCT and Extended sequential DCT compressed image formats.

Table 3-11-:  DCTDecode Filter

| Field | Specification |
| --- | --- |
| <All Details> | AS SPECIFIED, except as noted below. |

- Images MUST NOT have interleaved scans.

- Images MUST NOT be encoded using ‘Progressive JPEG’.

- The Consumer MUST adhere to the Memory requirements specified in Section 11 “RAM Requirements” of [ps-jpeg] for the Consumers Supported image resolution(s).

### File Trailer

See [pdf] Table 3.12.

Table 3-12-:  File Trailer

| Field | Specification |
| --- | --- |
| ‘Size’ | AS SPECIFIED |
| ‘Prev’ | PROHIBITED |
| ‘Root’ | AS SPECIFIED |
| ‘Encrypt’ | AS SPECIFIED, but PROHIBITED if the Document is to be PDF/X-3 Compliant (See [pdf-x3]). |
| ‘Info’ | REQUIRED. |
| ‘ID’ | REQUIRED. MUST use a pseudo-random number in place of ‘File Size’ when generating this value. See [pdf] Section 9.3 for guidelines on how to generate this value. Rationale: Using a random number in place of file size is due to the requirements of using this field in generating the encryption key for the ‘standard encryption’ algorithm ([pdf] Step 5 of Algorithm 3.2, pg. 78): file size will not be known at the time this field is needed. |

### Encryption Dictionary

See [pdf] Table 3.13 and [pdf-ppk] Table 3.

Note that if a Document is Standard encrypted (Profile <STD-ENC>), the ‘ID’ field of the File Trailer MUST be calculated before the Encryption Dictionary is written.  The ‘ID’ MUST then be cached until the ‘File Trailer’ is written.

Table 3-13-:  Encryption Dictionary

| Field | Specification |
| --- | --- |
| ‘Filter’ | REQUIRED: MUST have a value of ‘Standard’ if <STD-ENC> is Implemented, otherwise AS SPECIFIED. |
| ‘V’ | MUST have a value of ‘2’. |
| ‘Length’ | AS SPECIFIED |
| ‘R’ | AS SPECIFIED |
| ‘O’ | REQ if <STD-ENC>, PROH otherwise |
| ‘U’ | REQ if <STD-ENC>, PROH otherwise |
| ‘P’ | REQ if <STD-ENC>, PROH otherwise |
| ‘SubFilter’ | MUST be ‘adbe.pkcs7.s4’ if <PPK-ENC>, PROH otherwise |
| ‘Recipients’ | REQ if <STD-ENC>, PROH otherwise |

### Document Catalog

See [pdf] Table 3.16.

Table 3-14-:  Document Catalog

| Field | Specification |
| --- | --- |
| ‘Type’ | AS SPECIFIED |
| ‘Version’ | AS SPECIFIED |
| ‘Pages’ | AS SPECIFIED |
| ‘PageLabels’ | IGNORED |
| ‘Names’ | IGNORED. |
| ‘Dests’ | IGNORED. |
| ‘ViewerPreferences’ | IGNORED. |
| ‘PageLayout’ | IGNORED. |
| ‘PageMode’ | IGNORED. |
| ‘Outlines’ | IGNORED. |
| ‘Threads’ | IGNORED. |
| ‘OpenAction’ | IGNORED. |
| ‘AA’ | IGNORED. |
| ‘URI’ | IGNORED. |
| ‘AcroForm’ | REQ if <DIG-SIG>, PROH otherwise |
| ‘Metadata’ | IGNORED. |
| ‘StructTreeRoot’ | IGNORED. |
| ‘MarkInfo’ | AS SPECIFIED., See below. |
| ‘Lang’ | IGNORED. |
| ‘SpiderInfo’ | IGNORED. |
| ‘OutputIntents’ | PROHIBITED. |

‘Tagged PDF’ ([pdf] Section 9.7) MAY be used to enter searchable text in a Document.  A Producer MAY apply Optical Character Recognition (OCR) on the images of each page in a Document to generate searchable text.  Since ‘Tagged PDF’ information can be used for Document searching and does not affect printed output, its usage is OPTIONAL for the Producer and MAY be IGNORED by a conforming Consumer.

Page Tree Nodes

See [pdf] Table 3.17.

Table 3-15-:  Page Tree Nodes

| Field | Specification |
| --- | --- |
| ‘Type’ | AS SPECIFIED |
| ‘Parent’ | AS SPECIFIED |
| ‘Kids’ | AS SPECIFIED |
| ‘Count’ | AS SPECIFIED |
| <All ‘Page Object’ Fields, see [pdf] Table 3.18> | PROHIBITED |

### Page Objects

See [pdf] Table 3.18.

Table 3-16-:  Page Objects

| Field | Specification |
| --- | --- |
| ‘Type’ | AS SPECIFIED |
| ‘Parent’ | AS SPECIFIED |
| ‘LastModified’ | AS SPECIFIED |
| ‘Resources’ | MUST NOT be inherited |
| ‘MediaBox’ | MUST NOT be inherited |
| ‘CropBox’ | MUST NOT be inherited. If Present, the TrimBox MUST NOT extend beyond the boundaries of the CropBox. |
| ‘BleedBox’ | AS SPECIFIED. If Present, the TrimBox MUST NOT extend beyond the boundaries of the BleedBox. |
| ‘TrimBox’ | REQUIRED. |
| ‘ArtBox’ | PROHIBITED. |
| ‘BoxColorInfo’ | PROHIBITED. |
| ‘Contents’ | AS SPECIFIED. |
| ‘Rotate’ | MUST NOT be inherited |
| ‘Group’ | PROHIBITED. |
| ‘Thumb’ | IGNORED. |
| ‘B’ | IGNORED. |
| ‘Dur’ | IGNORED. |
| ‘Trans’ | IGNORED. |
| ‘Annots’ | IGNORED. |
| ‘AA’ | IGNORED. |
| ‘Metadata’ | IGNORED. |
| ‘PieceInfo’ | IGNORED. |
| ‘StructParents’ | IGNORED. |
| ‘ID’ | IGNORED. |
| ‘PZ’ | IGNORED. |
| ‘SeparationInfo’ | PROHIBITED. |
| ‘Fis_NextPage’ | REQUIRED: An Indirect Object Reference to the next ‘Page’ object or a ‘Page Node’ if this is the last page. |

The size of the current page can be determined by the value of the ‘MediaBox’.  The value associated with ‘MediaBox’ is an array of the coordinates of the page rectangle in default user space units (1/72 of an inch).  An 8.5 X 11 inch page, oriented Portrait, would be:

/MediaBox [0 0 612 792]

### Content Stream Operators

See [pdf] Table 4.1.  A conforming Consumer MUST be able to parse the Content Stream operators listed below, but only must be able to act upon the operators that are not listed as IGNORED.

Table 3-17-:  Content Stream Operators

| Operators | Specification | Reference |
| --- | --- | --- |
| ‘q’ | AS SPECIFIED | [pdf] Table 4.7 |
| ‘Q’ | AS SPECIFIED | [pdf] Table 4.7 |
| ‘cm’ | MUST be [Sx 0 0 Sy Tx Ty], See Below | [pdf] Table 4.7 |
| ‘Do’ | AS SPECIFIED | [pdf] Table 4.34 |
| ‘MP’ | IGNORED | [pdf] Table 9.8 |
| ‘DP’ | IGNORED except for ‘Tiling operator’ and ‘Cache operator’, see below | [pdf] Table 9.8 |
| ‘BMC’ | IGNORED | [pdf] Table 9.8 |
| ‘BDC’ | IGNORED | [pdf] Table 9.8 |
| ‘EMC’ | IGNORED | [pdf] Table 9.8 |
| ‘BX’ | AS SPECIFIED | [pdf] Table 3.20 |
| ‘EX’ | AS SPECIFIED | [pdf] Table 3.20 |
| <All elements between a ‘BMC’ or ’BDC’ and an ‘EMC’> | IGNORED | [pdf] Table 9.8 |
| <All other Operators> | PROHIBITED |  |

#### cm:

See [pdf] Table 4.7 for definition of ‘cm’ operator.

Given:

Wi = Width (X-direction) of the Image in inches.

Hi = Height (Y-direction) of the Image in inches.

Xi = Horizontal translation, in inches, from the left edge of the page to the top of the image.

Yi = Vertical translation, in inches, from the top edge of the page to the top of the image.

The Producer MUST ensure that the following is true:

Sx = Wi * 72

Sy = Hi * 72

Tx = Xi * 72

Ty = Yi * 72

#### Do:

#### See [pdf] Table 4.34 for definition of ‘Do’ operator.

Given:

Img = The ‘Image XObject’ associated with the ‘Do’ operator.

Cm = The current ‘cm’ operation in effect for ‘Img’.

Wp = ‘Width’ field of ‘Img’.

Hp = ‘Height’ field of ‘Img’.

Sx = ‘Sx’ value of ‘Cm’.

Sy = ‘Sy’ value of ‘Cm’.

The following MAY be assumed by either the Producer or the Consumer:

Rx = (Wp * 72 / Sx) = The resolution, in the X-direction, of ‘Img’, in dots per inch.

Ry = (Hp * 72 / Sy) =  The resolution, in the Y-direction, of ‘Img’, in dots per inch.

DP:

See [pdf] Table 9.8 for a definition of the ‘DP’ Operator.

The only ‘Marked Content’ flags that are not ignored in a PDF/is Document are the ‘Tiling Operator’ and the ‘Cache operator’.

The Tiling Operator:

Tiling facilitates the creation of a complex series of images on a PDF/is page to a Producer or Consumer that may be memory constrained and unable to otherwise create or display the page.  If the Producer of the Document is able to determine that the current page will violate the cache memory constraints of the Consumer; the Consumer MUST break up the current page into non-overlapping regions to be displayed (Tiling) or free up resources using the ‘Cache Operator’ (see below).  Tiling is specified in the content stream of the page.  Tiling indicates that all previous images or masks in the stream up to the “Tiling operator” do not overlay, and are not overlaid by, any images or masks that follow in the stream.

To indicate that a new ‘tile’ is beginning, the content stream MUST contain the following operator syntax, exactly as shown:

/Fis_tile <</Fis_tile [X Y]>> DP

Where:

X:  The user-space relative direction with regard to the X-axis that will not be overlapped.

Y:  The user-space relative direction with regard to the Y-axis that will not be overlapped.

X and Y MUST only have values of ‘-1’, ‘0’, or ‘1’.

A value of ‘0’ indicates that the entire width of this axis will not be overlapped by images and masks to follow in the content stream.  This value would be used if each ‘Tile’ were a full width ‘band’ of the page or it was the last tile on a row or column.  For example, if a band spanned the width (the X axis) of the page, the ‘X’ value should be ‘0’.

A value of ‘-1’ indicates that all remaining images and masks in the content stream have lesser values of this axis.  For example, for a band that spanned the top of a page, the ‘Y’ value would be ‘-1’ (since Y values decrease as you move down the page).

A value of ‘1’ indicates that all remaining images and masks in the content stream have greater value for this axis.  For example, for a band that spanned the left edge of a page, the ‘X’ value would ‘1’ (X axis values increase as you move to the right).

It should be noted that tiles may progress from the top to the bottom, bottom to top, left to right, or right to left as necessary.  The order and progression of the Tiles SHOULD be determined by either the capabilities of the Producer or the Consumer.  The specification of how this should be done is outside the scope of this specification.

See the following examples to help illuminate this feature.  The shaded area is the area that is specified to be non-overlapping by the parameters of the /Fis_tile operator of the tile in Bold.  The number before the colon is the order in which the tile appears in the content stream.

Example #1, Tile #1 is detailed:

| 1: [1, -1] | 2: [0, -1] |
| --- | --- |
| 3: [0, -1] | 4: [0, 0] |

Example #2:

| 3: [1, 0] | 6: [1, 0] | 9: [0, 0] |
| --- | --- | --- |
| 2: [1, 1] | 5: [1, 1] | 8: [0, 1] |
| 1: [1, 1] | 4: [1, 1] | 7: [0, 1] |

Example #3:

1: [0, -1]

2: [0, -1]

3: [0, 0]

A ‘Tile Operator’ MUST only occur between displayed images on a page, and MUST NOT occur at the beginning and/or end of the content stream.  A ‘Tile Operator’ occurring immediately before any Do operators in the content stream MUST be IGNORED.  A ‘Tile Operator’ that occurs after all Do operators MUST also be IGNORED.

To illustrate this feature:

A page with two tiles, each tile running across the page, might have a content stream that looks like this:

500 0 0 100 25 767 cm	% region of first ’tile’.  500 units wide, 100 units high,

% 25 units from top left corner (page is 11” tall, 792 units high).

/Im1 Do			% Display image in first band.

/Fis_tile <</Fis_tile [0 -1]>> DP	% 'Tile Operator' .

500 0 0 100 25 667 cm	% Second region, does not overlap first band-- notice Y offset of % 667 does not overlap bottom of first band.

/Im2 Do			% Display image in second band.

##### The Cache Operator:

The ‘Cache Operator’ allows the Producer of the Document to specify that certain ‘cached’ objects (See ‘Cached Objects’ section in this specification) may be released from the cache at a certain point in the content stream.  See ‘Cache Release’ section in this document for use of this operation.  This operation would allow a Consumer to Discard specified objects to free resources for image operations.  This operator has the following syntax:

/Fis_cache <</Fis_cache [OBJECTS]>> DP

Resource Dictionaries

See [pdf] Table 3.21.

The Resource Dictionary MUST reference all Image XObjects and ColorSpaces that are used on the current page.  The position of the image objects, their masks, and color spaces with respect to each other is defined in the Image XObject section of this specification.

Table 3-18-:  Resource Dictionaries

| Field | Specification |
| --- | --- |
| ‘ExtGState’ | PROHIBITED. |
| ‘ColorSpace’ | AS SPECIFIED. |
| ‘Pattern’ | PROHIBITED. |
| ‘Shading’ | PROHIBITED. |
| ‘XObject’ | AS SPECIFIED. |
| ‘Font’ | PROHIBITED. |
| ‘ProcSet’ | ‘Text’ Proc Sets PROHIBITED, all others AS SPECIFIED. |
| ‘Properties’ | IGNORED. |

### Color Spaces

See [pdf] Section 4.5.

Table 3-19-:  Color Spaces

| Field | Specification |
| --- | --- |
| ‘Lab’ | AS SPECIFIED |
| ‘DeviceGray’ | AS SPECIFIED |
| ‘DeviceRGB’ | AS SPECIFIED, but see below. |
| ‘DeviceCMYK’ | PROHIBITED |
| ‘CalGray’ | PROHIBITED |
| ‘CalRGB’ | PROHIBITED |
| ‘ICCBased’ | AS SPECIFIED, but see below. |
| ‘Indexed’ | AS SPECIFIED, but see below. |
| ‘Pattern’ | PROHIBITED |
| ‘Separation’ | PROHIBITED |
| ‘DeviceN’ | PROHIBITED |

#### DeviceRGB Color Space

The Producers  who uses ‘DeviceRGB’ color space, and Consumers that interpret them, SHOULD Implement the color values assuming ‘DeviceRGB’ to be the ‘sRGB’ standard IEC 61966-2-1 (1999-10) (See [srgb]).

#### ICCBased Color Space

See [pdf] Table 4.16

Note that to minimize ICC profile data size, FlateDecode Filter compression MAY be used.

It should also be noted that a Document with an ICCBased color space can be decoded by a Consumer that does not support ICCBased color spaces.  In this case, the Consumer should use the ‘Alternate’ color space as defined by the Field of the same name.

Table 3-20:  ICCBased Color Space

| Field | Specification |
| --- | --- |
| ‘N’ | MUST be either ‘1’ or ‘3’. |
| ‘Alternate’ | MUST be either ‘/DeviceGray’, ‘/DeviceRGB’, or ‘/Lab’ |
| ‘Range’ | AS SPECIFIED. |
| ‘Metadata’ | AS SPECIFIED. |

#### Indexed Color Space

An Index may be applied to any other supported color space, although it has limited value when applied to ‘DeviceGray’.  The Producer of a Document that used an Indexed color space MAY apply the FlateDecode filter to the color space data to minimize data size.

### Image XObjects

See [pdf] Table 4.35 for description of the following table.

Table 3-21-:  Image XObjects

| Field | Specification |
| --- | --- |
| ‘Type’ | MUST be ‘XObject’ |
| ‘Subtype’ | MUST be ‘Image’ |
| ‘Width’ | AS SPECIFIED |
| ‘Height’ | AS SPECIFIED |
| ‘ColorSpace’ | AS SPECIFIED, and see below. |
| ‘BitsPerComponent’ | AS SPECIFIED |
| ‘Intent’ | PROHIBITED. |
| ‘ImageMask’ | AS SPECIFIED, if Profile <MASK> |
| ‘Mask’ | AS SPECIFIED, if Profile <MASK>, and see below. |
| ‘SMask’ | PROHIBITED. |
| ‘Decode’ | AS SPECIFIED. |
| ‘Interpolate’ | MUST be ‘true’ |
| ‘Alternates’ | IGNORED |
| ‘Name’ | IGNORED. |
| ‘StructParent’ | IGNORED. |
| ‘ID’ | IGNORED. |
| ‘OPI’ | PROHIBITED. |
| ‘Metadata’ | IGNORED. |

- An ‘ImageMask’, if indicated in an Image XObject, MUST appear in the Document before the Image XObject that references it.

- If an ‘ICCBased’ or ‘Indexed’ color space is indicated in an Image XObject, the data for the color space MUST appear in the Document before the Image XObject that references it.

### Masked Images

See [pdf] Section 4.8.5.

Table 3-22-:  Masked Images

| Field | Specification |
| --- | --- |
| <All Fields> | AS SPECIFIED |

### Interactive Form Dictionary

See [pdf] Table 8.47.

Table 3-23-:  Interactive Form Dictionary

| Field | Specification |
| --- | --- |
| ‘Fields’ | MUST be an indirect object of an ‘Annotation Field Dictionary’. |
| ‘NeedAppearances’ | PROHIBITED |
| ‘SigFlags’ | MUST be ‘3’ |
| ‘CO’ | PROHIBITED |
| ‘DR’ | PROHIBITED |
| ‘DA’ | PROHIBITED |
| ‘Q’ | PROHIBITED |

### Annotation Field Dictionary

See [pdf] Tables 8.10 & 8.49.  This dictionary consists of entries from both a ‘Annotation Dictionary (Table 8.10) and a ‘Field Dictionary’ (Table 8.49).

Table 3-24-:  Annotation Field Dictionary

| Field | Specification |
| --- | --- |
| ‘Type’ | MUST be ‘Annot’ |
| ‘Subtype’ | MUST be ‘Widget’ |
| ‘Contents’ | IGNORED |
| ‘P’ | IGNORED |
| ‘Rect’ | MUST be ‘[0 0 0 0]’ |
| ‘NM’ | IGNORED |
| ‘F’ | IGNORED |
| ‘BS’ | IGNORED |
| ‘Border’ | IGNORED |
| ‘AP’ | IGNORED |
| ‘AS’ | IGNORED |
| ‘C’ | IGNORED |
| ‘CA’ | IGNORED |
| ‘T’ | IGNORED |
| ‘Popup’ | IGNORED |
| ‘A’ | IGNORED |
| ‘AA’ | IGNORED |
| ‘StructParent’ | IGNORED |
| ‘FT’ | MUST be ‘Sig’ |
| ‘Parent’ | PROHIBITED. |
| ‘Kids’ | PROHIBTED. |
| ‘T’ | AS SPECIFIED. |
| ‘TU’ | AS SPECIFIED. |
| ‘TM’ | IGNORED. |
| ‘Ff’ | MUST be ‘1’. |
| ‘V’ | MUST be an indirect object to a ‘Signature Dictionary’. |
| ‘DV’ | IGNORED. |
| ‘AA’ | IGNORED. |

### Signature Dictionary

See [pdf] Table 8.60 and [pdf-ppk] Table 2.

The Digital Signature format MUST only be in the ‘Raw Format’, see [pdf-ppk] Section 2.2.

Table 3-25-:  Signature Dictionary

| Field | Specification |
| --- | --- |
| ‘Type’ | MUST be ‘Sig’ |
| ‘Filter’ | AS SPECIFIED. |
| ‘SubFilter’ | MUST be ‘adbe.x509.rsa_sha1’ |
| ‘Name’ | AS SPECIFIED. |
| ‘Reason’ | AS SPECIFIED. |
| ‘Location’ | AS SPECIFIED. |
| ‘M’ | AS SPECIFIED. |
| ‘ByteRange’ | PROHIBITED (Implies all bytes in the Document with the exclusion of the bytes represented by the value of the ‘Cert’ field. See [pdf] for this field) |
| ‘Contents’ | AS SPECIFIED. |
| ‘Cert’ | AS SPECIFIED. |
| ‘R’ | AS SPECIFIED. |
| ‘V’ | AS SPECIFIED. |
| ‘ADBE_Build’ | AS SPECIFIED. |
| ‘ADBE_AuthType’ | AS SPECIFIED. |
| ‘ADBE_PwdTime’ | AS SPECIFIED. |

### Document Information Dictionary

See [pdf] Table 9.2.

Table 3-26-:  Document Information Dictionary

| Field | Specification |
| --- | --- |
| ’Title’ | REQUIRED* |
| ‘Author’ | REQUIRED* |
| ‘Subject’ | AS SPECIFIED |
| ‘Keywords’ | AS SPECIFIED |
| ‘Producer’ | AS SPECIFIED |
| ‘Producer’ | AS SPECIFIED |
| ‘CreationDate’ | REQUIRED* |
| ‘ModDate’ | REQUIRED* |
| ‘Trapped’ | REQUIRED, MUST be either ‘TRUE’ or ‘FALSE’. Partially Trapped files are PROHIBITED. |
| ‘GTS_PDFXVersion’ | PROHIBITED if Profile <STD-ENC> or <PPK-ENC> is Implemented; otherwise MUST be “(PDF/X-3:2002)” |

*Some fields in this object are required due to the specification of PDF/X-3 (See [pdf-x3]).

## Object Lifetime

Some Consumer’s may be limited in the amount of storage they may have to cache the Document as it’s received from the Producer.  This storage limitation may prohibit the Consumer from holding the entire Document before beginning to render the first page.  To facilitate this storage constraint, PDF/is has a mechanism of “object lifetime”.  This mechanism defines how long an object must be held in storage before it is no longer needed.

If a Document can be fully maintained in the Consumer’s storage, the Document’s Cross-Reference table should be used to access objects as they are needed.  In this case, the Consumer should follow the parsing model as spelled out in the PDF Reference [pdf].

If a Document cannot be fully maintained within the Consumers storage, the Document MUST be linearly parsed and the following parsing rules MUST be adhered to:

- Documents MUST be parsed in order, from beginning to end.

- The first object, the “PDF/is” object MUST never be Discarded.

- All non-IGNORED objects that are referenced from other Cached objects MUST not be Discarded.

- All Cached non-Page-Relative Objects (See Terminology) MUST be not be Discarded until the Document rendering is complete.

- All ‘Page-Relative’ Objects MUST NOT be Discarded until the next ‘Page’ object or the ‘Document Catalog’ is reached; unless the object is held in the ‘Cache Hold’ (See next section).  This also implies that all rendering of the current page MUST be complete before “reaching” the next ‘Page’ object or the ‘Document Catalog’.

- If rendering of a “Band” (See Section 3.3.11.3) is complete, objects that are referenced in the ‘content stream’ of the completed ‘band’ may be Discarded, if the object is not referenced in the remainder of the ‘content stream’ and is not ‘Cached’ (See next section).

## Cached Objects

If a ‘Page-Relative’ object MAY be used on more than one page, it will be necessary to specify the object as ‘Cached’.  Once an object is cached, it no longer has to abide by ‘Object Lifetime’ requirements 5 and 6.  This will allow an object to be used throughout the Document that otherwise would be discarded.

An object that is held in the Consumers cache by the ‘Cache Hold’ mechanism MUST be maintained in the cache until one of the following conditions is met:

The ‘Cache Release’ mechanism is invoked.

The ‘Document Catalog’ is reached.

### Cache Hold

To specify that an object should not be discarded once the current page is rendered, the object to be ‘cached’ should have the following ‘Dictionary Object’ (See [pdf] Section 3.2.6):

/Fis_Cache [ ]

### Cache Release

To release an object from the Consumer’s memory; the following ‘Dictionary Object’ MUST be placed in the ‘Page Object’ of the first page in which the object is no longer needed.  For example, if the object is question was first found on page 1 and was last used on page 3, the ‘Cache Release’ should occur in the ‘Page Object’ for page 4.

/Fis_Cache   [OBJECTS]

Where:

OBJECTS:  is an array (contained in ‘[]’s) of indirect object references of the objects that were previously cached and are no longer needed.  Indication of an object number that was never cached MUST be ignored.

Example:

3 0 obj

<<

/Fis_Cache []			%First object to be cached.

…

>>

endobj

…

7 0 obj				%Second object to be cached.

<<

/Fis_Cache []

…

>>

endobj

…				%One or more Page objects in between.

45 0 obj

<<

/Type /Page			%Page object

/Fis_Cache [3 0 R 7 0 R]	%Objects 3 and 7 are no longer needed.

...

>>

endobj

# Conformance Requirements

This section specifies the conformance requirements for Consumers and Producers.

## Producer conformance requirements

In order to conform to this specification, a Document Producer:

- MUST specify the version of PDF (See [pdf] Section 3.4.1) as being ‘PDF 1.4’.

- MUST place the ‘PDF/is’ object as the first object in the PDF.

- MUST place any ‘Encryption Dictionary’ object as the second object in the PDF/is Document, if the Document is encrypted.

- MUST NOT include any private ‘PDF Name Registry’ values/objects (See [pdf] – Appendix E) that affect printed output.

- MUST place the objects:  ‘Interactive Form Dictionary’, ‘Field Dictionary’ and ‘Digital Signature’ object as the last three objects (in that order) in the Document, if the Document is Digitally Signed.  Note that in a situation where the Consumer cannot cache the entire document before rendering, the detection of a valid or invalid Digital Signature will only occur after rendering of the entire Document.

- MUST ensure that there is at least one Forward-Reference to each object.  The only object that does not have to follow this rule is the ‘PDF/is Object’. Rationale:  This will aid the Consumer with knowing which objects will need to be cached and which can be ignored.

- MUST ensure that all objects appear in the PDF AFTER the object in which they are first referenced (Satisfied by Requirement 6) and BEFORE the next ‘Page Object’ unless the object is a Cached Object (See Section 3.4).

- MUST ensure that all object identifiers ([pdf] Section 3.2.9) start at the beginning of a line.

- MUST ensure that all ‘endobj’ keywords ([pdf] Section 3.2.9) start at the beginning of a line.

- MUST ensure that all ‘stream’ data ([pdf] Section 3.2.7) does not contain a line beginning with the word “endstream”, aside from the required “endstream” that delimits the end of the stream.

## Consumer conformance requirements

In order to conform to this specification, a Document Consumer:

- MUST Support all of the REQUIRED PDF/is objects.

- MUST Interpolate images up or down in resolution, as required, to properly match the Documents image resolution(s) to the Consumer’s device capabilities.

- MAY ignore all IGNORED objects that the Producer added to the PDF/is Document.

- MUST indicate to the Producer, which OPTIONAL features the Consumer Supports.

- MUST abide by the “Object Lifetime” rules in Section 3.5 if unable to Cache the entire Document.

## File Layout

Given that a Document is fully compliant with this specification, a PDF/is Document will, nominally, take on the following format:

Table 4-27-:  File Layout

|  | Object |
| --- | --- |
| A | ‘PDF/is’ object. |
| B | Encryption Object (if Profile <STD-ENC> XOR <PPK-ENC>) |
| C | Document Information Dictionary |
| D | Page object for page 1 |
| E | Resources for page 1 |
| F | Content object for page 1 |
| G | Color Space(s) for page 1 |
| H | Image Mask(s) for page 1 |
| I | Image XObject(s) for page 1 |
| J | [Repeat D – I for all remaining pages, in order] |
| K | Document Catalog |
| L | Page Node(s) |
| M | Interactive Form Dictionary (if Profile <DIG-SIG>) |
| N | Annotation Field Dictionary (if Profile <SIG-SIG>) |
| O | Signature Dictionary (if Profile <DIG-SIG>) |
| P | File Trailer |

# Issues

1) In the interest of blind-interchange, should JBIG2 rendering support be required of all consumers?

The only other 'Optional' features in the spec, as it now stands are:

A) Standard Encryption.

B) PPK Encryption.

C) Digital Signaturing.

Here are my feelings on each of these:

A - May require licensing of RC4 encryption software.  Standard encryption requires a target device that can query and take a password as input: this may not be practical for all types of devices.  This should remain an option.

B - May require licensing of encryption software.  PPK encryption requires that the consumer have a public key that the producer can retrieve via IPP.  A 'profile' isn't necessary for this feature:  if the producer is unable to get the consumer's public key, the producer will not be able to use this feature.

C - A Digital Signature may be applied to any document.  The consumer doesn't have to validate the signature if they don't wish to, or are not able to do so.

2) Should the 'DeviceRGB' color space be defined to be some version of sRGB?

3) Should we "hard code" a buffer size for the memory cache value (Section 3.3.1.1.4)?

4) A proposal from Xerox that I'm not sure I can answer right now:

"General comment about DID and Annotation fields, and the possibility

of using one or the other as a mechanism for including a "fax transmit

header" or sender-uri value, per Sec. 9.5 in IPPFAX 1.0 Protocol Draft.

Right now the recommendation is to burn it into the image data, but the

DID or Annotation field could be used for this attribute value--consider

text to this effect in 3.3.19 or 3.3.17."

Sample PDF/is PDFs

The ‘source’ of all of the sample documents in this section can be viewed with any text editor but should only be modified with a binary editor, as the stream data contained therein is not compatible with text editors.  Comments on the format of the documents are contained within the documents themselves.

All of the samples are different versions of the same document.

1:  The first sample is an unencrypted, single page, ‘CCITTFaxDecode’ masked, ‘DCTDecode’ color ICCBased color space foreground image with a ‘FlateDecode’ gray scale Indexed ICCBased color space background image.  The images use ‘FlateDecode’ compression on the ‘ICCBased’ and ‘Indexed’ Color Spaces.

ftp://pwg.org/pub/pwg/QUALDOCS/SamplePDFax/base-02.pdf

2:  The next sample has been encrypted with ‘Standard’ encryption.  The ‘user’ password is ‘12345’; the ‘owner’ password is ‘54321’.  The document has also been Digitally Signed:  the document will fail a digital signature check since it has been tampered with.  To see the digital signature in Acrobat (or Acrobat Reader), select the ‘Signature’ tab on the left side of the screen.

ftp://pwg.org/pub/pwg/QUALDOCS/SamplePDFax/stdEncryptSigned-02.pdf

# Normative References

[pdf]

Adobe Systems, “PDF Reference, third edition, Adobe Portable Document Format Version 1.4”, Addison-Wesley, December 2001, http://partners.adobe.com/asn/developer/acrosdk/docs/filefmtspecs/PDFReference.pdf.  Also see errata:  http://partners.adobe.com/asn/developer/acrosdk/docs/PDF14errata.txt.

[pdf-ppk]

Pravetz, J., “PDF Public-Key Digital Signature and Encryption Specification”, Version 3.2, Adobe Systems, September 2001, http://partners.adobe.com/asn/developer/pdfs/tn/ppk_pdfspec.pdf

[pdf-x3]

ISO/TC 130, “Complete exchange suitable for colour-managed workflows (PDF/X-3)”, ISO 15930-3:2002, September 2002.

[ps-jpeg]

Adobe Systems Incorporated, “Supporting the DCT Filters in PostScript Level 2”, November 1992, http://partners.adobe.com/asn/developer/pdfs/tn/5116.DCT_Filter.pdf

[ps]

Adobe Systems Incorporated, “PostScript Language Reference third edition”, Addiseon-Wesley, 1999, http://partners.adobe.com/asn/developer/pdfs/tn/PLRM.pdf.  Also see errata:  http://partners.adobe.com/asn/developer/pdfs/tn/PSerrata.txt.

[ifx]

Moore, Songer, Hastings, Seeler “IPPFAX/1.0 Protocol” PWG Proposed Standard P0.13, 2002, ftp://pwg.org/pub/pwg/QUALDOCS/pwg-ifx-ippfax-P13-021122.pdf

[ifx-req]

Moore, P., “IPP Fax transport requirements”, October 16, 2000, ftp://pwg.org/pub/pwg/QUALDOCS/requirements/ifx-transport-requirements-01.pdf

[t.4]

ITU-T Recommendation T.4, “Standardization of group 3 facsimile apparatus for document transmission”, October 1997

[t.6]

ITU-T Recommendation T.6, “Facsimile coding schemes and coding control functions for group 4 facsimile apparatus”, November 1988

[t.89]

ITU-T Recommendation T.89, “Application profiles for Recommendation T.88 – Lossy/lossless coding of bi-level images (JBIG2) for facsimile”, September 2001

[rfc2119]

Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, September 2000, ftp://ftp.rfc-editor.org/in-notes/pdfrfc/rfc2911.txt.pdf.

[rfc2911]

Hastings, Herriot, deBry, Isaacson, Powell, “Internet Printing Protocol/1.1: Model and Semantics”, September 2000, ftp://ftp.rfc-editor.org/in-notes/pdfrfc/rfc2911.txt.pdf.

[jpeg]

JTC 1/SC 29, “Information technology – Digital compression and coding of continuous-tone images:  Requirements and guidelines”, ISO/IEC 10918-1:1994, 1994.

[jbig2]

JTC 1/SC 29, “Information technology – Lossy/lossless coding of bi-level images”, ISO/IEC 14492:2001, December 2001.

[rfc1950]

Deutsch, Gailly, “ZLIB Compressed Data Format Specification version 3.3”, May 1996, ftp://ftp.isi.edu/in-notes/rfc1950.pdf.

[rfc1951]

Deutsch, “DEFLATE Compressed Data Format Specification version 1.3”, May 1996, ftp://ftp.isi.edu/in-notes/rfc1951.pdf.

[srgb]

International Electrotechnical Commission (IEC), IEC/3WD 61966-2.1, “Colour Measurement and Management in Multimedia Systems and Equipment, Part 2.1:  Default RGB Colour Space – sRGB”, 1999.

Informative References

[rfc2542]

Masinter , "Terminology and Goals for Internet Fax", RFC2542, March 1999, ftp://ftp.rfc-editor.org/in-notes/pdfrfc/rfc2542.txt.pdf.

[ifx-goals]

Klyne, Shockey, "Additional Goals for Quality Document Transfer", October 1999, ftp://ftp.pwg.org/pub/pwg/QUALDOCS/Internet-Drafts/draft-klyne-qualdoc-goals-02.txt.

Revision History (to be removed when standard is approved)

| Revision | Date | Author | Notes |
| --- | --- | --- | --- |
| 1 | 10/9/02 | Rick Seeler, Adobe Systems | Initial version |
| 2 | 10/23/02 | Rick Seeler, Adobe Systems |  |
| 3 | 11/19/02 | Rick Seeler, Adobe Systems |  |
| 4 | 11/22/02 | Rick Seeler, Adobe Systems |  |

# Contributors

John Pulera 	-  Minolta		mailto:jpulera@minolta-mil.com

Gail Songer 	-  Peerless		mailto:gsonger@peerless.com

Tom Hastings 	-  Xerox			mailto:hastings@cp10.es.xerox.com

Rob Buckley	-  Xerox			mailto:rbuckley@crt.xerox.com

Lloyd McIntyre	-  Xerox			mailto:Lloyd.McIntyre@pahv.xerox.com

# Acknowledgments

Kari Poysa	- Xerox			mailto:Kari.Poysa@usa.xerox.com

# Author’s Address

Rick Seeler

Adobe Systems Incorporated

321 Park Ave., E13

San Jose, CA 95110

Phone: 1+408 536-4393

Fax:	1+408 537-8077

e-mail:	mailto:rseeler@adobe.com

# Appendix A

## Intellectual Property Statement – Adobe Systems Incorporated

The following statement is in addition to the Intellectual Property Statement in the PDF Reference (See [pdf] Section 1.4).

Patent Clarification Notice Specific to Use of PDF for IPP G4 Protocol

Adobe has a number of patents covering technology that is disclosed in the Portable Document Format (PDF) Specification, version 1.4 and later, as documented in PDF Reference and associated Technical Notes (the “PDF Specification”). Adobe desires to promote the use of PDF as the file format for a future,

IPP G4 Protocol to be proposed, recommended, finalized and published by the IEEE Printer Working

Group (the “IPP G4 Standard”).

This Patent Clarification Notice is in addition to the permissions statement set forth in Section 1.4 of the

PDF Reference which shall also apply to Adobe’s contribution to the IPP G4 Standard.

Accordingly, Adobe agrees to provide a Royalty Free License to all Essential Claims solely for the purpose of implementing the IPP G4 Standard. Adobe and the IEEE Printer Working Group will identify and establish, within the final, published release of the IPP G4 Standard, a process whereby implementers of the IPP G4 Standard can request and obtain the above license.

No license shall be extended to those implementing only draft versions of the IPP G4 Standard.

A “Royalty Free License” shall mean a license that:

- shall be available to all implementers of the IPP G4 Standard worldwide, whether or not members of the IEEE Printer Working Group;

- shall extend to all Essential Claims owned or controlled by Adobe and its Affiliates;

- shall not be conditioned on payment of royalties, fees or other consideration except as described in (iv) and (v) below;

- may be conditioned on a grant of a reciprocal license on identical terms to all Essential Claims owned or controlled by the licensee and its Affiliates; and

- may include reasonable, customary terms relating to operation or maintenance of the license relationship including but not limited to the following: choice of law, dispute resolution, and patent notices.

“Essential Claims” shall mean all claims in any patent or patent application, in any jurisdiction in the world, that (A) Adobe and/or its Affiliates own and (B) that would be necessarily infringed by implementation of the IPP G4 Standard. A claim is necessarily infringed hereunder only when a licensee can prove that it is not possible to avoid infringing it because there is no non-infringing alternative for implementing the required portions of the IPP G4 Standard. Existence of a non-infringing alternative shall be judged based on the state of the art at the time a licensee implements the IPP G4 Standard.

The following are expressly excluded from and shall not be deemed to constitute Essential Claims:

- any claims other than as set forth above even if contained in the same patent as Essential Claims; and

- claims that would be infringed only by

  - portions of an implementation that are not required by the IPP G4 Standard

  - enabling technologies that may be necessary to make or use any product or portion thereof that complies with the IPP G4 Standard but are not themselves expressly set forth in the IPP G4 Standard; or

  - the implementation of technology developed elsewhere and merely incorporated by reference into the IPP G4 Standard.

For purposes of the Essential Claims definition, the “IPP G4 Standard” shall be deemed to include only architectural and interoperability requirements and shall not include any implementation examples or any other material that merely illustrates the requirements of the IPP G4 Standard.

An “Affiliate” of a first entity is a second entity that is controlled (greater than 50%) by, in control of, or under common control with the first entity.