Internet FAX RFC Abstracts

Date: 12/01/00

File: ftp://ftp.pwg.org/pub/pwg/QUALDOCS/rfcs/internet-fax-rfc-abstracts-001201.doc

RFC 2301 - File Format for Internet Fax, March 1998

This document describes the TIFF (Tag Image File Format)

representation of image data specified by the ITU-T Recommendations

for black-and-white and color facsimile. This file format

specification is commonly known as TIFF-FX. It formally defines

minimal, extended and lossless JBIG modes (Profiles S, F, J) for

black-and-white fax, and base JPEG, lossless JBIG and Mixed Raster

Content modes (Profiles C, L, M) for color and grayscale fax. These

modes or profiles correspond to the content of the applicable ITU-T

Recommendations. Files formatted according to this specification use

the image/tiff MIME Content Type.

RFC 2302 - Tag Image File Format (TIFF) - image/tiff MIME Sub-type Registration, March 1998

This document describes the registration of the MIME sub-type

image/tiff.  The baseline encoding is defined by [TIFF].  This

document refines an earlier sub-type registration in RFC 1528

[TPC.INT].

RFC 2303 - Minimal PSTN address format in Internet Mail, March 1998

This memo describes the MINIMAL addressing method to encode PSTN

addresses into e-mail addresses and the standard extension mechanism

to allow definition of further standard elements. The opposite

problem, i.e. to allow a traditional numeric-only PSTN device user to

access the e-mail transport service, is not discussed here.

RFC 2304 - Minimal FAX address format in Internet Mail, March 1998

This memo describes the MINIMAL addressing method and standard

extensions to encode FAX addresses in e-mail addresses, as required

in reference [13]. The opposite problem, i.e. to allow a traditional

numeric-only fax device user to access the e-mail transport service,

is not discussed here.

RFC 2305 - A Simple Mode of Facsimile Using Internet Mail, March 1998

This specification provides for "simple mode" carriage of facsimile

data over the Internet.  Extensions to this document will follow.

The current specification employs standard protocols and file formats

such as TCP/IP, Internet mail protocols [1, 2, 3], MIME [4, 16, 17],

and TIFF for Facsimile [5,6,19].  It can send images not only to

other Internet-aware facsimile devices but also to Internet-native

systems, such as PCs with common email readers which can handle MIME

mail and TIFF for Facsimile data.  The specification facilitates

communication among existing facsimile devices, Internet mail agents,

and the gateways which connect them.

RFC 2306 - Tag Image File Format (TIFF) - F Profile for Facsimile, March 1998

This document describes in detail the definition of TIFF-F that is

used to store facsimile images.  The TIFF-F encoding has been

folklore with no standard reference definition before this document.

RFC 2426 - vCard MIME Directory Profile - September 1998

This memo defines the profile of the MIME Content-Type [MIME-DIR] for

directory information for a white-pages person object, based on a

vCard electronic business card. The profile definition is independent

of any particular directory service or protocol. The profile is

defined for representing and exchanging a variety of information

about an individual (e.g., formatted and structured name and delivery

addresses, email address, multiple telephone numbers, photograph,

logo, audio clips, etc.). The directory information used by this

profile is based on the attributes for the person object defined in

the X.520 and X.521 directory services recommendations. The profile

also provides the method for including a [VCARD] representation of a

white-pages directory entry within the MIME Content-Type defined by

the [MIME-DIR] document.

RFC 2506 - Media Feature Tag Registration Procedure, March 1999

Recent Internet applications, such as the World Wide Web, tie

together a great diversity in data formats, client and server

platforms, and communities.  This has created a need for media

feature descriptions and negotiation mechanisms in order to identify

and reconcile the form of information to the capabilities and

preferences of the parties involved.

Extensible media feature identification and negotiation mechanisms

require a common vocabulary in order to positively identify media

features.  A registration process and authority for media features is

defined with the intent of sharing this vocabulary between

communicating parties. In addition, a URI tree is defined to enable

sharing of media feature definitions without registration.

This document defines a registration procedure which uses the

Internet Assigned Numbers Authority (IANA) as a central registry for

the media feature vocabulary.

Please send comments to the CONNEG working group at <ietf-

medfree@imc.org>.  Discussions of the working group are archived at

<URL: http://www.imc.org/ietf-medfree/>.

RFC 2530 - Indicating Supported Media Features Using Extensions to DSN and MDN, March 1999

There is a need in Internet mail and Internet fax for a recipient to

indicate the media features it supports so that messages can be

generated by senders without exceeding the recipient's abilities.

This memo describes a format for generating Message Disposition

Notifications [RFC2298] and Delivery Status Notifications [RFC1894]

which contain such information.  This information can be used by

senders to avoid exceeding the recipient's capabilities when sending

subsequent messages.

RFC 2531 - Content Feature Schema for Internet Fax, March 1999

Obsoleted by RFC 2879

This document defines a content feature schema that is a profile of

the media feature registration mechanisms [1,2,3] for use in

performing capability identification between extended Internet fax

systems [5].

This document does not describe any specific mechanisms for

communicating capability information, but does presume that any such

mechanisms will transfer textual values.  It specifies a textual

format to be used for describing Internet fax capability information.

RFC 2532 - Extended Facsimile Using Internet Mail, March 1999

This document describes extensions to "Simple Mode of Facsimile Using

Internet Mail" [RFC2305] and describes additional features, including

transmission of enhanced document characteristics (higher resolution,

color) and confirmation of delivery and processing.

These additional features are designed to provide the highest level

of interoperability with the existing and future standards-compliant

email infrastructure and mail user agents, while providing a level of

service that approximates the level currently enjoyed by fax users.

The IETF has been notified of intellectual property rights claimed in

regard to some or all of the specification contained in this

document.  For more information consult the online list of claimed

rights in <http://www.ietf.org/ipr.html>.

RFC 2533 - A Syntax for Describing Media Feature Sets, March 1999

Updated by RFC 2738 and by RFC 2938

A number of Internet application protocols have a need to provide

content negotiation for the resources with which they interact [1].

A framework for such negotiation is described in [2], part of which

is a way to describe the range of media features which can be handled

by the sender, recipient or document transmission format of a

message.  A format for a vocabulary of individual media features and

procedures for feature registration are presented in [3].

This document introduces and describes a syntax that can be used to

define feature sets which are formed from combinations and relations

involving individual media features.  Such feature sets are used to

describe the media feature handling capabilities of message senders,

recipients and file formats.

An algorithm for feature set matching is also described here.

RFC 2534 - Media Features for Display, Print, and Fax, March 1999

This specification defines some common media features for describing

image resolution, size, color, and image representation methods that

are common to web browsing, printing, and facsimile applications.

These features are registered for use within the framework of [REG].

RFC 2542 - Terminology and Goals for Internet Fax, March 1999

This document defines a number of terms useful for the discussion of

Internet Fax. In addition, it describes the goals of the Internet Fax

working group and establishes a baseline of desired functionality

against which protocols for Internet Fax can be judged. It

encompasses the goals for all modes of facsimile delivery, including

'real-time', 'session', and 'store and forward'.  Different levels of

desirability are indicated throughout the document.

RFC 2703 - Protocol-independent Content Negotiation Framework, September 1999

A number of Internet application protocols have a need to provide

content negotiation for the resources with which they interact.  MIME

media types [1,2] provide a standard method for handling one major

axis of variation, but resources also vary in ways which cannot be

expressed using currently available MIME headers.

This memo sets out terminology, an abstract framework and goals for

protocol-independent content negotiation, and identifies some

technical issues which may need to be addressed.

The abstract framework does not attempt to specify the content

negotiation process, but gives an indication of the anticipated scope

and form of any such specification.  The goals set out the desired

properties of a content negotiation mechanism.

RFC 2738 - Corrections to "A Syntax for Describing Media Feature Sets", Dec 1999

Updates 2533

In RFC 2533, "A Syntax for Describing Media Feature Sets", an

expression format is presented for describing media feature

capabilities using simple media feature tags.

This memo contains two corrections to that specification:  one fixes

an error in the formal syntax specification, and the other fixes an

error in the rules for reducing feature comparison predicates.

RFC 2879 - Content Feature Schema for Internet Fax (V2), August 2000

Obsoletes RFC 2531

This document defines a content media feature schema for Internet

fax.

It is a profile of the media feature registration mechanisms [1,2,3]

for use in performing capability identification between extended

Internet fax systems [5].  It replaces and updates the feature schema

defined in RFC 2531.

RFC 2880 - Internet Fax T.30 Feature Mapping, August 2000

This document describes how to map Group 3 fax capability

identification bits, described in ITU T.30 [6], into the Internet fax

feature schema described in "Content feature schema for Internet fax"

[4].

This is a companion to the fax feature schema document [4], which

itself defines a profile of the media feature registration mechanisms

[1,2,3], for use in performing capability identification between

extended Internet fax systems [5].

RFC 2912 - Indicating Media Features for MIME Content, September 2000

In "A Syntax for Describing Media Feature Sets", an expression format

is presented for describing media feature capabilities using simple

media feature tags.

This memo defines a Multipurpose Internet Mail Extensions (MIME)

'Content-features:' header that can be used to annotate a MIME

message part using this expression format, and indicates some ways it

might be used.

RFC 2913 - MIME Content Types in Media Feature Expressions, September 2000

In "A Syntax for Describing Media Feature Sets", an expression format

is presented for describing media feature capabilities using simple

media feature tags.

This memo defines a media feature tag whose value is a Multipurpose

Internet Mail Extensions (MIME) content type.  This allows the

construction of feature expressions that take account of the MIME

content type of the corresponding data.

RFC 2938 - Identifying Composite Media Features, September 2000

Updates 2533

In RFC 2533, an expression format is presented for describing media

feature capabilities as a combination of simple media feature tags.

This document describes an abbreviated format for a composite media

feature set, based upon a hash of the feature expression describing

that composite.