IEEE-ISTO

Printer Working Group

IPP Fax Project

Standard for IPPFAX/1.0 Protocol

Working Draft

Maturity:  Initial

Version 1.0

April 7, 2004

Abstract: This document specifies the IPPFAX/1.0 protocol.  The IPPFAX requirements [ifx-req] are derived from the requirements for Internet Fax [RFC2542].

In summary, IPPFAX is used to provide a synchronous, reliable exchange of image Documents between clients and servers.  The primary use envisaged of this protocol is to provide a synchronous image transmission service for the Internet.  Contrast this with the Internet FAX protocol specified in [RFC2305] and [RFC2532] that uses the SMTP mail protocol as a transport.

The IPPFAX/1.0 protocol is a specialization of the IPP/1.1 [RFC2911], [RFC2910] protocol supporting a subset of the IPP operations with increased conformance requirements in some cases, some restrictions in other cases, and some additional REQUIRED attributes.  The IPPFAX Protocol uses the ‘ippfax’ URL scheme (instead of the ‘ipp’ URL scheme) in all its operations.  Most of the new attributes defined in this document MAY be supported by IPP Printers as OPTIONAL extensions to IPP as well

An IPPFAX Printer object is called a Receiver.  A Receiver MUST support at least the PDF/is as specified in [PWG5102.3-2004] which is defined for the ‘application/pdf’ document format MIME type .  A Print System MAY be configured to support both the IPPFAX and IPP protocols concurrently, but each protocol requires separate Printer objects with distinct URLs.

This document is available electronically at: 			wd-ifx10-20040518.pdf, .doc

A version showing the changes from the previous version is available at: 	wd-ifx10-20040518-rev.pdf

The latest version of this specification is available at: 		ftp://pwg.org/pub/pwg/QUALDOCS/wd-ifx10-latest.pdf, .doc

Copyright (C) 2004, IEEE ISTO. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  The IPPFAX/1.0 Protocol

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

About the IEEE-ISTO

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit http://www.ieee-isto.org.

About the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:  http://www.pwg.org

Contact information:

IFX Web Page:  http://www.pwg.org/qualdocs

IFX Mailing List:  ifx@pwg.org

To subscribe to the ipp mailing list, send the following email:

1) send it to majordomo@pwg.org

2) leave the subject line blank

3) put the following two lines in the message body:

subscribe ifx

end

Implementers of this specification are encouraged to join the IFX Mailing List in order to participate in any discussions of clarifications or review of registration proposals for additional names.

Contents

1 Introduction	7

1.1 Typical exchange	8

2 Terminology	8

2.1 Conformance Terminology	8

2.2 Other Terminology	9

3 IPPFAX Model	10

3.1 Printer Object Relationships	10

3.2 A Printer object with multiple URLs	11

4 Common IPPFAX Operation Attribute Semantics	11

4.1 printer-uri (uri) operation attribute	11

4.2 version-number parameter	12

4.3 ippfax-version (type2 keyword) operation attribute	12

5 IPPFAX Printer Description Attributes	12

5.1 printer-uri-supported (1setOf uri)	13

5.2 ipp-versions-supported (1setOf type2 keyword)	13

5.3 ippfax-versions-supported (1setOf type2 keyword)	14

5.4 operations-supported (1setOf type2 enum)	14

5.5 document-format-supported (1setOf mimeMediaType)	15

5.6 document-format-version-supported (1setOf text(127))	15

5.7 digital-signatures-supported (1setOf type2 keyword)	15

5.8 pdl-override-supported (type2 keyword)	15

6 IPPFax Job Description Attributes	16

6.1 sending-user-vcard (text(MAX))	16

6.2 receiving-user-vcard (text(MAX))	16

6.3 xxx-supplied attributes	17

7 IPPFAX Operations	17

7.1 Required Operations and Features	17

7.2 Get-Printer-Attributes	18

7.3 Print-Job	18

7.3.1 Operation Attributes	19

7.3.2 Job Template Attributes	22

7.3.3 Delivery Confirmation using the Print-Job response	24

7.3.4 Originator identifier image	24

7.4 Cancel-Job operation	24

7.5 Get-Job-Attributes	24

7.6 Get-Jobs	24

8 Security considerations	25

8.1 Data Integrity and authentication	25

8.2 Data Privacy (encryption)	25

8.3 uri-authentication-supported (1setOf type2 keyword)	26

8.4 uri-security-supported (1setOf type2 keyword)	27

8.5 Using IPPFAX with TLS	28

8.6 Access control	29

8.7 Reduced feature set	29

9 Attribute Syntaxes	30

10 Status codes	30

11 Conformance Requirements	30

11.1 Operation Conformance Requirements	30

12 IPPFAX URL Scheme	32

12.1 IPPFAX URL Scheme Applicability and Intended Usage	33

12.2 IPPFAX URL Scheme Associated IPPFAX Port	33

12.3 IPPFAX URL Scheme Associated MIME Type	33

12.4 IPPFAX URL Scheme Character Encoding	33

12.5 IPPFAX URL Scheme Syntax in ABNF	34

12.6 IPPFAX URL Examples	34

12.7 IPPFAX URL Comparisons	35

13 IANA Considerations	35

14 References	36

14.1 Normative	36

14.2 Informative	36

15 Authors’ addresses	39

16 Appendix B: vCard Example	41

17 Revision History (to be removed when standard is approved)	42

Table of Tables

Table 1 - Printer Description attributes conformance requirements	13

Table 2 - Summary of Job Description attributes	16

Table 3 - Print-Job operation attributes	19

Table 4 - IPPFAX Defaults for unsupported Job-Template Attributes	22

Table 5 - Authentication Requirements	26

Table 6 - Digest Authentication Conformance Requirements	27

Table 7 - Security (Integrity and Privacy) Requirements	27

Table 8 - Transport Layer Security (TLS) Conformance Requirements	28

Table 9 - Conformance for IPPFax/1.0 Operations	31

# Introduction

This document specifies the IPPFAX/1.0 protocol.  The IPPFAX requirements [ifx-req] are derived from the requirements for Internet Fax [RFC2542].

In summary IPPFAX is used to provide a synchronous, reliable exchange of image documents between clients and servers.  The primary use envisaged of this protocol is to provide a synchronous image transmission service for the Internet.  Contrast this with the Internet FAX protocol specified in [RFC2305] and [RFC2532] that uses the SMTP mail protocol as a transport.

IPPFAX is primarily intended as a method of supporting a synchronous, secure, high quality document distribution protocol over the Internet.  It therefore discusses paper, pages, scanning and printing, etc. There is, however, no requirement that the input documents come from actual paper nor is there a requirement that the output of the process be printed paper.  The only conformance requirements are those associated with the exchange of data over the network.

The IPPFAX/1.0 protocol is a specialization of the IPP/1.1 [RFC2911], [RFC2910] protocol supporting a subset of the IPP operations with increased conformance requirements in some cases, some restrictions in other cases, and some additional REQUIRED attributes.  The IPPFAX Protocol uses the ‘ippfax’ URL scheme (instead of the ‘ipp’ URL scheme) for all operations.

An IPPFAX Printer object is called a Receiver.  A Receiver must support at least PDF/is [PWG5102.3-2004] which is defined for the ‘application/pdf’ document format MIME type..

An IPPFAX client is called a Sender.  The user of the Sender is called the Sending User.  The Sending User either (1a) loads the Document into the Sender or (1b) causes the Sender to generate the Document data by means outside the scope of this standard, (2) indicates the Receiver’s network location, and (3) starts the exchange.

The target market for an IPPFAX receiver is a midrange imaging device that can support the minimum memory requirements that are required by the data format PDF/is, but the image format is structured in such a way that the Receiver is not required to include a disk or other permanent storage.

IPPFax Senders and Receivers must support the operations, Get-Printer-Attributes, Print-Job, Get-Job-Attributes, and should support for authorized administrators Get-Jobs and Cancel-Job.  See Section 7

## Typical exchange

This section lists a typical exchange of information between a Sender and a Receiver using the four operations listed in section 1.1.

- The Sending User determines the network location of the Receiver (value of the “printer-uri” operation attribute) – see section 4.1.  This document does not specify how the Sending User does this.  Possible methods include directory lookup, search engines, business cards, network discovery protocols such as SLP, etc.  See Appendix E Generic Directory Schema of IPP/1.1 [RFC 2911].

- The Sending User either (1) loads the Document into the Sender or (2) causes the Sender to generate the Document data by means outside the scope of this document, indicates the Receiver’s network location and starts the exchange.

- The Sender can determine other PDF versions supported by the Receiver and the Sender can discover “media-supported” and “media-ready”.

- The Sender converts the document, if necessary, into PDF/is or another PDF subset depending on the Receiver’s capabilities. The PDF/is data format is described in detail in the “PDF Image-Streamable (PDF/is)” specification [PWG5102.3-2004].

- The Sender submits the document in a Print-Job request to the Receiver.  The Sender can include the sending user vCard[RFC2426, RFC2425]  and receiving user vCard in the Print-Job operations.

- The Receiver returns a Print-Job response to the Sender, who in turns informs the Sending-User.

- The Sender can use Get-Job-Attributes to check for successful job completion unless the Sending User requests otherwise.

# Terminology

This section defines the following additional terms that are used throughout this standard.

## Conformance Terminology

Capitalized terms, such as MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, MAY, NEED NOT, and OPTIONAL, have special meaning relating to conformance to this specification.  These terms are defined in [RFC2911] section 13.1 on conformance terminology, most of which is taken from RFC 2119 [RFC2119].  In order to help the reader compare and contrast the IPP and IPPFAX protocols, this document uses lower case “must”, “may” etc., to reproduce IPP Protocol conformance requirements for IPP clients and IPP Printer objects as stated in other documents.  If such reproduction in this document contradicts an IPP document, it is a mistake, and that IPP document prevails.

## Other Terminology

This standard defines a logical model of an IPPFAX interchange.  The following terms are introduced and capitalized in order to indicate their specific meaning:

IPP Protocol  The protocol defined in [RFC2911] and [RFC2910] and any IPP Protocol Extension document (see section 14).  For the IPP/1.1 Protocol each operation request must use the ‘ipp’ URL scheme.

IPPFAX Protocol  The protocol defined in this or a future revision document and any future extension document.  For the IPPFAX Protocol each operation request MUST use the ‘ippfax’ URL scheme (see section 4.1 and 12).  Unless a specific version number is appended to “IPPFAX”, such as “IPPFAX/1.0”, the term IPPFAX applies to all versions.

Printer object (or Printer)  A hardware or software entity that accepts protocol operation requests and returns protocol responses.  A Printer object MAY be: (1) an IPP Printer object or (2) an IPPFAX Printer object, DEPENDING ON IMPLEMENTATION  but MUST NOT be both (since they support some different operations and attributes and are really two different kinds of Print Services).  A Printer object MAY support multiple URLs with different security, authentication, and/or access control (see [RFC2911] sections 4.4.1, 4.4.2, 4.4.3, and 8).  However, each URL for a Printer object MUST support the same operations and attributes with the same values, except as restricted depending on the security, authentication, and/or access control implied by the URL.  In other words, each URL for a given Printer object is offering the same Print Service.

Note:  For brevity, this document uses the term “Receiver” instead of “IPPFAX Printer object”.  This document uses the term “Printer object” (and “Printer”) when the statement is intended to apply to a Printer object that MAY support the IPP Protocol or the IPPFAX protocol (but not both).

Print Service  The print functionality offered by a Printer object.  Several different Printer objects MAY offer the same Print Service. A Print Service MUST support only one printer object.

IPP Printer object  A Printer object that supports the IPP Protocol and offers the IPP Print Service (by definition).

Receiver  The Printer object that accepts IPPFAX protocol operations and receives the Document sent by the Sender.  A Receiver offers the IPPFAX Print Service (by definition).

Print System  All of the Printer objects on a single managed host network node.  A Print System MAY support IPP and IPPFAX protocols concurrently for a single output device (or multiple output devices), but each protocol requires separate Printer objects with distinct URLs.

client  A hardware and/or software entity that initiates protocol operation requests and accepts responses.  A client MAY be: (1) an IPP client, (2) an IPPFAX client, or (3) both.  However, this document uses the term “Sender”, instead of “IPPFAX client”.  This document uses the term “client” when the statement is intended to apply to a client that MAY support the IPP Protocol, the IPPFAX protocol, or both protocols.

IPP client  A client that uses the IPP Protocol to interact with an IPP Printer object.

Sender  A client that uses the IPPFAX Protocol to query a Receiver and transmit a Document to that Receiver.

Document  The electronic representation of a set of one or more pages that the Sender sends to the Receiver.

Sending User  The person interacting with the Sender.

Receiving User  The intended human recipient of the Document being sent by the Sender to the Receiver.

IPP Job  A job submitted by an IPP client to an IPP Printer object using the IPP Protocol.

IPPFAX Job  A job submitted by a Sender to a Receiver using the IPPFAX Protocol.

PDF/is The file format defined by [PWG5102.3-2004].

The terminology defined in [RFC2911], such as attribute, operation, request, response, operation attribute, Printer Description attribute, Job Description attribute, integrity, and privacy is also used in this document with the same capitalization conventions and semantics.

# IPPFAX Model

This sub-section defines the IPPFAX Model and its relationship to the IPP Protocol and Model.

## Printer Object Relationships

A Print System MAY support one or more Printer objects on a single network host.  RFC 2911 [RFC2911] defines the relationship between Printer objects and output devices to be many to many (see [RFC2911] section 2.1).  So one Printer object can represent one or more output devices and an output device can be represented by one or more Printer objects.  The same relationships hold for the IPPFAX Protocol so that the relationship between Receivers and output devices is many to many.

## A Printer object with multiple URLs

For a Printer object that has multiple URLs, the multiple URLs MUST only be aliases for the Printer object, not connections to different Print Services.  In other words, the semantics of operations and attributes accessed by the different URLs for a given Printer object MUST differ only in the security, authentication, and/or access control depending on the URL used.

The three parallel “printer-uri-supported” (1setOf uri), “uri-authentication-supported” (1setOf type2 keyword), and “uri-security-supported” (1setOf type2 keyword) Printer Description attributes (see [RFC2911] sections 4.4.1, 4.4.2, and 4.4.3, respectively) MUST contain the URLs, authentication, and security, respectively, supported by the Printer object.

# Common IPPFAX Operation Attribute Semantics

This section describes the IPPFAX/1.0 operation attribute semantics that are common to all operations.  IPPFAX/1.0 does not define any new operations.  Instead, IPPFAX/1.0 semantics are provided using existing IPP operations in [RFC2911],  with increased conformance requirements as specified in this document.

## printer-uri (uri) operation attribute

This operation attribute specifies the transfer path to the Receiver for the operation.  As in IPP/1.1, the client MUST supply the “printer-uri” operation attribute in every IPPFAX request (see [RFC2911] section 3.1.5).  For IPPFAX, the attribute value MUST be a URL using the ‘ippfax’ scheme (see section 12) specifying the Receiver’s network location.

The following is an example value of the target “printer-uri” operation attribute and “printer-uri-supported” Printer Description attribute:

ippfax://www.acme.com/ippfax-printers/printer5

As in IPP/1.1 [RFC2911] for each operation, the Receiver NEED NOT validate that the “printer-uri” operation attribute is present and that the value supplied by the Sender matches one of the Receiver’s “printer-uri-supported” Printer Description attribute (see section 5.1).  For URI matching rules see section 12.7.  If the Receiver does validate the “printer-uri” operation attribute and the URI value supplied does not match any value of the Receiver’s “printer-uri-supported” Printer Description attribute, the Receiver MUST reject the request, return the ‘client-error-attributes-or-values-not-supported’ status code, and return the attribute and value in the Unsupported Attributes Group.

## version-number parameter

This IPP/1.1 operation parameter ([RFC2911] section 3.1.8) specifies the major and minor version number of the IPP Protocol being used as part of the IPPFAX Protocol.  As in IPP/1.1, the Sender MUST supply this parameter in every request and the Receiver MUST return this parameter in every response.

For IPPFAX version 1.0 as specified in this document, the Sender MUST supply the IPP version number parameter with a value of  ‘1.1’ or a higher minor version number.

## ippfax-version (type2 keyword) operation attribute

The value of this operation attribute indicates the version of the IPPFAX Protocol and encoding that the Sender is requesting and the Receiver is returning.  The Sender MUST supply this operation attribute in every request and the Receiver MUST return this operation attribute in every response.  This operation attribute MUST be placed in the Operation Attributes Group immediately after the operation attributes whose order is specified in IPP/1.1 [RFC2911].  The semantics of the “ippfax-version” operation attribute are the same for the IPPFAX Protocol as the “version-number” parameter for IPP 1.1(see [RFC2911] section 3.1.8).

For IPPFAX version 1.0 as specified in this document, the Sender MUST supply the IPPFax version operation attribute with the keyword value of  ‘1.0’.

The Receiver MUST list the IPPFAX versions supported in the “ippfax-versions-supported” (1setOf type2 keyword) Printer Description attribute (see section 5.3).

The Sender MUST send and the Receiver MUST check both the IPP (see section 4.2) and IPPFAX version numbers supplied by the Sender in each request, not just the IPPFAX version number.

# IPPFAX Printer Description Attributes

This section defines the IPPFAX Printer Description attributes and the IPP Printer Description attributes whose semantics are augmented for IPPFAX.

Table 1 lists all the IPPFAX conformance requirements for IPP and IPPFAX Printer Description attributes whose semantics are defined in this document.

All Printer Description attributes not listed in Table 1 have the same conformance requirements as defined in IPP/1.1 [RFC2911] or other IETF or PWG standards track IPP documents.

See section 7.3.2 for the Receiver conformance requirements for the “xxx-supported”, “xxx-default”, and “xxx-ready” Job Template Printer attributes.

Table 1 - Printer Description attributes conformance requirements

| Attribute Name (attribute syntax) | IPP Fax Receiver support | Section |
| --- | --- | --- |
| printer-uri-supported (1setOf uri) * | MUST | 5.1 |
| ipp-versions-supported (1setOf type2 keyword) * | MUST | 5.2 |
| ippfax-versions-supported (1setOf type2 keyword) | MUST | 5.3 |
| operations-supported (1setOf type2 enum) * | MUST | 5.4 |
| document-format-supported (1setOf mimeMediaType) * | MUST | 5.5 |
| document-format-version-supported (1setOf text(127)) ** | MUST | 5.6 |
| digital-signature-supported (1setOf type2 keyword) ** | MUST | 5.7 |
| pdl-override-supported (type2 keyword) * | MUST | 5.8 |

* These IPP/1.1 attributes are defined in [RFC2911], but have enhanced semantics defined in this document.

** These IPP attributes are defined in [PWG 5100.7], but have enhanced or constrained semantics defined in this document.

## printer-uri-supported (1setOf uri)

This attribute (see [RFC2911] section 4.4.1) contains the set of target URIs that the Receiver supports, i.e., the URI values that a client can supply as values of the “printer-uri” target operation attribute in requests.  A Receiver MUST support this Printer Description attribute. This attrbribute MUST only contain URIs using the ‘ippfax’ scheme.

## ipp-versions-supported (1setOf type2 keyword)

This attribute (see [RFC2911] section 4.4.1.4) identifies the version or versions of the IPP encoding that this Receiver supports as part of the IPPFAX Protocol (rather than indicating that the Receiver supports the IPP Protocol), including major and minor versions, i.e., the version numbers for which this Receiver meets the conformance requirements.  The Receiver MUST support this Printer Description attribute.  The Receiver MUST compare the “version-number” parameter (see section 4.2), with the values of this attribute in order to determine whether the Printer supports the IPP version requested by the Sender as part of the IPPFAX Protocol.

Standard keyword values are (from [RFC2911]):

‘1.1’:  The IPPFAX operations meets encoding conformance requirements of IPP version 1/1 as specified in [RFC2911] and [RFC2910].

## ippfax-versions-supported (1setOf type2 keyword)

This attribute identifies the version or versions of the IPPFAX Protocol that this Receiver supports, including major and minor versions, i.e., the version numbers for which this Receiver meets the conformance requirements.  The support of this attribute indicates that this Printer object is a Receiver as opposed to a regular IPP Printer object

The Receiver MUST compare the “ippfax-version” operation attribute (see section 4.3) supplied by the Sender in each request, with the values of this attribute in order to determine whether the Receiver supports the IPPFAX version requested by the Sender.

Standard keyword values are:

‘1.0’:  Meets the conformance requirements of IPPFAX 1/0 as specified in this document.

## operations-supported (1setOf type2 enum)

This attribute (see [RFC 2911] section 4.4.15) identifies the set of supported operations for this Receiver and contained Job objects.  A Receiver MUST support this Printer Description attribute.

The values of this attribute MAY depend on the URL supplied in the “printer-uri” operation attribute and/or MAY depend on the authority of the authenticated requesting user.  For example, a Receiver that supports administrative operations MUST NOT support administrative operations for use by end users, but such a Receiver MAY return the administrative operation enums to end users.  See section 9 for conformance requirements for these operations.

A receiver MUST only support the following operations:

- get-printer-attributes

- print-job

- cancel-job

- get-jobs

- get-job-attributes

A receiver MUST NOT support any other operation.

## document-format-supported (1setOf mimeMediaType)

This attribute (see [RFC 2911] section 4.4.22) identifies which document formats the Receiver supports.  The Receiver MUST support this Printer Description attribute. Both the Sender and Receiver MUST only support ‘application/pdf’.

## document-format-version-supported (1setOf text(127))

This attribute (see [PWG 5100.7] section 7.8 ) identifies which PDF subsets the Receiver supports.  A Receiver MUST support this attribute and a Sender MAY support this attribute. Both the Sender and Receiver MUST support the ‘PDF/is-1.0’ subset of PDF. The Receiver MAY support other subsets of PDF and if it does then the Receiver MUST only list subsets that it fully supports.

## digital-signatures-supported (1setOf type2 keyword)

This attribute (see [PWG 5100.7] section 7.4) identifies which digital signature technologies are supported by the Receiver.  A Receiver MUST support this Printer Description attribute.

If the Receiver cannot validate the digital signature or if the digital signature fails to verify, then the Receiver MUST notify the Receiving User using an implementation specific method.

## pdl-override-supported (type2 keyword)

This attribute (see [RFC 2911] section 4.4.28) identifies Receiver implementation support for overriding document data instructions with IPPFax job attributes. A Receiver MUST support this printer subscription attribute with the value ‘attempted’. . A Receiver MUST attempt to override at least the media attribute.

# IPPFax Job Description Attributes

This section defines the IPPFAX Printer Description attributes and the IPP Printer Description attributes whose semantics are augmented for IPPFAX or are new to IPPFax.  .

Table 2 - Summary of Job Description attributes

| Attribute | Sender supplies * | Receiver supports |
| --- | --- | --- |
| sending-user-vcard (text(MAX)) | MAY | MUST |
| receiving-user-vcard (text(MAX)) | SHOULD | MUST |
| compression-supplied (type3 keyword) ** | MUST NOT | MUST |
| document-charset-supplied (charset) ** | MUST NOT | MUST |
| document-digital-signature-supplied (type2 keyword)** | MUST NOT | MUST |
| document-format-details-supplied (1setOf collection) ** | MUST NOT | MUST NOT |
| document-format-supplied (mimeMediaType)** | MUST NOT | MUST |
| document-format-version-supplied (text(127)) ** | MUST NOT | MUST |
| document-message-supplied (text(MAX))** | MUST NOT | MUST NOT |
| document-name-supplied (name (MAX)) ** | MUST NOT | MUST |
| document-natural-language-supplied (naturalLanguage)** | MUST NOT | MUST |

*Sender supplies as an operation attribute in a Print-Job operation.

** These IPP attributes are defined in [PWG 5100.7]

## sending-user-vcard (text(MAX))

This Job Description attribute identifies the Sending User in MIME vCard v3.0 [RFC2426, RFC2425] format (See Appendix B for a sample vCard).  The Receiver MUST support this job description attribute according to the vCard v3.0 specification and MUST populate it with the value of the corresponding Print-Job operation attribute.  The Receiver MUST support MAX (1023) octets of text.  However, the Receiver MAY ignore any image, logo, and sound parts of the vCard, in which case it MUST still accept the Print-Job request and return the ‘successful-ok-ignored-or-substituted-attributes’ status code (see [RFC2911] section 13.1.2.2). The Receiver MAY choose to use this information on a job start and end sheet (banner page) for the job.

## receiving-user-vcard (text(MAX))

This Job Description attribute identifies the intended Receiving User in MIME vCard v3.0 [RFC2426, RFC2425] format (See Appendix B for a sample vCard).   The Receiver MUST support this Job Description operation attribute and MUST populate it with the value of the corresponding Print-Job operation attribute.  The Receiver MUST support MAX (1023) octets of text.  However, the Receiver MAY ignore any image, logo, and sound parts of the vCard, in which case it MUST still accept the Print-Job request and return the ‘successful-ok-ignored-or-substituted-attributes’ status code (see [RFC2911] section 13.1.2.2).The Receiver MAY choose to use this information on a job start and end sheet (banner page) for the job.

## xxx-supplied attributes

An IPPFax Receiver implementation MUST supported compression-supplied, document-charset-supplied, document-digital-signature-supplied, document-format-supplied, document-format-version-supplied, document-name-supplied, and document-natural-language-supplied Job-Description attributes as defined in [PWG 5100.7]

An IPPFax Receiver MUST NOT implement document-format-details-supplied and document-message-supplied Job-Description attributes.

SHOULD WE INCLUDE Job-Progress attributes job-impressions-completed, job-media-sheets-completed, job-k-octets-processed  from RFC 2911? Nothing from RFC3381 applies

# IPPFAX Operations

An IPPFax Receiver implementation MUST support the Get-Printer Attributes, Print Job, Get-Job Attributes, Get-Jobs and Cancel-Job as defined in this section. An IPPFax Receiver MUST NOT support any other IPP operations.

An IPPFax Receiver MUST NOT support any optional job-template attributes features of IPP unless explicitly stated in this document. An IPPFax Receiver MAY support any optional operation attributes in the Print-Job operation and MAY support Job-Description attributes in Job Objects.

## Required Operations and Features

All IPPFax Senders and Receivers MUST support the following operations:

- Get-Printer-Attributes - If the document-format-version is not PDF/is or the media is not iso_a4_210x297mm or na_letter_8.5x11in, then the Sender MUST verify that the Receiver can support the alternate attributes. Rational: Using Get-Printer-Attributes would avoid rejection of the job which is important if the document data is very large.

- Print-Job - Sender MUST submit the IPPFAX job with a single document (Create-Job, Send-document and Send-URI and Print-URI MUST NOT be supported by Senders or Receivers).

- Get-Job-Attributes - The Sender MUST support and MUST use this operation to check for successful job completion unless the Sending User wishes otherwise. Job-History MUST be retained by the Receiver for at least 5 minutes after job completion. See 4.3.7.2 of RFC2911 for printer object Job-History discussion.

- Get-Jobs – Receivers MUST support this operation but only for authenticated Administrators or Operators.

- Job-Cancel – Receivers MUST support this operation but only for authenticated Administrators or Operators.

All IPPFax Senders and Receivers MUST NOT support any other IPP operations including job operations and administrative operation.

All IPPFax Receivers MUST support receiving PFD/is version 1.0 as defined in [PWG5102.3-2004].

All IPPFax Senders MUST support generating and transmitting PFD/is version 1.0 as defined in [PWG5102.3-2004].

## Get-Printer-Attributes

The Sender and Receiver MUST support the discovery of Receiver capabilities using the Get-Printer-Attributes operation.

See Section 5 IPPFAX Printer Description Attributes for required Printer Description Attributes for IPPFax Receivers.

## Print-Job

The Sender and Receiver MUST support creating IPPFAX Jobs using the Print-Job operation.  The Sender and Receiver MUST NOT support print by reference, i.e., MUST NOT support any other print operation, i.e. Create-Job, Send-Document, Print-URI and Send-URI operations.

### Operation Attributes

Table 3 lists the operation attributes for Print-Job operations for Senders, and Receivers. The Receiver MUST NOT support operations attributes defined in other IPP extension documents.

Table 3 - Print-Job operation attributes

| Operation attribute | Section | Sender supplies | Receiver Supports |
| --- | --- | --- | --- |
| attributes-charset (charset) |  | MUST | MUST |
| attributes-natural-language (naturalLanguage) |  | MUST | MUST |
| printer-uri (uri) | 4.1 | MUST | MUST |
| requesting-user-name (name(MAX)) |  | SHOULD | MUST |
| job-name (name(MAX)) |  | MAY | MUST |
| ipp-attribute-fidelity (boolean) | 7.3.1.1 | MUST with ‘true’ value | MUST |
| document-name (name(MAX)) * | 7.3.1.2 | MAY | MUST |
| compression (type3 keyword) * |  | MAY | MUST |
| document-format (mimeMediaType) * | 7.3.1.3 | MUST | MUST |
| document-format-version (type2 keyword) * | 7.3.1.4 | MUST3 | MUST |
| document-charset (charset) * | 7.3.1.5 | MAY | MUST |
| document-natural-language (naturalLanguage) * | 7.3.1.6 | MAY | MUST |
| document-digital-signature (type2 keyword) | 7.3.1.7 | MAY | MUST |
| job-k-octets (integer(0:MAX)) |  | MAY | MAY |
| job-impressions (integer(0:MAX)) |  | MAY | MAY |
| job-media-sheets (integer(0:MAX)) |  | MAY | MAY |
| sending-user-vcard (1setOf text(MAX)) | 6.1 | SHOULD | MUST |
| receiving-user-vcard (text(MAX)) | 6.2 | SHOULD3 | MUST |

* These IPPFax attributes MUST be copied to their corresponding xxx-supplied Job-Description attributes by the Receiver.

#### ipp-attribute-fidelity

This operation attribute (see [RFC2911] section 3.2.1.1) indicates whether or not the client requires the Printer to support all Job Template attributes and values supplied.  The Sender MUST supply this operation attribute in the Print-Job operations and the value MUST be ‘true’.  A Receiver MUST validate and support this operation attribute.

If the Sender does not supply this attribute or supplies the ‘false’ value, the Receiver MUST reject the operation, MUST return the ‘client-error-bad-request’ status code, and SHOULD return the ‘ipp-attribute-fidelity’ attribute name keyword in the Unsupported Attributes Group.

#### document-name (naturalLanguage)

A Sender MAY supply this operation attribute. A Receiver MUST support this operation attribute. The Receiver MUST copy the value of this attribute to the corresponding document-name-supplied Job Description attribute. (See section 5.2.8 of [PWG5100.7])

#### document-format (mimeMediaType)

This operation attribute (see [RFC2911] section 3.2.1.1) identifies the MIME Media Type of the document that the Sender is sending.  The Sender MUST supply this operation attribute in the Print-Job operation with a value of “application/PDF”.  A Receiver MUST validate that the value of attribute is “application/pdf”. The Receiver MUST copy the value of this attribute to the corresponding document-format-supplied Job Description attribute. (See section 5.2.5 of [PWG5100.7])

If the Sender does not supply this attribute, the Receiver MUST reject the operation, MUST return the ‘client-error-bad-request’ status code, and SHOULD return the ‘document-format’ attribute name keyword in the Unsupported Attributes Group

Because only one document-format MAY be supported, attribute coloring is not relevant for IPPFax.  If the Sender desires to send a different format, then it should use a different transmission protocol than IPPFax.

#### document-format-version (type2 keyword)

This operation attribute is defined in section 3.2.5.7 in [PWG5100.7].

This operation attribute identifies the type2 keyword of the subset of PDF.  The Sender MUST supply this operation attribute in the Print-Job operation to specify a subset of PDF.  A Receiver MUST support and validate this operation attribute. If the supplied document-format-version is not in the Receivers document-format-version-supported list then the Receiver MUST reject the job with a status code “client-error-document-fomat-not-supported”. The Receiver MUST copy the value of this attribute to the corresponding document-format-version-supplied Job Description attribute. (See section 5.2.6 of [PWG5100.7])

IPPFax Senders and Receivers MUST support PDF/is-1.0.

See section 5.6.

#### document-charset (charset)

A Sender MAY supply this operation attribute. A Receiver MUST support this operation attribute. The Receiver MUST copy the value of this attribute to the corresponding document-charset-supplied Job Description attribute. (See section 5.2.2 of [PWG5100.7])

#### document-natural-language (naturalLanguage)

A Sender MAY supply this operation attribute. A Receiver MUST support this operation attribute. The Receiver MUST copy the value of this attribute to the corresponding document-natural-language-supplied Job Description attribute. (See section 5.2.9 of [PWG5100.7])

#### document-digital-signature (type2 keyword)

A Sender MAY supply this operation attribute. A Receiver MUST support this operation attribute. The Receiver MUST copy the value of this attribute to the corresponding document-digital-signature-supplied Job Description attribute. (See section 5.2.3 of [PWG5100.7])

### Job Template Attributes

As in [RFC2911], the term “Job Template attribute” is actually up to four attributes:  the “xxx” Job attribute, and the “xxx-default”, “xxx-supported”, and possibly the “xxx-ready” Printer attributes.

As in [RFC2911], if a Receiver supports the “xxx” Job Template attribute, then it MUST support the corresponding “xxx-default” (if defined) and “xxx-supported” Printer attributes as well, and MAY support the “xxx-ready” attribute (if defined).

Senders MUST supply and Receivers MUST support the Job-Template attribute except “media”[RFC2911] job-template attribute section 7.3.2.1.  Senders MUST NOT supply and Receivers MUST NOT support any other Job-Template attributes.

Table 4 - IPPFAX Defaults for unsupported Job-Template Attributes

| Job Template attribute | IPPFax default behavior |
| --- | --- |
| copies (integer(1:MAX)) | 1 copy |
| finishings (1setOf type2 enum) | Administrator configuration |
| job-hold-until (type3 keyword ¦ name(MAX)) | ‘no-hold’ |
| job-priority (integer(1:100) | Administrator configuration |
| job-sheets (type3 keyword ¦ name(MAX)) | Administrator configuration |
| multiple-document-handling (type2 keyword) | No multiple document jobs |
| number-up (integer(1:MAX)) | 1 |
| orientation-requested (type2 enum) | Administrator configuration |
| page-ranges (1setOf rangeOfInteger(1:MAX)) | 1:MAX |
| print-quality (type2 enum) | Administrator’s choice |
| printer-resolution (resolution) | Administrator configuration |
| sides (type2 keyword) | Administrator configuration |

#### media (type2 keyword | name(MAX))

This Job Template attribute (see [RFC2911] section 4.2.11) identifies the medium to be used for all sheets of the job.  The Sender MUST supply and the Receiver MUST support the “media” Job Template attribute in Print-Job requests. The Receiver MUST support the “media-default”, and “media-supported” Printer attributes and SHOULD support the “media-ready” Printer attribute.

The Sender MUST supply Media Size Self Describing names defined in [PWG5101.1].

A Receiver MUST at least support  the sizes ‘na_letter_8.5x11in’ and ‘iso_a4_210x297mm’ and MUST be able to print on at least one of those two sizes. The Receiver MAY scale down at most 10% (PDF/is directives may prohibit this scaling for quality reasons), overflow to another page, or truncate.  If the Receiver does truncate then it MUST notify the Receiving User.  A Receiver MUST perform only isomorphic scaling.

A Sender SHOULD use PDF Crop boxes when the Sender knows that the imageable region is less than the media size.  If the crop box is the union of the lesser size of iso_a4_210x297mm and na_letter_8.5x11in minus 1/2 of an inch, then the Sender can be sure that the majority of Receivers can print the complete image without loss of data.  However, this does not eliminate that the possibility that data may be lost.

#### media-supported

The following standard keywords MUST be supported.  Any other paper sizes supported MUST use the self-describing names as defined in ([PWG5101.1]):

‘na_letter_8.5x11in’

‘iso_a4_210x297mm’

‘choice_iso_a4_210x297mm_na_letter_8.5x11in’ - represents both ‘na_letter_8.5x11in’ and ‘iso_a4_210x297mm’ and indicates that either is acceptable. See [PWG5100.7].

### Delivery Confirmation using the Print-Job response

The Sender knows when the Receiver has successfully received the entire Job when the Receiver returns the ‘successful-ok’ status code in the Print-Job Response.  The Sender MUST then inform the Sending User by means outside the scope of this standard that the Job has successfully been transmitted, unless the Sending User requests otherwise.

### Originator identifier image

Consistent with ITU-T T.30 facsimile, the Document Originator (generating application or Sender) MUST include an originator identifier image as required by PDF/is. [PWG5102.3-2004] section 7.1.

The Document Originator MUST include in the originator identifier image a human readable name of the person, organization or host system that generated this document and MAY include additional data such as Sending User vCard, Receiving User vCard, etc..

Cancel-Job operation

The Sender MAY support and the Receiver MUST support the Cancel-Job operation but only for authenticated Operators/Administrators.

Get-Job-Attributes

The Sender and Receiver MUST support the query of Job-Attributes using the Get-Job-Attributes operation.

## Get-Jobs

The Sender MAY support and the Receiver MUST support the Get-Jobs operation but only for authenticated Operators/Administrators.

Security considerations

Entire section needs rewritting

IPPFAX presents an interesting challenge of balancing security and openness. Many of the envisaged uses of IPPFAX require confidentiality of the data – at the same time the Receiver typically has no prior knowledge of the Sender or the Sending User. This last point will normally rule out all user-based authentication and access control. This is the reason for the restrictions placed on querying and canceling IPPFAX Jobs.

## Data Integrity and authentication

Any exchange between a Sender and a Receiver MUST be carried using the data integrity mechanism specified in IPP/1.1 namely TLS/1.0 [RFC2246] or later versions of TLS.

A Receiver MUST have a TLS certificate and be authenticated by the sender.

A  Sender MAY have a TLS certificate for client authentication.  A Receiver MAY decide to reject requests that come from Senders that do not have a TLS certificate and return the ‘client-error-not-authenticated’ status code.

A Sender MAY use its own TLS certificate or it can use one associated with the Sending User.

A Receiver MUST have a TLS certificate, and the Send MUST have the public keys of the top level public key Certificate Authorities (as current browsers do). If a Sender gets a public key from a Receiver that is doesn’t recognize, the Sender MUST resolve the unrecognized key or inform the Sending User that data integrity has been lost and MUST abort the job.

The distribution of private keys to Senders or Receivers is outside the scope of this document, but if it is done over the network, it MUST be over a secure channel.  See Internet Key Exchange (IKE) [RFC2409].

## Data Privacy (encryption)

A Sender MAY chose use data privacy (encryption) as defined in TLS/1.0 [RFC2246].

## uri-authentication-supported (1setOf type2 keyword)

This attribute (see [RFC2911] section 4.4.2) identifies the Client Authentication mechanism associated with each URI listed in the “printer-uri-supported” attribute (see section 5.1).

Table 5 - Authentication Requirements

| “uri-authentication-supported” keyword | Sender support and usage | Receiver support and usage |
| --- | --- | --- |
| none | MAY support and MAY use | MAY support and MAY use. If the ‘none’ value is supported by an implementation, then the administrator MUST be able to configure the Printer to not support the ‘none’ value (by means outside the scope of this document) |
| requesting-user-name | MUST NOT | MUST NOT |
| basic | MAY support and MAY use when the TLS channel is secured with Data Privacy using the cipher suites indicated below* or stronger | MAY support and MAY use when the TLS channel is secured with Data Privacy using the cipher suites indicated below* or stronger |
| digest | MUST support and MUST use, including the MD5 and MD5-sess algorithms and Message Integrity, unless using ‘certificate’ or ‘negotiate’ | MUST support and MAY use, including the MD5 and MD5-sess algorithms and Message Integrity |
| certificate | SHOULD support and MAY use when not using any of the above | MUST support and MAY use. For this value, the Receiver MUST validate the certificate for all client requests |

* TLS_DHE_DSS_WITH_3DES_EDE_CBC_SHA mandated by [RFC2246].

Table 6 compares the Digest Authentication requirements for IPP/1.1 clients, IPP/1.1 Printers, IPPFAX Senders, and IPPFAX Receivers.

Table 6 - Digest Authentication Conformance Requirements

| Feature | IPP/1.1 Client | IPP/1.1 Printer | IPPFAX Sender | IPPFAX Receiver |
| --- | --- | --- | --- | --- |
| MD5 and MD5-sess | must support must use | should support should use | MUST support MUST use | MUST support MUST use |
| The Message Integrity feature | must support may use | should support may use | MUST support MUST use | MUST support MUST use |

## uri-security-supported (1setOf type2 keyword)

This attribute (see [RFC2911] section 4.4.3) identifies the security (Integrity and Privacy) mechanisms used for each URI listed in the “printer-uri-supported” attribute (see section 5.1).

Table 7 - Security (Integrity and Privacy) Requirements

| uri-security-supported | Sender support and usage | Receiver support and usage |
| --- | --- | --- |
| none | MUST NOT | MUST NOT |
| ssl2 | MUST NOT | MUST NOT |
| ssl3 | MUST NOT | MUST NOT |
| tls | TLS Data Integrity - MUST support and MUST use | MUST support and MUST use |
|  | TLS Data Privacy - MUST support and MAY use. The Sender (device) MUST query the Sending User (human) before omitting Privacy (encryption). | MUST support and MAY use |

Table 8 compares the TLS conformance requirements for IPP/1.1 clients, IPP/1.1 Printers, IPPFAX Senders, and IPPFAX Receivers.

Table 8 - Transport Layer Security (TLS) Conformance Requirements

| TLS Feature | IPP/1.1 Client | IPP/1.1 Printer | IPPFAX Sender | IPPFAX Receiver |
| --- | --- | --- | --- | --- |
| Server Authentication | must support should use | should support may use | MUST use | MUST support |
| Client Authentication* | may support may use | may support may use | SHOULD support | MUST support MAY use |
| Data Integrity | may support may use | should support should use | MUST use | MUST support |
| Data Privacy | may support may use | should support may use | MUST support MAY** use. | MUST support |

* The ‘certificate’ keyword value for the “uri-authentication-supported” attribute [RFC2911].

** The Sender MUST query the Sending User before omitting the Data Privacy encryption.

Senders and Receivers MUST support the TLS_DHE_DSS_WITH_3DES_EDE_CBC_SHA cipher suite as mandated by RFC 2246 [RFC2246].  All stronger cipher suites are OPTIONAL; weaker cipher suites MUST NOT be supported or used by Senders or Receivers.

A Receiver MAY support Basic Authentication (described in HTTP/1.1 [RFC2617]) for Client Authentication if the TLS channel is secured with Data Privacy.  TLS with the above mandated cipher suite or stronger can provide such a secure channel.

## Using IPPFAX with TLS

The Sender MUST use only TLS for all IPPFAX operations on the IPPFAX URL.  The client MUST start the transaction in TLS, rather than using HTTP upgrade requests.  The following paragraph of [RFC2818] further explains:

The agent acting as the HTTP client should also act as the TLS client.  It should initiate a connection to the server on the appropriate port and then send the TLS ClientHello to begin the TLS handshake. When the TLS handshake has finished. The client may then initiate the first HTTP request.  All HTTP data MUST be sent as TLS “application data”.  Normal HTTP behavior, including retained connections should be followed.

Contrast this IPPFAX requirement with the IPP requirement in section 8.2 of [RFC2910].  The following client actions compare IPP with IPPFAX from a client’s point of view:

IPP/1.1 sequence:

- Start TCP connection

- Zero or more HTTP/IPP requests

- HTTP/IPP request with Upgrade to TLS header

- TLS handshake

- Finish the HTTP/IPP request securely

- Send more HTTP/IPP requests securely ...

IPPFAX sequence:

- Start TCP connection

- Send TLS ClientHello

- Rest of TLS handshake

- Send HTTP/IPPFAX requests securely ... (which usually will be a Get-Printer-Attributes, followed by the Print-Job operation).

## Access control

Needs re-writting

It is expected that the majority of IPPFAX Receivers will operate in a public mode when operating on the Internet, so that anonymous users can send documents without requiring client authentication (corresponding to the ‘none’ value for the “uri-authentication-supported” attribute - see section 8.3). However a Receiver MAY protect itself using any Client Authentication method specified in [RFC2911] (digest authentication [RFC2069] for example) to restrict access to any or all of its functionality.

However, the primary intent of IPPFAX is to create a controlled public access mode. It therefore does not really make much sense to combine IPPFAX and user authentication; they are achieving the same thing.

## Reduced feature set

Needs re-writting

An administrator or device implementer MAY choose to setup up a Print Service so that it only works as an IPPFAX Receiver (i.e., offers no ‘native’ IPP operations and does not accept IPP Jobs). In this mode it offers a restricted set of features and MAY be more safely connected to the Internet.

A Receiver that is operating in this mode MUST do so by rejecting any non-IPPFAX request and return a ‘client-error-attributes-or-values-not-supported’ error status code as indicated in section 4.1 for an unsupported value of the “printer-uri” operation attribute.  For job operations attempted on IPPFAX Jobs, the Receiver MUST return the ‘client-error-not-authorized’ error status code, unless the Sender is authenticated as the system administrator and the Receiver supports such access.

# Attribute Syntaxes

No new attribute syntaxes are defined in this document.

# Status codes

No new Status codes are defined and semantics for existing status codes have not been modified in this document.

# Conformance Requirements

Need to be re-worked.

The Sender MUST:

- Support PDF/is, see section 1

- Support the only the operations listed in Section 1.1

- Multiple URL’s must conform to the rules in section 3.2

- Implement Operations defined in section 7 as required for Senders

The Receiver MUST:

Document Originator MUST:

## Operation Conformance Requirements

Error: Reference source not found lists the conformance requirements for Printer operations for (1) an IPP/1.1 Printer (‘ipp’ URL), (2) the non-privileged IPPFAX Sender, (3) an IPPFAX Receiver receiving a request from a non-privileged User, and (4) an IPPFAX Receiver receiving a request from an authenticated and authorized operator or administrator, if the Receiver supports operator/administrator authentication and authorization.

Error: Reference source not found lists the conformance requirements for Job and Subscription operations for (1) an IPP/1.1 Printer (‘ipp’) URL, (2) the non-privileged IPPFAX Sender which MUST be on the same URL as the job was created (the target “printer-uri” MUST match the Job’s “job-printer-uri” Job Description attribute), (3) an IPPFAX Receiver receiving a request from the Job or Subscription Object Owner, (4) from some other non-privileged user, and (5) if the operation is supported at all - from an authenticated and authorized operator or administrator.

Table 9 - Conformance for IPPFax/1.0 Operations

| Operation Name | IPPFAX Sender support for a User | IPPFAX Receiver from a User | IPPFAX Receiver from an Operator | Reference |
| --- | --- | --- | --- | --- |
| Print-Job | MUST | MUST | MUST | section |
| Get-Jobs | MUST NOT | MUST NOT | MUST | section 7.4 |
| Get-Printer-Attributes | MUST | MUST | MUST | sections Error: Reference source not found, 5 |
| Cancel-Job |  |  |  |  |
| Get-Job-Attributes |  |  |  |  |

Legend:

Legend:

MAY* - Get-Job-Attributes restricts certain. See section 7.4.

Owner refers to the owner of the Job or Subscription object.

This section summarizes the conformance requirements for Senders and Receivers that are defined elsewhere in this document.

- A Sender and Receiver MUST observe the attribute name space conventions specified in section Error: Reference source not found.

- The Sender MUST supply and the Receiver MUST support (1) the “printer-uri” operation attribute with the ‘ippfax’ scheme, (2) the “version-number” parameter with the IPP/1.1 ‘1.1’ (or higher minor version) value, and (3) the “ippfax-version” operation attribute with the IPPFAX/1.0 ‘1.0’ keyword value in all operations to get the IPPFAX semantics as described in section 4.

- The Receiver MUST support the Get-Printer-Attributes operation as described in sections Error: Reference source not found.

- The Receiver MUST support the Printer Description attributes as specified in section 5.

- The Sender MUST validate that the target Printer is IPPFAX-capable using the Get-Printer-Attributes operation and validate that the Receiver supports the job using the Validate-Job operation as specified in section Error: Reference source not found.

- The Sender MUST supply and the Receiver MUST support the operation/Job Description attributes for Identify Exchange as described in section Error: Reference source not found.

- The Sender MUST support submitting and the Receiver MUST accept IPPFAX Jobs as defined in section Error: Reference source not found.

- The Sender MUST place the Sender’s identity in the document according to section Error: Reference source not found.

- The Sender and Receiver MUST support the operations as indicated in section 7.

- The Sender and Receiver MUST support the security mechanisms indicated in section 8, including TLS.

The [set-ops], enable-printer and disable-printer operations MUST only be preformed on a connection that has been authenticated by TLS and the user has the rights to perform them.

# IPPFAX URL Scheme

Use pwg-ippfax rather than ippfax

Need to be re-worked to be consistent RFC 3510

Need to register a port with IANA for IPPFax.

This section is intended for use in registering the ‘ippfax’ URL scheme with IANA and fully conforms to the requirements in [RFC2717].

## IPPFAX URL Scheme Applicability and Intended Usage

This document defines the ‘ippfax’ URL (Uniform Resource Locator) scheme for specifying the location of an IPPFAX Receiver which implements the IPPFAX Protocol specified in this document.

The ‘ippfax’ URL scheme defined in this document is based on the ABNF for the basic hierarchical URL syntax in [RFC2396]; however relative URL forms, parameters, and/or query parts are NOT allowed in an IPPFAX URL.  The ‘ippfax’ URL scheme is case-insensitive in the host name or host address part; however the path part is case-sensitive, as in [RFC2396].  Codepoints outside [US-ASCII] MUST be hex escaped by the mechanism defined in [RFC2396].

The intended usage of the ‘ippfax’ URL scheme is COMMON.

## IPPFAX URL Scheme Associated IPPFAX Port

All IPPFAX URLs which do NOT explicitly specify a port MUST be used over IANA-assigned well-known port xxx [TBA by IANA] for the IPPFAX Protocol.

See:  IANA Port Numbers Registry [IANA-PORTREG].

## IPPFAX URL Scheme Associated MIME Type

All IPPFAX protocol operations (requests and responses) MUST be conveyed in an ‘application/ipp’ MIME media type [RFC2910] as registered in [IANA-MT].  IPPFAX URLs MUST refer to IPPFAX Receivers which support this ‘application/ipp’ operation encoding.

See:  IANA MIME Media Types Registry [IANA-MT].

## IPPFAX URL Scheme Character Encoding

The IPPFAX URL scheme defined in this document is based on the ABNF for the HTTP URL scheme defined in HTTP/1.1 [RFC2616], which is derived from the URI Generic Syntax [RFC2396] and further updated by [RFC2732] and [RFC2373] (for IPv6 addresses in URLs).  The IPPFAX URL scheme is case-insensitive in the ‘scheme’ and ‘host’ (host name or host address) part; however, the ‘abs_path’ part is case-sensitive, as in [RFC2396].  Code points outside [US-ASCII] MUST be hex escaped by the mechanism specified in [RFC2396].

## IPPFAX URL Scheme Syntax in ABNF

The IPP protocol places a limit of 1023 octets (NOT characters) on the length of a URI (see section 4.1.5 ‘uri’ in [RFC2911]).  An IPPFAX Receiver MUST return ‘client-error-request-value-too-long’ (see section 13.1.4.10 in [RFC2911]) when a URI received in a request is too long.

Note:  IPPFAX Receivers ought to be cautious about depending on URI lengths above 255 bytes, because some older client or proxy implementations might not properly support these lengths.

IPPFAX URLs MUST be represented in absolute form.  Absolute URLs always begin with a scheme name followed by a colon.  For definitive information on URL syntax and semantics, see “Uniform Resource Identifiers (URI): Generic Syntax and Semantics” [RFC2396].  This specification adopts the definitions of “port”, “host”, “abs_path”, and “query” from [RFC2396], as updated by [RFC2732] and [RFC2373] (for IPv6 addresses in URLs).

The IPPFAX URL scheme syntax in ABNF is as follows:

ippfax_URL = “ippfax:” “//” host [ “:” port ] [ abs_path [ “?” query ]]

If the port is empty or not given, the IANA-assigned port as defined in section 12.2 is assumed.  The semantics are that the identified resource (see section 5.1.2 of [RFC2616]) is located at the IPPFAX Notification Recipient listening for HTTP connections on that port of that host, and the Request-URI for the identified resource is ‘abs_path’.

Note:  The use of IP addresses in URLs SHOULD be avoided whenever possible (see [RFC1900]).

If the ‘abs_path’ is not present in the URL, it MUST be given as “/” when used as a Request-URI for a resource (see section 5.1.2 of [RFC2616]).  If a proxy receives a host name which is not a fully qualified domain name, it MAY add its domain to the host name it received.  If a proxy receives a fully qualified domain name, the proxy MUST NOT change the host name.

## IPPFAX URL Examples

The following are examples of valid IPPFAX URLs for Notification Recipient objects (using DNS host names):

ippfax://abc.com

ippfax://abc.com/listener

Note:  The use of IP addresses in URLs SHOULD be avoided whenever possible (see [RFC1900]).

The following literal IPv4 addresses:

192.9.5.5                    ; IPv4 address in IPv4 style

186.7.8.9                    ; IPv4 address in IPv4 style

are represented in the following example IPPFAX URLs:

ippfax://192.9.5.5/listener

ippfax://186.7.8.9/listeners/tom

The following literal IPv6 addresses (conformant to [RFC2373]):

::192.9.5.5                  ; IPv4 address in IPv6 style

::FFFF:129.144.52.38         ; IPv4 address in IPv6 style

2010:836B:4179::836B:4179    ; IPv6 address per RFC 2373

are represented in the following example IPPFAX URLs:

ippfax://[::192.9.5.5]/listener

ippfax://[::FFFF:129.144.52.38]/listener

ippfax://[2010:836B:4179::836B:4179]/listeners/tom

## IPPFAX URL Comparisons

When comparing two IPPFAX URLs to decide if they match or not, the comparer MUST use the same rules as those defined for HTTP URI comparisons in [RFC2616], with the sole following exception:

- A port that is empty or not given MUST be treated as equivalent to the port as defined in section 12.2 for that IPPFAX URL;

# IANA Considerations

IANA shall register the ippfax URL scheme as defined in section 12 according to the procedures of [RFC2717] and assign a well known port.

Operation Attributes:

ippfax-version (type2 keyword)            IEEE-ISTO 510n.y 4.3

Operation/Job Description attributes:

sending-user-vcard (text(MAX))                   IEEE-ISTO 510n.y 6.1

receiving-user-vcard (text(MAX))                 IEEE-ISTO 510n.y 6.2

Printer Description Attributes:

ippfax-versions-supported (1setOf type2 keyword) IEEE-ISTO 510n.y 5.3

# References

## Normative

[IANA-MT]

IANA Registry of Media Types:  ftp://ftp.iana.orgisi.edu/in-notes/iana/assignments/media-types/.

[IANA-PORTREG]

IANA Port Numbers Registry.  ftp://ftp.isi.edu/in-notes/iana/assignments/port-numbers.

[PWG5102.3-2004]

Seeler, R., “PDF Image-Streamable (PDF/is)”, Work in Progress,

ftp://pwg.org/pub/pwg/QUALDOCS/pwg-ifx-pdfis-latest.pdf.

[jobx]

Hastings, T. and P. Zehler, "IPP Job Extensions", May 19, 2000, ftp://ftp.pwg.org/pub/pwg/ipp/new_JOBX/wd-ippjobx10-20030518.pdf, work in progress.

## Informative

[ifx-req]

Moore, P., “IPP Fax transport requirements”, October 16, 2000, ftp://ftp.pwg.org//pub/pwg/QUALDOCS/requirements/ifx-transport-requirements-01.pdf.

[RFC2542]

Masinter , “Terminology and Goals for Internet Fax”, RFC2542.

[RFC3380]

Kugler, C, Hastings, T., Lewis, H., “Internet Printing Protocol (IPP): Job and Printer Administrative Operations”, <draft-ietf-RFC3380-03.txt>, July 17, 2001.

[RFC 3382]

deBry, R., , Hastings, T., Herriot, R., “Internet Printing Protocol (IPP): collection attribute syntax”,RFC 3382, September, 2002 .

[ipp-get-method]

Herriot, Kugler, and Lewis, “The ‘ippget’ Delivery Method for Event Notifications” , <draft-ietf-ipp-notify-get-06.txt>, November 19, 2001.

[ipp-iig-bis]

Hastings, T., Manros, C., Zehler, P., Kugler, C., and H. Holst, “Internet Printing Protocol/1.1: Implementer’s Guide”, draft-ietf-ipp-implementers-guide-v11-04.txt, work in progress, intended to obsolete RFC 3196 [RFC3196], October 8, 2001.

[RFC 3381]

Hastings, T., Bergman, R., Lewis, H., “Internet Printing Protocol (IPP): Job Progress Attributes”, RFC 3381, September, 2002.

[ipp-ntfy]

Isaacson, S., Martin, J., deBry, R., Hastings, T., Shepherd, M., Bergman, R., “Internet Printing Protocol/1.1: IPP Event Notification Specification”, <draft-ietf-ipp-not-spec-08.txt>, November 19, 2001.

[ipp-output-bin]

Hastings, T., and R. Bergman, “Internet Printing Protocol (IPP): output-bin attribute extension”, IEEE-ISTO 5100.2-2001, February 7, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.2.pdf.

[ipp-prod-print]

Ocke, K., Hastings, T., “Internet Printing Protocol (IPP): Production Printing Attributes - Set1”, IEEE-ISTO 5100.3-2001, February 12, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.3.pdf.

[ipp-set-ops]

Hastings, Herriot, Kugler, and Lewis, “Job and Printer Set Operations”, <draft-ietf-ipp-job-printer-set-ops-05.txt>, August 28, 2001.

[ipp-uri-scheme]

Herriot, McDonald, “IPP URL Scheme”, <draft-ietf-ipp-url-scheme-03.txt>,April 3, 2001.

[pwg-media]

Bergman, Hastings, “Media Standardized Names”, work in progress, when approved: ftp://ftp.pwg.org/pub/pwg/standards/pwg5101.1.pdf;  current draft: ftp://ftp.pwg.org/pub/pwg/media-sizes/pwg-media-12.pdf, September 24, 2001.

[RFC1900]

B. Carpenter, Y. Rekhter.  Renumbering Needs Work, RFC 1900, February 1996.

[RFC2069]

Franks, Hallam-Baker, Hostetler, Leach, Luotonen,, Sink, Stewart, “An Extension to HTTP: Digest Access Authentication”, RFC2069.

[RFC2119]

Bradner, S., “Key words for use in RFCs to Indicate Requirement Level”, RFC2119.

[RFC2246]

Dierks, Allen “The TLS Protocol  Version 1.0”, RFC 2246.

[RFC2305]

Toyoda, Ohno, Murai, Wing “A Simple Mode of Facsimile Using Internet Mail”, RFC2305.

[RFC2373]

R. Hinden, S. Deering.  IP Version 6 Addressing Architecture, RFC 2373, July 1998.

[RFC2396]

Berners-Lee, T. et al.  Uniform Resource Identifiers (URI): Generic Syntax, RFC 2396, August 1998.

[RFC2409]

Harkins, D., and D. Carrel, “The Internet Key Exchange (IKE)”, RFC 2409, November 1998.

[RFC2425]

T. Howes, M. Smith, F. Dawson, “A MIME Content-Type for Directory Information”, RFC 2425, September 1998.

[RFC2426]

Dawson, Howes, “vCard MIME Directory Profile”, RFC 2426, September 1998 [version v3.0].

[RFC2532]

Masinter, Wing, “Extended Facsimile Using Internet Mail”, RFC2532.

[RFC2616]

R. Fielding, J. Gettys, J. Mogul, H. Frystyk, L. Masinter, P. Leach, T. Berners-Lee, “Hypertext Transfer Protocol - HTTP/1.1”, RFC 2616, June 1999.

[RFC2617]

J. Franks, P. Hallam-Baker, J. Hostetler, S. Lawrence, P. Leach, A. Luotonen, L. Stewart, “HTTP Authentication: Basic and Digest Access Authentication”, RFC 2617, June 1999.

[RFC2732]

R. Hinden, B. Carpenter, L. Masinter.  Format for Literal IPv6 Addresses in URL’s, RFC 2732, December 1999.

[RFC2818]

E. Rescorla, “HTTP Over TLS”, May 2000.

[RFC2910]

Herriot, Butler, Moore, Turner, Wenn, “Internet Printing Protocol/1.1: Encoding and Transport”, RFC2910, September 2000.

[RFC2911]

deBry, Hastings, Herriot, Isaacson, Powell, “Internet Printing Protocol/1.1: Model and Semantics”, RFC2911, September 2000.

[RFC3196]

Hastings, T., Manros, C., Zehler, P., Kugler, C., and H. Holst, “Internet Printing Protocol/1.1: Implementer’s Guide”, RFC 3196, November, 2001.

[X509]

CCITT. Recommendation X.509: “The Directory - Authentication Framework”, 1988.

# Authors’ addresses

| Thomas N. Hastings Xerox Corporation 701 Aviation Blvd. El Segundo, CA 90245 | Ira McDonald High North Inc 221 Ridge Ave Grand Marais, MI 49839 |
| --- | --- |
| Phone: +1 310-333-6413 FAX: +1 310-333-5514 email: hastings@cp10.es.xerox.com | Phone: +1 906-494-2434 Email: imcdonald@sharplabs.com |
|  | Gail Songer Peerless Systems Corp 2381 Rosecrans Ave El Segundo, CA 90245 |
|  | Phone: +1 650-358 8875 Email: gsonger@peerless.com |
|  | Rick Seeler Adobe Systems Incorporated 321 Park Ave. San Jose, CA 95110 |
|  | Phone: +1 408- 536-4393 Email: rseeler@adobe.com |
| Dennis Carney IBM 6300 Diagonal Highway Boulder, CO 80301 |  |
| Phone: +1 303-924-0565 Email: dcarney@us.ibm.com |  |

Contact Information:

IPPFAX Web Page:  http://www.pwg.org/qualdocs/

IPPFAX Mailing List:  ifx@pwg.org

To subscribe to the IPPFAX mailing list, send the following email:

1) send it to majordomo@pwg.org

2) leave the subject line blank

3) put the following two lines in the message body:

subscribe ifx

end

Implementers of this specification document are encouraged to join the IPPFAX Mailing List in order to participate in any discussions of clarification issues and review of registration proposals for additional attributes and values.  In order to reduce spam the mailing list rejects mail from non-subscribers, so you must subscribe to the mailing list in order to send a question or comment to the mailing list.

Other Participants:

| Aisushi Uchino - Epson | Marty Joel - Peerless |
| --- | --- |
| Bill Wagner - NetSilicon/DPI | Michael Wu - Heidelberg Digital |
| Carl-Uno Manros - Xerox | Mike Kuindersma - PrinterOn |
| Charles Kong - Panasonic | Norbert Schade - Oak Technology |
| Dan Calle - Digital Paper | Patrick Pidduck - PrinterOn |
| David Kellerman – Northlake | Peter Zehler – Xerox |
| Don Wright - Lexmark | Rich Heckelmann - Panasonic USA |
| Elliott Bradshaw – Oak Technologies | Richard Shockey - Newstar |
| Frank Martin - Brother | Rob Buckley - Xerox |
| Fumio Nagasaka – Epson | Robert Herriot - Xerox |
| Geoff Soord - Software 2000 | Roelop Hamberg - Oce |
| Harry Lewis - IBM | Ron Bergman - Hitachi Koki |
| Howard Sidorski - Netreon | Satoshi Fujitani - Ricoh |
| Hugo Parra - Novell | Shigeru Udea - Canon |
| Jeff Christensen - Novell | Shinichi Tsuruyama - Epson |
| Jerry Thrasher - Lexmark | Stuart Rowley - Kyocera |
| John Thomas - Sharp Labs | Ted Tronson - Novell |
| Koichi “Hurry” Izuhara - Minolta | Toru Maeda - Canon |
| Lee Farrell - Canon Info Systems | Yiruo Yang – Epson |
| Lloyd McIntyre | Yuji Sasaki - JCI |
| Mark VanderWiele - IBM | Paul Moore - |
| John Pulera - Minolta |  |

- Appendix A:

# Appendix B: vCard Example

Update the example

The following ASCII text is a complete vCard v3.0 [RFC2426, RFC2425] example:

BEGIN:VCARD

VERSION:3.0

N:Moore;Paul

FN:Paul Moore

ORG:Netreon

TEL;CELL;VOICE:1+206-251-7008

ADR;WORK:;;10900 NE 8th St;Bellvue;WA;98004;United States of America

EMAIL;PREF;INTERNET:pmoore@netreon.com

REV:19991207T215341Z

END:VCARD

# Revision History (to be removed when standard is approved)

| Revision | Date | Author | Notes |
| --- | --- | --- | --- |
| 1 | 1/16/01 | Paul Moore, Netreon | Initial version |
| 2 | 2/27/01 | Paul Moore, Gail Songer, Netreon | Specify TLS as MUST Removed Cover page and combined device Added need for big text types |
| 3 | 4/11/01 | Gail Songer, Netreon | Move attribute definition to first reference |
| 4 | 5/24/01 | Tom Hastings | Editorially updated the document to follow the style of the IPP standard documents. Added 23 issues to be reviewed. Capitalized the special terms throughout without showing revisions in order to make the document with revisions more readable. |
| 5 | 5/21/01 | Tom Hastings, John Pulera, Ira McDonald | Updated from the 6/6/01 telecon agreements on most of the 23 issues. There are 20 issues remaining, mostly new. |
| 6 | 7/27/01 | Tom Hastings, Ira McDonald | Updated from the 6/29/01 telecon. There are 41 issues remaining, mostly new. |
| 7 | 10/8/01 | Tom Hastings, Ira McDonald | Updated with all the resolutions to the 41 ISSUES from the August 1, 2001 IPPFAX WG meeting in Toronto, and the subsequent telecons: August, 9, 14, and 17, 2001. There are 4 (new) issues remaining. |
| 8 | 11/17/01 | Tom Hastings | Updated with the agreements from the IPPFAX WG meeting, 10/24/01, Texas. See minutes. There are 5 issues remaining. |
| 9 | 12/31/01 | Tom Hastings | Updated with the agreements reached at the 12/14/01 telecon. |
| 10 | 2/19/02 | Tom Hastings | Updated with the agreements reached as the 2/5/02 IPPFAX WG meeting. There are no remaining issues. |
| 11 | 9/20/02 | Tom Hastings | Replaced all occurrences of UIF with PDFax and uif with PDFax. |
| 12 | 10/16/02 10/24/02 | Rick Seeler Gail Songer | Updated to reflect PDF/is as file format. Replace CONNEG with UPDF. Attributes for OPTIONAL PDF/is functionality. |
| 13 | 11/22/02 | Rick Seeler | Replaced ‘PDFax’ with ‘PDF/is’ or ‘pdfis’. Updated spec to match 0.3 PDF/is specification. |
| 14 | 03/18/03 | Gail Songer | Removed pdfis-profile-requested and pdfis-profile-supported and pdfis-profiles; all image formats are required Removed pdfis-cache-size-k-octets (now fixed value) Removed pdfis-banding-direction-supported Started to split references into two sections, “normative” and “informative” and update descriptions to references Other editorial changes |
| 15 | 03/24/03 | Gail Songer | Added digital-signatures-supported. Added pdf-format and pdf-format supported. Put “coloring” back to optional. Removed PDF data encryption (leave for a future version of PDF/is and IPPFax) |
| 16 |  | Gail Songer Dennis Carney | Remove all references to coloring Changed pdf-format to document-format-version Remove the requirement that [set-ops] supports document-format coloring (we only allow document-format==PDF) ALL admin operations require TLS to have authenticated the user and the user has admin rights Other editorial changes |
| 17 | 05/21/03 05/28/03 | Dennis Carney Tom Hastings | Editorial updates Added new ‘choice_iso_a4_210x297mm_na_letter_8.5x11in’ value for “media” and a reference to [jobx]. Fixed conformance for “media-ready”. |
| 18 | 10/03 11/03 | Gail Songer | Reviewed in light of the Requirements specification. Noted lots of places in which the document MUST be changed. |

Allow Cancel-job for Administrators.

[RFC2911] does not require the client to supply the “ipp-attribute-fidelity” and allows the client to supply either the ‘true’ or ‘false’ value.

The [RFC2911] does not require the IPP client to supply the “document-format” operation attribute.

These attributes were not defined in [RFC2911].