Printer MIB and IPP MFD Alerts 
(MFD Alerts)

Status: Approved

Abstract: This document defines an update to the IANA-PRINTER-MIB (originally published in RFC 3805) to provide support for SNMP alerts in a multifunction device (MFD) and an equivalent update to IPP “printer-state-reasons” (RFC 2911) and IPP “printer-alert” (PWG 5100.9).  An MFD is typically based on a printer with added scan- and fax-specific components in order to support print, copy, scan, and facsimile (fax) services.

This document defines an update to the IANA-PRINTER-MIB to provide support for new MFD components and component-specific alerts and analogous Printer extension alerts for the existing Input, Output, and MediaPath components.

This document is a PWG Candidate Standard. For a definition of a "PWG Candidate Standard ", see:

ftp://ftp.pwg.org/pub/pwg/general/pwg-process30.pdf

This document is available at:

ftp://ftp.pwg.org/pub/pwg/candidates/cs-pmpmfdalerts10-20120629-5107.3.pdf

Copyright © 2012 The Printer Working Group. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  Printer MIB and IPP MFD Alerts (MFD Alerts)

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at: ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary. The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

About the IEEE-ISTO

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit:

http://www.ieee-isto.org.

About the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:

http://www.pwg.org

Contact information:

The Printer Working Group

c/o The IEEE Industry Standards and Technology Organization

445 Hoes Lane

Piscataway, NJ 08854

USA

About the Workgroup for Imaging Management Solutions

The Workgroup for Imaging Management Solutions (WIMS) is concerned with the definition of new and recasting of previously defined imaging device and service management elements for mapping into standard managment semantics and protocols. These protocols include, but are not limited to, SNMP, CIM and Web Services Management. WIMS provides continuing support for printing-related IETF RFCs including the Printer MIB (RFC 3805), Finisher MIB (RFC 3806) and the Job Monitoring MIB (RFC 2707).

WIMS Web Page:

http://www.pwg.org/wims/

PMP and WIMS Mailing Lists:

pmp@pwg.org

wims@pwg.org

Instructions for subscribing to the PMP and WIMS mailing lists can be found at the following link:

http://www.pwg.org/mailhelp.html

Implementers of this specification are encouraged to join the PMP (SNMP MIBs) and WIMS (all management protocols) mailing lists in order to participate in any discussions of the specification.  Suggested additions, changes, or clarification to this specification, should be sent to the PMP and WIMS mailing lists for consideration.

Table of Contents

1.	Introduction	7

2.	Terminology	8

2.1 Conformance Terminology	8

2.2 Printing Terminology	8

3.	Requirements	9

3.1 Rationale for MFD Alerts	9

3.2 Use Cases for MFD Alerts	9

3.2.1 MFDs with OEM Components	9

3.2.2 MFDs with Alert Messages	10

3.2.3 MFDs with Web-based Fleet Management	10

3.3 Out of-Scope for MFD Alerts	10

3.4 Design Requirements for MFD Alerts	11

4.	Printer Model Extensions	12

4.1 ScanDevice	14

4.2 FaxDevice	14

4.3 OutputChannel	14

5.	MFD and Printer Extension Alerts	15

5.1 MFD Alert Groups	15

5.2 MFD and Printer Extension Subunit Alerts	16

5.3 IPP printer-state-reasons (1setOf type2 keyword)	18

6.	Conformance Requirements	20

6.1 Printer MIB Agent Conformance Requirements	20

6.2 Printer MIB Client Conformance Requirements	20

6.3 IPP Printer Conformance Requirements	21

6.4 IPP Client Conformance Requirements	21

7.	Internationalization Considerations	22

8.	Security Considerations	22

9.	IANA Considerations	23

9.1 Alert Groups	23

9.2 Alert Codes	24

9.3 IPP Attribute and Keyword Value Registrations	26

10.	References	28

10.1 Normative References	28

10.2 Informative References	28

11.	Authors' Addresses	30

List of Figures

Figure 1 – System Object in MFD Model	12

Figure 2 – SystemConfiguration Element in MFD Model	13

List of Tables

Table 1 – MFD Alert Groups	15

Table 2 – MFD and Printer Subunit Alerts	16

Table 3 – IPP printer-state-reasons	18

- Introduction

This document defines simple extensions to the originally printer-specific IETF Printer MIB v2 [RFC3805] (new enumeration values in prtAlertCode) and IETF IPP/1.1 [RFC2911] (new keyword values in “printer-state-reasons”) to add support for alert information for multifunction devices (MFDs), which are now very popular alternatives to using separate printer, copier, and facsimile equipment.  Prior to the introduction of MFDs, printer vendors and application developers had already created tools, management systems, and device drivers based upon the Printer MIB v2 [RFC3805] and the prtAlertTable.    MFDs are typically less expensive than the equivalent set of individual devices, and have the additional advantage of occupying much less office space.

The printer portion of an MFD is used by the print, copy, and facsimile (fax) functions. Additional scanner and scan media path components are used by the copy and fax functions.  The fax function also uses a fax modem component with a PSTN interface.

The Printer Working Group (PWG) developed the IETF Printer MIB v2 [RFC3805], which is now implemented in most network printers sold today and defines the prtAlertTable that may be used, with or without SNMP traps, to implement an effective warning and error reporting system.

- Terminology

2.1 Conformance Terminology

Capitalized terms, such as MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, RECOMMENDED, MAY, and OPTIONAL, have special meaning relating to conformance as defined in IETF Key words for use in RFCs to Indicate Requirement Levels [RFC2119].

2.2 Printing Terminology

Normative definitions and semantics of printing terms are imported from IETF Printer MIB v2 [RFC3805], IETF Finisher MIB [RFC3806], and IETF IPP/1.1 [RFC2911].

This document also defines the following protocol roles in order to specify unambiguous conformance requirements:

IPP Client - Initiator of outgoing IPP session requests and sender of outgoing IPP operation requests (HTTP/1.1 [RFC2616] User Agent).

IPP Printer - Listener for incoming IPP session requests and receiver of incoming IPP operation requests (HTTP/1.1 [RFC2616] Server).

Printer MIB Agent:  Listener for incoming SNMP Get and Set management requests and sender of optional outgoing SNMP notifications for a Printer or MFD (i.e., an SNMP Agent).

Printer MIB Client:  Initiator of outgoing SNMP Get and Set management requests and receiver of optional incoming SNMP notifications for a Printer or MFD (i.e., an SNMP Manager).

- Requirements

3.1 Rationale for MFD Alerts

The IETF, and PWG standards in the printing industry include:

- An abstract model of a PrintDevice in section 2.2 of the IETF Printer MIB v2 [RFC3805].

- An SNMP Alert table for a PrintDevice to support the service and maintenance functions in section 2.2.13 of the IETF Printer MIB v2 [RFC3805].

- A set of design goals for status monitoring in a printing protocol in section 3.1.3 “Viewing the status and capabilities of a printer” (for End User), section 3.2.1 “Alerting” (for Operator), and section 3.3 “Administrator” (the bullet requirement to “administrate billing or other charge-back mechanisms”) of the IETF IPP Design Goals [RFC2567].

- A set of MFD service types for Imaging Systems in the JmJobServiceTypesTC textual convention in section 4 of the IETF Job Monitoring MIB [RFC2707].

- An abstract model of an MFD job in section 2 of the IETF Job Monitoring MIB [RFC2707].

- An abstract model of an MFD in the PWG MFD Model and Common Semantics [PWG5108.1].

In the years since the Printer MIB v2 [RFC3805] was published printers have evolved into MFDs.  Prior to the introduction of MFDs, printer vendors and application developers had already created tools, management systems, and device drivers based upon the Printer MIB v2 [RFC3805] and the prtAlertTable.  Now that these same vendors are building MFDs, there is an urgent need to leverage these existing tools and management applications.

This document defines a new set of MFD alert groups and MFD component alerts that will allow the applications currently using the prtAlertTable to support MFDs.

3.2 Use Cases for MFD Alerts

3.2.1 MFDs with OEM Components

Company A markets complete systems, including a full range of computers, printers, and other office peripheral devices.  Most of the equipment included with these systems are manufactured by Company A.  The remaining equipment is Company A branded (i.e., OEM), but manufactured by others.  All of these systems include a management application that monitors all systems components and automatically initiates service calls.  For printer maintenance, the management system uses the prtAlertTable.  New system configurations now offer MFDs as options for printers. By including the MFD Alerts in the MFDs and in Company A's management system, Company A can now offer full management and maintenance support for these new MFDs.

3.2.2 MFDs with Alert Messages

Company B is now adding a new series of MFDs to its extensive line of printers.  The current printer families include a deluxe driver that monitors the prtAlertTable to provide status information to the end user.  The monitor function does not interpret the prtAlertCode or the prtAlertLocation values, but instead queries and displays the prtAlertDescription value to indicate the fault condition.  This feature allows the end user to initiate any action that may be required to complete the user’s jobs.  The fault information may be related to a job that precedes the user’s current job so, if the owner of the previous job is not able or to does not wish to act, the owner of the new job may take the appropriate action so that normal operation can resume.  By including the MFD Alerts in their new MFD family, Company B can now offer the monitor function for these new MFDs.

3.2.3 MFDs with Web-based Fleet Management

Company C provides a fleet management system based upon DMTF WS-Management [WS-MGMT], OASIS WSDM [WSDM], or any other web-based protocol that is appropriate for fleet management.  The communication between the local fleet management server and the local printers is accomplished via SNMP and the information available in the prtAlertTable is queried to maintain the logs in the remote fleet management server.  When MFDs are added to the local network, the fleet management system can monitor all the MFD functions with only minor modifications to support the MFD Alerts.

3.3 Out of-Scope for MFD Alerts

This MFD Alerts specification should not:

- Define any components that are not already defined in the PWG MFD Model and Common Semantics [PWG5108.1].

- Define any semantics for workflow applications.

- Define any semantics for document repositories.

- Define any application-specific semantics for MFD monitoring using MFD Alerts.

3.4 Design Requirements for MFD Alerts

This MFD Alerts specification should satisfy the following design requirements:

  - Define a set of alert groups to provide alert capability for MFDs equivalent to the capability currently provided for printers for registration in the PrtAlertGroupTC in the IANA Printer MIB [IANAPRT].

  - Define new alert groups for MFD components only where functionally equivalent groups do not already exist for the PrintDevice.  For example, a ScanMediaPath is inherently entirely separate from any print MediaPath.

  - Do not define new alert groups for MFD components where functionally equivalent groups already exist for the PrintDevice.  For example, ScanDevice covers should be modeled using the existing Cover group.

  - Define a set of component-specific alerts for new ScanDevice and FaxDevice components for registration in the PrtAlertCodeTC in the IANA Printer MIB [IANAPRT].

  - Define a set of component-specific extension alerts for existing Input, Output, and MediaPath alert groups that correspond to extensions for the ScanMediaPath alert group.

- Printer Model Extensions

This section briefly summarizes extensions to the abstract Printer Model, originally defined in section 2 of IETF Printer MIB v2 [RFC3805], based on the PWG MFD Model and Common Semantics [PWG5108.1], to include the ScanDevice and FaxDevice,  their additional subunits, and the new OutputChannel subunit.  The following two figures are taken directly from [PWG5108.1].

Figure 1 – System Object in MFD Model

Figure 2 – SystemConfiguration Element in MFD Model

4.1 ScanDevice

The ScanDevice uses the following subunits: Console, Cover, Interface, Interpreter, OutputChannel, Processor, ScanMediaPath, Scanner, Storage, and optionally the VendorSubunit.

4.2 FaxDevice

The FaxDevice uses the following subunits: Console, Cover, FaxModem, Finisher, InputChannel, InputTray, Interface, Interpreter, Marker, MediaPath, OutputChannel, OutputTray, Processor, ScanMediaPath, Scanner, Storage, and optionally the VendorSubunit.

4.3 OutputChannel

An OutputChannel is the opposite of an InputChannel – it sends jobs and user data from an MFD via a configured application protocol (e.g., SMTP) to specified destinations.

- MFD and Printer Extension Alerts

5.1 MFD Alert Groups

The new MFD alert groups and the associated alert group values are defined in this section for registration in PrtAlertGroupTC in IANA Printer MIB [IANAPRT].

Table 1 – MFD Alert Groups

| MFD Alert Group | PrtAlertGroupTC Value |
| --- | --- |
| scanDevice | 50 |
| scanner | 51 |
| scanMediaPath | 52 |
| faxDevice | 60 |
| faxModem | 61 |
| outputChannel | 70 |

5.2 MFD and Printer Extension Subunit Alerts

The new MFD and Printer extension subunit alerts and the associated alert values are defined in this section for registration in PrtAlertCodeTC in IANA Printer MIB [IANAPRT].

Note:  The original Printer MIB v1 [RFC1759] and subsequent Printer MIB v2 [RFC3805] did not define any (Input)Channel-specific alerts.  Therefore this MFD Alerts specification does not define any OutputChannel-specific alerts.  The generic alerts (subunitXxx) originally defined in [RFC3805] and registered in [IANAPRT] may be used for both (Input)Channel and OutputChannel subunits.

Table 2 – MFD and Printer Subunit Alerts

| Subunit Alert | PrtAlertCodeTC |
| --- | --- |
| -- Input Group |  |
| inputMediaTrayFeedError | 814 |
| inputMediaTrayJam | 815 |
| inputMediaTrayFailure | 816 |
| inputPickRollerLifeWarn | 817 |
| inputPickRollerLifeOver | 818 |
| inputPickRollerFailure | 819 |
| inputPickRollerMissing | 820 |
| -- Output Group |  |
| outputMediaTrayFeedError | 905 |
| outputMediaTrayJam | 906 |
| outputMediaTrayFailure | 907 |
| -- Media Path Group |  |
| mediaPathFailure | 1305 |
| mediaPathJam | 1306 |
| mediaPathInputRequest | 1310 |
| mediaPathInputFeedError | 1311 |
| mediaPathInputJam | 1312 |
| mediaPathInputEmpty | 1313 |
| mediaPathOutputFeedError | 1321 |
| mediaPathOutputJam | 1322 |
| mediaPathOutputFull | 1323 |
| mediaPathPickRollerLifeWarn | 1331 |
| mediaPathPickRollerLifeOver | 1332 |
| mediaPathPickRollerFailure | 1333 |
| mediaPathPickRollerMissing | 1334 |
| -- Scanner Group |  |
| scannerLightLifeAlmostOver | 5101 |
| scannerLightLifeOver | 5102 |
| scannerLightFailure | 5103 |
| scannerLightMissing | 5104 |
| scannerSensorLifeAlmostOver | 5111 |
| scannerSensorLifeOver | 5112 |
| scannerSensorFailure | 5113 |
| scannerSensorMissing | 5114 |
| -- Scan Media Path Group |  |
| scanMediaPathTrayMissing | 5201 |
| scanMediaPathTrayAlmostFull | 5202 |
| scanMediaPathTrayFull | 5203 |
| scanMediaPathFailure | 5205 |
| scanMediaPathJam | 5206 |
| scanMediaPathInputRequest | 5210 |
| scanMediaPathInputFeedError | 5211 |
| scanMediaPathInputJam | 5212 |
| scanMediaPathInputEmpty | 5213 |
| scanMediaPathOutputFeedError | 5221 |
| scanMediaPathOutputJam | 5222 |
| scanMediaPathOutputFull | 5223 |
| scanMediaPathPickRollerLifeWarn | 5231 |
| scanMediaPathPickRollerLifeOver | 5232 |
| scanMediaPathPickRollerFailure | 5233 |
| scanMediaPathPickRollerMissing | 5234 |
| -- Fax Modem Group |  |
| faxModemMissing | 6101 |
| faxModemLifeAlmostOver | 6102 |
| faxModemLifeOver | 6103 |
| faxModemTurnedOn | 6104 |
| faxModemTurnedOff | 6105 |
| faxModemInactivityTimeout | 6110 |
| faxModemProtocolError | 6111 |
| faxModemEquipmentFailure | 6112 |
| faxModemNoDialTone | 6113 |
| faxModemLineBusy | 6114 |
| faxModemNoAnswer | 6115 |
| faxModemVoiceDetected | 6116 |
| faxModemCarrierLost | 6117 |
| faxModemTrainingFailure | 6118 |

5.3 IPP printer-state-reasons (1setOf type2 keyword)

The new MFD alert values of “printer-state-reasons” [RFC2911] are defined in this section for registration in IANA IPP Registry [IANAPRT].  The table below defines new MFD alert values of "printer-state-reasons" [RFC2911] and their mapping to/from new MFD alert values of 'PrtAlertCodeTC' [IANAPRT] defined above in sections 5.2 and 5.3.

Table 3 – IPP printer-state-reasons

| PrtAlertCodeTC | printer-state-reasons |
| --- | --- |
| -- Input Group |  |
| inputMediaTrayFeedError(814) | input-media-tray-feed-error |
| inputMediaTrayJam(815) | input-media-tray-jam |
| inputMediaTrayFailure(816) | input-media-tray-failure |
| inputPickRollerLifeWarn(817) | input-pick-roller-life-warn |
| inputPickRollerLifeOver(818) | input-pick-roller-life-over |
| inputPickRollerFailure(819) | input-pick-roller-failure |
| inputPickRollerMissing(820) | input-pick-roller-missing |
| -- Output Group |  |
| outputMediaTrayFeedError(905) | output-media-tray-feed-error |
| outputMediaTrayJam(906) | output-media-tray-jam |
| outputMediaTrayFailure(907) | output-media-tray-failure |
| -- Media Path Group |  |
| mediaPathFailure(1305) | media-path-failure |
| mediaPathJam(1306) | media-path-jam |
| mediaPathInputRequest(1310) | media-path-input-request |
| mediaPathInputFeedError(1311) | media-path-input-feed-error |
| mediaPathInputJam(1312) | media-path-input-jam |
| mediaPathInputEmpty(1313) | media-path-input-empty |
| mediaPathOutputFeedError(1321) | media-path-output-feed-error |
| mediaPathOutputJam(1322) | media-path-output-jam |
| mediaPathOutputFull(1323) | media-path-output-full |
| mediaPathPickRollerLifeWarn(1331) | media-path-pick-roller-life-warn |
| mediaPathPickRollerLifeOver(1332) | media-path-pick-roller-life-over |
| mediaPathPickRollerFailure(1333) | media-path-pick-roller-failure |
| mediaPathPickRollerMissing(1334) | media-path-pick-roller-missing |
| -- Scanner Group |  |
| scannerLightLifeAlmostOver(5101) | scanner-light-life-almost-over |
| scannerLightLifeOver(5102) | scanner-light-life-over |
| scannerLightFailure(5103) | scanner-light-failure |
| scannerSensorLifeAlmostOver(5104) | scanner-sensor-life-almost-over |
| scannerSensorLifeOver(5105) | scanner-sensor-life-over |
| scannerSensorFailure(5106) | scanner-sensor-failure |
| -- Scan Media Path Group |  |
| scanMediaPathTrayMissing(5201) | scan-media-path-tray-missing |
| scanMediaPathTrayAlmostFull(5202) | scan-media-path-tray-almost-full |
| scanMediaPathTrayFull(5203) | scan-media-path-tray-full |
| scanMediaPathFailure(5205) | scan-media-path-failure |
| scanMediaPathJam(5206) | scan-media-path-jam |
| scanMediaPathInputRequest(5210) | scan-media-path-input-request |
| scanMediaPathInputFeedError(5211) | scan-media-pathinput-feed-error |
| scanMediaPathInputJam(5212) | scan-media-path-input-jam |
| scanMediaPathInputEmpty(5213) | scan-media-path-input-empty |
| scanMediaPathOutputFeedError(5221) | scan-media-path-output-feed-error |
| scanMediaPathOutputJam(5222) | scan-media-path-output-jam |
| scanMediaPathOutputFull(5223) | scan-media-path-output-full |
| scanMediaPathPickRollerLifeWarn(5231) | scan-media-path-pick-roller-life-warn |
| scanMediaPathPickRollerLifeOver(5232) | scan-media-path-pick-roller-life-over |
| scanMediaPathPickRollerFailure(5233) | scan-media-path-pick-roller-failure |
| scanMediaPathPickRollerMissing(5234) | scan-media-path-pick-roller-missing |
| -- Fax Modem Group |  |
| faxModemMissing(6101) | fax-modem-missing |
| faxModemLifeAlmostOver(6102) | fax-modem-life-almost-over |
| faxModemLifeOver(6103) | fax-modem-life-over |
| faxModemTurnedOn(6104) | fax-modem-turned-on |
| faxModemTurnedOff(6105) | fax-modem-turned-off |
| faxModemInactivityTimeout(6110) | fax-modem-inactivity-timeout |
| faxModemProtocolError(6111) | fax-modem-protocol-error |
| faxModemEquipmentFailure(6112) | fax-modem-equipment-failure |
| faxModemNoDialTone(6113) | fax-modem-no-dial-tone |
| faxModemLineBusy(6114) | fax-modem-line-busy |
| faxModemNoAnswer(6115) | fax-modem-no-answer |
| faxModemVoiceDetected(6116) | fax-modem-voice-detected |
| faxModemCarrierLost(6117) | fax-modem-carrier-lost |
| faxModemTrainingFailure(6118) | fax-modem-training-failure |

- Conformance Requirements

6.1 Printer MIB Agent Conformance Requirements

To claim conformance to this specification, a Printer MIB Agent implementation for a Multifunction Device:

- MUST implement the prtAlertTable defined in IETF Printer MIB v2;

- SHOULD implement the prtAlertTable defined in IETF Printer MIB v2 [RFC3805] as persistent across power cycles and hardware reconfigurations, for reliable fleet management.

- MUST support the MFD alert groups defined in section 5.1 of this specification which are registered in PrtAlertGroupTC in IANA Printer MIB [IANAPRT], if the corresponding functionality (e.g., scan) is supported on the MFD;

- MUST support the MFD and Printer alert codes defined in section 5.2 of this specification which are registered in PrtAlertCodeTC in IANA Printer MIB [IANAPRT], if the corresponding functionality (e.g., scan) is supported on the MFD;

- MUST encode and interpret values of the prtAlertGroup and prtAlertCode objects defined in IETF Printer MIB v2 [RFC3805] according to the registry in IANA Printer MIB [IANAPRT].

6.2 Printer MIB Client Conformance Requirements

To claim conformance to this specification, a Printer MIB Client implementation that supports Multifunction Devices:

- MUST support the prtAlertTable defined in IETF Printer MIB v2;

- MUST support the MFD alert groups defined in section 5.1 of this specification which are registered in PrtAlertGroupTC in IANA Printer MIB [IANAPRT], if the corresponding functionality (e.g., scan) is supported on the Printer MIB Client;

- MUST support the MFD and Printer alert codes defined in section 5.2 of this specification which are registered in PrtAlertCodeTC in IANA Printer MIB [IANAPRT], if the corresponding functionality (e.g., scan) is supported on the Printer MIB Client;

- MUST decode and interpret values of the prtAlertGroup and prtAlertCode objects defined in IETF Printer MIB v2 [RFC3805] according to the registry in IANA Printer MIB [IANAPRT].

6.3 IPP Printer Conformance Requirements

To claim conformance to this specification, an IPP Printer implementation for a Multifunction Device:

- MUST support the IPP Printer “printer-alert” and “printer-alert-description” attributes defined in PWG IPP Printer State Extensions [PWG5100.9];

- MUST support the MFD alert groups defined in section 5.1 of this specification which are registered in PrtAlertGroupTC in IANA Printer MIB [IANAPRT] for keyword values in “printer-alert”, if the corresponding functionality (e.g., scan) is supported on the MFD;

- MUST support the MFD and Printer alert codes defined in section 5.2 of this specification which are registered in PrtAlertCodeTC in IANA Printer MIB [IANAPRT] and IANA IPP Registry [IANAIPP] for keyword values in “printer-alert” and “printer-state-reasons”, if the corresponding functionality (e.g., scan) is supported on the MFD;

- MUST encode and interpret values of “printer-alert” and “printer-state-reasons”  according to the IANA Printer MIB [IANAPRT] and IANA IPP Registry [IANAIPP];

- MUST implement the prtAlertTable defined in IETF Printer MIB v2, if a Printer MIB Agent is implemented.

6.4 IPP Client Conformance Requirements

To claim conformance to this specification, an IPP Client implementation that supports Multifunction Devices:

- MUST support the IPP Printer “printer-alert” and “printer-alert-description” attributes defined in PWG IPP Printer State Extensions [PWG5100.9];

- MUST support the MFD alert groups defined in section 5.1 of this specification which are registered in PrtAlertGroupTC in IANA Printer MIB [IANAPRT] for keyword values in “printer-alert”, if the corresponding functionality (e.g., scan) is supported on the IPP Client;

- MUST support the MFD and Printer alert codes defined in section 5.2 of this specification which are registered in PrtAlertCodeTC in IANA Printer MIB [IANAPRT] and IANA IPP Registry [IANAIPP] for keyword values in “printer-alert” and “printer-state-reasons”, if the corresponding functionality (e.g., scan) is supported on the IPP Client;

- MUST decode and interpret values of “printer-alert” and “printer-state-reasons”  according to the IANA Printer MIB [IANAPRT] and IANA IPP Registry [IANAIPP];

- MUST support the prtAlertTable defined in IETF Printer MIB v2, if a Printer MIB Client is implemented.

- Internationalization Considerations

The MFD alert groups and alert codes defined in this document do not add any internationalization considerations beyond those covered in section 8 of the IETF Printer MIB v2 [RFC3805].  The MFD extensions to the IPP Printer “printer-alert” and “printer-state-reasons” attributes defined in this document do not add any internationalization considerations beyond covered in section 7 of IPP/1.1 Model and Semantics [RFC2911].

- Security Considerations

The MFD alert groups and alert codes defined in this document do not add any security considerations beyond those covered in section 9 of the IETF Printer MIB v2 [RFC3805].  The MFD extensions to the IPP Printer “printer-alert” and “printer-state-reasons” attributes defined in this document do not add any security considerations beyond covered in section 8 of IPP/1.1 Model and Semantics [RFC2911].

- IANA Considerations

9.1 Alert Groups

This section contains the exact registration information for IANA to update the IANA-PRINTER-MIB PrtAlertGroupTC Registry [IANAPRT], according to the procedures defined in the IETF Printer MIB v2 [RFC3805] section 5, to cover the new alert groups defined in section 5.1 of this document.  Add to PrtAlertGroupTC the following:

-- Values for the ScanDevice

scanDevice(50),          -- MFD Extension

scanner(51),             -- MFD Extension

scanMediaPath(52),       -- MFD Extension

-- Values (50) to (59) reserved for the ScanDevice

-- Values for the FaxDevice

faxDevice(60),           -- MFD Extension

faxModem(61),            -- MFD Extension

-- Values (60) to (69) reserved for the FaxDevice

-- Values for other common subunits

outputChannel(70),       -- MFD Extension

-- Values (70) to (79) reserved for common subunits

9.2 Alert Codes

This section contains the exact registration information for IANA to update the IANA-PRINTER-MIB PrtAlertCodeTC Registry [IANAPRT], according to the procedures defined in the IETF Printer MIB v2 [RFC3805] section 5, to cover the new alert codes defined in sections 5.2 and 5.3 of this document.  Add to PrtAlertCodeTC the following:

-- Input Group

inputMediaTrayFeedError(814),

inputMediaTrayJam(815),

inputMediaTrayFailure(816),

inputMediaTrayPickRollerLifeWarn(817),

inputMediaTrayPickRollerLifeOver(818),

inputMediaTrayPickRollerFailure(819),

inputMediaTrayPickRollerMissing(820),

-- Output Group

outputMediaTrayFeedError(905),

outputMediaTrayJam(906),

outputMediaTrayFailure(907),

-- Media Path Group

mediaPathFailure(1305),

mediaPathJam(1306),

mediaPathInputRequest(1310),

mediaPathInputFeedError(1311),

mediaPathInputJam(1312),

mediaPathOutputFeedError(1321),

mediaPathOutputJam(1322),

mediaPathOutputFull(1323),

mediaPathPickRollerLifeWarn(1331),

mediaPathPickRollerLifeOver(1332),

mediaPathPickRollerFailure(1333),

mediaPathPickRollerMissing(1334),

-- Scanner Group

scannerLightLifeAlmostOver(5101),

scannerLightLifeOver(5102),

scannerLightFailure(5103),

scannerSensorLifeAlmostOver(5111),

scannerSensorLifeOver(5112),

scannerSensorFailure(5113),

-- Scan Media Path Group

scanMediaPathTrayMissing(5201),

scanMediaPathTrayAlmostFull(5202),

scanMediaPathTrayFull(5203),

scanMediaPathFailure(5205),

scanMediaPathJam(5206),

scanMediaPathInputRequest(5210),

scanMediaPathInputFeedError(5211),

scanMediaPathInputJam(5212),

scanMediaPathOutputFeedError(5221),

scanMediaPathOutputJam(5222),

scanMediaPathOutputFull(5223),

scanMediaPathPickRollerLifeWarn(5231),

scanMediaPathPickRollerLifeOver(5232),

scanMediaPathPickRollerFailure(5233),

scanMediaPathPickRollerMissing(5234),

-- Fax Modem Group

faxModemMissing(6101),

faxModemLifeAlmostOver(6102),

faxModemLifeOver(6103),

faxModemTurnedOn(6104),

faxModemTurnedOff(6105),

faxModemInactivityTimeout(6110),

faxModemProtocolError(6111),

faxModemEquipmentFailure(6112),

faxModemNoDialTone(6113),

faxModemLineBusy(6114),

faxModemNoAnswer(6115),

faxModemVoiceDetected(6116),

faxModemCarrierLost(6117),

faxModemTrainingFailure(6118),

## 9.3 IPP Attribute and Keyword Value Registrations

This section contains the exact registration information for IANA to update according to the procedures defined in [RFC2911].

The registry entry will contain the following information:

Section 9 (References)

[PWG5107.3] PWG Multifunction Device Alerts, PWG 5107.3, June 2012.

ftp://ftp.pwg.org/pub/pwg/candidates/

cs-pmpmfdalerts10-20120629-5107.3.pdf

Section 2 (Keyword Attribute Values)

Attribute Name (attribute syntax)               Reference

---------------------------------               ---------

printer-state-reasons (1setOf type2 keyword)    [RFC2911]

input-media-tray-feed-error                   [PWG5107.3]

input-media-tray-jam                          [PWG5107.3]

input-media-tray-failure                      [PWG5107.3]

input-pick-roller-life-warn                   [PWG5107.3]

input-pick-roller-life-over                   [PWG5107.3]

input-pick-roller-failure                     [PWG5107.3]

input-pick-roller-missing                     [PWG5107.3]

output-media-tray-feed-error                  [PWG5107.3]

output-media-tray-jam                         [PWG5107.3]

output-media-tray-failure                     [PWG5107.3]

media-path-failure                            [PWG5107.3]

media-path-jam                                [PWG5107.3]

media-path-input-request                      [PWG5107.3]

media-path-input-feed-error                   [PWG5107.3]

media-path-input-jam                          [PWG5107.3]

media-path-input-empty                        [PWG5107.3]

media-path-output-feed-error                  [PWG5107.3]

media-path-output-jam                         [PWG5107.3]

media-path-output-full                        [PWG5107.3]

media-path-pick-roller-life-warn              [PWG5107.3]

media-path-pick-roller-life-over              [PWG5107.3]

media-path-pick-roller-failure                [PWG5107.3]

media-path-pick-roller-missing                [PWG5107.3]

scanner-light-life-almost-over                [PWG5107.3]

scanner-light-life-over                       [PWG5107.3]

scanner-light-failure                         [PWG5107.3]

scanner-sensor-life-almost-over               [PWG5107.3]

scanner-sensor-life-over                      [PWG5107.3]

scanner-sensor-failure                        [PWG5107.3]

scan-media-path-tray-missing                  [PWG5107.3]

scan-media-path-tray-almost-full              [PWG5107.3]

scan-media-path-tray-full                     [PWG5107.3]

scan-media-path-failure                       [PWG5107.3]

scan-media-path-jam                           [PWG5107.3]

scan-media-path-input-request                 [PWG5107.3]

scan-media-path-input-feed-error              [PWG5107.3]

scan-media-path-input-jam                     [PWG5107.3]

scan-media-path-output-feed-error             [PWG5107.3]

scan-media-path-output-jam                    [PWG5107.3]

scan-media-path-output-full                   [PWG5107.3]

scan-media-path-pick-roller-life-warn         [PWG5107.3]

scan-media-path-pick-roller-life-over         [PWG5107.3]

scan-media-path-pick-roller-failure           [PWG5107.3]

scan-media-path-pick-roller-missing           [PWG5107.3]

fax-modem-missing                             [PWG5107.3]

fax-modem-life-almost-over                    [PWG5107.3]

fax-modem-life-over                           [PWG5107.3]

fax-modem-turned-on                           [PWG5107.3]

fax-modem-turned-off                          [PWG5107.3]

fax-modem-inactivity-timeout                  [PWG5107.3]

fax-modem-protocol-error                      [PWG5107.3]

fax-modem-equipment-failure                   [PWG5107.3]

fax-modem-no-dial-tone                        [PWG5107.3]

fax-modem-line-busy                           [PWG5107.3]

fax-modem-no-answer                           [PWG5107.3]

fax-modem-voice-detected                      [PWG5107.3]

fax-modem-carrier-lost                        [PWG5107.3]

fax-modem-training-failure                    [PWG5107.3]

- References

10.1 Normative References

[IANAIPP]	“IANA IPP Registry”, IANA Registry, http://www.iana.org/assignments/ipp-registrations

[IANAPRT]	“IANA Printer MIB”, IANA Registry, http://www.iana.org/assignments/ianaprinter-mib

[PWG5100.9]	I. McDonald, C. Whittle, “IPP Printer State Extensions”, PWG 5100.9-2009, July 2009, ftp://ftp.pwg.org/pub/pwg/candidates/cs-ippstate10-20090731-5100.9.pdf

[PWG5108.1]	W. Wagner, P. Zehler, “MFD Model and Common Semantics”, PWG 5108.1-2011, April 2011, ftp://ftp.pwg.org/pub/pwg/candidates/cs-sm20-mfdmodel10-20110415-5108.1.pdf

[RFC2119]	S. Bradner, “IETF Key words for use in RFCs to Indicate Requirement Levels”, RFC 2119 / BCP14, March 1997, http://www.ietf.org/rfc/rfc2119.txt

[RFC2616]	R. Fielding, J. Gettys, J. Mogul, H. Frystyk, L. Masinter, P. Leach, T. Berners-Lee, "Hypertext Transfer Protocol -- HTTP/1.1", RFC 2616, http://www.ietf.org/rfc/rfc2616.txt

[RFC3805]	R. Bergman, H. Lewis, I. McDonald, “IETF Printer MIB v2”, RFC 3805, June 2004, http://www.ietf.org/rfc/rfc3805.txt

[RFC3806]	R. Bergman, H. Lewis, I. McDonald, "Printer Finishing MIB", RFC 3806, June 2004, http://www.ietf.org/rfc/rfc3806.txt

10.2 Informative References

[RFC1759]	R. Smith, F. Wright, T. Hastings, S. Zilles, J. Gyllenskog, “IETF Printer MIB”, RFC 1759, March 1995, http://www.ietf.org/rfc/rfc1759.txt

[RFC2567]	F.D. Wright, "IETF Design Goals for an Internet Printing Protocol", RFC 2567, April 1999, http://www.ietf.org/rfc/rfc2567.txt

[RFC2707]	R. Bergman, T. Hastings, S. Isaacson, H. Lewis, "IETF Job Monitoring MIB - V1.0", RFC 2707, September 1999, http://www.ietf.org/rfc/rfc2707.txt

[WS-MGMT]	DMTF, "DMTF Web Services for Management (WS-Management) v1.0.0", DSP0226, February 2008, http://dmtf.org/sites/default/files/standards/documents/DSP0226_1.0.0.pdf

[WSDM]	OASIS, “OASIS Web Services Distributed Management (WSDM)”, composed of [WSDM-MOWS], [WSDM-MUWS1], [WSDM-MUWS2], August 2006, http://www.oasis-open.org/committees/download.php/20571/wsdm-1.1-os-01.zip

[WSDM-MOWS]	K. Wilson, I. Sadukin, "OASIS Web Services Distributed Management: Management of Web Services (WSDM-MOWS 1.1)", August 2006, http://www.oasis-open.org/committees/download.php/20576/wsdm-muws1-1.1-spec-os-01.pdf

[WSDM-MUWS1]	V. Bullard, W. Vambenepe, "OASIS Web Services Distributed Management: Management Using Web Services (MUWS 1.1) Part 1", August 2006, http://www.oasis-open.org/committees/download.php/20576/wsdm-muws1-1.1-spec-os-01.pdf

[WSDM-MUWS2]	V. Bullard, W. Vambenepe, "OASIS Web Services Distributed Management: Management Using Web Services (MUWS 1.1) Part 2", August 2006, http://www.oasis-open.org/committees/download.php/20575/wsdm-muws2-1.1-spec-os-01.pdf

- Authors' Addresses

Ira McDonald

High North Inc

PO Box 221

Grand Marais, MI  49839

Phone:  906-494-2434

Email:  blueroofmusic@gmail.com

Ron Bergman

Email:  RGBergman@hotmail.com

The following individuals also contributed to the development of this document:

| Charles Baxter | Xerox |  |  |
| --- | --- | --- | --- |
| John Boyd | Toshiba |  |  |
| Lee Farrell |  |  |  |
| Walt Filbrich |  |  |  |
| Gail Giansiracusa | Kyocera Mita |  |  |
| Sheng Lee | Toshiba |  |  |
| Harry Lewis Stuart Rowley | InfoPrint Solutions |  |  |
| Michael Sweet | Apple |  |  |
| Ole Skov Thomas Silver | MPI Tech Xerox |  |  |
| Jerry Thrasher | Lexmark |  |  |
| Paul Tykodi | Tykodi Consulting Services |  |  |
| Bill Wagner | TIC |  |  |
| Craig Whittle | Sharp Labs |  |  |
| Peter Zehler | Xerox |  |  |