# Annex F

(informative)

Document Format Definitions

This annex lists document formats that are implemented by printer vendors and print system implementors.  This annex also specifies certain document attributes that are specific to certain document formats.  These attributes are to be considered adjunct to the document production instructions specified in clause 9.

## F.1 Document formats

This sub-clause lists the standard document-format values that may be used in the various ISO DPA attributes that have document-formats as values.  The document-formats listed in the following syntax have been extracted from the Internet Printer MIB prtInterpreterLangFamily object specification (RFC 1759), and augmented by newly registered values.  These document formats, and future additions to the list, are registered with the Internet Assigned Number Authority.  The list may be obtained by anonyous ftp from the ftp-server containing the iana/assignments directory:

--   The ftp server is ftp.isi.edu.

--   The subdirectory is /in-notes/iana/assignments/.

--   The file name is printer-language-numbers.

prtInterpreterLangFamily OBJECT-TYPE
    -- This value is a type 2 enumeration
    SYNTAX     INTEGER {
          other(1),
          langPCL(3),        -- PCL.  Starting with PCL version 5, 
                             -- HP-GL/2 is included as part of the
                             -- PCL language.
                             -- PCL and HP-GL/2 are registered 
                             -- trademarks of Hewlett-Packard Company.
          langHPGL(4),       -- Hewlett-Packard Graphics Language. 
                             -- HP-GL is a registered trademark of
                             -- Hewlett-Packard Company.
          langPJL(5),        -- Peripheral Job Language. Appears in the
                             -- data stream between data intended for a
                             -- page description language.
                             -- Hewlett-Packard Co.
          langPS(6),         -- PostScript Language (tm)
                             -- Postscript - a trademark of Adobe
                             -- Systems Incorporated which may be
                             -- registered in certain jurisdictions
          langPSPrinter(42), -- The PostScript Language used for
                             -- control (with any PDLs)
                             -- Adobe Systems Incorporated
          langIPDS(7),       -- Intelligent Printer Data Stream
                             -- Bi-directional print data stream for
                             -- documents consisting of data objects
                             -- (text, image, graphics, bar codes),
                             -- resources (fonts, overlays) and page,
                             -- form and finishing instructions.
                             -- Facilitates system level device
                             -- control, document tracking and error
                             -- recovery throughout the print process.
                             -- Pennant Systems, IBM

langPPDS(8),       -- IBM Personal Printer Data Stream.
                             -- Originally called IBM ASCII, the name
                             -- was changed to PPDS when the Laser
                             -- Printer was introduced in 1989.
                             -- Lexmark International, Inc.
          langEscapeP(9),
          langEpson(10),
          langDDIF(11),      -- Digital Document Interchange Format
                             -- Digital Equipment Corp., Maynard MA
          langInterpress(12),
          langISO6429(13),   -- ISO/IEC 6429.  Control functions for Coded
                             -- Character Sets (has ASCII control
                             -- characters, plus additional controls for
                             -- character imaging devices.)
                             -- ISO Standard, Geneva, Switzerland
          langLineData(14),  -- line-data:  Lines of data as separate
                             -- ASCII or EBCDIC records and containing
                             -- no control functions (no CR, LF, HT, FF,
                             -- etc.). For use with traditional line
                             -- printers.  May use CR and/or LF to
                             -- delimit lines, instead of records.  See
                             -- ISO/IEC 10175 Document Printing Application
                             -- (DPA)
                             -- ISO standard, Geneva, Switzerland
          langMODCA(15),     -- Mixed Object Document Content Architecture
                             -- Definitions that allow the composition,
                             -- interchange -- and presentation of final
                             -- form documents as a collection of data
                             -- objects (text, image, graphics, bar
                             -- codes), resources (fonts, overlays) and
                             -- page, form and finishing instructions.
                             -- Pennant Systems, IBM
          langREGIS(16),     -- Remote Graphics Instruction Set,
                             -- Digital Equipment Corp., Maynard MA
          langSCS(17),       -- SNA Character String
                             -- Bi-directional print data stream for SNA
                             -- LU-1 mode of communications
                             -- IBM
          langSPDL(18),      -- ISO/IEC 10180 Standard Page Description
                             -- Language
                             -- ISO Standard
          langTEK4014(19),
          langPDS(20),
          langIGP(21),
          langCodeV(22),     -- Magnum Code-V, Image and printer control
                             -- language used to control impact/dot-
                             -- matrix printers.
                             -- QMS, Inc., Mobile AL
          langDSCDSE(23),    -- DSC-DSE:  Data Stream Compatible and 
                             -- Emulation Bi-directional print data 
                             -- stream for non-SNA (DSC) and SNA LU-3 
                             -- 3270 controller (DSE) communications
                             -- IBM
          langWPS(24),       -- Windows Printing System, Resource based
                             -- command/data stream used by Microsoft At
                             -- Work Peripherals.
                             -- Developed by the Microsoft Corporation.
          langLN03(25),      -- Early DEC-PPL3, Digital Equipment Corp.
          langCCITT(26),
          langQUIC(27),      -- QUIC (Quality Information Code), Page
                             -- Description Language for laser printers.
                             -- Included graphics, printer control
                             -- capability and emulation of other well-
                             -- known printer .
                             -- QMS, Inc.
          langCPAP(28),      -- Common Printer Access Protocol
                             -- Digital Equipment Corp.
          langDecPPL(29),    -- Digital ANSI-Compliant Printing Protocol
                             -- (DEC-PPL)
                             -- Digital Equipment Corp.
          langSimpleText(30),-- simple-text:  character coded data,
                             -- including NUL, CR , LF, HT, and FF
                             -- control characters.  See ISO/IEC 10175
                             -- Document Printing Application (DPA)
                             -- ISO standard, Geneva, Switzerland
          langNPAP(31),      -- Network Printer Alliance Protocol
                             -- IEEE 1284.1
          langDOC(32),       -- Document Option Commands, Appears in the
                             -- data stream between data intended for a
                             -- page description .
                             -- QMS, Inc.
          langimPress(33),   -- imPRESS, Page description language
                             -- originally developed for the ImageServer
                             -- line of systems. A binary language
                             -- providing representations for text,
                             -- simple graphics (rules, lines, conic
                             -- sections), and some large forms (simple
                             -- bit-map and CCITT group 3/4 encoded).The
                             -- language was intended to be sent over an
                             -- 8-bit channel and supported early
                             -- document preparation languages (e.g. TeX
                             -- and TROFF).
                             -- QMS, Inc.
          langPinwriter(34), -- 24 wire dot matrix printer  for
                             -- USA, Europe, and Asia except Japan.
                             -- More widely used in Germany, and some
                             -- Asian countries than in US.
                             -- NEC
          langNPDL(35),      -- Page printer  for Japanese
                             -- market.
                             -- NEC
          langNEC201PL(36),  -- Serial printer language used in the
                             -- Japanese market.
                             -- NEC
          langAutomatic(37), -- Automatic PDL sensing.  Automatic
                             -- sensing of the interpreter language
                             -- family by the printer examining the
                             -- document content.  Which actual
                             -- interpreter language families are sensed
                             -- depends on the printer implementation.
          langPages(38),     -- Page printer Advanced Graphic Escape Set
                             -- IBM Japan
          langLIPS(39),      -- LBP Image Processing System
          langTIFF(40),      -- Tagged Image File Format (Aldus)
          langDiagnostic(41),-- A hex dump of the input to the
                             -- interpreter
          langCaPSL(43),     -- Canon Print Systems Language
          langEXCL(44),      -- Extended Command Language
                             -- Talaris Systems Inc.
          langLCDS(45),      -- Line Conditioned Data Stream
                             -- Xerox Corporation
          langXES(46)        -- Xerox Escape Sequences
                             -- Xerox Corporation
          langPCLXL(47)      -- Printer Control Language. Extended language
                             -- features for printing, and printer control.
                             -- Technical reference manual # TBD.
                             -- Hewlett-Packard Co.
          langART(48)        -- Advanced Rendering Tools (ART). Page
                             -- Description language originally developed for
                             -- the Laser Press printers. Technical reference
                             -- manual: "ART IV Reference Manual", No F33M.
                             -- Fuji Xerox Co., Ltd.
          langTIPSI(49)      -- Transport Independent Printer System Interface
                             -- Tech Reference - see IEEE Std. 1284.1
               }
    MAX-ACCESS read-only
    STATUS     current
    DESCRIPTION
        "The family name of a Page Description Language (PDL) or
        control language which this interpreter in the printer can interpret
        or emulate.  This type 2 list of enumerations requires review before
        additional entries are made."
    ::= { prtInterpreterEntry 2 }

### F.1.1 Relationship between ISO/IEC 10175 OIDs and IANA enum values

ISO/IEC 10175 uses Object Identifiers to identify document formats, as ISO/IEC 10175 does for all attributes for which implementor extension is desired with implementor-defined Object Identifiers.  However, the Internet Printer MIB (RFC 1759) uses simple enum integer values that are registered with the IANA.  Implementors must submit requests to IANA in order to obtain unique values.

ISO/IEC 10175 assigns a value-class Object Identifier for purposes of assigning full Object Identifiers: id-vc-document-format:

iso(1) standard(0) dpa(10175) part-1(1) vc(6) document-format(14)

The complete ISO/IEC 10175 id-val-document-format-xxx object identifier is formed by appending the Printer MIB prtInterpreterLangFamily enum value to the id-vc-document-format OID.

For example, consider the PCL enum value 3.  The ISO/IEC 10175 document-format OID for PCL is:

iso(1) standard(0) dpa(10175) part-1(1) vc(6) document-format(14) PCL(3)

See F.5 Document-format object identifiers for the complete list of OID assignments of document-format object identifiers.

## F.2 Document-production-instruction (DPI) attributes

Many document formats support document production instruction (DPI) attributes that the client may specify to control the processing of a document.  The following attributes are available for use by various document formats (see table in F.4):

### F.2.1 New-line-option

This attribute identifies how the server shall determine how lines are delimited in the document data.  Document data is represented as stream or record data:

new-line-option ATTRIBUTE
        WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax
        SINGLE VALUE
        ::= id-att-new-line-option

- In stream data, the lines are delimited by the ASCII control codes: CARRIAGE RETURN (CR = hex 0D) and LINE FEED (LF = hex 0A) to reflect the various actual implementations.  The value of the new-line-option attribute identifies whether LF, CR, neither, or both shall be interpreted to start a new line.  See ANSI ANS X3.4 and ISO/IEC 6429.

- In record data, the lines are delimited by a record count embedded in the data.

The following standard values are defined:

| Simple Name | Object Identifier | Descriptor Text |
| --- | --- | --- |
| cr-and-lf | id-val-nl-option-cr-and-lf | Both CR and LF are needed for each new line. The server shall interpret CR as strict horizontal movement of the active position (to the first character position on the same line) and LF as strict vertical movement (to the same character position on the next line). So both a CR and an LF are needed to signify a new line as specified in ISO/IEC 6429. |
| lf | id-val-nl-option-lf | Interpret LF as a new-line. The server shall interpret LF as implying a CR as well, i.e., a single LF shall move the active position to the first character position on the next line. NOTE: This option is the so-called "new-line" option, permitted by ASCII (ANS X3.4) and used by UNIX(tm), C, and C++. CR shall have strict horizontal movement of the active (to the first character position on the same line). |
| cr | id-val-nl-option-cr | Interpret CR as a new-line. The server shall interpret CR as moving the active position to the first character position on the next line and LF as strict vertical movement (to the same character position on the next line). NOTE: This option is used in a number of PC applications. |
| cr-or-lf | id-val-nl-option-cr-or-lf | Interpret either CR or LF as new-lines. The server shall interpret either a CR or a LF as moving the active position to the first character position on the next line. |
| counted-4-octet-aligned | id-val-nl-option-counted-4-octet-aligned | The document data is a sequence of octets that is broken up into aligned counted records. Each count is a four-octet unsigned integer specifying the number of octets following in the record, not counting the 0 to 3 pad octets at the end of the record data so that the integers (and records) are four-octet aligned. |
| unknown | id-val-nl-option-unknown | The document delimits lines according to one of the four options using 1) CR and LF, 2) LF, 3) CR, or 4) CR or LF, and does not use counted records. The server shall provide some implementation default action, which may correspond to one of the actions of the other id-val-nl-option-xxx options, except id-val-nl-option-counted-4-octet-aligned, or may include the server auto-sensing between the other id-val-nl-option-xxx options, except id-val-nl-option-counted-4-octet-aligned, by examining an implementation-defined number of octets of document data. |

### F.2.2 Carriage-control-type

This attribute identifies the type of carriage control that the server shall assume when interpreting the document.  Carriage control specifies the amount of advancement, if any, of the active position (carriage on old technologies), at the beginning of each line or record.  When carriage control information is stored in the document content, the first octet(s) of each line or record specify the amount of the advancement.

carriage-control-type ATTRIBUTE
        WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax
        SINGLE VALUE
        ::= id-att-carriage-control-type

The following standard values are defined:

| Simple Name | Object Identifier | Descriptor Text |
| --- | --- | --- |
| none | id-val-generic-none | The document does not contain carriage control information. See 9.1.2. |
| ansi-ascii | id-val-cc-type-ansi-ascii | The first character of each line (or record) of the document shall be interpreted as carriage control specified according to the ANSI ANS X3.78 standard and coded in the ASCII (ANS X3.4) character set. Note: ansi-ascii is the so-called “FORTRAN carriage control” for use with the ASCII coded character set. |
| ansi-ebcdic | id-val-cc-type-ansi-ebcdic | The first character of each line (or record) of the document shall be interpreted as carriage control as specified according to the ANSI ANS X3.78 standard and coded in the EBCDIC character set. Note: ansi-ebcdic is the so-called “FORTRAN carriage control” for use with the EBCDIC coded character set. |
| machine | id-val-cc-type-machine | The first character of each line (or record) shall be interpreted as carriage control. The carriage controls are somewhat machine dependent but there are a standard set defined by the IBM System/370 Extended Architecture Reference Summary with IBM Document Order Number GX20-0157. It is intended that the server pass this carriage control information directly to the printer. |

### F.2.3 Table-reference-characters

This attribute specifies whether the first character of each line (or record) is a table reference character (TRC) which selects a character arrangement table.  When TRC is specified with carriage controls (see F2.2 carriage-control-type), e.g.  id-val-cc-type-ansi-xxx, the first character of each line (or record) is a print carriage control character and the second character (instead of the first character) of each line contains a table reference character (TRC) which selects a character arrangement table.

table-reference-characters ATTRIBUTE
        WITH ATTRIBUTE-SYNTAX booleanSyntax
        SINGLE VALUE
        ::= id-att-table-reference-characters

### F.2.4 Shift-out-shift-in

This attribute specifies that the server shall change scanning modes when processing EBCDIC line-data that is to be printed with both a single-byte and a double-byte font.  The server shall scan single-octet data one octet at a time until the server finds a shift-out control code (hex 0E), after which it shall scan data two octets as a time.  When the server finds a shift-in control code (hex 0F), the server shall return to scanning one octet at a time.

shift-out-shift-in ATTRIBUTE
        WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax
        SINGLE VALUE
        ::= id-att-shift-out-shift-in

The following standard values are defined:

| Simple Name | Object Identifier | Descriptor Text |
| --- | --- | --- |
| none | id-val-generic-none | The document does not contain shift-out or shift-in control codes that are to be interpreted as specified for the other values of this attribute, i.e., this value specifies that shift-out and shift-in controls, if present, shall be interpreted as in ISO/IEC 2022. See 9.1.2. |
| one | id-val-shift-out-shift-in-one | Specifies that each shift-out code shall be converted to a blank followed by a Set Coded Font Local text-control code for the second font and each shift-in code be converted to a Set Coded Font Local text-control code for the first font followed by a blank. |
| two | id-val-shift-out-shift-in-two | Specifies that each shift-out code shall be converted to a Set Coded Font Local text-control code for the second font and each shift-in code be converted to a Set Coded Font Local text-control code for the first font. |
| three | id-val-shift-out-shift-in-three | Specifies that each shift-out code shall be converted to a Set Coded Font Local text-control code for the second font and each shift-in code be converted to a Set Coded Font Local text-control code for the first font followed by two blanks. |

## F.3	Printer object attributes

The following document-format-specific printer object attributes are defined:

### F.3.1	New-line-options-supported

This attribute identifies the particular new-line-option attribute values that are supported by this printer.

new-line-options-supported ATTRIBUTE
        WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax
        MULTI VALUE
        ::= id-att-new-line-options-supported

The standard values are those of the new-line-option document attribute (see F.2.1).

### F.3.2 Carriage-control-types-supported

carriage-control-types-supported ATTRIBUTE
        WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax
        MULTI VALUE
        ::= id-att-carriage-control-types-supported

The standard values are those of the carriage-control-type document attribute (see F.2.2).

### F.3.3 Table-reference-characters-supported

This attribute identifies the particular table-reference-characters attribute values that are supported by this printer.

table-reference-characters-supported ATTRIBUTE
        WITH ATTRIBUTE-SYNTAX booleanSyntax
        SINGLE VALUE
         ::= id-att-table-reference-characters-supported

### F.3.4 Shift-out-shift-in-supported

This attribute identifies the particular shift-out-shift-in attribute values that are supported by this printer.

shift-out-shift-in-supported ATTRIBUTE
        WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax
        MULTI VALUE
        ::= id-att-shift-out-shift-in-supported

The standard values are those of the shift-out-shift-in document attribute (see F.2.4).

## F.4	Document-format-variants

The following table lists the values of the document-format-variants attributes that are currently defined, along with the document-formats that they qualify.  A brief description of some of the document formats and associated document format variants follows the table.  Other document-formats (see F.1) may also use some of these document-format-variant values as well as their own, depending on implementation.

NOTE - The document-format-variant attribute corresponds to the Printer MIB prtInterpreterLangLevel object/attribute having the same semantics, data type, and values.  However, the Printer MIB specification does not list any of the values; each implementor is free to invent variants, without the need of standardization or registration.

The following table also lists the document production instruction (DPI) attributes defined for use with some of the document-formats (the usage of these DPI attributes is not repeated in the description of each document-format in F.4.  These DPI attributes are specified in F.2.

| document-format | document-format-variant (strings, not OIDs) | DPI attributes |
| --- | --- | --- |
| DDIF | "text", "graphics", "images", "colour" |  |
| DEC-PPL | "DEC-PPL2", "DEC-PPL3" | new-line-option, carriage-control-type, default-character-set |
| PCL | "1", "2", "3", "4", "5", "5e" | new-line-option, default-character-set |
| PPDS | "level 1", "level 2", "level 3" |  |
| Interpress | "level 1", "level 2", "level 3" |  |
| ISO-6429 | "iso-10538-pif-level1", "iso-10538-pif-level2", "iso-10538-pif-level3", "iso-10538-pif-level4", "iso-10538-fft-level1", "iso-10538-fft-level2a", "iso-10538-fft-level", "iso-10538-fft-level2b", "iso-10538-fft-level2c", "ht", "sgr-underline-bold", "cr-overstrike", "bs-overstrike" | new-line-option, default-character-set |
| line-data |  | new-line-option, carriage-control-type, table-reference-characters, shift-out-shift-in, default-character-set |
| MODCA | "is1", "pt1", "dr2", "dr3", "fs10", "fs20", "bcd1", "im1" |  |
| PostScript | "1", "2" (mandatory); "dsc1", "dsc2", "dsc3" (optional) |  |
| ReGIS | "color" |  |
| SCS |  | new-line-option, carriage-control-type, table-reference-characters, shift-out-shift-in |
| simple-text | "sgr-underline-bold", "cr-overstrike", "bs-overstrike" | new-line-option, carriage-control-type, default-character-set, size-unit-type, width, length, left-margin, right-margin, top-margin, bottom-margin, repeated-tab-stops, explicit-tab-stops, header-text, footer-text, number-pages |
| SPDL | "binary", "clear-text" |  |

Multiple values shall be represented as a comma list, e.g., "b,c".

The remainder of this sub-clause lists the document formats, in alphabetical order, that have known document-format-variants and specifies those variants.  This sub-clause does not list the document production instructions attributes that these document formats employ; instead, see the table in F.1.

### F.4.1	DDIF (Digital Document Interchange Format)

DDIF is the CDA(tm) Architecture document format used to represent formattable text, graphics, and images by Digital Equipment Corporation and third party applications.  DDIF uses ASN.1 for its representation and is capable of representing the semantics of the popular de-facto and de-jure formattable document formats.  DDIF is defined in the CDA Reference Manual, Digital Order Number AA-PBD3A-TE.

For purposes of ISO/IEC 10175, the DDIF document-format does not specify or imply any subsets. The following document-format-variant attributes may be used as hints of the content which may help in selecting an appropriate printer, but are not required:

| Document-format-variant | Description |
| --- | --- |
| “text” | the document contains text |
| “graphics” | the document contains synthetic graphics, e.g., line drawings |
| “images” | the document contains raster image data in CCITT Group 3 or Group 4 compressed or uncompressed representation. |
| “colour” | the document contains colour specifications |

### F.4.2	DEC ANSI-Compliant Printing Protocol (DEC-PPL)

DEC ANSI-Compliant Printing Protocol (DEC-PPL) is a conforming implementation of ISO/IEC 6429 with extensions.  There are two levels:

| Document-format-variant | Description |
| --- | --- |
| “dec-ppl2” | Level 2 subset (with conforming private extensions) for nonpage printers, such as the LA70, LA75, and LA324. See LA324 Programming Reference Manual, Digital document Order number EK-LA324-PM-002. |
| “dec-ppl3” | Level 3 subset (with conforming private extensions) for page printers, such as the LN03 and DEClaser2000. See Digital Printer Protocol Level 3, order number AA-PBWGA-TE. |

### F.4.3	HPGL (Hewlett Packard Graphics Language)

HPGL is a vector graphics language designed for pen-plotters. It consists of two letter mnemonic instructions designed to remind the user of its function.  HPGL/2 is designed to meet a wider range of technologies such as electrostatic, laser or ink jet devices.  Commands are grouped in areas like: configuration and status, vector, polygon, line and fill, character attributes.

HPGL is defined in the following documents:

F.4.4	Interpress

HPGL Programming Reference Manual.

HPGL/2 The HPGL/2 Reference Guide. ISBN 0-201-56308-8.

Levels, or subsets of Interpress may be identified via the document-format-variant attribute which may include one of the following applicable values:

| Document-format-variant | Description |
| --- | --- |
| “level 1” | Commercial Set |
| “level 2” | Publication Set |
| “level 3” | Professional Graphics Set |

### F.4.5	ISO/IEC 6429 (Control Functions for Coded Character Sets)

The ISO/IEC 6429 document format consists of control functions coded according to ISO/IEC 6429 Control Functions for Coded Character Sets.  ISO/IEC 6429:1992 contains the 7-bit ASCII (ANS X3.4) control characters (CR, LF, HT, FF, etc.) as the C0 set (hex 00 to 1F) and additional controls for character imaging devices as the C1 set (hex 80 to 9F).  The control functions that may be included in the document include graphic character set selection, font selection, line spacing, and character spacing.  These will take precedence over ISO DPA document production attributes included with the job, if any.

For purposes of ISO/IEC 10175, levels and subsets of ISO/IEC 6429 may be identified by the document-format-variant attribute which may include one the following applicable values:

| Document-format-variant | Description |
| --- | --- |
| “iso-10538-pif-level1” | Level 1 Page Image Format (PIF) from ISO/IEC 10538 Control Functions for Text Communication. |
| “iso-10538-pif-level2” | Level 2 PIF from ISO/IEC 10538 Control Functions for Text Communication. |
| “iso-10538-pif-level3” | Level 3 PIF from ISO/IEC 10538 Control Functions for Text Communication. |
| “iso-10538-pif-level4” | Level 4 PIF from ISO/IEC 10538 Control Functions for Text Communication. |
| “iso-10538-fft-level1” | Level 1 Formatted and Formattable Text (FFT) from ISO/IEC 10538 Control Functions for Text Communication. |
| “iso-10538-fft-level2” | Level 2 FFT from ISO/IEC 10538 Control Functions for Text Communication. |
| “iso-10538-fft-level2a” | Level 2a FFT from ISO/IEC 10538 Control Functions for Text Communication. |
| “iso-10538-fft-level2b” | Level 2b FFT from ISO/IEC 10538 Control Functions for Text Communication. |
| “iso-10538-fft-level2c” | Level 2c FFT from ISO/IEC 10538 Control Functions for Text Communication. |

The document-format-variant attribute may include any of the following additional values, indicating additional information about the data, including the presence of additional control characters:

| Document-format-variant | Description |
| --- | --- |
| “ht” | indicates that the HORIZONTAL TAB (HT) control character may occur. The tab stops may be specified by the repeated-tab-stops or the explicit-tab-stops document attribute (see F.4.13 simple-text document format description). If these attributes are omitted, the server may use implementation-defined tab stops, such as every 8 positions. |
| “sgr-underline-bold” | the underline or bold rendition. See ISO/IEC 6429 for coding of the underline and bold SGR parameters. |
| “cr-overstrike” | indicates that the CARRIAGE RETURN (CR) control character may occur to overstrike entire lines to represent an underline or bold rendition. |
| “bs-overstrike” | indicates that the BACKSPACE (BS) control character may occur to overstrike the previous character to represent an underline or bold rendition. |

### F.4.6	Line-data

The line-data document format consists solely of 8-bit printing characters and SPACE (ASCII hex 20, EBCDIC hex 40). The new-line-option attribute specifies how lines of data are delimited, namely, by ASCII or EBCDIC CARRIAGE RETURN (CR) and/or LINE FEED (LF), or by a record count in the data (id-val-nl-option-counted-4-octet-aligned).  If the new-line-option attribute is not specified, the default action shall be the action of id-val-nl-option-unknown.

NOTE: line-data is frequently used with counted records.

Line-data shall not include any other ASCII or EBCDIC control characters, such as HORIZONTAL TAB (HT), BACK SPACE (BS), VERTICAL TAB (VT), or FORM FEED (FF).

If the server accesses the document data directly through a (local or distributed) file system, that file system may provide a record access method to the server that delimit each record, but such a document transfer-method is not specified by ISO/IEC 10175.

For the purposes of ISO/IEC 10175, the line-data document-format does not delineate or specify any particular subset or set of towers within the data stream.

### F.4.7	MODCA (Mixed object document content architecture)

MODCA is an IBM SAA data format which has gained wide spread acceptance and usage within IBM. It is a self defining document format based on an architected set of structured fields supporting text, image and graphics. A structured field consists of a length, introducer identifying MODCA and an ID which identifies the semantic content and syntax of the field. MODCA is defined by IBM publication SC31-6802.

For the purposes of ISO/IEC 10175, the MODCA document-format does not delineate or specify any particular subset or set of towers within the data stream. Levels, or subsets of MODCA may be identified via the document-format-variant document attribute which may include one or more of the following applicable values:

| Document-format-variant | Description |
| --- | --- |
| “is1” | MODCA Interchange Set 1 (SC31-6802). This is a formatted final form document interchange subset of IBM's Mixed Object Document Content Architecture. |
| “pt1” | PTOCA Presentation Text Interchange Subset 1 (SC31-6803). This is the MODCA Object Content Architecture for final form, presentation text. |
| “dr2” | GOCA Graphics Interchange Subset 2 (SC31-6804). This is Subset Level 2 of the MODCA Object Content Architecture for geometric vector graphics. |
| “dr3” | GOCA Graphics Interchange Subset 3 (SC31-6804). This is Subset Level 3 of the MODCA Object Content Architecture for geometric vector graphics. |
| “fs10” | IOCA Image Function Set 10 (SC31-6805). This identifies an Interchange function set of the MODCA Object Content Architecture for Raster Image data. |
| “fs20” | IOCA Image Function Set 20 (SC31-6805). This identifies an Interchange function set of the MODCA Object Content Architecture for Raster Image data. |
| “bcd1” | Bar Code Object Interchange Subset 1 (SH544-3202). This identifies an interchange subset of the MODCA Object Content Architecture for Bar Code data. |
| “im1” | Compatibility Image (SH544-3202). This is a device dependent image object format for bi-level raster data. |

### F.4.8	PCL (Hewlett Packard Printer Control Language)

PCL is a popular PDL containing escape control sequences developed by Hewlett-Packard.

PCL printer language structure has evolved through five major levels of functionality driven by the combination of printer technology development, changing user needs and application software improvements.  The five levels are:

| Document-format-variant | Description |
| --- | --- |
| “1” | Print and Space functionality for simple, convenient, single user workstation output. |
| “2” | EDP (Electronic Data Processing)/Transaction functionality is a superset of PCL1. Functions were added for general purpose, multi-user system printing. |
| “3” | Office Word Processing functionality is a superset of PCL2. Functions were added for high quality, office document production. |
| “4” | Page Formatting functionality is a superset of PCL3. Functions were added for new page printing capabilities. |
| “5” | Office Publishing functionality is a superset of PCL4. New publishing capabilities include font scaling and HP-GL/2 graphics. |
| “5e” | Level 5 plus extensions. |

PCL is defined in the Laserjet III Technical Reference Manual, Hewlett Packard order number 33449-90903.

For the purposes of ISO/IEC 10175, the PCL format specifier does not delineate or specify any particular subset or set of towers within the data stream.  Levels, or subsets of PCL may be identified via the document-format-variant.  Each level is a subset of the level that follows it.

### F.4.9	PostScript

The PostScript language (PS) is a high function PDL supported in many environments.  For the purposes of ISO/IEC 10175, the PostScript language is defined by the PostScript Language Reference Manual, Second Edition, ISBN 0-201-18127-4.

PS must have a mandatory level variant specified by the document-format-variant attribute with meanings as follows:

| Document-format-variant | Description |
| --- | --- |
| “1” | Level 1. This variant identifies a PostScript language data stream that uses only operators from the PostScript Language Reference Manual, Second Edition, that are not noted as being Level 2 or DPS-specific. |
| “2” | Level 2. This variant identifies a PostScript language data stream that uses only operators from the PostScript Language Reference Manual, Second Edition, that are not noted as being DPS-specific. |

An optional document-format-variant value may specify a variant that, if present, indicates both the fact that the document conforms to the Adobe Document Structuring Conventions and the version of the Document Structuring Conventions to which it adheres.  Currently valid values for this variant are:

| Document-format-variant | Description |
| --- | --- |
| “dsc1” | This variant value identifies a PostScript language data stream that adheres to the commenting conventions set forth in Appendix C of the PostScript Language Reference Manual, ISBN 0-201-10174-2. |
| “dsc2” | This variant value identifies a PostScript language data stream that adheres to the commenting conventions set forth in the document, Document Structuring Conventions Specification, Version 2.1, dated January 16, 1989, available free of charge from Adobe Systems Incorporated; 1585 Charleston Road; P.O. Box 7900; Mountain View, CA 94039-7900. |
| “dsc3” | This variant value identifies a PostScript language data stream that adheres to the commenting conventions set forth in Appendix G of the PostScript Language Reference Manual, Second Edition, ISBN 0-201-18127-4. |

Version may be specified (in the future) by the document-format-version attribute.  This would identify major language changes or additions between levels of the PostScript Language.

### F.4.10	PPDS (Page Printer Data Stream)

Page Printer Data Stream (PPDS) is similar to ISO/IEC 6429 and includes support for APA printing and image in a fashion very similar to PCL.  PPDS is defined within the IBM document, IBM Laser Printer Technical Reference, S544-4226-1.

For the purposes of ISO/IEC 10175, the PPDS format specifier does not delineate or specify any particular subset or set of towers within the data stream.  Levels, or subsets of PPDS may be identified via the document-format-variant attribute.  Each level of PPDS is a subset of the level that follows it.

The following levels provide a progression of capabilities from one level to the next.

| Document-format-variant | Description |
| --- | --- |
| “level 1” | This is the basic level of PPDS, represented by the 9- and 24-wire Proprinter family of printers. These printers are categorized as wire matrix printers using continuous forms paper. |
| “level 2” | This level of PPDS, characterized by data stream enhancements in font selection, print quality selection, and paper handling, is represented by the Quietwriter and Quietwriter families of printers. |
| “level 3” | This level provides enhancements to the previous PPDS levels for page type printers such as the IBM 4019. The enhancements to the data stream allow random placement of text and graphics images. The font selection commands of Level 2 are enhanced to allow selection of typographic fonts. |

### F.4.11	ReGIS (Remote graphics instruction set)

ReGIS is the Digital Equipment Corporation's Remote Graphics Instruction Set which images text and graphics in a resolution-independent manner on video terminals and hardcopy printers (using a translator).  See PostScript Translator's Manual for ReGIS, order number AA-PBWFA-TE.

The following variant values may be included in the document-format-variant attribute:

| Document-format-variant | Description |
| --- | --- |
| “colour” | the document contains color |

### F.4.12	SCS (SNA character string)

SCS (SNA Character String) data streams contain EBCDIC character data and embedded escape control sequences for formatting line sequential text in a presentation space.

For the purposes of ISO/IEC 10175, the SCS document format does not delineate or specify any particular subset or set of towers within the data stream.

### F.4.13	Simple-text

The simple-text document format consists of 8-bit character coded data that is imaged character by character and line by line on one or more pages without loss of information.  By default the values from hex 00 to hex 7F shall consist only of the following characters:

| Name | hex | Server action - the server shall: |
| --- | --- | --- |
| Null (NUL) | 00 | ignore NUL and images nothing for it. |
| HORIZONTAL TAB (HT) | 09 | advance the active character position 1 to 8 character positions simulating tab stops set every 8 character positions by default. The repeated-tab-stops or the explicit-tab-stops attributes, if implemented, shall be used to specify different tab stops than the default values. |
| LINE FEED (LF) | 0A | advance the active character position (1) to the corresponding character position on the next line or (2) to the first character position on the next line, depending on the value of the new-line-option document attribute. |
| FORM FEED (FF) | 0C | advance the active character position to the corresponding character position on the first line of the next logical page. |
| CARRIAGE RETURN (CR) | 0D | return the active character position to the first character position (1) on the current line or (2) on the next line, depending on the value of the new-line-option document attribute. |
| SPACE (SP) | 20 | advance the active character position one character position along the current line. |

and the ASCII graphic characters (hex 21 to hex 7E).  See American National Standard ANS X3.4 (ASCII) which is also specified in ISO/IEC 646-1991 and ISO/IEC 6429.  If other control characters are encountered in the range hex 00 to 1F, the results are implementation-dependent.

The meaning of characters in the range hex 80 to FF is implementation defined and may be specified by the default-character-set document attribute.  The default-character-set document attribute may also be used to specify a different default character set for characters in the range hex 00 to hex 7F.  The default-font document attribute may be used to specify the font that is to be used with the simple-text document format.

The width of margins (white space not used to image simple-text characters) on all four edges of a logical page is implementation-dependent.  The left-margin, right-margin, top-margin and bottom-margin attributes, if implemented, shall be used to specify explicit margins.

The number of characters per line and the number of lines per page, is implementation-dependent.  A minimum of 80 characters per line and a minimum of 60 lines per page is recommended.  The width and length attributes, if implemented, shall be used to specify explicit values for the width and length of the area to be used to image simple-text.

If a character would be imaged beyond the last character position on a line, i.e., in the right margin area, the server shall advance the active character positions to an implementation-defined character position on the next line, so that no data is lost.  Similarly, if a line would be imaged beyond the last line on a logical page, i.e., in the bottom margin area, the server shall advance the active character position to an implementation-defined line on the next logical page.

Whether the server interprets lines as delimited by:

depends on the value of the new-line-option attribute (see F.2.1).

1.  CARRIAGE RETURN (CR) LINE FEED (LF) pairs

2.  LF only

3.  CR only

4.  either CR or LF

To give the user and print client more control over the placement of the text on the page, any of the following 12 additional attributes may be specified. These attributes are defined in the M.I.T. Athena Palladium Printing System specification, entitled: "Additional Simple Text Formatting Attributes for use with ISO/IEC 10175, Document Printing Application standard".  Requests for this specification may be sent to:

MIT Project Athena	
	Massachusetts Institute of Technology	
	1 Amherst St.	
	Cambridge, MA 02139

NOTE –  While these attributes were designed to be used with the ISO/IEC 6429 simple-text document format, the specification of other document formats may make use of these attributes as well.

The M.I.T. specification includes the following per-document attributes and MIT Palladium assigned object identifiers:

| Attribute | Description |
| --- | --- |
| size-unit-type | identifies the unit of measure for the following 8 attributes. Values are: character (default), millimeter, computer-decipoint, decididot, mil, basic-measuring-unit, micrometer, pixel, decipoint, TWIPS (one-twentieth of a point). If this attribute is omitted or not implemented, the values are in units of characters. |
| width | specifies the width of the text area, i.e., the maximum line width before wrapping (in units of characters, millimeters, e tc., depending on the size-unit-type attribute, if any). |
| length | specifies the length of the text area, i.e., the maximum page length before lines are automatically placed on the next page. |
| left-margin | specifies the distance between the left edge of the logical page and the left edge of the text area when held in the intended reading orientation. |
| right-margin | specifies the distance between the right edge of the logical page and the right edge of the text area when held in the intended reading orientation. |
| top-margin | specifies the distance between the top edge of the logical page and the top edge of the text area when held in the intended reading orientation. |
| bottom-margin | specifies the distance between the bottom edge of the logical page and the bottom edge of the text area when held in the intended reading orientation. |
| repeated-tab-stops | specifies the SINGLE VALUE Integer number of units between tab stops. |
| explicit-tab-stops | specifies the MULTI VALUE Integer placement of each tab stop. |
| header-text | specifies the text for the first line of each page. |
| footer-text | specifies the text for the last line of each page. |
| number-pages | indicates whether to number pages or not. |

In addition to any of the above attributes, the print server may use additional attributes, such as default-font, default-medium, content-orientation, assured-reproduction-area, page-size, and presentation-directions-contained to determine the size and/or spacing of the font used and the placement and/or orientation of the text on the page, depending on implementation.

### F.4.14	SPDL (Standard Page Description Language)

This document format describes the interchange formats for the Standard Page Description Language (ISO/IEC 10180), as defined in that standard.  The applicable format is identified via the required document-format-variant attribute, which must specify either binary or cleartext, where:

| Document-format-variant | Description |
| --- | --- |
| “binary” | This variant identifies the ASN.1 binary interchange format for SPDL structure and the binary format for SPDL content. |
| “cleartext” | This variant identifies the SGML cleartext interchange format for SPDL structure and the cleartext format for SPDL content. |

Version may be specified by the document-format-version attribute.

## F.5	Document-format identifiers

The Object Identifiers used to identify document-formats are assigned according to the transformation specified in F.1.1 from the  Printer MIB prtInterpreterLangFamily object/attribute enum values to produce the final arc of the ISO/IEC 10175 Object Identifier.

The list of document-format OIDs corresponds to the Printer MIB prtInterpreterLangFamily object/attribute enum values that were registered at the time of publication of this International Standard.  Implementors registering additional values with the IANA may automatically use the corresponding ISO/IEC 10175 Object Identifier using the algorithm specified in F.1.

NOTE: The simple names corresponding to these OIDs were derived from the IANA assigned enum symbol names.  However, the case of some of the letters has been changed in some instances to conform to documentation and naming conventions used in this International Standard.  Simple names that are proper names or acronyms of proper names are rendered in upper-case when constructiong the simple-names used in ISO/IEC 10175.  For simple names that would normally contain spaces (e.g., simple text), a hyphen is inserted to follow ISO DPA conventions for simple OID names (e.g., simple-text).

See the ASN.1 in F.7 for the OID assignments.

## F.6	Simple-text Object Identifiers

The Object Identifiers used to identify attributes for use with the simple-text document format (see F.4.13) were assigned by MIT. These attributes may be used with other document formats as well.  See F.7 for the OID assignments.

## F.7	ASN.1 for document-format specific attributes of ANNEX F

This sub-clause formally defines the document-format specific attribute-types applicable to DPA.  It employs ASN.1 with the ATTRIBUTE and ATTRIBUTE-SYNTAX macros.

The order of the ASN.1 production in this sub-clause follows the order of the attribute definitions in ANNEX F.

The ASN.1 in this sub-clause has been compiled with Annexes A, B, C, and D by a recognized ASN.1 compiler.  In case of discrepancies between this sub-clause and the rest of ANNEX F, this sub-clause shall take precedence.

--  File:  DPA_0F.ASN,  Version 0.5, 15-May-1995

--  Version 0.5, 15-May-1995: Add simple-text attribute OID assignments

--  This sub-clause formally defines the document-format specific

--  attribute-types applicable to DPA.  It employs ASN.1 with the

--  ATTRIBUTE and ATTRIBUTE-SYNTAX macros.

--  The order of the ASN.1 production in this sub-clause follows the

--  order of the attribute definitions in ANNEX F.  The ASN.1 in this

--  sub-clause has been compiled with Annexes A, B, C, and D by

--  a recognized ASN.1 compiler.  In case of discrepancies between

--  this sub-clause and the rest of ANNEX F, this sub-clause shall

--  take precedence.

DPAStandardDocumentFormatDefinitions { iso standard dpa(10175) part-1(1)

modules(0) document-format-definitions(5)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- PROLOGUE --

-- EXPORTS  everything --

IMPORTS

-- some attribute-type OIDs from ANNEX A:

id-att-carriage-control-type,

id-att-carriage-control-types-supported,

id-att-new-line-option,

id-att-new-line-options-supported,

id-att-shift-out-shift-in,

id-att-shift-out-shift-in-supported,

id-att-table-reference-characters,

id-att-table-reference-characters-supported,

id-vc-document-format

FROM DPAStandardObjectIdentifiers

{ iso standard dpa(10175) part-1(1) modules(0) object-identifiers(0) }

-- Some macros from ANNEX C:

ATTRIBUTE, ATTRIBUTE-SYNTAX

FROM DPAStandardAttributes

{ iso standard dpa(10175) part-1(1) modules(0) attributes(2) }

;

-- end of IMPORTS --

-- Attribute-type OID assignments from MIT Palladium

ID ::=  OBJECT IDENTIFIER

id-pd          ID ::= { iso(1) member-body(2) usa(840)

mit(113554) project-athena(1) palladium(1) }

id-pd-oc     ID ::= { id-pd  3 }

id-pd-vc     ID ::= { id-pd  6 }

id-pd-job    ID ::= { id-pd-oc  1 }

id-att-size-unit-type                       ID ::= { id-pd-job    1 }

id-vc-size-unit                            ID ::= { id-pd-vc   199 }

id-val-size-unit-character                ID ::= { id-vc-size-unit  1 }

id-val-size-unit-millimeter               ID ::= { id-vc-size-unit  2 }

id-val-size-unit-computer-decipoint       ID ::= { id-vc-size-unit  3 }

id-val-size-unit-decididot                ID ::= { id-vc-size-unit  4 }

id-val-size-unit-mil                      ID ::= { id-vc-size-unit  5 }

id-val-size-unit-basic-measuring-unit     ID ::= { id-vc-size-unit  6 }

id-val-size-unit-micrometer               ID ::= { id-vc-size-unit  7 }

id-val-size-unit-pixel                    ID ::= { id-vc-size-unit  8 }

id-val-size-unit-decipoint                ID ::= { id-vc-size-unit  9 }

id-val-size-unit-TWIPS                    ID ::= { id-vc-size-unit 10 }

id-att-width                                ID ::= { id-pd-job  2 }

id-att-length                               ID ::= { id-pd-job  3 }

id-att-left-margin                          ID ::= { id-pd-job 4 }

id-att-right-margin                         ID ::= { id-pd-job 5 }

id-att-top-margin                           ID ::= { id-pd-job 6 }

id-att-bottom-margin                        ID ::= { id-pd-job 7 }

id-att-repeated-tab-stops                   ID ::= { id-pd-job 8 }

id-att-explicit-tab-stops                   ID ::= { id-pd-job 9 }

id-att-header-text                          ID ::= { id-pd-job 10 }

id-att-footer-text                          ID ::= { id-pd-job 11 }

id-att-number-pages                         ID ::= { id-pd-job 12 }

--  document-format attribute-value assignments --

id-val-document-format-other        ID ::= { id-vc-document-format  1 }

id-val-document-format-unknown      ID ::= { id-vc-document-format  2 }

--  value 2 is reserved by Internet Printer MIB in case

--  the “unknown” value is needed.

--  The “unknown” OID may be used in ISO DPA.

id-val-document-format-PCL          ID ::= { id-vc-document-format  3 }

id-val-document-format-HPGL         ID ::= { id-vc-document-format  4 }

id-val-document-format-PJL          ID ::= { id-vc-document-format  5 }

id-val-document-format-PostScript   ID ::= { id-vc-document-format  6 }

id-val-document-format-IPDS         ID ::= { id-vc-document-format  7 }

id-val-document-format-PPDS         ID ::= { id-vc-document-format  8 }

id-val-document-format-EscapeP      ID ::= { id-vc-document-format  9 }

id-val-document-format-Epson        ID ::= { id-vc-document-format 10 }

id-val-document-format-DDIF         ID ::= { id-vc-document-format 11 }

id-val-document-format-Interpress   ID ::= { id-vc-document-format 12 }

id-val-document-format-ISO6429      ID ::= { id-vc-document-format 13 }

id-val-document-format-line-data    ID ::= { id-vc-document-format 14 }

id-val-document-format-MODCA        ID ::= { id-vc-document-format 15 }

id-val-document-format-ReGIS        ID ::= { id-vc-document-format 16 }

id-val-document-format-SCS          ID ::= { id-vc-document-format 17 }

id-val-document-format-SPDL         ID ::= { id-vc-document-format 18 }

id-val-document-format-Tek4014      ID ::= { id-vc-document-format 19 }

id-val-document-format-PDS          ID ::= { id-vc-document-format 20 }

id-val-document-format-IGP          ID ::= { id-vc-document-format 21 }

id-val-document-format-CodeV        ID ::= { id-vc-document-format 22 }

id-val-document-format-DSC-DSE      ID ::= { id-vc-document-format 23 }

id-val-document-format-WPS          ID ::= { id-vc-document-format 24 }

id-val-document-format-LN03         ID ::= { id-vc-document-format 25 }

id-val-document-format-CCITT        ID ::= { id-vc-document-format 26 }

id-val-document-format-QUIC         ID ::= { id-vc-document-format 27 }

id-val-document-format-CPAP         ID ::= { id-vc-document-format 28 }

id-val-document-format-DEC-PPL      ID ::= { id-vc-document-format 29 }

id-val-document-format-simple-text  ID ::= { id-vc-document-format 30 }

id-val-document-format-NPAP         ID ::= { id-vc-document-format 31 }

id-val-document-format-DOC          ID ::= { id-vc-document-format 32 }

id-val-document-format-imPRESS      ID ::= { id-vc-document-format 33 }

id-val-document-format-Pinwriter    ID ::= { id-vc-document-format 34 }

id-val-document-format-NPDL         ID ::= { id-vc-document-format 35 }

id-val-document-format-201PL        ID ::= { id-vc-document-format 36 }

id-val-document-format-automatic    ID ::= { id-vc-document-format 37 }

id-val-document-format-PAGES        ID ::= { id-vc-document-format 38 }

id-val-document-format-LIPS         ID ::= { id-vc-document-format 39 }

id-val-document-format-TIFF         ID ::= { id-vc-document-format 40 }

id-val-document-format-Diagnostic   ID ::= { id-vc-document-format 41 }

id-val-document-format-PSPrinter    ID ::= { id-vc-document-format 42 }

id-val-document-format-CaPSL        ID ::= { id-vc-document-format 43 }

id-val-document-format-EXCL         ID ::= { id-vc-document-format 44 }

id-val-document-format-LCDS         ID ::= { id-vc-document-format 45 }

id-val-document-format-XES          ID ::= { id-vc-document-format 46 }

id-val-document-format-PCLXL        ID ::= { id-vc-document-format 47 }

id-val-document-format-ART          ID ::= { id-vc-document-format 48 }

id-val-document-format-TIPSI        ID ::= { id-vc-document-format 49 }

--***************************************************************************--

--  attribute definitions --

new-line-option ATTRIBUTE

WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax

SINGLE VALUE

::= id-att-new-line-option

carriage-control-type ATTRIBUTE

WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax

SINGLE VALUE

::= id-att-carriage-control-type

table-reference-characters ATTRIBUTE

WITH ATTRIBUTE-SYNTAX booleanSyntax

SINGLE VALUE

::= id-att-table-reference-characters

shift-out-shift-in ATTRIBUTE

WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax

SINGLE VALUE

::= id-att-shift-out-shift-in

new-line-options-supported ATTRIBUTE

WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax

MULTI VALUE

::= id-att-new-line-options-supported

carriage-control-types-supported ATTRIBUTE

WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax

MULTI VALUE

::= id-att-carriage-control-types-supported

table-reference-characters-supported ATTRIBUTE

WITH ATTRIBUTE-SYNTAX booleanSyntax

SINGLE VALUE

::= id-att-table-reference-characters-supported

shift-out-shift-in-supported ATTRIBUTE

WITH ATTRIBUTE-SYNTAX objectIdentifierSyntax

MULTI VALUE

::= id-att-shift-out-shift-in-supported

END -- of DPAStandardDocumentFormatDefinitions --

PostScript is a trademark of Adobe Systems, Incorporated, which may be registered in some jurisdictions.

The M.I.T. Project Athena Palladium Printing System is a public domain implementation of the ISO/IEC (DIS) 10175 Draft International Standard and an open systems distributed printing system.