FINISHER MIB                                      October 22,1997

Expires April 22,1998

Finisher-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, experimental,

Integer32                                     FROM SNMPv2-SMI

TEXTUAL-CONVENTION                             FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP             FROM SNMPv2-CONF;

hrDeviceIndex                         FROM HOST-RESOURCES-MIB

PrtSubUnitStatusTC, PrtMarkerSuppliesSupplyUnitTC,

PrtCapacityUnitTC, PrtOutputStackingOrderTC,

PrtMarkerColorantRoleTC, prtOutputIndex,

PrtMarkerSuppliesClassTC, PresentOnOff,

PrtOutputPageDeliveryOrientationTC           FROM Printer-MIB

finisherMIB MODULE-IDENTITY

LAST-UPDATED "9710150000Z"

ORGANIZATION "IETF Printer MIB Working Group"

CONTACT-INFO

"Ron Bergman

Dataproducts Corp.

1757 Tapo Canyon Road

Simi Valley, CA 91063-3394

rbergma@dpc.com

Send comments to the printmib WG using the Finisher MIB

Project (FIN) Mailing List:  fin@pwg.org

For further information, access the PWG web page under

'FIN':   http://www.pwg.org/"

DESCRIPTION

"The MIB module for management of printer finisher units.

The Finisher MIB is an extension of the Printer MIB."

::= { mib-2 43 }

-- Textual conventions for this MIB module

FinStitchingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined stitching type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

staple(3),

stapleTopLeft(4),

stapleBottomLeft(5),

stapleTopRight(6),

Ron Bergman, Dataproducts Corp.                         [page 1]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

stapleBottomRight(7),

saddleStitch(8),

edgeStitch(9),

stitch(10)

}

FinBindingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined binding type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

bind(3),

tape(4),

plastic(5),

velo(6),

perfect(7),

spiral(8)

}

FinSlittingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined slitting type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

slit(3),

slitAndSeparate(4),

slitAndMerge(5)

}

FinOutputTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined output type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

removableBin(3),

unremovableBin(4),

continuousRollDevice(5),

mailbox(6),

continuousFanFold(7),

conveyer(8),

smartCart(9)

}

Ron Bergman, Dataproducts Corp.                         [page 2]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

FinSupplyTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined finishing supply type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

-- toner(3),

-- wasteToner(4),

-- ink(5),

-- inkCartridge(6),

-- inkRibbon(7),

-- wasteInk(8),

-- opc(9), -- photo conductor

-- developer(10),

-- fuserOil(11),

-- solidWax(12),

-- ribbonWax(13),

-- wasteWax(14),

-- fuser(15),

-- coronaWire(16),

-- fuserOilWick(17),

-- cleanerUnit(18),

-- fuserCleaningPad(19),

-- transferUnit(20),

-- tonerCartridge(21),

-- fuserOiler(22),

water(23),

wasteWater(24),

glueWaterAdditive(25),

wastePaper(26),

bindingTape(27),

bandingTape(28),

stitchingWire(29),

shrinkWrap(30),

paperWrap(31)

}

-- Finisher Device Group

--

-- A finisher component may have multiple document finisher

-- devices associated with it.  Each finishing device may support

-- one or more finishing features.

-- For example, one finisher device may cut and sequence sheets

-- into physical pages.  Another finisher device may fold the

-- pages for insertion into envelopes.  Each entry in the table

-- corresponds to a finisher device supported by this conponent.

Ron Bergman, Dataproducts Corp.                         [page 3]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

finDeviceTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinDeviceEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A device that performs one or more finishing operations on

printed media other than marking.  Examples of operations

performed by finisher devices are stapling, punching,

folding, die cutting, trimming, and banding."

::= { finisherMIB 18 }

finDeviceEntry OBJECT-TYPE

SYNTAX     FinDeviceEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Entries may exist in the table for each device index with

a device type of 'printer' and for each printer output

path with one or more finisher devices."

INDEX  { hrDeviceIndex, prtOutputIndex, finDeviceIndex }

::= { finDeviceTable 1 }

FinDeviceEntry ::= SEQUENCE {

finDeviceIndex                Integer32,

finDeviceName                 DisplayString,

finDeviceVendorName           DisplayString,

finDeviceModel                DisplayString,

finDeviceVersion              OCTET STRING,

finDeviceSerialNumber         DisplayString,

finDEviceDescription          OCTET STRING,

finDeviceStatus               PrtSubUnitStatusTC

}

finDeviceIndex OBJECT-TYPE

SYNTAX      Integer32(0..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique value used by this component to identify an

associated document finisher device. Although these values

may change due to a major reconfiguration of the component

(e.g. the addition of new finishing devices), values are

normally expected to remain stable across successive power

cycles."

::= { finDeviceEntry 1 }

finDeviceName OBJECT-TYPE

SYNTAX      DisplayString(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The name assigned to this document finisher device."

Ron Bergman, Dataproducts Corp.                         [page 4]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

::= { finDeviceEntry 2 }

finDeviceVendorName OBJECT-TYPE

SYNTAX      DisplayString(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The name of the vendor of this device."

::= { finDeviceEntry 3 }

finDeviceModel OBJECT-TYPE

SYNTAX      DisplayString(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The model name of this device."

::= { finDeviceEntry 4 }

finDeviceVersion OBJECT-TYPE

SYNTAX      OCTET STRING(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The version string for this device."

::= { finDeviceEntry 5 }

finDeviceSerialNumber OBJECT-TYPE

SYNTAX      DisplayString(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The serial number assigned to this device."

::= { finDeviceEntry 6 }

finDeviceDescription OBJECT-TYPE

SYNTAX      OCTET STRING(0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A free-form text description of this document finishing

device."

::= { finDeviceEntry 7 }

finDeviceStatus OBJECT-TYPE

SYNTAX      PrtSubUnitStatusTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the state of the device."

::= { finDeviceEntry 8 }

Ron Bergman, Dataproducts Corp.                         [page 5]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

-- Finisher Features Group

--

-- A finisher device may support one or more finishing functions

-- for for each of the input sources associated with it.

finFeaturesTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinFeaturesEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Finishing features associated with the processing of input

media including information regarding functions supported,

possible configuration options and status for each input

source."

::= { finisherMIB 19 }

finFeaturesEntry OBJECT-TYPE

SYNTAX     FinFeaturesEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"There is an entry in the finishing features table for each

finisher device. Each entry, in turn, specifies a list of

finishing features and identifies whether or not they are

present, and if so, their current status and

characteristics."

INDEX  { hrDeviceIndex, prtOutputIndex, finDeviceIndex,

finFeatureIndex }

::= { finFeaturesTable 1 }

FinFeaturesEntry ::= SEQUENCE {

finStitching                  PresentOnOff,

finStitchingType              FinStitchingTypeTC,

finMaxPagesStitched           Integer32,

finFolding                    PresentOnOff,

finFoldingType                Integer32,

finMaxPagesFolded             Integer32,

finBinding                    PresentOnOff,

finBindingType                finBindingTypeTC,

finMaxPagesBound              Integer32,

finBindingColor               DisplayString,

finTrimming                   PresentOnOff,

finTrimmingType               Integer32,

finMaxPagesTrimmed            Integer32,

finDieCutting                 PresentOnOff,

finDieCuttingType             Integer32,

finMaxPagesDieCut             Integer32,

finPunching                   PresentOnOff,

finPunchType                  Integer32,

finMaxPagesPunched            Integer32,

finPerforating                PresentOnOff,

finPerforationType            Integer32,

Ron Bergman, Dataproducts Corp.                         [page 6]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

finSlitting                   PresentOnOff,

finSlittingType               FinSlittingTypeTC,

finSeparationCut              PresentOnOff,

finSeparationCutType          Integer32,

finImprinting                 PresentOnOff,

finImprintingType             Integer32,

finWrapping                   PresentOnOff,

finWrappingType               Integer32,

finMaxPagesWrapped            Integer32,

finBanding                    PresentOnOff,

finBandingType                Integer32,

finMaxPagesBanded             Integer32

}

finStitching OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if stitching is supported as a finishing

feature option for this input source and whether the

feature is enabled."

::= { finFeatureEntry 1 }

finStitchingType OBJECT-TYPE

SYNTAX      FinStitchingTypeTC

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of stitching that is presently enabled for page sets

in the input media source associated with this entry.

Staple indicates the staple type is the finisher defined

staple default. Stitch indicates the stitch type is the

finisher defined stitch default."

::= { finFeatureEntry 2 }

finMaxPagesStitched OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of pages that can be in a page set to be

stitched by this finishing device. The value of this

object may be used for device setups and/or bypassing

(diverting) stitching operations for sets that are too

large to handle."

::= { finFeatureEntry 3 }

finFolding OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

Ron Bergman, Dataproducts Corp.                         [page 7]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

DESCRIPTION

"Indicates if folding is supported as a finishing feature

option for this input source and whether the feature is

enabled."

::= { finFeatureEntry 4 }

finFoldingType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

zFold(3),

halfFold(4),

letterFold(5)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of folding that is presently enabled for page sets in

the input media source associated with this entry."

::= { finFeatureEntry 5 }

finMaxPagesFolded OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of pages that can be in a page set to be

folded by this finishing device. The value of this object

may be used for device setups and/or bypassing (diverting)

folding for sets that are too large to handle."

::= { finFeatureEntry 6 }

finBinding OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if binding is supported as a finishing feature

option for this input source and whether the feature is

enabled."

::= { finFeatureEntry 7 }

finBindingType OBJECT-TYPE

SYNTAX      FinBindingTypeTC

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of binding that is presently enabled for page sets in

the input media source associated with this entry.  Bind

indicates that the bind type that is the finisher defined

binding default."

::= { finFeatureEntry 8 }

Ron Bergman, Dataproducts Corp.                         [page 8]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

finMaxPagesBound OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of pages that can be in a page set to be

bound by this finishing device. The value of this object

may be used for device setups and/or bypassing (diverting)

binding for sets that are too large to handle."

::= { finFeatureEntry 9 }

finBindingColor OBJECT-TYPE

SYNTAX      DisplayString(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The name of the color of the binding associated with this

finishing feature.  Standard values are standardized

strings from ISO-10175 [DPA] and ISO-10180 [SPDL].  These

are white, red, green, blue, pink, yellow, black, buff,

goldenrod and other.  Additional strings with names that

do not conflict with standard names may also be

specified."

::= { finFeatureEntry 10 }

finTrimming OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if trimming is supported as a finishing feature

option for this input source and whether the feature is

enabled."

::= { finFeatureEntry 11 }

finTrimmingType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

trim(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of trimming that is presently enabled for page sets

in the input media source associated with this entry.

Trim indicates the trim type that is the finisher defined

default."

::= { finFeatureEntry 12 }

finMaxPagesTrimmed OBJECT-TYPE

Ron Bergman, Dataproducts Corp.                         [page 9]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of pages that can be in a page set to be

trimmed by this finishing device. The value of this object

may be used for device setups and/or bypassing (diverting)

trimming for sets that are too large to handle."

::= { finFeatureEntry 13 }

finDieCutting OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if die cutting is supported as a finishing

feature option for this input source and whether the

feature is enabled."

::= { finFeatureEntry 14 }

finDieCuttingType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

dieCut(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of die cutting that is presently enabled for page

sets in the input media source associated with this entry.

Die cut indicates the die cut type that is the finisher

defined default."

::= { finFeatureEntry 15 }

finMaxPagesDieCut OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of pages that can be in a page set to be

die cut by this finishing device. The value of this object

may be used for device setups and/or bypassing (diverting)

die cutting for sets that are too large to handle."

::= { finFeatureEntry 16 }

finPunching OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if Punching is supported as a finishing feature

Ron Bergman, Dataproducts Corp.                         [page 10]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

option for this input source and whether the feature is

enabled."

::= { finFeatureEntry 17 }

finPunchType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

punch(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of punching that is presently enabled for page sets

in the input media source associated with this entry.

Punch indicates the punch type that is the finisher

defined default."

::= { finFeatureEntry 18 }

finMaxPagesPunched OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of pages that can be in a page set to be

punched by this finishing device. The value of this object

may be used for device setups and/or bypassing (diverting)

punching for sets that are too large to handle."

::= { finFeatureEntry 19 }

finPerforating OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if perforating is supported as a finishing

feature option for this input source and whether the

feature is enabled."

::= { finFeatureEntry 20 }

finPerforationType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

perforate(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of perforation that is presently enabled for page

sets in the input media source associated with this entry.

The value perforate indicates the perforation type that is

Ron Bergman, Dataproducts Corp.                         [page 11]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

the finisher defined default."

::= { finFeatureEntry 21 }

finSlitting OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if slitting is supported as a finishing feature

option for this input source and whether the feature is

enabled."

::= { finFeatureEntry 22 }

finSlittingType OBJECT-TYPE

SYNTAX      FinSlittingTypeTC

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of slitting that is presently enabled for page sets

in the input media source associated with this entry.

The value Slit indicates the slitting type that is the

finisher defined slitting default."

::= { finFeatureEntry 23 }

finSeparationCut OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if separation cuts are supported as a finishing

feature option for this input source and whether the

feature is enabled."

::= { finFeatureEntry 24 }

finSeparationCutType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

separationCut(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of separation cut that is presently enabled for page

sets in the input media source associated with this entry.

The value separationCut indicates the separation cut type

that is the finisher defined default."

::= { finFeatureEntry 25 }

finImprinting OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

Ron Bergman, Dataproducts Corp.                         [page 12]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

STATUS      current

DESCRIPTION

"Indicates if imprinting is supported as a finishing

feature option for this input source and whether the

feature is enabled."

::= { finFeatureEntry 26 }

finImprintingType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

imprinting(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of imprinting that is presently enabled for page

sets in the input media source associated with this entry.

The value imprinting indicates the imprinting type that is

the finisher defined default."

::= { finFeatureEntry 27 }

finWrapping OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if wrapping is supported as a finishing feature

option for this input source and whether the feature is

enabled."

::= { finFeatureEntry 28 }

finWrappingType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

wrap(3),

shrinkWrap(4),

paperWrap(5)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of wrapping that is presently enabled for page

sets in the input media source associated with this entry.

The value wrap indicates the wrapping type that is the

finisher defined default."

::= { finFeatureEntry 29 }

finMaxPagesWrapped OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

Ron Bergman, Dataproducts Corp.                         [page 13]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

STATUS      current

DESCRIPTION

"Maximum number of pages that can be in a page set to be

wrapped by this finishing device."

::= { finFeatureEntry 30 }

finBanding OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if banding is supported as a finishing feature

option for this input source and whether the feature is

enabled."

::= { finFeatureEntry 31 }

finBandingType OBJECT-TYPE

SYNTAX      Integer32 {

other(1),

unknown(2),

band(3)

}

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Type of banding that is presently enabled for page

sets in the input media source associated with this entry.

The value band indicates the banding type that is the

finisher defined default."

::= { finFeatureEntry 32 }

finMaxPagesBanded OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Maximum number of pages that can be in a page set to be

banded by this finishing device."

::= { finFeatureEntry 33 }

-- Finisher Output Group

--

-- A finisher may have multiple output devices associated with it

-- For example, a finisher can have several output trays.

finOutputTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinOutoutEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The output devices associated with a finisher are

Ron Bergman, Dataproducts Corp.                         [page 14]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

identified in this table.  Each entry in the table is an

output device definition with an ordered set of attributes

describing the output characteristics and indicating

current status or capacity. Only those attributes

applicable to the output device need be specified."

::= { finisherMIB 20 }

finOutputEntry OBJECT-TYPE

SYNTAX     FinOutputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A list of output devices capable of receiving media

delivered from the printing process including information

regarding their identity, possible and current

configuration, and status."

INDEX  { hrDeviceIndex, prtOutputIndex, finDeviceIndex,

finOutputIndex }

::= { finOutputTable 1 }

FinOutputEntry ::= SEQUENCE {

finOutputIndex                Integer32,

finOutoutType                 FinOutputTypeTC,

finOutputCapacityUnit         PrtCapacityUnitTC,

finOutputMaxCapacity          Integer32,

finOutputRemainingCapacity    Integer32,

finOutputStatus               PrtSubUnitStatusTC,

finOutputName                 OCTET STRING,

finOutputVendorName           OCTET STRING,

finOutputModel                OCTET STRING,

finOutputVersion              OCTET STRING,

finOutputSerialNumber         OCTET STRING,

finOutputDescription          OCTET STRING,

finOutputSecurity             PresentOnOff,

finOutputDimUnit              MediaUnit,

finOutputMaxDimFeedDir        Integer32,

finOutputMaxDimXFeedDir       Integer32,

finOutputMinDimFeedDir        Integer32,

finOutputMinDimXFeedDir       Integer32,

finOutputStackingOrder        PrtOutputStackingOrderTC,

finOutputPageDeliveryOrient

PrtOutputPageDeliveryOrientationTC,

finOutputBursting             PresentOnOff,

finOutputDecollating          PresentOnOff,

finOutputPageCollated         PresentOnOff,

finOutputOffsetStacking       PresentOnOff

}

finOutputIndex OBJECT-TYPE

SYNTAX      Integer32(0..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

Ron Bergman, Dataproducts Corp.                         [page 15]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

DESCRIPTION

"A unique value used by this component to identify an

associated output device. Although these values may change

due to a major reconfiguration of the component (e.g. the

addition of new output devices to the finisher), values

are normally expected to remain stable across successive

power cycles."

::= { finOutputEntry 1 }

finOutputType OBJECT-TYPE

SYNTAX      FinOutputTypeTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The type of technology employed by this output unit."

::= { finOutputEntry 2 }

finOutputCapacityUnit OBJECT-TYPE

SYNTAX      PrtCapacityUnitTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unit of measure for specifying capacity."

::= { finOutputEntry 3 }

finOutputMaxCapacity OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The maximum capacity of this device in Output Capacity

Units.  There is no convention associated with the media

itself so this value is claimed capacity.  If the device

can reliably sense this value, the value is sensed by the

finisher and is read-only: otherwise the value may be

written by a management or control console application.

The value (-1) means other and specifically indicates

that the device places no restrictions on this parameter.

The value (-2) means unknown."

::= { finOutputEntry 4 }

finOutputRemainingCapacity OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The remaining capacity of this device in Output Capacity

Units.  If the output device can reliably sense this

value, the value is sensed by the finisher and is

read-only: otherwise the value may be written by a

management or control console application.  The value (-1)

means other and specifically indicates that the device

Ron Bergman, Dataproducts Corp.                         [page 16]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

places no restrictions on this parameter.  The value (-2)

means unknown."

::= { finOutputEntry 5 }

--               Capacity Attribute Relationships

--

--

--                 OUTPUT MEASUREMENT

--

--       _______ |                     | _______

--        ^      |                     |      ^

--        |      |                     |      |

--        |      |                     | RemainingCapacity

--   MaxCapacity |                     |      |

--        |      |                     |      |       ^

--        |      |_ _ _ _ _ _ _ _ _ _ _| _____v______ | direction

--        |      |                     |              |

--        |      |  Sheets in output   |

--        v      |                     |

--       ------- +---------------------+

--

finOutputStatus OBJECT-TYPE

SYNTAX      PrtSubUnitStatusTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the current status of this output device."

::= { finOutputEntry 6 }

-- The Extended Finisher Output Group

--

-- This group is optional.  However, to claim conformance to this

-- group, it is necessary to implement every object in the group.

finOutputName OBJECT-TYPE

SYNTAX      OCTET STRING(0..63)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The name assigned to this output device."

::= { finOutputEntry 7 }

finOutputVendorName OBJECT-TYPE

SYNTAX      OCTET STRING(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The vendor name of this output device."

Ron Bergman, Dataproducts Corp.                         [page 17]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

::= { finOutputEntry 8 }

finOutputModel OBJECT-TYPE

SYNTAX      OCTET STRING(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The model name of this output device."

::= { finOutputEntry 9 }

finOutputVersion OBJECT-TYPE

SYNTAX      OCTET STRING(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The version string for this output device."

::= { finOutputEntry 10 }

finOutputSerialNumber OBJECT-TYPE

SYNTAX      OCTET STRING(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The serial number assigned to this output device."

::= { finOutputEntry 11 }

finOutputDescription OBJECT-TYPE

SYNTAX      OCTET STRING(0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A free form text description of this output device in the

localization specified by prtGeneralCurrentLocalization."

::= { finOutputEntry 12 }

finOutputSecurity OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if this device has some security associated with

it."

::= { finOutputEntry 13 }

-- The Finisher Output Dimensions Group

--

-- This group is optional.  However, to claim conformance to this

-- group, it is necessary to implement every object in the group.

finOutputDimUnit OBJECT-TYPE

SYNTAX      Integer32 {

Ron Bergman, Dataproducts Corp.                         [page 18]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

other(1),

unknown(2),

tenThousandthsOfInches(3),

micrometers(4),

sheets(8)

}

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unit of measure for specifying dimensional values for

this output device."

::= { finOutputEntry 14 }

finOutputMaxDimFeedDir OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object provides the value of the maximum dimension in

the feed direction of the media that is placed or will be

placed in this output device.  Feed dimension measurements

are taken parallel relative to the feed direction of the

device and measured in Output Dim Units. If the device can

reliably sense this value, the value is sensed by the

device and is read-only; otherwise the value may be

written (set) by a management application."

::= { finOutputEntry 15 }

finOutputMaxDimXFeedDir OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The maximum dimensions supported by this output device for

measurements taken ninety degrees relative to the feed

direction associated with this device in Output Dim Units.

If the device can reliably sense this value, the value is

sensed by the device and is read-only; otherwise the value

may be written (set) by a management application."

::= { finOutputEntry 16 }

finOutputMinDimFeedDir OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object provides the value of the minimum dimension in

the feed direction of the media that is placed or will be

placed in this output device.  Feed dimension measurements

are taken parallel relative to the feed direction of the

device and measured in Output Dim Units. If the device can

reliably sense this value, the value is sensed by the

Ron Bergman, Dataproducts Corp.                         [page 19]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

device and is read-only; otherwise the value may be

written (set) by a management application."

::= { finOutputEntry 17 }

finOutputMinDimXFeedDir OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The minimum dimensions supported by this output device for

measurements taken ninety degrees relative to the feed

direction associated with this device in Output Dim Units.

If the device can reliably sense this value, the value is

sensed by the device and is read-only; otherwise the value

may be written (set) by a management application."

::= { finOutputEntry 18 }

-- The Finisher Output Features Group

--

-- This group is optional.  However, to claim conformance to this

-- group, it is necessary to implement every object in the group.

--

-- This group is identical to the Output Features Group in the

-- Printer MIB and should be used when the outputs of the

-- finisher do not have identical characteristics and can be

-- described by the objects in this group.

finOutputStackingOrder OBJECT-TYPE

SYNTAX      PrtOutputStackingOrderTC

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The current state of the stacking order for this output

device. firstToLast means that as pages are output, the

front of the next page is placed  against the back of the

previous page. lastToFirst means as pages are output, the

back of the next page is placed against the front of the

previous page."

::= { finOutputEntry 19 }

finOutputPageDeliveryOrient OBJECT-TYPE

SYNTAX      PrtOutputPageDeliveryOrientationTC

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The reading surface that will be up when pages are

delivered to the output device."

::= { finOutputEntry 20 }

finOutputBursting OBJECT-TYPE

SYNTAX      PresentOnOff

Ron Bergman, Dataproducts Corp.                         [page 20]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates if the output unit supports

bursting, and if so, whether the feature is enabled.

Bursting is the process by which continuous media is

separated into individual sheets typically by bursting

along pre-formed perforations."

::= { finOutputEntry 21 }

finOutputDecollating OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates if the output device supports

decollating, and if so, whether the feature is enabled.

Decollating is the process by which the parts of a

multi-part form are separated and sorted into separate

stacks for each part."

::= { finOutputEntry 22 }

finOutputPageCollated OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates if the output device supports

page collation, and if so, whether the feature is

enabled."

::= { finOutputEntry 23 }

finOutputOffsetStacking OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object indicates if the output device supports

offset stackin, and if so, whether the feature is

enabled."

::= { finOutputEntry 24 }

-- Finisher Supply Group

--

-- A finisher may have one or more supplies associated with it.

-- For example a finisher may use both binding tape and stitching

-- wire supplies. A finisher may also have more than one source

-- for a given type of supply e.g. multiple supply sources of ink

-- for imprinters.

Ron Bergman, Dataproducts Corp.                         [page 21]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

finSupplyTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinSupplyEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Each unique source of supply is an entry in the finisher

supply table. Each supply entry has its own

characteristics associated with it such as colorant and

current supply level."

::= { finisherMIB 21 }

finSupplyEntry OBJECT-TYPE

SYNTAX     FinSupplyEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A list of finisher devices, with their associated

supplies and supplies characteristics."

INDEX  { hrDeviceIndex, prtOutputIndex, finDeviceIndex,

finSupplyIndex, finSupplyColorantIndex }

::= { finSupplyTable 1 }

FinSupplyEntry ::= SEQUENCE {

finSupplyIndex                Integer32,

finSupplyClass                PrtMarkerSuppliesClassTC,

finSupplyType                 FinSupplyTypeTC,

finSupplyDescription          OCTET STRING,

finSupplyUnit

PrtMarkerSuppliesSupplyUnitTC,

finSupplyMaxCapacity          Integer32,

finSupplyCurrentLevel         Integer32

}

finSupplyIndex OBJECT-TYPE

SYNTAX      Integer32(0..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique value used by a finisher to identify this supply

container/receptacle.  Although these values may change

due to a major reconfiguration of the finisher (e.g. the

addition of new supply sources to the finisher), values

are normally expected to remain stable across successive

power cycles."

::= { finSupplyEntry 1 }

finSupplyClass OBJECT-TYPE

SYNTAX      PrtMarkerSuppliesClassTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of the Index corresponding to the finisher

Ron Bergman, Dataproducts Corp.                         [page 22]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

device with which this supply entry is associated."

::= { finSupplyEntry 2 }

finSupplyType OBJECT-TYPE

SYNTAX      FinSupplyTypeTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The type of this supply."

::= { finSupplyEntry 3 }

finSupplyDescription OBJECT-TYPE

SYNTAX      OCTET STRING(0..255)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The description of this supply/receptacle in text useful

for operators and management applications."

::= { finSupplyEntry 4 }

finSupplyUnit OBJECT-TYPE

SYNTAX      PrtMarkerSuppliesSupplyUnitTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Unit of measure of this finisher supply container or

receptacle."

::= { finSupplyEntry 5 }

finSupplyMaxCapacity OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The maximum capacity of this supply container/receptacle

expressed in Supply Units.  If this supply container/

receptacle can reliably sense this value, the value is

sensed  and is read-only; otherwise the value may be

written by a control panel or management application.  The

value (-1) means other and places no restrictions on this

parameter.  The value (-2) means unknown."

::= { finSupplyEntry 6 }

finSupplyCurrentLevel OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The current level if this supply is a container; the

remaining space if this supply is a receptacle. If this

supply container/receptacle can reliably sense this value,

the value is sensed  and is read-only; otherwise the value

Ron Bergman, Dataproducts Corp.                         [page 23]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

may be written by a control panel or management

application.  The value (-1) means other and places no

restrictions on this parameter. The value (-2) means

unknown."

::= { finSupplyEntry 7 }

-- Finisher Supply Colorant Group

--

-- A Finisher may have one or more colorants associated with it.

finSupplyColorantTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinSupplyColorantEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of all the colorants available on the finisher."

::= { finisherMIB 22 }

finSupplyColorantEntry OBJECT-TYPE

SYNTAX     FinSupplyColorantEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Each colorant supported by the finisher is an entry in

this table."

INDEX  { hrDeviceIndex, prtOutputIndex, finDeviceIndex,

finSupplyColorantIndex }

::= { prtChannelTable 1 }

FinSupplyColorantEntry ::= SEQUENCE {

finSupplyColorantIndex        Integer32,

finSupplyColorantRole         PrtMarkerColorantRoleTC,

finSupplyColorantValue        OCTET STRING,

finSupplyColorantTonality     Integer32

}

finSupplyColorantIndex OBJECT-TYPE

SYNTAX      Integer32(0..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique value used by a finisher to identify this

colorant.  Although these values may change due to a major

reconfiguration of the finisher (e.g. the addition of new

colorants to the finisher), values are normally expected

to remain stable across successive power cycles."

::= { finSupplyColorantEntry 1 }

finSupplyColorantRole OBJECT-TYPE

SYNTAX      PrtMarkerColorantRoleTC

MAX-ACCESS  read-only

Ron Bergman, Dataproducts Corp.                         [page 24]

FINISHER MIB                                      October 22,1997

Expires April 22,1998

STATUS      current

DESCRIPTION

"The role played by this colorant."

::= { finSupplyColorantEntry 2 }

finSupplyColorantValue OBJECT-TYPE

SYNTAX      OCTET STRING(0..63)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The name of the color of this colorant using standardized

string names from ISO 10175 (DPA) and ISO 10180 (SPDL)

which are: other, unknown, white, red, green, blue, cyan,

magenta, yellow and black.  Implementors may add

additional string values.  The naming conventions in ISO

9070 are recommended in order to avoid potential name

clashes."

::= { finSupplyColorantEntry 3 }

finSupplyColorantTonality OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The distinct levels of tonality realizable by a finisher

device when using this colorant. This value does not

include the number of levels of tonal difference that an

interpreter can obtain by techniques such as half toning.

This value must be at least 2."

::= { finSupplyColorantEntry 4 }

END

Ron Bergman, Dataproducts Corp.                         [page 25]