INTERNET-DRAFT                                               Ron Bergman

Dataproducts Corp.

Harry Lewis

IBM Corp.

July 15, 1998

Printer Finishing MIB

<draft-ietf-printmib-finishing-xx.txt>

Expires January 15, 1999

Status of this Memo

This document is an Internet-Draft.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six

months and may be updated, replaced, or obsoleted by other

documents at any time.  It is inappropriate to use Internet-Drafts

as reference material or to cite them other than as "work in

progress".

To learn the current status of any Internet-Draft, please check the

"1id-abstracts.txt" listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Copyright Notice

Copyright (C) The Internet Society (1998).  All Rights Reserved.

Abstract

This document defines a printer industry standard SNMP MIB for the

management of printer finishing device subunits.  The finishing

device subunits applicable to this MIB are an integral part of the

Printer System.  This MIB does not apply to a Finisher Device that

is not connected to a Printer System.

The Finisher MIB is defined as an extension of the Printer MIB

[PrtMIB] and it is expected that the information defined in this

document will be incorporated into a future update of the Printer

MIB.

TABLE OF CONTENTS

1.  INTRODUCTION

1.1  Scope

1.2  Rational

2.  TERMINOLOGY

2.1  General Terminology

2.2  Process Specific Terminology

3.  FINISHER SUBUNITS INTEGRATED INTO THE PRINTER MODEL

4.  FINISHING SPECIFICATIONS

4.1  Multiple finDeviceTable Entries

4.2.1  FinPunchPatternTC

4.2.2  FinPunchHoleTypeTC, punchHoleSizeMaxDim, punchHoleSizeMinDim

5.  THE ATTRIBUTE MECHANISM

5.1  Conformance of Attribute Implementation

5.2  Useful, 'Unknown', and 'Other' Values for Objects and Attributes

5.3  Data Sub-types and Attribute Naming Conventions

5.4  Single-Value (Row) Versus Multi-Value (MULTI-ROW) Attributes

5.6  Index Value Attributes

6.  Enumerations

6.1  Registering Additional Enumerated Values

7.  PRINTER FINISHING MIB SPECIFICATION

8.  REFERENCES

9.  SECURITY CONSIDERATIONS

10.  FULL COPYRIGHT STATEMENT

11.  AUTHORS

# 1.  INTRODUCTION

This document describes an SNMP Management Information Base (MIB) to

provide for the management of in-line post-processing in a fashion

that is currently provided for printers, using the Printer MIB

[PrtMIB].  The Printer Finishing MIB includes the following features:

- Provides the status of the finishing device.

- Queries and controls the features and configuration of the

finishing device.

- Enables and disables the finishing processes.

- Allows unsolicited status from the finishing device.

## 1.1  Scope

This document provides a robust set of finishing devices, features,

and functions, based upon today's state of the art of in-line

finishing.  Since finishing typically accompanies higher speed

network printers and copiers, in contrast to simple desktop devices,

no attempt is made to limit the scope to "bare minimum".  On the

other hand, the Printer Finishing MIB does not duplicate the

production mail preparation, custom insertion, franking, and reprints

that are covered by the DMTF Large Mailing Operations standard [LMO].

Information supplied by the Printer Finishing MIB may be utilized by

printer and finisher management applications engaged in monitoring

status and managing configuration, and also used by print and

finishing submission applications which are engaged in:

- print-job-level finishing processes that are applied to a

complete print job,

- document-level finishing processes that are applied individually

to each document in the print job,

- document-level finishing processes that are applied to a selected

document in the print job.

Note that not all combinations of finishing processes are

permitted.  Compatible combinations of finishing processes are

implementation specific.  The MIB allows invalid combinations to be

identified.

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED",  "MAY", and "OPTIONAL" in this

document are to be interpreted as described in [RFC2119].

## 1.2  Rational

The Printer MIB [PrtMIB] is now successfully deployed in a large

segment of the network printer market.  SNMP and/or HTTP enabled

printers and software management applications are growing in numbers.

There is an increase in the availability of network printers and

copiers that include in-line finishing processes.  Thus a well

defined and ordered set of finishing objects is now necessary for

printer management.

The printer model defined in the Printer MIB includes finishing

processes and the MIB was designed to later incorporate finisher

objects or to be referenced by a future Finisher MIB.

# 2.  TERMINOLOGY

Where appropriate, the Printer Finishing MIB will conform to the

terminology, syntax, and semantics from the DMTF Large Mailing

Operations standard [LMO], the Internet Printing Protocol [IPP],

and/or the ISO Document Printing Application [DPA].

## 2.1  General Terminology

Finisher Input:  An input tray on the finisher and not otherwise

associated with the printer.  An example of a finisher input is a

tray that holds finishing "inserts".

Finisher Output:  The output of the finisher.  Because processing is

in-line, the finisher outputs are a direct extension of the set of

printer outputs.

^ Y

|

|<---- Reference Edge

|

|  |<--- Finishing Process Axis

|  |

--->|  |<--- Finishing Process Offset

|  |

Head              +=========================+ (X2,Y4)

Locations         #  |                      #

+-----#----+                    #

-----Y3--|-----#--O | <--+- Head         #

^        +-----#----+    |  Mechanisms   #

|              #  |      |               #

|              #  |      |               #

|              #  |      |               #

|              #  |      |               #

|        +-----#----+    |               #

| ---Y2--|-----#--O | <--+               #

| ^      +-----#----+    |               #

| |            #  |      |               #

| |            #  |      |               #

| |            #  |      |               #

| |            #  |      |               #

| |      +-----#----+    |               #

| | -Y1--|-----#--O | <--+               #

| | ^    +-----#----+       bottom right #

| | |          #  |               corner #    X

--------------- +==+======================+ ---->

(0,0)  (X1,0)

Figure 1 - Finishing Process Axis Parallel to Y Axis

^ Y

|        Head Locations

|<---------------->|---X2

|<---->|---X1      |

|      |           |

|    +-|-+       +-|-+

|    | | |       | | |  (X3,Y2)

+======|===========|======+

#    | | |       | | |    # Finishing Process Axis

#----| O |-------| O |----#----- Y1

#    +---+       +---+    #   ^

#      ^           ^      #   |

#      |           |      #   |

#      +-----------+      #   |

#      |                  #   |

#     Head                #   |

#     Mechanisms          #   |

#                         #   |

#                         # Finishing Process Offset

#                         #   |

#                         #   |

#                         #   |

#                         #   |

#                         #   |

#                         #   |

#            bottom right #   |

#                  corner #   v   X

+=========================+ ------>

(0,0)              Reference Edge

Figure 2 - Finishing Process Axis Parallel to X Axis

Media Orientation:  All Finishing Processes are defined relative to a

portrait orientation of the medium, regardless of the orientation of

the printed image or the direction of feed.  The 'X' and 'Y' axis,

therefore, will always reference the medium as shown in figures 1

and 2, with the 'X' axis always along the short edge of the medium.

All edges and corners are also defined with the medium orientation

as shown using the syntax top, bottom, left, and right.  Thus the

bottom edge of the medium is at Y = 0, the left edge is at X = 0,

and the bottom right corner is at (X2,0) as shown in the figure 1

and at (X3,0) as shown in figure 2.

Finishing:  Defined by DPA as an operation on a document following the

completion of the image process.  Finishing processes defined within

this document are those applied to one or more instances of

rectangular paper sheet media.

Finishing Process:  Defined by DPA as an operation applied by a

machine such as trimming a document, folding the sheets in a

document, and applying a binding to a document.

Finishing Specification:  Defined by DPA as the specific sequence of

operations for a serial combination of finishing processes.  The

exact sequential order of the processes, in many cases, is

critical to the obtaining the desired result.  For example, a

folding operation followed by trimming could provide a very

different result than if the trimming was followed by the folding.

Finishing Process Parameters:  This parameter set is used to create a

detailed definition of the finishing process.  Generic Finishing

Process Parameters are applicable to any Finishing Specification.

- Head Mechanism:  Defined by DPA as the physical mechanism that is

used to perform a finishing process.  The head position may be

fixed or variable depending upon the capabilities of the device.

- Reference Edge:  Defined by DPA as the edge of the document

relative to the axis to which the finishing process is applied.

The edge of the medium defined to be the Reference Edge may be

either the 'X' or the 'Y' axis, depending upon the finishing

process to be performed.

Note that the Reference Edge may change from one finishing process

to another for one of two reasons.  First, a subsequent process may

require a different Reference Edge.  Second, the actual dimensions

of the document may change, for example as a result of a folding or

a trimming operation.

- Jog Edge:  Defined by DPA as one of the two edges that is

perpendicular to the Reference Edge.  Specifying the Jog Edge

parameter indicates the edges of all sheets which correspond to

the Jog Edge are aligned.

- Finishing Process Axis:  Defined by DPA as the axis to which some

finishing processes are applied to or referenced from by the Head

Mechanism.  Examples are the axis for a fold process or the axis

for a punch process.

- Head Locations:  Defined by DPA as the position of the Heads on the

Finishing Process Axis.

- Finishing Process Offset:  The offset from the Reference Edge to

the Finishing Process Axis at which the finishing process takes

place or is applied.

## 2.2  Process Specific Terminology

FOLDING:

Z Fold:  A fold in which two folds are placed in the sheet in opposite

directions.  The first fold is located at 25% of the sheet length,

and the second is located at 50% of the sheet length (i.e., the

center of the sheet).  Z Folding is often used on 11x17 inch or A3

size sheets, when they are included in sets containing 8.5x11 inch

or A4 size sheets.

Half Fold:  To fold a sheet in half so that one of the resulting

dimensions are exactly half the original sheet.  Often used for

signatures or booklets.

Letter Fold:  Folding a sheet roughly in thirds.  Usually performed on

8.5x11 inch or A4 size sheets for insertion into an envelope.

Signature:  The process by which images are placed on a large sheet of

paper in correct panel areas and in the proper orientation such that

when the sheet is folded it will produce a booklet with each page in

the proper order and orientation.

BINDING:

Adhesive Binding:  A method of attaching sheets together to form a

book or booklet using glue or adhesive.  Some adhesive binding

methods apply the glue to sheets individually, before merging them

together for form a book, but most methods involve the application

of adhesive to an entire book of sheets.

Comb Binding:  A method of binding in which a series of small

rectangular holes is placed along the bind edge the sheets.  The

sheets are then held together using a tube shaped plastic binding

strip with comb like fingers that fit through the holes in the

sheets.

Spiral Binding:  Sometimes referred to as wire binding, this binding

method is a mechanical bind in which the individual leaves are held

together by a wire or plastic spiral that is fed through small holes

in the paper binding edge.

Padding:  Applying a non-penetrating adhesive to the edge of a stack

of sheets such that the sheets can be easily peeled off one at a

time.  Frequently used for forms.

Velo Binding:  A bind formed by punching holes into the edge of the

sheets, placing a two piece plastic strip (one side formed with

plastic pins that pass through the holes) along the edge and then

staking the two pieces together.

Perfect Binding:  A method of binding in which all pages are cut and

roughed up at the back or binding edge and held together by an

adhesive.

Tape Binding:  The act of placing tape over the bind edge of a set.

Sometimes contains adhesive to provide a functional bind to the set,

and sometimes done for decorative purposes on a set that has been

edge stapled.

SLITTING/CUTTING/TRIMMING:

Trim:  To cut the edges of a sheet or set of sheets.

Face Trim:  To cut the edges of a set of sheets on a booklet of sheets

that have been folded to eliminate the "creep" or edge shingling

that results from the folding process.

Gutter Trim:  To cut a larger sheet into smaller sheets eliminating

the gutter between adjacent images.  This operation requires a

minimum of two cuts for each gutter.

Tab Cutting:  The act of cutting the edge of a sheet to form an index

tab, thereby allowing quick identification and access.  The external

tabs are sequentially placed along the book edge for visibility and

ease of grasping.

Perforating:  The act of cutting a series of very small, closely

spaced holes or slots into a sheet to allow for ease of separation

of a portion of the sheet.  Sometimes also used to ease

bending/hinging of heavy weight papers.

Scoring:  A means of applying small linear grooves or impressions

along a sheet to allow easy folding.   Often used on heavy weight

sheets and book covers.

Slitting:  The action of cutting apart a large sheet to form smaller

sheets.  Usually done using a sharp circular roll system.

STITCHING/STAPLING:

Staple:  The process of binding a set of sheets together using a 'U'

shaped piece of metal wire that is punched through the set. The ends

of the metal staple are then bent over, or 'clinched' to hold the

staple in place.   Technically the term 'stapler' refers to devices

that use pre-cut metal staples, but the term is also commonly used

to refer to devices that use wire spools and then cut/form the

staple. (see the definition of Stitch)

Stitch:  The process of binding a set of sheets together using a 'U'

shaped piece of metal wire that is punched through the set.  The

wire used to form the staple is cut and formed into a 'U' shape in

the stitcher head, and the staple 'leg' length is often varied

depending on the number of sheets to be bound together.  The ends of

the metal staple are bent over, or 'clinched' to hold the staple in

place.

Stitching can also refer to the process of sewing the edges of the

signatures of a book together.

Saddle Stitch:  The process of stapling a set along its center line as

part of a booklet making process.   Usually 2 or 3 staples are used.

Dual Stapling:  The process of placing 2 staples along the bind edge

of a set.   The staples are typically located at 25% and 75% of the

length of the bind edge.   Although dual stapling is often performed

on the long edge of a set, legal documents are frequently dual

stapled along the top, or short edge of the set.

Triple Stapling:  Same as above, but using 3 staples along the bind

edge, and usually applies to the long edge only.

WRAPPING:

Shrink Wrap:  A wrap of thin plastic which when heated will shrink and

wrap tightly around the stack thus preparing it for shipment.

BANDING:

Band Wrap:  Bundling a finished stack to prepare for shipment.  Also

known as Strap Wrap.

ROTATING:

Sheet Rotator:  A device that rotates each sheet as received from the

Media Path to the proper orientation for the finisher processing.

# 3.  FINISHER SUBUNITS INTEGRATED INTO THE PRINTER MODEL

The Printer Finisher Device subunits receive media from one or more

Printer Media Path subunits and deliver the media to one or more

Printer Output subunits after the completion of the finishing

processes.  The Printer Model, as described in the Printer MIB

[PRTMIB], is modified adding the finisher subunit(s) and finisher

supplies between the media path and output subunits as follows:

+----------+

+----------+ |

|  Marker  | |

| Supplies |-+

+----------+

\

+-----+               \ +------+                +--------+    +------+

|     |                \|      |                |        |    |      |

+-----+ |    +-----+    +------+ |   +------+   +--------+ |  +------+ |

|Input|-+  +------+|    |Marker|-+  +------+|   |Finisher|-+  |Output|-+

|     |===>|      |+<==>|      |<==>|      |+==>|        |===>|      |

+-----+    +-+  +-+     +------+    +-+  +-+    +--------+    +------+

\         |  ||                    |  ||         \

\        |  ||                    |  ||          \

\       |  ||                    |  ||    +----------+

+-------+  |  |+--------------------|  ||    | Finisher |-+

|       |  |  +---------------------+  ||    | Supplies | |

+-------+ |  |         Media Path        |+    +----------+ |

| Media |-+  +---------------------------+       |          |

|(opt.) |                                        +----------+

+-------+

# 4.  FINISHING SPECIFICATIONS

The Finisher MIB is able to provide most of the information that is

required to generate a Finishing Specification.  This includes;

1. Finishing operations that can be performed on media that are

associated with a specific printer media path and output subunit.

2. Combinations of operations that cannot be performed.

3. The location of the operation on the medium, if applicable.

4. The physical characteristics of the result of the operation.

For example, the size and shape of a punched hole, or if a fold

operation creates a letter fold or a "Z" fold.

The Finisher MIB permits an agent to describe the order that

operations can be performed.

## 4.1  Multiple finDeviceTable Entries

Each finishing operation supported by the printer is represented by

one or more entries in the finDeviceTable.  Each entry in this table

defines a "logical" finishing device, since the function of several

table entries may be performed by a single finisher mechanism.

Multiple entries may also exist in the table as a result of the

existence of multiple finisher mechanisms that perform the same type

of operation.

One example of possible multiple entries for a single finisher

device, is a hole punch operation that creates more than one hole.

This could be performed using a single die punch that moves to each

required position or a multi-die punch that simultaneously creates

all holes.  In either case, each defined hole position may be defined

as a separate table entry.

In both cases, if the punch positions can be individually selected,

a table entry for each position would be necessary.

For the multi-die punch, each head mechanism may have a different

hole pattern or size.  If these differences are to be properly

disclosed, a table entry for each head mechanism would be required.

4.2  Implicit Parameters

Finishing operations that are specified by an enum define a

standard operation and in many cases an implicit set of physical

characteristics is to be included when specifying the enum.  If

explicit values for these characteristics are not provided in the

attributes table, the values defined in this section are to be

implied.

## 4.2.1  FinPunchPatternTC

enum pattern       |Reference| Reference | Hole spacing

| Edge    |Axis Offset| (see note 1)

-------------------+---------+-----------+---------------------------

twoHoleUSTop(4)    | topEdge |   note 2  | 2.75 inches

threeHoleUS(5)     |  note 3 |   note 2  | 4.25 inches

twoHoleDIN(6)      |  note 4 |   note 5  | 80 mm

fourHoleDIN(7)     |  note 4 |   note 5  | 80 mm

twentyTwoHoleUS(8) |  note 3 |   note 2  | .5 inches

nineteenHoleUS(9)  |  note 3 |   note 9  | .5625 inches

twoHoleMetric(10)  |  note 6 |   note 8  | 80 mm

swedish4Hole(11)   |  note 4 |   note 5  | 21, 70, 21 mm

twoHoleUSSide(12)  |  note 3 |   note 2  | 2.75 inches

fiveHoleUS(13)     |  note 3 |   note 2  | 2, 2.25, 2.25, 2 in

sevenHoleUS(14)    |  note 3 |   note 2  | 1, 1, 2.25, 2.25, 1, 1 in

mixed7H4S(15)      |  note 4 |   note 5  | note 7

norweg6Hole(16)    |  note 4 |   note 5  | note 8

metric26Hole(17)   |  note 6 |   note 5  | 9.5 mm

metric30Hole(18)   |  note 4 |   note 5  | 9.5 mm

Notes:

1. All hole to hole patterns are centered along the process edge.

2. Offset is 0.18 inches to 0.51 inches.

3. Reference edge is leftEdge(5) for letter and topEdge(3)

for ledger.

4. Reference edge is leftEdge(5) for A4 and topEdge(3) for A3.

5. Offset is 4.5 mm to 13 mm.

6. Reference edge is leftEdge(5) for B5 and topEdge(3) for B4.

7. 7 holes and 4 slots are punched in a H-S-H-H-S-H-S-H-H-S-H

pattern with 15, 25, 23, 20, 37, 37, 20, 23, 25, 15 mm spacing.

8. 4 holes and 2 slots are punched in a H-H-S-S-H-H pattern with

a 64, 18.5, 75, 18.5, 64 mm spacing.

9. Offset is .188 inches.

## 4.2.2  FinPunchHoleTypeTC, punchHoleSizeMaxDim, punchHoleSizeMinDim

enum pattern       | Hole Description

-------------------+----------------------------------------

twoHoleUSTop(4)    | round(3), .2 - .32 inch diameter

threeHoleUS(5)     | round(3), .2 - .32 inch diameter

twoHoleDIN(6)      | round(3),  5 - 8 mm diameter

fourHoleDIN(7)     | round(3),  5 - 8 mm diameter

twentyTwoHoleUS(8) | round(3), .2 - .32 inch diameter

nineteenHoleUS(9)  | rectang(6), .313 inches X .125 inches

twoHoleMetric(10)  | round(3),  5 - 8 mm diameter

swedish4Hole(11)   | round(3),  5 - 8 mm diameter

twoHoleUSSide(12)  | round(3), .2 - .32 inch diameter

fiveHoleUS(13)     | round(3), .2 - .32 inch diameter

sevenHoleUS(14)    | round(3), .2 - .32 inch diameter

mixed7H4S(15)      | round(3),  5 - 8 mm diameter

| rectang(6),  12 mm X 6 mm

norweg6Hole(16)    | round(3),  5 - 8 mm diameter

| rectang(6),  10 mm  X 5.5 mm

metric26Hole(17)   | round(3),  5 - 8 mm

metric30Hole(18)   | round(3),  5 - 8 mm

Note: Hole size ranges are typical and are provided as a reference

only.  Exact tolerances should be site defined.

# 5.  THE ATTRIBUTE MECHANISM

Attributes provide a function similar to information objects, except

that attributes are identified by an enum, instead of an OID.  Thus

new attributes may be registered without requiring a change to the

MIB.  In addition, an implementation that does not have the

functionality represented by the attribute can omit the attribute

entirely, rather than having to return a distinguished value.  The

agent is free to materialize an attribute in the Attribute Table as

soon as the agent is aware of the value of the attribute.

The agent materializes finishing subunit attributes in a four-indexed

finDeviceAttributeTable:

1. hrDeviceIndex - which device in the host

2. finDeviceIndex - which finisher subunit in the printer device

3. finDeviceAttributeTypeIndex - which attribute

4. finDeviceAttributeInstanceIndex - which attribute instance for

those attributes that can have multiple values per finishing

subunit.

## 5.1  Conformance of Attribute Implementation

An agent SHALL implement any attribute if (1) the device supports the

functionality represented by the attribute and (2) the information is

available to the agent.  The agent MAY create the attribute row in

the finDeviceAttributeTable when the information is available or MAY

create the row earlier with the designated 'unknown' value

appropriate for that attribute.  See next section.

If the device does not implement or does not provide access to the

information about an attribute, the agent SHOULD NOT create the

corresponding row in the finDeviceAttributeTable.

## 5.2  Useful, 'Unknown', and 'Other' Values for Objects and Attributes

Some attributes have a 'useful' Integer32 value, some have a 'useful'

OCTET STRING value, some MAY have either or both depending on

implementation, and some MUST have both.  See the

finDeviceAttributeTypeTC textual convention for the specification of

each attribute.

SNMP requires that if an object cannot be implemented because its

values cannot be accessed, then a compliant agent SHALL return an

SNMP error in SNMPv1 or an exception value in SNMPv2.  However, this

MIB has been designed so that 'all' objects can and SHALL be

implemented by an agent, so that neither the SNMPv1 error nor the

SNMPv2 exception value SHALL be generated by the agent.  This MIB has

also been designed so that when an agent materializes an attribute,

the agent SHALL materialize a row consisting of both the

finDeviceAttributeValueAsInteger and

finDeviceAttributeValueAsOctets objects.

In general, values for objects and attributes have been chosen so

that a management application will be able to determine whether a

'useful', 'unknown', or 'other' value is available.  When a useful

value is not available for an object that agent SHALL return a zero-

length string for octet strings, the value 'unknown(2)' for enums, a

'0' value for an object that represents an index in another table,

and a value '-2' for counting integers.

Since each attribute is represented by a row consisting of both the

finDeviceAttributeValueAsInteger and finDeviceAttributeValueAsOctets

MANDATORY objects, SNMP requires that the agent SHALL always create

an attribute row with both objects specified.  However, for most

attributes the agent SHALL return a "useful" value for one of the

objects and SHALL return the 'other' value for the other object.  For

integer only attributes, the agent SHALL always return a zero-length

string value for the finDeviceAttributeValueAsOctets object.  For

octet string only attributes, the agent SHALL always return a '-1'

value for the finDeviceAttributeValueAsInteger object.

## 5.3  Data Sub-types and Attribute Naming Conventions

Many attributes are sub-typed to give a more specific data type than

Integer32 or OCTET STRING.  The data sub-type of each attribute is

indicated on the first line(s) of the description.  Some attributes

have several different data sub-type representations.  When an

attribute has both an Integer32 data sub-type and an OCTET STRING

data sub-type, the attribute can be represented in a single row in

the finDeviceAttributeTable.  In this case, the data sub-type name is

not included as the last part of the name of the attribute.  When the

data sub-types cannot be represented by a single row in the

finDeviceAttributeTable, each such representation is considered a

separate attribute and is assigned a separate name and enum value.

For these attributes, the name of the data sub-type is the last part

of the name of the attribute.

## 5.4  Single-Value (Row) Versus Multi-Value (MULTI-ROW) Attributes

Most attributes SHALL have only one row per finishing subunit.

However, a few attributes can have multiple values per finishing

subunit, where each value is a separate row in the

finDeviceAttributeTable.  Unless indicated with 'MULTI-ROW:' in the

finDeviceAttributeTypeTC description, an agent SHALL ensure that each

attribute occurs only once in the finDeviceAttributeTable for a

finishing subunit.  Most of the 'MULTI-ROW' attributes do not allow

duplicate values, i.e., the agent SHALL ensure that each value occurs

only once for a finishing subunit.  Only if the specification of the

'MULTI-ROW' attribute also says "There is no restriction on the same

xxx occurring in multiple rows" can the agent allow duplicate values

to occur for a single finishing subunit.

5.5  Linked MUTI-ROW Values

Some MULTI-ROW attributes are intended to go together.  Thus a set

of value instances represent a single instance.  For example, the

puncher attributes indicate the location, maximum size, minimum size

and shape of the various holes that the puncher can produce.  So the

first set of values could represent one kind of hole, and the second

set another kind of hole, etc.

## 5.6  Index Value Attributes

A number of attributes are indexes in other tables.  Such attribute

names end with the word 'Index'.  If the agent has not (yet) assigned

an index value for a particular index attribute for a finishing

subunit, the agent SHALL either: (1) return the value 0 or (2) not

add this attribute to the finDeviceAttributeTable until the index

value is assigned.  In the interests of brevity, the semantics for 0

is specified once here and is not repeated for each index attribute

specification and a DEFVAL of 0 is indicated.

# 6.  Enumerations

Enumerations (enums) are sets of symbolic values defined for use

with one or more objects.  Some common enumeration sets are

assigned a symbolic data type name (textual convention).  These

enumerations are listed at the beginning of this specification.

## 6.1  Registering Additional Enumerated Values

The Printer Working Group (PWG) has defined several type of

enumerations.  These enumerations differ in the method employed

to control the addition of new enumerations.  Throughout this

document, references to "enumeration (n)", where n can be 1, 2 or

3 can be found in the various tables.  The definitions of these

types of enumerations are:

enumeration (1)  All the values are defined in the Printer Finisher

MIB specification (RFC for the Printer Finisher MIB).  Additional

enumerated values require a new RFC.  Type 1 enumerations are

typically used where changes to the enumeration are either unlikely

or will have a significant impact on the structure of the MIB or

implementation of the MIB in management applications.

Some criteria that suggest using a type 1 enumeration are:

a) the set of values in the enumeration is thought to be known,

e.g., faceUp and faceDown

b) the enumeration defines a set of units of measure which must

be understood by a management application to be able to

correctly display the value of an object that measurement unit

controls;

and

c) the enumeration is tied to the structure of the MIB or the

model on which the MIB is based, i.e., the enumeration is tied

to the OIDs for the related tables.

There are no type 1 enums in the current draft.

enumeration (2)  An initial set of values are defined in the

Printer Finisher MIB specification.  Additional enumerated values

are registered after review by this working group.  The initial

versions of the MIB will contain the values registered so far.

After the MIB is approved, additional values will be registered

through IANA after approval by this working group.  The current

set of approved values should always be obtained from the IANA

registry. Type 2 enumerations are typically used where it is

important to ensure consistent usage of the enumeration values;

that is, to ensure that the same entity does not get two

different enumerations values, or two different entities do not

get the same enum value.

enumeration (3)  An initial set of values are defined in the

Printer Finisher MIB specification.  Additional enumerated values

are registered without working group review.  The initial versions

of the MIB will contain the values registered so far.  After the MIB

is approved, additional values will be registered through IANA

without approval by this working group.  The current set of

approved values should always be obtained from the IANA registry.

Type 3 enumerations are used for enumerations that can be extended

without any controls; an example is the prtMarkerSuppliesType, which

can be extended as needed by any manufacturer to describe the

supplies required by a new printer.

# 7.  PRINTER FINISHING MIB SPECIFICATION

Finisher-MIB DEFINITIONS ::= BEGIN

IMPORTS

mib-2                                          FROM RFC1213-MIB

MODULE-IDENTITY, OBJECT-TYPE, Integer32         FROM SNMPv2-SMI

TEXTUAL-CONVENTION                               FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP                FROM SNMPv2-CONF

hrDeviceIndex                           FROM HOST-RESOURCES-MIB

printmib, PrtMarkerSuppliesTypeTC,

PrtSubUnitStatusTC, PrtInputTypeTC,

PrtMarkerSuppliesSupplyUnitTC,

PrtCapacityUnitTC, PrtMarkerSuppliesClassTC,

PresentOnOff, prtMIBConformance               FROM Printer-MIB;

finisherMIB MODULE-IDENTITY

LAST-UPDATED "9801120000Z"

ORGANIZATION "IETF Printer MIB Working Group"

CONTACT-INFO

"Ron Bergman

Dataproducts Corp.

1757 Tapo Canyon Road

Simi Valley, CA 91063-3394

rbergma@dpc.com

Send comments to the printmib WG using the Finisher MIB

Project (FIN) Mailing List:  fin@pwg.org

For further information, access the PWG web page under 'FIN':

http://www.pwg.org/

Implementers of this specification are encouraged to join the

fin mailing list in order to participate in discussions on any

clarifications needed and registration proposals being reviewed

in order to achieve consensus."

DESCRIPTION

"The MIB module for management of printer finisher units.

The Finisher MIB is an extension of the Printer MIB."

::= { mib-2 43 }

-- Textual conventions for this MIB module

FinDeviceTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined finishing device subunit process

enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

stitcher(3),

folder(4),

binder(5),

trimmer(6),

dieCutter(7),

puncher(8),

perforater(9),

slitter(10),

separationCutter(11),

imprinter(12),

wrapper(13),

bander(14),

makeEnvelope(15),

stacker(16),

sheetRotator(17)

}

FinAttributeTypeTC  ::= TEXTUAL-CONVENTION

-- This is a type 3 enumeration.

STATUS     current

DESCRIPTION

"This textual convention defines a set of enums for use in

the finDeviceAttributeTable.  The data type tag definitions

'INTEGER:' or 'OCTETS', indicate if the attribute can be

represented using the object finDeviceAttributeAsInteger or

the object finDeviceAttributeAsOctets, respectively.  In some

cases, a choice between the two data types is possible and for

a few attributes both objects may be required at the same time

to properly present the value.

NOTE - The enum assignments are grouped logically with values

assigned in groups of 10, so that additional values may be

registered in the future and assigned a value that is part of

their logical grouping.

Values in the range 2**30 to 2**31-1 are reserved for private

or experimental usage.  This range corresponds to the same

range reserved in IPP.  Implementers are warned that use of

such values may conflict with other implementations.

Implementers are encouraged to request registration of enum

values following the procedures in Section 5.1.

The attribute types defined at the time of completion of this

specification are:

finAttributeTypeIndex              Data type

---------------------              ---------

other(1),                          Integer32

AND/OR

OCTET STRING (SIZE(0..63))

INTEGER: and/or OCTETS:  An attribute that is not currently

approved and registered.

-- Generic finisher subunit attributes that apply to all

-- finisher subunit types.  (3..)

deviceName(3),                     OCTET STRING (SIZE(0..63))

OCTETS:  The name assigned to this finisher device subunit.

deviceVendorName(4),               OCTET STRING (SIZE(0..63))

OCTETS:  The name of the vendor of this finisher device

subunit.

deviceModel(5),                    OCTET STRING (SIZE(0..63))

OCTETS:  The model name of this finisher device subunit.

deviceVersion(6),                  OCTET STRING (SIZE(0..63))

OCTETS:  The version string for this finisher device

subunit.

deviceSerialNumber(7),             OCTET STRING (SIZE(0..63))

OCTETS:  The serial number assigned to this finisher device

subunit.

maximumSheets(8),                  Integer32 (-2..32767)

INTEGER:  Defines the maximum number of media sheets that a

finisher device is able to process.

finProcessOffsetUnits(9),          PrtMediaUnitTC

INTEGER:  An enumeration which defines the units of measure

for the attributes finAxisOffset and finHeadLocation.

finReferenceEdge(10),              FinEdgeTC

INTEGER:  An enumeration which defines which edge of the

form is the reference for this finishing process.  The

Finishing Process Axis will be parallel to this axis.

finAxisOffset(11),                 Integer32 (-2..2147483647)

INTEGER:  Defines the parallel offset of the Finishing

Process Axis from the Reference Edge.  For a value of

finEdgeTC equal to TopEdge or RightEdge, the value

given is to interpreted as a negative offset from the

reference edge.  The units of measure are defined by the

attribute finReferenceEdgeTC.

finJogEdge(12),                    FinEdgeTC

INTEGER:  An enumeration which defines a second edge of the

document to which the media is aligned.  The jog edge must

be perpendicular to the edge defined by finReferenceEdge.

finHeadLocation(13),               Integer32 (-2..2147483647)

INTEGER: MULTI-ROW:  Defines the position of the Head

Mechanism relative to the axis, 'X' or 'Y', that is

perpendicular to the Process Axis.  The units of measure

are defined by the attribute finProcessOffsetUnits.

finOperationRestrictions(14),      Integer32 (0..65535)

INTEGER: MULTI-ROW:  Defines the finDeviceIndex of a

finishing process which cannot be combined with the

process defined by the finDeviceIndex for this

finDeviceAttributeTable instance.  When this condition

occurs this attribute SHALL be presented in the

attribute tables for both finishing processes that cannot

be combined.

finNumberOfPositions(15),          Integer32 (0..65535)

INTEGER:  Defines the total number of head positions for

this finishing process.  Each position many be realized by

a unique head mechanism or a single head mechanism may be

automatically moved to each position.

namedConfiguration(16),            OCTET STRING (SIZE(0..63))

OCTETS:  Contains an administratively define name to define

the finishing specification configured for this device.

finMediaTypeRestriction(17),       OCTET STRING (SIZE(0..63))

OCTETS: MULTI-ROW:  Defines the media type which cannot be

combined with the process defined by the finDeviceIndex

for this finDeviceAttributeTable instance.  Values are the

same as defined for finSupplyMediaInputMediaName.

finPrinterInputTraySupported(18),  Integer32 (0..65535)

INTEGER: MULTI-ROW:  Defines the value of prtInputIndex

corresponding to the printer input tray that can be used

with the process defined by the finDeviceIndex for this

finDeviceAttributeTable instance.  If this attribute is

not present, this process can be used with any input tray

in the printer.  For example, this attribute can indicate

the current stapling capabilities for a stapler device

for the input trays that depend upon the size and feed

orientation.  So if there were two letter trays, one with

A size and the other with B size, a two position stapler

might specify in one row: upper-left and upper-right for

the input tray with A size, but only upper-left for the

one with B size.

finPreviousFinishingOperation(19),         Integer32 (0..65535)

INTEGER:  Defines the finDeviceIndex of the previous

finishing process for implementations in which the

finishing processes are performed in a prescribed order.

Each finishing process in the fixed sequence is either

performed or not performed according to the finishing

instructions submitted with the job.  A value of 0

indicates that this finishing process is the first in a

sequence.  Finishing processes which are not part of a

fixed sequence SHALL NOT have this attribute.

finNextFinishingOperation(20),             Integer32 (0..65535)

INTEGER:  Defines the finDeviceIndex of the next finishing

process for implementations in which the finishing

processes are performed in a prescribed order.  Each

finishing process in the fixed sequence is either

performed or not performed according to the finishing

instructions submitted with the job.  A value of 0

indicates that this finishing process is the last in a

sequence.  Finishing processes which are not part of a

fixed sequence SHALL NOT have this attribute.

-- Stitcher type-specific attributes (30..)

stitchingType(30),                 FinStitchingTypeTC

INTEGER: MULTI-ROW:  Provides additional information

regarding the stitching operation.

-- Folder type-specific attributes (40..)

foldingType(40),                   FinFoldingTypeTC

INTEGER:  Provides additional information regarding the

folding process.

-- Binder type-specific attributes (50..)

bindingType(50),                   FinBindingTypeTC

INTEGER:  Provides additional information regarding the

binding process.

-- Trimmer type-specific attributes (60..)

-- Die cutter type-specific attributes (70..)

-- Puncher type-specific attributes (80..)

punchHoleType(80),                 FinPunchHoleTypeTC

INTEGER:  Provides information regarding the shape of the

punched hole.

punchHoleSizeLongDim(81),          Integer32 (-2..2147483647)

INTEGER:  Defines the size of the punched hole in the

longest dimension.  This dimension is typically measured

parallel to either the long edge or the short edge of the

media and the longest dimension will always be measured 90

degrees from the shortest dimension.  For a symmetrical

hole, such as a round or square hole, the shortest and

longest dimensions will be identical.

punchHoleSizeShortDim(82),         Integer32 (-2..2147483647)

INTEGER:  Defines the size of the punched hole in the

shortest dimension.  This dimension is typically measured

parallel to either the long edge or the short edge of the

media and the shortest dimension will always be measured

90 degrees from the longest dimension.  For a symmetrical

hole, such as a round or square hole, the shortest and

longest dimensions will be identical.

punchPattern(83),                  FinPunchPatternTC

INTEGER:  Defines the hole pattern produced by the punch

process.

-- Perforator type-specific attributes (90..)

-- Slitter type-specific attributes (100..)

slittingType(100),                 FinSlittingTypeTC

INTEGER:  Provides additional information regarding the

slitting process.

-- Separation cutter type-specific attributes (110..)

-- Imprinter type-specific attributes (120..)

-- Wrapper type-specific attributes (130..)

wrappingType(130),                 FinWrappingTypeTC

INTEGER:  Provides additional information regarding the

wrapping process.

-- Bander type-specific attributes (140..)

-- Make Envelopes type-specific attributes (150..)

-- Stacker type-specific attributes (160..)

stackOutputType(160)               FinStackOutputTypeTC

INTEGER:  Defines the job-to-job orientation produced by

the stacker.

stackOffset(161)                   Integer32 (-2..2147483647)

INTEGER:  Defines the copy-to-copy output stack offset as

a positive offset distance.  The units of measure are

defined by finProcessOffsetUnits.

stackRotation(162)                 Integer32 (-2..180)

INTEGER:  Defines the copy-to-copy output stack rotation

measured in degrees.  The value is the positive

copy-to-copy rotation.

SYNTAX       INTEGER {

other(1),

deviceName(3),

deviceVendorName(4),

deviceModel(5),

deviceVersion(6),

deviceSerialNumber(7),

maximumSheets(8),

finProcessOffsetUnits(9),

finReferenceEdge(10),

finAxisOffset(11),

finJogEdge(12),

finHeadLocation(13),

finOperationRestrictions(14),

finNumberOfPositions(15),

namedConfiguration(16),

finMediaTypeRestriction(17),

finPrinterInputTraySupported(18),

finPreviousFinishingOperation(19),

finNextFinishingOperation(20),

stitchingType(30),

foldingType(40),

bindingType(50),

punchHoleType(80),

punchHoleSizeLongDim(81),

punchHoleSizeShortDim(82),

punchPattern(83),

slittingType(100),

wrappingType(130),

stackOutputType(160),

stackOffset(161),

stackRotation(162)

}

FinEdgeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"Specifies an edge for a Finishing Process."

SYNTAX       INTEGER {

topEdge(3),

bottomEdge(4),

leftEdge(5),

rightEdge(6)

}

FinStitchingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined stitching type enumerations."

SYNTAX       INTEGER {

other(1),         -- More information in other attributes

unknown(2),

stapleTopLeft(4),

stapleBottomLeft(5),

stapleTopRight(6),

stapleBottomRight(7),

saddleStitch(8),

edgeStitch(9),

stitch(10)

}

FinFoldingTypeTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined folding device process enumerations."

SYNTAX       INTEGER {

other(1),        -- More information in other attributes

unknown(2),

zFold(3),

halfFold(4),

letterFold(5)

}

FinBindingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined binding type enumerations."

SYNTAX       INTEGER {

other(1),          -- More information in other attributes

unknown(2),

tape(4),

plastic(5),

velo(6),

perfect(7),

spiral(8),

adhesive(9),

comb(10),

padding(11)

}

FinPunchHoleTypeTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined hole type punch process enumerations."

SYNTAX       INTEGER {

other(1),        -- More information in other attributes

unknown(2),

round(3),

oblong(4),

square(5),

rectangular(6),

star(7)

}

FinPunchPatternTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined hole pattern punch process enumerations."

SYNTAX       INTEGER {

other(1),           --Pattern to be defined in other attributes

unknown(2),

twoHoleUSTop(4),    --Letter/legal, 8.5 inch edge

threeHoleUS(5),     --Letter/ledger, 11 inch edge

twoHoleDIN(6),      --A4/A3, 297 mm edge

fourHoleDIN(7),     --A4/A3, 297 mm edge

twentyTwoHoleUS(8), --Letter/ledger, 11 inch edge

nineteenHoleUS(9),  --Letter/ledger, 11 inch edge

twoHoleMetric(10),  --B5/B4, 257 mm edge

swedish4Hole(11),   --A4/A3, 297 mm edge

twoHoleUSSide(12),  --Letter/ledger, 11 inch edge

fiveHoleUS(13),     --Letter/ledger, 11 inch edge

sevenHoleUS(14),    --Letter/ledger, 11 inch edge

mixed7H4S(15),      --A4/A3, 297 mm edge

norweg6Hole(16),    --A4/A3, 297 mm edge

metric26Hole(17),   --B5/B4, 257 mm edge

metric30Hole(18)    --A4/A3, 297 mm edge

}

FinSlittingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined slitting type enumerations."

SYNTAX       INTEGER {

other(1),         -- More information in other attributes

unknown(2),

slitAndSeparate(4),

slitAndMerge(5)

}

FinWrappingTypeTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined wrapping device process enumerations."

SYNTAX       INTEGER {

other(1),        -- More information in other attributes

unknown(2),

shrinkWrap(4),

paperWrap(5)

}

FinStackOutputTypeTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined stack output type enumerations."

SYNTAX       INTEGER {

other(1),        -- More information in other attributes

unknown(2),

straight(4),     -- No offset, one on top of another

offset(5),

crissCross(6)    -- Rotated

}

-- Finisher Device Group (Mandatory)

--

-- A printer may support zero or more finishing subunits.  A

-- finishing device subunit may be associated with one or more

-- output subunits and one or more media path subunits.

finDevice  OBJECT IDENTIFIER  ::=  { printmib 30 }

finDeviceTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinDeviceEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This table defines the finishing device subunits,

including information regarding possible configuration

options and the status for each finisher device subunit."

::= { finDevice 1 }

finDeviceEntry OBJECT-TYPE

SYNTAX      FinDeviceEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"There is an entry in the finishing device table for each

possible finisher process."

INDEX  { hrDeviceIndex, finDeviceIndex }

::= { finDeviceTable 1 }

FinDeviceEntry ::= SEQUENCE {

finDeviceIndex                 Integer32,

finDeviceType                  FinDeviceTypeTC,

finDevicePresentOnOff          PresentOnOff,

finDeviceCapacityUnit          PrtCapacityUnitTC,

finDeviceMaxCapacity           Integer32,

finDeviceCurrentCapacity       Integer32,

finDeviceAssociatedMediaPaths  OCTET STRING,

finDeviceAssociatedOutputs     OCTET STRING,

finDeviceStatus                PrtSubUnitStatusTC,

finDeviceDescription           OCTET STRING

}

finDeviceIndex OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique value used to identify a finisher process.

Although these values may change due to a major

reconfiguration of the printer system (e.g. the addition

of new finishing processes), the values are normally

expected to remain stable across successive power cycles."

::= { finDeviceEntry 1 }

finDeviceType OBJECT-TYPE

SYNTAX      FinDeviceTypeTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Defines the type of finishing process associated with this

table row entry."

::= { finDeviceEntry 2 }

finDevicePresentOnOff OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if this finishing device subunit is available

and whether the device subunit is enabled."

DEFVAL      { notPresent }

::= { finDeviceEntry 3 }

finDeviceCapacityUnit OBJECT-TYPE

SYNTAX      PrtCapacityUnitTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unit of measure for specifying the capacity of this

finisher device subunit."

::= { finDeviceEntry 4 }

finDeviceMaxCapacity OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The maximum capacity of this finisher device subunit in

finDeviceCapacityUnits.  If the device can reliably sense

this value, the value is sensed by the finisher device

and is read-only: otherwise the value may be written by a

management or control console application.  The value (-1)

means other and specifically indicates that the device

places no restrictions on this parameter.  The value (-2)

means unknown."

DEFVAL      { -2 }      -- unknown

::= { finDeviceEntry 5 }

finDeviceCurrentCapacity OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The current capacity of this finisher device subunit in

finDeviceCapacityUnits.  If the device can reliably sense

this value, the value is sensed by the finisher and is

read-only: otherwise the value may be written by a

management or control console application.  The value (-1)

means other and specifically indicates that the device

places no restrictions on this parameter.  The value (-2)

means unknown."

DEFVAL      { -2 }      -- unknown

::= { finDeviceEntry 6 }

finDeviceAssociatedMediaPaths OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the media paths which can supply media for this

finisher device.  The value of this object is a bit map in an

octet string with each position representing the value of a

prtMediaPathIndex.  For a media path that can be a source

for this finisher device subunit, the bit position equal

to one less than the value of prtMediaPathIndex will be set.

The bits are numbered starting with the most significant bit of

the first byte being bit 0, the least significant bit of the

first byte being bit 7, the most significant of the second byte

being bit 8, and so on."

::= { finDeviceEntry 7 }

finDeviceAssociatedOutputs OBJECT-TYPE

SYNTAX      OCTET STRING

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the printer output subunits this finisher device

subunit services.  The value of this object is a bit map in an

octet string with each position representing the value of a

prtOutputIndex.  For an output subunit that is serviced

by this finisher device subunit, the bit position equal to

to one less than the value of prtOutputIndex will be set.

The bits are numbered starting with the most significant bit of

the first byte being bit 0, the least significant bit of the

first byte being bit 7, the most significant of the second byte

being bit 8, and so on."

::= { finDeviceEntry 8 }

finDeviceStatus OBJECT-TYPE

SYNTAX      PrtSubUnitStatusTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the current status of this finisher device

subunit."

DEFVAL      { 5 }      -- unknown

::= { finDeviceEntry 9 }

finDeviceDescription OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A free form text description of this device subunit in the

localization specified by prtGeneralCurrentLocalization."

::= { finDeviceEntry 10 }

-- Finisher Supply Group (Mandatory)

--

-- A finisher device, but not all finisher devices, may have one or more

-- supplies associated with it.  For example a finisher may use both

-- binding tape and stitching wire supplies. A finisher may also have

-- more than one source for a given type of supply e.g. multiple supply

-- sources of ink for imprinters.

finSupply  OBJECT IDENTIFIER  ::=  { printmib 31 }

finSupplyTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinSupplyEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Each unique source of supply is an entry in the finisher

supply table. Each supply entry has its own

characteristics associated with it such as colorant and

current supply level."

::= { finSupply 1 }

finSupplyEntry OBJECT-TYPE

SYNTAX     FinSupplyEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A list of finisher devices, with their associated

supplies and supplies characteristics."

INDEX  { hrDeviceIndex, finSupplyIndex }

::= { finSupplyTable 1 }

FinSupplyEntry ::= SEQUENCE {

finSupplyIndex               Integer32,

finSupplyDeviceIndex         Integer32,

finSupplyClass               PrtMarkerSuppliesClassTC,

finSupplyType                PrtMarkerSuppliesTypeTC,

finSupplyDescription         OCTET STRING,

finSupplyUnit                PrtMarkerSuppliesSupplyUnitTC,

finSupplyMaxCapacity         Integer32,

finSupplyCurrentLevel        Integer32,

finSupplyColorName           OCTET STRING

}

finSupplyIndex OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique value used by a finisher to identify this supply

container/receptacle.  Although these values may change

due to a major reconfiguration of the finisher (e.g. the

addition of new supply sources to the finisher), values

are normally expected to remain stable across successive

power cycles."

::= { finSupplyEntry 1 }

finSupplyDeviceIndex OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of finDeviceIndex corresponding to the finishing

device subunit with which this finisher supply is associated."

::= { finSupplyEntry 2 }

finSupplyClass OBJECT-TYPE

SYNTAX      PrtMarkerSuppliesClassTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value indicates whether this supply entity

represents a supply that is consumed or a container that

is filled."

::= { finSupplyEntry 3 }

finSupplyType OBJECT-TYPE

SYNTAX      PrtMarkerSuppliesTypeTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The type of this supply."

::= { finSupplyEntry 4 }

finSupplyDescription OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The description of this supply/receptacle in text useful

for operators and management applications and in the

localization specified by prtGeneralCurrentLocalization."

::= { finSupplyEntry 5 }

finSupplyUnit OBJECT-TYPE

SYNTAX      PrtMarkerSuppliesSupplyUnitTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Unit of measure of this finisher supply container or

receptacle."

::= { finSupplyEntry 6 }

finSupplyMaxCapacity OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The maximum capacity of this supply container/receptacle

expressed in Supply Units.  If this supply container/

receptacle can reliably sense this value, the value is

sensed  and is read-only; otherwise the value may be

written by a control panel or management application.  The

value (-1) means other and places no restrictions on this

parameter.  The value (-2) means unknown."

DEFVAL      { -2 }      -- unknown

::= { finSupplyEntry 7 }

finSupplyCurrentLevel OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The current level if this supply is a container; the

remaining space if this supply is a receptacle. If this

supply container/receptacle can reliably sense this value,

the value is sensed  and is read-only; otherwise the value

may be written by a control panel or management

application.  The value (-1) means other and places no

restrictions on this parameter. The value (-2) means

unknown."

DEFVAL      { -2 }      -- unknown

::= { finSupplyEntry 8 }

--             Capacity Attribute Relationships

--

--                 MEDIA INPUT MEASUREMENT

--

--     _______ |                   |

--      |      |                   |

--      |      |                   |                   |

--      |      |_ _ _ _ _ _ _ _ _ _| ________________  |direction

--      |      |                   |      |            v

-- MaxCapacity |                   |      |

--      |      | Sheets remaining  | CurrentLevel

--      |      |                   |      |

--      v      |                   |      v

--     _______ +___________________+ _______

finSupplyColorName OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..63))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The name of the color of this colorant using standardized

string names from ISO 10175 (DPA) and ISO 10180 (SPDL)

which are: other, unknown, white, red, green, blue, cyan,

magenta, yellow and black.  Implementors may add

additional string values.  The naming conventions in ISO

9070 are recommended in order to avoid potential name

clashes."

::= { finSupplyEntry 9 }

-- Finisher Supply, Media Input Group (Conditionally Mandatory)

--

-- A finisher device may have one or more associated supply media

-- inputs.  Each entry in this table defines an input for a

-- supply media type such as inserts, covers, etc.

--

-- This group is mandatory only if the printer system contains a

-- finisher device that requires a media supply used exclusively by a

-- finishing process.  Examples are inserts or covers that are not

-- supplied by an input subunit that provides media to the marker.

finSupplyMediaInput  OBJECT IDENTIFIER  ::=  { printmib 32 }

finSupplyMediaInputTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinSupplyMediaInputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The input subunits associated with a finisher supply media

are each represented by an entry in this table."

::= { finSupplyMediaInput 1 }

finSupplyMediaInputEntry OBJECT-TYPE

SYNTAX     FinSupplyMediaInputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A list of finisher supply media input subunit features and

characteristics."

INDEX  { hrDeviceIndex, finSupplyMediaInputIndex }

::= { finSupplyMediaInputTable 1 }

FinSupplyMediaInputEntry ::= SEQUENCE {

finSupplyMediaInputIndex            Integer32,

finSupplyMediaInputDeviceIndex      Integer32,

finSupplyMediaInputSupplyIndex      Integer32,

finSupplyMediaInputType             PrtInputTypeTC,

finSupplyMediaInputDimUnit          PrtCapacityUnitTC,

finSupplyMediaInputMediaDimFeedDir  Integer32,

finSupplyMediaInputMediaDimXFeedDir Integer32,

finSupplyMediaInputStatus           PrtSubUnitStatusTC,

finSupplyMediaInputMediaName        OCTET STRING,

finSupplyMediaInputName             DisplayStrin

finSupplyMediaInputDescription      DisplayStrin

finSupplyMediaInputSecurity         PresentOnOff,

finSupplyMediaInputMediaWeight      Integer32,

finSupplyMediaInputMediaThickness   Integer32,

finSupplyMediaInputMediaType        DisplayStrin

}

finSupplyMediaInputIndex OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique value used by a finisher to identify this supply

media input subunit.  Although these values may change

due to a major reconfiguration of the finisher (e.g. the

addition of new supply media input sources to the

finisher), values are normally expected to remain stable

across successive power cycles."

::= { finSupplyMediaInputEntry 1 }

finSupplyMediaInputDeviceIndex OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of finDeviceIndex corresponding to the finishing

device subunit with which this finisher media supply is

associated."

::= { finSupplyMediaInputEntry 2 }

finSupplyMediaInputSupplyIndex OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The value of finSupplyIndex corresponding to the finishing

supply subunit with which this finisher media supply is

associated."

::= { finSupplyMediaInputEntry 3 }

finSupplyMediaInputType OBJECT-TYPE

SYNTAX      PrtInputTypeTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The type of technology (discriminated primarily according

to the feeder mechanism type) employed by the input

subunit."

::= { finSupplyMediaInputEntry 4 }

finSupplyMediaInputDimUnit OBJECT-TYPE

SYNTAX      PrtCapacityUnitTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unit of measure for specifying dimensional values for

this input device."

::= { finSupplyMediaInputEntry 5 }

finSupplyMediaInputMediaDimFeedDir OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object provides the value of the dimension in the

feed direction of the media that is placed or will be

placed in this input device.  Feed dimension measurements

are taken parallel to the feed direction of the device and

measured in finSupplyMediaInputDimUnits.  If this input

device can reliably sense this value, the value is sensed

and is read-only access. Otherwise the value is read-write

access and may be written by management or control panel

applications. The value (-1) means other and specifically

indicates that this device places no restrictions on this

parameter. The value (-2) indicates unknown. "

::= { finSupplyMediaInputEntry 6 }

finSupplyMediaInputMediaDimXFeedDir OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object provides the value of the dimension across the

feed direction of the media that is placed or will be

placed in this input device.  The cross feed direction is

ninety degrees relative to the feed direction on this

device and measured in finSupplyMediaInputDimUnits.  If

this input device can reliably sense this value, the value

is sensed and is read-only access. Otherwise the value is

read-write access and may be written by management or

control panel applications. The value (-1) means other and

specifically indicates that this device places no

restrictions on this parameter. The value (-2) indicates

unknown. "

::= { finSupplyMediaInputEntry 7 }

finSupplyMediaInputStatus OBJECT-TYPE

SYNTAX      PrtSubUnitStatusTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"This value indicates the current status of this input

device."

DEFVAL      { 5 }      -- unknown

::= { finSupplyMediaInputEntry 8 }

finSupplyMediaInputMediaName OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..63))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The name of the current media contained in this input

device. Examples are Print Job Output A, Triple A Billing

Statements or ISO standard names."

::= { finSupplyMediaInputEntry 9 }

finSupplyMediaInputName OBJECT-TYPE

SYNTAX      DisplayStrin))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The name assigned to this input subunit."

::= { finSupplyMediaInputEntry 10 }

finSupplyMediaInputDescription OBJECT-TYPE

SYNTAX      DisplayStrin5))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A free form text description of this input subunit in the

localization specified by prtGeneralCurrentLocalization."

::= { finSupplyMediaInputEntry 11 }

finSupplyMediaInputSecurity OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if this subunit has some security associated

with it."

::= { finSupplyMediaInputEntry 12 }

finSupplyMediaInputMediaWeight OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The weight of the media associated with this Input device

in grams per meter squared.  The value (-1) means other

and specifically indicates that the device places no

restriction on this parameter.  The value (-2) means

unknown.  This object can be used to calculate the weight

of individual pages processed by the document finisher.

This value, when multiplied by the number of pages in a

finished set, can be used to calculate the weight of a set

before it is inserted into a mailing envelope."

::= { finSupplyMediaInputEntry 13 }

finSupplyMediaInputMediaThickness OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"This object identifies the thickness of the input media

processed by this document input subunit measured in

micrometers.  This value may be used by devices (or

operators) to set up proper machine tolerances for the

feeder operation.  The value (-2) indicates that the media

thickness is unknown or not used in the setup for this

input subunit."

::= { finSupplyMediaInputEntry 14 }

finSupplyMediaInputMediaType OBJECT-TYPE

SYNTAX      DisplayStrin))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"The name of the type of the type of medium associated with

this input subunit.  Valid values are standardized strings

from ISO 10175 (DPA) and ISO 10180 (SPDL) which are:

stationary, transparency, envelope, envelope-plain,

envelope window, continuous-long, continuous-short,

tab-stock, labels, multi-layer."

::= { finSupplyMediaInputEntry 15 }

-- Finisher Device Attribute Group (Mandatory)

--

-- A finisher device subunit may have one or more parameters that

-- cannot be specified by any other objects in the MIB.  The

-- Device Attribute group facilitates the definition of these

-- parameters.  The objects which define the attributes are

-- read-write, to allow both Set and Get operations.

--

-- At least one table entry must exist for each finisher device defined

-- by the MIB.  If no other entry is possible for a finisher device, the

-- deviceName(3) attribute MUST be returned.

finDeviceAttribute  OBJECT IDENTIFIER  ::=  { printmib 33 }

finDeviceAttributeTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinDeviceAttributeEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The attribute table defines special parameters that are

applicable only to a minority of the finisher devices.

An attribute table entry is used, rather than unique

objects, to minimize the number of MIB objects and to

allow for expansion without the addition of MIB objects.

Each finisher device is represented by a separate row

in the device subunit attribute table."

::= { finDeviceAttribute 1 }

finDeviceAttributeEntry OBJECT-TYPE

SYNTAX     FinDeviceAttributeEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Each entry defines a finisher function parameter that

cannot be represented by an object in the finisher

device subunit table."

INDEX  { hrDeviceIndex, finDeviceIndex,

finDeviceAttributeTypeIndex,

finDeviceAttributeInstanceIndex }

::= { finDeviceAttributeTable 1 }

FinDeviceAttributeEntry ::= SEQUENCE {

finDeviceAttributeTypeIndex        FinAttributeTypeTC,

finDeviceAttributeInstanceIndex    Integer32,

finDeviceAttributeValueAsInteger   Integer32,

finDeviceAttributeValueAsOctets    OCTET STRING

}

finDeviceAttributeTypeIndex OBJECT-TYPE

SYNTAX      FinAttributeTypeTC

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Defines the attribute type represented by this row."

::= { finDeviceAttributeEntry 1 }

finDeviceAttributeInstanceIndex OBJECT-TYPE

SYNTAX      Integer32 (0..65535)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index that allows the discrimination of an attribute

instance when the same attribute occurs multiple times for

a specific instance of a finisher function.  The value of

this index shall be 1 if only a single instance of the

attribute occurs for the specific finisher function."

::= { finDeviceAttributeEntry 2 }

finDeviceAttributeValueAsInteger OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Defines the integer value of the attribute.  The value of

the attribute is represented as an integer if the

finAttributeTypeTC description for the attribute has the

tag 'INTEGER:'.

Depending upon the attribute enum definition, this object

may be either an integer, a counter, an index, or an enum.

Attributes for which the concept of an integer value is

not meaningful SHALL return a value of -1 for this

attribute."

DEFVAL      { -2 }      -- unknown

::= { finDeviceAttributeEntry 3 }

finDeviceAttributeValueAsOctets OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..63))

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Contains the octet string value of the attribute.  The

value of the attribute is represented as a string if the

finAttributeTypeTC description for the attribute has the

tag 'OCTETS:'.

Depending upon the attribute enum definition, this object

may be either a coded character set string (text) or a

binary octet string.  Attributes for which the concept of

an octet string value is not meaningful SHALL contain a

zero length string."

DEFVAL      { ''H }      -- empty string

::= { finDeviceAttributeEntry 4 }

-- Conformance Information

-- compliance statements

finMIBCompliance MODULE-COMPLIANCE

STATUS  current

DESCRIPTION

"The compliance statement for agents that implement the

finisher MIB."

MODULE -- this module

MANDATORY-GROUPS { finDeviceGroup, finSupplyGroup,

finDeviceAttributeGroup }

OBJECT      finDevicePresentOnOff

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finDeviceMaxCapacity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finDeviceCurrentCapacity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMaxCapacity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyCurrentLevel

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMediaInputMediaDimFeedDir

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMediaInputMediaDimXFeedDir

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMediaInputMediaName

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMediaInputName

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMediaInputSecurity

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMediaInputMediaWeight

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMediaInputMediaThickness

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finSupplyMediaInputMediaType

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finDeviceAttributeValueAsInteger

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      finDeviceAttributeValueAsOctets

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

::= { prtMIBConformance 3 }

finMIBGroups      OBJECT IDENTIFIER ::= { prtMIBConformance 4 }

finDeviceGroup OBJECT-GROUP

OBJECTS { finDeviceType, finDevicePresentOnOff,

finDeviceCapacityUnit, finDeviceMaxCapacity,

finDeviceCurrentCapacity, finDeviceAssociatedMediaPaths,

finDeviceAssociatedOutputs, finDeviceStatus,

finDeviceDescription }

STATUS  current

DESCRIPTION

"The finisher device group."

::= { finMIBGroups 1 }

finSupplyGroup OBJECT-GROUP

OBJECTS { finSupplyDeviceIndex, finSupplyClass, finSupplyType,

finSupplyDescription, finSupplyUnit, finSupplyMaxCapacity,

finSupplyCurrentLevel, finSupplyColorName }

STATUS  current

DESCRIPTION

"The finisher supply group."

::= { finMIBGroups 2 }

finSupplyMediaInputGroup OBJECT-GROUP

OBJECTS { finSupplyMediaInputDeviceType,

finSupplyMediaInputSupplyIndex, finSupplyMediaInputType,

finSupplyMediaInputDimUnit,

finSupplyMediaInputMediaDimFeedDir,

finSupplyMediaInputMediaDimXFeedDir,

finSupplyMediaInputStatus, finSupplyMediaInputMediaName,

finSupplyMediaInputName, finSupplyMediaInputDescription,

finSupplyMediaInputSecurity,

finSupplyMediaInputMediaWeight,

finSupplyMediaInputMediaThickness,

finSupplyMediaInputMediaType }

STATUS  current

DESCRIPTION

"The finisher supply, media input group."

::= { finMIBGroups 3 }

finDeviceAttributeGroup OBJECT-GROUP

OBJECTS { finDeviceAttributeValueAsInteger,

finDeviceAttributeValueAsOctets }

STATUS  current

DESCRIPTION

"The finisher device attribute group."

::= { finMIBGroups 4 }

END

# 8.  REFERENCES

[PRTMIB]  The Printer MIB, RFC 1759, IETF standards track document.

[RFC2119] Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119, March 1997.

[LMO] Large Mailing Operations Specification, DMTF.  See

http://www.dmtf.org/tech/apps.html

[DPA] ISO/IEC 10175 Document Printing Application (DPA).  See

ftp://ftp.pwg.org/pub/pwg/dpa/

[IPP] Internet Printing Protocol/1.0: Model and Semantics, work

in progress on the IETF standards track.  See draft-ietf-ipp-

model-09.txt.

# 9.  SECURITY CONSIDERATIONS

The Finisher MIB specifies a database and not necessarily a protocol

for accessing the database. With regards to the security of the

information within the database, it is anticipated that the primary

vehicle for accessing this data will be through the use of the Simple

Network Management Protocol (SNMP). SNMP Version 1 has known security

vulnerabilities due to the fact that only a "clear text" community

name is used for accessing MIB objects within a particular entity.

SNMPv2 attempted to provide additional security features but is still

not considered to be an adequate solution.  SNMP Version 3 provides

much more robust security mechanisms with regards to with support

authentication, as well as privacy.

# 10.  FULL COPYRIGHT STATEMENT

Copyright (C) The Internet Society (1998).  All Rights Reserved.

This document and translations of it may be copied and furnished to

others, and derivative works that comment on or otherwise explain it

or assist in its implementation may be prepared, copied, published

and distributed, in whole or in part, without restriction of any

kind, provided that the above copyright notice and this paragraph are

included on all such copies and derivative works.  However, this

document itself may not be modified in any way, such as by removing

the copyright notice or references to the Internet Society or other

Internet organizations, except as needed for the purpose of

developing Internet standards in which case the procedures for

copyrights defined in the Internet Standards process must be

followed, or as required to translate it into languages other than

English.

The limited permissions granted above are perpetual and will not be

revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an

"AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING

TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING

BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION

HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

# 11.  AUTHORS

This document was created with significant contributions from the

following individuals.

Ron Bergman (Editor)

Dataproducts Corp.

1757 Tapo Canyon Road

Simi Valley, CA 93063-3394

Phone: 805-578-4421

Fax:  805-578-4001

Email: rbergman@dpc.com

Harry Lewis (chairman)

IBM Corporation

6300 Diagonal Hwy

Boulder, CO 80301

Phone: (303) 924-5337

Fax:   (303) 924-4662

Email: harryl@us.ibm.com

Send comments to the Printer Working Group (PWG) using the Finisher

MIB Project (FIN) Mailing List:  fin@pwg.org

Implementers of this specification are encouraged to join this email

distribution list in order to participate in any discussions of

clarification issues and review registration proposals for

additional attributes and enum values.

For further information, access the PWG web page under "FIN":

http://www.pwg.org/

Other Participants:

Chuck Adams - Tektronix

Carlos Becerra - HP

Andy Davidson - Tektronix

Mabry Dozier - QMS

Lee Farrell - Canon

Jennifer Gattis - Duplo USA

Paul Gloger - Xerox

Richard Hart - Digital

Tom Hastings - Xerox

Scott Isaacson - Novell

David Kellerman - Northlake Software

Henrik Holst - i-data International

Rick Landau - Digital

Jay Martin - Underscore

Gary Padlipski - Xerox

Kevin Palmer - Duplo USA

Bob Pentecost - HP

Stuart Rowley - Kyocera

Yuki Sacchi - Japan Computer Industry

Philip Thambidunai - Okidata

William Wagner - DPI/Osicom

Chris Wellens - Interworking Labs

Don Wright - Lexmark

Lloyd Young - Lexmark