INTERNET-DRAFT                                               Ron Bergman

Dataproducts Corp.

Harry Lewis

IBM Corp.

March 5, 1998

Printer Finishing MIB

<draft-ietf-printmib-finishing-00.txt>

Expires September 5, 1998

Status of this Memo

This document is an Internet-Draft.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six

months and may be updated, replaced, or obsoleted by other

documents at any time.  It is inappropriate to use Internet-Drafts

as reference material or to cite them other than as "work in

progress".

To learn the current status of any Internet-Draft, please check the

"1id-abstracts.txt" listing contained in the Internet-Drafts Shadow

Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe),

munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or

ftp.isi.edu (US West Coast).

Abstract

This document defines a printer industry standard SNMP MIB for the

management of printer finishing device subunits.  The finishing

device subunits applicable to this MIB are an integral part of the

Printer System.  This MIB does not apply to a Finisher Device that

is external to a Printer System.

The Finisher MIB is defined as an extension of the Printer MIB

[PrtMIB] and it is expected that the information defined in this

document will be incorporated into a future update of the Printer

MIB.

TABLE OF CONTENTS

1.0  INTRODUCTION

1.1  Scope

1.2  Rational

2.0  TERMINOLOGY

3.0  FINISHER SUBUNITS INTEGRATED INTO THE PRINTER MODEL:

4.0  PRINTER FINISHING MIB SPECIFICATION

5.0  REFERENCES

6.0  AUTHORS

# 1	INTRODUCTION

This document describes an SNMP Management Information Base (MIB) to provide for the management of in-line post-processing in a fashion that is currently provided for printers, using the Printer MIB [PrtMIB].  The Printer Finishing MIB includes the following features:

- Provides the status of the finishing device.

- Manages the features and configuration of the finishing device.

- Enables and disables the finishing operations.

- Allows unsolicited status from the finishing device.

## 1.1	Scope

This document provides a robust set of finishing devices, features, and functions, based upon today's state of the art of in-line finishing.  Since finishing typically accompanies higher speed network printers and copiers, in contrast to simple desktop devices, no attempt is made to limit the scope to "bare minimum".  On the other hand, the Printer Finishing MIB does not duplicate the production mail preparation, custom insertion, franking, and reprints that are covered by the DMTF Large Mailing Operations standard [LMO].

Information supplied by the Printer Finishing MIB may be utilized by printer and finisher management applications engaged in monitoring status and managing configuration, and also used by print and finishing submission applications which are engaged in:

- print-file-level finishing operations that are applied to a

complete print file,

- document-level finishing operations that are applied individually

to each document in the print file,

- document-level finishing operations that are applied to a selected

document in the print file.

Note that not all combinations of finishing operations are compatible.  Compatible combinations of finishing operations are device specific.

## 1.2	Rational

The Printer MIB [PrtMIB] is now successfully deployed in a large segment of the network printer market.  SNMP and/or HTTP enabled printers and software management applications are growing in numbers.

There is an increase in the availability of network printers and copiers that include in-line finishing operations.  Thus a well defined and ordered set of finishing objects is now necessary for printer management.

The printer model defined in the Printer MIB includes finishing operations and the MIB was designed to later incorporate finisher objects or to be referenced by a future Finisher MIB.

# 2	TERMINOLOGY

Where appropriate, the Printer Finishing MIB will conform to the terminology, syntax, and semantics from the DMTF Large Mailing Operations standard [LMO], the Internet Printing Protocol [IPP], and/or the ISO Document Printing Application [DPA].

Finisher Input:  An input tray on the finisher and not otherwise associated with the printer.  An example of a finisher input is a tray that holds finishing "inserts".

Finisher Output:  The output of the finisher.  Because processing is in-line, the finisher outputs are a direct extension of the set of printer outputs.

Finishing Operation Axis:  Defined by DPA as the axis to which some finishing operations are applied to or referenced from.  An example is the axis for a fold operation.

Finishing Axis Offset:  The offset from a finishing operation axis at which the finishing operation takes place or is applied.

# 3	FINISHER SUBUNITS INTEGRATED INTO THE PRINTER MODEL:

The Printer Finisher Device subunits receive media from one or more Printer Media Path subunits and deliver the media to one or more Printer Output subunits after the completion of the finishing operations.  The Printer Model, as described in the Printer MIB [PRTMIB], is modified adding the finisher subunit(s) and finisher supplies as follows:

+-----+                 +------+                +--------+    +------+

|     |                 |      |                |        |    |      |

+-----+ |    +-----+    +------+ |   +------+   +--------+ |  +------+ |

|Input|-+  +------+|    |Marker|-+  +------+|   |Finisher|-+  |Output|-+

|     |===>|      |+<==>|      |<==>|      |+==>|        |===>|      |

+-----+    +-+  +-+     +------+    +-+  +-+    +--------+    +------+

\         |  ||                    |  ||         ||

\        |  ||                    |  ||         ||

\       |  ||                    |  ||    +----------+

+-------+  |  |+--------------------|  ||    | Finisher |-+

|       |  |  +---------------------+  ||    | Supplies | |

+-------+ |  |         Media Path        |+    +----------+ |

| Media |-+  +---------------------------+       |          |

|(opt.) |                                        +----------+

+-------+

ISSUE:  I made the finisher use the input and marker supplies tables in the Printer MIB, so that the Finisher Supplies isn't really a separate subunit.  Ok?  If so, then shouldn't we remove the Finisher Supplies box above?

# 4	THE ATTRIBUTE MECHANISM

Attributes are similar to information objects, except that attributes are identified by an enum, instead of an OID, so that attributes may be registered without requiring a new MIB.  Also an implementation that does not have the functionality represented by the attribute can omit the attribute entirely, rather than having to return a distinguished value.  The agent is free to materialize an attribute in the finDeviceAttributeTable as soon as the agent is aware of the value of the attribute.

The agent materializes finishing subunit attributes in a four-indexed finDeviceAttributeTable:

- hrDeviceIndex - which device in the host

- finDeviceIndex - which finisher subunit in the printer device

- finDeviceAttributeTypeIndex - which attribute

- finDeviceAttributeInstanceIndex - which attribute instance for those attributes that can have multiple values per finishing subunit.

### 4.1.1	Conformance of Attribute Implementation

An agent SHALL implement any attribute if (1) the device supports the functionality represented by the attribute and (2) the information is available to the agent.  The agent MAY create the attribute row in the finDeviceAttributeTable when the information is available or MAY create the row earlier with the designated 'unknown' value appropriate for that attribute.  See next section.

If the server or device does not implement or does not provide access to the information about an attribute, the agent SHOULD NOT create the corresponding row in the finDeviceAttributeTable.

### 4.1.2	Useful, 'Unknown', and 'Other' Values for Objects and Attributes

Some attributes have a 'useful' Integer32 value, some have a 'useful' OCTET STRING value, some MAY have either or both depending on implementation, and some MUST have both.  See the FinAttributeTypeTC textual convention for the specification of each attribute.

SNMP requires that if an object cannot be implemented because its values cannot be accessed, then a compliant agent SHALL return an SNMP error in SNMPv1 or an exception value in SNMPv2.  However, this MIB has been designed so that 'all' objects can and SHALL be implemented by an agent, so that neither the SNMPv1 error nor the SNMPv2 exception value SHALL be generated by the agent.  This MIB has also been designed so that when an agent materializes an attribute, the agent SHALL materialize a row consisting of both the finDeviceAttributeValueAsInteger and finDeviceAttributeValueAsOctets objects.

In general, values for objects and attributes have been chosen so that a management application will be able to determine whether a 'useful', 'unknown', or 'other' value is available.  When a useful value is not available for an object that agent SHALL return a zero-length string for octet strings, the value 'unknown(2)' for enums, a '0' value for an object that represents an index in another table, and a value '-2' for counting integers.

Since each attribute is represented by a row consisting of both the finDeviceAttributeValueAsInteger and finDeviceAttributeValueAsOctets MANDATORY objects, SNMP requires that the agent SHALL always create an attribute row with both objects specified.  However, for most attributes the agent SHALL return a "useful" value for one of the objects and SHALL return the 'other' value for the other object.  For integer-only attributes, the agent SHALL always return a zero-length string value for the finDeviceAttributeValueAsOctets object.  For octet string only attributes, the agent SHALL always return a '-1' value for the finDeviceAttributeValueAsInteger object.

### 4.1.3	Data Sub-types and Attribute Naming Conventions

Many attributes are sub-typed to give a more specific data type than Integer32 or OCTET STRING.  The data sub-type of each attribute is indicated on the first line(s) of the description.  Some attributes have several different data sub-type representations.  When an attribute has both an Integer32 data sub-type and an OCTET STRING data sub-type, the attribute can be represented in a single row in the finDeviceAttributeTable.  In this case, the data sub-type name is not included as the last part of the name of the attribute.  When the data sub-types cannot be represented by a single row in the finDeviceAttributeTable, each such representation is considered a separate attribute and is assigned a separate name and enum value.  For these attributes, the name of the data sub-type is the last part of the name of the attribute.  For example, deviceAssociatedSupplyIndex(20) is an index.

### 4.1.4	Single-Value (Row) Versus Multi-Value (MULTI-ROW) Attributes

Most attributes SHALL have only one row per finishing subunit.  However, a few attributes can have multiple values per finishing subunit, where each value is a separate row in the finDeviceAttributeTable.  Unless indicated with 'MULTI-ROW:' in the FinAttributeTypeTC description, an agent SHALL ensure that each attribute occurs only once in the finDeviceAttributeTable for a finishing subunit.  Most of the 'MULTI-ROW' attributes do not allow duplicate values, i.e., the agent SHALL ensure that each value occurs only once for a finishing subunit.  Only if the specification of the 'MULTI-ROW' attribute also says "There is no restriction on the same xxx occurring in multiple rows" can the agent allow duplicate values to occur for a single finishing subunit.

### 4.1.5	Linked MULTI-ROW values

Some multi-row attributes are intended to go together.  Thus a set of value instances represent a single instance.  For example, the puncher attributes indicate the shape, max size, min size, and shape of the various holes that the puncher can produce.  So the 1st set of values could represent one kind of hole, and the 2nd set of values a another kind of hole, etc.

### 4.1.6	Index Value Attributes

A number of attributes are indexes in other tables.  Such attribute names end with the word 'Index'.  If the agent has not (yet) assigned an index value for a particular index attribute for a finishing subunit, the agent SHALL either: (1) return the value 0 or (2) not add this attribute to the finDeviceAttributeTable until the index value is assigned.  In the interests of brevity, the semantics for 0 is specified once here and is not repeated for each index attribute specification and a DEFVAL of 0 is indicated.

# 5	PRINTER FINISHING MIB SPECIFICATION

Finisher-MIB DEFINITIONS ::= BEGIN

IMPORTS

MODULE-IDENTITY, OBJECT-TYPE, experimental,

Integer32                                     FROM SNMPv2-SMI

TEXTUAL-CONVENTION                             FROM SNMPv2-TC

MODULE-COMPLIANCE, OBJECT-GROUP              FROM SNMPv2-CONF

hrDeviceIndex                         FROM HOST-RESOURCES-MIB

PrtSubUnitStatusTC, PrtInputTypeTC,

PrtMarkerSuppliesSupplyUnitTC,

PrtCapacityUnitTC, prtOutputIndex,

PrtMarkerSuppliesClassTC, PresentOnOff,

PrtMediaPathIndex                            FROM Printer-MIB

finisherMIB MODULE-IDENTITY

LAST-UPDATED "9803050000Z"

ORGANIZATION "IETF Printer MIB Working Group"

CONTACT-INFO

"Ron Bergman

Dataproducts Corp.

1757 Tapo Canyon Road

Simi Valley, CA 91063-3394

rbergma@dpc.com

Send comments to the printmib WG using the Finisher MIB

Project (FIN) Mailing List:  fin@pwg.org

For further information, access the PWG web page under 'FIN':

http://www.pwg.org/

Implementers of this specification are encouraged to join the

fin mailing list in order to participate in discussions on any

clarifications needed and registration proposals being reviewed

in order to achieve consensus."

DESCRIPTION

"The MIB module for management of printer finisher units.

The Finisher MIB is an extension of the Printer MIB."

::= { mib-2 43 }

-- Textual conventions for this MIB module

FinDeviceTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined finishing device subunit types."

"

SYNTAX       INTEGER {

other(1),

unknown(2),

stitcher(3),

folder(4),

binder(5),

trimmer(6),

dieCutter(7),

puncher(8),

perforater(9),

slitter(10),

separationCutter(11),

imprinter(12),

wrapper(13),

bander(14)

}

FinAttributeTypeTC  ::= TEXTUAL-CONVENTION

STATUS     current

DESCRIPTION

"This textual convention defines a set of enums for use in

the finDeviceAttributeTable.  The data type tag definitions

'INTEGER:' or 'OCTETS', indicate if the attribute can be

represented using the object finDeviceAttributeAsInteger or

the object finDeviceAttributeAsOctets, respectively.  In some

cases, a choice between the two data types is possible and for

a few attributes both objects may be required at the same time

to properly present the value.

The attribute types defined at the time of completion of this

specification are:"

finAttributeTypeIndex	Data type

---------------------	---------

other(1),	Integer32

AND/OR
	OCTET STRING (SIZE(0..63))
INTEGER: and/or OCTETS:  An attribute that is not currently approved and registered.

-- Generic Finisher subunit attributes that apply to all

-- Finisher subunit types (3..):

deviceCapacityUnit(3), 	PrtCapacityUnitTC
INTEGER:  The unit of measure for specifying the capacity of this finisher device subunit.

deviceMaxCapacity(4),	Integer32
INTEGER:  The maximum capacity of this finisher device subunit in deviceCapacityUnits.  If the device can reliably sense this value, the value is sensed by the finisher device and is read-only: otherwise the value may be written by a management or control console application.  The value (-1) means other and specifically indicates that the device places no restrictions on this parameter.  The value (-2) means unknown.

deviceCurrentCapacity(5),	Integer32
INTEGER:  The current capacity of this finisher device subunit in deviceCapacityUnits.  If the device can reliably sense this value, the value is sensed by the finisher and is read-only: otherwise the value may be written by a management or control console application.  The value (-1) means other and specifically indicates that the device places no restrictions on this parameter.  The value (-2) means unknown.

deviceAssociatedMediaPaths(6),	Integer32
INTEGER:  Indicates the media paths which can supply media for this finisher device.  The value of this object is a bit map with each position representing the value of a prtMediaPathIndex.  For a media path that can be a source for this finisher device subunit, the bit position equal to 2 raised to the power of one less than the value of prtMediaPathIndex will be set.

For example, a value of hexadecimal 21 indicates the media paths with a prtMediaPathIndex of 1 or 6 will supply media to this  finisher device subunit.

The most significant bit position SHALL not be used, which allows a maximum of 31 media paths to be defined.

ISSUE: Why not just have a multi-valued attribute that is each prtMediaPathIndex value?  Then no limit and is more like all other associations, instead of this bit mask.

deviceAssociatedOutputs(7),	Integer32
INTEGER:  MULTI-VALUE:  Indicates the printer output subunits this finisher device subunit services.  The value of this object is a bit map with each position representing the value of a prtOutputIndex.  For an output subunit that is serviced by this finisher device subunit, the bit position equal to 2 raised to the power of one less than the value of prtOutputIndex will be set.

For example, a value of hexadecimal 49 indicates the output subunits with a prtOutputIndex of 1, 4 or 7 can be serviced by this  finisher device subunit.

The most significant bit position SHALL not be used, which allows a maximum of 31 output subunits to be defined.

If more than 31 are needed, use the next attribute value.

ISSUE: Why not just have a multi-valued attribute that is each prtMediaPathIndex value?  Then no limit and is more like all other associations, instead of this bit mask.

deviceDescription(8),	OCTET STRING(0..255)
OCTETS:  A free form text description of this device subunit in the localization specified by prtGeneralCurrentLocalization.

deviceName(9),	OCTET STRING(0..63)
OCTETS:  The name assigned to this finisher device subunit.

deviceVendorName(10),	OCTET STRING(0..63)
OCTETS:  The vendor name of this finisher device subunit.

deviceModel(11),	OCTET STRING(0..63)
OCTETS:  The model name of this finisher device subunit."

deviceVersion(12),	OCTET STRING(0..63)
OCTETS:  The version string for this finisher device subunit.

deviceSerialNumber(13),	OCTET STRING(0..63)
OCTETS:  The serial number assigned to this finisher device subunit.

-- Finisher Supply Attributes

--

-- A finisher subunit may have one or more supplies associated with

-- it.  For example a finisher may use both binding tape and

-- stitching wire supplies. A finisher may also have more than one

-- source for a given type of supply e.g. multiple supply sources of

-- ink for imprinters.

--

-- This finisher subunit references each marker supply row in the

-- Printer MIB with which this finisher subunit is associated.

deviceAssociatedSupplyIndex(20)	Integer32 (0..2147483647)
INTEGER:  MULTI-ROW:  The index in the prtMarkerSuppliesTable in the Printer MIB [PRTMIB] that is associated with this finisher subunit.  The marker supplies table contains the following objects:

prtMarkerSuppliesIndex	Integer32,

prtMarkerSuppliesMarkerIndex	Integer32,

prtMarkerSuppliesColorantIndex Integer32,

prtMarkerSuppliesClass	PrtMarkerSuppliesClassTC,

prtMarkerSuppliesType	PrtMarkerSuppliesTypeTC,

prtMarkerSuppliesDescription	OCTET STRING,

prtMarkerSuppliesSupplyUnit PrtMarkerSuppliesSupplyUnitTC,

prtMarkerSuppliesMaxCapacity	Integer32,

prtMarkerSuppliesLevel	Integer32

-- Finisher Supply Media Input Attributes

--

-- A finisher subunit may have one or more associated supply media

-- inputs.  Each entry in this table defines an input for a

-- supply media type such as inserts, covers, etc.

--

-- This finisher subunit references each input row in the Printer

-- MIB with which this finisher subunit is associated.

deviceAssociatedInputIndex(21)	Integer32 (0..2147483647)
INTEGER:  MULTI-ROW:  The index in the prtInputTable in the Printer MIB [PRTMIB] that is associated with this finisher subunit.  The input table contains the following objects:

prtInputIndex	Integer32,

prtInputTypePrt	PrtInputTypeTC,

prtInputDimUnit	PrtMediaUnitTC,

prtInputMediaDimFeedDirDeclared Integer32,

prtInputMediaDimXFeedDirDeclared Integer32,

prtInputMediaDimFeedDirChosen	Integer32,

prtInputMediaDimXFeedDirChosen Integer32,

prtInputCapacityUnit	PrtCapacityUnitTC,

prtInputMaxCapacity	Integer32,

prtInputCurrentLeve	Integer32,

prtInputStatus	PrtSubUnitStatusTC,

prtInputMediaName	OCTET STRING,

prtInputNameOCTET	STRING,

prtInputVendorName	OCTET STRING,

prtInputModel	OCTET STRING,

prtInputVersion	OCTET STRING,

prtInputSerialNumber	OCTET STRING,

prtInputDescription	OCTET STRING,

prtInputSecurity	PresentOnOff,

prtInputMediaWeight	Integer32,

prtInputMediaType	OCTET STRING,

prtInputMediaColor	OCTET STRING,

prtInputMediaFormParts	Integer32,

prtInputMediaLoadTimeout	Integer32,

prtInputNextIndex	Integer32

-- Finisher type-specific attributes:

-- stitcher attributes (30..):

stitchingType(30),	FinStitchingTypeTC
INTEGER:  MULTI-VALUE:  The stitching operations supported by this finisher subunit.

-- folder attributes (40..):

foldingType(40),	FinFoldingTypeTC
INTEGER:  The folding operations supported by this finisher subunit.

-- binder attributes (50..):

bindingType(50),	FinBindingTypeTC
INTEGER:  The binding operations supported by this finisher subunit.

-- trimmer attributes (60..):

-- die cutter attributes (70..):

-- puncher attributes (80..)  These attributes are linked, so that the ith value of each attribute go together:

punchHoleType(80),	FinPunchHoleTypeTC
INTEGER:  MULTI-VALUE:  The shapes of the punched holes supported by this finisher subunit.

punchHoleSizeMaxDim(81),	Integer32
INTEGER:  MULTI-VALUE:  Defines the size of the punched hole in the maximum dimension.  This dimension shall always be measured parallel to either the long edge or the short edge of the media and the maximum dimension will always be measured 90 degrees from the minimum dimension.  The minimum and maximum dimensions may be identical.

punchHoleSizeMinDim(82),	Integer32
INTEGER:  Defines the size of the punched hole in the minimum dimension.  This dimension shall always be measured parallel to either the long edge or the short edge of the media and the minimum dimension will always be measured 90 degrees from the maximum dimension.  The minimum and maximum dimensions may be identical.

punchPattern(83),	FinPunchPatternTC
INTEGER:  MULTI-VALUE:  Defines the hole patterns produced by the punch operation.

-- perforater attributes (90..):

-- slitter attributes (100..):

slittingType(100),	FinSlittingTypeTC
INTEGER:  The slitting operations supported by this finisher subunit.

-- separation cutter attributes (110..):

-- imprinter attributes (120..):

-- wrapper attributes (130..):

wrappingType(130),	FinWrappingTypeTC
INTEGER:  Provides additional information regarding the wrapping operations supported by this finishing subunit.

-- bander attributes:

SYNTAX	INTEGER{

other(1),

deviceCapacityUnit(3),

deviceMaxCapacity(4),

deviceCurrentCapacity(5),

deviceAssociatedMediaPaths(6),

deviceAssociatedOutputs(7),

deviceDescription(8),

deviceName(9),

deviceVendorName(10),

deviceModel(11),

deviceVersion(12),

deviceSerialNumber(13),

stitchingType(30),

foldingType(40),

bindingType(50),

punchHoleType(80),

punchHoleSizeMaxDim(81),

punchHoleSizeMinDim(82),

punchPattern(83),

slittingType(100),

wrappingType(130),

}

FinStitchingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined stitching type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

staple(3),

stapleTopLeft(4),

stapleBottomLeft(5),

stapleTopRight(6),

stapleBottomRight(7),

saddleStitch(8),

edgeStitch(9),

stitch(10)

}

FinFoldingTypeTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined folding device operation enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

zFold(3),

halfFold(4),

letterFold(5)

}

FinBindingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined binding type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

bind(3),

tape(4),

plastic(5),

velo(6),

perfect(7),

spiral(8)

}

FinPunchHoleTypeTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined hole type punch operation enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

round(3),

oblong(4),

square(5),

rectangular(6),

star(7)

}

FinPunchPatternTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined hole pattern punch operation enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

punch(3),            --Pattern to be defined in other

--attributes

twoHoleUS(4),        --Letter top edge, 8.5 inch side

threeHoleUS(5),      --Letter/ledger left edge, 11 inch side

twoHoleMetric(6),    --A4/A3 left edge, 297 mm side

fourHoleMetric(7),   --A4/A3 left edge, 297 mm side

twentyTwoHoleUS(8),  --Letter/ledger left edge, 11 inch side

nineteenHoleUS(9)    --Letter/ledger left edge, 11 inch side

)

FinSlittingTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined slitting type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

slit(3),

slitAndSeparate(4),

slitAndMerge(5)

}

FinWrappingTypeTC ::= TEXTUAL-CONVENTION

--This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined wrapping device operation enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

wrap(3),

shrinkWrap(4),

paperWrap(5)

}

ISSUE:  Where is the following TC used?  Can we delete it?

FinOutputTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration.

STATUS       current

DESCRIPTION

"The defined output type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

removableBin(3),

unremovableBin(4),

continuousRollDevice(5),

mailbox(6),

continuousFanFold(7),

conveyer(8),

smartCart(9)

}

FinSupplyTypeTC ::= TEXTUAL-CONVENTION

-- This is a type 2 enumeration that is an extension to the

-- Printer MIB textual convention PrtMarkerSuppliesTypeTC.

-- ******************************************************************

-- ISSUE: Should this just define new enums added to the Printer MIB

--        or keep it separate?  If separate, should the duplicates

--        from the Printer MIB be eliminated?

-- ******************************************************************

STATUS       current

DESCRIPTION

"The defined finishing supply type enumerations."

SYNTAX       INTEGER {

other(1),

unknown(2),

toner(3),

wasteToner(4),

ink(5),

inkCartridge(6),

inkRibbon(7),

wasteInk(8),

opc(9),               --photo conductor

developer(10),

fuserOil(11),

solidWax(12),

ribbonWax(13),

wasteWax(14),

fuser(15),

coronaWire(16),

fuserOilWick(17),

cleanerUnit(18),

fuserCleaningPad(19),

transferUnit(20),

tonerCartridge(21),

fuserOiler(22),       --Supply types 3 to 22 are from the

--Printer MIB

water(23),

wasteWater(24),

glueWaterAdditive(25),

wastePaper(26),

bindingTape(27),

bandingTape(28),

stitchingWire(29),

shrinkWrap(30),

paperWrap(31),

staples(32),

inserts(33),

covers(34)

}

-- Finisher Device Group (Mandatory)

--

-- A printer may support zero or more finishing subunits.  A

-- finishing device subunit may be associated with one or more

-- output subunits and one or more media path subunits.

--

-- NOTE: The objects in this table could not have been made attributes

-- because the Printer MIB trap mechanism needs to be able to indicate

-- alerts in subunits which are rows in tables.

finDeviceTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinDeviceEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This table defines the finishing device subunits,

including information regarding possible configuration

options and the status for each finisher device subunit."

::= { finisherMIB 18 }

finDeviceEntry OBJECT-TYPE

SYNTAX      FinDeviceEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"There is an entry in the finishing device table for each

possible finisher operation."

INDEX  { hrDeviceIndex, finDeviceIndex }

::= { finDevicesTable 1 }

FinDeviceEntry ::= SEQUENCE {

finDeviceIndex                 Integer32,

finDeviceType                  FinDeviceTypeTC,

finDevicePresentOnOff          PresentOnOff,

finDeviceStatus                PrtSubUnitStatusTC

}

finDeviceIndex OBJECT-TYPE

SYNTAX      Integer32(0..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"A unique value used to identify a finisher operation.

Although these values may change due to a major

reconfiguration of the printer system (e.g. the addition

of new finishing operations), the values are normally

expected to remain stable across successive power cycles."

::= { finDeviceEntry 1 }

finDeviceType OBJECT-TYPE

SYNTAX      FinDeviceTypeTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Defines the type of finishing operation associated with this

table row entry."

::= { finDeviceEntry 2 }

finDevicePresentOnOff OBJECT-TYPE

SYNTAX      PresentOnOff

MAX-ACCESS  read-write

STATUS      current

DESCRIPTION

"Indicates if this finishing device subunit is available

and whether the device subunit is enabled."

::= { finDeviceEntry 3 }

finDeviceStatus OBJECT-TYPE

SYNTAX      PrtSubUnitStatusTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Indicates the current status of this finisher device

subunit."

::= { finDeviceEntry 4 }

"

"

"

"

"

"

"

"

""

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

"

""

""

""

""

""

"

"

"

"

"

"

"

"

"

"

"

"

-- Finisher Device Attribute Group (Mandatory)

--

-- A finisher device subunit may have one or more parameters that

-- cannot be specified by any other objects in the MIB.  The

-- Device Attribute group allows the definition of these

-- parameters.

finDeviceAttributeTable OBJECT-TYPE

SYNTAX     SEQUENCE OF FinDeviceAttributeEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"The attribute table defines special parameters that are

applicable only to a minority of the finisher devices.

An attribute table entry is used, rather than unique

objects, to minimize the number of MIB objects and to

allow for expansion without the addition of MIB objects.

Each finisher device is represented by a separate row

in the device subunit attribute table."

::= { finisherMIB 21 }

finDeviceAttributeEntry OBJECT-TYPE

SYNTAX     FinDeviceAttributeEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Each entry defines a finisher function parameter that

cannot be represented by an object in the finisher

device subunit table."

INDEX  { hrDeviceIndex, finDeviceIndex,

finDeviceAttributeTypeIndex }

::= { finDeviceAttributeTable 1 }

FinDeviceAttributeEntry ::= SEQUENCE {

finDeviceAttributeTypeIndex        FinAttributeTypeTC,

finDeviceAttributeInstanceIndex    Integer32,

finDeviceAttributeValueAsInteger   Integer32,

finDeviceAttributeValueAsOctets    OCTET STRING

}

finDeviceAttributeTypeIndex OBJECT-TYPE

SYNTAX      FinAttributeTypeTC

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Defines the attribute type represented by this row."

::= { finDeviceAttributeEntry 1 }

finDeviceAttributeInstanceIndex OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An index that allows the discrimination of an attribute

instance when the same attribute occurs multiple times for

a specific instance of a finisher function.  The value of

this index shall be 1 if only a single instance of the

attribute occurs for the specific finisher function."

::= { finDeviceAttributeEntry 2 }

finDeviceAttributeValueAsInteger OBJECT-TYPE

SYNTAX      Integer32

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Defines the integer value of the attribute.  The value of

the attribute is represented as an integer if the

FinAttributeTypeTC description for the attribute has the

tag 'INTEGER:'.

Depending upon the attribute enum definition, this object

may be either an integer, a counter, an index, or an enum.

Attributes for which the concept of an integer value is

not meaningful SHALL return a value of -1 for this

attribute."

::= { finDeviceAttributeEntry 3 }

finDeviceAttributeValueAsOctets OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..63))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Contains the octet string value of the attribute.  The

value of the attribute is represented as a string if the

FinAttributeTypeTC description for the attribute has the

tag 'OCTETS:'.

Depending upon the attribute enum definition, this object

may be either a coded character set string (text) or a

binary octet string.  Attributes for which the concept of

an octet string value is not meaningful SHALL contain a

zero length string."

::= { finDeviceAttributeEntry 4 }

END

# 6	REFERENCES

[PRTMIB]  The Printer MIB, RFC 1759, IETF standards track document.

[LMO] Large Mailing Operations Specification, DMTF.  See http://www.dmtf.org/tech/apps.html

[DPA] ISO/IEC 10175 Document Printing Application (DPA).  See

ftp://ftp.pwg.org/pub/pwg/dpa/

[IPP] Internet Printing Protocol/1.0: Model and Semantics, work

in progress on the IETF standards track.  See draft-ietf-ipp-model-

09.txt.

# 7	AUTHORS

This document was created with significant contributions from the following individuals.

Ron Bergman (Editor)

Dataproducts Corp.

1757 Tapo Canyon Road

Simi Valley, CA 93063-3394

Phone: 805-578-4421

Fax:  805-578-4001

Email: rbergman@dpc.com

Harry Lewis (chairman)

IBM Corporation

6300 Diagonal Hwy

Boulder, CO 80301

Phone: (303) 924-5337

Fax:   (303) 924-4662

Email: harryl@us.ibm.com

Send comments to the Printer Working Group (PWG) using the Finisher

MIB Project (FIN) Mailing List:  fin@pwg.org

For further information, access the PWG web page under "FIN":

http://www.pwg.org/

Other Participants:

Chuck Adams - Tektronix

Andy Davidson - Tektronix

Mabry Dozier - QMS

Lee Ferrel - Canon

Paul Gloger - Xerox

Richard Hart - Digital

Tom Hastings - Xerox

Scott Isaacson - Novell

David Kellerman - Northlake Software

Henrik Holst - i-data International

Rick Landau - Digital

Jay Martin - Underscore

Gary Padlipski - Xerox

Bob Pentecost - HP

Stuart Rowley - Kyocera

Yuki Sacchi - Japan Computer Industry

Philip Thambidunai - Okidata

William Wagner - DPI/Osicom

Chris Wellens - Interworking Labs

Don Wright - Lexmark

Lloyd Young - Lexmark