Internet Printing Protocol Working Group                    Tom Hastings

INTERNET DRAFT                                                     Xerox

Expires 6 January 2001                                      Ira McDonald

High North

6 July 2000

Internet Printing Protocol (IPP):

Notifications over SNMP via Job Monitoring MIB Traps

<draft-ietf-ipp-not-over-snmp-03.txt>

Copyright (C) The Internet Society (2000). All Rights Reserved.

Status of this Memo

This document is an Internet-Draft and is in full conformance with

all provisions of Section 10 of RFC2026.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

To view the list of Internet-Draft Shadow Directories, see

http://www.ietf.org/shadow.html.

Abstract

This document is a submission to the Internet Printing Protocol

Working Group of the Internet Engineering Task Force (IETF).

Comments should be submitted to the ipp@pwg.org mailing list.

This document proposes a mapping of IPP notifications over SNMP via

new service and job trap extensions defined for the Job Monitoring

MIB [RFC-2707].  This mapping may be used to deliver printer

notifications for any printer (not just IPP-capable ones) and also

job notifications for any job (not just ones submitted via IPP).

This document proposes (4) new object groups and (4) new SNMP traps

for addition to the Job Monitoring MIB [RFC 2707].  A working copy of

this proposal integrated into the original Job Monitoring MIB ASN.1

is at:

ftp://ftp.pwg.org/pub/pwg/ipp/new_NOT/jmp-trap-000706.mib

Hastings, McDonald            Expires 6 January 2001            [Page 1]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

Table of Contents

1.  Introduction ...............................................       3

1.1.  Terminology for Conformance ............................       3

2.  SNMP Network Management Framework ..........................       3

3.  Managed Object Mapping .....................................       4

3.1.  SNMP Mapping for IPP Notifications .....................       4

3.1.1.  SNMP Mapping for IPP Printer Events ................       4

3.1.2.  SNMP Mapping for IPP Job Events ....................       5

3.2.  Subscription for IPP Notifications .....................       6

3.2.1.  Subscription via IPP ...............................       6

3.2.2.  Subscription via SNMP ..............................       6

3.3.  Relationship to other MIBs .............................       7

3.3.1.  MIB-II (RFC 1213) ..................................       7

3.3.2.  Host Resources MIB (RFC 2790) ......................       7

3.3.3.  Printer MIB (RFC 1759) .............................       7

4.  Managed Object Definitions .................................       8

4.1.  Notification Definitions ...............................       8

4.1.1.  Service Basic Event - Trap .........................       8

4.1.2.  Job Basic Event - Trap .............................       9

4.1.3.  Job Completed Event - Trap .........................      11

4.1.4.  Job Progress Event - Trap ..........................      12

4.2.  Object Definitions .....................................      13

4.2.1.  Service Group - Objects ............................      13

4.2.2.  Service Event Group - Objects ......................      16

4.2.3.  Job Event Group - Objects ..........................      19

4.2.4.  Job Progress Group - Objects .......................      22

5.  IANA Considerations ........................................      24

6.  Internationalization Considerations ........................      24

7.  Security Considerations ....................................      24

8.  References .................................................      25

9.  Change Log .................................................      26

10.  Intellectual Property Notice ..............................      29

11.  Authors' Addresses ........................................      29

12.  Full Copyright Statement ..................................      30

Hastings, McDonald            Expires 6 January 2001            [Page 2]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

1.  Introduction

This document is a submission to the Internet Printing Protocol

Working Group of the Internet Engineering Task Force (IETF).

Comments should be submitted to the ipp@pwg.org mailing list.

This document proposes a mapping of IPP notifications over SNMP via

new service and job trap extensions defined for the Job Monitoring

MIB [RFC-2707].  This mapping may be used to deliver printer

notifications for any printer (not just IPP-capable ones) and also

job notifications for any job (not just ones submitted via IPP).

This document proposes (4) new object groups and (4) new SNMP traps

for addition to the Job Monitoring MIB [RFC 2707].  A working copy of

this proposal integrated into the original Job Monitoring MIB ASN.1

is at:

ftp://ftp.pwg.org/pub/pwg/ipp/new_NOT/jmp-trap-000706.mib

The IPP protocol [IPP-PRO] supports passive monitoring of IPP Printer

and Job objects, via client polling of IPP object attributes using

the 'Get-Printer-Attributes' and 'Get-Job-Attributes' operations.

This document proposes support for dynamic monitoring of IPP Printer

and Job objects via SNMP traps generated by the service provider

(server or device).

1.1.  Terminology for Conformance

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted per [RFC-2119].

2.  SNMP Network Management Framework

See:  Section 1.1 'SNMPv1', section 1.2 'SNMPv2', and section 1.3

'SNMPv3' of [RFC-2576].

Hastings, McDonald            Expires 6 January 2001            [Page 3]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

3.  Managed Object Mapping

3.1.  SNMP Mapping for IPP Notifications

3.1.1.  SNMP Mapping for IPP Printer Events

IPP Printer event attribute         Job Monitoring MIB object mapping

---------------------------         ---------------------------------

version-number                      [no mapping - not useful]

status-code                         [implicit in each generated trap]

notify-sequence-number              [request-id in SNMP trap header]

notify-charset                      [no mapping - strings are UTF-8]

notify-natural-language             [no mapping - no text bindings]

subscription-id                     [no mapping]

notify-subscribed-event             jmServiceEventNotifyEvent

notify-printer-uri                  jmServiceURI

printer-up-time                     [sysUpTime in SNMP trap bindings]

printer-current-time                [hrSystemDate in Host Res MIB]

notify-user-data                    [no mapping]

notify-text                         [no mapping]

printer-state                       jmServiceState

printer-state-reasons               jmServiceStateReasons

printer-is-accepting-jobs           [jmServiceStateReasons keyword]

Hastings, McDonald            Expires 6 January 2001            [Page 4]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

3.1.2.  SNMP Mapping for IPP Job Events

IPP Job event attribute             Job Monitoring MIB object mapping

-----------------------             ---------------------------------

version-number                      [no mapping - not useful]

status-code                         [implicit in each generated trap]

notify-sequence-number              [request-id in SNMP trap header]

notify-charset                      [no mapping - strings are UTF-8]

notify-natural-language             [no mapping - no text bindings]

subscription-id                     [no mapping]

notify-subscribed-event             jmJobEventNotifyEvent

notify-printer-uri                  jmServiceURI

job-id                              [via jmJobState OID instance]

printer-up-time                     [sysUpTime in SNMP trap bindings]

printer-current-time                [hrSystemDate in Host Res MIB]

notify-user-data                    [no mapping]

notify-text                         [no mapping]

job-state                           jmJobState

job-state-reasons                   jmJobEventJobStateReasons

[jmJobStateReasons1 plus zero or]

[more jobStateReasonsN in a strin

[job-completed and job-purged - extra bindings]

job-k-octets-processed              jmJobKOctetsProcessed

job-impressions-completed           jmJobImpressionsCompleted

[job-progress - extra bindings]

job-k-octets                        jmJobKOctetsPerCopyRequested

job-impressions                     jmJobImpressionsPerCopyRequested

job-copies                          jmProgressJobCopiesRequested

job-collation-type                  jmProgressJobCollationType

job-media-sheets-completed          jmProgressMediaSheetsCompleted

sheet-completed-copy-number         jmProgressSheetCompletedCopyNum

sheet-completed-document-number     jmProgressSheetCompletedDocNum

Hastings, McDonald            Expires 6 January 2001            [Page 5]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

3.2.  Subscription for IPP Notifications

IPP Clients MAY subscribe for IPP Notifications delivered via SNMP by

either of the following two standard methods:

3.2.1.  Subscription via IPP

IPP Job creation (Create-Job, Print-Job, Print-URI) and

Create-Subscription operations MAY be used to create per-Printer or

per-Job IPP Subscription objects and MAY specify

"notify-recipient-uri" = 'snmpnotify://hostname[.port]'

"notify-events" = <events of interest, e.g., 'job-completed'>

3.2.2.  Subscription via SNMP

The SNMP Notification MIB and SNMP Target MIB (both in [RFC-2573])

MAY be used to create SNMP subscriptions.

To create a subscription via SNMP, the IPP Client MAY use an SNMP

Set-Request operation to create an appropriate row in the SNMP

Notification MIB with the following variable-bindings:

1)  'snmpNotifyTag'

- tag of this notification - see 'snmpTargetAddrTagList' below

2)  'snmpNotifyType'

- deliver via SNMP Trap (unacknowledged) or Inform (acknowledged)

3)  'snmpNotifyStorageType'

- persistence (volatile, nonVolatile, permanent, readOnly)

Note:  The SNMP Notification MIB uses persistence instead of the

lease duration supported in IPP Subscription objects.

To complete a subscription via SNMP, the IPP Client MAY use an SNMP

Set-Request operation to create an appropriate row in the SNMP Target

MIB with the following variable-bindings:

1)  'snmpTargetAddrTDomain' and 'snmpTargetAddrTAddress'

- notification target (client) transport protocol and address

(corresponds to IPP 'notify-recipient-uri')

2)  'snmpTargetAddressTimeout' and 'snmpTargetAddrRetryCount'

- retry timeout and limit (for acknowledged SNMP Inform only)

3)  'snmpTargetAddrTagList'

- tags of notifications to be sent to this target (client)

4)  'snmpTargetAddrParamsEntry'

Hastings, McDonald            Expires 6 January 2001            [Page 6]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

- notification security and SNMP protocol version

5)  'snmpTargetAddrStorageType'

- persistence (volatile, nonVolatile, permanent, readOnly)

Note:  The SNMP Target MIB uses persistence instead of the lease

duration supported in IPP Subscription objects.

3.3.  Relationship to other MIBs

3.3.1.  MIB-II (RFC 1213)

All SNMPv1 trap messages include 'time-stamp' which is the value of

the 'sysUpTime' object from MIB-II [RFC-1213].

All SNMPv2 trap bindings include the 'sysUpTime' object from MIB-II

[RFC-1213].

3.3.2.  Host Resources MIB (RFC 2790)

The 'jmServiceTable' defined in this document MAY have a sparse

mapping to the 'hrDeviceTable' (same indices) in Host Resources MIB

[RFC-2790].

3.3.3.  Printer MIB (RFC 1759)

The 'jmServiceTable' defined in this document MAY have a sparse

mapping to the 'hrDeviceTable' (same indices) in Host Resources MIB

[RFC-2790], thus offering access to the Printer MIB via a

'hrDeviceIndex' value.

Hastings, McDonald            Expires 6 January 2001            [Page 7]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

4.  Managed Object Definitions

4.1.  Notification Definitions

4.1.1.  Service Basic Event - Trap

--  Service Basic Event Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send this Service traps via SNMP.

--  The jmServiceBasicEventGroup consists entirely of the

--  jmServiceBasicV2Event notification.

jmServiceBasicV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a Service basic event sent by this managed system."

::= { jobmonMIBNotifications 1 }

jmServiceBasicV2EventPrefix

OBJECT IDENTIFIER ::= { jmServiceBasicV1Enterprise 0 }

jmServiceBasicV2Event NOTIFICATION-TYPE

OBJECTS {

jmServiceEventNotifyEvent,

jmServiceState,

jmServiceStateReasons

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to an IPP Printer basic event.

The value of 'jmServiceIndex' for

use with 'jmServiceTable' for this Service is conveyed in the

instance qualifier (OID suffix) of 'jmServiceState'.

This trap is sent when requested by a prior subscription.

The subscribed event type is in 'jmServiceEventNotifyEvent'.

Standard Printer event types defined in [IPP-NOT] are:

- 'printer-state-changed'

- 'printer-restarted'

- 'printer-shutdown'

Hastings, McDonald            Expires 6 January 2001            [Page 8]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

- 'printer-config-changed'

- 'printer-media-changed'

- 'printer-finishings-changed'

- 'printer-queue-order-changed'

- 'printer-full'

- 'printer-no-longer-full'

- 'printer-almost-idle'

- 'printer-not-almost-idle'

Standard Service event types generalized from [IPP-NOT] are:

- 'service-state-changed'

- 'service-restarted'

- 'service-shutdown'

- 'service-config-changed'

- 'service-media-changed'

- 'service-finishings-changed'

- 'service-queue-order-changed'

- 'service-full'

- 'service-no-longer-full'

- 'service-almost-idle'

- 'service-not-almost-idle'

- and (optionally) vendor extension event types

Additional variable-bindings SHOULD be appended to this trap:

- Systems with the Host Resources MIB (RFC 2790) SHOULD add

'hrSystemDate'

(compare to IPP 'printer-current-time')

Additional variable-bindings MAY be appended to this trap

for all printer-specific events:

- Systems with the Host Resources MIB (RFC 2790) MAY add

'hrDeviceStatus'

(compare to IPP 'printer-state')

'hrPrinterStatus'

(compare to IPP 'printer-state') and

'hrPrinterDetectedErrorState'

(compare to IPP 'printer-state-reasons')

Systems MAY add other variable-bindings from any MIB.

See:    Section 5.3.2 'notify-events' in [IPP-NOT];

Section 8.1 'notify-subscribed-event' in [IPP-NOT];

Section 9 'Event Notification Content' in [IPP-NOT]."

::= { jmServiceBasicV2EventPrefix 1 }

4.1.2.  Job Basic Event - Trap

--  Job Basic Event Group (CONDITIONALLY MANDATORY)

--

Hastings, McDonald            Expires 6 January 2001            [Page 9]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send this Job traps via SNMP.

--  The jmJobBasicEventGroup consists entirely of the

--  jmJobBasicV2Event notification.

jmJobBasicV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a Job basic event sent by this managed system."

::= { jobmonMIBNotifications 2 }

jmJobBasicV2EventPrefix

OBJECT IDENTIFIER ::= { jmJobBasicV1Enterprise 0 }

jmJobBasicV2Event NOTIFICATION-TYPE

OBJECTS {

jmJobEventNotifyEvent,

jmJobState,

jmJobEventJobStateReasons

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to an IPP Job basic event.

The values of 'jmGeneralJobSetIndex' and 'jmJobIndex' for

use with 'jmJobTable' for this Job are conveyed in the

instance qualifier (OID suffix) of 'jmJobState'.

This trap is sent when requested by a prior subscription.

The subscribed event type is in 'jmJobEventNotifyEvent'.

Event types reported via 'jmJobBasicV2Event' include:

- 'job-state-changed'

- 'job-created'

- 'job-completed'

- 'job-purged'

- 'job-config-changed'

- and (optionally) vendor extension event types

Additional variable-bindings SHOULD be appended to this trap:

- Systems with the Host Resources MIB (RFC 2790) SHOULD add

'hrSystemDate'

(compare to IPP 'printer-current-time')

Systems MAY add other variable-bindings from any MIB.

See:    Section 5.3.2 'notify-events' in [IPP-NOT];

Section 8.1 'notify-subscribed-event' in [IPP-NOT];

Section 9 'Event Notification Content' in [IPP-NOT]."

Hastings, McDonald           Expires 6 January 2001            [Page 10]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

::= { jmJobBasicV2EventPrefix 1 }

4.1.3.  Job Completed Event - Trap

--  Job Completed Event Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send this Job traps via SNMP.

jmJobCompletedV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a Job completed event sent by this managed system."

::= { jobmonMIBNotifications 3 }

jmJobCompletedV2EventPrefix

OBJECT IDENTIFIER ::= { jmJobCompletedV1Enterprise 0 }

jmJobCompletedV2Event NOTIFICATION-TYPE

OBJECTS {

jmJobEventNotifyEvent,

jmJobState,

jmJobEventJobStateReasons,

jmJobKOctetsProcessed,

jmJobImpressionsCompleted

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to an IPP 'job-completed' event.

The values of 'jmGeneralJobSetIndex' and 'jmJobIndex' for

use with 'jmJobTable' for this Job are conveyed in the

instance qualifier (OID suffix) of 'jmJobState'.

This trap is sent when requested by a prior subscription.

The subscribed event type is in 'jmJobEventNotifyEvent'.

Event types reported via 'jmJobCompletedV2Event' include:

- 'job-completed'

- 'job-purged'

- and (optionally) vendor extension event types

Additional variable-bindings SHOULD be appended to this trap:

- Systems with the Host Resources MIB (RFC 2790) SHOULD add

'hrSystemDate'

(compare to IPP 'printer-current-time')

Systems MAY add other variable-bindings from any MIB.

Hastings, McDonald           Expires 6 January 2001            [Page 11]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

See:    Section 5.3.2 'notify-events' in [IPP-NOT];

Section 8.1 'notify-subscribed-event' in [IPP-NOT];

Section 9 'Event Notification Content' in [IPP-NOT]."

::= { jmJobCompletedV2EventPrefix 1 }

4.1.4.  Job Progress Event - Trap

--  Job Progress Event Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send this Job traps via SNMP.

jmJobProgressV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a Job progress event sent by this managed system."

::= { jobmonMIBNotifications 4 }

jmJobProgressV2EventPrefix

OBJECT IDENTIFIER ::= { jmJobProgressV1Enterprise 0 }

jmJobProgressV2Event NOTIFICATION-TYPE

OBJECTS {

jmJobKOctetsPerCopyRequested,

jmJobKOctetsProcessed,

jmJobImpressionsPerCopyRequested,

jmJobImpressionsCompleted,

jmProgressJobCopiesRequested,

jmProgressJobCollationType,

jmProgressMediaSheetsCompleted,

jmProgressSheetCompletedCopyNum,

jmProgressSheetCompletedDocNum

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to an IPP 'job-progress' event.

The values of 'jmGeneralJobSetIndex' and 'jmJobIndex' for

use with 'jmJobTable' for this Job are conveyed in the

instance qualifier (OID suffix) of 'jmJobKOctetsProcessed'.

This trap is sent when requested by a prior subscription.

The event type is 'job-progress'.

Additional variable-bindings SHOULD be appended to this trap:

- Systems with the Host Resources MIB (RFC 2790) SHOULD add

'hrSystemDate'

Hastings, McDonald           Expires 6 January 2001            [Page 12]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

(compare to IPP 'printer-current-time')

Systems MAY add other variable-bindings from any MIB.

See:    Section 5.3.2 'notify-events' in [IPP-NOT];

Section 8.1 'notify-subscribed-event' in [IPP-NOT];

Section 9 'Event Notification Content' in [IPP-NOT]."

::= { jmJobProgressV2EventPrefix 1 }

4.2.  Object Definitions

4.2.1.  Service Group - Objects

--  The Service Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which show Service state via SNMP.

--  The jmServiceGroup consists entirely of the jmServiceTable

jmService  OBJECT IDENTIFIER ::= { jobmonMIBObjects 7 }

jmServiceTable  OBJECT-TYPE

SYNTAX      SEQUENCE OF JmServiceEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The jmServiceTable consists of basic service state and status

information for each service which offers one or more job

services on this managed system.

An entry SHALL exist in this table for each service, no matter

what the state of that service.  A service MAY support multiple

configured job sets and configured devices.

See:    'jmServiceJobSetsConfigured' and

'jmServiceDevicesConfigured' bit-arrays in this MIB."

::= { jmService 1 }

jmServiceEntry  OBJECT-TYPE

SYNTAX      JmServiceEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Basic service state and status information."

INDEX  { jmServiceIndex }

Hastings, McDonald           Expires 6 January 2001            [Page 13]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

::= { jmServiceTable 1 }

JmServiceEntry ::= SEQUENCE {

jmServiceIndex                      Integer32 (1..2147483647),

jmServiceName                       JmUTF8StringTC (SIZE (0..63)),

jmServiceURI                        JmUTF8StringTC (SIZE (0..63)),

jmServiceJobServiceTypes            JmJobServiceTypesTC,

jmServiceJobSetsConfigured          OCTET STRING (SIZE (0..255)),

jmServiceDevicesConfigured          OCTET STRING (SIZE (0..255)),

jmServiceState                      JmServiceStateTC,

jmServiceStateReasons               SnmpAdminString (SIZE (0..255))

}

jmServiceIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The unique identifier for this service on this managed system.

See:    'jmServiceEventServiceIndex' object in this MIB."

::= { jmServiceEntry 1 }

jmServiceName OBJECT-TYPE

SYNTAX      JmUTF8StringTC (SIZE(0..63))    -- 127 in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The human readable name of this managed service.

See:    'deviceNameRequested' job attribute in this MIB;

'physicalDevice' job attribute in this MIB;

Section 4.4.4 'printer-name' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no service name

::= { jmServiceEntry 2 }

jmServiceURI OBJECT-TYPE

SYNTAX      JmUTF8StringTC (SIZE(0..63))    -- 1023 in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A URI for this managed service (valid for job services).

See:    Section 4.3.3 'job-printer-uri' in [IPP-MOD];

Section 4.4.1 'printer-uri-supported' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no service URI

::= { jmServiceEntry 3 }

jmServiceJobServiceTypes OBJECT-TYPE

SYNTAX      JmJobServiceTypesTC

Hastings, McDonald           Expires 6 January 2001            [Page 14]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The types of job services supported by this managed service.

See:    'JmJobServiceTypesTC' textual convention in this MIB;

'jobServiceTypes' job attribute in this MIB."

DEFVAL      { 0 }                   -- no job services

::= { jmServiceEntry 4 }

jmServiceJobSetsConfigured OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A bit-array that specifies the job sets configured for this

service, where each bit '2**n' is set if 'jmGeneralJobSetIndex'

is a configured job set.   Uses network byte order (big-endian)

rules - the high-order bit of the first octet corresponds to

'jmGeneralJobSetIndex' of '0' (reserved) - the low-order bit of

the first octet corresponds to 'jmGeneralJobSetIndex' of '7'.

Supports values of 'jmGeneralJobSetIndex' from '1' to '2039'.

Compare to the BITS pseudotype defined in IETF SMIv2 (RFC 2578)

which has the same bit ordering rules (big-endian).

See:    'queueNameRequested' job attribute in this MIB;

'jmGeneralJobSetIndex' table index in this MIB."

DEFVAL      { ''H }                 -- no job sets configured

::= { jmServiceEntry 5 }

jmServiceDevicesConfigured OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A bit-array that specifies the devices configured for this

service, where each bit '2**n' is set if 'hrDeviceIndex'

is a configured device.   Uses network byte order (big-endian)

rules - the high-order bit of the first octet corresponds to

'hrDeviceIndex' of '0' (reserved) - the low-order bit of

the first octet corresponds to 'hrDeviceIndex' of '7'.

Supports values of 'hrDeviceIndex' from '1' to '2039'.

Compare to the BITS pseudotype defined in IETF SMIv2 (RFC 2578)

which has the same bit ordering rules (big-endian).

See:    'physicalDevice' job attribute in this MIB;

'hrDeviceIndex' in IETF Host MIB (RFC 2790)."

DEFVAL      { ''H }                 -- no devices configured

::= { jmServiceEntry 6 }

Hastings, McDonald           Expires 6 January 2001            [Page 15]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

jmServiceState OBJECT-TYPE

SYNTAX      JmServiceStateTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The current state of this managed service.

See:    'jmServiceEventServiceState' object in this MIB;

'jmJobState' object in this MIB;

Section 4.4.11 'printer-state' in [IPP-MOD]."

DEFVAL      { unknown }             -- unknown service state

::= { jmServiceEntry 7 }

jmServiceStateReasons OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255)) -- multi-valued in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The service state reasons associated with this service state

(as a comma-separated list) or the empty string (if none).

See:    'jmServiceEventServiceStateReasons' object in this MIB;

'jmJobStateReasons1' object in this MIB;

Section 4.4.12 'printer-state-reasons' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no service state reasons

::= { jmServiceEntry 8 }

4.2.2.  Service Event Group - Objects

--  The Service Event Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which show Service events via SNMP.

--  The jmServiceEventGroup consists entirely of the jmServiceEventTable

jmServiceEvent  OBJECT IDENTIFIER ::= { jobmonMIBObjects 8 }

jmServiceEventTable  OBJECT-TYPE

SYNTAX      SEQUENCE OF JmServiceEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The jmServiceEventTable contains service event entries for the

services which offer job services on this managed system."

::= { jmServiceEvent 1 }

Hastings, McDonald           Expires 6 January 2001            [Page 16]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

jmServiceEventEntry  OBJECT-TYPE

SYNTAX      JmServiceEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Basic service event information."

INDEX  { jmServiceEventIndex }

::= { jmServiceEventTable 1 }

JmServiceEventEntry ::= SEQUENCE {

jmServiceEventIndex                 Integer32 (1..2147483647),

jmServiceEventNotifyEvent           SnmpAdminString (SIZE (0..63)),

jmServiceEventNotifyTime            TimeTicks,

jmServiceEventServiceIndex          Integer32 (1..2147483647),

jmServiceEventServiceState          JmServiceStateTC,

jmServiceEventServiceStateReasons   SnmpAdminString (SIZE (0..255))

}

jmServiceEventIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The unique identifier for this event on this managed system."

::= { jmServiceEventEntry 1 }

jmServiceEventNotifyEvent OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..63))   -- 255 in [IPP-NOT]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The event type that created this row in 'jmServiceEventTable'.

Standard Printer event types defined in [IPP-NOT] are:

- 'printer-state-changed'

- 'printer-restarted'

- 'printer-shutdown'

- 'printer-config-changed'

- 'printer-media-changed'

- 'printer-finishings-changed'

- 'printer-queue-order-changed'

- 'printer-full'

- 'printer-no-longer-full'

- 'printer-almost-idle'

- 'printer-not-almost-idle'

Standard Service event types generalized from [IPP-NOT] are:

- 'service-state-changed'

- 'service-restarted'

Hastings, McDonald           Expires 6 January 2001            [Page 17]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

- 'service-shutdown'

- 'service-config-changed'

- 'service-media-changed'

- 'service-finishings-changed'

- 'service-queue-order-changed'

- 'service-full'

- 'service-no-longer-full'

- 'service-almost-idle'

- 'service-not-almost-idle'

Conformance:    The natural language for keywords

in subscribed event type SHALL always be US English.

Conformance:    This subscribed event type SHALL be valid

and reported in ALL Job Monitoring MIB notifications.

See:    Section 8.1 'notify-subscribed-event' in [IPP-NOT]."

DEFVAL      { ''H }                 -- no notify subscribed event

::= { jmServiceEventEntry 2 }

jmServiceEventNotifyTime OBJECT-TYPE

SYNTAX      TimeTicks

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The time of this service event.

Usage:  Conforming management agents, which MUST implement the

IETF MIB-II (RFC 1213), SHALL set 'jmServiceEventNotifyTime' to

'sysUpTime' when a service event row is created.

See:    'sysUpTime' in IETF MIB-II (RFC 1213);

Section 5.4.4 'notify-printer-up-time' in [IPP-NOT];

Section 4.4.29 'printer-up-time' in [IPP-MOD]."

::= { jmServiceEventEntry 3 }

jmServiceEventServiceIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unique identifier for this service on this managed system.

See:    'jmServiceIndex' object in this MIB."

::= { jmServiceEventEntry 4 }

jmServiceEventServiceState OBJECT-TYPE

SYNTAX      JmServiceStateTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

Hastings, McDonald           Expires 6 January 2001            [Page 18]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

"The state of this managed service at the time of this event.

See:    'jmServiceState' object in this MIB;

'jmJobState' object in this MIB;

Section 4.4.11 'printer-state' in [IPP-MOD]."

DEFVAL      { unknown }             -- unknown service state

::= { jmServiceEventEntry 5 }

jmServiceEventServiceStateReasons OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255)) -- multi-valued in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The service state reasons associated with this service state

(as a comma-separated list) or the empty string (if none).

See:    'jmServiceStateReasons' object in this MIB;

'jmJobStateReasons1' object in this MIB;

Section 4.4.12 'printer-state-reasons' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no service state reasons

::= { jmServiceEventEntry 6 }

4.2.3.  Job Event Group - Objects

--  The Job Event Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which show Job events via SNMP.

--  The jmJobEventGroup consists entirely of the jmJobEventTable

jmJobEvent  OBJECT IDENTIFIER ::= { jobmonMIBObjects 9 }

jmJobEventTable  OBJECT-TYPE

SYNTAX      SEQUENCE OF JmJobEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The jmJobEventTable contains job event entries for the

jobs present on this managed system."

::= { jmJobEvent 1 }

jmJobEventEntry  OBJECT-TYPE

SYNTAX      JmJobEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Basic job event information."

Hastings, McDonald           Expires 6 January 2001            [Page 19]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

INDEX  { jmJobEventIndex }

::= { jmJobEventTable 1 }

JmJobEventEntry ::= SEQUENCE {

jmJobEventIndex                     Integer32 (1..2147483647),

jmJobEventNotifyEvent               SnmpAdminString (SIZE (0..63)),

jmJobEventNotifyTime                TimeTicks,

jmJobEventJobSetIndex               Integer32 (1..32767),

jmJobEventJobIndex                  Integer32 (1..2147483647),

jmJobEventJobState                  JmJobStateTC,

jmJobEventJobStateReasons           OCTET STRING (SIZE (4..16))

}

jmJobEventIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The unique identifier for this event on this managed system."

::= { jmJobEventEntry 1 }

jmJobEventNotifyEvent OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..63))   -- 255 in [IPP-NOT]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The event type that created this row in 'jmJobEventTable'.

Standard Job event types defined in [IPP-NOT] are:

- 'job-state-changed'

- 'job-created'

- 'job-completed'

- 'job-purged'

- 'job-config-changed'

- 'job-progress'

Conformance:    The natural language for keywords

in subscribed event type SHALL always be US English.

Conformance:    This subscribed event type SHALL be valid

and reported in ALL Job Monitoring MIB notifications.

See:    Section 8.1 'notify-subscribed-event' in [IPP-NOT]."

DEFVAL      { ''H }                 -- no notify subscribed event

::= { jmJobEventEntry 2 }

jmJobEventNotifyTime OBJECT-TYPE

SYNTAX      TimeTicks

MAX-ACCESS  read-only

STATUS      current

Hastings, McDonald           Expires 6 January 2001            [Page 20]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

DESCRIPTION

"The time of this job event.

Usage:  Conforming management agents, which MUST implement the

IETF MIB-II (RFC 1213), SHALL set 'jmJobEventNotifyTime' to

'sysUpTime' when a job event row is created.

See:    'sysUpTime' in IETF MIB-II (RFC 1213);

Section 5.4.4 'notify-printer-up-time' in [IPP-NOT];

Section 4.4.29 'printer-up-time' in [IPP-MOD]."

::= { jmJobEventEntry 3 }

jmJobEventJobSetIndex OBJECT-TYPE

SYNTAX      Integer32 (1..32767)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unique identifier for this job set on this managed system.

See:    'jmGeneralJobSetIndex' object in this MIB."

::= { jmJobEventEntry 4 }

jmJobEventJobIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unique identifier for this job on this managed system,

when prefixed with 'jmJobEventJobSetIndex'.

See:    'jmJobIndex' object in this MIB."

::= { jmJobEventEntry 5 }

jmJobEventJobState OBJECT-TYPE

SYNTAX      JmJobStateTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The state of this managed job at the time of this event.

See:    'jmJobState' in this MIB;

Section 4.3.7 'job-state' in [IPP-MOD]."

DEFVAL      { unknown }             -- unknown job state

::= { jmJobEventEntry 6 }

jmJobEventJobStateReasons OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE (4..16)) -- multi-valued in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The job state reasons associated with this job state

Hastings, McDonald           Expires 6 January 2001            [Page 21]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

represented as one to four concatenated 32-bit integers

in network byte order (big-endian).

Usage:  Conforming management stations SHALL always report the

value of the object 'jmJobStateReasons1' in the first four

octets of 'jmJobEventJobStateReasons' and SHOULD report values

of the attributes 'jobStateReasons2', 'jobStateReasons3', and

'jobStateReasons4' in subsequent octets.

See:    'jmJobStateReasons1' in this MIB;

Section 4.3.8 'job-state-reasons' in [IPP-MOD]."

DEFVAL      { '00000000'H }         -- no job state reasons

::= { jmJobEventEntry 7 }

4.2.4.  Job Progress Group - Objects

--  The Job Progress Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send Job progress traps via SNMP.

--  The jmProgressGroup consists entirely of leaf objects for traps

jmProgress  OBJECT IDENTIFIER ::= { jobmonMIBObjects 10 }

jmProgressJobCopiesRequested OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of copies of this job requested by the client.

See:    'jobCopiesRequested' attribute in this MIB."

DEFVAL      { -2 }                  -- unknown job copies

::= { jmProgress 1 }

jmProgressJobCollationType OBJECT-TYPE

SYNTAX      JmJobCollationTypeTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of copies of this job requested by the client.

See:    'jobCollationType' attribute in this MIB;

'job-collation-type' in [IPP-PROG];

Section 9 'Event Notification Content' in [IPP-NOT]."

DEFVAL      { unknown }             -- unknown job collation type

::= { jmProgress 2 }

Hastings, McDonald           Expires 6 January 2001            [Page 22]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

jmProgressMediaSheetsCompleted OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of media sheets completed for this job.

See:    'sheetsCompleted' attribute in this MIB;

Section 4.3.18.3 'job-media-sheets-completed'

in [IPP-MOD];

Section 9 'Event Notification Content' in [IPP-NOT]."

DEFVAL      { -2 }                  -- unknown job progress

::= { jmProgress 3 }

jmProgressSheetCompletedCopyNum OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of the job copy currently being stacked for the

current document or zero (if none) or '-2' (unknown).

See:    'sheetCompletedCopyNumber' attribute in this MIB;

'sheet-completed-copy-number' in [IPP-PROG];

Section 9 'Event Notification Content' in [IPP-NOT]."

DEFVAL      { -2 }                  -- unknown sheet complete copy

::= { jmProgress 4 }

jmProgressSheetCompletedDocNum OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of the job document currently being stacked for the

current job or zero (if none) or '-2' (unknown).

See:    'sheetCompletedDocumentNumber' attribute in this MIB;

'sheet-completed-document-number' in [IPP-PROG];

Section 9 'Event Notification Content' in [IPP-NOT]."

DEFVAL      { -2 }                  -- unknown sheet complete doc

::= { jmProgress 5 }

Hastings, McDonald           Expires 6 January 2001            [Page 23]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

5.  IANA Considerations

None.

6.  Internationalization Considerations

The 'jmServiceStateReasons', 'jmServiceEventServiceStateReasons',

'jmServiceEventNotifyEvent', and 'jmJobEventNotifyEvent', text string

objects defined in this document are of type 'SnmpAdminString' (UTF-8

stream-encoded ISO 10646/Unicode text) defined in the SNMPv3

Framework MIB [RFC-2571].  The natural language of these objects is

US English (they contain keywords).

7.  Security Considerations

This IPP Notifications over SNMP mapping defines only 'read-only'

objects.  It is suitable for use with any version of SNMP, as no

update security is required (because no configuration updates are

supported).

No sensitive information is available via IPP Notifications over

SNMP.

Hastings, McDonald           Expires 6 January 2001            [Page 24]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

8.  References

[IPP-MOD] R. deBry, T. Hastings, R. Herriot, S. Isaacson, P. Powell.

IPP/1.1 Model and Semantics, <draft-ietf-ipp-model-v11-07.txt>

(work-in-progress), May 2000.

[IPP-NOT] S. Isaacson, J. Martin, R. deBry, T. Hastings, M. Shepherd,

R. Bergman.  IPP Event Notification Specification,

<draft-ietf-ipp-not-spec-02.txt> (work-in-progress), March 2000.

[IPP-PROG] T. Hastings, R. Bergman, H. Lewis.  Proposed Job Progress

Attributes for IPP, <draft-ietf-ipp-job-prog-00.txt>

(work-in-progress), February 2000.

[RFC-1213] K. McCloghrie, M. Rose.  MIB-II, RFC 1213, March 1991.

[RFC-1759] R. Smith, F. Wright, T. Hastings, S. Zilles,

J. Gyllenskog.  Printer MIB, RFC 1759, March 1995.

[RFC-2571] D. Harrington, R. Presuhn, B. Wijnen.  Architecture for

SNMP Management Frameworks, RFC 2571, April 1999.

[RFC-2573] D. Levi, P. Meyer, B. Stewart.  SNMP Applications, RFC

2573, April 1999.

[RFC-2576] R. Frye, D. Levi, S. Routhier, B. Wijnen.  Coexistence

between Version 1, Version 2, and Version 3 of the Internet-standard

Network Management Framework, RFC 2576, March 2000.

[RFC-2707] R. Bergman, T. Hastings, S. Isaacson, H. Lewis.  Job

Monitoring MIB v1.0, RFC 2707, November 1999

[RFC-2790] P. Grillo, S. Waldbusser.  Host Resources MIB, RFC 2790,

March 2000.

Hastings, McDonald           Expires 6 January 2001            [Page 25]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

9.  Change Log

Changes in reverse chronological order (most recent first).

<draft-ietf-ipp-not-over-snmp-03.txt> - 6 July 2000

1)  Added 'SnmpAdminString' to IMPORTS clause for new objects.

2)  Corrected OID in MODULE-IDENTITY to use forward reference to

definition of 'pwg' from 'enterprises' and 'mibs' from 'pwg'.

3)  Added 'JmServiceStateTC' textual convention.

4)  Added 'jmMirrorAttr' and 'jmSystem' object identifiers reserved

for future extensions.

5)  Major rewrite, per email discussion on IETF IPP WG list, to

specify four new small (traditional) SNMP traps for:

'jmServiceBasicV2Event' (generalized from IPP Printer event),

'jmJobBasicV2Event' (corresponds IPP Job event),

'jmJobCompletedV2Event' (corresponds IPP Job completed event),

'jmJobProgressV2Event' (corresponds IPP Job progress event).

6)  Major rewrite, per email discussion on IETF IPP WG list, to

specify four new SNMP object groups:  'jmServiceTable' (name,

URI, state, etc.  - from IPP Printer), 'jmServiceEventTable'

(records IPP Printer events for polling), 'jmJobEventTable'

(records IPP Job events for polling), 'jmJobProgressGroup' (leaf

objects for IPP Job progress event).

7)  Revised section 3.1 'SNMP Mapping for IPP Printer Events' and

section 3.2 'SNMP Mapping for IPP Job Events', to agree with

above.

8)  Deleted obsolete section 3.3 'Rules for Encoding Notifications',

as event bindings now always fit over all SNMP transport

protocols.

<draft-ietf-ipp-not-over-snmp-02.txt> - 19 March 2000

1)  Renamed Printer Event notification group to Device Basic Event

and 'jmPrinterEventV2Event' notification to

'jmDeviceBasicV2Event', to better align with IPP and to support

non-printing jobs.

2)  Revised 'jmDeviceBasicV2Event' notification to remove

'hrDeviceStatus', 'hrPrinterStatus',

'hrPrinterDetectedErrorState' from mandatory trap bindings

because they were redundant, per request of Ron Bergman.

3)  Renamed Job Event notification group to Job Basic Event and

'jmJobEventV2Event' notification to 'jmJobBasicV2Event', to

better align with IPP and to support variant job events.

4)  Defined new Job Completed Event notification group and defined

new Job Progress Event notification group, to better align with

IPP and to support variant job events.

5)  Renamed Event object group to Event Binding,

'jmEventPrinterState' to 'jmEventDeviceState',

'jmEventPrinterStateReasons' to 'jmEventDeviceStateReasons',

'jmEventPrinterIsAcceptingJobs' to

Hastings, McDonald           Expires 6 January 2001            [Page 26]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

'jmEventDeviceIsAcceptingJobs', to support non-printing jobs.

6)  Revised Event Binding object group, adding explicit objects

'jmEventDeviceURI', 'jmEventDeviceName', 'jmEventJobSetIndex',

'jmEventJobIndex', 'jmEventJobName', 'jmEventJobState',

'jmEventJobStateReasons', 'jmEventJobKOctets',

'jmEventJobKOctetsProcessed', 'jmEventJobImpressions',

'jmEventJobImpressionsCompleted', 'jmEventJobMediaSheets',

'jmEventJobMediaSheetsCompleted',

'jmEventJobImpressionsCompletedCC', 'jmEventJobCollationType',

'jmEventJobSheetCompletedCopyNum',

'jmEventJobSheetCompletedDocNum', per request of Ron Bergman.

7)  Revised SYNTAX of 'jmEventTriggerEvent' object from from

'JmUTF8StringTC' (string) to IPP-aligned enumeration, per request

of Ron Bergman.

8)  Removed all references to Printer MIB v2, as they were of limited

value, per request of Ron Bergman.

9)  Revised 'SNMP Mapping for IPP Printer Events' section for renamed

event binding objects, per request of Ron Bergman.

10) Revised 'Rules for Encoding Notifications' section to truncate

additional string bindings, per request of Ron Bergman.

11) Revised 'Registration via IPP' section, to change scheme name

from 'ipp-snmp:' to 'snmpnotify:', per request of Ron Bergman.

<draft-ietf-ipp-not-over-snmp-01.txt> - 1 December 1999

1)  Deleted 'JmTriggerEventTC' textual convention (see below).

2)  Revised SYNTAX of 'jmEventTriggerEvent' object from

'JmTriggerEventTC' (enumeration) to 'JmUTF8StringTC' (string), to

support use of IPP standard keywords.

3)  Added 'jmEventPrinterState', 'jmEventPrinterStateReasons', and

'jmEventPrinterIsAcceptingJobs' objects for consistency w/

[IPP-NOT] and to reduce ambiguity about printer states inherent

in RFC 1759.

4)  Revised DESCRIPTION of 'jmPrinterEventV2Event' notification to

add SHOULD (recommendation) for 'jmEventPrinterState',

'jmEventPrinterStateReasons', and 'jmEventPrinterIsAcceptingJobs'

objects.

5)  Revised 'SNMP Mapping for IPP Printer Events' section to add

direct mapping of IPP notification attributes to

'jmEventPrinterState', 'jmEventPrinterStateReasons', and

'jmEventPrinterIsAcceptingJobs' objects.

6)  Revised 'Rules for Encoding Notifications' section to add

'jmEventPrinterState' and 'jmEventPrinterStateReasons'.

7)  Revised 'IANA Considerations' section to specify there are none -

no enumerated or keyword textual conventions are now defined in

this document.

8)  Revised 'Internationalization Considerations' section to specify

that US English keywords are used in 'jmEventTriggerEvent',

'jmEventPrinterState', and 'jmEventPrinterStateReasons' objects

and thus no explicit natural language tagging is required.

<draft-ietf-ipp-not-over-snmp-00.txt> - 10 October 1999

Hastings, McDonald           Expires 6 January 2001            [Page 27]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

1)  Initial version.

Hastings, McDonald           Expires 6 January 2001            [Page 28]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

10.  Intellectual Property Notice

The IETF takes no position regarding the validity or scope of any

intellectual property or other rights that might be claimed to

pertain to the implementation or use of the technology described in

this document or the extent to which any license under such rights

might or might not be available; neither does it represent that it

has made any effort to identify any such rights.  Information on the

IETF's procedures with respect to rights in standards-track and

standards-related documentation can be found in BCP-11.  Copies of

claims of rights made available for publication and any assurances of

licenses to be made available, or the result of an attempt made to

obtain a general license or permission for the use of such

proprietary rights by implementers or users of this specification can

be obtained from the IETF Secretariat.

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights which may cover technology that may be required to practice

this standard.  Please address the information to the IETF Executive

Director.

11.  Authors' Addresses

Tom Hastings

Xerox Corporation

701 S Aviation Blvd, MS 834-03E

El Segundo, CA  90245

Phone: +1 310-333-6413

Email: hastings@cp10.es.xerox.com

Ira McDonald

High North Inc

221 Ridge Ave

Grand Marais, MI  49839

Phone: +1 906-494-2434 or +1 906-494-2697

Email: imcdonald@sharplabs.com

Email: imcdonal@sdsp.mc.xerox.com

Hastings, McDonald           Expires 6 January 2001            [Page 29]

Internet Draft     IPP Notify - Job Monitoring MIB Traps     6 July 2000

12.  Full Copyright Statement

Copyright (C) The Internet Society (2000).  All Rights Reserved.

This document and translations of it may be copied and furnished to

others, and derivative works that comment on or otherwise explain it

or assist in its implementation may be prepared, copied, published

and distributed, in whole or in part, without restriction of any

kind, provided that the above copyright notice and this paragraph are

included on all such copies and derivative works.  However, this

document itself may not be modified in any way, such as by removing

the copyright notice or references to the Internet Society or other

Internet organizations, except as needed for the purpose of

developing Internet standards in which case the procedures for

copyrights defined in the Internet Standards process must be

followed, or as required to translate it into languages other than

English.

The limited permissions granted above are perpetual and will not be

revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an

"AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING

TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING

BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION

HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

Hastings, McDonald           Expires 6 January 2001            [Page 30]