List of Printer MIB, UPnP, and Bluetooth Print Document Formats

ISSUES are highlighted like this.

File: Print-MIME-tab.doc, .pdf

Date: 1/16/2001

| Enum (decimal) | Enum (hex) | Name or Acrnoym | MIME Media Type (already registered) | Description, including full title | Reference | Company or Organization (without location, since it changes) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 01 | Other |  |  |  |  |
|  | 02 | Unknown |  |  |  |  |
|  | 03 | PCL | application/vnd.hp-PCL | Starting with PCL version 5, HP-GL/2 is included as part of the PCL language. PCL and HP-GL/2 are registered trademarks of Hewlett-Packard Company. | PCL-PJL Technical Reference Manual Documentation Package HP Part No. 5012-0330 | Hewlett-Packard Company |
|  | 04 | HPGL | application/vnd.hp-HPGL | Hewlett-Packard Graphics Language. HP-GL is a registered trademark of Hewlett-Packard Company. | The HP-GL/2 and HP RTL Reference Guide Addison-Wesley Publishing Co. ISBN: 0-201-63325-6 | Hewlett-Packard Company |
|  | 05 | PJL |  | Peripheral Job Language. Appears in the data stream between data intended for a page description language |  | Hewlett-Packard Co. |
|  | 06 | PostScript (tm) Language | application/postscript | Postscript – a trademark of Adobe Systems Incorporated which may be registered in certain jurisdictions. | MIME Registry has: [RFC2046] | Adobe Systems |
|  | 07 | IPDS |  | Intelligent Printer Data Stream. Bi-directional print data stream for documents consisting of data objects (text, image, graphics, bar codes), resources (fonts, overlays) and page, form and finishing instructions. Facilitates system level device control, document tracking and error recovery throughout the print process. |  | IBM Corporation |
|  | 08 | PPDS |  | IBM Personal Printer Data Stream. Originally called IBM ASCII, the name was changed to PPDS when the Laser Printer was introduced in 1989 |  | Lexmark International, Inc. |
|  | 09 | Escape/P |  |  |  | Epson Corp. |
|  | 0A | Epson |  | What is this? |  | Epson Corp. |
|  | 0B | DDIF |  | Digital Document Interchange Format |  | Digital Equipment Corp. (now Compaq) |
|  | 0C | Interpress |  | Need a Description or title |  | Xerox Corp. |
|  | 0D | ISO 6429 |  | Control functions for Coded Character Sets (has ASCII control characters, plus additional controls for character imaging devices.) | ISO 6429 ontrol functions for Coded Character Sets | ISO |
|  | 0E | LineData |  | Lines of data as separate ASCII or EBCDIC records and containing no control functions (no CR, LF, HT, FF, etc.) For use with traditional line printers. May use CR and/or LF to delimit lines, instead of records. | ISO 10175 Document Printing Application (DPA) | ISO |
|  | 0F | MODCA | application/vnd.ibm.modcap is this correct? | Mixed Object Document Content Architecture. Definitions that allow the composition, interchange, and presentation of final form documents as a collection of data objects (text, image, graphics, bar codes), resources (fonts, overlays) and page, form and finishing instructions | MIME Registry talks about MO:DCA-P. Is this the same as the Printer MIB enum? | IBM Corporation |
|  | 10 | ReGIS |  | Remote Graphics Instruction Set |  | Digital Equipment Corp. (now Compaq) |
|  | 11 | SCS |  | SNA Character String Bi-directional print data stream for SNA LU-1 mode of communication |  | IBM Corporation |
|  | 12 | SPDL |  | ISO 10180 Standard Page Description Language. | ISO 10180 Standard Page Description Language | ISO |
|  | 13 | TEK4014 |  |  |  | Tektronix Corp. |
|  | 14 | PDS |  |  |  | Who? |
|  | 15 | IGP |  |  |  | Printronix Corp. |
|  | 16 | CodeV |  | Magnum Code-V, Image and printer control language used to control impact/dot-matrix printers |  | QMS, Inc., Mobile AL |
|  | 17 | DSC-DSE |  | Data Stream Compatible and Emulation Bi-directional print data stream for non-SNA (DSC) and SNA LU-3 3270 controller (DSE) communications |  | IBM Corporation. |
|  | 18 | WPS |  | Windows Printing System, Resource based command/data stream used by Microsoft At Work Peripherals |  | Developed by the Microsoft Corporation |
|  | 19 | LN03 |  | Early DEC-PPL3 |  | Digital Equipment Corp. |
|  | 1A | CCITT |  | ISSUE: I think the intent was Group 3 or 4 FAX. How would this entry relate to the TIFF (40)? |  |  |
|  | 1B | QUIC |  | QUIC (Quality Information Code), Page Description Language for laser printers. Included graphics, printer control capability and emulation of other well-known printer. |  | QMS, Inc. |
|  | 1C | CPAP |  | Common Printer Access Protocol |  | Digital Equipment Corp. (now COMPAQ) |
|  | 1D | DEC-PPL |  | Digital ANSI-Compliant Printing Protocol |  | Digital Equipment Corp. (now COMPAQ) |
|  | 1E | SimpleText | Is the ISO DPA Annex F of Simple-Text close enough to the IETF text/plain to be able to use ‘text/plain’ as the MIME media type? | Character coded data, including NUL, CR , LF, HT, and FF control characters |  | See ISO 10175 Document Printing Application (DPA) |
|  | 1F | NPAP |  | Network Printer Alliance Protocol. This protocol has been superseded by the IEEE 1284.1. |  | Network Printer Allicance (NPA) |
|  | 20 | DOC |  | Document Option Commands, Appears in the data stream between data intended for a page description |  | QMS, Inc. |
|  | 21 | ImPRESS |  | Page description language originally developed for the ImageServer product line. A binary language providing representations of text, simple graphics, and some large forms (simple bit-map and CCITT group 3/4 encoded). The language was intended to be sent over an 8-bit channel and supported early document preparation languages (e.g., TeX and TROFF) |  | QMS, Inc. |
|  | 22 | Pinwriter |  | 24 wire dot matrix printer for USA, Europe, and Asia except Japan. More widely used in Germany, and some Asian countries than in US |  | NEC Corp. |
|  | 23 | NPDL |  | Page printer for Japanese market |  | NEC Corp. |
|  | 24 | NEC201PL |  | Serial printer language used in the Japanese market |  | NEC Corp. |
|  | 25 | Automatic | application/octet-stream | Automatic PDL sensing. Automatic sensing of the interpreter language family by the printer examining the document content. Which actual interpreter language families are sensed depends on the printer implementation. | RFC 2911, Internet Printing Protocol/1.1 Model and Semantics But MIME registry already has: [RFC2045,RFC2046] | IETF |
|  | 26 | Pages |  | Page printer Advanced Graphic Escape Set |  | IBM Japan |
|  | 27 | LIPS |  | LBP Image Processing System |  |  |
|  | 28 | TIFF | image/tiff image/tiff;faxbw image/tiff;faxcolor | Tagged Image File Format | MIME Registry has: [RFC2302] | Aldus (now Adobe Systems Incorporated |
|  | 29 | Diagnostic |  | A hex dump of the input to the interpreter | RFC 1759 | IETF |
|  | 2A | PSPrinter |  | The PostScript Language used for control (with any PDLs) |  | Adobe Systems Incorporated |
|  | 2B | CaPSL |  | Canon Print Systems Language |  | Canon |
|  | 2C | EXCL |  | Extended Command Language |  | Talaris Systems Inc. |
|  | 2D | LCDS |  | Line Conditioned Data Stream |  | Xerox Corporation |
|  | 2E | XES |  | Xerox Escape Sequences |  | Xerox Corporation |
|  | 2F | PCLXL | application/vnd.hp-PCLXL | Printer Control Language eXtended Language features for printing, and printer control | MIME Registry has: Specification by example: Within a Windows application select: Print and select as the printer HP 'LaserJet 5/5M - Enhanced Driver' (this driver is available from www.hp.com). Select: 'Print to File' and then 'Okay' | Hewlett-Packard Co. |
|  | 30 | ART |  | Advanced Rendering Tools (ART). Page Description language originally developed for the Laser Press printers. | Technical reference manual: "ART IV Reference Manual", No F33M | Fuji Xerox Co., Ltd. |
|  | 31 | TIPSI |  | Transport Independent Printer System Interface (ref. IEEE Std. 1284.1) | IEEE Std. 1284.1 | IEEE |
|  | 32 | Prescribe |  | Page description and printer control language. It can be described with ordinary ASCII | Technical reference manual: "PRESCRIBE II Programming Manual" | What company? |
|  | 33 | LinePrinter |  | A simple-text character stream which supports the control codes LF, VT, FF, and plus Centronics or Dataproducts Vertical Format Unit (VFU) language is commonly used on many older model line and matrix printers. |  |  |
|  | 34 | IDP |  | Imaging Device Protocol |  | Apple Computer. |
|  | 35 | XJCL |  | Xerox Job Control Language (JCL). A Job Control language originally developed for the LaserPress printers and is capable of switching PDLs. | Technical reference manual: "ART IV Reference Manual", No F33M | Fuji Xerox Co., Ltd. |
|  | 36 | PDF | application/pdf | Adobe Portable Document Format | Version 1.0: Portable Document Format Reference Manual, Adobe Systems, Inc, Addison-Wesley Publishing Co. ISBN 0-201-62628-4, 1993. Version 1.1: An Adobe Systems, Inc Technical Note | Adobe Systems, Inc. |
|  | 37 | RPDL |  | Ricoh Page Description Language for printers. | Technical manual "RPDL command reference" No.307029 | RICOH, Co. LTD |
|  | 38 | IntermecIPL |  | Intermec Printer Language for label printers. | Technical Manual: "IPL Programmers Reference Manual" | Intermec Corporation |
|  | 39 | UBIFingerprint |  | An intelligent basic-like programming language for label printers. | Reference Manual: "UBI Fingerprint 7.1", No. 1-960434-00 | United Barcode Industries |
|  | 3A | UBIDirectProtocol |  | An intelligent control language for label printers. | Programmers Guide: "UBI Direct Protocol", No. 1-960419-00 | United Barcode Industries |
|  | 3B | Fujitsu |  | Fujitsu Printer Language | Reference Manual: "FM Printer Sequence" No. 80HP-0770 | FUJITSU LIMITED |

The following Print Document Formats have been proposed for use in Blue tooth with the following additional enum values assigned.  They need to be added to the Printer MIB as well.

| Enum (decimal) | Enum (hex) | Name or Acrnoym | MIME Media Type (already registered) | Description, including full title | Reference | Company or Organization (without location, since it changes) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3C | SPCL. |  | Standard PCL |  |  |
|  | 3D | VCard |  |  |  |  |
|  | 3E | VCal |  |  |  |  |
|  | 3F | VNote |  |  | Infrared Data Association, IrMC (Ir Mobile Communications) Specification with Published Errata, Version 1.1, February 1999 |  |
|  | 40 | VMessage |  |  | Infrared Data Association, IrMC (Ir Mobile Communications) Specification with Published Errata, Version 1.1, February 1999 |  |
|  | 41 | HTML | text/html | The 'text/html' Media Type | RFC 2854, The 'text/html' Media Type. D. Connolly, L. Masinter. June 2000. | IETF |
|  | 42 | XHTML | application/xhtml+xml (Internet-Draft only) | The Extensible HyperText Markup Language |  | W3C |
|  | 43 | XHTML-Basic | no separate MIME registry planned See draft-baker-xhtml-media-reg-00.txt | XHTML Basic |  | W3C |
|  | 44 | XHTML-Print |  |  |  |  |