The Printer Working Group (PWG)

Proposed Standard for
The Internet Printing Protocol (IPP):

Document Object

6 ISSUES are highlighted like this.

Version 0.5, October 25, 2002

The Printer Working Group (PWG)
Proposed Standard for

The Internet Printing Protocol (IPP):

Document Object

Version 0.5, October 25, 2002

Abstract: This IPP specification extends the IPP Model and Semantics [RFC2911] object model by defining a Document object.  The [RFC2911] Job object is extended to contain one or more Document objects which are passive objects operated on by the Job.  The Document object is created by the [RFC2911] Send-Document and Send-URI operations and represents an Input Document to the Job.  This specification also adds a Document Attributes group (and tag) to these Send-Document and Send-URI requests.  The Document Attributes group contains any Document Template attributes to be applied to this Document object being created, overriding any corresponding Job Template attribute supplied at the job level (including the “document-overrides” operation attributes at the Job or Document level).  A new Create-Document operation allows a client to supply the Document Attributes without the data (like Create-Job does for a Job object).  Then the client sends the data content to that Document object using the new Send-Data operation.  The client can also supply Document Attributes with the existing Send-Document and Send-URI operations to create Document objects with content data or a document reference, respectively.

This specification also defines seven new operations for Document objects once they have been created: Cancel-Current-Document, Cancel-Document, Get-Document-Attributes, Get-Documents, Delete-Document, Set-Document-Attributes, and Validate-Document.

This specification also lists all of the attributes defined in other IPP specifications to show their relationship to corresponding attributes defined in this IPP specification for use with the Document object.  The full semantics of the “document-state”, “document-state-reasons”, and “document-state-message” Document Description attributes are given along side of the corresponding [RFC2911] “job-state”,  “job-state-reasons”, and “job-state-message” Job Description attributes.

The purpose for specifying the Document object is so that the print industry can have a common semantic specification for use in IPP, the PWG Semantic Model, the PWG Print Service Interface (PSI) project, and the Free Standards Group Job Ticket API which all have a Document object.

This version of the PWG Proposed Standard is available electronically at:

ftp://ftp.pwg.org/pub/pwg/ipp/new_DOC/pwg-ipp-document-object-v05-021025.pdf, .doc, .rtf

This document is an IEEE-ISTO PWG Proposed standard.  For a definition of a "PWG Proposed Standard" and its transition to a “PWG Draft Standard”, see:  ftp://ftp.pwg.org/pub/pwg/general/pwg-process.pdf.  After approval by the PWG (by a Last Call) to transition this Proposed Standard to a Draft Standard, an IEEE-ISTO number will be assigned and this PWG Draft Standard will be available electronically at:

ftp://ftp.pwg.org/pub/pwg/standards/

Copyright (C) 2002, IEEE ISTO. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  The Printer Working Group Standard for the Internet Printing Protocol (IPP): Document Object

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

About the IEEE-ISTO

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit http://www.ieee-isto.org.

About the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:  http://www.pwg.org

Contact information:

IPP Web Page:  http://www.pwg.org/ipp/

IPP Mailing List:  ipp@pwg.org

To subscribe to the ipp mailing list, send the following email:

1) send it to majordomo@pwg.org

2) leave the subject line blank

3) put the following two lines in the message body:

subscribe ipp

end

Implementers of this specification are encouraged to join the IPP Mailing List in order to participate in any discussions of clarifications or review of registration proposals for additional names.  Requests for additional media names, for inclusion in this specification, should be sent to the IPP Mailing list for consideration.

Contents

1	Introduction and Summary	2

1.1	Scope	2

2	Terminology	2

2.1	Conformance Terminology	2

2.2	Other Terminology	2

3	Document Operations	2

3.1	Create-Document Operation	2

3.1.1 Create-Document Request	2

3.1.2 Create-Document Response	2

3.2	Send-Data Operation	2

3.2.1 Send-Data request	2

3.2.2 Send-Data Response	2

3.3	Send-Document Operation	2

3.3.1 Send-Document Request	2

3.3.2 Send-Document Response	2

3.4	Send-URI Operation	2

3.5	Validate-Document Operation	2

3.6	Get-Document-Attributes Operation	2

3.6.1 Get-Document-Attributes Request	2

3.6.2 Get-Document-Attributes Response	2

3.7	Get-Documents Operation	2

3.7.1 Get-Documents Request	2

3.7.2 Get-Documents Response	2

3.8	Set-Document-Attributes Operation	2

3.8.1 Settable and READ-ONLY Document Description attributes	2

3.8.2 Set-Document-Attributes Request	2

3.8.3 Set-Document-Attributes Response	2

3.9	Cancel-Document Operation	2

3.9.1 Cancel-Document Request	2

3.9.2 Cancel-Document Response	2

3.10	Cancel-Current-Document Operation	2

3.10.1 job-id (integer(1:MAX))	operation attribute	2

3.10.2 document-number (integer(1:MAX))	operation attribute	2

3.10.3 document-message (text(127))	operation attribute	2

3.11	Delete-Document Operation	2

4	Semantic extensions to Job operations	2

4.1	Cancel-Current-Job ([adm-ops] section 4.2)	2

4.2	Cancel-Job ([RFC2911] section 3.3.3)	2

4.3	Create-Job ([RFC2911] section 3.2.4)	2

4.4	Print-Job ([RFC2911] section 3.2.1)	2

4.5	Print-URI ([RFC2911] section 3.2.2)	2

4.6	Purge-Jobs ([RFC2911] section 3.2.9)	2

4.7	Release-Job  ([RFC2911] section 3.3.6)	2

4.8	Reprocess-Job ([adm-ops] section 4.1)	2

4.9	Restart-Job ([RFC2911] section 3.3.7)	2

4.10	Send-Document ([RFC2911] section 3.3.1)	2

4.11	Send-URI ([RFC2911] section 3.3.2)	2

4.12	Suspend-Current-Job ([adm-ops] section 4.3.1)	2

5	Attribute Precedence	2

6	Operation Attributes for the Job and Document Creation operations	2

6.1	Operation Attributes supplied at the Job and Document Levels for Creation	2

6.2	Operation Attributes defined for use in Document Creation operations	2

6.2.1 ipp-attribute-fidelity (boolean) (RFC2911] sections 3.2.1 and 15)	2

6.2.2 job-mandatory-attributes (1setOf type2 keyword)	2

6.2.3 document-mandatory-attributes (1setOf type2 keyword)	2

6.3	Operation Attributes returned in Job and Document Creation responses	2

7	Job Template and Document Template attributes	2

7.1	Job Template attributes defined for use with the Document object	2

7.1.1 job-copies (integer(1:MAX))	Job Template attribute	2

7.1.2 job-cover-back (collection)	Job Template attribute	2

7.1.3 job-cover-front (collection)	Job Template attribute	2

7.1.4 job-finishings (1setOf type2 enum)	Job Template attribute	2

7.1.5 job-finishings-col (1setOf collection)	Job Template attribute	2

8	Job Description and Document Description attributes	2

8.1	document-number (integer(1:MAX)) Document Description attribute	2

8.2	document-id-uri (uri) Document Description attribute	2

8.3	document-uri (uri) Document Description attribute	2

8.4	job-state (type1 enum) Job Description attribute	2

8.5	document-state (type1 enum) Document Description attribute	2

8.5.1 The “job-state” Job Description attribute and the Job Life Cycle	2

8.5.2 The “document-state” Document Description attribute and the Document object life Cycle	2

8.5.3 Forwarding Servers	2

8.5.4 Partitioning of Job/Document States	2

8.6	job-state-reasons  (1setOf type2 keyword)  Job Description attribute	2

8.7	document-state-reasons  (1setOf type2 keyword)  Document Description attribute	2

8.8	job-state-message (text(MAX)) Job Description attribute	2

8.9	document-state-message (text(MAX)) Document Description attribute	2

8.10	document-message (text(127)) Document Description	2

8.11	document-printer-up-time (integer(1:MAX)) Document Description	2

9	Printer attributes	2

9.1	document-creation-attributes-supported (1setOf type2 keyword)	2

9.2	printer-state-reasons (1setOf type2 keyword)	2

10	New Values for Existing Printer Description Attributes	2

10.1	operations-supported (1setOf type2 enum)	2

11	Conformance Requirements	2

11.1	Printer conformance requirements	2

11.2	Client conformance requirements	2

12	Normative References	2

13	Informative References	2

14	IANA Considerations	2

14.1	Attribute Registrations	2

14.2	Additional Enum Attribute Value Registrations	2

14.3	Operation Registrations	2

15	Internationalization Considerations	2

16	Security Considerations	2

17	Contributors	2

18	Acknowledgments	2

19	Author’s Address	2

20	Appendix A: Document operations compared to Job operations	2

21	Annex B: Change Log (informative)	2

Table of Tables

Table 1 - List of Document operations	2

Table 2 - Job Creation and Document Creation operations	2

Table 3 - Client-supplied Document Creation and Validation Request Signatures	2

Table 4 - Printer-returned Document Creation and Validation Response Signatures	2

Table 5 - Document State Table for the Set-Document-Attributes operation	2

Table 6 - Operation Attributes supplied at the Job and Document Levels in Creation Requests	2

Table 7 - Operation attributes returned in Job and Document Creation operation responses	2

Table 8 - Job Template and Document Template attributes	2

Table 9 - Job and Document Description attributes	2

Table 10 - "job-state" and "document-state" attribute values	2

Figure 1 - The “job-state” Job Description attribute and the Job object life cycle	2

Figure 2 - The “document-state” Document Description attribute and the Document object life Cycle	2

Table 11 - "job-state-reasons" and "document-state-reasons" attribute values	2

Table 12 - Printer attributes (Job Template and Printer Description)	2

Table 13 – Operation-id assignments	2

Table 14 - Document operations compared to Job operations	2

Table of Figures

Figure 1 - The “job-state” Job Description attribute and the Job object life cycle	2

Figure 2 - The “document-state” Document Description attribute and the Document object life Cycle	2

# Introduction and Summary

This IPP specification extends the IPP Model and Semantics [RFC2911] object model by defining a Document object.  The [RFC2911] Job object is extended to contain one or more Document objects which are passive objects operated on by the Job.  The Document object is created by the [RFC2911] Send-Document and Send-URI operations and represents an Input Document to the Job.  This specification also adds a Document Attributes group (and tag) to these Send-Document and Send-URI requests.  The Document Attributes group contains any Document Template attributes to be applied to this Document object being created, overriding any corresponding Job Template attribute supplied at the job level (including the “document-overrides” operation attributes at the Job or Document level).  A new Create-Document operation allows a client to supply the Document Attributes without the data (like Create-Job does for a Job object).  Then the client can send the data content to that Document object using the new Send-Data operation.

This specification also defines seven new operations for Document objects once they have been created: Cancel-Current-Document, Cancel-Document, Get-Document-Attributes, Get-Documents, Delete-Document, Set-Document-Attributes, and Validate-Document.

This specification also lists all of the attributes defined in other IPP specifications to show their relationship to corresponding attributes defined in this IPP specification for use with the Document object.  The full semantics of the “document-state”, “document-state-reasons”, and “document-state-message” Document Description attributes are given along side of the corresponding [RFC2911] “job-state”,  “job-state-reasons”, and “job-state-message” Job Description attributes.

The intent is to propose this specification as an IPP extension and which will be referenced as part of the PWG Semantic Model.  The purpose for specifying the Document object is so that the print industry can have a common semantic specification for use in IPP, the PWG Semantic Model [pwg-sm], the PWG Print Service Interface (PSI) project [pwg-psi], and the Free Standards Group Job Ticket API [fsg-jt] which all have a Document object.

## Scope

Part of the extension is to allow the client to supply printing instructions, called Document Template attributes, when creating each Document object.  An IPP Printer object implementation MAY support most of the existing Job Template attributes as Document Template attributes as well.  (The PWG Semantic Model [pwg-sm] currently calls the Job Template attributes and Document Template attributes simply “Job Processing” and “Document Processing” attributes).  Note:  There are a number of footnotes which relate this IPP specification to the PWG Semantic model and other PWG projects, such as the Print Service Interface (PSI) [pwg-psi].

The Document object is a passive object and is operated upon by the Job.  The client uses the new Create-Document operation or either of the existing Send-Document and Send-URI operations to create a Document object.  In addition there are 8 new operations that affect the Document object after it has been created as summarized in Table 1.

Table 1 - List of Document operations

| Operation Name | Printer support | Brief Description |
| --- | --- | --- |
| Create-Document | REQUIRED | Creates a Document object with Document attributes, but no data. |
| Send-Data | REQUIRED | Sends data to the Document object created by Create-Document. |
| Send-Document * | REQUIRED | Creates a Document object with Document attributes and data. |
| Send-URI * | OPTIONAL | Creates a Document object with Document attributes and a URL reference to the data (print-by-reference). |
| Validate-Document | REQUIRED | Validate that the supplied Document attributes are supported. |
| Get-Document-Attributes | REQUIRED | Get requested attributes of specified Document in the specified Job. |
| Get-Documents | REQUIRED | Get requested attributes of all of the Documents in the specified Job. |
| Set-Document-Attributes | OPTIONAL | Set the supplied Document attributes on the specified Document. |
| Cancel-Document | REQUIRED | Cancel the specified Document in the specified Job, but don’t remove it. |
| Cancel-Current-Document | OPTIONAL | Cancel the current document that the Printer is printing. |
| Delete-Document | OPTIONAL | Remove the specified Document object and its data from the Job. |

* The existing IPP/1.1 operations defined in [RFC2911] with an extension so the client can supply Document Template attributes in a new attribute Group in the request.

This specification also specifies the life cycle of the Document object by defining the “document-state”, “document-state-reasons”, and “document-state-message” attributes.  Finally, the analogous Document Descriptions attributes that correspond to the Job Description attributes are defined.

# Terminology

This section defines terminology used throughout this document.

## Conformance Terminology

Capitalized terms, such as MUST, MUST NOT, REQUIRED, SHOULD, SHOULD NOT, MAY, NEED NOT, and OPTIONAL, have special meaning relating to conformance as defined in RFC 2119 [RFC2119] and [RFC2911] section 12.1.  If an implementation supports the extension defined in this document, then these terms apply; otherwise, they do not.  These terms define conformance to this document (and [RFC2911]) only; they do not affect conformance to other documents, unless explicitly stated otherwise.  To be more specific:

REQUIRED - an adjective used to indicate that a conforming IPP Printer implementation MUST support the indicated operation, object, attribute, attribute value, status code, or out-of-band value in requests and responses.  See [RFC2911] “Appendix A - Terminology for a definition of “support”.  Since support of this entire Document Object specification is OPTIONAL for conformance to IPP/1.1, the use of the term REQUIRED in this document means “REQUIRED if this OPTIONAL Document Object  specification is implemented”.

RECOMMENDED - an adjective used to indicate that a conforming IPP Printer implementation is recommended to support the indicated operation, object, attribute, attribute value, status code, or out-of-band value in requests and responses.  Since support of this entire Document Object specification is OPTIONAL for conformance to IPP/1.1, the use of the term RECOMMENDED in this document means “RECOMMENDED if this OPTIONAL Document Object  specification is implemented”.

OPTIONAL - an adjective used to indicate that a conforming IPP Printer implementation MAY, but is NOT REQUIRED to, support the indicated operation, object, attribute, attribute value, status code, or out-of-band value in requests and responses.

## Other Terminology

This document uses the same terminology as [RFC2911], such as “client”, “Printer”, “attribute”, “attribute value”, “keyword”, “Job Template attribute”, “Operation attribute”, “operation”, “request”, “response”, and “support” with the same meaning.  In addition, the following terms are defined for use in this document:

Document object - the object to represent the logical concept of a document in a Job.  A Job object can contain one or more Document objects.

Job Creation operation - operations that create Job objects, specifically: Print-Job, Print-URI, and Create-Job as defined in [RFC2911].

Document Creation operation - operations that create Document objects.  Specifically: Create-Document, Print-Job, Print-URI, Send-Document and Send-URI.  Note:  the Print-Job and Print-URI are both Job Creation and Document Creation operations, since they create both Job and a Document object at the same time.  Also this specification does not extend the Print-Job and Print-URI operations.  However, this specification does extend the Send-Document and Send-URI operations to allow the client to supply Document Template attributes in each request.

Job Level - attributes supplied in the Print-Job, Print-URI, and Create-Job are referred to as being supplied at the Job Level.  Attributes supplied at the Job Level are a mixture of Job attributes and Document attributes.  Job attributes apply to the Job as a whole, while Document attributes provide defaults for the Document objects in case the client does not supply a Document attribute at the Document Level for a Document object.  Put another way, Document objects inherit Document attributes from the Job object which are used if the client didn’t supply a corresponding attribute for the Document object.

Document Level - attribute supplied in the Send-Document, Send-URI, and Create-Document operations are referred to as being supplied at the Document Level.  Note: attribute supplied in the Print-Job and Print-URI requests are considered as being supplied at the Job Level only, not also at the Document Level, even though Print-Job and Print-URI also create Document objects.

Attribute Precedence - the precedence rules which dictate which attribute value takes precedence when the same attribute is supplied at different points in the work flow with different values, such as at both the Job Level and the Document Level.  See section 5.

Job Attribute - an Operation attribute or a Job Template attribute that applies to the Job (object) as a whole.  For example, “job-name” is an Operation attribute and “job-priority” is a Job Template attribute.

Document Template attribute - an attribute that the client supplies in a Create-Document, Send-Document, or Send-URI operation request in the Document Attributes group.  For example, “media”.

Document attribute - an Operation attribute or a Document Template attribute that applies to a Document object as a whole.  For example, “document-format” is an Operation attribute and “media” is a Document Template attribute.

Table 2 shows this terminology as applied to the six operations.  Note that Print-Job and Print-URI are both Job Creation operations and Document Creation operations, though they only create single Document jobs.  In other words, for Print-Job and Print-URI, there is only one document per job, so whether the attributes are considered to be submitted at the Job Level and then inherited by the Document object, is moot, since these operations do not have a separate Document group in which the client can supply Document attributes.  Note also that for the “Level” concept, no operation supplies attributes at more than one level.

Table 2 - Job Creation and Document Creation operations

| Job Creation operation | Document Creation operation | Operation | Attribute Group in Request |  | Level |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Job | Document |  |
| yes |  | Create-Job | Job attributes and Document attributes |  | Job Level |
| yes | yes | Print-Job | Job attributes and Document attributes |  |  |
| yes | yes | Print-URI | Job attributes and Document attributes |  |  |
|  | yes | Create-Document |  | Document attributes | Document Level |
|  | yes | Send-Document |  | Document attributes |  |
|  | yes | Send-URI |  | Document attributes |  |

Document Overrides - a way for a client to specify attributes to apply to specified documents in a multi-document job.  Specifically, as member attributes of the “document-overrides” Job Template attribute in a Job Creation operation or the “document-overrides” operation attribute in a Document Creation operation as defined in [override].  Note:  The Document Template attributes supplied in a Create-Document, Send-Document, or Send-URI operation also “override” any Document Template attributes supplied in the Job Creation operation.

# Document Operations

The Document object semantics are as parallel to the Job object semantics as possible.  Therefore, most of the Job operations have analogous Document operations as shown in Table 14 in section “Appendix A: Document operations compared to Job operations”.  Operations that can apply only to an entire job, have no analogous document operation as indicated.  This extension adds 8 new Document operations that operate on the Document object once it is created by the new Create-Document operation or either of the existing two operations: Send-Document and Send-URI operations.

Note:  By Document object we mean the object once it become a contained part of a Job object, not before or after.  So in this specification a Document object does not represent data in a repository.

The signatures of the operation requests that create and/or validate Document objects are very similar as shown in Table 3 and defined in section 3.1 through section 3.5.  The entries in the table show what a client MUST, MAY, or MUST NOT supply in the indicated operation requests.

Table 3 - Client-supplied Document Creation and Validation Request Signatures

|  | Create-Document | Send-Data | Send-Document | Send-URI | Validate-Document |
| --- | --- | --- | --- | --- | --- |
| Target job | MUST | MUST | MUST | MUST | MAY |
| “input-document-number” | MAY | MUST | MAY | MAY | MAY |
| “document-uri” | MUST NOT | MUST NOT | MUST NOT | MUST | MAY |
| Document Template attributes | MAY | MUST NOT | MAY | MAY | MAY |
| Data | MUST NOT | MUST | MUST | MUST NOT | MUST NOT |

The entries in Table 4 show what a Printer MUST, MAY, or MUST not support and return in the indicated operations responses.

Table 4 - Printer-returned Document Creation and Validation Response Signatures

|  | Create-Document | Send-Data | Send-Document | Send-URI | Validate-Document |
| --- | --- | --- | --- | --- | --- |
| Status code/message | MUST | MUST | MUST | MUST | MUST |
| Unsupported Attributes | MUST, if any | MUST, if any | MUST, if any | MUST, if any | MUST, if any |
| Job status attributes | MUST | MUST | MUST | MUST | MUST NOT |
| Document status attributes: |  |  |  |  |  |
| “document-number” | MUST | MUST | MUST | MUST | MUST NOT |
| “document-id-uri” | MUST | MUST | MUST | MUST | MUST NOT |
| “document-state” | MUST | MUST | MUST | MUST | MUST NOT |
| “document-state-reasons | MUST | MUST | MUST | MUST | MUST NOT |
| “document-state-message | MAY | MAY | MAY | MAY | MUST NOT |

## Create-Document Operation

This REQUIRED operation allows a client to create a Document object, supply Document attributes, and have the Printer validate those attributes without sending any document content data.  If the Printer rejects the request, the client won’t have sent the possibly time-consuming data.  The client MUST perform the operations in the following order:  (1) Create-Job to create the Job object, (2) Create-Document to create the Document object, (3) Send-Data (see section 3.2) to send the content data for the Document object.

The client can supply multiple documents for a single Job using pairs of Create-Document and Send-Data operations.  The order of the corresponding Send-Data operations is not important, so that the client can send pairs of Create-Document and Send-Data operations or can send a number of Create-Document operations followed by the corresponding Send-Data operations.  The latter approach is useful if the client wants to make sure that all Documents will be accepted by the Printer before sending any of the Document data to the Printer.

If a Printer supports the Document object, the Printer:

- MUST also support the Create-Document and Send-Data operations

- MUST support the "multiple-operation-time-out" Printer Description attribute (see [RFC2911] section 4.4.31).

- MUST support the "multiple-document-jobs-supported" Printer Description attribute (see [RFC2911] section 4.4.16) with a ‘true’ value, that is the Printer MUST support multiple document jobs.

- MUST support the "multiple-document-handling" Job Template job attribute with at least one value (see [RFC2911] section 4.2.4) and the associated "multiple-document-handling-default" and "multiple-document-handling-supported" Job Template Printer attributes (see [RFC2911] section 4.2).

- MUST support the Attribute Precedence semantics specified in section 5 for the levels that the Printer supports.

The Printer sets the value of the Document’s "document-state" Document Description attribute to ‘pending’, if the Printer accepts the Create-Document request.  A Printer MAY set the 'job-data-insufficient' value of the Document’s "document-state-reason" Document Description attribute to indicate that processing cannot begin until sufficient data has arrived and set the "document-state" to 'pending'.  A non-spooling printer that doesn't implement the 'pending' document state may even set the "document-state" to 'processing', even though there is not yet any data to process.  See sections 8.5 and 8.7.

Access Rights: The authenticated user (see [RFC2911] section 8.3) performing this operation must either be the job owner (as determined in the Create-Job operation) or an operator or administrator of the Printer object (see [RFC2911] sections 1 and 8.5).  Otherwise, the Printer MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

### Create-Document Request

The client supplies the following groups as part of the Create-Document Request:

Group 1: Operation Attributes

Natural Language and Character Set:

First, the client MUST supply the "attributes-charset" and "attributes-natural-language" Operation attributes as described in [RFC2911] section 3.1.4.1.  The Printer object MUST copy these values to the corresponding Job Description attributes described in [RFC2911] sections 4.3.19 and 4.3.20.

Target:

Next, the client MUST supply either (1) the "printer-uri" (uri) plus "job-id" (integer(1:MAX)) or (2) the "job-uri" (uri) operation attribute(s) which define the target Job object for this operation as described in [RFC2911] section 3.1.5.  The client MAY supply the remaining Operation attributes in any order following:

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [RFC2911] section 8.3.

#### input-document-number (integer(1:MAX)) 	operation attribute

The client MAY supply the “input-document-number” Operation attribute to indicate which Document object in the Job to which this Create-Document request is to apply.  If the Printer supports this attribute, the Printer MUST also support the corresponding “document-number” (text(127)) Document Description attribute (see section 8.1) and copy this value to the “document-number” Document Description attribute.  Usually the client omits this Operation attribute, so that the Printer assigns the next integer value starting at 1 for each Job.  A client that is creating multiple Document objects in parallel MAY wish to supply this attribute in order to control the order of the Documents in the job.  If supplied, the Printer MUST copy the value to the Document object’s “document-number” Document Description attribute.  If the Job already contains a Document with the same “document-number” value as the one supplied by this attribute, the Printer MUST reject the request and return the ‘client-error-client-error-conflicting-attributes (see [RFC2911] section 13.1.4.15) and the “input-document-number” and “document-number” attributes.

#### last-document (boolean)	operation attribute

The client MUST supply this attribute (as in the Send-Document and Send-URI operation requests).  The Printer object MUST support this attribute.  This attribute is a boolean flag that the client supplies with a 'true' value if this is the last document for the Job, 'false' otherwise.

#### document-message (text(127))	operation attribute

The client MAY supply this operation attribute.  If the Printer supports this attribute, the Printer MUST also support the corresponding “document-message” (text(127)) Document Description attribute (see section 8.10) and copy this value to the “document-message” Document Description attribute.  The end user can use this Document Description attribute as a message to the operator and the operator can use it as a message to the user.  The Printer MAY print this message on a document start sheet for this document, if any.

Additional Operation attributes:

Any of the other Operation attributes indicated in the “Level Supplied” column labeled with a “y” in the “Doc” column in Table 6, such as “document-name” and “document-format”.  These operation attributes are said to be supplied at the Document Level and apply to the Document object, overriding any operation attributes supplied at the Job Level.  For example, if “document-format” = ‘application/postscript’ at the Job Level and “document-format” = ‘text/plain’ in this Create-Document request, this Document will be ‘text/plain’.  If the next Create-Document request for this Job omits the “document-format” operation attribute, that Document will be ‘application/postscript’.

Group 2: Document Template Attributes

Any Document Template attributes in any order:

The client OPTIONALLY supplies a set of Document Template attributes, that is any attributes indicated in Table 8 in the “Defined for use” column labeled with a “y” in the “D” column.  These Document Template attributes are said to be supplied at the Document Level and apply to the Document object, overriding any Document Template attributes supplied at the Job Level.  For example, if “media” = ‘na-letter-white’ at the Job Level and “media” = ‘na-letter-transparency’ in this Create-Document request, this Document will be ‘na-letter-transparency’.  If the next Create-Document operation for this Job omits the “media” operation attribute, that Document will be ‘na-letter-white’ which it inherits from the Job.

If the client is not supplying any Document Template attributes in the request, the client SHOULD omit Group 2 rather than sending an empty group.  However, a Printer object MUST be able to accept an empty group.

The client MUST NOT supply the document data in this operation request.  See Send-Data operation.

Group 3: Subscription Attributes (see [ntfy] section 11.1.3.1):  MUST NOT be present.  Notification is only at the Job Level.

In addition to the MANDATORY parameters required for every operation request (see [RFC2911]), the simplest Create-Document request consists of just the "attributes-charset" and "attributes-natural-language" operation attributes; either (1) the "printer-uri" target operation attribute and the “job-id” or (2) the “job-uri” and nothing else.  In this simple case, the Printer object:

- creates a new Document object and adds it to the specified Job object,

- assigns the lowest document number not yet used by this Job, if the client did not supply an “input-document-number” operation attribute (see section 3.1.1.1).  Copy the document number assigned or supplied to the Document object’s “document-number” Document Description attribute.

- stores a generated Document name in the "document-name" Document Description attribute in the natural language and charset requested (see [RFC2911] section 3.1.4.1) (if those are supported, otherwise using the Printer object's default natural language and charset), and

- at job processing time, uses its corresponding default value attributes for the supported Document Template attributes that were (1) not supplied by the client as IPP attributes and (2) not embedded instructions in the document data.

### Create-Document Response

The Printer object MUST return the following sets of attributes to the client as part of the Create-Job response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [RFC2911] sections 13 and 3.1.6.

The Printer MUST reject the operation:

If the client supplies the “ipp-attribute-fidelity” operation attribute (see section 6.2.1) with a ‘true’ value and the client supplies any unsupported and/or conflicting Job Template or Document Template attributes.

If the client omits or supplies the “ipp-attribute-fidelity” operation attribute (section 6.2.3) with a ‘false’ value and supplies keyword values of the “document-mandatory-attributes” operation attribute that are unsupported and/or conflicting.

Otherwise, the Printer MUST accept the operation and ignore or substitute unsupported and/or conflicting attribute values supplied by the client.  See the Implementer's Guide [RFC3196] for a complete description of the suggested steps for processing the attributes in a create request.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.2.

Group 2: Unsupported Attributes

See [RFC2911] section 3.1.7 for details on returning Unsupported Attributes.

The value of the "ipp-attribute-fidelity" and “document-mandatory-attributes” operation attributes supplied by the client does not affect what attributes the Printer object returns in this group.  The value of these two operation attributes only affects whether the Create-Job operation is accepted or rejected.  If the Document is accepted, the client may query the Document object using the Get-Document-Attributes operation (see section 3.5) requesting the unsupported attributes that were returned in the create response to see which attributes were ignored (not stored on the Document object) and which attributes were stored with other (substituted) values.

Group 3: Job Object Attributes

In addition, the Printer MUST return the same Job status attributes that [RFC2911] REQUIRES for the Send-Document and Send-URI operation responses (which are the same as for the Print-Job response - [RFC2911] section 3.2.1.2):

"job-uri" (uri) - see [RFC2911] section 4.3.1

"job-id" (integer(1:MAX)) - see [RFC2911] section 4.3.2

"job-state" (type1 enum) - see [RFC2911] section 4.3.7

"job-state-reasons" (1setOf type2 keyword) - see [RFC2911] section 4.3.8

The Printer MAY return:

"job-state-message" (text(MAX)) - see [RFC2911] section 4.3.9

"number-of-intervening-jobs" (integer(0:MAX)) - see [RFC2911] section 4.3.15

Group 4: Document Object Attributes

document-number (integer(1:MAX))	Document Description attribute

The Printer MUST return the document number of this Document object that was either assigned by the Printer (see section 3.1.1) or supplied by the client (see section 3.1.1.1).

#### document-id-uri (uri)	Document Description attribute

The Printer MUST return the document id of this Document object as a URI as assigned by the Printer (see section 8.2).

#### document-state (type1 enum)	Document Description attribute

The Printer MUST return the state of this Document object (see section 8.5).

#### document-state-reasons (1setOf type2 keyword)	Document Description attribute

The Printer MUST return the associated state reasons for this Document (see section 8.7).

#### document-state-message (text(MAX))	Document Description attribute

The Printer MAY return the localized description of the “document-state” and “document-state-reasons” values for this Document (see section 8.9).

Send-Data Operation

This REQUIRED operation allows the client to create a Document object using the Create-Document operation (see section 3.1) that the Printer validates before the client sends any document content data.  Then the client can skip sending the data to the Printer, if the Printer rejects the Document Creation operation.

Since the Create-Document and the Send-Data operation that follows could occur over an arbitrarily long period of time for a particular Document, a client MUST send the Send-Data operation within an IPP Printer defined minimum time interval after the receipt of the previous request for the job.  If a Printer object supports the Document object, the Printer object MUST support the "multiple-operation-time-out" attribute (see [RFC2911] section 4.4.31).  This attribute indicates the minimum number of seconds the Printer object will wait for the next send operation before taking some recovery action.  See the description under Send-Document in [RFC2911] section 3.3.1, for the possible Printer actions, if the client does not supply a Send-Data request in time.

Access Rights: The authenticated user (see [RFC2911] section 8.3) performing this operation must either be the job owner (as determined in the Create-Job operation) or an operator or administrator of the Printer object (see [RFC2911] sections 1 and 8.5).  Otherwise, the Printer MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

### Send-Data request

The following attribute sets are part of the Send-Data Request:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in section [RFC2911] 3.1.4.1.

Target:

The “document-number” (integer(1:MAX)) operation attribute MUST be supplied immediately after either (1) the "printer-uri" (uri) plus "job-id" (integer(1:MAX)) or (2) the "job-uri" (uri) and “operation attribute(s) which define the target Document object for this operation as described in section [RFC2911] 3.1.5.  If the client omits the “document-number” operation attribute, the Printer MUST reject the request and return the ‘client-error-bad-request’ status code.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [RFC2911] section 8.3.

Group 2: Document Content

The client MUST supply the document data in-line to be processed for this Document, just like supplying data in the Send-Document and Print-Job requests.

### Send-Data Response

The following sets of attributes are part of the Send-Data Response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [RFC2911] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.2.

Group 2: Unsupported Attributes

See [RFC2911] section 3.1.7 for details on returning Unsupported Attributes.

Group 3: Job Object Attributes

The Printer MUST return the same set of Job status attributes as for the Print-Job, Send-Document, and Create-Document responses (see section 3.1.2).

Group 4: Document Object Attributes

The Printer MUST return the same set of Document status attributes as for the Create-Document operation response (see section 3.1.2)

ISSUE 01:  OK for the Send-Data operation to return Document status attributes (as well as Job status attributes) so at to be like the other Document operations?

## Send-Document Operation

This REQUIRED operation allows a client to submit multiple documents in a single Job by making a Send-Document request for each Document.  In this specification, the Send-Document creates a Document object ([RFC2911] did not have a Document object).  A Printer MUST support this operation with the semantics specified in [RFC2911] and with the extension defined in this specification, since this specification REQUIRES the Printer to support multiple document jobs.

See [RFC2911] section 3.3.1 for the semantics of the Send-Document operation not reproduced here.

Access Rights: The authenticated user (see [RFC2911] section 8.3) performing this operation must either be the job owner (as determined in the Create-Job operation) or an operator or administrator of the Printer object (see [RFC2911] sections 1 and 8.5).  Otherwise, the Printer MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

### Send-Document Request

The following attribute sets are part of the Send-Document Request:

Group 1: Operation Attributes (as in [RFC2911])

As specified in [RFC2911] section 3.3.1 Send-Document request, plus any extensions indicated in Table 6.  For example, “compression”, “document-format”, and “document-name”.  So this group is the same as for Create-Document (see section 3.1.1).

Group 2: Document Attributes (new - not in [RFC2911])

The client OPTIONALLY supplies a set of Document Template attributes as listed in Table 8.  For example, “media” and “number-up”.  If the client is not supplying any Document Template attributes in the request, the client SHOULD omit Group 2 rather than sending an empty group.  However, a Printer object MUST be able to accept an empty group.

Group 3: Document Content (as in [RFC2911] Group 2)

The client MUST supply the document data in-line to be processed for this Document if the "last-document" flag is set to 'false'.  However, since a client might not know that the previous Document object created with a Create-Job, Send-Document, or Send-URI operation was the last document (i.e., the "last-document" attribute was set to 'false'), it is legal to send a Send-Document request with no document data where the "last-document" flag is set to 'true'.  For this case, the Printer MUST NOT increment the value of the Job object's "number-of-documents" attribute, since no Document object was added to the job.  In addition, it is not an error for a client to submit a job with no actual document data, i.e., only a single Create-Job and Send-Document request with a "last-document" operation attribute set to 'true' with no document data; it is only an empty job which is scheduled and processed normally.

### Send-Document Response

The following sets of attributes are part of the Send-Document Response:

Group 1: Operation Attributes (as in [RFC2911] Group 1)

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [RFC2911] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.2.

Group 2: Unsupported Attributes (as in [RFC2911] Group 2)

See [RFC2911] section 3.1.7 for details on returning Unsupported Attributes.

Group 3: Job Object Attributes (as in [RFC2911] Group 3)

The Printer MUST return the same set of Job status attributes as for the Print-Job, Send-Document, and Create-Document responses (see section 3.1.2).

Group 4: Document Object Attributes (new - as in Create-Document Group 3 - see section 3.1.2)

The Printer MUST return the same set of Document status attributes as for the Create-Document operation response (see section 3.1.2).

## Send-URI Operation

This OPTIONAL operation has the same request and response semantics as the Send-Document operation (see section 3.3) except that a client MUST supply a URI reference ("document-uri" operation attribute) rather than the document data itself.  If a Printer object supports this operation, clients can use both Send-URI or Send-Document operations to add new documents to an existing multi-document Job object.  However, if a client needs to indicate that the previous Send-URI or Send-Document was the last document,  the client MUST use the Send-Document operation with no document data and the "last-document" flag set to 'true' (rather than using a Send-URI operation with no "document-uri" operation attribute).

If a Printer object supports this operation, it MUST also support the Print-URI operation (see [RFC2911] section 3.2.2).

The Printer object MUST validate the syntax and URI scheme of the supplied URI before returning a response, just as in the Print-URI operation.  The IPP Printer MAY validate the accessibility of the document as part of the operation or subsequently (see [RFC2911] section 3.2.2).

## Validate-Document Operation

This REQUIRED operation allows the client to verify capabilities of a Printer against whatever attributes the client supplies in the Validate-Document request in combination with the previously submitted partial Job, if any.  This operation has the same inputs as the Create-Document operation request (section 3.1) except the Printer does not create a new Document object.  The Printer object returns the same status codes, Operation Attributes (Group 1) and Unsupported Attributes (Group 2) as the Create-Document operation (see section 3.1.2).  However, no Job Object Attributes (Group 3) and no Document Object Attributes (Group 4) are returned, since no Document object is created.

The client MAY supply a Job object Target, that MUST be either (1) a “printer-uri” and a “job-id” operation attribute or (2) a “job-uri” operation attribute.  If supplied, the Job Target MUST identify a job that has been created by Create-Job ([RFC2911] section 3.2.4), but not yet closed, that is, the client hasn’t yet supplied the “last-document” operation attribute with a ‘true’ value.  When the client supplies the Job object Target, the Printer MUST validate that an identical Create-Document, Send-Document, or Send-URI operation would have been accepted in the context of the specified Job and its other Document objects.

If the client omits the Job Target by including only the “printer-uri” operation attribute, the Printer performs the same validation as for a Create-Document in which there were no Job attributes supplied in the Create-Job and no other Documents.  The Printer MUST support the Validate-Document operation with and without the Job object Target being present.

In addition to the attributes and groups defined for a Create-Document request (see section 3.1.1), the client MAY supply the “document-uri” operation attribute (defined in the Send-URI operation in [RFC2911] section 3.3.2).  If the client supplies the “document-uri” operation attribute, the Printer MUST validate that the URI scheme is supported and that the referenced data is accessible.

If the client discovers that a Validate-Document operation fails, it has the choice of (1) canceling the entire partially created job, if any, (2) closing the partially created Job as is (with a Send-Document supplying the “last-document” with a ‘true’ value) so that the Printer will process the Job, or (3) adding other Document objects (with Create-Document, Send-Document, and/or Send-URI) and then closing the Job.

By using the Validate-Document operation a client can validate that an identical Create-Document operation (or a Send-Document operation with the document data or a Send-URI operation) would be accepted by the Printer in the context of the partially submitted Job.  The Validate-Document operation also performs the same security negotiation as the Create-Document operation (see [RFC2911[ section 8), so that a client can check that the client and Printer object security requirements can be met before performing a Create-Document, Send-Document, or Send-URI operation.

## Get-Document-Attributes Operation

This REQUIRED operation allows a client to request the values of selected attributes of a Document object.  This operation is almost identical to the Get-Job-Attributes operation (see [RFC2911] section 3.3.4).  The only differences are that the operation is directed at a Document object rather than a Job object and the returned attribute group is a set of Document object attributes rather than a set of Job object attributes.

The Printer MUST return the attributes that a client supplied in the Create-Document, Send-Document, or Send-URI operations, plus any Document Description attributes that the Printer has generated.  The Printer MUST NOT return any Job Level attributes that the Document object inherits, including “document-overrides” (1setOf collection) member attributes (see [override]), and MUST NOT factor out common Document object attributes and return them as Job object attributes.  Thus the Get-Document-Attributes returns to clients the attributes that clients submitted with the Job object and with the Document object.

The client MAY display to the user any Document attributes inherited from the Job object.  However, such a client MUST query both the Job object using Get-Job-Attributes and the Document object using Get-Document-Attributes in order to be able to merge the inherited attributes with the explicitly-supplied Document attributes.  Furthermore, the client MAY factor out common explicitly-supplied Document object attributes and display them once as Job object attributes, instead of with each Document object.  However, any such regularization of the display for users is the policy of the client and is outside the scope of this client-Printer protocol specification.  Note: specify any such regularization or canonicalization policy in the Printer response would have been impossible to agree to.

For Document objects, the possible names of attribute groups are:

- ‘document-template': the subset of the Document Template attributes that apply to a Document object (the “D” column in Table 8) that the implementation supports for Document objects.

- ‘document-description': the subset of the Document Description attributes specified in the “Doc” column of Table 9 that the implementation supports for Document objects.

- 'all': the special group 'all' that includes all attributes that the implementation supports for Document objects.

Since a client MAY request specific attributes or named groups, there is a potential that there is some overlap.  For example, if a client requests, ‘document-name' and ‘document-description', the client is actually requesting the "document-name" attribute once by naming it explicitly, and once by inclusion in the ‘document-description' group.  In such cases, the Printer object NEED NOT return the attribute only once in the response even if it is requested multiple times.  The client SHOULD NOT request the same attribute in multiple ways.

It is NOT REQUIRED that a Document object to support all attributes belonging to a group (since some attributes are OPTIONAL).  However, it is REQUIRED that the Printer support all these group names for the Document object.

### Get-Document-Attributes Request

The following groups of attributes are part of the Get-Document-Attributes Request when the request is directed at a Document object:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.1.

Target:

The “document-number” (integer(1:MAX)) operation attribute MUST be supplied immediately after either (1) the "printer-uri" (uri) plus "job-id" (integer(1:MAX)) or (2) the "job-uri" (uri) operation attribute(s) which define the target Document object for this operation as described in [RFC2911] section 3.1.5.  If the client omits the “document-number” operation attribute, the Printer MUST reject the request and return the ‘client-error-bad-request’ status code.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [RFC2911] section 8.3.

"requested-attributes" (1setOf keyword):

The client OPTIONALLY supplies this attribute.  The Printer MUST support this attribute.   It is a set of attribute names and/or attribute group names in whose values the requester is interested.  If the client omits this attribute, the Printer MUST respond as if this attribute had been supplied with a value of 'all'.

### Get-Document-Attributes Response

The Printer object returns the following sets of attributes as part of the Get-Document-Attributes Response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [RFC2911] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.2.  The "attributes-natural-language" MAY be the natural language of the Document object, rather than the one requested.

Group 2: Unsupported Attributes

See [RFC2911] section 3.1.7 for details on returning Unsupported Attributes.

The response NEED NOT contain the "requested-attributes" operation attribute with any supplied values (attribute keywords) that were requested by the client but are not supported by the Printer. If the Printer object does return unsupported attributes referenced in the "requested-attributes" operation attribute and that attribute included group names, such as 'all', the unsupported attributes MUST NOT include attributes described in the standard but not supported by the implementation.

Group 3: Document Object Attributes

This is the set of requested attributes and their current values.  The Printer ignores (does not respond with) any requested attribute or value which is not supported.  Furthermore, the Printer MUST NOT return Document Template attributes that were not supplied by a client in a Create-Document, Send-Document, or Send-URI request or set by a Set-Document-Attributes request.  So the response reflects what was submitted.  The Printer MAY omit returning attributes that are restricted by the Printer’s security policy in force, including whether the requesting user is the user that submitted the job (job originating user) or not (see [RFC2911] section 8).  However, the Printer MUST respond with the 'unknown' value for any supported attribute (including all REQUIRED attributes) for which the Printer does not know the value, unless it would violate the security policy.  See the description of the "out-of-band" values in the beginning of [RFC2911] Section 4.1.

## Get-Documents Operation

This REQUIRED operation allows a client to retrieve the list of Document objects belonging to the target Job object.  The client MAY also supply a list of Document attribute names and/or attribute group names.  A group of Document object attributes will be returned for each Document object in the Job.

This operation is similar to the Get-Document-Attributes operation (see section 3.5), except that this Get-Documents operation returns attributes from all Document objects contained in the Job object, instead of from a single selected Document object in the Job object.  As with the Get-Document-Attributes operation the Printer MUST only return attributes that were submitted by a client when the Document object was created by the Create-Document, Send-Document, or Send-URI operations and possibly modified by the Set-Document-Attributes operation (see section 3.8).

### Get-Documents Request

The client submits the Get-Documents request to a Printer object.  The Get-Documents is similar to the Get-Jobs operations (see [RFC2911] section 3.2.6) except that there are no equivalents to the “which-jobs” and “my-jobs” operation attributes.

The following groups of attributes are part of the Get-Documents Request:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.1.

Target:

Either (1) the "printer-uri" (uri) plus "job-id" (integer(1:MAX)) or (2) the "job-uri" (uri) operation attribute(s) which define the target Job object for this operation as described in [RFC2911] section 3.1.5.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [RFC2911] section 8.3.

"limit" (integer(1:MAX)):

The client OPTIONALLY supplies this attribute.  The Printer object MUST support this attribute. It is an integer value that determines the maximum number of Documents that a client will receive from the Printer.  The limit is a "stateless limit" in that if the value supplied by the client is 'N', then only the first 'N' Documents are returned in the Get-Documents Response.  There is no mechanism to allow for the next 'M' Documents after the first 'N' Documents.  If the client does not supply this attribute, the Printer object responds with all Documents.

"requested-attributes" (1setOf type2 keyword):

The client OPTIONALLY supplies this attribute.  The Printer object MUST support this attribute.  It is a set of Document attribute names and/or attribute groups names in whose values the requester is interested.  This set of attributes is returned for each Document object that is returned.  The allowed attribute group names are the same as those defined in the Get-Document-Attributes operation in section 3.6.1.

If the client does not supply this attribute, the Printer MUST respond as if the client had supplied this attribute with the two values:

‘document-number’ (see section 8.1)

‘document-id-uri’ (see section 8.2).

### Get-Documents Response

The Printer object returns all of the Document objects up to the number specified by the "limit" attribute that match the criteria as defined by the attribute values supplied by the client in the request.  If the client requests any Document attributes at all, there is a set of Document Object Attributes returned for each Document object.

It is not an error for the Printer to return 0 Documents. If the response returns 0 Documents because there are no Documents in the Job, then the status code for 0 Documents MUST be 'successful-ok'.

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [RFC2911] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.2.

Group 2: Unsupported Attributes

See [RFC2911] section 3.1.7 for details on returning Unsupported Attributes.

The response NEED NOT contain the "requested-attributes" operation attribute with any supplied values (attribute keywords) that were requested by the client but are not supported by the Printer. If the Printer object does return unsupported attributes referenced in the "requested-attributes" operation attribute and that attribute included group names, such as 'all', the unsupported attributes MUST NOT include attributes described in the standard but not supported by the implementation.

Groups 3 to N: Document Object Attributes

The Printer object responds with one set of Document Object Attributes for each returned Document object.  The Printer object ignores (does not respond with) any requested attribute or value which is not supported or which is restricted by the security policy in force, including whether the requesting user is the user that submitted the job (job originating user) or not (see [RFC2911] section 8).  However, the Printer object MUST respond with the 'unknown' value for any supported attribute (including all REQUIRED attributes) for which the Printer object does not know the value, unless it would violate the security policy.  See the description of the "out-of-band" values in the beginning of [RFC2911] section 4.1.

The Printer MUST return the Groups for the requested Job in order of increasing “document-number” Document Description attribute value.

## Set-Document-Attributes Operation

This OPTIONAL operation allows a client to set the values of the attributes of a Document object after the Document object was submitted to the Printer.  In the request, the client supplies the set of Document keyword attribute names and values that are to be set.  In the response, the Printer returns success or rejects the entire request with indications of which attribute or attributes could not be set.

This operation is almost identical to the Set-Job-Attributes operation and follows the same rules for validation (see [set-ops] section 4.2).  The only differences are that the Set-Document-Attributes operation is directed at a Document object rather than a Job object.  As with the Set-Job-Attributes operation,  The Set-Document-Attributes operation can add an attribute to the Target object, accepts the 'delete-attributes' out-of-band value to remove an attribute from the Target object.  In addition the validation is the same as the Create-Document, Send-Document, and Send-URI operations that create Document objects, i.e., the Printer validation depends on the "xxx-supported" Printer Description attributes (see [RFC2911] section 3.1).

If a client supplies a document attribute in a Set-Document-Attributes request that the Printer supports, and the document was originally submitted without supplying that attribute, the Printer adds the attribute to the Document object.

If the client supplies a document attribute with the "out-of-band" value 'delete-attribute' (see [set-ops] section 8.2), then the Printer MUST remove the attribute and all of its values from the Document object, if present.  The semantic effect of the client supplying the 'delete-attribute' value in a Set-Document-Attributes operation MUST be the same as if the attribute had not been supplied with the Document object in the Document Creation operation, i.e., the Printer applies its default attribute or behavior with lower precedence that the PDL (see the beginning of [RFC2911] section 4.2 and [RFC2911] 3.2.1.1).  Any subsequent query of the Document object using Get-Document-Attributes or Get-Documents MUST NOT return any attribute that has been deleted using the 'delete-attribute' out-of-band value.  However, a client can re-establish such a deleted Document attribute with any supported value(s) using a subsequent Set-Document-Attributes operation.

If the client supplies an attribute in a Set-Document-Attributes request with the 'delete-attribute' value and that attribute is not present on the Document object, the Printer ignores that supplied attribute in the request, does not return the attribute in the Unsupported Attributes group, and returns the 'successful-ok' status code, if there are no other problems with the request.

The validation of the Set-Document-Attributes request is performed by the Printer as if the document had been submitted originally with the new attribute values (and the deleted attributes removed) and with "ipp-attribute-fidelity" set to 'true', i.e., all modified attributes Document attributes and values MUST be supported in combination with the Document attributes not modified.  If such a Document Creation operation would have been accepted, then the Set-Document-Attributes MUST be accepted.  If such a Document Creation operation would have been rejected, then the Set-Document-Attributes MUST be rejected and the Document MUST be unchanged.  In addition, if any of the supplied attributes are not supported, are not settable, or the values are not supported, the Printer object MUST reject the entire operation; the Printer object MUST NOT partially set some of the supplied attributes.  In other words, after the operation, all the supplied attributes MUST be set or none of them MUST be set, thus making the Set-Document-Attributes an atomic operation.

The Printer MUST accept or reject this operations when the Document's READ-ONLY "document-state" attribute has the values shown in Table 5.  The document's current state MUST affect whether thetPrinter accepts or rejects the request.  However, the Set-Document-Attributes operation MUST NOT change the state of the Document object (since the Document is a passive object and the Document state is a subset of the Job State).  For example, in the case where the operation creates a request for unavailable resources, the Job (but not the Document) transitions to a new state.  Table 5 shows the allowed behaviors in each document state.

Table 5 - Document State Table for the Set-Document-Attributes operation

| Current “document-state” | New “document-state” (no change) | Printer’s response status code and “action”: |
| --- | --- | --- |
| 'pending' | 'pending' | 'successful-ok' |
| 'pending' | 'pending' | 'successful-ok' - needed resources are not ready (the Job MAY change state, but the Document MUST NOT). |
| 'processing' | 'processing' | 'successful-ok' or 'client-error-not-possible' depending on implementation, including the attributes being set, whether the document has started marking media, etc. |
| 'completed' | 'completed' | 'client-error-not-possible' |
| 'canceled' | 'canceled' | 'client-error-not-possible' |
| 'aborted' | 'aborted' | 'client-error-not-possible' |

This operation MUST NOT change the value of attributes not specified in the operation unless the definition of the attribute explicitly specifies such side-effects.  In general, Document attribute definitions that are settable will not define side-effects on other attributes that are settable, only side effects on READ-ONLY attributes, if any.

### Settable and READ-ONLY Document Description attributes

If the Printer supports the Set-Document-Attributes operation, then it SHOULD support setting of:

all Document Template document ("xxx") attributes

that the implementation supports (see the column labeled “D” in Table 8).

Almost all Document Description attributes (see [RFC2911] section 4.3) MUST NOT be settable, i.e., they are defined to be READ-ONLY.  Only Document Description attributes marked with “(r/w) in Table 8 MAY be settable using Set-Document-Attributes.  The Printer MUST reject a Set-Document-Attributes request that attempts to set any attributes in Table 8 that is not marked “(r/w).

Note:  From now on, all extensions that define new object attributes will indicate whether or not the attributes are READ-ONLY, by including the "READ-ONLY" adjective in their descriptions and/or explicitly stating whether they MAY be settable.

Access Rights: The authenticated user (see [RFC2911] section 8.3) performing this operation must either be the job owner (as determined in the Job Creation operation) or an operator or administrator of the Printer object (see [RFC2911] Sections 1 and 8.5).

### Set-Document-Attributes Request

The following sets of attributes are part of the Set-Document-Attributes Request:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.1.

Target:

The client MUST supply the “document-number” (integer(1:MAX)) operation attribute immediately after either (1) the "printer-uri" (uri) plus "job-id" (integer(1:MAX)) or (2) the "job-uri" (uri) operation attribute(s) which define the target Document object for this operation as described in [RFC2911] section 3.1.5.  If the client omits the “document-number” operation attribute, the Printer MUST reject the request and return the ‘client-error-bad-request’ status code.

Note:  There is no way in a single operation for the client to set all of a Job’s Document objects to the same attribute value.  Either (1) the client sets the corresponding attribute in each Document object in the Job with separate Set-Document-Attributes requests, or (2) the client sets the corresponding attribute at the Job Level using the Set-Job-Attributes operation.  Setting the corresponding attribute at the Job Level (or supplying that attribute initially in the Job Creation operation) causes each Document object to inherit the Job Level attribute value, if the Document object doesn’t have a corresponding explicit attribute supplied at the Document Level.  In other words, supplying an attribute at the Job Levels will affect all the Job’s Document objects that don’t have that attribute also explicitly supplied at the Document Level.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [RFC2911] section 8.3.

Group 2: Document Attributes

The client MUST supply a set of Document Template attributes with one or more values (including explicitly allowed out-of-band values) as listed in column labeled “D” in Table 8 - Job Template and Document Template attributes and Document Description attributes flagged with “(r/w)” and listed in the column labeled “Doc” in Table 9 - Job and Document Description attributes, and any attribute extensions supported by the Printer.  The value(s) of each Document attribute supplied in Group 2 replaces the value(s) of the corresponding Document attribute on the target Document object.  For attributes that can have multiple values (1setOf), all values supplied by the client replace all values of the corresponding Document object attribute.

In order to set the Document’s “document-message” Document Description attribute (see section 8.10), the client MUST supply the attribute in Group 2, rather than supplying a “document-message” operation attribute in Group 1.

If the client supplies an "xxx" attribute with the 'delete-attribute' out-of-band value (see [set-ops] section 8.2), the Printer MUST remove the "xxx" attribute from the Document object, if present.

### Set-Document-Attributes Response

The Printer returns the following sets of attributes as part of the Set-Document-Attributes Response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [RFC2911] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.2.

Group 2: Unsupported Attributes

See [RFC2911] section 3.1.7 for details on returning Unsupported Attributes.

If some of the attributes in the operation fail to validate, the Printer MUST reject the operation, MUST NOT change any Document attributes, and MUST return the indicated status code below.  In this group, the Printer MUST also return all attributes that fail to validate.  The following are the reasons that an attribute fails to validate and the value returned for the attribute, along with the indicated status code and order of detection:

- The number of attributes supplied by the client exceeds the maximum number that the Printer supports in a Set-Printer-Attributes request:  return the 'client-error-request-entity-too-large' (see [RFC2911] section 13.1.4.9).

- The Printer doesn't support the attribute:  return the attribute with the 'unsupported' out-of-band attribute value (see [RFC2911] section 3.1.7 and [RFC2910]) and the 'client-error-attributes-or-values-not-supported (see [RFC2911] section 13.1.4.12).

- The attribute is READ-ONLY (in its definition) or is not-settable in this implementation:  return the attribute with the 'not-settable' out-of-band attribute value (see [set-ops] section 8.1) and the 'client-error-attributes-not-settable' status code (see [set-ops] section 7.1).

- The Printer doesn't support the value:  if the attribute in the operation has a single value return it.  If the attribute in the operation is multi-valued, return only those values in a 1setOf that are not supported.  Return the 'client-error-attributes-or-values-not-supported' status code (see [RFC2911] section 13.1.4.12).

- The values of some of the supplied attributes conflict with one another and/or other Document attribute values not being set:  if the conflicting attribute in the operation has a single value return the attribute and the value.  If the attribute in the operation is multi-valued, return only the attribute and those values in a 1setOf that are conflicting with other attributes.  Return the 'client-error-conflicting-attributes' status code (see [RFC2911] section 13.1.4.15).

## Cancel-Document Operation

This REQUIRED operation allows a client to cancel a specified Document in a specified Print Job from the time the Document is created up to, but not including, the time it is completed, canceled, or aborted.  Since a Document might already be printing by the time a Cancel-Document is received, some media sheet pages might be printed before the document processing is actually terminated.

The Cancel-Document operation does not remove the Document from the Job and Printer, but only sets the specified Document’s “document-state” Document Description attribute to ‘canceled’ and the Document’s “document-state-reasons” Document Description attribute to ‘canceled-by-user’ or ‘cancel-by-operator’.  If the Job object is printed again using operations such as the Reprocess-Job operation (see [adm-ops] section 4.1) or the Restart-Job operation (see [RFC2911] section 3.3.7), any canceled Documents are also printed again.  Thus Cancel-Document has the same semantics as Cancel-Job (see [RFC2911] section 3.3.3), which only cancels the processing of the job, and doesn’t delete the Job object itself.  See the Delete-Document operation (section 3.11) to remove the Document object itself immediately from the Job and Printer.

The Cancel-Document operation does not affect the states of any of the other Document objects in the job.  If the Job was processing and there are more Documents to be processed, the Printer does so.  If there are no further Documents to process, the Job moves to the ‘completed’ state as usual whenever there is no more processing to be done.

The Printer MUST accept or reject the request based on the document's current state and transition the document to the indicated new state as follows:

| Current "document-state" | New "document-state" | Printer object's response status code and action: |
| --- | --- | --- |
| 'pending' | 'canceled' | 'successful-ok' |
| 'processing' | 'canceled' | 'successful-ok' |
| 'processing' | 'processing' | 'successful-ok' See Rule 1 |
| 'processing' | 'processing' | 'client-error-not-possible' See Rule 2 |
| 'completed' | 'completed' | 'client-error-not-possible' |
| 'canceled' | 'canceled' | 'client-error-not-possible' |
| 'aborted' | 'aborted' | 'client-error-not-possible' |

Rule 1:  If the implementation requires some measurable time to cancel the document in the 'processing' document states, the Printer object MUST add the 'processing-to-stop-point' value to the document's "document-state-reasons" attribute and then transition the document to the 'canceled' state when the processing ceases (see [RFC2911] section 4.3.8).

Rule 2:  If the Document object already has the 'processing-to-stop-point' value in its "document-state-reasons" attribute, then the Printer object MUST reject a Cancel-Document operation.

Access Rights: The authenticated user (see [RFC2911] section 8.3) performing this operation must either be the job owner or an operator or administrator of the Printer object (see [RFC2911] sections 1 and 8.5).  Otherwise, the Printer MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.

### Cancel-Document Request

The following groups of attributes are part of the Cancel-Document Request:

Group 1: Operation Attributes

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.1.

Target:

The client MUST supply the “document-number” (integer(1:MAX)) operation attribute immediately after either (1) the "printer-uri" (uri) plus "job-id"  (integer(1:MAX)) or (2) the "job-uri" (uri) operation attribute(s) which define the target Document object for this operation as described in section [RFC2911] 3.1.5.  If the client omits the “document-number” operation attribute, the Printer MUST reject the request and return the ‘client-error-bad-request’ status code.

Requesting User Name:

The "requesting-user-name" (name(MAX)) attribute SHOULD be supplied by the client as described in [RFC2911] section 8.3.

#### document-message (text(127))	operation attribute

The client MAY supply this operation attribute.  If the Printer supports this attribute, the Printer MUST also support the corresponding “document-message” (text(127)) Document Description attribute (see section 8.10) and copy this value to the “document-message” Document Description attribute.  The end user can use this Document Description attribute as a message to the operator and the operator can use it as a message to the user.  The Printer MAY print this message on a document start sheet for this document, if any.

The Printer MUST ignore the new “job-message-from-operator” operation attribute (see [adm-ops] section 6) for this operation, since that is a job only attribute and Cancel-Document is a Document operation.

### Cancel-Document Response

The following sets of attributes are part of the Cancel-Document Response:

Group 1: Operation Attributes

Status Message:

In addition to the REQUIRED status code returned in every response, the response OPTIONALLY includes a "status-message" (text(255)) and/or a "detailed-status-message" (text(MAX)) operation attribute as described in [RFC2911] sections 13 and 3.1.6.

Natural Language and Character Set:

The "attributes-charset" and "attributes-natural-language" attributes as described in [RFC2911] section 3.1.4.2.

Group 2: Unsupported Attributes

See [RFC2911] section 3.1.7 for details on returning Unsupported Attributes.

Once a successful response has been sent, the implementation guarantees that the Document will eventually end up in the 'canceled' state.  Between the time of the Cancel-Document operation is accepted and when the document enters the 'canceled' document-state (see section 8.5), the "document-state-reasons" attribute (see section 8.7) SHOULD contain the 'processing-to-stop-point' value which indicates to later queries that although the Document might still be 'processing', it will eventually end up in the 'canceled' state, not the 'completed' state.

## Cancel-Current-Document Operation

This OPTIONAL operation allows a client to cancel either (1) any current Document of the current Job on the target Printer or (2) the specified document in a specified job if they are the current document and job on the Printer.  One of the uses of this operation is by operators in an emergency, where the job-id and document-number might be unknown to the operator.  See section 3.9 for the semantics of canceling a Document and what values the Printer sets the Document’s Document Description attributes.  Since a Document might already be marking by the time a Cancel-Current-Document is received, some media sheet pages might be printed before the document is actually terminated.

If the client does not supply a “job-id” and “document-number” operation attributes, the Printer MUST accept the request and cancel the current document if there is a current document in the ‘processing’ state; otherwise, it MUST reject the request and return the ‘client-error-not-possible’ status code.  If more than one document is in the ‘processing’ or ‘processing-stopped’ states, the one that is marking is canceled and the others are unaffected.

Warning:  On a shared printer, there is a race condition.  Between the time that a user issues this operation and its acceptance, the current document might change to a different document or even a different job.  If the user or operator is authenticated to cancel the new job, the wrong document is canceled.  To prevent this race from canceling the wrong document, the client MAY supply the “job-id” and “document-number” operation attributes which is checked against the current job’s “job-id” and the current Document’s “document-number”.  If the job identified by the “job-id” attribute is not the current job on the Printer and the “document-number” is not the current Document in that current Job, i.e., is not in the ‘processing’  state, the Printer MUST reject this operation and return the ‘client-error-not-possible’ status code.  Otherwise, the Printer cancels the specified document.

Access Rights: The authenticated user (see [RFC2911] section 8.3) performing this operation must either be the job owner (as determined in the Job Creation operation) or an operator or administrator of the Printer object (see [RFC2911] Sections 1 and 8.5).

The Cancel-Current-Document Request and Cancel-Current-Document Response have the same attribute groups and attributes as the Pause-Printer operation (see [RFC2911] section 3.2.8) with the addition of the following Group 1 Operation attributes in the request:

### job-id (integer(1:MAX))	operation attribute

The client OPTIONALLY supplies this Operation attribute in order to verify that the identified job is still the current job on the target Printer object.  The Printer MUST supports this operation attribute, if it supports this operation.

### document-number (integer(1:MAX))	operation attribute

The client OPTIONALLY supplies this Operation attribute in order to verify that the identified document is still the current document on the target Printer object.  The Printer MUST supports this operation attribute, if it supports this operation.

### document-message (text(127))	operation attribute

The client MAY supply this operation attribute.  If the Printer supports this attribute, the Printer MUST also support the corresponding “document-message” (text(127)) Document Description attribute (see section 8.10) and copy this value to the “document-message” Document Description attribute.  The end user can use this Document Description attribute as a message to the operator and the operator can use it as a message to the user.  The Printer MAY print this message on a document start sheet for this document, if any.

The Printer MUST ignore the new “job-message-from-operator” operation attribute (see [adm-ops] section 6) for this operation, since that is a job only attribute and Cancel-Current-Document is a Document operation.

## Delete-Document Operation

This OPTIONAL operation removes the specified Document object and its data immediately from the specified Job.  The Printer MUST reject the request if the Document is in any state, but ‘pending” (there is no ‘pending-held’ state for Documents).  The Printer does not reassign the “document-number” Document Description attribute values of the remaining Document objects in the Job, so that the deleted Document object leaves a gap in the document numbering space for the Job.  The Printer MUST return a ‘client-error-not-found’ status code, in a subsequent Get-Document-Attributes operation for this Document object specifying its former “document-number”.  This operation is similar to the Purge-Jobs operation which immediately deletes all jobs from the Printer.  However, the Delete-Document operation only deletes a single Document from the specified Job.

Access Rights: The authenticated user (see [RFC2911] section 8.3) performing this operation MUST be an operator or administrator of the Printer object (see [RFC2911] sections 1 and 8.5).  Otherwise, the Printer MUST reject the operation and return: 'client-error-forbidden', 'client-error-not-authenticated', or 'client-error-not-authorized' as appropriate.  This operation requires operator/administrator privileges, otherwise a rogue client could submit, then delete, at will using up valuable spooling space.  Instead of using this operation, unprivileged users use Cancel-Document which goes through the normal accounting, if any.

The Delete-Document request and Delete-Document response have the same attribute groups and attributes as the Cancel-Document operation (see section 3.9).

# Semantic extensions to Job operations

This section defines effects that Job operations have on the Document object.  Printer and Subscription object (Event Notification [ntfy]) operations MUST have no effect on the Document object and the Document object has no effect on the semantics of the Printer and Subscription operations.  The Job operations that MUST NOT have any effect on the Document object because they are defined to affect the Job object as a whole are: Get-Job-Attributes ([RFC2911] section 3.3.4), Get-Jobs ([RFC2911] section 3.2.6), Hold-Job ([RFC2911] section 3.3.5), Promote-Job ([adm-ops] section 4.4.1), Release-Held-New-Jobs ([adm-ops] section 3.3.2), Resume-Job ([adm-ops] section 4.3.2),  Schedule-Job-After ([adm-ops] section 4.4.2), Set-Job-Attributes ([set-ops] section 4.2), and Validate-Job ([RFC2911] section 3.2.3).

The Job operations that affect Document objects or that Document objects affect the semantics of the Job operations are, in alphabetical order:

## Cancel-Current-Job ([adm-ops] section 4.2)

The Printer sets the “document-state” Document Description attribute to ‘canceled’ and the “document-state-reasons” Document Description attribute to ‘canceled-by-xxx’ for any documents in the current Job that have not yet reached a terminal state (‘completed’, ‘canceled’, or ‘aborted’), where “xxx” is ‘user’ or  ‘operator’.  See [adm-ops] section 4.2.

## Cancel-Job ([RFC2911] section 3.3.3)

The Printer sets “document-state” Document Description attribute to ‘canceled’ and the “document-state-reasons” Document Description attribute to ‘canceled-by-user’ or ‘canceled-by-operator’ for all Document objects in the identified Job that have not yet reached a terminal state (‘completed’, ‘canceled’, or ‘aborted’).

## Create-Job ([RFC2911] section 3.2.4)

The Printer creates the Job object with all of the mixture of Job and Document attributes supplied in the Create-Job operation.  The Document objects that are subsequently created for the specified Job inherit the Document attributes from the Job object.  However, the Printer MUST NOT copy down any Job Level attributes, including “document-overrides” ([override]) to the Document object as observable by a client in any Get-Documents or Get-Document-Attributes operation responses.

## Print-Job ([RFC2911] section 3.2.1)

The Printer creates one Document object as a side effect of the Print-Job operation.  All of the attributes supplied by the client MUST remain at the Job Level as part of the Job object.  Any Document attributes supplied have affect on the Document object because the Document object inherits them from the Job object.  The Printer MUST populate the single Document object’s “document-state”, “document-state-reasons”, and “document-number” Document Description attributes with appropriate values.  Thus a Get-Documents or Get-Document-Attributes for a Job created by Print-Job will return no attributes supplied by the client, but will return the populated Document Description attributes.

## Print-URI ([RFC2911] section 3.2.2)

The Printer creates one Document object as a side effect of the Print-URI operation.  All of the attributes supplied by the client MUST remain at the Job Level as part of the Job object.  Any Document attributes supplied have affect on the Document object because the Document object inherits them from the Job object.  The Printer MUST populate the Document object’s “document-state”, “document-state-reasons”, and “document-number” Document Description attributes with appropriate values.  Thus a Get-Documents or Get-Document-Attributes for a Job created by Print-Job will return no attributes supplied by the client, but will return the populated Document Description attributes.

## Purge-Jobs ([RFC2911] section 3.2.9)

The Printer removes all Job and all of their contained Document objects.

## Release-Job  ([RFC2911] section 3.3.6)

The Printer moves the job to the ‘pending’ state, if there are no other “job-state-reasons” to hold the job.  The values of the “document-state-reasons” attributes MUST NOT affect job scheduling (see Table 11), only the Job’s “job-state-reasons” values. .

## Reprocess-Job ([adm-ops] section 4.1)

The Printer processes the Job again by processing all Documents in the Job, including ones that have been canceled by a previous Cancel-Document operation.  Therefore, the Printer removes any ‘canceled-by-xxx’ values from any of the “document-state-reasons” attributes and the Job’s “job-state-reasons” attributes.

## Restart-Job ([RFC2911] section 3.3.7)

The Printer starts the Job over again by processing all Documents in the Job, including ones that have been canceled by a previous Cancel-Document operation.  Therefore, the Printer removes any ‘canceled-by-xxx’ values from any of the “document-state-reasons” attributes and the Job’s “job-state-reasons” attributes and

## Send-Document ([RFC2911] section 3.3.1)

The Printer accepts a new Document Attributes group (Group 4) that the client can supply Document Template attribute and creates a Document object (see section 3.3).

## Send-URI ([RFC2911] section 3.3.2)

The Printer accepts a new Document Attributes group (Group 4) that the client can supply Document Template attribute and creates a Document object (see section 3.4).

## Suspend-Current-Job ([adm-ops] section 4.3.1)

The Printer suspends the current job and sets the Job’s “job-state” Job Description attribute to ‘processing-stopped’ and “job-state-reasons” to ‘job-suspended-by-xxx’, where ‘xxx’ is ‘user’ or ‘operator’.  The Documents’ “document-state” and “document-state-reasons” Document Description attributes are unaffected (but the Documents are not processed any further, since the Job is stopped).

# Attribute Precedence

This section defines the Attribute Precedence rules for Queue Override attributes, Page Override attributes, Document Object attributes, Document Override attributes, Job  Object attributes, PDL instructions, PDL Interpreter initialization files, and Printer object defaults.  Since each attribute can be specified for all of these levels, it is important to define which level takes precedence when the same attribute occurs at different levels with differing values.

The attribute precedence model has the following features:

- In principle, each defined attribute can occur at each of the precedence levels.

- Attributes can be introduced and modified at different points in the job workflow, e.g., in the application, print driver, submission client, protocol, and Printer:

- Attributes are supplied by different agents in the system:  application, user, print driver, administrator, operator, Printer object, and the output device hardware.

- Attributes are supplied at different points in time:  PDL generation, job submission, job acceptance, job pending, job processing.

- Attributes can be introduced and modified at different points in the job workflow, e.g., in the application, print driver, submission client, protocol, and Printer.

- The later that an attribute is introduced into the work flow or the closer the specified scope of the attribute is to the affected object, the higher the precedence is assigned to that attribute, since precedence is intended to reflect the user’s wishes.

The following levels of Attribute Precedence are defined in order of decreasing priority:

- queue override - The value that the Printer enforces for any job submitted to it (see [prod-print2] section 3.1).  Set by the administrator when configuring the Printer by setting an "xxx-supported" Printer attribute to contain a single value, i.e., the override value.  Example:  the administrator sets the Printer's "sides" = 'two-sided-long' so that all jobs submitted to that (logical) Printer are printed two-sided flipped along the long edge.

Note:  The queue override level is only available for Printers that support the 'guaranteed' value for its "pdl-override-supported" attribute (see [prod-print2] section 8).  All other levels are available for all Printers independent of the "pdl-override-supported" value.

- Document Level - page override programming - The special “page-overrides” collection Document Template attribute (see [override]) supplied at the Document Level, that is, in a Create-Document, Send-Document, or Send-URI operation, that contains the attributes that are to have the page override status for the specified ranges of pages in the Input-Document or the Output-Document.  Supplied by the client when submitting a Document object, or set by the user or operator after the Document has been accepted either using the Set-Document-Attributes operation, or by the operator using means local to the Printer.  Example:  "page-overrides" = {"page-number" = '1'; "media" = 'letterhead'}

- Job Level - page override programming - The special "page-overrides" collection Job Template attribute (see [override]) supplied at the Job Level, that is, in a Job Creation operation, that contains the attributes that are to have the page override status for the specified ranges of pages in the Input-Document or the Output-Document.  Supplied by the client when submitting a job either in the protocol, or set by the user or operator after the job has been accepted either using the Set-Job-Attributes operation, or by the operator using means local to the Printer.  Example:  "page-overrides" = {"page-number" = '1'; "media" = 'letterhead'}

- Document Level - Document object attributes - The Document attributes supplied at the Document Level, that is, in a Create-Document, Send-Document, or Send-URI operation to apply to the Input Document.  Supplied by the client when submitting a Document object, or set by the user or operator after the Document has been accepted either using the Set-Job-Attributes operation, or by the operator using means local to the Printer.  Examples: "media" = 'na-letter' and "copies" = '2'.

- Document Level - document override programming - The special "document-overrides" collection operation attribute (see [override]) supplied by the client in the Send-Document or Send-URI request.  The collection contains the member attributes that are to override attributes specified at lower precedence for the specified range of documents, including the current Document and subsequently created Documents.  Clients that support the Document object SHOULD supply the attributes as Document Template attributes directly when creating the Document, instead of using the operation attribute mechanism.  Example:  "document-overrides" = {"input-documents" = '5:5’; "media" = 'transparency'}

- Job Level - document override programming - The special "document-overrides" collection Job Template attribute (see [override]) that contains the attributes that are to have the document override status for the specified Input-Documents or Output-Documents.  Supplied by the client when submitting a job either in the protocol, or set by the user or operator after the job has been accepted either using the Set-Job-Attributes operation, or by the operator using means local to the Printer.  Example:  "document-overrides" = {"input-documents" = '5:5’; "media" = 'transparency'}

- Job Level - Job object and Document object attributes - The Job attributes and Document attributes supplied at the Job Level, i.e., with a Create-Job, Print-Job, or Print-URI operation.  Supplied by the client when submitting a Job in the protocol, or set by the user or operator after the job has been accepted either using the Set-Job-Attributes operation, or by the operator using means local to the Printer.  Examples: "media" = 'na-letter' and "copies" = '2'.

- PDL document data - an instruction in the PDL document data.  Supplied by the application or print driver that created the PDL.  Example:  PostScript setPageDevice specifying na-letter media.  Whether PDL document data is actually lower precedence than the preceding levels depends on the implementation as expressed in the “pdl-override-supported” Printer Description attribute (see [RFC2911] section 4.4.28 and [prod-print2] section 8.1).

Job Template and Document Template attributes of the form “xxx-requested”, such as “orientation-requested” (see [RFC2911] section 4.2.10) have a lower priority than the PDL.  If the PDL contains a corresponding instruction, that PDL instruction will be used instead of the supplied “xxx-requested” attribute.  For example, PostScript PDL data always has a well-defined orientation, so that “orientation-requested” never affects the result.  On the other hand, PDLs such as text/plain never has orientation and PCL NEED NOT have orientation, in which case, “orientation-requested” takes affect.

- PDL Init File data - a file that the Printer uses to initialize the PDL Interpreter before it starts interpreting each document in a Job.  See description of “pdl-init-file” Job Template attribute in [prod-print2] Section 5.8. It is immaterial how this file is specified, whether it be by a queue override, job object attribute or printer default – the precedence of the contents of this file is at this level.

- printer default - a Printer "xxx-default" attribute that is applied by the Printer provided that none of the higher levels have supplied a value.  The Printer implementation MUST supply values for all of the Printer's "xxx-default" attributes that reflect the out-of-the-box action by the output device.  For example, if the output device will use media from the large-capacity input tray if not directed otherwise, the implementation MUST supply the Printer's "media-default" attribute with the 'large-capacity' keyword value without requiring the administrator to configure that value.  The implementation MUST allow the administrator to configure the Printer's "xxx-default" attributes to other values, if other value are supported by the implementation.  Example:  The administrator changes the Printer's "media-default" from 'large-capacity' to 'na-letter'.

Note: 	Because of the requirement that the implementation supply pre-configured values for the Printer's "xxx-default" attributes, there is no need for a lower "hardware default" precedence level.

# Operation Attributes for the Job and Document Creation operations

This section lists all the Operation attributes that have been defined in other IPP specifications that are used in Job Creation operations and extends their use to the Document Creation operations defined in the IPP specification.  Operation attributes relate to the operation and do not depend on the printing capabilities of the Printer.

## Operation Attributes supplied at the Job and Document Levels for Creation

Table 6 lists the Operation attributes that clients supply at the Job Level and/or the Document Level.  The value of the “ipp-attribute-fidelity” operation attribute (see section 6.2.1) does not affect the Printer’s handling of the other operation attributes, only its handling of the Job Template and Document Template attributes (see section 7).

The “Level Supplied” column indicates whether the Operation attribute can be supplied at the Job Level (Print-Job, Print-URI, and Create-Job) and at the Document Level (Create-Document, Send-Document, and Send-URI) .  The Conformance column indicates the conformance requirements for what the client supplies in a  request and the Printer supports in a request.  The Reference and Section columns list the IETF or IEEE-ISTO specification which defines the semantics of the attribute.

Legend:

[job-]	indicates an attribute that shouldn’t have had a “job-” prefix in its name in [RFC2911], so that the same attribute name could also be used for the Document object as a Document Description attribute.  Note: For the PWG Semantic Model, the “job-” prefix is dropped.

“-”	indicates that the Job or Document attribute is not defined.

*	these operation attributes are defined for all operations, not just Job Creation and Document Creation operations

**	the Printer MUST copy these operation attributes to the corresponding Job Description or Document Description attributes.   See Table 9 for the corresponding Job and Document Description attributes.

Table 6 - Operation Attributes supplied at the Job and Document Levels in Creation Requests

| Operation attribute name for Job and Document Creation requests | Attribute Syntax | Level Supplied |  | Conformance |  | Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Job | Doc | Client supply | Printer support |  |
| attributes-charset * ** | charset | y | y | MUST | MUST | [RFC2911] §3.1.4 |
| attributes-natural-language * ** | naturalLanguage | y | y | MUST | MUST | [RFC2911] §3.1.4 |
| compression ** | type3 keyword | y | y | MAY | MUST | [RFC2911] §3.2.1.1 |
| document-format ** | mimeMediaType | y | y | MAY | MUST | [RFC2911] §3.2.1.1 |
| document-mandatory-attributes ** | 1setOf type2 keyword | - | y | MAY | MAY | [this spec] §6.2.3 |
| document-message * ** | text(127) | - | y | MAY | MAY | [this spec] §8.10 |
| document-name ** | name(MAX) | y | y | MAY | MUST | [RFC2911] §3.2.1.1 |
| document-natural-language ** | naturalLanguage | y | y | MAY | MAY | [RFC2911] §3.2.1.1 |
| document-overrides | 1setOf collection | - | y | MAY | MAY | [override] §5.1, §9.2.2 |
| See “document-overrides” in Table 8 for the list of member attributes |  |  |  |  |  |  |
| document-uri ** | uri | y | y | MAY | MAY | [RFC2911] §3.2.2 |
| input-document-number | integer(1:MAX) | - | y | MAY | MAY | [override] §9.2, [this spec] §3.1.1.1 |
| ipp-attribute-fidelity ** | boolean | y | y | MAY | MUST | [RFC2911] §3.2.1.1, [this spec] §6.2.1 |
| [job-]impressions | integer(0:MAX) | y | y | MAY | MAY | [RFC2911] §3.2.1.1 |
| [job-]k-octets ** | integer(0:MAX) | y | y | MAY | MAY | [RFC2911] §3.2.1.1 |
| job-mandatory-attributes ** | 1setOf type2 keyword | y | - | MAY | MAY | [this spec] §6.2.2 |
| [job-]media-sheets | integer(0:MAX) | y | y | MAY | MAY | [RFC2911] §3.2.1.1 |
| job-name ** | name(MAX) | y | - | MAY | MUST | [RFC2911] §4.3.5 |
| job-password ** | octetString(255) | y | - | MAY | MAY | [prod-print2] §4.1 |
| job-password-encryption ** | type3 keyword ¦ name(MAX) | y | - | MAY | MAY | [prod-print2] §4.2 |
| last-document ** | boolean | - | y** | MUST | MUST | [RFC2911] §3.3.1.1 |
| operation-id * | integer(1:0x8FFF) | y | y | MUST | MUST | [RFC2911] §3.1.2 |
| original-requesting-user-name | name(MAX) | y | y | MAY | MAY | [adm-ops] §11.7.2.2 |
| page-overrides | 1setOf collection | - | y | MAY | MAY | [override] §5.2, §9.2.3 |
| See “page-overrides” in Table 8 for the list of member attributes |  |  |  |  |  |  |
| printer-uri * | uri | y | y | MUST | MUST | [RFC2911] §3.1.5 |
| request-id * | integer(1:MAX) | y | y | MUST | MUST | [RFC2911] §3.1.2 |
| requesting-user-name * | name(MAX) | y | y | SHOULD | MUST | [RFC2911] §3.2.1.1 |
| version-number * | X.Y | y | y | MUST | MUST | [RFC2911] §3.1.8 |

* and **.  See Legend at the beginning of the Table.

## Operation Attributes defined for use in Document Creation operations

This section defines extensions to existing Operation attributes in Job and Document Creation operations and new Operation attributes for use in Document Creation operations.

### ipp-attribute-fidelity (boolean) (RFC2911] sections 3.2.1 and 15)

This operation attribute indicates whether or not the Printer MUST honor all supplied Job Template and Document Template attributes in the Job Creation and Document Creation operation, respectively.  The client MAY supply this attribute in Job Creation and Document Creation operations.  The Printer MUST support this attribute in Job Creation and Document Creation operations.  The value 'true' indicates that total fidelity to client supplied Job Template and Document Template attributes and values is required, else the Printer MUST reject the request.  The value 'false' indicates that a reasonable attempt to print the Job or Document object is acceptable and the Printer object MUST accept the request. If not supplied, the Printer assumes the value is 'false'.  All Printer implementations MUST support both types of job processing.  See [RFC2911] section 15 for a full description of "ipp-attribute-fidelity" and its relationship to other attributes, especially the Printer object's "pdl-override-supported" attribute.

### job-mandatory-attributes (1setOf type2 keyword)

This operation attribute identifies which Job Template attributes the Printer MUST honor, in a Job Creation operation, otherwise, the Printer MUST reject the operation.  The client MAY supply this attribute in Job Creation operations.  The Printer MUST support this attribute in Job Creation operations.  The Printer MUST ignore this attribute if the client supplies the “ipp-attribute-fidelity” operation attribute (see section 6.2.1) with a ‘true’ value, since a ‘true’ value indicates that the Printer MUST honor all supplied Job Template and Document Template attributes.

The values of the “job-mandatory-attributes” attribute are the keyword attribute names that identify Job Template attributes, for example, ‘job-hold-until’, ‘job-finishings’ (see section 7.1.4), ‘copies’ and ‘media’.  In order to identify a member attribute of a collection attribute, the keyword names of both the collection attribute and the member attribute are given separated by a period (.) character.  For example, to make the “media” attribute of the “cover-front” collection attribute mandatory, supply the keyword value: ‘cover-front.media’.

### document-mandatory-attributes (1setOf type2 keyword)

This operation attribute identifies which Document Template attributes the Printer MUST honor in a Document Creation operation, otherwise, the Printer MUST reject the operation.  The client MAY supply this attribute in Document Creation operations.  The Printer MUST support this attribute in Document Creation operations.  The Printer MUST ignore this attribute if the client supplies the “ipp-attribute-fidelity” operation attribute (see section 6.2.1) with a ‘true’ value, since a ‘true’ value indicates that the Printer MUST honor all supplied Job Template attributes and Document Template attributes.

The values of the “document-mandatory-attributes” attribute are the keyword attribute names that identify Document Template attributes, for example, ‘copies’ and ‘media’.  In order to identify a member attribute of a collection attribute, the keyword names of both the collection attribute and the member attribute are given separated by a period (.) character.  For example, to make the “media” attribute of the “cover-front” collection attribute mandatory, supply the keyword value: ‘cover-front.media’.

Operation Attributes returned in Job and Document Creation responses

Table 7 lists the Operation attributes that Printers return in Job Creation operation responses: Print-Job, Print-URI, and Create-Job and in Document Creation operation responses: Send-Document and Send-URI.  The column labeled “Conformance” indicates the conformance for what the Printer returns and what the Printer supports.  The Reference and Section columns list the IETF or IEEE-ISTO specification which defines the semantics of the attribute.

Table 7 - Operation attributes returned in Job and Document Creation operation responses

| Operation attribute name |  | Attribute Syntax | Conformance |  |  |  |  | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Values | Printer return |  | Printer support |  |  |  |
| attributes-charset * | charset |  |  | MUST |  | MUST | [RFC2911] §3.1.4 |  |
|  | ‘utf-8’ |  |  | MAY |  | MUST | [RFC2911] §4.1.7 |  |
|  | ‘us-ascii’ |  |  | MAY |  | MAY | [RFC2911] §4.1.7 |  |
|  | ‘iso-8859-1’ |  |  | MAY |  | MAY | [RFC2911] §4.1.7 |  |
|  | ‘iso-10646-ucs-2’ |  |  | MAY |  | MAY | [RFC2911] §4.1.7 |  |
| attributes-natural-language * | naturalLanguage |  |  | MUST |  | MUST | [RFC2911] §3.1.4 |  |
|  | ‘en-US’ |  |  | MAY |  | MAY | [RFC2911] §4.1.8 |  |
| detailed-status-message * | text |  |  | MAY |  | MAY | [RFC2911] §3.1.6.3 |  |
| document-access-error | text |  |  | MAY |  | MAY | [RFC2911] §3.1.6.4 |  |
| document-id-uri | uri |  |  | MUST |  | MUST | [this spec] §8.2 |  |
| document-number | integer(1:MAX) |  |  | MUST |  | MUST | [this spec] §8.1 |  |
| job-id | integer(1:MAX) |  |  | MUST |  | MUST | [RFC2911] §4.3.2 |  |
| job-uri | uri |  |  | MUST |  | MUST | [RFC2911] §4.3.1 |  |
| redirect-uri (uri) | uri |  |  | MAY |  | MAY | [not-srv] §5.2 |  |
| request-id * | integer(1:32767) |  |  | MUST |  | MUST | [RFC2911] §3.1.2 |  |
| status-code * | integer(0:32767) |  |  | MUST |  | MUST | [RFC2911] §3.1.6.1 |  |
| status-message * | text(255) |  |  | MAY |  | MAY | [RFC2911] §3.1.6.2 |  |
| version-number * | integer(1:255), integer(0:255) |  |  | MUST |  | MUST | [RFC2911] §3.1.8 |  |

* - indicates attributes that are defined for all operations, not just Job Creation operations

# Job Template and Document Template attributes

Table 8 lists all of the Job Template attributes defined in other IPP specifications and lists their corresponding Document Template attributes defined in this IPP specification.  The Job Template attributes can be supplied in the Job Creation operations: Print-Job, Print-URI, and Create-Job operations.

A subset of these Job Template attributes  (e.g., “document-format” and “media”, but not “job-priority”) are also defined to apply to Documents and are called Document Template attributes when supplied at the Document Level.  Document Template attribute can be supplied in the Create-Document, Send-Document, and Send-URI operations.

These Document Template attributes can also be supplied as Document Overrides using the “document-overrides” Job Template attribute (see [overrides]).

A subset of the Document Template attributes that apply to pages (e.g., “media”, but not “document-format” or “job-priority”) can also be supplied as Page Overrides using the “page-overrides” Job Template collection attribute (see [override]) or as a Document Template attribute.

According to [override], the “document-overrides” (collection) attribute MAY be supplied by the client in a Send-Document or Send-URI request as an Operation attribute to apply document overrides to this and/or subsequent documents in the job.  See the “document-overrides” Job Template attribute in Table 8 for the listing of the member attributes.  However, with the introduction of the Document object, the “document-overrides” (collection) attribute SHOULD NOT be used (either as a Job Template attribute or an Operation attribute).  Instead, the client simply supplies the Document Template attributes (see Table 8) for each Document Creation request (in a new Document Template attribute group) without needing a collection.   A Printer MAY support “document-overrides” as specified in [override] for backward compatibility with clients that don’t support the Document object.

The “document-overrides” attribute is also useful in combination with the “pages-per-subset” attribute (see [overrides]) which divides up the Input Page stream concatenated across the Input Documents into separate Output Documents.  For example, making every 10 Input Pages be a separate Output Document but the client only wants to staple the first Output Document.

Similarly, according to [override], the “page-overrides” (collection) attribute MAY be supplied by the client in a Send-Document or Send-URI request as an Operation attribute to apply page overrides to this and/or subsequent documents in the job.  See the “page-overrides” Job Template attribute in Table 8 for the listing of the member attributes.  However, with the introduction of the Document object, the “page-overrides” (collection) attribute SHOULD be more simply supplied as one of the Document Template attributes for the document being created only.  However, a Printer MAY also support the “page-overrides” attribute as an operation attribute for backward compatibility with clients that do not support the Document object.

The “Defined for use” column indicates whether the client MAY supply and the Printer support the attribute at the Job, Document, or Page Level with the following semantics:

| Supplied | Description |
| --- | --- |
| J=y: | Job Level: The attributes MAY be supplied as Job Template attributes at the Job Level in a Job Creation request (Print-Job, Print-URI, Create-Job) in which case each Document inherits the Job Template attribute as a default which is used unless an attribute at the document level is also supplied. Note: The “multiple-document-handling” attribute controls whether extensive attributes, such as “finishing”, “copies”, affect the entire job as a whole (‘single-xxx’ values) or affect each document separately (‘separate-xxx’ values). |
| D=y: | Document Level: The attribute MAY be supplied at the Document Level as Document Template attributes in a Document Creation operation (Send-Document, Send-URI), in which case the supplied Document Templates override any Job Template attributes supplied at the Job Level and any “document-overrides” supplied at the Job or Document level. |
| P=y: | Page Level: The attribute MAY be supplied as a member of the (1) “page-ranges” Job Template attribute at the Job Level in a Job Creation operation or (2) “page-ranges” Document Template attribute at the Document Level to affect the specified range of pages. |

Job Template and Document Template attributes are OPTIONAL for a Printer to support and for a client to supply in a Job Creation or Document Creation request or a Document Overrides or Page Overrides.  If a Printer supports a Job Template or Document Template attribute, then it MUST copy the supplied attribute to the Job or Document object, respectively so that a client MAY query the attributes in subsequent Get-Job-Attributes/Get-Jobs and Get-Document-Attributes/Get-Documents operations, respectively.  The effect of Job Template Attributes supplied in Job Creation requests are inherited by the Document objects, unless the Document Creation operation supplies the attribute with a different value.

If a Job Template collection attribute is supported (see [coll]), then there are conformance requirements for which member attributes MUST be supported and which member attributes a client MUST supply as indicated in the Conformance column for each collection member attribute in Table 8.

Table 8 - Job Template and Document Template attributes

| Job Template / Document Template attribute name | Attribute Syntax | Defined for use |  |  | Conformance |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | J | D | P | Client supply | Printer support |  |
| copies | integer(1:MAX) | y | y | n | MAY | MAY | [RFC2911] §4.2.5 |
| cover-back | collection | y | y | n | MAY | MAY | [prod-print] §3.1 |
| cover-type | type2 keyword |  |  |  | MUST | MUST | [prod-print] §3.1.2 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.1.1 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.1.1 |
| cover-front | collection | y | y | n | MAY | MAY | [prod-print] §3.1 |
| cover-type | type2 keyword |  |  |  | MUST | MUST | [prod-print] §3.1.2 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.1.1 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.1.1 |
| document-overrides | collection | y | y | n | MAY | MAY | [override] §5.1 |
| input-documents | 1setOf rangeOfInteger(MAX) |  |  |  | MUST be one or the other, but NOT both | MUST | [override] §5.1.1 |
| output-documents | 1setOf rangeOfInteger(MAX) |  |  |  |  | MUST | [override] §5.1.2 |
| document-copies | 1setOf rangeOfInteger(MAX) |  |  |  | MAY | MAY | [override] §5.1.3 |
| document-format | mimeMediaType |  |  |  | MAY | MUST | [override] §5.1.4 |
| document-name | name (MAX) |  |  |  | MAY | MUST | [override] §5.1.5 |
| compression | type3 keyword |  |  |  | MAY | MUST | [override] §5.1.6 |
| document-natural-language | naturalLanguage |  |  |  | MAY | MAY | [override] §5.1.7 |
| page-ranges | 1setOf rangeOfInteger (1:MAX) |  |  |  | MAY | MAY | [override] §5.1.8 |
| any other Job Template attributes that affect documents, such as “media” or “media-col” |  |  |  |  | MAY | MAY |  |
| feed-orientation | type3 keyword | y | y | n | MAY | MAY | [prod-print2] §5.1 |
| finishings | 1setOf type2 enum | y | y | y | MAY be neither or one of, but NOT both | MAY, MUST if “finishing-col” is supported | [RFC2911] §4.2.6 [finish] §2 |
| finishings-col | collection | y9 | y | y |  | MAY | [prod-print] §3.2 |
| finishing-template | name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.2.1 |
| stitching | collection |  |  |  | MAY | MAY | [prod-print] §3.2.2 |
| stitching-reference-edge | type2 keyword |  |  |  | MUST | MUST | [prod-print] §3.2.2.1 |
| stitching-offset | integer (0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.2.2.2 |
| stitching-locations | 1setOf integer(0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.2.2.3 |
| font-name-requested | name(MAX) | y | y | y | MAY | MAY | [prod-print2] §5.2 |
| font-size-requested | integer (1:MAX) | y | y | y | MAY | MAY | [prod-print2] §5.3 |
| force-front-side | 1setOf integer(1:MAX) | y | y | y | MAY | MAY | [prod-print] §3.3 |
| imposition-template | type2 keyword ¦ name(MAX) | y | y | y | MAY | MAY | [prod-print] §3.4 |
| insert-sheet | 1setOf collection | y10 | y | y | MAY | MAY | [prod-print] §3.5 |
| insert-after-page-number | integer(0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.5.1 |
| insert-count | integer(0:MAX) |  |  |  | MAY | MUST | [prod-print] §3.5.2 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MUST be one or the other, but NOT both | MUST | [prod-print] §3.5.3 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.5.3 |
| job-account-id | name(MAX) | y | n | n | MAY | MAY | [prod-print] §3.6 |
| job-accounting-sheets | collection | y | n | n | MAY | MAY | [prod-print] §3.8 |
| job-accounting-sheets-type | type3 keyword ¦ name(MAX) |  |  |  | MUST | MUST | [prod-print] §3.8.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.8.2 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.8.2 |
| job-accounting-output-bin | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.8.3 |
| job-accounting-user-id | name(MAX) | y | n | n | MAY | MAY | [prod-print] §3.7 |
| job-copies | integer(1:MAX) | y | n | n | MAY | MAY | [this spec] §7.1.1 |
| job-cover-back | collection | y | n | n | MAY | MAY | [this spec] §7.1.2 |
| same as cover-back | see cover-back |  |  |  |  |  |  |
| job-cover-front | collection | y | n | n | MAY | MAY | [this spec] §7.1.3 |
| same as cover-front | see cover-front |  |  |  |  |  |  |
| job-error-sheet | collection | y | n | n | MAY | MAY | [prod-print] §3.9 |
| job-error-sheet-type | type3 keyword ¦ name(MAX) |  |  |  | MUST | MUST | [prod-print] §3.9.1 |
| job-error-sheet-when | type2 keyword |  |  |  | MAY | MAY | [prod-print] §3.9.2 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.9.3 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.9.3 |
| job-finishings | 1setOf type2 enum | y | n | n | MAY | MAY | [this spec] §7.1.4 |
| job-finishings-col | collection | y | n | n | MAY | MAY | [this spec] §7.1.5 |
| same as finishings-col | see finishings-col |  |  |  |  |  |  |
| job-hold-until | type3 keyword ¦ name(MAX) | y | n | n | MAY | MAY | [RFC2911] §4.2.2 |
| job-hold-until-time | dateTime | y | n | n | MAY | MAY | [prod-print2] §5.4 |
| job-message-to-operator | text(MAX) | y | n | n | MAY | MAY | [prod-print] §3.10 |
| job-phone-number | text(127) | y | n | n | MAY | MAY | [prod-print2] §5.5 |
| job-priority | integer(1:100 | y | n | n | MAY | MAY | [RFC2911] §4.2.1 |
| job-recipient-name | name(MAX) | y | n | n | MAY | MAY | [prod-print2] §5.6 |
| job-save-disposition | collection | y | n | n | MAY | MAY | [prod-print2] §5.7 |
| save-disposition | type3 keyword | y | n | n | MAY | MAY | [prod-print2] §5.7.1.1 |
| save-info | 1setOf collection | y | n | n | MAY | MAY | [prod-print2] §5.7.1.2 |
| save-location | uri | y | n | n | MAY | MAY | [prod-print2] §5.7.1.2.3.1 |
| save-name | name(MAX) | y | n | n | MAY | MAY | [prod-print2] §5.7.1.2.3.2 |
| save-document-format | mimeMediaType | y | n | n | MAY | MAY | [prod-print2] §5.7.1.2.3.3 |
| job-sheet-message | text(MAX) | y | n | n | MAY | MAY | [prod-print] §3.12 |
| job-sheets | type3 keyword ¦ name(MAX) | y | n | n | MAY be neither or one of, but NOT both | MAY, MUST if “job-sheets-col” supported | [RFC2911] §4.2.3 |
| job-sheets-col | collection | y | n | n |  | MAY | [prod-print] §3.11 |
| job-sheets | type3 keyword ¦ name(MAX) |  |  |  | MUST | MUST | [prod-print] §3.11.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MUST be one or the other, but NOT both | MUST | [prod-print] §3.11.2 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.11.2 |
| media | type3 keyword ¦ name(MAX) | y | y | y | MAY be neither or one of, but NOT both | MAY, MUST if “media-col” supported | [RFC2911] §4.2.11 |
| media-col | collection | y | y | y |  | MAY | [prod-print] §3.13 |
| media-back-coating | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.10 |
| media-color | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.4 |
| media-front-coating | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.10 |
| media-grain | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print2] §8.4.2 |
| media-hole-count | integer(0:MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.6 |
| media-info | text(255) |  |  |  | MAY | MAY | [prod-print] §3.13.3 |
| media-key | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.1 |
| media-material | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print2] §8.4.3 |
| media-order-count | integer(1:MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.7 |
| media-preprinted | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.5 |
| media-recycled | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.11 |
| media-size | collection |  |  |  | MAY | MUST | [prod-print] §3.13.8 |
| x-dimension | integer(0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.13.8.1 |
| y-dimension | integer(0:MAX) |  |  |  | MUST | MUST | [prod-print] §3.13.8.2 |
| media-tooth | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print2] §8.4.1 |
| media-thickness | integer(1:MAX |  |  |  | MAY | MAY | [prod-print2] §8.4.4 |
| media-type | type3 keyword ¦ name(MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.2 |
| media-weight-metric | integer(0:MAX) |  |  |  | MAY | MAY | [prod-print] §3.13.9 |
| media-input-tray-check | type3 keyword ¦ name(MAX) | y | y | y | MAY | MAY | [prod-print] §3.14 |
| multiple-document-handling | type2 keyword | y | n | n | MAY | MUST if multiple document jobs supported | [RFC2911] §4.2.4 |
| number-up | integer(1:MAX) | y | y | y | MAY | MAY | [RFC2911] §4.2.9 |
| orientation-requested | type2 enum | y | y | y | MAY | MAY | [RFC2911] §4.2.10 |
| output-bin | type2 keyword ¦ name(MAX) | y | n | n | MAY | MAY | [output-bin] §2.1 |
| page-delivery | type2 keyword | y | y | n | MAY | MAY | [prod-print] §3.15 |
| page-order-received | type2 keyword | y | y | n | MAY | MAY | [prod-print] §3.16 |
| page-overrides | 1setOf collection | y | y | n | MAY | MAY | [override] §5.2 |
| input-documents | 1setOf rangeOfInteger(MAX) |  |  |  | MUST be one or the other, but NOT both | MUST | [override] §5.2.1 |
| output-documents | 1setOf rangeOfInteger(MAX) |  |  |  |  | MUST | [override] §5.2.2 |
| document-copies | 1setOf rangeOfInteger(MAX) |  |  |  | MAY | MAY | [override] §5.2.3 |
| pages | 1setOf rangeOfInteger(MAX) |  |  |  | MUST | MUST | [override] §5.2.4 |
| any other Job Template attributes that affect pages, such as “media” or “media-col” |  |  |  |  | MAY | MAY |  |
| page-ranges | 1setOf rangeOfInteger(1:MAX) | y | y | n | MAY | MAY | [RFC2911] §4.2.7 |
| pages-per-subset | 1setOf integer(1:MAX) | y | n | n | MAY | MAY | [prod-print] §5.3 |
| pdl-init-file | 1setOf collection | y | y | n | MAY | MAY | [prod-print2] §5.8 |
| pdl-init-file-location | uri |  |  |  | MAY | MAY | [prod-print2] §5.8.1.1 |
| pdl-init-file-name | name(MAX) |  |  |  | MAY | MAY | [prod-print2] §5.8.1.2 |
| pdl-init-file-entry | name(MAX) |  |  |  | MAY | MAY | [prod-print2] §5.8.1.3 |
| presentation-direction-number-up | type2 keyword | y | y | y | MAY | MAY | [prod-print] §3.17 |
| print-quality | type2 enum | y | y | y | MAY | MAY | [RFC2911] §4.2.13 |
| printer-resolution | resolution | y | y | y | MAY | MAY | [RFC2911] §4.2.12 |
| proof-print | collection | y | n | n | MAY | MAY | [prod-print2] §5.9 |
| proof-print-copies | integer (0:MAX) | y | n | n | MAY | MAY | [prod-print2] §5.9.1.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print2] §5.8.1.2 |
| media-col | collection |  |  |  |  | MAY | [prod-print2] §5.8.1.2 |
| separator-sheets | collection | y | y | n | MAY | MAY | [prod-print] §3.18 |
| separator-sheets-type | type3 keyword ¦ name(MAX) |  |  |  | MUST | MUST | [prod-print] §3.18.1 |
| media | type3 keyword ¦ name(MAX) |  |  |  | MAY be neither or one of, but NOT both | MUST | [prod-print] §3.18.2 |
| media-col | collection |  |  |  |  | MAY | [prod-print] §3.18.2 |
| sheet-collate | type2 keyword | y | y | n | MAY | MAY | [job-prog] §3.1 |
| sides | type2 keyword | y | y | y | MAY | MAY | [RFC2911] §4.2.8 |
| x-image-position | type2 keyword | y | y | y | MAY | MAY | [prod-print] §3.19.2 |
| x-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.3 |
| x-side1-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.4 |
| x-side2-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.5 |
| y-image-position | type2 keyword | y | y | y | MAY | MAY | [prod-print] §3.19.6 |
| y-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.7 |
| y-side1-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.8 |
| y-side2-image-shift | integer(MIN:MAX) | y | y | y | MAY | MAY | [prod-print] §3.19.9 |

## Job Template attributes defined for use with the Document object

This section defines additional Job Template attributes for use with the Document object.

### job-copies (integer(1:MAX))	Job Template attribute

This attribute specifies the number of job copies, i.e., with all of the Document objects taken together.  Its interpretation is independent of the value of the “multiple-document-handling” Job Template attribute.  See the “copies” Job Template attribute defined in [RFC2911] section 4.2.5 which switches between job copies and document copies depending on the value of the “multiple-document-handling” Job Template attribute.

### job-cover-back (collection)	Job Template attribute

This attribute specifies the cover for the back of the job, i.e., with all of the Document objects taken together and is independent of the value of the “multiple-document-handling” Job Template attribute.  The member attributes are the same as “cover-back” (see [prod-print] section 3.1).  However, the “cover-back” Job Template attribute switches between a job back cover and a document back cover depending on the value of the “multiple-document-handling” Job Template attribute.

### job-cover-front (collection)	Job Template attribute

This attribute specifies the cover for the front of the job, i.e., with all of the Document objects taken together and is independent of the value of the “multiple-document-handling” Job Template attribute.  The member attributes are the same as the “cover-front” Job Template attribute (see [prod-print] section 3.2).  However, the “cover-front” Job Template attribute switches between a job front cover and a document front cover depending on the value of the “multiple-document-handling” Job Template attribute.

### job-finishings (1setOf type2 enum)	Job Template attribute

This attribute specifies the job finishings, i.e., with all of the Document objects taken together.  Its interpretation is independent of the value of the “multiple-document-handling” Job Template attribute.  See the “finishings” Job Template attribute defined in [RFC2911] section 4.2.6 which switches between job finishings and document finishings depending on the value of the “multiple-document-handling” Job Template attribute.

### job-finishings-col (1setOf collection)	Job Template attribute

This attribute specifies the job finishings collection, i.e., with all of the Document objects taken together.  Its interpretation is independent of the value of the “multiple-document-handling” Job Template attribute.  See the “finishings” Job Template attribute defined in [RFC2911] section 4.2.6 which switches between job finishings collection and document finishings collection depending on the value of the “multiple-document-handling” Job Template attribute.

# Job Description and Document Description attributes

Table 9 lists all the Job Description defined in other IPP specifications and the corresponding Document Description attributes defined in this IPP specification.  Job Description and Document Description attributes are Job object and Document object attributes, respectively, that are filled in by the Printer.  Some Job/Document Description attribute values are supplied by the client in the Job/Document Creation operation using corresponding Operation attributes as flagged with “**”.

The column labeled “attribute syntax” indicates the attribute syntax (see [RFC2911] section 4.1).  The Printer conformance is indicated along with the reference to the specification in which the attribute is defined.  The conformance for Document Description attributes (if the Printer supports the Document object) is the same as the corresponding Job Description attributes.

Note:  There are no corresponding Page Range (or Page Override) Description attributes.  The Page Range is not an object with a state and it cannot be queried.

Legend:

[job-]	indicates an attribute that shouldn’t have had a “job-” prefix in its name in [RFC2911], so that the same attribute name could also be used for the Document object as a Document Description attribute.  Note: For the PWG Semantic Model, the “job-” prefix is dropped.

“-”	indicates that the Job or Document attribute is not defined.

**	indicates a Job and/or Document Description attribute that the client MAY supply the values for in a corresponding Operation attribute.   See Table 6 for the corresponding operation attribute.

(r/w)	indicates that the Job Description or Document Description attribute is read/write, so that the client can changes its value using the Set-Job-Attributes or Set-Document-Attributes operation (see section 3.8).

Table 9 - Job and Document Description attributes

| IPP Job Description / Document Description attribute name | IPP Attribute Syntax | Job | Doc | Printer conformance | Reference |
| --- | --- | --- | --- | --- | --- |
| attributes-charset ** | charset | y | y | MUST | [RFC2911] §4.3.19 |
| attributes-natural-language ** | naturalLanguage | y | y | MUST | [RFC2911] §4.3.20 |
| compression ** | type3 keyword | y | y | MUST | [RFC2911] §3.2.1.1 |
| current-page-order | type2 keyword | y | y | MAY | [prod-print] §4.1 |
| date-time-at-completed | dateTime | y | y | MAY | [RFC2911] §4.3.14.7 |
| date-time-at-creation | dateTime | y | y | MAY | [RFC2911] §4.3.14.5 |
| date-time-at-processing | dateTime | y | y | MAY | [RFC2911] §4.3.14.6 |
| document-format ** | documentFormat | y | y | MUST | [RFC2911] §3.2.1.1 |
| document-id-uri | uri | - | y | MUST | [this spec] §8.2 |
| document-message (r/w) | text(127) | - | y | MAY | [this spec] §8.10 |
| document-name (r/w) ** | name(MAX) | y | y | MUST | [RFC2911] §3.2.1.1 |
| document-natural-language ** | naturalLanguage | y | y | MAY | [RFC2911] §3.2.1.1 |
| document-number | integer(0:MAX) | - | y | MUST | [this spec] §8.1 |
| document-printer-up-time | integer(1:MAX) | - | y | MUST | [this spec] §8.11 |
| document-state | type1 enum | - | y | MUST | [this spec] §8.5.2 |
| document-state-message | text(MAX) | - | y | MAY | [this spec] §8.9 |
| document-state-reasons | 1setOf type2 keyword | - | y | MUST | [this spec] §8.7 |
| document-uri ** | uri | - | y | MAY | [this spec] §8.3 |
| impressions-completed-current-copy | integer(0:MAX) | y | y | MAY | [job-prog] §4.4 |
| job-collation-type | type2 enum | y | - | MAY | [job-prog] §4.1 |
| [job-]detailed-status-message | 1setOf text (MAX) | y | y | MAY | [RFC2911] §4.3.10 |
| [job-]document-access-errors | 1setOf text(MAX) | y | y | MAY | [RFC2911] §4.3.11 |
| job-id | integer(1:MAX) | y | - | MUST | [RFC2911] §4.3.2 |
| [job-]impressions ** | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.17.2 |
| [job-]impressions-completed | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.18.2 |
| [job-]k-octets ** | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.17.1 |
| [job-]k-octets-processed | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.18.1 |
| [job-]media-sheets ** | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.17.3 |
| [job-]media-sheets-completed | integer(0:MAX) | y | y | MAY | [RFC2911] §4.3.18.3 |
| job-message-from-operator | text(127) | y | - | MAY | [RFC2911] §4.3.16 |
| [job-]more-info | uri | y | y | MAY | [RFC2911] §4.3.4 |
| job-name (r/w) | name(MAX) | y | - | MUST | [RFC2911] §4.3.5 |
| job-originating-user-name | name (MAX) | y | - | MUST | [RFC2911] §4.3.6 |
| job-printer-make-and-model | text(127) | y | - | CMUST | [RFC2911] §4.3.14.4 |
| job-printer-up-time | integer (1:MAX) | y |  | MUST | [RFC2911] §4.3.14.4 |
| job-printer-uri | uri | y | - | MUST | [RFC2911] §4.3.3 |
| job-state | type1 enum | y | - | MUST | [RFC2911] §4.3.7 |
| job-state-message | text(MAX) | y | - | MAY | [RFC2911] §4.3.9 |
| job-state-reasons | 1setOf type2 keyword | y | - | MUST | [RFC2911] §4.3.8 |
| job-uri | uri | y | - | MUST | [RFC2911] §4.3.1 |
| [job-]warnings-count | integer(0:MAX) | y | y | MAY | [override] §6.1 |
| last-document ** | boolean | - | y | MAY | [RFC2911] §4.3.12 |
| number-of-documents | integer(0:MAX) | y | - | MAY | [RFC2911] §4.3.15 |
| number-of-intervening-jobs | integer(0:MAX) | y | - | MAY | [RFC2911] §4.3.13 |
| output-device-assigned | name(127) | y | - | MAY | [RFC2911] §4.3.7.2 |
| sheet-completed-copy-number | integer(0:MAX) | y | y | MAY | [job-prog] §4.2 |
| sheet-completed-document-number | integer(0:MAX) | y | - | MAY | [job-prog] §4.3 |
| time-at-completed | integer (MIN:MAX) | y | y | MUST | [RFC2911] §4.3.14.3 |
| time-at-creation | integer (MIN:MAX) | y | y | MUST | [RFC2911] §4.3.14.1 |
| time-at-processing | integer (MIN:MAX) | y | y | MUST | [RFC2911] §4.3.14.2 |

“**”	See the description in the Legend before the table.

Most of the existing “job-state” and “job-state-reasons” Job Description attribute values apply equally well to individual documents in a multi-document job (with removal of the ‘job-’ prefix for those that apply to both Job and Document).  Furthermore, the job states are completely service-type neutral, so they can apply to scanning. faxing, formatting processes as well.  The job state reasons have a number of reasons that are print service specific, but most can apply to other types of service as well.  The reason for separating state from reasons, is so that the states can be the fundamental states that most implementations have, while the state reasons can vary depending on the sophistication of the system.  The states are the ones that clients are most likely to base actions upon, and the state reasons are more information, both for programs and people.

The descriptions of the new “document-state” and “document-state-reasons” attributes are merged in with the existing “job-state” and “job-state-reasons”.  Such merging shows the parallelism in the description to aid understanding and implementation and so that future maintenance will continue this strong parallelism.  The “/” character separates the words: Job from Document so that the same description can serve for both Job and Document objects.

## document-number (integer(1:MAX))	Document Description attribute

This REQUIRED Document Description attribute is the ordinal number of the document within the job which determines its order of being printed in a multi-document job.  The Printer generates the values monotonically increasing from 1 for a job.  However, the client MAY supply the value in the “input-document-number” operation attribute (see section 3.1.1.1).

## document-id-uri (uri)	Document Description attribute

This REQUIRED Document Description attribute is generated by the Printer as a unique URI to identify a Document object, in the same way that the “job-uri” (uri) Job Description attribute uniquely identifies a Job object.  Unlike the “job-uri” (uri) attribute, the client cannot supply this attribute as a target of an operation.

document-uri (uri)	Document Description attribute

This Document Description attribute is the URI that the client supplied in a Send-URI operation.  The Printer MUST support this attribute if the Printer supports the Send-URI operation (see section 3.4).

## job-state (type1 enum)	Job Description attribute

## document-state (type1 enum)	Document Description attribute

The REQUIRED “job-state” and the REQUIRED “document-state” attribute identify the current state of the job and document, respectively.  Even though IPP defines seven values for job/document states (plus the out-of-band 'unknown' value - see [RFC2911] section 4.1), implementations only need to support those states which are appropriate for the particular implementation.  In other words, an IPP Printer object implementation supports only those job/document states implemented by the output device and available to the IPP Printer object implementation.

Table 10 lists the “job-state” and “document-state” attribute values.

Table 10 - "job-state" and "document-state" attribute values

| Values | “job-state” | “document-state” |
| --- | --- | --- |
| 3 pending | y | y |
| 4 pending-held | y | n/a |
| 5 processing | y | y |
| 6 processing-stopped | y | n/a |
| 7 canceled | y | y |
| 8 aborted | y | y |
| 9 completed | y | y |

The (JD) and (J) notation below indicates whether the keyword is defined for the “job-state” and “document-state” attributes or just the “job-state” attribute as indicated in Table 10.  Standard enum values are:

| Value |  | Symbolic Name and Description |
| --- | --- | --- |
| '3' |  | 'pending' (JD): The job is a candidate to start processing, but is not yet processing. The document has not started to be processed at all. |
| '4' |  | 'pending-held'' (J): The job is not a candidate for processing for any number of reasons but will return to the 'pending' state as soon as the reasons are no longer present. The job's "job-state-reason" attribute MUST indicate why the job is no longer a candidate for processing. The ‘pending-held’ state does not apply to the Document object’s “document-state” attribute. |
| '5' |  | 'processing' (JD): One or more of: - The job is using, or is attempting to use, one or more purely software processes that are analyzing, creating, or interpreting a PDL, etc., - The job is using, or is attempting to use, one or more hardware devices that are interpreting a PDL, making marks on a medium, and/or performing finishing, such as stapling, etc., - The Printer object has made the job ready for printing, but the output device is not yet printing it, either because the job hasn't reached the output device or because the job is queued in the output device or some other spooler, awaiting the output device to print it. When the job is in the 'processing' state, the entire job/document state includes the detailed status represented in the Printer object's "printer-state", "printer-state-reasons", and "printer-state-message" attributes. On the other hand, the ‘processing’ state for the Document indicates that the Document has begun to be processed. Even if the job stops being processed, the Document remains in the ‘processing’ state until it moves to one of the three terminal states. Implementations MAY, though they NEED NOT, include additional values in the job's/document’s "job-state-reasons"/“document-state-reasons” attribute to indicate the progress of the job/document, such as adding the 'job-printing'/‘printing’ value to indicate when the output device is actually making marks on paper and/or the 'processing-to-stop-point' value to indicate that the Printer is in the process of canceling or aborting the job/document. Most implementations won't bother with this nuance. |
| '6' |  | 'processing-stopped' (J): The job has stopped while processing for any number of reasons and will return to the 'processing' state as soon as the reasons are no longer present. The job's "job-state-reason" attribute MAY indicate why the job has stopped processing. For example, if the output device is stopped, the 'printer-stopped' value MAY be included in the job's "job-state-reasons" attribute. Note: When an output device is stopped, the device usually indicates its condition in human readable form locally at the device. A client can obtain more complete device status remotely by querying the Printer object's "printer-state", "printer-state-reasons" and "printer-state-message" attributes. The ‘processing-stopped’ state does not apply to the Document object’s “document-state” attribute. |
| '7' |  | 'canceled' (JD): The job/document has been canceled by a Cancel-Job/Cancel-Document operation and the Printer object has completed canceling the job/document and all job/document status attributes have reached their final values for the job/document. While the Printer object is canceling the job/document, the job/document remains in its current state, but the job's/document’s "job-state-reasons"/“document-state-reasons” attribute SHOULD contain the 'processing-to-stop-point' value and one of the 'canceled-by-user', 'canceled-by-operator', or 'canceled-at-device' value. When the job/document moves to the 'canceled' state, the 'processing-to-stop-point' value, if present, MUST be removed, but the 'canceled-by-xxx', if present, MUST remain. |
| '8' |  | 'aborted' (JD): The job/document has been aborted by the system, usually while the job/document was in the 'processing' or 'processing-stopped' state and the Printer has completed aborting the job/document and all job/document status attributes have reached their final values for the job/document. While the Printer object is aborting the job/document, the job/document remains in its current state, but the job's/document’s "job-state-reasons"/“document-state-reasons” attribute SHOULD contain the 'processing-to-stop-point' and 'aborted-by-system' values. When the job/document moves to the 'aborted' state, the 'processing-to-stop-point' value, if present, MUST be removed, but the 'aborted-by-system' value, if present, MUST remain. |
| '9' |  | 'completed' (JD): The job/document has completed successfully or with warnings or errors after processing and all of the job/document media sheets have been successfully stacked in the appropriate output bin(s) and all job/document status attributes have reached their final values for the job/document. The job's/document’s "job-state-reasons"/“document-state-reasons” attribute SHOULD contain one of: 'completed-successfully', 'completed-with-warnings', or 'completed-with-errors' values. |

The final value for this attribute MUST be one of: 'completed', 'canceled', or 'aborted' before the Printer removes the job/document altogether.  The length of time that jobs/document remain in the 'canceled', 'aborted', and 'completed' states depends on implementation.

Jobs/documents reach one of the three terminal states: 'completed', 'canceled', or 'aborted', after the jobs/documents have completed all activity, including stacking output media, after the jobs/documents have completed all activity, and all job/document status attributes have reached their final values for the job/document.

### The “job-state” Job Description attribute and the Job Life Cycle

The “job-state” attribute is one of the most important Job Description attributes.  Figure 1 shows the values of the “job-state” attribute and the Job object life cycle as affected by actions on the Job, Printer, and job processing.

Figure 1 - The “job-state” Job Description attribute and the Job object life cycle

Normally a job progresses from left to right.  Other state transitions are unlikely, but are not forbidden.  Not shown are the transitions to the 'canceled' state from the 'pending', 'pending-held', and 'processing-stopped' states.

### The “document-state” Document Description attribute and the Document object life Cycle

The “document-state” attribute is one of the most important Document Description attributes.  Figure 2 shows the values of the “document-state” attribute and the Document object life cycle as affected by actions on the Job, Document, Printer, and job processing.  Documents are not active objects and their life cycle is closely tied to the lifecycle of a Job.  Documents basically have three states.   The first is waiting to be processed by a Job (i.e. pending).  The second state is from the time the Job first starts processing the Document (i.e., processing) and until it reaches one of its terminal states.  The third state for a Document is one of its terminal states (i.e., completed, canceled, or aborted).

Figure 2 - The “document-state” Document Description attribute and the Document object life Cycle

A Document object has a much simpler life cycle than a Job object.  A Document always progresses from left to right in the state diagram.  No other state transitions are defined.

### Forwarding Servers

As with all other IPP attributes, if the implementation can not determine the correct value for this attribute, it SHOULD respond with the out-of-band value 'unknown' (see [RFC2911] Section 4.1) rather than try to guess at some possibly incorrect value and give the end user the wrong impression about the state of the Job/Document object.  For example, if the implementation is just a gateway into some printing system from which it can normally get status, but temporarily is unable, then the implementation should return the 'unknown' value.  However, if the implementation is a gateway to a printing system that never provides detailed status about the print job/document, the implementation MAY set the IPP Job/Document object's state to 'completed', provided that it also sets the 'queued-in-device' value in the job's/document’s "job-state-reasons"/“document-state-reasons” attribute (see “job-state-reasons”/“document-state-reasons” attribute below).

### Partitioning of Job/Document States

This section partitions the job/document states into phases:  Job/Document Not Completed, Job/Document Retention, Job/Document History, and Job/Document Removal.  This section also explains the 'job-restartable' value of the "job-state-reasons" Job Description attribute for use with the Restart-Job operation.

Job/Document Not Completed:  When a job is in the 'pending', 'pending-held', 'processing', or 'processing-stopped' states, the job is not completed.  When a Document is in the ‘pending’ or ‘processing’ state, it is not completed.

Job Retention:  When a job enters one of the three terminal job states:  'completed', 'canceled', or 'aborted', the IPP Printer object MAY "retain" the job in a restartable condition for an implementation-defined time period.  This time period MAY be zero seconds and MAY depend on the terminal job state.  This phase is called Job Retention.  While in the Job Retention phase, the job's document data is retained and a client may restart the job using the Restart-Job operation.  If the Printer supports the Restart-Job operation, then it SHOULD indicate that the job is restartable by adding the 'job-restartable' value to the job's "job-state-reasons" attribute (see Section 8.6) during the Job Retention phase.  If the Printer supports the Document object, then the Get-Document-Attributes and Get-Documents operations MUST be supported while the job is in the Job Retention phase.

Job History:  After the Job Retention phase expires for a job, the Printer object deletes the document data for the job and the job becomes part of the Job History.  The Printer object MAY also delete any number of the job and document attributes.  Since the job is no longer restartable, the Printer object MUST remove the 'job-restartable' value from the job's "job-state-reasons" attribute, if present.  If the implementation supports the Document object, it SHOULD retain at least the “document-state” and “document-state-reasons” attributes on the Document object during the Job History phase which can be queried using the Get-Document-Attributes and Get-Documents operations.

Job Removal:  After the job has remained in the Job History for an implementation-defined time, such as when the number of jobs exceeds a fixed number or after a fixed time period (which MAY be zero seconds), the IPP Printer removes the Job and Document objects from the system.

Using the Get-Jobs/Get-Documents operation and supplying the 'not-completed' value for the "which-jobs" operation attribute, a client is requesting jobs/documents in the Job/Document Not Completed phase.  Using the Get-Jobs/Get-Documents operation and supplying the 'completed' value for the "which-jobs" operation attribute, a client is requesting jobs/documents in the Job Retention and Job History phases.  Using the Get-Job-Attributes/Get-Document-Attributes operation, a client is requesting a job/documents in any phase except Job Removal.  After Job Removal, the Get-Job-Attributes/Get-Document-Attributes and Get-Jobs/Get-Documents operations no longer are capable of returning any information about a job/document.

NOTE:  The semantics of this attribute or the set of valid values may be different for different document formats.

## job-state-reasons  (1setOf type2 keyword) 	Job Description attribute

## document-state-reasons  (1setOf type2 keyword) 	Document Description attribute

The REQUIRED “job-state-reasons” and the REQUIRED “document-state-reasons” attributes provide additional information about the job's and each document’s current state, i.e., information that augments the value of the job's "job-state" attribute and each Document’s “document-state” attribute, respectively.  The Printer SHOULD support the same values for both the “job-state-reasons” and the “document-state-reasons” attribute for those attributes that are defined for both as indicated in the “Job” and “Doc” columns in Table 11.

These values MAY be used with any job or document state or states for which the reason makes sense.  Some of these value definitions indicate conformance requirements; the rest are OPTIONAL.  Furthermore, when implemented, the Printer MUST return these values when the reason applies and MUST NOT return them when the reason no longer applies whether the value of the Job's "job-state" attribute or the Document’s “document-state” attribute changed or not.  When the Job or Document does not have any reasons for being in its current state, the value of the Job's "job-state-reasons" attribute or Document’s “document-state-reasons” attribute MUST be 'none'.

Some of the Job’s “job-state-reasons” values affect job scheduling, such as ‘resources-are-not-ready’ and ‘job-hold-until-specified’.  However, “document-state-reasons” values MUST NOT affect job scheduling, since Document objects are passive objects operated on by the containing Job object.

Note: While values cannot be added to the “job-state” or “document-state” attributes without impacting deployed clients that take actions upon receiving "job-state" or “document-state” values, it is the intent that additional "job-state-reasons" and “document-state-reasons’ values can be defined and registered without impacting such deployed clients.  In other words, the "job-state-reasons" and the “document-state-reasons” attributes are intended to be extensible.

Table 11 lists the standard keyword values for the “job-state-reasons” and/or “document-state-reasons” attributes defined in this and other IPP specifications as indicated in the References colum Table 11.  In Table 11 the keywords are listed in alphabetic order (observing the “[job-]” prefix), while in their descriptions following Table 11, they are listed in the most likely order of occurrence to help understanding.

When the Document object is supported, the Printer implementation SHOULD still support the corresponding Job level “job-state-reasons”, so that clients that do not support the Document object will receive the same information.  Also it is usually more convenient for the client to obtain just the Job level status.  However, the client SHOULD query both Job and Document attributes and convert them to some canonical form for display to the user, so that all Jobs and their Documents are displayed in a uniform manner, no matter from which clients they were submitted.

Notation:

[job-]	indicates a state reason keyword value that has the ‘job-‘ prefix when it is a value of the “job-state-reasons” attribute, but does not have the ‘job-‘ prefix when it is a value of the “document-state-reasons” attribute.  For purposes of sorting in Table 11, the “[job-]” prefix is observed, since this is an IPP specification.

“-”	indicates that the Job or Document value is not defined.

Table 11 - "job-state-reasons" and "document-state-reasons" attribute values

| IPP Job Description / Document Description attribute name | IPP Attribute Syntax plus Values | Job | Doc | Printer conformance | Reference |
| --- | --- | --- | --- | --- | --- |
| job-state-reasons | 1setOf type2 keyword | y | - | MUST | [RFC2911] §4.3.8 |
| document-state-reasons | 1setOf type2 keyword | - | y | MUST | [this spec] §8.7 |
|  | aborted-by-system | y | y | MAY | [RFC2911] §4.3.8 |
|  | compression-error | y | y | MAY | [RFC2911] §4.3.8 |
|  | document-access-error | y | y | MAY | [RFC2911] §4.3.8 |
|  | document-format-error | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]canceled-at-device | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]canceled-by-operator | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]canceled-by-user | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]completed-successfully | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]completed-with-errors | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]completed-with-warnings | y | y | MAY | [RFC2911] §4.3.8 |
|  | job-data-insufficient | y | - | MAY | [RFC2911] §4.3.8 |
|  | job-hold-until-specified | y | - | MAY | [RFC2911] §4.3.8 |
|  | [job-]incoming | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]interpreting | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]outgoing | y | y | MAY | [RFC2911] §4.3.8 |
|  | job-password-wait | y | - | MAY | [prod-print2] §8.3.1 |
|  | [job-]printing | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]queued | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]queued-for-marker | y | y | MAY | [RFC2911] §4.3.8 |
|  | job-restartable | y | - | MAY | [RFC2911] §4.3.8 |
|  | job-resuming | y | - | MAY | [prod-print2] §8.3.1 |
|  | job-saved-successfully | y | - | MAY | [prod-print2] §8.3.1 |
|  | job-save-error | y | - | MAY | [prod-print2] §8.3.1 |
|  | job-saving | y | - | MAY | [prod-print2] §8.3.1 |
|  | job-scheduling | y | - | MAY | [prod-print2] §8.3.1 |
|  | [job-]spooling | y | y | MAY | [prod-print2] §8.3.1 |
|  | [job-]streaming | y | y | MAY | [prod-print2] §8.3.1 |
|  | job-suspended | y | - | MAY | [adm-ops] §9.1 |
|  | job-suspended-by-operator | y | - | MAY | [prod-print2] §8.3.1 |
|  | job-suspended-by-system | y | - | MAY | [prod-print2] §8.3.1 |
|  | job-suspended-by-user | y | - | MAY | [prod-print2] §8.3.1 |
|  | job-suspending | y | - | MAY | [prod-print2] §8.3.1 |
|  | [job-]transforming | y | y | MAY | [RFC2911] §4.3.8 |
|  | [job-]warnings-detected | y | y | MAY | [override] §7.1 |
|  | none | y | y | MUST | [RFC2911] §4.3.8 |
|  | printer-stopped | y | - | MAY | [RFC2911] §4.3.8 |
|  | printer-stopped-partly | y | - | MAY | [RFC2911] §4.3.8 |
|  | processing-to-stop-point | y | - | MAY | [RFC2911] §4.3.8 |
|  | proof-print-wait | y | - | MAY | [prod-print2] §8.3.1 |
|  | queued-in-device | y | y | MAY | [RFC2911] §4.3.8 |
|  | resources-are-not-ready | y | - | MAY | [RFC2911] §4.3.8 |
|  | resources-are-not-supported | y | y | MAY | [prod-print] §6.1 |
|  | service-off-line | y | - | MAY | [RFC2911] §4.3.8 |
|  | submission-interrupted | y | y | MAY | [RFC2911] §4.3.8 |
|  | unsupported-compression | y | y | MUST | [RFC2911] §4.3.8 |
|  | unsupported-document-format | y | y | MUST | [RFC2911] §4.3.8 |

For ease of understanding, the following value descriptions are presented in the order in which the reasons are likely to occur (if implemented), starting with the ‘none’ value.  Note: The order is improved over that in [RFC2911].  The (JD), (J), and (D) notation is identical to the “Job” and “Doc” columns in Table 11 indicating whether the keyword is defined for the “job-state-reasons” and/or “document-state-reasons” attributes:

'none' (JD):  There are no reasons for the job's or document’s current state. This state reason is semantically equivalent to "job-state-reasons" or “document-state-reasons” attribute without any value and MUST be used when there is no other value, since the 1setOf attribute syntax requires at least one value.

'job-hold-until-specified' (J):  The value of the job's "job-hold-until" attribute was specified with a time period that is still in the future.  The job MUST NOT be a candidate for processing until this reason is removed and there are no other reasons to hold the job.  This value SHOULD be supported if the "job-hold-until" Job Template attribute is supported.

'job-password-wait' (J): The Job is currently being held until the correct password is entered at the device.  If the "job-password" (octetString(255)) Operation attribute (See [prod-print2] section 4.1) is supported, this value MUST be supported.

'proof-print-wait' (J): The job is currently being held until the operator verifies the output of the proof print and performs a Release-Job operation.  If the "proof-print" (collection) Job Template attribute (See [prod-print2] section 5.9) is supported, this value MUST be supported.

'resources-are-not-supported' (JD): At least one of the resources needed by the job/document, such as media, fonts, resource objects, etc., is not supported on any of the physical printers for which the job/document is a candidate.  This condition MAY be detected when the job/document is accepted, or subsequently while the job/document is pending or processing, depending on implementation.  The job/document may (1) remain in its current state, (2) be moved to the 'pending-held' state, depending on implementation and/or job scheduling policy, or (3) scheduled normally, but the Printer is put into the 'stopped' state when the job/document is attempted to be processed on the Printer.  This value is intended for use with an implementation that supports the "user-defined-values-supported" Printer attribute (see [prod-print] section 6.1) which allows a job to be accepted with an unsupported 'name' value.

'resources-are-not-ready' (JD):  At least one of the resources needed by the job/document, such as media, fonts, resource objects, etc., is not ready on any of the physical printer's for which the job/document is a candidate.  This condition MAY be detected when the job/document is accepted, or subsequently while the job/document is pending or processing, depending on implementation.  The job/document may remain in its current state or be moved to the 'pending-held' state, depending on implementation and/or job/document scheduling policy.

'[job-]incoming' (JD):  Either (1) the Printer has accepted the Create-Job operation and is expecting additional Send-Document and/or Send-URI operations or (2) the Printer is retrieving/accepting document data as a result of a Print-Job, Print-URI, Send-Document or Send-URI operation.

'[job-]spooling' (JD): Same as 'job-incoming' with the specialization that the Printer is spooling the Job/Document data before processing it.  This value corresponds to the 'spool' or 'automatic' value of the "job-spooling-supported" Printer Description attribute.

'[job-]streaming' (JD): Same as 'job-incoming' with the specialization that the Printer is processing the Job/Document data as it is being received (that is, the document is not being spooled, but rather is being processed in chunks by the output device and is being imaged during reception).  This value corresponds to the 'stream' or 'automatic' value of the "job-spooling-supported" Printer Description attribute.

'job-data-insufficient' (J):  The Create-Job operation has been accepted by the Printer, but the Printer is expecting additional document data before it can move the job into the 'processing' state.  If a Printer starts processing before it has received all data, the Printer removes the  'job-data-insufficient' reason, but the 'job-incoming' remains.  If a Printer starts processing after it has received all data, the Printer removes the 'job-data-insufficient' reason and the 'job-incoming' at the same time.

'unsupported-compression' (JD): The job/document was aborted by the system because the Printer determined while attempting to decompress the document-data's that the compression is actually not among those supported by the Printer.  This value MUST be supported, since "compressions is a REQUIRED operation attribute.

'compression-error' (JD): The job/document was aborted by the system because the Printer encountered an error in the document-data while decompressing it.  If the Printer posts this reason, the document-data has already passed any tests that would have led to the 'unsupported-compression' job-state-reason/document-state-reason.

'document-access-error' (JD):  After accepting a Print-URI or Send-URI request, the Printer could not access one or more documents passed by reference.  This reason is intended to cover any file access problem, including file does not exist and access denied because of an access control problem.  The Printer MAY also indicate the document access error using the "job-document-access-errors" Job Description attribute (see the description of "job-document-access-errors" above).  Whether the Printer aborts the job/document and moves the job/document to the 'aborted' job state or prints all documents that are accessible and moves the job/document to the 'completed' job/document state and adds the 'completed-with-errors' value in the job's "job-state-reasons" or document’s “document-state-reasons” attribute depends on implementation and/or site policy.  This value SHOULD be supported if the Print-URI or Send-URI operations are supported.

'unsupported-document-format' (JD): The job/document was aborted by the system because the document-data's document-format is not among those supported by the Printer.  If the client specifies the document-format as 'application/octet-stream', the printer MAY abort the job/document and post this reason even though the format is a member of the "document-format-supported" printer attribute, but not among the auto-sensed document-formats.  This value MUST be supported, since "document-format" is a REQUIRED operation attribute.

'document-format-error' (JD): The job/document was aborted by the system because the Printer encountered an error in the document-data while processing it.  If the Printer posts this reason, the document-data has already passed any tests that would have led to the 'unsupported-document-format' job-state-reason/document-state-reason.

'submission-interrupted' (JD):  The job or document was not completely submitted for some unforeseen reason, such as: (1) the Printer has crashed before the job was closed by the client, (2) the Printer or the document transfer method has crashed in some non-recoverable way before the document data was entirely transferred to the Printer, (3) the client crashed or failed to close the job before the time-out period.  See the description of the “multiple-operation-time-out” Printer attribute in [RFC2911] Section 4.4.31.

'[job-]outgoing' (JD):  The Printer is transmitting the job/document to the output device.

'job-scheduling' (J): The printer is in the process of scheduling the processing for the Job.

'[job-]queued' (JD): Job is in the 'processing' state, but more specifically, the Printer has queued the Document data.

'[job-]transforming' (JD): Job/document is in the 'processing' state, but more specifically, the Printer is interpreting document data and producing another electronic representation.

'[job-]interpreting' (JD): Job/document is in the 'processing' state, but more specifically, the Printer is interpreting the document data.

'[job-]queued-for-marker' (JD): Job/document is in any of the 'pending-held', 'pending', or 'processing' states, but more specifically, the Printer has completed enough processing of the document to be able to start marking and the job/document is waiting for the marker.  Systems that require human intervention to release jobs using the Release-Job operation, put the job into the 'pending-held' job state.  Systems that automatically select a job to use the marker put the job into the  'pending' job state or keep the job in the 'processing' job state while waiting for the marker, depending on implementation.  All implementations put the job/document into (or back into) the 'processing' state when marking does begin.

'[job-]printing' (JD):  The output device is marking media. This value is useful for Printers which spend a great deal of time processing (1) when no marking is happening and then want to show that marking is now happening or (2) when the job/document is in the process of being canceled or aborted while the job/document remains in the 'processing' state, but the marking has not yet stopped so that impression or sheet counts are still increasing for the job/document.

'printer-stopped-partly' (J):  The value of the Printer's "printer-state-reasons" attribute contains the value 'stopped-partly'.

'printer-stopped' (J):  The value of the Printer's "printer-state" attribute is 'stopped'.

'job-suspending' (J): The printer is in the process of moving the Job from a processing condition to a suspended condition where other jobs can be processed.

'job-suspended' (J): The job has been suspended while processing using the Suspend-Current-Job operation and other jobs can be processed on the Printer.  The Job can be resumed using the Resume-Job operation which removes this value.  Note: Documents cannot be suspended and there is no Suspend-Current-Document operation.

'job-suspended-by-operator' (J): The Job has been indefinitely suspended by the printer operator.

'job-suspended-by-user' (J): The Job has been indefinitely suspended by the user.

'job-suspended-by-system' (J): The Job has been indefinitely suspended by the Printer’s system software during normal processing of the job.

'job-resuming' (J): The printer is in the process of moving the Job from a suspended condition to become a candidate for processing.

'[job-]canceled-by-user' (JD):  The job/document was canceled by the owner of the job using the Cancel-Job/Cancel-Document request, i.e., by a user whose authenticated identity is the same as the value of the originating user that created the Job object, or by some other authorized end-user, such as a member of the job owner's security group.  This ‘job-canceled’ value SHOULD be supported.  The ‘canceled-by-user’ value SHOULD be supported for the “document-state-reasons” attributes for use with the Cancel-Document.

'[job-]canceled-by-operator' (JD):  The job/document was canceled by the operator using the Cancel-Job/Cancel-Document request, i.e., by a user who has been authenticated as having operator privileges (whether local or remote).  If the security policy is to allow anyone to cancel anyone's job/document, then these values may be used when the job/document is canceled by other than the owner of the job.  For such a security policy, in effect, everyone is an operator as far as canceling jobs/documents is concerned.  These values SHOULD be supported if the implementation permits canceling by other than the owner of the job.

'[job-]canceled-at-device' (JD):  The job/document was canceled by an unidentified local user, i.e., a user at a console at the device.  This value SHOULD be supported if the implementation supports canceling jobs/documents at the console.

'aborted-by-system' (JD):  The job/document (1) is in the process of being aborted, (2) has been aborted by the system and placed in the 'aborted' state, or (3) has been aborted by the system and placed in the 'pending-held' state, so that a user or operator can manually try the job/document again.  This value SHOULD be supported.

'processing-to-stop-point' (J):  The requester has issued a Cancel-Job/Cancel-Document operation or the Printer object has aborted the job/document, but is still performing some actions on the job/document until a specified stop point occurs or job/document termination/cleanup is completed.
If the implementation requires some measurable time to cancel the job/document in the 'processing' or 'processing-stopped' job/document states, the Printer MUST use this value to indicate that the Printer object is still performing some actions on the job/document while the job/document remains in the 'processing' or 'processing-stopped' state.  After all the job's/document’s job/document description attributes have stopped incrementing, the Printer object moves the job/document from the 'processing' state to the 'canceled' or 'aborted' job/document states.

'service-off-line' (J):  The Printer is off-line and accepting no jobs.  All 'pending' jobs are put into the 'pending-held' state.  This situation could be true if the service's or document transform's input is impaired or broken.

'[job-]warnings-detected' (JD): If the Printer supports these values, the Printer MUST add the value to “job-state-reasons” / “document-state-reasons” when it generates the first warning message. That is, a single occurrence of this value is present in the “job-state-reasons” / “document-state-reasons” if the Printer has generated one or more warnings.

'[job-]completed-successfully' (JD):  The job/document completed successfully. There were no warnings or errors in printing.  This value SHOULD be supported.

'[job-]completed-with-warnings' (JD):  The print part of the job/document completed with warnings (whether or not there were save errors).  This value SHOULD be supported if the implementation detects warnings.  [RFC2911]

'[job-]completed-with-errors' (JD):  The print part of the job/document completed with errors (and possibly warnings too) (whether or not there were save errors).  This value SHOULD be supported if the implementation detects errors.  [RFC2911]

'job-saving' (J): The Printer is transmitting the Job to the save location. This is similar to the 'job-printing' value.

'job-saved-successfully' (J): The Printer saved the Job successfully.

'job-save-error' (J): The Printer did not save the Job successfully (whether or not it was printed successfully, printed with warnings, printed with errors, or not printed).  The Printer MUST ensure that none of the saved job is accessible, if it was unable to successfully save all of the job.

'job-restartable' (J): This job is retained (see the description of "job-state" above, specifically "Partitioning of Job States") and is currently able to be restarted using the Restart-Job operation (see [RFC2911] Section 3.3.7).  If 'job-restartable' is a value of the job's "job-state-reasons" attribute, then the Printer MUST accept a Restart-Job operation for that job.  This value SHOULD be supported if the Restart-Job operation is supported.

'queued-in-device' (JD): The job/document has been forwarded to a device or print system that is unable to send back status.  The Printer sets the job's/document’s "job-state" / “document-state” attribute to 'completed'  and adds the 'queued-in-device' value to the job's/document’s "job-state-reasons" / “document-state-reasons” attribute to indicate that the Printer has no additional information about the job/document and never will have any better information.  See the description of the "job-state" / “document-state” attribute above, specifically the portion on "Forwarding Servers".

NOTE:  The semantics of this attribute or the set of valid values may be different for different document formats.

## job-state-message (text(MAX))	Job Description attribute

## document-state-message (text(MAX))	Document Description attribute

This OPTIONAL attribute specifies information about the "job-state"/”document-state” and "job-state-reasons"/”document-state-reasons” attributes in human readable text.  If the Printer object supports this attribute, the Printer object MUST be able to generate this message in any of the natural languages identified by the Printer's "generated-natural-language-supported" attribute (see the "attributes-natural-language" operation attribute specified in [RFC2911] Section 3.1.4.1).

The value SHOULD NOT contain additional information not contained in the values of the "job-state" and "job-states-reasons" attributes, such as interpreter error information.  Otherwise, application programs might attempt to parse the (localized text).  For such additional information such as interpreter errors for application program consumption or specific document access errors, new attributes with keyword values, needs to be developed and registered.

## document-message (text(127))	Document Description

ISSUE 02:  Should the “document-message” Document Description attribute have a maximum of 127 octets, like the “job-message-from-operator” (text(127)) Job Description attribute in [RFC2911] section 4.3.16 or MAX (1023 octets) like “job-message-to-operator” (text(MAX)) Job Template attribute in IEEE-ISTO 5100.3 [prod-print] section 3.10?

This OPTIONAL Document Description contains a message from the user to the operator or a message from the operator or administrator system administrator or "intelligent" process to indicate to the end user the reasons for modification or other management action taken on the Document.  The Printer populates this Document Description attribute from the “document-message” (text(127) operation attribute value supplied by the client in any of the Document Creation operations (see section 3.1.1) or in a Cancel-Document or Cancel-Current-Document operations (see sections 3.9 or 3.10).  In addition, the user and the operator can set the value of this Document Description attribute explicitly using the Set-Document-Attributes operation (see section 3.8).

## document-printer-up-time (integer(1:MAX))	Document Description

This REQUIRED Document Description attribute indicates the amount of time (in seconds) that the Printer implementation has been up and running.  This attribute is an alias for the "printer-up-time" Printer Description attribute (see [RFC2911] section 4.4.29).

A client MAY request this attribute in a Get-Document-Attributes or Get-Documents request and use the value returned in combination with other requested Event Time Job Description Attributes in order to display time attributes to a user.  The difference between this attribute and the 'integer' value of a "time-at-xxx" attribute is the number of seconds ago that the "time-at-xxx" event occurred.  A client can compute the wall-clock time at which the "time-at-xxx" event occurred by subtracting this difference from the client's wall-clock time.

ISSUE 03:  The following 4 Job Creation operation attributes have corresponding Job Description attributes, but there isn’t a definition or an IANA Registration for any of these Job Description attributes:  “ipp-attribute-fidelity”, “job-mandatory-attributes”, “job-password”, and “job-password-encryption”.  OK to add Job Description attribute descriptions for each of them?  That would correspond to the PWG Semantic Model where they are Job Description attributes.

ISSUE 04 repeat:  The following 4 Job Creation operation attributes have corresponding Job Description attributes, since they are really Document attributes: “compression”, “document-format”, “document-name”, “document-natural-language”?  OK to add Document Description attribute descriptions for each of them?    That would correspond to the PWG Semantic Model where they are Document Description attributes.

ISSUE 05:  The Printer copies the following 11 Document Creation operation attributes to corresponding Document Description attributes, but there isn’t a definition or an IANA Registration for any of these Document Description attributes:  “attributes-charset”, “attributes-natural-language”, “compression”, “document-format”, “document-name”, “document-natural-language”, “impressions”, “k-octets”, “media-sheets”, “job-printer-uri”, and “last-document”.  OK to add operation attribute and Document Description attribute descriptions for each of them?

ISSUE 06:  The definitions of the following 16 remaining Document Description attributes are derived from the corresponding Job Description attributes that are not set by the client:  “current-page-order”, “date-time-at-completed”, “date-time-at-creation”, “date-time-at-processing”, “detailed-status-message”, “document-access-errors”,  “impressions-completed”, “impressions-completed-current-copy”, “k-octets-processed”, “media-sheets-completed”, “more-info”, “sheet-completed-copy-number”, “time-at-completed”, “time-at-creation”, “time-at-processing”, and “warnings-count”,  OK to add Document Description attribute descriptions for each of them?

# Printer attributes

This section lists all of the Printer attributes defined in other IPP documents.  Printer attributes are divided into three disjoint groups:

| Job Template attributes (JT) | These Printer attributes are related directly to the “xxx” Job Template attributes that a client can supply in a Job Creation request as follows: - “xxx-supported” indicate the supported values - “xxx-default” indicate the values that will be used if the client and the PDL omit the attribute. - “xxx-ready” indicate the supported values that are available without human intervention, for those few attributes that require human intervention, such as “media” and possibly “finishing”. |
| --- | --- |
| Printer Description attributes (PD) | These Printer attributes describe the Printer, such as “printer-name” and “printer-location”. Some of them are “xxx-supported” and “xxx-default” for Operation attributes that the client can supply in a Job Creation request. |
| neither (-) | These Printer attributes describe supported and defaults for member attributes of collection Job Template attributes. |

In a Get-Printer-Attributes request, a client can request all of the Printer attributes in the Job Template (JT) or Printer Description (PD) groups by supplying the ‘job-template’ or ‘printer-description’ group name, respectively.  The client can also request “all”.  The column labels are:

“Attribute Syntax” - indicates the attribute syntax (see [RFC2911] section 4.1).  Many “xxx-default” and “xxx-supported” attributes have the ‘collection’ syntax which consists of a number of member attributes.  These member attributes are not listed in this table when they are the same as the corresponding “xxx” attribute in Table 8.  Only those few Printer attribute collection attributes that do not have a corresponding Job Template attributes show the member attributes in this table.

“Grp” - indicates to which group the attribute belongs.

“Printer Conf.” - indicates the IPP/1.1 Printer conformance requirements for supporting the attribute.  Those attributes labeled as “MAY” may have conditional requirements that require their support, if the Printer supports a related attribute, operation, or feature.  For example, all Job Template attributes are OPTIONAL for a Printer to support.  However, if a Printer supports Job Template attribute “xxx”, then it MUST also support the corresponding “xxx-default” and “xxx-supported” attributes.  Similarly, if Event Notification is supported, then some of the attributes indicated as “MAY” in this table become MUST.

“Reference” - lists the IETF or IEEE-ISTO specification which defines the semantics of the attribute.

Table 12 - Printer attributes (Job Template and Printer Description)

| Printer attribute name (Job Template & Printer Description) | Attribute Syntax | Grp | Printer Conf. | Reference |
| --- | --- | --- | --- | --- |
| charset-configured | charset | PD | MUST | [RFC2911] §4.4.17 |
| charset-supported | 1setOf charset | PD | MUST | [RFC2911] §4.4.18 |
| color-supported | boolean | PD | MAY | [RFC2911] §4.4.26 |
| compression-supported | type3 keyword | PD | MUST | [RFC2911] §4.4.32 |
| copies-default | integer(1:MAX) | JT | MAY | [RFC2911] §4.2.5 |
| copies-supported | rangeOfInteger(1:MAX) | JT | MAY | [RFC2911] §4.2.5 |
| cover-back-default | collection | JT | MAY | [prod-print] §3.1.3 |
| cover-back-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.1.4 |
| cover-front-default | collection | JT | MAY | [prod-print] §3.1.3 |
| cover-front-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.1.4 |
| cover-type-supported | 1setOf type2 keyword | - | MAY | [prod-print] §3.1.2.1 |
| document-creation-attributes-supported | 1setOf type2 keyword | PD | MAY | [this-spec] §9.1 |
| document-format-default | mimeMediaType | PD | MUST | [RFC2911] §4.4.21 |
| document-format-supported | 1setOf mimeMediaType | PD | MUST | [RFC2911] §4.4.22 |
| document-format-varying-attributes | 1setOf type2 keyword | PD | MAY | [set-ops] §6.3 |
| document-overrides-supported | 1setOf type2 keyword |  | MAY | [override] §5.1.14 |
| finishings-col-default | collection | JT | MAY | [prod-print] §3.2.3 |
| finishings-col-ready | 1setOf collection | JT | MAY | [prod-print] §3.2.4 |
| finishings-col-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.2.5 |
| finishings-default | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.6 |
| finishings-ready | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.6 |
| finishings-supported | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.6 |
| finishing-template-supported | 1setOf name(MAX) | - | MAY | [prod-print] §3.2.1.1 |
| force-front-side-default | 1setOf integer(1:MAX) | JT | MAY | [prod-print] §3.3 |
| force-front-side-supported | rangeOfInteger(1:MAX) | JT | MAY | [prod-print] §3.3 |
| generated-natural-language-supported | 1setOf naturalLanguage | PD | MUST | [RFC2911] §4.4.20 |
| imposition-template-default | type3 keyword ¦ name(MAX) | - | MAY | [prod-print] §3.4 |
| imposition-template-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.4 |
| insert-after-page-number-supported | rangeOfInteger(0:MAX) | - | MAY | [prod-print] §3.5.1.1 |
| insert-count-supported | rangeOfInteger(0:MAX) | - | MAY | [prod-print] §3.5.2.1 |
| insert-sheet-default | 1setOf collection | JT | MAY | [prod-print] §3.5.4 |
| insert-sheet-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.5.5 |
| ippget-event-life | integer(15:MAX) | PD | MAY | [get-method] §8.1 |
| ipp-versions-supported | 1setOf type2 keyword | PD | MUST | [RFC2911] §4.4.20 |
| job-account-id-default | name(MAX) | JT | MAY | [prod-print] §3.6 |
| job-account-id-supported | boolean | JT | MAY | [prod-print] §3.6 |
| job-accounting-output-bin-default | type3 keyword ¦ name(MAX) | - | MAY | [prod-print] §3.8.3.1 |
| job-accounting-output-bin-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.8.3.2 |
| job-accounting-sheets-default | collection | JT | MAY | [prod-print] §3.8.4 |
| job-accounting-sheets-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.8.5 |
| job-accounting-sheets-type-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.8.1.1 |
| job-accounting-user-id-default | name(MAX) | JT | MAY | [prod-print] §3.7 |
| job-accounting-user-id-supported | boolean | JT | MAY | [prod-print] §3.7 |
| job-creation-attributes-supported | 1setOf type2 keyword | PD | MAY | [prod-print2] §7.1 |
| job-error-sheet-default | collection | JT | MAY | [prod-print] §3.7 |
| job-error-sheet-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.9.5 |
| job-error-sheet-type-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.9.1.1 |
| job-error-sheet-when-supported | 1setOf type2 keyword | - | MAY | [prod-print] §3.9.2.1 |
| job-hold-until-default | type3 keyword ¦ name(MAX) | JT | MAY | [RFC2911] §4.2.2 |
| job-hold-until-supported | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [RFC2911] §4.2.2 |
| job-impressions-supported | rangeOfInteger(0:MAX) | PD | MAY | [RFC2911] §4.4.34 |
| job-k-octets-supported | rangeOfInteger(0:MAX) | PD | MAY | [RFC2911] §4.4.33 |
| job-media-sheets-supported | rangeOfInteger(0:MAX) | PD | MAY | [RFC2911] §4.4.35 |
| job-message-to-operator-default | text(MAX) | JT | MAY | [prod-print] §3.12 |
| job-message-to-operator-supported | boolean | JT | MAY | [prod-print] §3.12 |
| job-password-supported | integer(0:127) | PD | MAY | [prod-print2] §7.2 |
| job-password-encryption-supported | 1setOf (type3 keyword ¦ name(MAX)) | PD | MAY | [prod-print2] §7.3 |
| job-priority-default | integer(1:100) | JT | MAY | [RFC2911] §4.2.1 |
| job-priority-supported | 1setOf integer(1:100) | JT | MAY | [RFC2911] §4.2.1 |
| job-settable-attributes-supported | 1setOf type2 keyword | PD | MAY | [set-ops] §6.2 |
| job-sheet-message-default | text(MAX) |  | MAY | [prod-print] §3.12 |
| job-sheet-message-supported | boolean |  | MAY | [prod-print] §3.12 |
| job-sheets-col-default | collection | JT | MAY | [prod-print] §3.11.3 |
| job-sheets-col-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.11.4 |
| job-sheets-default | type3 keyword ¦ name(MAX) | JT | MAY | [RFC2911] §4.2.3 |
| job-sheets-supported | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [RFC2911] §4.2.3 |
| job-spooling-supported | type2 keyword | PD | MAY | [prod-print2] §7.4 |
| max-save-info-supported | integer(1:MAX) | PD | MAY | [prod-print2] §7.5 |
| max-stitching-locations-supported | integer(1:MAX) | PD | MAY | [prod-print] §5.2 |
| media-back-coating-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.10.2 |
| media-col-database | 1setOf collection | - | MAY | [prod-print2] §7.6 |
| any “media-col” member attribute |  |  |  |  |
| media-col-default | collection | JT | MAY | [prod-print] §3.13.12 |
| any “media-col” member attribute |  |  |  |  |
| media-color-supported | 1setOf type2 keyword | - | MAY | [prod-print] §3.13.4.1 |
| media-col-ready | 1setOf collection | JT | MAY | [prod-print] §3.13.13 |
| any “media-col” member attribute |  |  |  |  |
| media-col-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.13.14 |
| media-default | type3 keyword ¦ name(MAX) | JT | MAY | [RFC2911] §4.2.11 |
| media-front-coating-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.10.1 |
| media-hole-count-supported | 1setOf rangeOfInteger(0:MAX) | - | MAY | [prod-print] §3.13.6.1 |
| media-info-supported | boolean | - | MAY | [prod-print] §3.13.3.1 |
| media-input-tray-check-default | type3 keyword ¦ name(MAX) | JT | MAY | [prod-print] §3.14 |
| media-input-tray-check-supported | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [prod-print] §3.14 |
| media-key-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.1.1 |
| media-order-count-supported | 1setOf rangeOfInteger(1:MAX) | - | MAY | [prod-print] §3.13.7.1 |
| media-pre-printed-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.5.1 |
| media-ready | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [RFC2911] §4.2.11 |
| media-recycled-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.11.1 |
| media-size-supported | 1setOf collection | - | MAY | [prod-print] §3.13.8.3 |
| media-supported | 1setOf (type3 keyword ¦ name(MAX)) | JT | MAY | [RFC2911] §4.2.11 |
| media-type-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.13.2.1 |
| media-weight-metric-supported | 1setOf rangeOfInteger(1:MAX) | - | MAY | [prod-print] §3.13.9.1 |
| multiple-document-handling-default | type2 keyword | JT | MAY | [RFC2911] §4.2.4 |
| multiple-document-handling-supported | 1setOf type2 keyword | JT | MAY | [RFC2911] §4.2.4 |
| multiple-document-jobs-supported | boolean | PD | MAY | [RFC2911] §4.4.16 |
| multiple-operation-time-out | integer(1:MAX) | PD | MAY | [RFC2911] §4.4.31 |
| natural-language-configured | naturalLanguage | PD | MUST | [RFC2911] §4.4.19 |
| notify-attributes-supported | 1setOf type2 keyword | PD | MAY | [ntfy] §5.3.4 |
| notify-events-default | 1setOf type2 keyword | PD | MAY | [ntfy] §5.3.3 |
| notify-events-supported | 1setOf type2 keyword | PD | MAY | [ntfy] §5.3.3 |
| notify-lease-duration-default | integer(0:67108863) | PD | MAY | [ntfy] §5.3.8 |
| notify-lease-duration-supported | 1setOf (integer(0: 67108863) ¦ rangeOfInteger(0:67108863)) | PD | MAY | [ntfy] §5.3.8 |
| notify-lease-expiration-time | integer(0:MAX) | PD | MAY | [ntfy] §5.4.3 |
| notify-max-events-supported | integer(2:MAX) | PD | MAY | [ntfy] §5.3.3 |
| notify-pull-method-supported | 1setOf type2 keyword | PD | MAY | [ntfy] §5.3.2 |
| notify-schemes-supported | 1setOf uriScheme | PD | MAY | [ntfy] §5.3.1 |
| number-up-default | integer(1:MAX) | JT | MAY | [RFC2911] §4.2.9 |
| number-up-supported | 1setOf (integer(1:MAX) ¦ rangeOfInteger(1:MAX)) | JT | MAY | [RFC2911] §4.2.9 |
| operations-supported | 1setOf type2 enum | PD | MUST | [RFC2911] §4.4.15 |
| orientation-requested-default | type2 enum | JT | MAY | [RFC2911] §4.2.10 |
| orientation-requested-supported | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.10 |
| output-bin-default | type2 keyword ¦ name(MAX) | JT | MAY | [output-bin] §2.1 |
| output-bin-supported | 1setOf (type2 keyword ¦ name(MAX)) | JT | MAY | [output-bin] §2.1 |
| page-delivery-default | type2 keyword | JT | MAY | [prod-print] §3.15 |
| page-delivery-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.15 |
| page-order-received-default | type2 keyword | JT | MAY | [prod-print] §3.16 |
| page-order-received-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.16 |
| page-overrides-supported | 1setOf type2 keyword | PD | MAY | [override] §5.2.9 |
| page-ranges-supported | boolean | JT | MAY | [RFC2911] §4.2.7 |
| pages-per-minute | integer(0:MAX) | PD | MAY | [RFC2911] §4.4.36 |
| pages-per-minute-color | integer(0:MAX) | PD | MAY | [RFC2911] §4.4.37 |
| pages-per-subset-supported | boolean | PD | MAY | [override] §5.3.2 |
| parent-printers-supported | 1setOf uri | PD | MAY | [adm-ops] §7.2 |
| pdl-override-supported | type2 keyword | PD | MUST | [RFC2911] §4.4.28 |
| presentation-direction-number-up-default | type2 keyword | JT | MAY | [prod-print] §3.17 |
| presentation-direction-number-up-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.17 |
| printer-current-time | dateTime | PD | MAY | [RFC2911] §4.4.30 |
| printer-detailed-status-messages | 1setOf text(MAX) | PD | MAY | [prod-print2] §7.7 |
| printer-driver-installer | uri | PD | MAY | [RFC2911] §4.4.8 |
| printer-info | text(127) | PD | MAY | [RFC2911] §4.4.6 |
| printer-is-accepting-jobs | boolean | PD | MUST | [RFC2911] §4.4.23 |
| printer-location | text(127) | PD | MAY | [RFC2911] §4.4.5 |
| printer-make-and-model | text(127) | PD | MAY | [RFC2911] §4.4.9 |
| printer-message-date-time | dateTime | PD | MAY | [set-ops] §6.5 |
| printer-message-from-operator | text(127) | PD | MAY | [RFC2911] §4.4.25 |
| printer-message-time | integer(MIN:MAX) | PD | MAY | [set-ops] §6.4 |
| printer-more-info | uri | PD | MAY | [RFC2911] §4.4.7 |
| printer-more-info-manufacturer | uri | PD | MAY | [RFC2911] §4.4.10 |
| printer-name | name(127) | PD | MUST | [RFC2911] §4.4.4 |
| printer-notify-server-uri | uri | PD | MAY | [not-srv] §4.1 |
| printer-resolution-default | resolution | JT | MAY | [RFC2911] §4.2.12 |
| printer-resolution-supported | 1setOf resolution | JT | MAY | [RFC2911] §4.2.12 |
| printer-settable-attributes-supported | 1setOf type2 keyword | PD | MAY | [set-ops] §6.1 |
| printer-state | type1 enum | PD | MUST | [RFC2911] §4.4.11 |
| printer-state-change-date-time | dateTime | PD | MAY | [ntfy] §6.2 |
| printer-state-change-time | integer(1:MAX) | PD | MAY | [ntfy] §6.1 |
| printer-state-message | text(MAX) | PD | MAY | [RFC2911] §4.4.13 |
| printer-state-reasons | 1setOf type2 keyword | PD | MUST | [RFC2911] §4.4.12 |
| printer-up-time | integer(1:MAX)) | PD | MUST | [RFC2911] §4.4.29 |
| printer-uri-supported | 1setOf uri | PD | MUST | [RFC2911] §4.4.1 |
| printer-xri-supported | 1setOf collection | PD | MAY | [set-ops] §6.6 |
| xri-uri | uri | PD | MUST | [set-ops] §6.6 |
| xri-authentication | 1setOf type2 keyword | PD | MUST | [set-ops] §6.6 |
| xri-security | 1setOf type2 keyword | PD | MUST | [set-ops] §6.6 |
| print-quality-default | type2 enum | JT | MAY | [RFC2911] §4.2.13 |
| print-quality-supported | 1setOf type2 enum | JT | MAY | [RFC2911] §4.2.13 |
| queued-job-count | integer(0:MAX) | PD | MUST | [RFC2911] §4.4.24 |
| reference-uri-schemes-supported | 1setOf uriScheme | PD | MAY | [RFC2911] §4.4.27 |
| separator-sheets-default | collection | JT | MAY | [prod-print] §3.18.3 |
| separator-sheets-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.18.4 |
| separator-sheets-type-supported | 1setOf (type3 keyword ¦ name(MAX)) | - | MAY | [prod-print] §3.18.1.1 |
| sheet-collate-default | type2 keyword | JT | MAY | [job-prog] §3.1 |
| sheet-collate-supported | 1setOf type2 keyword | JT | MAY | [job-prog] §3.1 |
| sides-default | type2 keyword | JT | MAY | [RFC2911] §4.2.8 |
| sides-supported | 1setOf type2 keyword | JT | MAY | [RFC2911] §4.2.8 |
| stitching-locations-supported | 1setOf (integer(0:MAX) ¦ rangeOfInteger(0:MAX)) | - | MAY | [prod-print] §3.2.2.3.1 |
| stitching-offset-supported | 1setOf (integer (0:MAX) ¦ rangeOfInteger(0:MAX)) | - | MAY | [prod-print] §3.2.2.2.1 |
| stitching-reference-edge-supported | 1setOf type2 keyword |  | MAY | [prod-print] §3.2.2.1.1 |
| subordinate-printers-supported | 1setOf uri | PD | MAY | [adm-ops] §7.1 |
| uri-authentication-supported | 1setOf type2 keyword | PD | MUST | [RFC2911] §4.4.2 |
| uri-security-supported | 1setOf type2 keyword | PD | MUST | [RFC2911] §4.4.3 |
| user-defined-values-supported | 1setOf type2 keyword | PD | MAY | [prod-print] §5.1 |
| which-jobs-supported | 1setOf type2 keyword | PD | MAY | [prod-print2] §7.8 |
| x-image-position-default | type2 keyword | JT | MAY | [prod-print] §3.19.2 |
| x-image-position-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.19.2 |
| x-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.3 |
| x-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.3 |
| xri-authentication-supported | 1setOf type2 keyword | PD | MAY | [set-ops] §6.8 |
| xri-security-supported | 1setOf type2 keyword | PD | MAY | [set-ops] §6.9 |
| xri-uri-scheme-supported | 1setOf uriScheme | PD | MAY | [set-ops] §6.7 |
| x-side1-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.4 |
| x-side1-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.4 |
| x-side2-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.5 |
| x-side2-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.5 |
| y-image-position-default | type2 keyword | JT | MAY | [prod-print] §3.19.6 |
| y-image-position-supported | 1setOf type2 keyword | JT | MAY | [prod-print] §3.19.6 |
| y-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.7 |
| y-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.7 |
| y-side1-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.8 |
| y-side1-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.8 |
| y-side2-image-shift-default | integer(MIN:MAX) | JT | MAY | [prod-print] §3.19.9 |
| y-side2-image-shift-supported | rangeOfInteger(MIN:MAX) | JT | MAY | [prod-print] §3.19.9 |

## document-creation-attributes-supported (1setOf type2 keyword)

This Printer Description attribute lists the keyword names of the Document Template and Operation attributes that the Printer will accept in the Create-Document, Send-Document, and Send-URI Document Creation operations.

## printer-state-reasons (1setOf type2 keyword)

| IPP Printer Description attribute name | IPP Attribute Syntax plus Values | Printer conformance | Reference |
| --- | --- | --- | --- |
| printer-state-reasons | 1setOf type2 keyword | MUST | [RFC2911] §4.4.12 |
|  | other | MAY | [RFC2911] §4.4.12 |
|  | none | MUST | [RFC2911] §4.4.12 |
|  | connecting-to-device | MAY | [RFC2911] §4.4.12 |
|  | cover-open | MAY | [RFC2911] §4.4.12 |
|  | deactivated | MAY | [adm-ops] §8.2 |
|  | developer-empty | MAY | [RFC2911] §4.4.12 |
|  | developer-low | MAY | [RFC2911] §4.4.12 |
|  | door-open | MAY | [RFC2911] §4.4.12 |
|  | fuser-over-temp | MAY | [RFC2911] §4.4.12 |
|  | fuser-under-temp | MAY | [RFC2911] §4.4.12 |
|  | hold-new-jobs | MAY | [adm-ops] §8.1 |
|  | input-tray-missing | MAY | [RFC2911] §4.4.12 |
|  | interlock-open | MAY | [RFC2911] §4.4.12 |
|  | interpreter-resource-unavailable | MAY | [RFC2911] §4.4.12 |
|  | marker-supply-empty | MAY | [RFC2911] §4.4.12 |
|  | marker-supply-low | MAY | [RFC2911] §4.4.12 |
|  | marker-waste-almost-full | MAY | [RFC2911] §4.4.12 |
|  | marker-waste-full | MAY | [RFC2911] §4.4.12 |
|  | media-empty | MAY | [RFC2911] §4.4.12 |
|  | media-jam | MAY | [RFC2911] §4.4.12 |
|  | media-low | MAY | [RFC2911] §4.4.12 |
|  | media-needed | MAY | [RFC2911] §4.4.12 |
|  | moving-to-paused | CMUST | [RFC2911] §4.4.12 |
|  | opc-life-over | MAY | [RFC2911] §4.4.12 |
|  | opc-near-eol | MAY | [RFC2911] §4.4.12 |
|  | output-area-almost-full | MAY | [RFC2911] §4.4.12 |
|  | output-area-full | MAY | [RFC2911] §4.4.12 |
|  | output-tray-missing | MAY | [RFC2911] §4.4.12 |
|  | paused | CMUST | [RFC2911] §4.4.12 |
|  | shutdown | MAY | [RFC2911] §4.4.12 |
|  | spool-area-full | SHOULD | [RFC2911] §4.4.12 |
|  | stopped-partly | MAY | [RFC2911] §4.4.12 |
|  | stopping | MAY | [RFC2911] §4.4.12 |
|  | timed-out | MAY | [RFC2911] §4.4.12 |
|  | toner-empty | MAY | [RFC2911] §4.4.12 |
|  | toner-low | MAY | [RFC2911] §4.4.12 |

This description is taken from [RFC2911] with additions from other IPP extension documents:

This REQUIRED Printer attribute supplies additional detail about the device's state.  Some of the these value definitions indicate conformance requirements; the rest are OPTIONAL.

Each keyword value MAY have a suffix to indicate its level of severity.  The three levels are: report (least severe), warning, and error (most severe).

'-report':  This suffix indicates that the reason is a "report". An implementation may choose to omit some or all reports. Some reports specify finer granularity about the printer state; others serve as a precursor to a warning. A report MUST contain nothing that could affect the printed output.

'-warning': This suffix indicates that the reason is a "warning". An implementation may choose to omit some or all warnings. Warnings serve as a precursor to an error. A warning MUST contain nothing that prevents a job from completing, though in some cases the output may be of lower quality.

'-error': This suffix indicates that the reason is an "error".  An implementation MUST include all errors. If this attribute contains one or more errors, printer MUST be in the stopped state.

If the implementation does not add any one of the three suffixes, all parties MUST assume that the reason is an "error".

If a Printer object controls more than one output device, each value of this attribute MAY apply to one or more of the output devices.  An error on one output device that does not stop the Printer object as a whole MAY appear as a warning in the Printer's "printer-state-reasons attribute".  If the "printer-state" for such a Printer has a value of 'stopped', then there MUST be an error reason among the values in the "printer-state-reasons" attribute.

The following standard keyword values are defined (in alphabetical order):

'other': The device has detected an error other than one listed in this document.

'none': There are not reasons. This state reason is semantically equivalent to "printer-state-reasons" without any value and MUST be used, since the 1setOf attribute syntax requires at least one value.

'connecting-to-device': The Printer object has scheduled a job on the output device and is in the process of connecting to a shared network output device (and might not be able to actually start printing the job for an arbitrarily long time depending on the usage of the output device by other servers on the network).

'cover-open': One or more covers on the device are open.

'deactivated':  A client has issued a Deactivate-Printer operation for the Printer object (see [adm-ops] section 3.4.1) and the Printer is in the process of becoming deactivated or has become deactivated. The Printer MUST reject all requests except Activate-Printer, queries (Get-Printer-Attributes, Get-Job-Attributes, Get-Jobs, etc.), Send-Document, and Send-URI (so that partial job submission can be completed - see [adm-ops] section 3.1.1) and return the ‘server-error-service-unavailable’ status code.

'developer-empty’: The device is out of developer.

'developer-low': The device is low on developer.

'door-open': One or more doors on the device are open.

'fuser-over-temp': The fuser temperature is above normal.

'fuser-under-temp': The fuser temperature is below normal.

'hold-new-jobs': The operator has issued the Hold-New-Jobs operation (see [adm-ops] section 3.3.1) or other means, but the output-device(s) are taking an appreciable time to stop.  Later, when all output has stopped, the “printer-state” becomes ‘stopped’, and the ‘paused’ value replaces the ‘moving-to-paused’ value in the “printer-state-reasons” attribute.  This value MUST be supported, if the Hold-New-Jobs operation is supported and the implementation takes significant time to pause a device in certain circumstances.

'input-tray-missing': One or more input trays are not in the device.

'interlock-open': One or more interlock devices on the printer are unlocked.

'interpreter-resource-unavailable': An interpreter resource is unavailable (i.e. font, form)

'marker-supply-empty': The device is out of at least one marker supply. (e.g. toner, ink, ribbon)

'marker-supply-low': The device is low on at least one marker supply. (e.g. toner, ink, ribbon)

'marker-waste-almost-full': The device marker supply waste receptacle is almost full.

'marker-waste-full': The device marker supply waste receptacle is full.

'media-empty': At least one input tray is empty.

'media-jam': The device has a media jam.

'media-low': At least one input tray is low on media.

'media-needed': A tray has run out of media.

'moving-to-paused':  Someone has paused the Printer object using the Pause-Printer operation (see [RFC2911] section 3.2.7) or other means, but the device(s) are taking an appreciable time to stop.  Later, when all output has stopped, the "printer-state" becomes 'stopped', and the 'paused' value replaces the 'moving-to-paused' value in the "printer-state-reasons" attribute.  This value MUST be supported, if the Pause-Printer operation is supported and the implementation takes significant time to pause a device in certain circumstances.

'opc-life-over': The optical photo conductor is no longer functioning.

'opc-near-eol': The optical photo conductor is near end of life.

'output-area-almost-full': One or more output area is almost full (e.g. tray, stacker, collator).

'output-area-full': One or more output area is full. (e.g. tray, stacker, collator)

'output-tray-missing': One or more output trays are not in the device

'paused': Someone has paused the Printer object using the Pause-Printer operation (see [RFC2911] section 3.2.7) or other means and the Printer object's "printer-state" is 'stopped'.  In this state, a Printer MUST NOT produce printed output, but it MUST perform other operations requested by a client.  If a Printer had been printing a job when the Printer was paused, the Printer MUST resume printing that job when the Printer is no longer paused and leave no evidence in the printed output of such a pause.  This value MUST be supported, if the Pause-Printer operation is supported.

'shutdown': Someone has removed a Printer object from service, and the device may be powered down or physically removed.  In this state, a Printer object MUST NOT produce printed output, and unless the Printer object is realized by a print server that is still active, the Printer object MUST perform no other operations requested by a client, including returning this value. If a Printer object had been printing a job when it was shutdown, the Printer NEED NOT resume printing that job when the Printer is no longer shutdown. If the Printer resumes printing such a job, it may leave evidence in the printed output of such a shutdown, e.g. the part printed before the shutdown may be printed a second time after the shutdown.

'spool-area-full': The limit of persistent storage allocated for spooling has been reached.  The Printer is temporarily unable to accept more jobs.  The Printer will remove this value when it is able to accept more jobs.  This value SHOULD be used by a non-spooling Printer that only accepts one or a small number jobs at a time or a spooling Printer that has filled the spool space.

'stopped-partly': When a Printer object controls more than one output device, this reason indicates that one or more output devices are stopped.  If the reason is a report, fewer than half of the output devices are stopped.  If the reason is a warning, fewer than all of the output devices are stopped.

'stopping': The Printer object is in the process of stopping the device and will be stopped in a while. When the device is stopped, the Printer object will change the Printer object's state to 'stopped'.  The 'stopping-warning' reason is never an error, even for a Printer with a single output device.  When an output-device ceases accepting jobs, the Printer will have this reason while the output device completes printing.

'timed-out': The server was able to connect to the output device (or is always connected), but was unable to get a response from the output device.

'toner-empty':  The device is out of toner.

'toner-low': The device is low on toner.

# New Values for Existing Printer Description Attributes

This section contains those attributes for which additional values are added.

## operations-supported (1setOf type2 enum)

The following “operation-id” values are added in order to support the new operations defined in this document:

Table 13 – Operation-id assignments

| Value | Operation Name | Reference |
| --- | --- | --- |
| 0x0032 | Cancel-Current-Document | [this-spec] §3.10 |
| 0x0033 | Cancel-Document | [this-spec] §3.9 |
| 0x0034 | Get-Document-Attributes | [this-spec] §3.5 |
| 0x0035 | Get-Documents | [this-spec] §3.7 |
| 0x0036 | Delete-Document | [this-spec] §3.11 |
| 0x0037 | Set-Document-Attributes | [this-spec] §3.8 |
| 0x0038 | Validate-Document | [this-spec] §3.5 |
| 0x0039 | Create-Document | [this-spec] §3.1 |
| 0x003A | Send-Data | [this-spec] §3.2 |

# Conformance Requirements

This section specifies the conformance requirements for Printers and clients.

## Printer conformance requirements

In order to conform to this specification, a Printer:

- MUST meet the conformance requirements for Printers specified in [RFC2911].

- MUST support the Document operations that section 3 REQUIRES a Printer to support.

- MUST support the operation attributes in the Send-Document operation and, if supported, the Send-URI operation that section 6 REQUIRES a Printer to support.

- MAY support any of the Document Template attributes in Document Creation operations as indicated in section 7 and MUST NOT support them for contexts for where they are not defined as indicated in the Table 9 with an  “n”.

- MUST support the Document Description attributes that section 8 REQUIRES a Printer to support.

- MUST support the Printer attributes that section 8.11 REQUIRES a Printer to support.

- MUST support the values of Printer Descriptions attribute that section 10 REQUIRES a Printer to support.

## Client conformance requirements

In order to conform to this specification, a client:

- MUST meet the conformance requirements for clients specified in [RFC2911]

- MAY support any of the Document operations defined in section 3.

- MUST supply the operation attributes that section 6 REQUIRES a client to supply in operation requests.

- MUST supply any Document Template attributes in the format that section 7 REQUIRES a client to supply in Document Creation operations and MUST NOT support them for contexts for where they are not defined as indicated in the Table 9 with an  “n”.

- SHOULD support any of the Document Description attributes that section 8 REQUIRES a Printer to support.

- MAY support any of the Printer attributes listed in section 8.11.

- MAY support any of the values of the Printer Descriptions attributes in section 10.

# Normative References

Note:  The references in the body of this IEEE-ISTO document use the more reader-friendly mnemonic symbols below, rather than the RFC number (except for [RFC2910] and [RFC2911]).  IETF RFCs and IANA registrations require using the RFC numbers in the body of the document for references.

[adm-ops]

Kugler, C, Hastings, T., Lewis, H., "Internet Printing Protocol (IPP): Job and Printer Administrative Operations", <draft-ietf-ipp-ops-set2-03.txt>, July 17, 2001.

[coll]

deBry, R., Hastings, T., Herriot, R., Ocke, K., and P. Zehler, "Internet Printing Protocol (IPP): The ‘collection’ attribute syntax", <draft-ietf-ipp-collection-05.txt>, work in progress, July 17, 2001.

[finish]

Hastings, T., Fullman, D., "Internet Printing Protocol (IPP): “finishings” attribute values extension”, IEEE-ISTO 5100.1-2001, February 5, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.1.pdf

[get-method]

Herriot, R., and T. Hastings, "Internet Printing Protocol (IPP):  The ‘ippget’ Delivery Method for Event Notifications", <draft-ietf-ipp-notify-get-08.txt>, September 5, 2002.

[job-prog]

Hastings, T., Lewis, H., and R. Bergman, "Internet Printing Protocol (IPP): Job Progress Attributes", RFC3381, September 2002.

[not-srv]

Hastings, T., Lewis, H, and Ira McDonald, “Distributed Notification Server”, September 26, 2002, work in progress to become IEEE-ISTO 5100.6-2002, ftp://ftp.pwg/org/pub/pwg/ipp/new_NOT/ipp-dist-not-service-020926.doc

[ntfy]

Herriot, R., Hastings, T., Shepherd, M., deBry, R., Isaacson, S., Martin, J., and R. Bergman, "Internet Printing Protocol/1.1: Event Notifications and Subscriptions", <draft-ietf-ipp-not-spec-08.txt>, November 19, 2001.

[output-bin]

Hastings, T., and R. Bergman, “Internet Printing Protocol (IPP): output-bin attribute extension”, IEEE-ISTO 5100.2-2001, February 7, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.2.pdf.

[override]

Herriot, R., Ocke, K., "Internet Printing Protocol (IPP): Override Attributes for Documents and Pages", IEEE-ISTO 5100.4-2001, February 7, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.4.pdf.

[prod-print]

Ocke, K., Hastings, T., "Internet Printing Protocol (IPP): Production Printing Attributes - Set1", IEEE-ISTO 5100.3-2001, February 12, 2001, ftp://ftp.pwg.org/pub/pwg/standards/pwg5100.3.pdf.

[prod-print2] Hastings, T., and D. Fullman, “Internet Printing Protocol (IPP): Production Printing Attributes - Set 2”, to become a PWG IEEE-ISTO standard, work in progress, August 21, 2002, ftp://ftp.pwg.org/pub/pwg/ipp/new_PPE/pwg-ipp-prod-print-set2-draft-v0_1-020821.pdf.

[RFC2911]

R. deBry, T. Hastings, R. Herriot, S. Isaacson, P. Powell, "Internet Printing Protocol/1.1: Model and Semantics", RFC 2911,  September 2000.

[set-ops]

Hastings, T., Herriot, R., Kugler, C., and H. Lewis, "Internet Printing Protocol (IPP): Job and Printer Set Operations", RFC 3380, September 2002.

# Informative References

[fsg-jt]

Free Software Group, Open Printing Job Ticket API, Visit: http://www.freestandards.org/openprinting/

[pwg-psi]

“Print Service Interface (PSI)”, work in progress to become a PWG IEEE-ISTO standard.  Visit http://www.pwg.org/.

[pwg-sm]

Zehler, P., and S. Albright, “PWG Semantic Model”, work in progress to become a PWG IEEE-ISTO standard, version 0.11, September 27, 2002.  ftp://ftp.pwg.org/pub/pwg/Semantic-Model/PWG-Semantic-Model.pdf

[RFC2565]

Herriot, R., Butler, S., Moore, P., and R.  Turner, "Internet Printing Protocol/1.0: Encoding and Transport", RFC 2565, April 1999.

[RFC2566]

R.  deBry, T.  Hastings, R.  Herriot, S.  Isaacson, and P.  Powell, "Internet Printing Protocol/1.0: Model and Semantics", RFC 2566, April 1999.

[RFC2567]

Wright, D., "Design Goals for an Internet Printing Protocol", RFC 2567, April 1999.

[RFC2568]

Zilles, S., "Rationale for the Structure and Model and Protocol for the Internet Printing Protocol", RFC 2568, April 1999.

[RFC2569]

Herriot, R., Hastings, T., Jacobs, N., Martin, J., "Mapping between LPD and IPP Protocols", RFC 2569, April 1999.

[RFC2616]

R.  Fielding, J.  Gettys, J.  Mogul, H.  Frystyk, L.  Masinter, P.  Leach, T.  Berners-Lee, “Hypertext Transfer Protocol - HTTP/1.1”, RFC 2616, June 1999.

[RFC3196]

Hastings, T., Manros, C., Zehler, P., Kugler, C., and H. Holst, “Internet Printing Protocol/1.1: Implementer’s Guide”, RFC 3196, November 2001.

[RFC3239]

Kugler, C., Lewis, H. and T. Hastings, “Internet Printing Protocol (IPP): Requirements for Job, Printer, and Device Administrative Operations”, RFC 3239, February 2002.

# IANA Considerations

This section contains the registration information for IANA to add to the various IPP Registries according to the procedures defined in RFC 2911 [RFC2911] section 6 to cover the definitions in this document.  The resulting registrations will be published in the http://www.iana.org/assignments/ipp-registrations registry.

Note to the RFC Editor:  Replace the References below with the RFC number for the appropriate document, so that it accurately reflects the content of the information for the IANA Registry.

## Attribute Registrations

The following table lists all the attributes defined in this document.  These are to be registered according to the procedures in RFC 2911 [RFC2911] section 6.2.

Attribute Name (attribute syntax):               Reference:   Section:

Document Description attributes:                 Reference:   Section:

attributes-charset (charset)                     [RFC2911]    4.3.19

attributes-natural-language (naturalLanguage)    [RFC2911]    4.3.20

compression (type3 keyword)                      [RFC2911]    3.2.1.1

current-page-order (type2 keyword)               [prod-print] 4.1

date-time-at-completed (dateTime)                [RFC2911]    4.3.14.7

date-time-at-creation (dateTime)                 [RFC2911]    4.3.14.5

date-time-at-processing (dateTime)               [RFC2911]    4.3.14.6

detailed-status-message (1setOf text (MAX))      [RFC2911]    4.3.10

document-access-errors (1setOf text(MAX))        [RFC2911]    4.3.11

document-format (documentFormat)                 [RFC2911]    3.2.1.1

document-id-uri (uri)                            [this spec]  8.2

document-message (text(127))                     [this spec]  8.10

document-name (name(MAX))                        [RFC2911]    3.2.1.1

document-natural-language (naturalLanguage)      [RFC2911]    3.2.1.1

document-number (integer(0:MAX))                 [this spec]  8.1

document-state (type1 enum)                      [this spec]  8.5

document-state-message (text(MAX))               [this spec]  8.10

document-state-reasons (1setOf type2 keyword)    [this spec]  8.7

document-uri (uri)                               [this spec]  8.3

impressions  (integer(0:MAX))                    [RFC2911]    4.3.17.2

impressions-completed (integer(0:MAX))           [RFC2911]    4.3.18.2

impressions-completed-current-copy               [job-prog]   4.4

(integer(0:MAX))

k-octets  (integer(0:MAX))                       [RFC2911]    4.3.17.1

k-octets-processed (integer(0:MAX))              [RFC2911]    4.3.18.1

last-document  (boolean)                         [RFC2911]    4.3.12

media-sheets  (integer(0:MAX))                   [RFC2911]    4.3.17.3

media-sheets-completed (integer(0:MAX))          [RFC2911]    4.3.18.3

more-info (uri)                                  [RFC2911]    4.3.4

printer-up-time (integer (1:MAX))                [RFC2911]    4.3.14.4

sheet-completed-copy-number (integer(0:MAX))     [job-prog]   4.2

time-at-completed (integer (MIN:MAX))            [RFC2911]    4.3.14.3

time-at-creation (integer (MIN:MAX))             [RFC2911]    4.3.14.1

time-at-processing (integer (MIN:MAX))           [RFC2911]    4.3.14.2

warnings-count (integer(0:MAX))                  [override]   6.1

Document Templates attributes:                   Reference:   Section:

copies (integer(1:MAX))                          [RFC2911]    4.2.5

cover-back (collection)                          [prod-print] 3.1

media (type3 keyword | name(MAX))                [prod-print] 3.1.1

media-col (collection)                           [prod-print] 3.1.1

cover-front (collection)                       [prod-print] 3.1

cover-type (type2 keyword)                     [prod-print] 3.1.2

media (type3 keyword | name(MAX))              [prod-print] 3.1.1

media-col (collection)                         [prod-print] 3.1.1

cover-type (type2 keyword)                       [prod-print] 3.1.2

finishings (1setOf type2 enum)                   [RFC2911]    4.2.6

[finish]     2

finishings-col (collection)                      [prod-print] 3.2

finishing-template (name(MAX))                 [prod-print] 3.2.1

stitching (collection)                         [prod-print] 3.2.2

stitching-locations (1setOf integer(0:MAX))  [prod-print] 3.2.2.3

stitching-offset (integer (0:MAX))           [prod-print] 3.2.2.2

stitching-reference-edge (type2 keyword)     [prod-print] 3.2.2.1

force-front-side (1setOf integer(1:MAX))         [prod-print] 3.3

imposition-template (type2 keyword | name(MAX))  [prod-print] 3.4

insert-sheet (1setOf collection)                 [prod-print] 3.5

insert-after-page-number (integer(0:MAX))      [prod-print] 3.5.1

insert-count (integer(0:MAX))                  [prod-print] 3.5.2

media (type3 keyword | name(MAX))              [prod-print] 3.5.3

media-col (collection)                         [prod-print] 3.5.3

media (type3 keyword | name(MAX))                [RFC2911]    4.2.11

media-col (collection)                           [prod-print] 3.13

media-back-coating (type3 keyword |            [prod-print] 3.13.10

name(MAX))

media-color (type3 keyword | name(MAX))        [prod-print] 3.13.4

media-front-coating (type3 keyword |           [prod-print] 3.13.10

name(MAX))

media-hole-count (integer(0:MAX))              [prod-print] 3.13.6

media-info (text(255))                         [prod-print] 3.13.3

media-key (type3 keyword | name(MAX))          [prod-print] 3.13.1

media-order-count (integer(1:MAX))             [prod-print] 3.13.7

media-preprinted (type3 keyword | name(MAX))   [prod-print] 3.13.5

media-recycled (type3 keyword | name(MAX))     [prod-print] 3.13.11

media-size (collection)                        [prod-print] 3.13.8

x-dimension (integer(0:MAX))                 [prod-print] 3.13.8.1

y-dimension (integer(0:MAX))                 [prod-print] 3.13.8.2

media-type (type3 keyword | name(MAX)          [prod-print] 3.13.2

media-weight-metric (integer(0:MAX))           [prod-print] 3.13.9

media-input-tray-check (type3 keyword |          [prod-print] 3.14

name(MAX))

number-up (integer(1:MAX))                       [RFC2911]    4.2.9

orientation-requested (type2 enum)               [RFC2911]    4.2.10

output-bin (type2 keyword | name(MAX))           [output-bin] 2.1

page-delivery (type2 keyword)                    [prod-print] 3.15

page-order-received (type2 keyword)              [prod-print] 3.16

page-overrides (1setOf collection)               [override]   5.2

document-copies (1setOf rangeOfInteger(MAX))   [override]   5.2.3

input-documents (1setOf rangeOfInteger(MAX))   [override]   5.2.1

output-documents (1setOf rangeOfInteger(MAX))  [override]   5.2.2

pages (1setOf rangeOfInteger(MAX))             [override]   5.2.4

<any other Job Template attributes that

affect pages, such as media or media-col (

page-ranges (1setOf rangeOfInteger(1:MAX))       [RFC2911]    4.2.7

pages-per-subset (1setOf integer(1:MAX))         [prod-print] 5.3

presentation-direction-number-up (type2          [prod-print] 3.17

keyword)

print-quality (type2 enum)                       [RFC2911]    4.2.13

printer-resolution (resolution)                  [RFC2911]    4.2.12

separator-sheets (collection)                    [prod-print] 3.18

media (type3 keyword | name(MAX))              [prod-print] 3.18.2

media-col (collection)                         [prod-print] 3.18.2

separator-sheets-type (type3 keyword |         [prod-print] 3.18.1

name(MAX))

sheet-collate (type2 keyword)                    [job-prog]   3.1

sides (type2 keyword)                            [RFC2911]    4.2.8

x-image-position (type2 keyword)                 [prod-print] 3.19.2

x-image-shift (integer(MIN:MAX))                 [prod-print] 3.19.3

x-side1-image-shift (integer(MIN:MAX))           [prod-print] 3.19.4

x-side2-image-shift (integer(MIN:MAX))           [prod-print] 3.19.5

y-image-position (type2 keyword)                 [prod-print] 3.19.6

y-image-shift (integer(MIN:MAX))                 [prod-print] 3.19.7

y-side1-image-shift (integer(MIN:MAX))           [prod-print] 3.19.8

y-side2-image-shift (integer(MIN:MAX))           [prod-print] 3.19.9

Job Template attributes:                         Reference:   Section:

job-copies (integer(1:MAX))                      [this spec]  7.1.1

job-cover-back (collection)                      [this spec]  7.1.2

<member attributes are the same as “cover-back”>

job-cover-front (collection)                     [this spec]  7.1.3

<member attributes are the same as “cover-front”>

job-finishings (1setOf type2 enum)               [this spec]  7.1.4

job-finishings-col (1setOf collection)           [this spec]  7.1.5

<member attributes are the same as “finishings-col”>

Printer Descriptions:                            Reference:   Section:

document-creation-attributes-supported (1setOf type2 keyword)

[this spec]  9.1

Operation attributes:                            Reference:   Section:

document-mandatory-attributes (1setOf type2 keyword)

[this spec]   6.2.3

document-message (text(127))                     [this spec]   3.9.1.1

input-document-number (integer(1:MAX))           [this spec]   3.1.1.1

job-mandatory-attributes (1setOf type2 keyword)  [this spec]   6.2.2

last-document (boolean)                          [this spec]   3.1.1.2

## Additional Enum Attribute Value Registrations

The following table lists all the new enum attribute values defined in this document as additional type2 enum values.  These are to be registered according to the procedures in RFC 2911 [RFC2911] section 6.1.

Value      Name                                Reference   Section:

--------   ----------------------              ---------   -------

document-state (type1 enum)                      [this spec]    §8.5

3   pending                                    [this spec]    §8.5

5   processing                                 [this spec]    §8.5

7   canceled                                   [this spec]    §8.5

8   aborted                                    [this spec]    §8.5

9   completed                                  [this spec]    §8.5

job-finishings (1setOf type2 enum)               [this spec]    §7.1.4

<enum values are the same as “finishings”>

operations-supported (type2 enum):

0x0032     Cancel-Current-Document             [this-spec]    §3.10

0x0033     Cancel-Document                     [this-spec]    §3.9

0x0034     Get-Document-Attributes             [this-spec]    §3.5

0x0035     Get-Documents                       [this-spec]    §3.7

0x0036     Delete-Document                     [this-spec]    §3.11

0x0037     Set-Document-Attributes             [this-spec]    §3.8

0x0038     Validate-Document                   [this-spec]    §3.5

0x0039     Create-Document                     [this-spec]    §3.1

0x003A     Send-Data                           [this-spec]    §3.2

Operation Registrations

The following table lists all of the operations defined in this document.  These are to be registered according to the procedures in RFC 2911 [RFC2911] section 6.4.

Operations:                                    Reference:   Section:

Cancel-Current-Document                        [this-spec]    §3.10

Cancel-Document                                [this-spec]    §3.9

Get-Document-Attributes                        [this-spec]    §3.5

Get-Documents                                  [this-spec]    §3.7

Delete-Document                                [this-spec]    §3.11

Set-Document-Attributes                        [this-spec]    §3.8

Validate-Document                              [this-spec]    §3.5

Create-Document                                [this-spec]    §3.1

Send-Data                                      [this-spec]    §3.2

# Internationalization Considerations

The internationalization considerations for this extension for handling the Document object are the same as for handling the Job object in [RFC2911].

# Security Considerations

The Document object should provide no more increased security considerations than the Job object.  The Printer MUST provide the same access control for Document objects as for Job objects.

# Contributors

# Acknowledgments

# Author’s Address

Tom Hastings

Xerox Corporation

737 Hawaii St.

El Segundo, CA 90245

Phone: 310 333-6413

Fax:  310 333-5514

e-mail: hastings@cp10.es.xerox.com

Peter Zehler

Xerox Corporation

MS: 0128-30E

800 Phillips Road

Webster, NY 14580-9701

Phone: 1+585+2658755

Fax:

e-mail: PZehler@crt.xerox.com

Additional contributors:

Don Fullman, Xerox

Ira McDonald, High North

# Appendix A: Document operations compared to Job operations

This appendix compares the Document operations with the existing Job operations to show the parallelism.

Table 14 compares the Job and Document operations.  The two Conf. columns indicate the conformance requirements (REQUIRED = REQ, OPTIONAL = OPT) for the Job and Document operations respectively.  A “REQ” in the Document Conformance column means if the Document object is supported.  Whether a Document operation is a method on the Job object or the Document object depends on the modeling regime and is not an essential part of the semantics described here, since these semantics are intended to apply to both to object-oriented and object-based protocols, APIs, and implementations.

Table 14 - Document operations compared to Job operations

| Job operations | Job Conf. | Document operations | Doc Conf |
| --- | --- | --- | --- |
| Cancel-Current-Job | OPT | Cancel-Current-Document | OPT |
| Cancel-Job | REQ | Cancel-Document | REQ |
| Create-Job | OPT | Create-Document | REQ |
| Get-Job-Attributes | REQ | Get-Document-Attributes | REQ |
| Get-Jobs | REQ | Get-Documents | REQ |
| Hold-Job |  | N/A, can only hold an entire job. |  |
| Print-Job | REQ | Send-Document (existing operation - see [RFC2911] with an additional Document attributes group defined in section 3.3) | REQ |
| Print-URI | OPT | Send-URI (existing operation - see [RFC2911] with an additional Document attributes group defined in section 3.4) | OPT |
| N/A |  | Send-Data | REQ |
| Promote-Job | OPT | N/A, can only promote an entire job |  |
| Purge-Jobs | OPT | Delete-Document | OPT |
| Release-Held-New-Jobs | OPT | N/A, can only release entire jobs |  |
| Release-Job | OPT | N/A, can only release an entire job |  |
| Reprocess-Job | OPT | N/A, can only reprocess an entire job |  |
| Restart-Job | OPT | N/A, can only restart an entire job |  |
| Resume-Job | OPT | N/A, can only resume an entire job |  |
| Schedule-Job-After | OPT | N/A, can only schedule an entire job |  |
| Set-Job-Attributes | OPT | Set-Document-Attributes | OPT |
| Suspend-Current-Job | OPT | N/A, can only suspend an entire job in order to run another one |  |
| Validate-Job | REQ | Validate-Document | REQ |

# Annex B: Change Log (informative)

Version 0.5, 25 October 2002, as a result of the PWG Semantic Model telecon, October 24, 2002:

- Fixed the title to say “Proposed Standard” and removed the 5100.n number until the document is approved by the PWG Last Call to progress as a Draft Standard.

- Changed Cancel-Document from OPTIONAL to REQUIRED.

- Added Table 3 and Table 4 to show the request and response signatures of the Document Creation and Validation operations.

- Renamed the “document-number” operation attribute to “input-document-number”, to agree with [override] and because the Document object is really the Input Document (as opposed to the Output Document).  The Printer copies this value to the Document’s “document-number” Document Description attribute.

- Clarified that the Printer MUST reject a Create-Document request if the “document-number” already exists.

- Clarified that the client cannot supply Subscription attributes in any Document Creation requests.

- Added “document-message” (text(127)) operation attribute to the Document Creation requests to go with the “document-message” agreement (see point #15 ).

- Fixed Create-Document response to return Job status attributes in Group 3 and Document status attributes in Group 4.

- Required the Printer to return just the “document-number” and “document-id-uri” (uri) Document Description attribute in a Document Creation response.

- Added the “document-mandatory-attributes” operation attribute.

- Separated the returned Job status attributes and the Document status attributes into two groups, one Job Attributes and the other Document attributes.

- Allowed the client to supply or omit the “document-uri” operation attribute in a Validate-Job request.

- Clarified that the “document-number” operation attribute MUST follow the Printer and Job Target in Group 1.

- Changed the Get-Documents response when there is no “requested-attributes” to simply “document-number” and “document-id-uri” to be parallel with the Get-Jobs operation.

- Changed the “message” operation attribute in Cancel-Document and Cancel-Current-Document to “document-message” operation attribute which the Printer copies to the “document-message” Document Description attribute for use both by the user and the operator.

- Clarified that the “document-state-reasons” values never affect Job scheduling.

- Made Job Level - page override programming higher precedence than Document level Document object attributes.

- Clarified PDL document data precedence and its relationship to Job Template attributes ending with “-requested”.

- In Table 6, flagged the operation attributes that the Printer copies to the corresponding Job and Document Description attributes in Document Creations operations.

- Fixed “job-mandatory-attributes” operation attribute so its only supplied at Job level, now that we have “document-mandatory-attributes” operation attribute.

- Clarified that “ipp-attribute-fidelity” can be supplied at both the Job and Document Levels and how it interacts with “job-mandatory-attributes” and “document-mandatory-attributes” operation attributes.

- Disallowed “output-bin” at the Document Level (and Page Level).  It subsets the single stream of Input-Pages across Input-Documents.

- Disallowed “page-ranges” at the Page Level.

- Disallowed “pages-per-subset” at the Document level (and Page Level).

- Disallowed “separator-sheets” at the Page Level.

- Added “document-printer-up-time” Document Description attribute for Get-Documents and Get-Document-Attributes responses that the Printer MUST support, just like “job-printer-up-time” operation attribute for Get-Jobs and Get-Job-Attributes.

Version 0.4, 14 October 2002:

- Added the operation specifications.

- Added the Create-Document and Send-Data to be more parallel with Jobs and support PWG PSI.

- Resolved the 15 issues in version 0.3 as discussed on the mailing list.

- Added the Attribute Precedence section 5.

- Added [prod-print2] attributes.

- Added job-mandatory-attributes, job-cover-back, job-cover-front, job-copies, job-finishings, job-finishings-col. document-id-uri, and redirect-uri.

Version 0.3, 9 September 2002:

- Reformatted as an IEEE-ISTO standard and added the usual sections for a standard.

- Added “original-requesting-user-name” Operation attribute to Table 6.

- Added “y” to “output-bin” in Table 8 so it MAY be a Document Template attribute.

- Clarified the * and ** in Table 9 - Job and Document Description attributes.

- Added section 8.11 all of the Printer attributes defined In any IPP standard.

- Allocated operation-id enums in section 10 to the 7 Document operations.

- Added the conformance section 11

- Completed the IANA Considerations section 14.

The Document object is not a sub-job.  If Job trees are desired where sub-jobs can be on different network nodes, such as in JDF, then a separate specification should be generated for Job trees.

The Create-Document operation maps to the PWG PSI Send-Document-By-Post operation and the Send-Data maps to the HTTP Post operation.

The PWG Semantic Model [pwg-sm] calls the Operation attributes supplied by the client: Job Description attributes or Document Description attributes for the Job Creation and Document Creation operations, respectively.  The [pwg-sm] also calls Job Template attributes and Document Template attributes: Job Processing and Document Processing attributes, respectively.  Each Processing attribute is defined to be a Job Processing or a Document Processing attribute, but is not both.  The client can supply both Job Processing and Document Processing attributes in a Job Creation operation, in which case the Document Processing attributes server as defaults for the Document objects.  The Printer copies all of these client’s supplied attributes to the Job or Document object.  Read-only Job and Document attributes that the system sets are called Job State and Document State attributes (instead of Description attributes).

The Send-Document operation is REQUIRED in this specification while it was OPTIONAL in [RFC2911].  This difference is because this specification REQUIRES Printer support of multiple document jobs, while [RFC2911] did not.

The PWG PSI [pwg-psi] specification includes a Validate-Document operation with the “document-uri” operation attribute that the Printer MUST validate.

The Cancel-Document operation is OPTIONAL, whereas the corresponding Cancel-Job is REQUIRED.

In some modeling regimes, the Cancel-Current-Document operation might be a Device method, rather than a Job or Document method.

The Printer MUST NOT copy up to the Job object any Document attributes supplied at the Document Level in Create-Document, Send-Document, or Send-URI to the Job object as observable by the client in any Get-Job-Attributes operation responses.

The Printer MUST NOT copy up to the Job object any Document attributes supplied at the Document Level in Create-Document, Send-Document, or Send-URI to the Job object as observable by the client in any Get-Job-Attributes operation responses.

The “compression”, “document-format”, “document-name”, and “document-natural-language” MAY be supplied in the Create-Job Job Creation operation as an extension to [RFC2911] as specified in [override].  If supplied in any Job Creation operation, the Printer copies them to corresponding Job Description attributes for subsequent Job query.  These supplied attributes become defaults for the Documents in the job and used if the client does not supply the corresponding operation attribute in the Send-Document or Send-URI Document Creation operation.  ISSUE 04:  With the Document object, do we want to also defines Job Description attributes for: “compression”, “document-format”, “document-name”, “document-natural-language”, since they are really Document attributes?

See long footnote for the “compression” operation attribute in this table.

See long footnote for the “compression” operation attribute in this table.

See long footnote for the “compression” operation attribute in this table.

According to [override] section 9.2.2, the “document-overrides” (collection) attribute MAY be supplied by the client in a Send-Document or Send-URI request as an Operation attribute to apply document overrides to this and/or subsequent documents in the job.  The Printer MUST merge this operation attribute into the “document-overrides” Document Template attribute, if any.  See the “page-overrides” Job Template attribute in Table 8 for the listing of the member attributes.  However, with the introduction of the Document object, the “document-overrides” (collection) attribute SHOULD NOT be used (either as a Job Template attribute or an Operation attribute).  Instead, the client simply supplies the Document Template attributes (see Table 8) for each Document Creation request (in a new Document Template attribute group) without needing a collection.

The “document-uri” operation attribute MUST be supplied in the Print-URI Job Creation operation and the Send-URI Document Creation operation, MAY be supplied in a Validate-Document operation, and MUST NOT be supplied in other operations.

The “input-document-number” operation attribute sets the Document’s “document-number” Document Description attribute (see section 8.1).

According to [override] section 9.2.3, the “page-overrides” (collection) attribute MAY be supplied by the client in a Send-Document or Send-URI request as an Operation attribute to apply page overrides to this and/or subsequent documents in the job.  The Printer MUST merge this operation attribute into the “page-overrides” Document Template attribute, if any.  See the “page-overrides” Job Template attribute in Table 8 for the listing of the member attributes.  However, with the introduction of the Document object, the “page-overrides” (collection) attribute SHOULD be more simply supplied as one of the Document Template attributes for this document only.

The Printer copies the “printer-uri” operation attribute to the Job’s “job-printer-uri” Job Description attribute.

The Printer or Notification Server returns the “redirect-uri” operation attribute in order to redirect the Get-Notification operation to a Notification Server or back to the original Printer.

The IPP “copies” attribute is an extensive attribute, so its effect when supplied at the job level is not always inherited by the documents in a multi-document job.  Instead, its effect depends on the value of the “multiple-document-handling” Job Template attribute.  The ‘single-document’ and ‘single-document-new-sheet’ values produce copies of the job as a whole with the multiple input documents concatenated into a single output document for each job copy.  The ‘separate-document-uncollated-copies’ value produce N copies of the first input document followed by N copies of the second input document, etc.  The ‘separate-document-collated-copies’ produce N successive job copies, each job copy consisting of 1 copy of the first document followed by 1 copy of the second document, etc.

The client can supply the “document-overrides” operation attribute in the Send-Document or Send-URI operations (see [override]).

The Printer MUST support “document-format”, “document-name”, “compression”, “document-natural-language”, and “page-ranges” with the “input-documents” member attribute, but not the “output-documents” member attribute.

The IPP “finishings” and “finishings-col” attributes are extensive attributes, so their effect when supplied at the Job Level is not always inherited by the documents in a multi-document job.  Instead, their effect depends on the value of the “multiple-document-handling” Job Template attribute.  The ‘single-document’ and ‘single-document-new-sheet’ cause the finishing to be applied to each job copy as a whole.  The ‘separate-documents-collated-copies’ and ‘separate-document-uncollated-copies’ values cause the finishing to be applied to each document.

The effect of the IPP “force-front-side” and “insert-sheet” attributes when supplied at the job level of a multi-document job depends on the value of the “multiple-document-handling” Job Template attribute.  For the ‘single-document’ and ‘single-document-new-sheet’ values, the pages are numbered as a single set from 1 to n for the job as a whole.  For the ‘separate-documents-collated-copies’ and ‘separate-document-uncollated-copies’ values, the pages are numbered from 1 to n for each document separately.

The “job-copies” Job Template attribute MUST NOT be supplied at the Document or Page Level.  The “job-copies” Job Template attribute was introduced for use in clients or Printers that don’t support “multiple-document-handling” = ‘single-document’ or ‘single-document-new-sheet’.

The “job-cover-back” Job Template attribute MUST NOT be supplied at the Document or Page Level.  The “job-cover-back” Job Template attribute was introduced for use in clients or Printers that don’t support “multiple-document-handling” = ‘single-document’ or ‘single-document-new-sheet’.

The “job-cover-front” Job Template attribute MUST NOT be supplied at the Document or Page Level.  The “job-cover-front” Job Template attribute was introduced for use with clients or Printers that don’t support “multiple-document-handling” = ‘single-document’ or ‘single-document-new-sheet’.

The “job-finishing” Job Template attribute MUST NOT be supplied at the Document or Page Level.  The “job-finishings” Job Template attribute was introduced for use with clients or Printers that don’t support “multiple-document-handling” = ‘single-document’ or ‘single-document-new-sheet’.

The “job-finishing-col” Job Template attribute MUST NOT be supplied at the Document or Page Level.  The “job-finishings-col” Job Template attribute was introduced for use with clients or Printers that don’t support “multiple-document-handling” = ‘single-document’ or ‘single-document-new-sheet’.

The “job-message-to-operator” Job Template attribute has no corresponding Document Template attribute.  Instead, the user and operator communicate with each other using the same “document-message” operation attribute which the Printer copies to the “document-message” Document Description attribute.

The effect of the IPP “page-overrides” Job Template attribute when supplied at the job level of a multi-document job depends on the value of the “multiple-document-handling” Job Template attribute.  For the ‘single-document’ and ‘single-document-new-sheet’ values, the pages are numbered as a single set from 1 to n for the job as a whole.  For the ‘separate-documents-collated-copies’ and ‘separate-document-uncollated-copies’ values, the pages are numbered from 1 to n for each document separately.

The effect of the IPP “page-ranges” Job Template attribute when supplied at the job level of a multi-document job depends on the value of the “multiple-document-handling” Job Template attribute.  For the ‘single-document’ and ‘single-document-new-sheet’ values, the pages are numbered as a single set from 1 to n for the job as a whole.  For the ‘separate-documents-collated-copies’ and ‘separate-document-uncollated-copies’ values, the pages are numbered from 1 to n for each document separately.

The IPP “pages-per-subset” Job Template attribute MUST be supplied at the Job Level and can only be used when the “multiple-document-handling” Job Template attribute is ‘separate-documents-collated-copies’ or ‘separate-documents-uncollated-copes’, since the purpose of the “pages-per-subset” attribute is to produce separate output documents for each subset.  Otherwise, the Printer MUST ignore the “pages-per-subset” attribute.

When documents in a multiple document job have the same page order, the Printer populates both the Job and Document “current-page-order” with the same value, so that clients that only know about the Job Description attribute will get a value.  When the page orders differ, the Printer doesn’t populate the “current-page-order” Job Description attribute, but only the Document Description attribute of each Document.

The “document-message” (text(127)) Document Description attribute has a corresponding “job-message-from-operator” (text(127)) Job Description attribute (see [RFC2911] section 4.3.16) which handles operator to user communication only and a “job-message-to-operator (text(MAX)) Job Template attribute (see [prod-print] section 3.10) for user to operator communication.

The Document Description attribute that corresponds to the “job-id” Job Description attribute is the (new) “document-number” attribute.

The “job-message-from-operator” Job Description attribute has no corresponding “document-message-from-operator” (text(127)) Document Description attribute.  Instead, both the user and the operator use the “document-message” (text(127)) operation/Document Description attribute (see section 8.10) to communicate with each other.

The Document Description attribute that corresponds to the “job-name” Job Description attribute is the (existing) “document-name” attribute defined in [RFC2911].

The “job-printer-make-and-model” Job Description attribute MUST be supported if the “job-save-disposition” Job Template attribute is supported.

The Document Description attribute that corresponds to the “job-state” Job Description attribute is the (new) “document-state” attribute.

The Document Description attribute that corresponds to the “job-state-message” Job Description attribute is the (new) “document-state-message” attribute.

The Document Description attribute that corresponds to the “job-state-reasons” Job Description attribute is the  (new) “document-state-reasons” attribute.

The Document Description attribute that corresponds to the “job-uri” (uri) Job Description attribute is the (new) “document-id-uri” (uri) attribute.  However, the value is only a unique ID.  The client cannot use the “document-id-uri” attribute URI value as a target of a Document object operation; the “job-uri” has proved problematic enough as a target.

The Document object is not really a sub-job.  If the Document object were its own independent active object, i.e., were a sub-job, then the Document state would affect the job state and there would be complex and debatable roll-up rules for how the various “document-state” values of all of the Job’s document are rolled up to give a single value for the Job’s “job-state” attribute.  Instead, the Job state reflects the overall processing of the job and is the same whether or not the Document object is supported.

For the PWG Semantic Model [pwg-sm], the “job-” prefix is being dropped altogether so that the same values can be used for both the JobStateReasons and DocumentStateReasons attributes.

The Document Description attribute that corresponds to the “job-state-reasons” Job Description attribute is the  (new) “document-state-reasons” attribute.

The “media-col-database” attribute is not returned when the client requests ‘all’ or ‘printer-description’ attribute group because there MAY be a large number of values.  Instead the client MUST query “media-col-database” explicitly in a Get-Printer-Attributes request.

The 'moving-to-paused' value MUST be supported if the Pause-Printer operation is supported and it takes significant time to move to paused.

The 'paused' value MUST be supported if the Pause-Printer operation is supported.

The Send-Document operation is OPTIONAL for a Printer to conform to [RFC2911], but is REQUIRED for a Printer to conform to this Document object specification.