DRAFT  Some issues are highlighted with yellow.

ipp-mib-access-980503.pdf

S. Isaacson

Novell, Inc.

T. Hastings

Xerox Corporation

R. Herriot

Sun Microsystems

May 3, 1998

IPP Device and MIB access

Version 0.03

Abstract

This document introduces a new Device object into the IPP object model.  An IPP Printer object may support one or more Device objects which each represent a physical device as shown in the Internet Printing Protocol/1.0: Model and Semantics specification.  This document provides read access to such device objects by defining a mapping from table entries in the Printer MIB to new algorithmically generated attribute names.  New algorithmically generated attribute group names allow convenient read-only access to any row, any column, and any entire table in the Printer MIB, as well as the entire Printer MIB.  A general algorithmically generated attribute name provides access to any single SNMP object in any MIB.  These new attribute names and attribute group names are supplied in the "requested-attributes" Operation attribute of the current Get-Printer-Attributes operation.  A new OPTIONAL "which-device" Operation attribute permits the requester to select a particular device for the case where an IPP Printer object is supporting more than one device.

Table of Contents

1.	Introduction	3

2.	Object Relationships	5

3.	Issues Resolved in the Mapping	8

3.1	Terminology	8

3.2	Name vs Index	8

3.3	Input objects and Media objects	8

3.4	Interpreter Object vs. Document Format	9

3.5	Scope Rules	10

3.6	MIB Object names vs IPP Attribute names	10

3.7	Mapping of SMI data types to IPP syntax types	13

4.	IPP Sub-Units	16

4.1	New Printer Attributes	16

4.2	Console Display Buffer	17

4.3	Channel	18

4.4	Cover	18

4.5	Input	19

4.6	Interpreter	20

4.7	Console Light	20

4.8	Marker	21

4.9	Marker Colorant	21

4.10	Marker Supplies	21

4.11	Media Path	22

4.12	Medium	22

4.13	Output	23

4.14	Alert	24

5.	New "Get-Sub-Units" IPP Printer Operation	25

5.1.1	Get-MIB-Objects Operation	25

5.1.1.1	Get-Sub-Units Request	25

5.1.1.2	Get-Sub-Units Response	27

6.	Conformance	28

6.1	Client Conformance Requirements	28

6.2	Server Conformance Requirements	28

6.2.1	Extensions	28

7.	Security Considerations	28

8.	References	29

9.	Copyright Notice	29

10.	Author's Address	29

# Introduction

This document introduces a new Device object into the IPP object model.  An IPP Printer object may support one or more Device objects which each represent a physical device as shown in the Internet Printing Protocol/1.0: Model and Semantics specification.  This document provides read access to such device objects by defining a mapping from table entries in the Printer MIB to new algorithmically generated attribute names.  New algorithmically generated attribute group names allow convenient read-only access to any row, any column, and any entire table in the Printer MIB, as well as the entire Printer MIB.  A general algorithmically generated attribute name provides access to any single SNMP object in any MIB.  These new attribute names and attribute group names are supplied in the "requested-attributes" Operation attribute of the current Get-Printer-Attributes operation.  A new OPTIONAL "which-device" Operation attribute permits the requester to select a particular device for the case where an IPP Printer object is supporting more than one device.

This mapping allows for Printer MIB information to be queried (no setting or modification of information) using the IPP protocol rather than SNMP.  This is not to say that the use of SNMP is invalid or improper, in fact just the opposite is true.  The implementation of SNMP based Printer MIBs is almost universal for modern, network-attached printers.  Therefore, since the information model as already been established and implemented, this document is simply a "IPP window" onto the Printer MIB.  This mapping gives yet another way to access the same information that has already been validated and deployed using SNMP and Printer MIB technologies.  It is expected that printer vendors that already support the Printer MIB and IPP/1.0 will find the implementation of the mapping defined in this document to be trivial with almost no additional overhead (i.e., no additional memory or processing resources).

The basic approach for this mapping is to define a simple, non-table driven, algorithm for converting an attribute name to an SNMP OID and for converting an attribute group name to a row, column, or table in the MIB.  Then the MIB objects can be retrieved using the Get-Printer-Attributes request by specifying the attribute name or attribute group name in the "requested-attributes" Operation attribute.  In order to be able to access each device when an IPP Printer object supports more than one device, a "which-device" Operation attribute MAY be supplied by the client.  If the IPP Printer object only supports one device, the "which-device" Operation attribute NEED NOT be supported by the IPP Printer and NEED NOT be supplied by a client in a request.

The following Printer MIB Tables are mapped to IPP object types:

| Printer MIB Table | IPP table number (t) |
| --- | --- |
| prtGeneral | 5 |
| prtCover | 6 |
| prtLocalization | 7 |
| Input | 8 |
| Output | 9 |
| Marker | 10 |
| MarkerSupplies | 11 |
| MarkerColorant | 12 |
| MediaPath | 13 |
| Channel | 14 |
| Interpreter | 15 |
| ConsoleDisplayBuffer | 16 |
| ConsoleLights | 17 |
| Alert | 18 |
| Finisher (future) | 19 |
| future Printer MIB table additions … | ?? |

In addition, references to external tables (external to the Printer MIB itself) are not mapped (such as the Host Resources MIB and the Interfaces MIB).  Instead, the Model introduces new "device-state" and "device-state-reasons" attributes which represents the state of the Device.  These states and reasons are a subset of the values of the IPP Printer "printer-state" and "printer-state-reasons" attributes.  These attributes provide a simpler state representation than the Host Resources MIB.

A new object type, not part of the existing Printer MIB or its tables is introduced.  This object type is called "medium".  This object type is introduced to support the idea of medium characteristic attributes for both "ready" and "supported" media.  The Printer MIB is only concerned with "ready" media (media loaded in one of the input trays).  The attributes of medium objects are requested using an algorithmic naming scheme that includes the medium characteristic attribute and the medium instance name.  For example, "medium-weight-iso-a4-white"

Additional objects can be added in the future, such as start-sheets, fonts, forms, etc., using a similar attribute naming scheme.

# Printer object and the Device object

This document adds the semantics of the Device object to the IPP model, including a query interface.  The Device object is represented in the IPP/1.0 Model and Semantics document, but without any defined interface.  From the Internet Printing Protocol/1.0 Model and Semantics document we have the following illustrations of the IPP Printer object and its relationship to devices:

Since a Printer object is an abstraction of a generic document output device and print service provider, a Printer object could be used to represent any real or virtual device with semantics consistent with the Printer object, such as a fax device, an imager, or even a CD writer.

Some examples of configurations supporting a Printer object include:

1) An output device, with no spooling capabilities

2) An output device, with a built-in spooler

3) A print server supporting IPP with one or more associated output devices

3a) The associated output devices might or might not be capable of spooling jobs

3b) The associated output devices might or might not support IPP

The following figures show some examples of how Printer objects can be realized on top of various distributed printing configurations.  The embedded case below represents configurations 1 and 2. The hosted and fan-out figures below represent configuration 3.

Legend:

##### indicates a Printer object which is

either embedded in an output device or is

hosted in a server.  The Printer object

might or might not be capable of queuing/spooling.

any   indicates any network protocol or direct

connect, including IPP

embedded printer:

output device

+---------------+

O   +--------+                         |  ###########  |

/|\  | client |------------IPP------------># Printer #  |

/ \  +--------+                         |  # Object  #  |

|  ###########  |

+---------------+

hosted printer:

+---------------+

O   +--------+        ###########      |               |

/|\  | client |--IPP--># Printer #-any->| output device |

/ \  +--------+        # Object  #      |               |

###########      +---------------+

+---------------+

fan out:                                 |               |

+-->| output device |

any/    |               |

O   +--------+      ###########   /     +---------------+

/|\  | client |-IPP-># Printer #--*

/ \  +--------+      # Object  #   \     +---------------+

########### any\    |               |

+-->| output device |

|               |

+---------------+

# Object Relationships

This document introduces the Device object and the Medium object.  Their object relationship to the IPP Printer object is shown below:

+-------------------+

|  Printer          |

+--+----------------+

|

| +-------------------+

+-| Device            |

| +-------------------+

+

| +-------------------+

+-| Medium            |

| +-------------------+

Figure 1 illustrates the containment relationship.  All IPP Sub-Unit objects are contained by an IPP Printer object.

ISSUE:  Why a containment relationship for the Device object?  Would association be good enough?

ISSUE:  Can a Device object be controlled by more than one IPP Printer object?  I think yes, so we need to show.

# Issues Resolved in the Mapping

Several issues must be reconciled when supporting the Printer MIB tables as IPP attributes contained by an IPP Printer object.

## Terminology

IPP Object Type (or just Object Type):  The IPP Model document uses this term to describe the attributes and operations associated with a modeled entity.  In other OO contexts, the term "class" is used.

IPP Object: The instantiation of an IPP Object Type.  Each IPP Object instance has a distinct name using the IPP 'name' attribute syntax.

ISSUE:  Are these two above terms still needed?

MIB Object:  A cell in a MIB table - maps to an attribute in the IPP model.

MIB Table:  A set of rows and columns in a MIB table.

## Input objects and Media objects

IPP uses "media-ready" and "media-supported" attributes in order to identify what media is ready and supported,  respectively.  The Printer MIB does not contain the notion of a MediaTable.  A separate descriptive object is useful for each media type when characteristics such as size, weight, color, grain, transparency, etc. need to be associated with the media.  The Printer MIB chooses to associate these characteristics with the each row in the InputTable.  The reasoning behind this decision is found in the Printer MIBs stated objective of only returning "ready" information (not "supported" type information).

To rationalize these two different models, the Medium object is introduced as a new object and an Input object already contains a named reference to a Medium object (prtInputMediaName).  The Medium object which has all the characteristics about that instances (size, color, weight, etc.).  The set of "media- ready" is the set  of Medium objects referenced by Input objects.   In addition,  the following Printer MIB objects in the Input object are duplicated in the Medium Object:

prtInputMediaWeight		"medium-weight-i"

prtInputMediaType		"medium-type-i"

prtInputMediaColor		"medium-color-i"

prtInputMediaFormParts	"medium-form-parts-i"

where i is a medium instance name, such as 'iso-a4-white', so that the full IPP attribute name is:

	"medium-weight-iso-a4-white"

Interpreter Object vs. Document Format

IPP uses a MIME media type ('mimeMediaType') to identify the "document-formats-supported" by a Printer and the format of any documents supplied to the Printer.  The Printer MIB uses rows the Interpreter Table with the following MIB Objects:

prtInterpreterLangFamily

prtInterpreterLangLevel

prtInterpreterLangVersion

If an IPP Printer object supports both the "document-format" Job Template attribute and one or more contained Interpreter objects, then the implementation for that IPP Printer object MUST correctly map between the two forms of representing the PDLs that are supported.

## Scope Rules

The existing Get-Printer-Attributes operation will be used to query ANY and/or ALL Device objects contained by an IPP Printer object.  This operation can be used to query:

1. All tables in the Printer MIB.

2. All MIB tables of a certain type (all Input or Output objects)

3. A specific row in the Printer MIB (Input object "tray 1")

4. A specific column in the Printer MIB (Input media names for all the input trays)

5. A specific cell in the Printer MIB (Input media name for tray 1)

## MIB Object names vs. IPP Attribute names

All MIB object names follow the ASN.1 naming convention of "prt" followed by the some string of multiple words or abbreviations with each word or abbreviation being all lower-case letters and except for an initial upper-case letter.   For example: "prtInputMediaType".

All IPP attribute names are special keywords (all lowercase letters and digits, starting with a letter and being separated by a ‘-').  The special characters "_" and "." can also be included.  For example: "document-format-supported", "prt-att-8-12-3", or "mib-1.3.6.1.2.1.43.8.2.1.12.1.3".

### Rules for IPP attribute names and group names from Printer MIB Object names

The following rules will apply in creating IPP attribute names and attribute group names from Printer MIB Object names:

1. The attribute name will begin with "prt-".

2. The rest of the words or abbreviations in the MIB object name (after the initial "prt") will be separated by a ‘-'

3. The remainder of the name will be decimal numbers separated by "-" to indicate table numbers, column numbers and row numbers.

For example:  prtInputMediaName for the 3rd tray becomes "prt-att-8-12-3"

For IPP attribute names from any MIB Object names:

1. The attribute name will begin with "mib-".

2. The rest of the attribute name will be the decimalized OBJECT IDENTIFIER of the Object in the MIB.

For example: "prtInputMediaName" for device 10 tray 3 becomes:

"mib-1.3.6.1.2.1.43.8.2.1.12.10.3""

### Attribute names for use in "requested-attributes" Operation attribute

The following are attribute names that can be values of the "requested-attributes" Operation attributes in a Get-Printer-Attributes request:

'mib-x' specifies a single object within any MIB, where x is the full decimalized OBJECT IDENTIFIER (OID).  For example, the attribute name 'mib-1.3.6.1.2.1.43.8.2.1.12.1.3' identifies the prtInputMediaName MIB object for the third input tray in device 1.  These attribute names are not restricted to the Printer MIB, so that any MIB, including private MIBs may be accessed.  In other words, the IPP Printer object has the attribute:

        mib-1.3.6.1.2.1.43.8.2.1.12.1.3 (name (127))

The Get-Printer-Attributes response for each requested attribute (that is supported) consists of the attribute name and the value, i.e., the attribute name: 'mib-1.3.6.1.2.1.43.8.2.1.12.1.3' and the text string value, say,  "iso-a4-white".

'prt-att-t-c-r' specifies a single object in the Printer MIB [prtmib], where t is the table number, c is the column number, and r is the row number.  For example, the attribute name 'prt-att-8-12-3' identifies the prtInputMediaName MIB object for the third input tray in the specified device.  These attribute names are restricted to the Printer MIB.  In other words, the IPP Printer object has the attribute:

        prt-att-8-12-3 (name (127))

The Get-Printer-Attributes response for each requested attribute (that is supported) consists of the attribute name and the value, i.e., the attribute name: 'prt-att-8-12-3' and the text string value, say,  "iso-a4-white".

### Attribute group names for use in "requested-attributes" Operation attribute

In addition to the above attribute names, the following attribute group names are defined to represent a group of attributes:

'prt-col-t-c' specifies a single column in the Printer MIB, where t is the table number and c is the column number.  For example, the attribute group name 'prt-col-8-12' identifies the prtInputMediaName MIB object for all input trays in the specified device.  Thus the client can request all of the values of a single input tray attribute by supplying a single column group name.

The Get-Printer-Attributes response for a requested column attribute group (that is supported) consists of the set of attribute names and their values in increasing attribute name order, i.e., the response to the 'prt-col-8-12' attribute group is:

        'prt-att-8-12-1' and the first tray's value, say, "letter-white", 
        'prt-att-8-12-2' and the second tray's value, say, "letter-transparency",
        'prt-att-8-12-3' and the third tray's value, say, "iso-a4-white"

'prt-row-t-r' specifies a single row in the Printer MIB, where t is the table number and r is the row number.  For example, the attribute group name 'prt-row-8-3' identifies the entire third row of the Input table, i.e., all the columns that are supported for the third input tray.  Thus the requester can request all of the attributes of a specified input tray by supplying a single row group name.

The Get-Printer-Attributes response for a requested row attribute group (that is supported) consists of the set of attribute names and their values in increasing attribute name order, i.e., the response to the 'prt-row-8-3' attribute group is:

        'prt-att-8-2-3' and the 3rd tray's prtInputType, say, sheetFeedManual(5), 
        'prt-att-8-3-3' and the 3rd tray's prtInputDimUnit, say, micrometers(4),
        ...
        'prt-att-8-12-3' and the 3rd tray's prtInputMediaName, say, 'iso-a4-white'
        ...

'prt-tab-t' specifies an entire table in the Printer MIB, where t is the table number.  For example, the attribute group name 'prt-tab-8' identifies the entire Input table, i.e., all the rows and columns that are supported for the Input table.  Thus the requester can request all of the attributes of all the input trays by supplying a single table group name.

The Get-Printer-Attributes response for a requested table attribute group (that is supported) consists of the set of attribute names and their values in increasing attribute name order, i.e., the response to the 'prt-tab-8' attribute group is:

        'prt-att-8-2-1' and the 1st tray's prtInputType, say, sheetFeedManual(5), 
        'prt-att-8-2-2' and the 2nd tray's prtInputType, say, sheetFeedManual(5),
        'prt-att-8-2-3' and the 3rd tray's prtInputType, say, sheetFeedManual(5),
        'prt-att-8-3-1' and the 1st tray's prtInputDimUnit, say, micrometers(4),
        'prt-att-8-3-2' ...
        ...
        'prt-att-8-12-3' and the 3rd tray's prtInputMediaName, say, 'iso-a4-white'
        ...

'prt-tab-all' specifies the entire Printer MIB.  Each attribute is returned separately in increasing attribute name order.

## Mapping of SMI data types to IPP syntax types

The following rules apply:

1. ‘Interger32' and ‘Counter32'  just map to ‘integer"

2. ‘OCTET STRING' generally maps to ‘text', however some may map to ‘name' and some may map to ‘octetString'

3.  ‘Prt<whatever>TC' maps to ‘enum'

4.  ‘presentOnOff' is just a special TC and it maps to ‘enum' as well

# New "which-device" Operation attribute for the Get-Printer-Attributes operation

The following new (post IPP/1.0) "which-device" Operation attribute is introduced for use with the existing Get-Printer-Attributes operation.:

"which-device" (name) :

The client OPTIONALLY supplies this attribute.  The Printer object MUST support this attribute, if it supports more than one device and supports the IPP attribute and attribute group names for Printer MIB access.  This attribute is useful to select which device for an IPP Printer object that supports more than on Device object instance.

If the client omits this attribute, but requests attributes from the Printer MIB and the Printer supports multiple devices, the Printer object SHALL select a particular Device in an implementation-dependent manner.

If the client supplies a value for the "which-device" Operation attribute that is not supported by the Printer, i.e., is not among the values of the Printer object's "devices-supported" attribute, the Printer object SHALL reject the operation and return the 'client-error-attributes-or-values-not-supported' status code.

## New "devices-supported" Printer attribute

The following description is for the new "devices-supported" Printer attribute:

devices-supported (1setOf name)

This OPTIONAL Printer attribute identifies the devices that this IPP Printer supports.

## New "device-state" Device attribute

Specifies the state of the Device.  Contains a subset of the enum values of the "printer-state" Printer attribute.

## New "device-state-reasons" Device attribute

Specifies the state reasons of the Device.  Contains a subset of the enum values of the "printer-state" Printer attribute.

# Conformance

TBD

## Client Conformance Requirements

TBD

## Server Conformance Requirements

TBD

### Extensions

TBD

# Security Considerations

TBD

# Appendix

This Appendix lists all of the Printer MIB objects and their data types, table and column number, and their equivalent IPP attribute name and attribute syntax.  IPP Device object types are defined in the same way as other IPP object types: for each type, all known attributes are listed and qualified.

It is OPTIONAL for an IPP Printer to support any Device object.  However, if the Printer object supports the functionality or physical characteristics corresponding the appropriate Table in the MIB, then the Printer object SHOULD implement support for that (those) corresponding attributes and attribute groups.

## The General Table (5)

prtGeneralConfigChanges         Counter32

prtGeneralCurrentLocalization   Integer32,

prtGeneralReset                 PrtGeneralResetTC,

prtGeneralCurrentOperator       OCTET STRING,

prtGeneralServicePerson         OCTET STRING,

prtInputDefaultIndex            Integer32,

prtOutputDefaultIndex           Integer32,

prtMarkerDefaultIndex           Integer32,

prtMediaPathDefaultIndex        Integer32,

prtConsoleLocalization          Integer32

prtConsoleNumberOfDisplayLines  Integer32,

prtConsoleNumberOfDisplayChars  Integer32,

prtConsoleDisable               INTEGER,

prtAuxiliarySheetStartupPage    PresentOnOff,

prtAuxiliarySheetBannerPage     PresentOnOff,

prtGeneralPrinterName           OCTET STRING,

prtGeneralSerialNumber          OCTET STRING,

prtAlertCriticalEvents          Counter32,

prtAlertAllEvents               Counter32

These map to:

prt-att-5-1(integer(0:MAX)

prt-5-2(integer(1:65535)

prt-5-3(enum)

prt-5-4(text(127))

prt-5-5(text(127))

...

Cover (6)

prtCoverDescription      OCTET STRING,

prtCoverStatus           PrtCoverStatusTC

## Localization (7)

## Console Display Buffer (16)

prtConsoleDisplayBufferText     OCTET STRING

This maps to:

prt-att-6-

## Channel(14)

prtChannelType                      PrtChannelTypeTC,

prtChannelProtocolVersion           OCTET STRING,

prtChannelCurrentJobCntlLangIndex   Integer32,

prtChannelDefaultPageDescLangIndex  Integer32,

prtChannelState                     PrtChannelStateTC,

prtChannelIfIndex                   Integer32,

prtChannelStatus                    PrtSubUnitStatusTC,

prtChannelInformation               OCTET STRING

## Input (8)

prtInputType                      PrtInputTypeTC,

prtInputDimUnit                   PrtMediaUnitTC,

prtInputMediaDimFeedDirDeclared   Integer32,

prtInputMediaDimXFeedDirDeclared  Integer32,

prtInputMediaDimFeedDirChosen     Integer32,

prtInputMediaDimXFeedDirChosen    Integer32,

prtInputCapacityUnit              PrtCapacityUnitTC,

prtInputMaxCapacity               Integer32,

prtInputCurrentLevel              Integer32,

prtInputStatus                    PrtSubUnitStatusTC,

prtInputMediaName                 OCTET STRING,

prtInputName                      OCTET STRING,

prtInputVendorName                OCTET STRING,

prtInputModel                     OCTET STRING,

prtInputVersion                   OCTET STRING,

prtInputSerialNumber              OCTET STRING,

prtInputDescription               OCTET STRING,

prtInputSecurity                  PresentOnOff,

prtInputMediaWeight               Integer32,

prtInputMediaType                 OCTET STRING,

prtInputMediaColor                OCTET STRING,

prtInputMediaFormParts            Integer32,

prtInputMediaLoadTimeout          Integer32,

prtInputNextIndex                 Integer32

Interpreter (15)

prtInterpreterLangFamily         PrtInterpreterLangFamilyTC,

prtInterpreterLangLevel             OCTET STRING,

prtInterpreterLangVersion           OCTET STRING,

prtInterpreterDescription           OCTET STRING,

prtInterpreterVersion               OCTET STRING,

prtInterpreterDefaultOrientation    PrtPrintOrientationTC,

prtInterpreterFeedAddressability    Integer32,

prtInterpreterXFeedAddressability   Integer32,

prtInterpreterDefaultCharSetIn      CodedCharSet,

prtInterpreterDefaultCharSetOut     CodedCharSet,

prtInterpreterTwoWay                PrtInterpreterTwoWayTC

## Console Lights (17)

prtConsoleOnTime                Integer32,

prtConsoleOffTime               Integer32,

prtConsoleColor                 PrtConsoleColorTC,

prtConsoleDescription           OCTET STRING

## Marker (10)

prtMarkerMarkTech               PrtMarkerMarkTechTC,

prtMarkerCounterUnit            PrtMarkerCounterUnitTC,

prtMarkerLifeCount              Counter32,

prtMarkerPowerOnCount           Counter32,

prtMarkerProcessColorants       Integer32,

prtMarkerSpotColorants          Integer32,

prtMarkerAddressabilityUnit     INTEGER,

prtMarkerAddressabilityFeedDir  Integer32,

prtMarkerAddressabilityXFeedDir Integer32,

prtMarkerNorthMargin            Integer32,

prtMarkerSouthMargin            Integer32,

prtMarkerWestMargin             Integer32,

prtMarkerEastMargin             Integer32,

prtMarkerStatus                 PrtSubUnitStatusTC

## Marker Colorant (12)

prtMarkerColorantMarkerIndex    Integer32,

prtMarkerColorantRole           PrtMarkerColorantRoleTC,

prtMarkerColorantValue          OCTET STRING,

prtMarkerColorantTonality       Integer32

## Marker Supplies (11)

prtMarkerSuppliesMarkerIndex    Integer32,

prtMarkerSuppliesColorantIndex  Integer32,

prtMarkerSuppliesClass          PrtMarkerSuppliesClassTC,

prtMarkerSuppliesType           PrtMarkerSuppliesTypeTC,

prtMarkerSuppliesDescription    OCTET STRING,

prtMarkerSuppliesSupplyUnit    PrtMarkerSuppliesSupplyUnitTC,

prtMarkerSuppliesMaxCapacity    Integer32,

prtMarkerSuppliesLevel          Integer32

## Media Path (13)

prtMediaPathMaxSpeedPrintUnit  PrtMediaPathMaxSpeedPrintUnitTC,

prtMediaPathMediaSizeUnit       PrtMediaUnitTC,

prtMediaPathMaxSpeed            Integer32,

prtMediaPathMaxMediaFeedDir     Integer32,

prtMediaPathMaxMediaXFeedDir    Integer32,

prtMediaPathMinMediaFeedDir     Integer32,

prtMediaPathMinMediaXFeedDir    Integer32,

prtMediaPathType                PrtMediaPathTypeTC,

prtMediaPathDescription         OCTET STRING,

prtMediaPathStatus              PrtSubUnitStatusTC

## Medium (needs more work)

From Mapping:

prt-sub-unit-name

prt-sub-unit-index

From DPA:

medium-associated-media

medium-assured-reproduction-area

medium-dimensions

medium-grain

medium-holes-axis-offset

medium-holes-count

medium-holes-diameter

medium-holes-locations

medium-holes-reference-edge

medium-realization

medium-sides

medium-size

medium-tooth

## Output (9)

prtOutputType                     PrtOutputTypeTC,

prtOutputCapacityUnit             PrtCapacityUnitTC,

prtOutputMaxCapacity              Integer32,

prtOutputRemainingCapacity        Integer32,

prtOutputStatus                   PrtSubUnitStatusTC,

prtOutputName                     OCTET STRING,

prtOutputVendorName               OCTET STRING,

prtOutputModel                    OCTET STRING,

prtOutputVersion                  OCTET STRING,

prtOutputSerialNumber             OCTET STRING,

prtOutputDescription              OCTET STRING,

prtOutputSecurity                 PresentOnOff,

prtOutputDimUnit                  PrtMediaUnitTC,

prtOutputMaxDimFeedDir            Integer32,

prtOutputMaxDimXFeedDir           Integer32,

prtOutputMinDimFeedDir            Integer32,

prtOutputMinDimXFeedDir           Integer32,

prtOutputStackingOrder        PrtOutputStackingOrderTC,

prtOutputPageDeliveryOrientation

PrtOutputPageDeliveryOrientationTC,

prtOutputBursting                 PresentOnOff,

prtOutputDecollating              PresentOnOff,

prtOutputPageCollated             PresentOnOff,

prtOutputOffsetStacking           PresentOnOff

## Alert (18)

prtAlertSeverityLevel       PrtAlertSeverityLevelTC,

prtAlertTrainingLevel       PrtAlertTrainingLevelTC,

prtAlertGroup               PrtAlertGroupTC,

prtAlertGroupIndex          Integer32,

prtAlertLocation            Integer32,

prtAlertCode                PrtAlertCodeTC,

prtAlertDescription         OCTET STRING,

prtAlertTime                TimeTicks

# References

[IPP-MOD]

[prtmib]

# Copyright Notice

None,

# Author's Address

Scott A. Isaacson (Editor)

Novell, Inc.

122 E 1700 S

Provo, UT   84606

Phone: 801-861-7366

Fax:   801-861-2517

e-mail: sisaacson@novell.com

Tom Hastings

Xerox Corporation

701 S. Aviation Blvd.

El Segundo, CA   90245

Phone: 310-333-6413

Fax:   310-333-5514

e-mail: hastings@cp10.es.xerox.com

Robert Herriot

Sun Microsystems Inc.

901 San Antonio.Road, MPK-17

Palo Alto, CA 94303

Phone: 650-786-8995

Fax:	 650-786-7077

e-mail: robert.herriot@eng.sun.com