Robert Herriot (editor)

Sun Microsystems

Sylvan Butler

Hewlett-Packard

Paul Moore
Microsoft.

Randy Turner
Sharp Labs

September  04, 1997

Internet Printing Protocol/1.0: Protocol Specification

draft-ietf-ipp-protocol-02.txt

Status of this Memo

This document is an Internet-Draft.  Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress".

To learn the current status of any Internet-Draft, please check the "1id-abstracts.txt" listing contained in the Internet-Drafts Shadow Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe), munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or ftp.isi.edu (US West Coast).

Abstract

This document is one of a set of documents, which together describe all aspects of a new Internet Printing Protocol (IPP).  IPP is an application level protocol that can be used for distributed printing using Internet tools and technology.  The protocol is heavily influenced by the printing model introduced in the Document Printing Application (ISO/IEC 10175 DPA) standard.  Although DPA specifies both end user and administrative features, IPP version 1.0 is focused only on end user functionality.

The full set of IPP documents includes:

Internet Printing Protocol: Requirements

Internet Printing Protocol/1.0: Model and Semantics

Internet Printing Protocol/1.0: Security

Internet Printing Protocol/1.0: Protocol Specification

Internet Printing Protocol/1.0: Directory Schema

The requirements document takes a broad look at distributed printing functionality, and it enumerates real-life scenarios that help to clarify the features that need to be included in a printing protocol for the Internet.  It identifies requirements for three types of users: end users, operators, and administrators.  The requirements document calls out a subset of end user requirements that MUST be satisfied in the first version of IPP.  Operator and administrator requirements are out of scope for v1.0. The model and semantics document describes a simplified model with abstract objects, their attributes, and their operations. The model introduces a Printer object and a Job object.  The Job object supports multiple documents per job.  The security document covers potential threats and proposed counters to those threats.  The protocol specification is formal document which incorporates the ideas in all the other documents into a concrete mapping using clearly defined data representations and transport protocol mappings that real implementers can use to develop interoperable client and server side components. Finally, the directory schema document shows a generic schema for directory service entries that represent instances of IPP Printers.

This document is the "Internet Printing Protocol/1.0: Protocol Specification" document.

Table of Contents

1. Introduction

2. Conformance Terminology

3. Encoding of  the Operation Layer

3.1 Picture of the Encoding

3.2 Syntax of Encoding

3.3 Version

3.4 Mapping of Operations

3.5 Mapping of Status-code

3.6 Tags

3.6.1 Delimiter Tags

3.6.2 Value Tags

3.7 Name-Lengths

3.8 Mapping of Attribute  Names

3.9 Value Lengths

3.10 Mapping of Attribute Values

3.11 Data

4. Encoding of Transport Layer

4.1 General Headers

4.2 Request  Headers

4.3 Response Headers

4.4 Entity  Headers

5. Security Considerations

6. References

7. Author's Address

8. Other Participants:

9. Appendix A: Protocol Examples

9.1 Print-Job Request

9.2 Print-Job Response (successful)

9.3 Print-Job Response (failure)

9.4 Print-URI Request

9.5 Create-Job Request

9.6 Get-Jobs Request

9.7 Get-Jobs Response

10. Appendix B: Mapping of Each Operation in the Encoding

# Introduction

This document contains the rules for encoding IPP operations and describes two layers: the transport layer and the operation layer.

The transport layer consists of an  HTTP/1.1 request or response. RFC 2068 [27] describes HTTP/1.1. This document specifies the HTTP headers that an IPP implementation supports.

The operation layer consists of  a message body in an HTTP request or response.  The document "Internet Printing Protocol/1.0: Model and Semantics" [21] defines the semantics of such a message body and the supported values. This document specifies the encoding of an IPP operation. The aforementioned document [21] is henceforth referred to as the “IPP model document”

# Conformance Terminology

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and  "OPTIONAL" in this document are to be interpreted as described in RFC 2119 [25].

# Encoding of  the Operation Layer

The operation layer SHALL contain a single operation request or operation response.

The encoding consists of octets as the most primitive type. There are several types built from octets, but two important  types are integers and characters, on which most  other data types are built. Every character in this encoding SHALL be a member of the UCS-2 coded character set and SHALL be encoded using UTF-8 which uses 1 to 3 octets per character. Every integer in this encoding SHALL be encoded as a signed integer using two’s-complement binary encoding with big-endian format (also known as “network order” and “most significant byte first”). The number of octets for an integer SHALL be 1, 2 or 4, depending on usage in the protocol. Such one-octet integers, henceforth called SIGNED-BYTE, are used for the version and tag fields. Such two-byte integers, henceforth called SIGNED-SHORT are used for the operation, status-code and length fields. Four byte integers, henceforth called SIGNED-INTEGER, are used for values fields.

The following two sections present the operation layer in two ways

- informally through pictures and description

- formally through Augmented Backus-Naur Form (ABNF), as specified by draft-ietf-drums-abnf-02.txt [29]

## Picture of the Encoding

The encoding for an operation request or response consists of:

-----------------------------------------------

|                    version                  |   2 bytes  - required

-----------------------------------------------

|operation (request) or status-code (response)|   2 bytes  - required

-----------------------------------------------------------

|                 attribute-tag               |   1 byte  |

-----------------------------------------------           |-0 or more

|               attribute-sequence            |   n bytes |

-----------------------------------------------------------

|                  data-tag                   |   1 byte   - required

-----------------------------------------------

|                     data                    |   q bytes  - optional

-----------------------------------------------

The attribute-tag and attribute-sequence may be omitted if the operation has no attributes or it may be replicated in special cases, such as  for an operation that contains attributes for multiple objects. The data is omitted from some operations, but the data-tag is present even when the data is omitted. Note, the attribute-tag and data-tag are called ‘delimiter-tags’.

An attributes-sequence consists of zero or more compound-attributes.

-----------------------------------------------

|              compound-attribute             |   s bytes - 0 or more

-----------------------------------------------

A compound-attribute consists an attribute with a single value followed by zero or more additional values.

Each attribute consists of:

-----------------------------------------------

|                   value-tag                 |   1 byte

-----------------------------------------------

|               name-length  (value is u)     |   2 bytes

-----------------------------------------------

|                     name                    |   u bytes

-----------------------------------------------

|              value-length  (value is v)     |   2 bytes

-----------------------------------------------

|                     value                   |   v bytes

-----------------------------------------------

An additional value consists of:

-----------------------------------------------------------

|                   value-tag                 |   1 byte  |

-----------------------------------------------           |

|            name-length  (value is 0x0000)   |   2 bytes |

-----------------------------------------------           |-0 or more

|              value-length (value is w)      |   2 bytes |

-----------------------------------------------           |

|                     value                   |   w bytes |

-----------------------------------------------------------

Note: an additional value is like an attribute whose name-length is 0.

From the standpoint of a parsing loop, the encoding consists of:

-----------------------------------------------

|                    version                  |   2 bytes  - required

-----------------------------------------------

|operation (request) or status-code (response)|   2 bytes  - required

-----------------------------------------------------------

|        tag (delimiter-tag or value-tag)     |   1 byte  |

-----------------------------------------------           |-0 or more

|           empty or rest of attribute        |   x bytes |

-----------------------------------------------------------

|                   data-tag                  |   2 bytes  - required

-----------------------------------------------

|                     data                    |   y bytes  - optional

-----------------------------------------------

The value of the tag determines whether the bytes following the tag are:

- attributes

- data

- the remainder of a single attribute where the tag specifies the type of the value.

## Syntax of Encoding

The syntax below is ABNF except ‘strings of literals’ SHALL be case sensitive. For example ‘a’ means lower case  ‘a’ and not upper case ‘A’.   In addition, SIGNED-BYTE and SIGNED-SHORT fields are represented as ‘%x’ values which show their range of values.

ipp-message = ipp-request / ipp-response

ipp-request = version operation

*(attribute-tag  attribute-sequence) data-tag data

ipp-response = version status-code 

         *(attribute-tag attribute-sequence)  data-tag  data

version = major-version minor-version

major-version = SIGNED-BYTE  ; initially %d1

minor-version = SIGNED-BYTE  ; initially %d0

operation = SIGNED-SHORT    ; mapping from model defined below

status-code = SIGNED-SHORT  ; mapping from model defined below

attribute-sequence = *compound-attribute

compound-attribute = attribute *additional-values

attribute = value-tag name-length name value-length value

additional-values = value-tag zero-name-length value-length value

name-length = SIGNED-SHORT    ; number of octets of ‘name’

name = LALPHA *( LALPHA / DIGIT / “-” / “_” / “.” )

value-length = SIGNED-SHORT  ; number of octets of ‘value’

value = OCTET-STRING

data = OCTET-STRING

zero-name-length = %x00.00 	; name-length of 0

attribute-tag	=  %x02    	; tag of 2

data-tag = %x03                	; tag of 3

value-tag = %x10-FF

SIGNED-BYTE = BYTE

SIGNED-SHORT = 2BYTE

DIGIT = %x30-39    ;  "0" to "9"

LALPHA = %x61-7A   ;  "a" to "z"

BYTE = %x00-FF

OCTET-STRING = *BYTE

The syntax allows an attribute-tag to be present when the attribute-sequence that follows is empty. The syntax is defined this way to allow for the response of Get-Jobs where no attributes are returned for some job-objects.  Although it is RECOMMENDED that the sender not send an attribute-tag if there are no attributes (except in the Get-Jobs response just mentioned), the receiver MUST be able to decode such syntax.

## Version

The version SHALL consist of a major and minor version, each of which SHALL be represented by a SIGNED-BYTE. The protocol described in this document SHALL have a major version of 1 (0x01) and a minor version of  0 (0x00).  The ABNF for these two bytes SHALL be %x01.00.

## Mapping of Operations

Operations are defined as enums in the model document. An perations enum value SHALL be encoded as a SIGNED-SHORT

## Note: the values 0x4000 to 0xFFFF are reserved for private extensions. Mapping of Status-code

Status-codes are defined as enums in the model document. A status-code enum value SHALL be encoded as a SIGNED-SHORT

If an IPP status-code is returned, then the HTTP Status-Code MUST be 200 (OK). With any other HTTP Status-Code value, the HTTP response SHALL NOT contain an IPP message-body, and thus no IPP status-code is returned.

## Tags

There are two kinds of tags:

- delimiter tags: delimit major sections of the protocol, namely attributes and data

- value tags: specify the type of each attribute value

### Delimiter Tags

The following table specifies the values for the delimiter tags:

| Tag Value (Hex) | Delimiter |
| --- | --- |
| 0x00 | reserved |
| 0x01 | reserved |
| 0x02 | attribute-tag |
| 0x03 | data-tag |
| 0x04-0x0F | reserved for future delimiters |

### Value Tags

The remaining tables show values for the value-tag, which is the first octet of  an attribute. The value-tag specifies the type of the value of the attribute. The value of the value-tag of an attribute SHALL either be a type value specified in the model document or an “out-of-band” value, such as “unsupported” or “default”. If  the value of value-tag for an attribute is not “out-of-band” and differs from the value type specified by the model document, then a server receiving such a request MAY reject it, and  a client receiving such a response MAY ignore the attribute.

The following table specifies the “out-of-band” values for the value-tag.

| Tag Value (Hex) | Meaning |
| --- | --- |
| 0x10 | unsupported |
| 0x11 | default |
| 0x12 | no-value |
| 0x13-0x1F | reserved for future “out-of-band” values. |

The “unsupported” value SHALL be used in the attribute-sequence of an error response for those attributes which the server does not support. The “default” value is reserved for future use of setting value back to their default value. The “no-value” value is used for the “no-value” value in model, e.g. when a document-attribute is returned as a set of values and an attribute has no specified value for one or more of the documents.

The following table specifies the integer values for the value-tag

| Tag Value (Hex) | Meaning |
| --- | --- |
| 0x20 | reserved |
| 0x21 | integer |
| 0x22 | boolean |
| 0x23 | enum |
| 0x24-0x2F | reserved for future integer types |

NOTE: 0x20 is reserved for “generic integer” if should ever be needed.

The following table specifies the octet-string values for the value-tag

| Tag Value (Hex) | Meaning |
| --- | --- |
| 0x30 | reserved |
| 0x31 | dateTime |
| 0x32 | resolution |
| 0x33-0x3F | reserved for future octet-string types |

The following table specifies the character-string values for the value-tag

| Tag Value (Hex) | Meaning |
| --- | --- |
| 0x40 | reserved |
| 0x41 | text |
| 0x42 | name |
| 0x43 | language |
| 0x44 | keyword |
| 0x45 | uri |
| 0x46 | uriScheme |
| 0x47-0x5F | reserved for future character string types |

NOTE: 0x40 is reserved for “generic character-string” if should ever be needed.

The values 0x60-0xFF are reserved for future types. There are no values allocated for private extensions. A new type must be registered via the type 2 process.

## Name-Lengths

The name-length field SHALL consist of a SIGNED-SHORT. This field SHALL specify the number of octets in the name field which follows the name-length field, excluding the two bytes of the name-length field.

If a name-length field has a value of zero, the following name field SHALL be empty, and the following value SHALL be treated as an additional value for the preceding attribute. Within an attribute-sequence, if two attributes have the same name, the first occurrence SHALL be ignored. The zero-length name is the only mechanism for multi-valued attributes.

## Mapping of Attribute  Names

Some attributes are encoded in a special position.  These attribute are:

- “printer-uri”: The printer-uri of each operation in the IPP model document SHALL be specified outside of  the operation layer as the request-URI on the Request-Line at the HTTP level,.

- “document-content”: The attribute named “document-content” in the IPP model document SHALL become the “data” in the operation layer.

- “status-code”: The attribute named “status-code” in the IPP model document SHALL become the “status-code” field in the operation layer response.

The remaining attributes defined in the model document represent either a specific attribute or a collection of attributes. These attributes SHALL be  encoded in the attribute-sequence according to the order specified in the model document. If an attribute in the model document represents a collection of object attributes, those attributes are encoded as a sequence of attributes in the position specified by the model document, but the attributes in this collection may be in any order.

ISSUE: coordinate this with the model document.

If a response contains unsupported attributes, these attributes SHALL be in a second attribute-sequence. Otherwise, if an operation contain attributes from exactly one object, there SHALL be exactly one attribute-sequence. If an operation contains attributes from more than one object (e.g. Get-Jobs response), the attributes from each object SHALL be in a separate attribute-sequence, such that the attributes from the ith object are in the ith attribute-sequence. See  Section 10 “Appendix B: Mapping of Each Operation in the Encoding” for table showing the application of the rules above.

## Value Lengths

Each attribute value SHALL be preceded by a SIGNED-SHORT which SHALL specify the number of octets in the value which follows this length, exclusive of the two bytes specifying the length.

For any of the types represented by binary signed integers, the sender MUST encode the value in exactly four octets..

For any of the types represented by character-strings, the sender MUST encode the value with all the characters of the string and without any padding characters.

If a value-tag contains an “out-of-band” value, such as “unsupported”, the value-length SHALL be 0 and the value empty — the value has no meaning when the value-tag has an “out-of-band” value. If a server or client receives an operation with a nonzero value-length in this case, it SHALL ignore the value field.

## Mapping of Attribute Values

The following SHALL be the mapping of attribute values to their IPP encoding in the value field. The syntax types are defined in the IPP model document.

| Syntax of Attribute Value | Encoding |
| --- | --- |
| text | an octet string where each character is a member of the UCS-2 coded character set and is encoded using UTF-8. The text is encoded in “network byte order” with the first character in the text (according to reading order) being the first character in the encoding. |
| name | same as text |
| language | same as text but with a syntax specified by RFC 1766 |
| keyword | same as text. Allowed text values are defined in the IPP model document |
| uri | same as text |
| uriScheme | same as text |
| boolean | one binary octet where 0x00 is ‘false’ and 0x01 is ‘true’ |
| integer | a SIGNED-INTEGER, defined previously as a signed integer using two’s-complement binary encoding in four octets with big-endian format (also known as “network order” and “most significant byte first”). |
| enum | same as integer. Allowed integer values are defined in the IPP model document |
| dateTime | eleven octets whose contents are defined by “DateAndTime” in RFC 1903. Although RFC 1903 also defines an eight octet format which omits the time zone, a value of this type in the IPP protocol MUST use the eleven octet format. |
| resolution | nine octets consisting of 2 SIGNED-INTEGERs followed by a SIGNED-BYTE. The values are the same as those specified in draft-ietf-printmib-mib-info-02.txt [30]. The first SIGNED-INTEGER contains the value of prtMarkerAddressabilityXFeedDir. The second SIGNED-INTEGER contains the value of prtMarkerAddressabilityFeedDir. The SIGNED-BYTE contains the value of prtMarkerAddressabilityUnit. Note: the latter value is either 3 (tenThousandsOfInches) or 4 (micrometers) and the addressability is in 10,000 units of measure. Thus the SIGNED-INTEGERS represent integral values in either dots-per-inch or dots-per-centimeter. |
| 1setOf X | encoding according to the rules for an attribute with more than more value. Each value X is encoded according to the rules for encoding its type. |
| rangeOf X | same 1setOf X where the number of values is 2. |

The type of the value in the model document determines the encoding in the value and the value of the value-tag.

## Data

The data part SHALL include any data required by the operation

# Encoding of Transport Layer

HTTP/1.1 shall be the transport layer for this protocol.

The operation layer has been designed with the assumption that the transport layer contains the following information:

- the URI of the target job or printer operation

- the total length of the data in the operation layer, either as a single length or as a sequence of chunks each with a length.

- the client’s language, the character-set and the transport encoding.

Each HTTP operation shall use the POST method where the request-URI is the object target of the operation, and where the “Content-Type” of the message-body in each request and response shall be “application/ipp”. The message-body shall contain the operation layer and shall have the syntax described in section 3.2 “Syntax of Encoding”. A client implementation SHALL adhere to the rules for a client described in RFC 2068. A server implementation SHALL adhere the rules for an origin server described in RFC 2068.In the following sections, there are a tables of all HTTP headers which describe their use in an IPP client or server.  The following is an explanation of each column in these tables.

- the “header” column contains the name of a header

- the “request/client” column indicates whether a client sends the header.

- the “request/server” column indicates whether a server supports the header when received.

- the “response/server” column indicates whether a server sends the header.

- the “response /client” column indicates whether a client supports the header when received.

- the “values and conditions” column specifies the allowed header values and the conditions for the header to be present in a request/response.

The table for “request headers” does not have columns for responses, and the table for “response headers” does not have columns for requests.

The following is an explanation of the values in the “request/client” and “response/server” columns.

- must: the client or server MUST send the header,

- must-if: the client or server MUST send the header when the condition described in the “values and conditions” column is met,

- may: the client or server MAY send the header

- not: the client or server SHOULD NOT send the header. It is not relevant to an IPP implementation.

The following is an explanation of the values in the “response/client” and “request/server” columns.

- must: the client or server MUST support the header,

- may: the client or server MAY support the header

- not: the client or server SHOULD NOT support the header. It is not relevant to an IPP implementation.

## General Headers

The following is a table for the general headers.

ISSUE: an HTTP expert should review these tables for accuracy.

| General-Header | Request |  | Response |  | Values and Conditions |
| --- | --- | --- | --- | --- | --- |
|  | Client | Server | Server | Client |  |
| Cache-Control | must | not | must | not | “no-cache” only |
| Connection | must-if | must | must-if | must | “close” only. Both client and server SHOULD keep a connection for the duration of a sequence of operations. The client and server MUST include this header for the last operation in such a sequence. |
| Date | may | may | must | may | per RFC 1123 [9] |
| Pragma` | must | not | must | not | “no-cache” only |
| Transfer-Encoding | must-if | must | must-if | must | “chunked” only . Header MUST be present if Content-Length is absent. |
| Upgrade | not | not | not | not |  |
| Via | not | not | not | not |  |

## Request  Headers

The following is a table for the request headers.

| Request-Header | Client | Server | Request Values and Conditions |
| --- | --- | --- | --- |
| Accept | may | must | “application/ipp” only. This value is the default if the client omits it |
| Accept-Charset | may | must | per IANA Character Set registry. ISSUE: is this useful for IPP? |
| Accept-Encoding | may | must | empty and per RFC 2068 [27] and IANA registry for content-codings |
| Accept-Language | may | must | see RFC 1766 [26]. A server SHOULD honor language requested by returning the values status-message, job-state-message and printer-state-reason in one of requested languages. |
| Authorization | must-if | must | per RFC 2068. A client MUST send this header when it receives a 401 “Unauthorized” response and does not receive a “Proxy-Authenticate” header. |
| From | not | not | per RFC 2068. Because RFC recommends sending this header only with the user’s approval, it is not very useful |
| Host | must | must | per RFC 2068 |
| If-Match | not | not |  |
| If-Modified-Since | not | not |  |
| If-None-Match | not | not |  |
| If-Range | not | not |  |
| If-Unmodified-Since | not | not |  |
| Max-Forwards | not | not |  |
| Proxy-Authorization | must-if | not | per RFC 2068. A client MUST send this header when it receives a 401 “Unauthorized” response and a “Proxy-Authenticate” header. |
| Range | not | not |  |
| Referer | not | not |  |
| User-Agent | not | not |  |

## Response Headers

The following is a table for the request headers.

| Response-Header | Server | Client | Response Values and Conditions |
| --- | --- | --- | --- |
| Accept-Ranges | not | not |  |
| Age | not | not |  |
| Location | must-if | may | per RFC 2068. When URI needs redirection. |
| Proxy-Authenticate | not | must | per RFC 2068 |
| Public | may | may | per RFC 2068 |
| Retry-After | may | may | per RFC 2068 |
| Server | not | not |  |
| Vary | not | not |  |
| Warning | may | may | per RFC 2068 |
| WWW-Authenticate | must-if | must | per RFC 2068. When a server needs to authenticate a client. |

## Entity  Headers

The following is a table for the entity headers.

| Entity-Header | Request |  | Response |  | Values and Conditions |
| --- | --- | --- | --- | --- | --- |
|  | Client | Server | Server | Client |  |
| Allow | not | not | not | not |  |
| Content-Base | not | not | not | not |  |
| Content-Encoding | may | must | must | must | per RFC 2068 and IANA registry for content codings. |
| Content-Language | may | must | must | must | see RFC 1766 [26]. |
| Content-Length | must-if | must | must-if | must | the length of the message-body per RFC 2068. Header MUST be present if Transfer-Encoding is absent.. |
| Content-Location | not | not | not | not |  |
| Content-MD5 | may | may | may | may | per RFC 2068 |
| Content-Range | not | not | not | not |  |
| Content-Type | must | must | must | must | “application/ipp” only |
| ETag | not | not | not | not |  |
| Expires | not | not | not | not |  |
| Last-Modified | not | not | not | not |  |

# Security Considerations

When utilizing HTTP 1.1 as a transport of IPP, the security considerations outlined in RFC 2068 apply.  Specifically, IPP servers can generate a 401 "Unauthorized" response code to request client authentication and IPP clients should correctly respond with the proper "Authorization" header.  Both Basic Authentication (RFC 2068) and Digest Authentication (RFC 2069) flavors of authentication should be supported.  The server chooses which type(s) of authentication to accept.  Digest Authentication is a more secure method, and is always preferred to Basic Authentication.

For secure communication (privacy in particular), IPP should be run using a secure communications channel.  Both Transport Layer Security - TLS (draft-ietf-tls-protocol-03) and IPSec (RFC 1825) provide necessary features for security.  It is possible to combine the two techniques, HTTP 1.1 client authentication (either basic or digest) with secure communications channel (either TLS or IPSec).  Together the two are more secure than client authentication and they perform user authentication.

Complete discussion of IPP security considerations can be found in the IPP  Security document

ISSUE: how does each security mechanism supply the job-originating-user and job-originating-host values?

# References

[1]	Smith, R., Wright, F., Hastings, T., Zilles, S., and Gyllenskog, J., "Printer MIB", RFC 1759, March 1995.

[2]	Berners-Lee, T, Fielding, R., and Nielsen, H., "Hypertext Transfer Protocol - HTTP/1.0", RFC 1945, August 1995.

[3]	Crocker, D., "Standard for the Format of ARPA Internet Text Messages", RFC 822, August 1982.

[4]	Postel, J., "Instructions to RFC Authors", RFC 1543, October 1993.

[5]	ISO/IEC 10175 Document Printing Application (DPA), June 1996.

[6]	Herriot, R. (editor), X/Open A Printing System Interoperability Specification (PSIS), August 1995.

[7]	Kirk, M. (editor), POSIX System Administration - Part 4: Printing Interfaces, POSIX 1387.4 D8, 1994.

[8]	Borenstein, N., and Freed, N., "MIME (Multi-purpose Internet Mail Extensions) Part One: Mechanism for Specifying and Describing the Format of Internet Message Bodies", RFC 1521, September, 1993.

[9]	Braden, S., "Requirements for Internet Hosts - Application and Support", RFC 1123, October, 1989,

[10]	McLaughlin, L. III, (editor), "Line Printer Daemon Protocol" RFC 1179, August 1990.

[11]	Berners-Lee, T., Masinter, L., McCahill, M. , "Uniform Resource Locators (URL)", RFC 1738, December, 1994.

[20]	Wright, F. D., “Requirements for an Internet Printing Protocol:”

[21]	Isaacson, S, deBry, R, Hasting, T, Herriot, R, Powell, P, “Internet Printing Protocol/1.0: Model and Semantics”

[22]	Internet Printing Protocol/1.0: Security

[23]	Herriot, R, Butler, S, Moore, P, Turner, R, “Internet Printing Protocol/1.0: Protocol Specification”  (This document)

[24]	Carter, K, Isaacson, S, “Internet Printing Protocol/1.0: Directory Schema”

[25]	S. Bradner, "Key words for use in RFCs to Indicate Requirement Levels", RFC 2119 , March 1997

[26]	H. Alvestrand, " Tags for the Identification of Languages", RFC 1766, March 1995.

[27]       R Fielding, et al, “Hypertext Transfer Protocol – HTTP/1.1” RFC 2068, January 1997

[28]       J. Case, et al. “Textual Conventions for Version 2 of the Simple Network Managment Protocol (SNMPv2)”, RFC 1903, January 1996.

[29]	D. Crocker et al., “Augmented BNF for Syntax Specifications: ABNF”, draft-ietf-drums-abnf-03.txt.

[30]       R. Turner, “Printer MIB”, draft-ietf-printmib-mib-info-02.txt, July 12, 1997.

# Author's Address

| Robert Herriot (editor) | Paul Moore |
| --- | --- |
| Sun Microsystems Inc. | Microsoft |
| 901 San Antonio.Road, MPK-17 | One Microsoft Way |
| Palo Alto, CA 94303 | Redmond, WA 98053 |
| Phone: 650-786-8995 | Phone: 425-936-0908 |
| Fax: 650-786-7077 | Fax: 425-93MS-FAX |
| Email: robert.herriot@eng.sun.com | Email: paulmo@microsoft.com |
| Sylvan Butler | Randy Turner |
| Hewlett-Packard | Sharp Laboratories |
| 11311 Chinden Blvd. | 5750 NW Pacific Rim Blvd |
| Boise, ID 83714 | Camas, WA 98607 |
| Phone: 208-396-6000 | Phone: 360-817-8456 |
| Fax: 208-396-3457 | Fax: : 360-817-8436 |
| Email: sbutler@boi.hp.com | Email: rturner@sharplabs.com |
| IPP Mailing List: ipp@pwg.org |  |
| IPP Mailing List Subscription: ipp-request@pwg.org |  |
| IPP Web Page: http://www.pwg.org/ipp/ |  |

# Other Participants:

| Chuck Adams - Tektronix | Harry Lewis - IBM |
| --- | --- |
| Ron Bergman - Data Products | Tony Liao - Vivid Image |
| Keith Carter - IBM | David Manchala - Xerox |
| Angelo Caruso - Xerox | Carl-Uno Manros - Xerox |
| Jeff Copeland - QMS | Jay Martin - Underscore |
| Roger Debry - IBM | Larry Masinter - Xerox |
| Lee Farrell - Canon | Bob Pentecost - Hewlett-Packard |
| Sue Gleeson - Digital | Patrick Powell - SDSU |
| Charles Gordon - Osicom | Jeff Rackowitz - Intermec |
| Brian Grimshaw - Apple | Xavier Riley - Xerox |
| Jerry Hadsell - IBM | Gary Roberts - Ricoh |
| Richard Hart - Digital | Stuart Rowley - Kyocera |
| Tom Hastings - Xerox | Richard Schneider - Epson |
| Stephen Holmstead | Shigern Ueda - Canon |
| Zhi-Hong Huang - Zenographics | Bob Von Andel - Allegro Software |
| Scott Isaacson - Novell | William Wagner - Digital Products |
| Rich Lomicka - Digital | Jasper Wong - Xionics |
| David Kellerman - Northlake Software | Don Wright - Lexmark |
| Robert Kline - TrueSpectra | Rick Yardumian - Xerox |
| Dave Kuntz - Hewlett-Packard | Lloyd Young - Lexmark |
| Takami Kurono - Brother | Peter Zehler - Xerox |
| Rich Landau - Digital | Frank Zhao - Panasonic |
| Greg LeClair - Epson | Steve Zilles - Adobe |

# Appendix A: Protocol Examples

## Print-Job Request

The following is an example of a Print-Job request with job-name, copies, and sides specified.

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0002 | PrintJob | operation |
| 0x02 | start attributes | attribute tag |
| 0x42 | name type | value-tag |
| 0x0008 |  | name-length |
| job-name | job-name | name |
| 0x0006 |  | value-length |
| foobar | foobar | value |
| 0x21 | integer type | value-tag |
| 0x0005 |  | name-length |
| copies | copies | name |
| 0x0004 |  | value-length |
| 0x00000014 | 20 | value |
| 0x44 | keyword type | value-tag |
| 0x0005 |  | name-length |
| sides | sides | name |
| 0x0013 |  | value-length |
| two-sided-long-edge | two-sided-long-edge | value |
| 0x03 | start-data | data-tag |
| %!PS... | <PostScript> | data |

## Print-Job Response (successful)

Here is an example of a Print-Job response which is successful:

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0000 | OK (successful) | status-code |
| 0x02 | start attributes | attribute tag |
| 0x41 | text type | value-tag |
| 0x000E |  | name-length |
| status-message | status-message | name |
| 0x0002 |  | value-length |
| OK | OK | value |
| 0x21 | integer | value-tag |
| 0x0007 |  | name-length |
| job-id | job-id | name |
| 0x0004 |  | value-length |
| 147 | 147 | value |
| 0x25 | name type | value-tag |
| 0x0008 |  | name-length |
| job-state | job-state | name |
| 0x0001 |  | value-length |
| 0x03 | pending | value |
| 0x03 | start-data | data-tag |

## Print-Job Response (failure)

Here is an example of a Print-Job response which fails because the printer does not support sides and because the value 20 for copies is not supported:

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0400 | client-error-bad-request | status-code |
| 0x02 | start attribures | attribute tag |
| 0x41 | text type | value-tag |
| 0x000E |  | name-length |
| status-message | status-message | name |
| 0x000D |  | value-length |
| bad-request | bad-request | value |
| 0x02 | start attributes | attribute tag |
| 0x21 | integer type | value-tag |
| 0x0005 |  | name-length |
| copies | copies | name |
| 0x0004 |  | value-length |
| 0x00000014 | 20 | value |
| 0x10 | unsupported (type) | value-tag |
| 0x0005 |  | name-length |
| sides | sides | name |
| 0x0000 |  | value-length |
| 0x03 | start-data | data-tag |

## Print-URI Request

The following is an example of Print-URI request with copies and job-name parameters.

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0003 | Print-URI | operation |
| 0x02 | start-attributes | attribute tag |
| 0x45 | uri type | value-tag |
| 0x000A |  | name-length |
| document-uri | document-uri | name |
| 0x11 |  | value-length |
| ftp://foo.com/foo | ftp://foo.com/foo | value |
| 0x42 | name type | value-tag |
| 0x0008 |  | name-length |
| job-name | job-name | name |
| 0x0006 |  | value-length |
| foobar | foobar | value |
| 0x21 | integer type | value-tag |
| 0x0005 |  | name-length |
| copies | copies | name |
| 0x0004 |  | value-length |
| 0x00000001 | 1 | value |
| 0x03 | start-data | data-tag |
| %!PS... | <PostScript> | data |

## Create-Job Request

The following is an example of Create-Job request with no parameters and no attributes

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0005 | Create-Job | operation |
| 0x03 | start-data | data-tag |

## Get-Jobs Request

The following is an example of Get-Jobs request with parameters but no attributes.

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x000A | Get-Jobs | operation |
| 0x02 | start-attributes | attribute-tag |
| 0x21 | integer type | value-tag |
| 0x0005 |  | name-length |
| limit | limit | name |
| 0x0004 |  | value-length |
| 0x00000032 | 50 | value |
| 0x44 | keyword type | value-tag |
| 0x0014 |  | name-length |
| requested-attributes | requested-attributes | name |
| 0x0006 |  | value-length |
| job-id | job-id | value |
| 0x44 | keyword type | value-tag |
| 0x0000 | additional value | name-length |
| 0x0008 |  | value-length |
| job-name | job-name | value |
| 0x03 | start-data | data-tag |

## Get-Jobs Response

The following is an of Get-Jobs response from previous request with 3 jobs. The Printer returns no information about the second job.

| Octets | Symbolic Value | Protocol field |
| --- | --- | --- |
| 0x0100 | 1.0 | version |
| 0x0000 | OK (successful) | status-code |
| 0x02 | start-attributes | attribute-tag |
| 0x41 | text type | value-tag |
| 0x000E |  | name-length |
| status-message | status-message | name |
| 0x0002 |  | value-length |
| OK | OK | value |
| 0x02 | start-attributes (1st object) | attribute-tag |
| 0x21 | integer type | value-tag |
| 0x0006 |  | name-length |
| job-id | job-id | name |
| 0x0004 |  | value-length |
| 147 | 147 | value |
| 0x42 | name type | value-tag |
| 0x0008 |  | name-length |
| job-name | job-name | name |
| 0x0003 |  | name-length |
| foo | foo | name |
| 0x02 | start-attributes (2nd object) | attribute-tag |
| 0x02 | start-attributes (3rd object) | attribute-tag |
| 0x21 | interger type | value-tag |
| 0x0006 |  | name-length |
| job-id | job-id | name |
| 0x0004 |  | value-length |
| 148 | 148 | value |
| 0x42 | name type | value-tag |
| 0x0008 |  | name-length |
| job-name | job-name | name |
| 0x0003 |  | name-length |
| bar | bar | name |
| 0x03 | start-data | data-tag |

# Appendix B: Mapping of Each Operation in the Encoding

The next three tables show the results of applying the rules above to the operations defined in the IPP model document. There is no information in these tables that cannot be derived from the rules presented in Section 3.8 “Mapping of Attribute  Names”.

The following table shows the mapping of all IPP model document request attributes to an attribute-sequence  or special position in the protocol. A request contains at most one attribute sequence.

| Operation | attribute sequence | special position |
| --- | --- | --- |
| Get-Operations |  |  |
| Print-Job | best-effort job-name job-template attributes document attributes | document-content |
| Print-URI | printer-uri best-effort job-namedocument-uri job-template attributes document attributes |  |
| Validate-Job or Create-Job | printer-uri best-effort job-name job-template attributes |  |
| Send-Document | job-id last-document document attributes | document-content |
| Send-URI | job-id last-document document-uri document attributes |  |
| Cancel-Job | job-id message |  |
| Get-Attributes (for a Printer) | printer-uri document-format requested-attributes |  |
| Get-Attributes (for a Job) | job-uri document-format requested-attributes |  |
| Get-Jobs | printer-uri limit requested-attributes |  |

The following table shows the mapping of all IPP model document response attributes to a 1st attribute-sequence, other attribute sequences or special position in the protocol.

| Operation | 1st attribute-sequence | other attribute-sequences | special position |
| --- | --- | --- | --- |
| Print-Job, Print-URI or Create-Job | status-message job-id job-status attributes |  | status-code |
| Send-Document or Send-URI | status-message job-status attributes |  | status-code |
| Validate-Job | status-message |  | status-code |
| Cancel-Job | status-message |  | status-code |
| Get-Attributes | status-message requested attributes |  | status-code |
| Get-Jobs | status-message | requested attributes (see the Note below) | status-code |

Note for Get-Jobs: there is a separate attribute-sequence containing requested-attributes for each job object in the response

The following table shows the mapping of all IPP model document error response attribute to an 1st attribute-sequence, 2nd attribute sequence or special position in the protocol. Those operations omitted don’t have special attributes for an error return.

| Operation | 1st attribute-sequence | 2nd attribute-sequence | special position |
| --- | --- | --- | --- |
| Print-Job, Print-URI, Validate-Job, Create-Job, Send-Document or Send-URI | status-message | unsupported attributes | status-code |