IPP Bake Off 3 Testing Outline

Table of Contents

1	General	2

1.1	Common restrictions for test purposes	2

1.2	Test print files	3

1.3	Result processing and documentation	3

1.3.1	Anonymity	3

1.3.2	Test data collection method	3

1.3.3	Results determination	3

1.3.4	Output	3

2	Procedure	4

2.1	IPP connectivity and basic printing	4

2.2	Version Interoperability	5

2.3	Notification Registration and Delivery Methods	5

2.4	Firewall and Proxy Interactions	5

2.5	Authentication and Security	5

3	Sample Tables and Checklists	6

3.1	IPP Components matrix	6

3.2	Simple Print Table	6

3.3	Version interoperability Table	7

3.4	Notification Table	10

3.5	Authentication & Security Table	10

3.6	Firewall Table	11

4	Issues	11

# General

## Common restrictions for test purposes

To facilitate testing and analysis of results we should conduct our tests under certain restrictions.  If time permits specific restrictions may be lifted for specific tests.

- Security. Authentication, authorization and privacy will not be used except where explicitly stated.  There will be a series of test specifically targeting interoperability of security implementations.  The core protocol testing will eliminate these variables.

- Character set.  We will use the UTF-8 character encoding of ISO646.  This will simplify the comparison of results.  The exception will be tests specifically targeted at characters sets.

- Language.  We will use en-us.  This will simplify the comparison of results.  Once again the exception will be tests that are intended to test this aspect of the protocol.

- IPP Scheme.  The URLs for printers will be ipp://* as specified in the IPP 1.1 specification.

- Http Chunking.  No IPP component being tested will chunk its requests or responses. Testing of chunking IPP requests and responses will specifically be performed.

- Set 1 operations.  Any implementations that support the set 1 extensions will use the PWG OP codes.  The PWG OP codes are from 0x0C to 0X12 inclusive.

- HTTP Version 1.1 will be used. Compatibility tests with 1.1 and 1.0 components will be performed.

- Firewalls and proxies.  .  The exception will be tests specifically targeted at firewalls and proxies.

## Test print files

Where possible a small standard file will be used.  A large test file will be used for IPP contention testing and real life printing scenario testing

## Result processing and documentation

### Anonymity

The participants of the tests will be aware of the identity of the participants.  Any posting or discussion of results will not identify the participants demonstrating the issue at hand.  The discussions will focus on issues regarding the IPP specification.  The objective is to clarify the specification’s language to resolve the issue.

### Test data collection method

Developers of IPP test tools should operate at the IPP application layer.  This will simplify the comparison of results.  The format of the traces will be test tool specific.   It is recommended that there be an indication of success or failure.  The tool should also report expected and received results.

Network analyzers are suggested to keep a detailed trace of various tests.  All participants are encouraged to bring their favorite.

### Results determination

Humans can determine the results for many of the tests.  An IPP Client submitting a job that is printed by an IPP printer would seem to indicate interoperability.  .

Some clients or tools will allow us to examine the attribute groups, tags, attributes and values.  The examination of these may be automated to some extent.

If all else fails the network analyzers will allow us to examine the bits on the wire.

### Output

#### Detailed results

The detailed results of the bake-off will be emailed directly to the participants of the test. Some of the detailed results will be made available during the bake-off.  All participants are encouraged to make their traces available to interested participants.  Implementers of IPP components involved in a test are entitled to the results of the test.

#### Summary of results

A summary of the test results will be circulated among the participants prior to posting on the IPP distribution list.  The results will be sanitized of any vendor specific information.  The object of the summary results is to give a high level overview of the bake-off results.

#### Issues

Issues will be recorded and tracked by the TES whip or designate.  The issue will be sent to the IPP Working Group for disposition.

# Procedure

After setting up and establishing basic IP connectivity, every IPP component will be assigned a designator.  The relevant information for each IPP component will be recorded in the table similar to the one in section 3.1.

Throughout the tests the term “interoperate” means that two independent IPP implementations correctly understands the feature.

## IPP connectivity and basic printing

Each IPP Client will send a simple print job to every IPP Printer.  The results will be recorded in a table similar to the one in section 3.2.This step should be performed during the initial set up day.  Any holes in the table can be filled in the following days.

Undecided on IPP coverage testing during Bake-Off

(NOTE: The remainder of the testing should be performed using a pair of independent implementations of the IPP protocol.)

Mandatory operations coverage

Check off the operations supported in the table in section

Optional operations coverage

Check off the attribute supported in the table in section

Mandatory printer attributes coverage

Check off the attribute supported in the table in section

Optional printer attribute coverage

Check off the attributes supported in the table in section

Add this type of testing if common error conditions will be tested

Printer State and Status

The objective of this step is to compare the state of printers under various conditions.  The get-printer-attributes operation will be used to obtain the printer-state and printer-state-reasons attributes. .

Out of paper

Submit a job of a known size to a printer with the input tray(s) empty.  The printer-state should be stopped and the printer-state-reasons should be 'media-empty'.

Put some paper into the input tray.  There should not be enough to complete the job.  While the printer is printing, examine the printer-state and printer-state-reasons.  The printer-state should be processing and the printer-state-reasons may contain media-low.

When the printer runs out of paper, examine the two attributes again.  The printer-state should be stopped.  The printer-state-reasons may contain media-needed

Cover open

Open the appropriate covers and doors.  The printer-state should move to ‘stopped’.  The printer-state-reasons may contain 'cover-open’ and/or 'door-open'

Tray missing

Remove an input tray and record the printer-state and printer-state-reasons.  The printer-state-reasons may contain 'input-tray-missing'.

Pausing a printer

If possible, pause the printer.  The printer-state should be stopped.  The printer-state-reasons may contain paused.

Paper jam

If possible, cause the printer to jam.  The printer-state should be stopped.  The printer-state-reasons may contain media-jam.

## Version Interoperability

Using Clients and Printers of differing versions submit simple print jobs.  Record results in table similar to the one in section 3.3.

Should other operations/scenarios be tested?

## Notification Registration and Delivery Methods

Need to fill out this section.  Register for events using supported methods.  Test job and printer related registrations.  Test some common event triggers.

## Firewall and Proxy Interactions

Need to fill out this section.    Test inbound and outbound printing.  Test inbound and outbound notification.  Is there any firewall specific test that should be run?  Test security and printing through a proxy.

## Authentication and Security

Authentication and security will be tested for interoperability.  The IPP printers and client participating will record the results in tables similar to the table in section 3.5   Authentication and security schemes include Basic, Digest, SSL3, TLS and Kerboros

# Sample Tables and Checklists

## IPP Components matrix

| ID | URL | Ethernet Address | Chunking (Send,Receive) | Security Schemes | Authentication Schemes | Notification Schemes |
| --- | --- | --- | --- | --- | --- | --- |
| Clients |  |  |  |  |  |  |
| C1 |  |  |  |  |  |  |
| C2 |  |  |  |  |  |  |
| C3 |  |  |  |  |  |  |
| Printers |  |  |  |  |  |  |
| P1 |  |  |  |  |  |  |
| P2 |  |  |  |  |  |  |
| P3 |  |  |  |  |  |  |
| Test Suites |  |  |  |  |  |  |
| TS1 |  |  |  |  |  |  |
| TS2 |  |  |  |  |  |  |
| TS3 |  |  |  |  |  |  |

## Simple Print Table

|  | Client ID |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Printer ID |  |  |  |  |  |  |  |  |

## Version interoperability Table

|  | 1.0 Client | 1.1 Client |
| --- | --- | --- |
| 1.0 Printer |  |  |
| 1.1 Printer |  |  |

X.X IPP Operation/Attribute Coverage Tables

X.X.X Mandatory Operations

| Mandatory Operations |  |
| --- | --- |
| Operation | Interoperate |
| print-job |  |
| validate-job |  |
| get-printer-attributes |  |
| get-jobs |  |
| cancel-job |  |
| get-job-attributes |  |

X.X.X Optional Operations

| Optional Operations |  |
| --- | --- |
| Operation | Interoperate |
| print-uri |  |
| create-job |  |
| pause-printer |  |
| resume-printer |  |
| purge-printer |  |
| send-document |  |
| send-uri |  |
| hold-job |  |
| release-job |  |
| restart-job |  |

X.X.X.Operational attributes

| Operational attributes |  |  |
| --- | --- | --- |
| Operational Attribute | Group | Interoperate |
| version-number | preamble |  |
| operation-id | preamble |  |
| request-id | preamble |  |
| attributes-charset | operational-attribute |  |
| attributes-natural-language | operational-attribute |  |
| printer-uri | operational-attribute |  |
| requesting-user-name | operational-attribute |  |
| status code | preamble |  |
| Status-message (Optional) | operational-attribute |  |

X.X.X Mandatory Printer Attributes

| Mandatory Printer Attributes |  |
| --- | --- |
| Attribute | Interoperate |
| printer-uri-supported |  |
| uri-security-supported |  |
| printer-name |  |
| printer-state |  |
| operations-supported |  |
| charset-configured |  |
| charset-supported |  |
| natural-language-configured |  |
| generated-natural-language-supported |  |
| printer-is-accepting-jobs |  |
| pdl-override-supported |  |
| document-format-default |  |
| document-format-supported |  |
| printer-up-time |  |

X.X.X Optional Printer Attributes

| Optional Printer Attributes |  |
| --- | --- |
| Attribute | Interoperate |
| printer-location |  |
| printer-info |  |
| printer-more-info |  |
| printer-driver-installer |  |
| printer-make-and-model |  |
| printer-more-info-manufacturer |  |
| printer-state-reasons |  |
| printer-state-message |  |
| queued-job-count |  |
| printer-message-from-operator |  |
| color-supported |  |
| reference-uri-schemes-supported |  |
| printer-current-time |  |
| multiple-operation-time-out |  |
| compression-supported |  |
| job-k-octets-supported |  |
| job-impressions-supported |  |
| job-media-sheets-supported |  |
| job-priority-default |  |
| job-priority-supported |  |
| job-hold-until-default |  |
| job-hold-until-supported |  |
| job-sheets-default |  |
| job-sheets-supported |  |
| multiple-document-handling-default |  |
| multiple-document-handling-supported |  |
| copies-default |  |
| copies-supported |  |
| finishings-default |  |
| finishings-supported |  |
| page-ranges-supported |  |
| sides-default |  |
| sides-supported |  |
| number-up-default |  |
| number-up-supported |  |
| orientation-requested-default |  |
| orientation-requested-supported |  |
| media-default |  |
| media-supported |  |
| media-ready |  |
| printer-resolution-default |  |
| printer-resolution-supported |  |
| print-quality-default |  |
| print-quality-supported |  |

X.X.X Mandatory job Attributes

| Mandatory job Attributes |  |
| --- | --- |
| Attribute | Interoperate |
| Job-uri |  |
| Job-id |  |
| Job-printer-uri |  |
| Job-name |  |
| Job-originating-user-name |  |
| Job-state |  |
| job-state-reasons |  |
| Time-at-creation |  |
| Time-at-processing |  |
| Time-at-completed |  |
| Attributes-charset |  |
| Attributes-natural-language |  |

X.X.X Optional Job Attributes

| Optional Job Attributes |  |
| --- | --- |
| Attribute | Interoperate |
| Job-more-info |  |
| Job-state-message |  |
| Job-detailed-status-message |  |
| Job-document-access-error |  |
| Number-of-documents |  |
| Output-device-assigned |  |
| Date-time-at-creation |  |
| Date-time-at-processing |  |
| Date-time-at-completed |  |
| Number-of-intervening-jobs |  |
| Job-message-from-operator |  |
| Job-k-octets |  |
| job-impressions |  |
| job-media-sheets |  |
| compression-supported |  |
| Job-k-octets-processed |  |
| job-impressions-completed |  |
| job-media-sheets-completed |  |
| job-priority |  |
| job-hold-until |  |
| job-sheets |  |
| multiple-document-handling |  |
| copies |  |
| finishings |  |
| page-ranges |  |
| sides |  |
| number-up |  |
| orientation-requested |  |
| media |  |
| printer-resolution |  |
| print-quality |  |

## Notification Table

|  | Printer ID & method |  |  |  |
| --- | --- | --- | --- | --- |
| Client ID & method | Registration |  |  |  |
|  | Delivery |  |  |  |

## Authentication & Security Table

| Security, Authentication | Client ID |  |  |  |
| --- | --- | --- | --- | --- |
| Printer ID |  |  |  |  |

## Firewall Table

|  | Inbound Print | Outbound Notify | Outbound Print | Inbound Notify |
| --- | --- | --- | --- | --- |
| Printer ID, Client ID |  |  |  |  |

# Issues

Use this area to record any issues, comments or concerns.  The items should relate to the IPP specification or interoperability testing.