INTERNET-DRAFT

<draft-ietf-ipp-output-bin-attr-00.txt>

C. Manros

Xerox Corporation

T. Hastings

Xerox Corporation

R. Bergman

Dataproducts Corp.

May 21, 1999

Internet Printing Protocol/1.0 & 1.1: "output-bin" attribute extension

Copyright (C) The Internet Society (1999). All Rights Reserved.

Status of this Memo

This document is an Internet-Draft and is in full conformance with all provisions of Section 10 of [RFC2026].  Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress".

The list of current Internet-Drafts can be accessed at http://www.ietf.org/ietf/1id-abstracts.txt

The list of Internet-Draft Shadow Directories can be accessed as http://www.ietf.org/shadow.html.

Abstract

This document defines an extension to the IPP/1.0 [RFC2566] & IPP/1.1 [ipp-mod] Model and Semantics specification for the OPTIONAL "output-bin" Job Template attribute.  This attribute allows the client to specify in which output bin a job is to be placed and to query the Printer's default and supported output bins.

TABLE OF CONTENTS

1	Add new "output-bin" Job Template attributes	3

1.1	Problem	3

1.2	Suggested solution	3

1.3	Proposed text	3

2	IANA Considerations	7

3	Internationalization Considerations	7

4	Security Considerations	7

5	Author's Addresses	7

6	Change History	8

6.1	Changes made to the December 14, 1998 version to make the May 21, 1999 version	8

7	Appendix A: Full Copyright Statement	8

# Add new "output-bin" Job Template attributes

## Problem

Many printers have multiple output bins, that the job submission protocol permits the submitter to select in which to put the entire job.

## Suggested solution

Add a single-valued "output-bin" Job Template attribute that captures existing practice.  Allow integer values, so that the number of output bins is not constrained.  Do not specify internal mechanisms, such as collators.  Do specify an externally accessible stacker, since current devices allow a user to  select a stacker.  Do not make the attribute multi-valued.  Add the corresponding Job Template Printer attributes: "output-bin-default" and "output-bin-supported".

Note:  If it is desired to allow the job submitter to select several output bin mail boxes that can be identified by number or recipient's name, propose a separate multi-valued attribute.  Since the destination may also be electronic and have a method associated with it, also allow the uri attribute syntax. Probably call this other attribute "output-destination" with an attribute syntax of (1setOf uri | name).  Or possibly the output-destination should be a parameter on the URL?  If both "output-bin" and "output-destination" are specified, the job is both printed and sent to the specified destination.  This note is provided so that the "output-bin" attribute will not suffer "scope creep" during the review and be changed into "output-destination".  Printers have been allowing something like the "output-bin" specification for many years.  Supporting something like "output-destination" is just starting to appear now.

## Proposed text

+===================+======================+======================+

| Job Attribute     |Printer: Default Value|  Printer: Supported  |

|                   |   Attribute          |   Values Attribute   |

+===================+======================+======================+

| output-bin        | output-bin-default   | output-bin-supported |

| (type2 keyword |  | (type2 keyword |     | (1setOf

(            |

|  name(MAX) |      |  name(MAX)) |        |     type2 keyword |  |

|  integer(1:MAX)   |  integer(1:MAX)      |     name(MAX) |      |

|                   |                      |     integer(1:MAX))) |

+===================+======================+======================+

output-bin (type2 keyword | name(MAX) | integer(1:MAX))

This Job Template attribute identifies the device output bin to which the job is to be delivered.  There are standard values whose attribute syntax is 'keyword', but there are no standard values whose attribute syntax is 'name' or 'integer'.  Output bins whose attribute syntax is 'name', if any, are assigned by local administrators (by means outside the scope of IPP/1.0 and IPP/1.1).  Output bins whose attribute syntax is 'integer', if any, are assigned by a printer vendor or local administrator to identify a number of similar output bins which are better differentiated by number than by one of the descriptive names defined in the following keyword list.

Each output bin may have implementation-dependent properties.  Output bins identified by 'integer' or 'name' values MAY possess any of the properties of the output bins identified by the following keywords, depending on implementation.  However, each output bin MUST be identified by only one value of any attribute syntax type.  Otherwise, clients might be mis-led as to the capabilities of the device when querying the associated Printer object's "output-bin-supported" attribute.

Note:  Output bin types, such as sorter(s) or collator(s), have not been included in the values of this attribute, since implementations that employ such internal or external bins, determine which to use by the values of other job attributes, such as "finishings", and "copies".

When validating a job in a create (or Validate-Job) operation, which subset of the output bins are allowed as a destination for a job MAY depend on the user submitting that job, the user's authentication, and possibly other job attributes, such as "finishings" and "copies".  When returning the values of the associated "output-bin-supported" attribute, the values returned MAY depend on the user issuing the Get-Printer-Attributes operation.  For example, some implementations MAY omit the 'my-mailbox' value for users who do not have a defined mailbox for this IPP Printer object, while others MAY always return 'my-mailbox' to all users even if only supported for certain users.

If this IPP Printer object is associated with multiple devices (fan-out) (see [ipp-mod] section 2.1), the value of its "output-bin-supported" attribute is the union of the values supported with duplicates removed.

Standard keyword values are:

'top':	The output-bin that, when facing the device, is best identified as the "top" bin with respect to the device.

'middle'	The output-bin

that, when facing the device, is best identified as the "middle" bin with respect to the device.

'bottom'	The output-bin that, when facing the device, is best identified as the "bottom" bin with respect to the device.

'side'	The output-bin that, when facing the device, is best identified as the "side" bin with respect to the device.

'left'	The output-bin that, when facing the device, is best identified as the "left" bin with respect to the device.

'right'	The output-bin that, when facing the device, is best identified as the "right" bin with respect to the device.

'center'	The output-bin that, when facing the device, is best identified as the "center" bin with respect to the device.

'rear':	The output-bin that, when facing the device, is best identified as the "rear" bin with respect to the device.

'face-up'	 The output-bin that is best identified as the "face-up" bin with respect to the device. The selection of this output bin does not cause output to be made face-up; rather this output bin is given this name because a sheet with printing on one-side arrives in the output bin in the face-up position.

'face-down’ The output-bin that is best identified as the "face-down" bin with respect to the device. The selection of this output bin does not cause output to be made face-down; rather this output bin is given this name because a sheet with printing on one-side arrives in the output bin in the face-down position.

'large-capacity'	The output-bin that is best identified as the "large-capacity" bin (in terms of the number of sheets) with respect to the device.

'stacker-N':		The output-bin that is best identified as the stacker with values 'stacker-1', 'stacker-2', ....  A stacker is typically used to collate sheets within a single document (not to be confused with collated copies in which document copies are collated within a job - see the description of the 'separate-documents-collated-copies' value of the "multiple-document-handling" attribute in [ipp-mod] section 4.2.4).  The correspondence between the 'stacker-N' keyword and the actual stacker in the device is implementation-dependent, as is the number of stackers.  If this group of values is supported, at least the 'stacker-1' value MUST be supported, unless the system administrator has assigned names or integer values.

For client implementations that require distinct keywords for each possible value, say, for localization purposes, it is recommended for interoperability with other vendor's Printer implementations that 'stacker-1' to 'stacker-10' keywords be represented.

'mailbox-N':	The output-bin that is best identified as a mailbox with values 'mailbox-1', 'mailbox-2', 'mailbox-3', ….  Each mailbox is typically used to collect jobs for an individual or group.  Whether the mailbox has doors and/or locks or is open, depends on implementation.  The correspondence between the 'mailbox-N' keyword and the actual output-bin in the device is implementation-dependent, as is the number of mailboxes.  A system administrator MAY be able to assign a name to each mailbox in order to make selection of a mailbox easier for the user.  If this group of values is supported, at least the 'mailbox-1' value MUST be supported, unless the system administrator has assigned names or integer values to mailboxes.

For client implementations that require distinct keywords for each possible value, say, for localization purposes, it is recommended for interoperability with other vendor's Printer implementations that 'mailbox-1' to 'mailbox-25' keywords be represented.

'my-mailbox':	The output-bin that is best identified as functioning like a private "mailbox" with respect to the device.  An output-bin functions like a private mailbox if a printer selects the actual output bin using additional implementation-dependent criteria, such as the "authenticated user" (see [ipp-mod] section 8.3) that depends on the user submitting the job.  Whether the mailbox has doors and/or locks or is open, depends on implementation, as is the number of mailboxes.

# IANA Considerations

This registration proposal will be forwarded to IANA for publication after approval by the IETF IPP WG as outlined in the procedures in RFC 2566] and [ipp-mod].

# Internationalization Considerations

Normally a client will provide localization of the keywords values of this attribute to the language of the user.  The numeric form may be simpler for a client to localize.

# Security Considerations

The 'my-mailbox' attribute requires some form of Client Authorization to be really secure.  See [ipp-mod] section 8.

# Author's Addresses

Tom Hastings

Xerox Corporation

737 Hawaii St.  ESAE 231

El Segundo, CA  90245

Phone: 310-333-6413

Fax: 310-333-5514

e-mail: hastings@cp10.es.xerox.com

Carl-Uno Manros

Xerox Corporation

737 Hawaii St.  ESAE 231

El Segundo, CA  90245

Phone: 310-333-8273

Fax: 310-333-5514

e-mail: cmanros@cp10.es.xerox.com

Ron Bergman (Editor)

Dataproducts Corp.

1757 Tapo Canyon Road

Simi Valley, CA 93063-3394

Phone: 805-578-4421

Fax:  805-578-4001

Email: rbergman@dpc.com

# Change History

## Changes made to the December 14, 1998 version to make the May 21, 1999 version

The following changes were made to the December 14, 1998 version to make the May 21, 1999 version:

- Added the 'integer' attribute syntax to reflect existing practice.

- Removed 'collator' on the grounds that the client doesn't specify such a internal mechanism.

- Recommended 10 stacker and 25 mailbox keywords for client implementations that require distinct keywords for, say, localization.

- Replaced the 'private' with 'my-mailbox' and clarified the Client Authentication required.

- Added the other sections required to be an Internet-Draft.

# Appendix A: Full Copyright Statement

Copyright (C) The Internet Society (1998,1999). All Rights Reserved

This document and translations of it may be copied and furnished to others, and derivative works that comment on or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice and this paragraph are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the Internet Society or other Internet organizations, except as needed for the purpose of developing Internet standards in which case the procedures for copyrights defined in the Internet Standards process must be followed, or as required to translate it into languages other than English.

The limited permissions granted above are perpetual and will not be revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an "AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.