Comparison of Job Monitoring MIB implementations
with the Job Monitoring MIB objects

# List of objects for the Job Monitoring MIB

## The MIB Instance Group

The JobSetGroup consists of objects that are for all Job Set instances, not just a single instance.  The jmJobSetGroup consists entirely of the jmJobSetEntry which is indexed by:

- jmJobSetIndex -  a running index of Job Set instances supported by this printer or server.

| JmJobSetGroup (M) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^31) | N/A | N/A |  |

## The General Group

The jmGeneralGroup consists of objects of a general nature that are not per-job.  The jmGeneralGroup consists entirely of the jmGeneralEntry which is indexed by:

- jmJobSetIndex -  a running index of Job Set instances supported by this printer or server.

| JmGeneralGroup (G) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^15-1) | N/A | N/A |  |
| - jmGeneralJobCompletedPolicy - the time in seconds that jobs are kept in the jmJobTable and the jmCompletedTable after processing. | Integer32(0..2^31-1) | N/A | N/A |  |
| - jmGeneralMaxNumberOfJobs - the maximum number of job; (-1) means no limit. | Integer32(0..2^31-1) | N/A | N/A |  |
| - jmGeneralCurrentNumberOfJobs - the total number of jobs currently in the Job Table (pending and completed). | Integer32(0..2^31-1) | N/A | N/A |  |
| - jmGeneralQueuingAlgorithm - the current scheduling algorithm being used or none (no queuing is possible). | JMQueuingAlgorigthm | N/A | N/A |  |

## The Queue Group

The jmQueueGroup is made up entirely of the jmQueueTable which is an ordered list of jobs that have not completed processing.  The jmQueueGroup consists of objects that are not needed after the job has completed processing.  The jmQueueGroup is conditionally mandatory and shall be implemented by a server or print that performs queuing (or spooling).  The jmQueueGroup shall not be implemented if the value of jmGeneralQueuingAlgorithm is none.  The jmQueueTable is indexed by:

- jmJobSetIndex -  a running index of Job Set instances supported by this printer or server.

- jmQueueIndex - a running index of the jobs that have not finished processing.

| JmQueueGroup (Q) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^15) | N/A | N/A |  |
| - jmQueueIndex - a running index of the jobs that have not finished processing. | Integer32(1..2^31-1) | N/A | N/A |  |
| - jmQueueIndex - the job’s identifier generated by the printer or server implementing this JM MIB | Integer32(0..2^31-1) | N/A | N/A |  |
| - jmQueueNumberOfInterveningJobs - the number of jobs in front of this job | Integer32(0..2^31-1) | N/A | N/A |  |
| - jmJobPriority - Job priority | Integer32(0..100) | N/A | N/A |  |
| - jmJobProcessAfterTime - process-after-time | GeneralizedTime | N/A | N/A |  |
| - jmJobMessageToOperator - job-message-to-operator from submitting user or device | OCTET STRING(SIZE((63)) | N/A | N/A |  |

## The Completed Group

The jmCompletedGroup consists entirely of the jmCompletedTable which is an ordered list of the job that have completed processing.  The jmCompletedTable is indexed by:

- jmJobSetIndex -  a running index of Job Set instances supported by this printer or server.

- jmCompletedIndex - a running index of the jobs that have finished processing.

| jmCompletedGroup (C) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^15-1) | N/A | N/A |  |
| - jmCompletedIndex - a running index of the jobs that have finished processing. | Integer32(1..2^31) | N/A | N/A |  |
| - jmJobIndex - the job’s identifier generated by the printer or server implementing this JM MIB | Integer32(1..) | N/A | N/A |  |

## The Job Group

The jmJobGroup consists of (1) job identification, (2) job parameters, and (3) job status and accounting objects that have a single value per job.  The jmJobGroup consists entirely of the jmJobTable which is indexed by:

- jmJobSetIndex -  an instance index to distinguish separate sets of tables when a server supports more than one printer.

- jmJobIndex -  the job identifier that was generated by the server or printer that accepted the job.

| jmJobGroup - Identification (I) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32(1..2^15-1) |  |  |  |
| - jmJobIndex - the job’s identifier generated by the server or printer implementing this JM MIB | Integer32(1..2^31-1) |  |  |  |
| - jmJobName - Job name assigned by job owner which is not necessarily unique. | OCTET STRING(SIZE(63)) | qmsPtrJobsTitle | DisplayString (SIZE (0..255)) | The information is taken from the %%Title command in the job stream. This is either put there by our driver(s) or by many applications. |
| - jmJobNameId - the job’s identifier name generated by the job submitting software using the job submission protocol. This name can be anything that helps identifer the job to the job submitter, including the name of the queue from which the job was submitted. | OCTET STRING(SIZE(63)) | see qmsPtrJobsTitle | DisplayString (SIZE (0..255)) | The information is taken from the %%Title command in the job stream. This is either put there by our driver(s) or by many applications. |
| - jmJobNumberId - the job’s identifier number generated by the job submitting software using the job submission protocol. A (-2) value shall indicate that the submitter did not supply a job identifier number. | Integer32(0..2^31-1) | qmsPtrJobsIndex | INTEGER |  |
| - jmJobTypes - Job types (print, fax, scan, etc.) - bit vector to get multiple values in a single object | JMJobType - enum encoded as bits | N/A | N/A |  |
| - jmJobOwner - Job owner (User name of the user that originally submitted print job) | OCTET STRING(SIZE(63)) | qmsPtrJobsOwner | DisplayString (SIZE (0..255)) | The information is taken from the %%For command in the job stream. This is either put there by our driver(s) or by many applications. |
| - jmJobDeviceNameRequested - Device name (Device-specific name of device) requested by the submitting user. | OCTET STRING(SIZE((63)) | N/A | N/A |  |
| - jmDeviceIndex - the host resources index of the corresponding Printer MIB that the job was submitted to or has been assigned to be printed on by the server. 0 indicates if the server has not assigned a printer to the job. | Integer32(0..2^31-1) | N/A | N/A |  |
| - jmJobSourceChannel - Source channel on which the job was submitted (index of channel row in the Printer MIB) | PrtChannelIndex | qmsPtrJobsChannel | INTEGER |  |
| - jmJobSubmissionTime - Date/Time of job submission by job owner | DateAndTime | N/A | N/A |  |
| - jmJobComment - Job comment | OCTET STRING(SIZE(63)) | qmsPtrJobsComment | DisplayString (SIZE (0..255)) |  |

| jmJobGroup - Parameters (J) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobTotalKOctets - total K octets to be processed in the job - rounded up to next higher K | Integer32(0..2^31-1) | N/A | N/A |  |

| jmJobGroup - Status and Accounting (S) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobCurrentState - Job state (pending, processing, completed, etc.) | JMJobState | qmsPtrJobsCurStatus | INTEGER |  |
| - jmJobStateReasons - Job state reasons - additional information about the job state: reasons being held, additional completed information such as successful, warnings, or errors. | OCTET STRING(SIZE(0..63)) -bit vector | N/A | N/A |  |
| - jmJobKOctetsCompleted - K Octets completed - should be rounded down to lower K until completed. | Integer32(0..2^31-1) | N/A | N/A |  |
| - jmJobStartedProcessingTime - Date/Time of day job started processing on device | DateAndTime | N/A | N/A |  |
| - jmJobCompletionTime - Date/Time of day job finished using the device | DateAndTime | N/A | N/A |  |
| - jmJobAccountName - Account Name | OCTET STRING(SIZE(63)) | N/A | N/A |  |

## The Resource Group

The jmResourceGroup consists of requested and used resources objects that can have multiple values per job.  The jmResourceGroup consists entirely of the jmResourceTable which is indexed by:

- jmJobSetIndex -  an instance index to distinguish separate sets of tables when a server supports more than one printer.

- jmJobIndex -  the job identifier that was generated by the server or printer that accepted the job.

- jmResourceIndex -  a running index of resources for each job

| jmResourceGroup (R) | DataType | Obj/attr name | Data type | Notes |
| --- | --- | --- | --- | --- |
| - jmJobSetIndex - a running index of Job Set instances supported by this printer or server. | Integer32 |  |  |  |
| - jmJobIndex - the job’s current identifier generated by the server or printer implementing this JM MIB | Integer32(0..) |  |  |  |
| - jmResourceIndex - a running index of the resources requested and/or used by the job. | Integer32 |  |  |  |
| - jmResourceType - Resources required/used (table): | JMResourceType |  |  |  |
| - documentName(3) - Document name(s) (or file-names) | OCTET STRING(63) | qmsPtrJobsTitle | DisplayString (SIZE (0..255)) |  |
| - jobCopiesRequested(4) - Number of job copies requested | Integer32(0..2^31-1) | N/A | N/A |  |
| - jobCopiesProduced(5) - Number of job copies produced | Integer32(0..2^31-1) | N/A | N/A |  |
| - documentCopiesRequested(6) - Number of document copies requested | Integer32(0..2^31-1) | N/A | N/A |  |
| - documentCopiesProduced(7) - Number of document copies produced | Integer32(0..2^31-1) | N/A | N/A |  |
| - sides(8) - Number of sides requested/used (one-sided, two-sided) | Integer32(1..2) | qmsPtrJobsPage | INTEGER |  |
| - interpreters(9) - PDLs requested/used | PrtInterpreterFamily | qmsPtrJobsEmulation | INTEGER |  |
| - physicalDevices(10) - physical devices requested/used | hrDeviceIndex |  |  |  |
| - faxPhoneNumber(10) - FAX phone number requested/used | OCTET STRING(255) |  |  |  |
| - impressionsCompleted(11) - Impressions (sides) completed | Counter32(0..2^31-1) | qmsPtrJobsPage | INTEGER |  |
| - sheetsCompleted(12) - Sheets completed for the job. | Counter32(0..2^31-1) | qmsPtrJobsSheet | INTEGER |  |
| - pagesSpooled(13) - logical pages spooled for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesInterpreted(14) - logical pages intepreted for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesSentToDevice(15) - logical pages sent to the device for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesCompleted(16) - logical pages completed for the job. | Counter32(0..2^31-1) |  |  |  |
| - pagesCompletedCurrentCopy(17) - logical pages completed on the current copy. | Integer32(0..2^31-1) |  |  |  |
| - processingTime(18) - Processing time so far | Integer32(0..2^31-1) |  |  |  |
| - processingMessage(19) - Processing Messages | OCTET STRING(63) |  |  |  |
| - jmResourceName - resource required/usage name | OCTET STRING(63) or Integer32 |  |  |  |
| - jmResourceUnits - resource required/used usage-unit | JMResourceUnits |  |  |  |
| - jmResourceAmount - resource amount requested/used; -2 - unknown | Integer32 |  |  |  |

# QMS Specific Methods of Monitoring Jobs

My understanding from the meeting was that we were to describe each of our methods (and not necessarily NMP based) of monitoring user's jobs.

I am currently working on filling in the word template posted on the ftp server, however some of the methods we use don't necessarily fit into the tables.

No doubt the interest is in what is being monitored, rather than the particular implementation.

QMS provides a couple of different methods of users monitoring the jobs that are submitted.  Our printers can spool many (20+) jobs internally to the printer.  The methods described below only apply to jobs internal to the printer.

## SNMP

Below is listed a part of the QMS vendor specific MIB.

-- The Jobs Group

-- The Current Job

qmsPtrJobsCurStatus OBJECT-TYPE

SYNTAX  INTEGER {

spooling(1),

interpreting(2),

rasterizing(4),

printing(8),

terminating(16),

waiting(32),

spooled(64),

internal(32768)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The current job entries status."

::= { qmsPtrJobs 1 }

qmsPtrJobsCurSheet OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The sheet being imaged for the current job."

::= { qmsPtrJobs 2 }

qmsPtrJobsCurPage OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The page being imaged for the current job."

::= { qmsPtrJobs 3 }

-- The Jobs Table

-- (includes all jobs currently being processed by the printer)

qmsPtrJobsTable OBJECT-TYPE

SYNTAX  SEQUENCE OF QMSPtrJobsEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"A list of the status, byte size, emulation, page number,

sheet number, channel number, comment string, job title,

and job originator string for all jobs currently being

processed by the printer."

::= { qmsPtrJobs 4 }

qmsPtrJobsEntry OBJECT-TYPE

SYNTAX  QMSPtrJobsEntry

ACCESS  not-accessible

STATUS  mandatory

DESCRIPTION

"The current job entry containing its current status, sheet

number, and page number."

INDEX   { qmsPtrJobsIndex }

::= { qmsPtrJobsTable 1 }

QMSPtrJobsEntry ::=

SEQUENCE {

qmsPtrJobsIndex

INTEGER,

qmsPtrJobsStatus

INTEGER,

qmsPtrJobsEmulation

INTEGER,

qmsPtrJobsPage

INTEGER,

qmsPtrJobsSheet

INTEGER,

qmsPtrJobsChannel

INTEGER,

qmsPtrJobsComment

DisplayString (SIZE (0..255)),

qmsPtrJobsTitle

DisplayString (SIZE (0..255)),

qmsPtrJobsOwner

DisplayString (SIZE (0..255))

}

qmsPtrJobsIndex OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A unique value for each currently active job.  Its value ranges

between 1 and the number of currently active jobs,

qmsPtrJobsNumber.  This value must remain the same between

successive printer power cycles."

::= { qmsPtrJobsEntry 1 }

qmsPtrJobsStatus OBJECT-TYPE

SYNTAX  INTEGER {

spooling(1),

interpreting(2),

rasterizing(4),

printing(8),

terminating(16),

waiting(32),

spooled(64),

internal(32768)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The job status."

::= { qmsPtrJobsEntry 2 }

qmsPtrJobsEmulation OBJECT-TYPE

SYNTAX  INTEGER {

ultrascript(1),

ibm-proprinter(2),

diablo-630(3),

ti-855(4),

hp-gl(5),

hp-pcl(6),

hex-dump(7),

ti-855-dp(8),

ti-810(9),

epson-fx(10),

dec-ansi(11),

ibm-ext-ascii(12),

ln03(13),

tektronix-4014(14),

quic-2(15),

impress(16),

code-v(17),

r1(18),

ccitt(19),

line-printer(20),

hp-pcl-5(21),

esp(201),

wild-card(65536)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The emulation selected for the processing of this job."

::= { qmsPtrJobsEntry 4 }

qmsPtrJobsPage OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of pages defined for this job.  Note: Sheets

are defined as the physical sheet of paper as opposed to

the number of logical page images appearing on the physical

sheet of paper."

::= { qmsPtrJobsEntry 5 }

qmsPtrJobsSheet OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The number of sheets defined for this job.  Note: Sheets

are defined as the physical sheet of paper as opposed to

the number of logical page images appearing on the physical

sheet of paper."

::= { qmsPtrJobsEntry 6 }

qmsPtrJobsChannel OBJECT-TYPE

SYNTAX  INTEGER {

other(1),

rs-232(12),

parallel(13),

appletalk(14),

optional-io(16)

}

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The interface over which this job was communicated to

the printer."

::= { qmsPtrJobsEntry 7 }

qmsPtrJobsComment OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A user defined text string sent with this job."

::= { qmsPtrJobsEntry 8 }

qmsPtrJobsTitle OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A user defined text string, usually the name of the job."

::= { qmsPtrJobsEntry 9 }

qmsPtrJobsOwner OBJECT-TYPE

SYNTAX  DisplayString (SIZE (0..255))

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"A user defined text string, usually the username of the

person originating the job."

::= { qmsPtrJobsEntry 10 }

-- The Job Completion Statistics

--     Accumulation of all jobs processed by the printer since the

--     last power-up.

qmsPtrJobsStatPages OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of pages printed for all jobs processed

by the printer since the last power cycle.  Note: Sheets

are defined as the physical sheet of paper as opposed to

the number of logical page images appearing on the

physical sheet of paper."

::= { qmsPtrJobs 6 }

qmsPtrJobsStatSheets OBJECT-TYPE

SYNTAX  INTEGER

ACCESS  read-only

STATUS  mandatory

DESCRIPTION

"The total number of sheets printed for all jobs processed

by the printer since the last power cycle.  Note: Sheets

are defined as the physical sheet of paper as opposed to

the number of logical page images appearing on the physical

sheet of paper."

::= { qmsPtrJobs 7 }

## REMOTE CONSOLE

I have captured below what QMS calls a remote console session, and sent several

jobs to the printer from a couple of different Ethernet protocols, as well as

from a parallel port.

The Owner and Name information is taken from the %%Title and %%For Postscript

DSC commands in the job stream.  These are described in the "PostScript Language

Reference Manual Second Edition"  in Appendix G.  If this information is omitted,

the Owner is "Unknown" and/or the Name is "Untitled".  These commands are generated by many applications and/or drivers.

[superman, 45] telnet cricket Trying 161.33.129.6 ... Connected to cricket. Escape character is '^]'. QMS CrownNet Rev E2.05 QMS 2425 Print System; Release 2.0; Firmware 9; Processor R4300; CROWN 0; QMS Console Manager Mabry-> Mabry->Idle Mabry->Starting job 00018 [Owner: Unknown Name: Untitled] Mabry->Input Idle Printing Mabry->Starting job 00019 [Owner: mabry Name: Untitled] Mabry->Input Idle Printing Mabry->Starting job 00020 [Owner: mabry Name: Untitled] Mabry->Starting job 00021 [Owner: mabry Name: Untitled] Mabry->Starting job 00022 [Owner: mabry Name: Untitled] Mabry->Input Idle Printing Mabry->Starting job 00023 [Owner: mabry Name: mickey.ps] Mabry->Input Idle Printing Mabry->Starting job 00024 [Owner: mabry Name: simpsons.ps] Mabry->Starting job 00025 [Owner: mabry Name: Untitled] Mabry->Starting job 00026 [Owner: Unknown Name: Ventura Publisher Document] Mabry->Starting job 00027 [Owner: Unknown Name: Untitled] Mabry->

A user can enter a command to view the status of the jobs spooled internally in the printer.

Note that some small history of jobs printed is listed below the dashed line.

Mabry->jobs Job Status Prio Owner Title Interface Sheet Page 00026 Printing 0001 Ventura Publisher D Parallel 0001 0002 00027 Printing 0001 Parallel 0001 0002 ----------------------------------------------------------------------------- 00021 Printed 0001 mabry Ethernet: 2 0004 0004 00022 Printed 0001 mabry Ethernet: 2 0004 0004 00023 Printed 0001 mabry mickey.ps Ethernet: 2 0001 0002 00024 Printed 0001 mabry simpsons.ps Ethernet: 2 0001 0002 00025 Printed 0001 mabry Ethernet: 2 0004 0004 End of job query Mabry-> Mabry->Starting job 00028 [Owner: mabry Name: at9.ps] Mabry->Starting job 00029 [Owner: mabry Name: at9.ps] Mabry->Starting job 00030 [Owner: mabry Name: simpsons.ps] Mabry->Starting job 00031 [Owner: mabry Name: mickey.ps] Mabry->Starting job 00032 [Owner: mabry Name: Untitled] Mabry->Starting job 00033 [Owner: mabry Name: Untitled] Mabry->

Below is the output if jobs are send from different interfaces.

Mabry->jobs Job Status Prio Owner Title Interface Sheet Page 00026 Printing 0001 Ventura Publisher D Parallel 0001 0002 00027 Printing 0001 Parallel 0001 0002 00028 Interpreting 0001 mabry at9.ps Ethernet: 2 0000 0000 00029 Spooled 0001 mabry at9.ps Ethernet: 2 0000 0000 00030 Spooled 0001 mabry simpsons.ps Ethernet: 2 0000 0000 00031 Spooled 0001 mabry mickey.ps Ethernet: 2 0000 0000 00032 Spooled 0001 mabry Ethernet: 2 0000 0000 00033 Spooled 0001 mabry Ethernet: 2 0000 0000 ----------------------------------------------------------------------------- 00021 Printed 0001 mabry Ethernet: 2 0004 0004 00022 Printed 0001 mabry Ethernet: 2 0004 0004 00023 Printed 0001 mabry mickey.ps Ethernet: 2 0001 0002 00024 Printed 0001 mabry simpsons.ps Ethernet: 2 0001 0002 00025 Printed 0001 mabry Ethernet: 2 0004 0004 End of job query Mabry->

## RFC 1179 (LPD)

A user can issue a lpq command to see what jobs are spooled internally to the printer, as well as the jobs in the UNIX queue in this case.

Example:  In this example jobs have been sent by both lpd and FTP.

The jobs numbered 12-18 are those internally queued in the printer, the jobs listed at the bottom are still queued on the UNIX host.

[superman, 50] lpq -Plpdcr Printer Status ---> IDLE Job # Title Queue User I/F Status 12 Sun Raster Dump exec2000 Ethernet:ETH.DB Interpreting 0 of 0 13 FTP Ethernet:ETH.DB Spooled 0 of 0 14 simpsons.ps exec2000 mabry Ethernet:ETH.DB Spooled 0 of 0 15 escher.pcl FTP joe Ethernet:ETH.DB Spooled 0 of 0 16 simpsons.ps exec2000 mabry Ethernet:ETH.DB Spooled 0 of 0 17 simpsons.ps exec2000 mabry Ethernet:ETH.DB Spooled 0 of 0 18 nozzle.plt FTP bill Ethernet:ETH.DB Spooled 0 of 0 superman: sending to exec2000 Rank Owner Job Files Total Size 1st mabry 582 rasfile2.ps 4415178 bytes 2nd mabry 583 rasfile2.ps 4415178 bytes [superman, 51]