7/8/96

Tom:

Looks good!  I made only some minor changes.  Talk to you tomorrow (Tuesday)

at 1:30.

Ron

Subj:  Pruned Summary of Mappings of Job Submission attributes to JMP objects/attributes

From:	Tom Hastings

Date:	06/28/96

File:	ftp://ftp.external.hp.com/snmpmib/jobs-mib/protomap/map-sum1.doc  .ps

Status: Jub submission protocol mappings (protomap.doc) with text removed.  Pruned from map-summ.doc at the 6/28/96 JMP meeting.  Reduced 69 objects to 36 objects (though Resources requested and Consumables consumed are a total of from 3 to 6 objects).

The object/attributes are divided into the following categories:

- Job Identification (I)

- Job Status and Accounting (S)

- Job Parameters (P)

In the following table, duplicates have been combined into a single object and put into the Job Status and Accounting group.  Objects marked with a leading ** are ones that were not in our original brainstorming list.  Combined objects from the original 69 are not marked with **, since they were counted in the job protocol survey.

| Job Identification (I) | ISO DPA | Apple PAP | IPDS | LPRLPD | NDPS | PJL | PSERVER | SMB | TIPSI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Job id on original client | x | x |  | x | x |  | x | x | x |
| - Job id on device (printer) | x |  | x |  | x | x | x |  | x |
| - Job id on upstream server implementing the JM MIB | x |  | x | x | x | x |  | x |  |
| - Job id on downstream server implementing the JM MIB | x |  | x | x | x | x |  | x |  |
| - Job type (print, fax, scan, etc.) - multi-valued (sub-jobs?) |  |  | x |  | x |  |  | x |  |
| - User name that originally submitting print job | x | x | x |  | x |  | x | x | x |
| - Source channel type (enum from Printer MIB) |  | x |  | x |  |  |  |  | x |
| - Source port |  | x |  | x |  |  |  |  | x |
| - Job name assigned by end-user | x |  | x |  | x | x | x |  |  |
| - Document name(s) (or file-names) | x | x | x | x | x |  | x |  | x |
| - Date/Time of job submission by end-user | x |  |  |  | x |  | x | x |  |
| - Job comment | x |  |  |  | x | x | x |  | x |
| - Device-specific name of device | x |  | x |  | x |  |  |  | x |

| Job Parameters (J) | ISO DPA | Apple PAP | IPDS | LPRLPD | NDPS | PJL | PSERVER | SMB | TIPSI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Number of copies requested | x |  |  |  | x | x | x |  |  |
| - Media path requested (one-sided, two-sided, LEF, SEF) | x |  | x |  | x | x |  |  | x |
| - PDLs requested | x |  |  | x | x | x |  |  | x |
| - Job priority | x |  |  |  | x |  |  |  | x |
| - Resources requested, including media, finishing, color ink. Presumably a type (enum) indicating the kind of resource and a name (text string) for each resource. Relates to Consumables consumed. |  |  |  |  |  |  |  |  |  |
| - Destination (including phone numbers for FAX, logical printers for printing) | x |  | x |  |  | x |  |  |  |
| - ** process-after-time | x |  |  |  |  |  |  |  |  |
| - ** job-message-to-operator from submitting user or device | x |  |  |  |  |  |  |  |  |

| Job Status and Accounting (S) | ISO DPA | Apple PAP | IPDS | LPRLPD | NDPS | PJL | PSERVER | SMB | TIPSI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Job state (held, pending, processing, printing, completed, etc.) | x | x |  | x | x | x |  | x | x |
| - Physical device(s) being used | x |  |  |  | x |  | x |  | x |
| - Job state reasons - additional information about the job state: reasons being held, additional executing information such as device(s) needs attention, additional completed information such as successful, warnings, or errors. (whether bits or text string is TBD). | x |  | x |  | x | x |  |  | x |
| - Octets completed | x |  |  |  | x |  |  |  | x |
| - Impressions (sides) completed | x |  |  |  | x | x |  |  |  |
| - Sheets completed | x |  |  |  | x |  |  |  |  |
| - Processing time completed | x |  |  |  | x |  |  |  |  |
| - Date/Time of day job started processing on device | x |  |  |  | x |  |  |  | x |
| - Date/Time of day job finished using the device |  |  |  |  |  |  |  |  |  |
| - warning count | x |  |  |  |  |  |  |  |  |
| - error-count | x |  |  |  |  |  |  |  |  |
| - ** number of pending jobs currently scheduled to process before this job | x |  |  |  |  |  |  |  |  |
| - Account Name | x |  |  |  | x |  |  |  | x |
| - Consumables consumed. Presumably a type (enum) indicating the kind of resource, a name (text string) for each resource, and an amount (and maybe an enum to indicate what units the amount is in). Relates to Resources requested. |  |  |  |  |  |  |  |  | x |
| - (See #2) | x |  |  |  |  |  |  |  | x |
| - PDLs used | x |  |  |  |  |  |  |  | x |