Media Standardized Names

Draft D0.4

March 21, 2001

ftp://ftp.pwg.org/pub/pwg/general/pwg-media-04.pdf

Abstract

This document specifies standard names to be used to indicate media types, media colors, and media sizes in other standards.  These lists of names are a superset of the names that are currently presented in the Printer MIB [RFC1759] and the IPP Model and Semantics [RFC2911] documents.  It is intended to supplement the currently defined lists as well as to provide a normative reference for all subsequent standards.

This document is a draft of an IEEE-ISTO PWG Proposed Standard and is in full conformance with all provisions of the PWG Process (see http://www.pwg.org/chair/pwg-process-990825.pdf).  PWG Proposed Standards are working documents of the IEEE-ISTO PWG and its working groups.  The list of current PWG projects and drafts can be obtained at http://www.pwg.org.

Copyright (C) 2000, IEEE Industry Standards and Technology Organization. All rights reserved.

This document may be copied and furnished to others, and derivative works that comment on, or otherwise explain it or assist in its implementation may be prepared, copied, published and distributed, in whole or in part, without restriction of any kind, provided that the above copyright notice, this paragraph and the title of the Document as referenced below are included on all such copies and derivative works.  However, this document itself may not be modified in any way, such as by removing the copyright notice or references to the IEEE-ISTO and the Printer Working Group, a program of the IEEE-ISTO.

Title:  Media Size Standardized Names

The IEEE-ISTO and the Printer Working Group DISCLAIM ANY AND ALL WARRANTIES, WHETHER EXPRESS OR IMPLIED INCLUDING (WITHOUT LIMITATION) ANY IMPLIED WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

The Printer Working Group, a program of the IEEE-ISTO, reserves the right to make changes to the document without further notice.  The document may be updated, replaced or made obsolete by other documents at any time.

The IEEE-ISTO takes no position regarding the validity or scope of any intellectual property or other rights that might be claimed to pertain to the implementation or use of the technology described in this document or the extent to which any license under such rights might or might not be available; neither does it represent that it has made any effort to identify any such rights.

The IEEE-ISTO invites any interested party to bring to its attention any copyrights, patents, or patent applications, or other proprietary rights which may cover technology that may be required to implement the contents of this document. The IEEE-ISTO and its programs shall not be responsible for identifying patents for which a license may be required by a document and/or IEEE-ISTO Industry Group Standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. Inquiries may be submitted to the IEEE-ISTO by e-mail at:

ieee-isto@ieee.org.

The Printer Working Group acknowledges that the IEEE-ISTO (acting itself or through its designees) is, and shall at all times, be the sole entity that may authorize the use of certification marks, trademarks, or other special designations to indicate compliance with these materials.

Use of this document is wholly voluntary.  The existence of this document does not imply that there are no other ways to produce, test, measure, purchase, market, or provide other goods and services related to its scope.

TABLE OF CONTENTS

1.	Introduction	5

1.1	Scope	5

2	Terminology	5

3	Media Type Names	6

4	Media Color Names	7

5	Media Size Self Describing Names	8

5.1	Media Size Self Describing Name Format	8

5.2	Custom Media Size Self Describing Name Format	10

6	Conformance Requirements	16

7	IANA Considerations	16

8	Internationalization Considerations	17

9	Security Considerations	17

10	References	17

11	Author’s Address	17

12	Appendix A: Description of the IEEE Industry Standards and Technology (ISTO)	18

13	Appendix B: Description of the IEEE-ISTO PWG	18

14	Appendix C: Change History	19

14.1	Changes to D.03, February 22, 2001, to make D.04, March 21, 2001	19

Table of Tables

Table 1 - Standardized Media Type Names	7

Table 2 - Media Color Names	8

Table 3 - North American Standard Sheet Media Sizes	11

Table 4 - North American Standard Envelope Media Sizes	12

Table 5 - ISO Standard Sheet Media Sizes	13

Table 6 - ISO Standard Envelope Media Sizes	14

Table 7 - Japanese Standard Sheet Media Sizes	14

Table 8 - Japanese Standard Envelope Media Sizes	15

Table 9 - Chinese Standard Sheet Media Sizes	15

Table 10 - Chinese Standard Envelope Media Sizes	15

Table 11 - Other Metric Standard Sheet Media Sizes	16

Table 12 - Other Metric Standard Envelope Media Sizes	16

# Introduction

Media types,  media colors, and media sizes have been defined in many previously published standards related to printing.  Examples are the ISO Document Printing Application [DPA], the IEEE Transport Independent Printer/System Interface [TIP/SI], the IETF Printer MIB [RFC1759], and the IETF Internet Printing Protocol [IPP].  Although there is a high degree of commonality in the set of media types, colors, and sizes presented in these documents, they do not represent a uniform set.  Several other standards developments, in process prior to the creation of this standard, also have a need for media type, color, and size definitions.  Also there is a large body of existing computer printing system practice around PPD and GPD files that describe a Printer’s capabilities that include media type, color, and size.  Thus this standard is a response to an urgent need to define a complete set of media types, colors, and sizes, in an independent document, that can be used a normative reference by other standards.

This standard is the result of extensive research to obtain an exhaustive list.  It provides a superset of the media types, sizes, and colors currently defined in the previously listed specifications.  This standard is intended to update the list that is currently presented in the Printer MIB and the IPP Model and Semantics specification and it also can be referenced by future printing standards.  This document will be periodically updated to include any additional types, sizes, and colors, as required.

## Scope

This document defines media types, media colors, and media sizes only.  Other media attributes such as name, weight, opacity, or coating are not included at this time, though they may be added in the future, if the need arises.

No provisions are included to specify roll paper.  All media sizes represent a cut sheet.  Media that is printed and then cut by the printing device can use this standard only to define the final size.

The color attribute that is included in a portion of the Media Name entries in both the Printer MIB and IPP are included as a separate independent set of Color Names.

The media size dimensions that are defined in this document are independent of the media feed direction (i.e. short edge feed or long edge feed) or printing orientation (i.e. portrait or landscape).  Both of these parameters are best handled by unique attributes rather than overloading the media size attribute.

# Terminology

This glossary defines certain terms used in this specification which may not be generally familiar or which may be used with very specific meaning.  These definitions are not intended to be absolute but do reflect the use of the terms within this specification.

Alias  An alternative name that is commonly used to mean the same as a name standardized in this document, but which is not defined for use that conforms to this standard.

ASCII  American Standards Code for Information Exchange as defined in ANSI X3.4-1986, “Coded Character Set - 7-bit American Standard Code for Information Interchange (ASCII).  A character set encoding with printable characters defined in the range 0x21 to 0x7E and the SPACE character (0x20).  Other encoded values MUST NOT be used.

IETF  Internet Engineering Task Force.  A volunteer group that develops and approves standards that are relative to the Internet.

ISO  International Organization for Standardization.

Legacy Name  A name used in the same contexts as the names defined in this standard, but which is deprecated from use when conforming to this standard.

media  The consumable upon which the marking engine marks so as to form a text and/or pictorial image, typically paper.

Media Color Name  The human readable name used to identify the color of the media.  Examples: ‘white’, ‘red’, ‘ivory’.

Media Dimensions  The short and long dimensions of the media.

Media Name  The human readable name used to identify media that possess the same characteristics and to distinguishes the media from others with different characteristics in the context in which the Media Name is used.  Examples:  ‘iso-a4-white’, na-letter-transparency’, ‘monarch-envelope’.  This standard does not define Media Names.

Media Size Name  The human readable name that identifies a particular media size.   Examples: ‘iso-a4’, ‘na-letter’, ‘monarch’.

Media Size Self Describing Name (or Media Size for short)  An ASCII string that contains a Media Size Name and the Media Dimensions of that correspond to its Media Size Name.  Examples: ‘iso-a4.2100-2970’, ‘na-letter.8500-11000’, ‘na-monarch.3875-7500’.

Media Type Name  The human readable name that identifies a particular medium type, i.e., the predominate characteristic of the media.  Examples:  ‘stationery’, ‘transparency’, ‘envelope’.

# Media Type Names

Table 1 defines the standardized Media Type Names.

The values and descriptions indicated with 'yes' are taken verbatim from the Printer MIB [RFC1759] and "Media Features for Display, Print, and Fax" [RFC2534] documents.  Bracketed text indicates additions to these Descriptions taken from other standards.  Additional values MAY be registered according to both [REG] and [RFC2911].

Table 1 - Standardized Media Type Names

| Keyword | Description | Printer MIB | RFC 2534 |
| --- | --- | --- | --- |
| stationery | Separately cut sheets of an opaque material | yes | yes |
| transparency | Separately cut sheets of a transparent material | yes | yes |
| envelope | Envelopes that can be used for conventional mailing purposes | yes | yes |
| envelope-plain | Envelopes that are not preprinted and have no windows | yes | yes |
| envelope-window | Envelopes that have windows for addressing purposes | yes | no |
| continuous | Continuously connected sheets of an opaque material - which edge is connected is not specified | no | yes |
| continuous-long | Continuously connected sheets of an opaque material connected along the long edge | yes | no |
| continuous-short | Continuously connected sheets of an opaque material connected along the short edge | yes | no |
| tab-stock | Media with tabs [either pre-cut or full-cut] | yes | no |
| pre-cut-tabs | Media with tabs that are cut so that more than one tab is visible extending out beyond the edge of non-tabbed media in an Output-Document. | no | no |
| full-cut-tabs | Media with a tab that runs the full length of the sheet so that only one tab is visible extending out beyond the edge of non-tabbed media in an Output-Document. | no | no |
| multi-part-form | Form medium composed of multiple layers not pre-attached to one another; each sheet may be drawn separately from an input source | yes | no |
| labels | Label stock [For example, a sheet of peel-off labels]. | yes | no |
| multi-layer | Form medium composed of multiple layers which are pre-attached to one another; e.g., for use with impact printers. | yes | no |
| screen | A refreshable display | no | yes |
| screen-paged | A refreshable display which cannot scroll | no | yes |
| photographic | Separately cut sheets of an opaque material to produce photographic quality images | no | no |
| cardstock | Separately cut sheets of a heavier or stiffer opaque material than stationery | no | no |
| other | The 'other' value is used when the media instance does not correspond to any of the other Media Type Names. | no | no |

# Media Color Names

Table 2 defines the standardized Media Color Names.

Table 2 - Media Color Names

| 'no-color' | The specified media should have no color. |
| --- | --- |
| 'white' | The specified media should be white. |
| 'pink' | The specified media should be pink. |
| 'yellow' | The specified media should be yellow. |
| 'blue' | The specified media should be blue. |
| 'green' | The specified media should be green. |
| 'buff' | The specified media should be buff. |
| 'goldenrod' | The specified media should be goldenrod. |
| 'red' | The specified media should be red. |
| 'gray' | The specified media should be gray. |
| 'ivory' | The specified media should be ivory. |
| 'orange' | The specified media should be orange. |

Note: These names are derived primarily from the Printer MIB [RFC1759] prtInputMediaColor standard values with the addition of 'blue', 'red', 'gray', 'ivory', 'orange', and 'no-color' (instead of 'transparent' - see ‘transparency’ Media Type Name in Table 1.

# Media Size Self Describing Names

The media size specifications defined in this document, labeled as Media Size Self Describing Names, are cross indexed to Legacy Names and Alias (common) names.  The Legacy Names define the names currently used in the ISO DPA, Printer MIB, or IPP documents.  A reference column is included in the tables to indicate which of these three documents contain the Legacy Name.

Ref column entry definitions:

1 =  Printer MIB and ISO DPA.   (Both documents contain an identical set.)

2 =  IPP

## Media Size Self Describing Name Format

This specification defines a new Media Size Self Describing Name format that is recommended to be used by all new implementations.  This new format has the Media Size Name and the Media Dimensions embedded within the string and allows a device to operate without a Media Size Name to Media Dimensions table.  The Media Size Self Describing Name format is structured as follows using ABNF:

media-size-self-describing-name = [prefix] size-name "." short-dim "-" long-dim

prefix = "na-"

size-name = lowalpha *( lowalpha | digit | "-" | "-" )

short-dim = *digit

long-dim = *digit

lowalpha = "a" | "b" | "c" | "d" | "e" | "f" | "g" | "h" | "i" |

"j" | "k" | "l" | "m" | "n" | "o" | "p" | "q" | "r" |

"s" | "t" | "u" | "v" | "w" | "x" | "y" | "z"

digit    = "0" | "1" | "2" | "3" | "4" | "5" | "6" | "7" | "8" | "9"

5.1.1   prefix   This string parameter is present to indicate the size dimensions are in English units.  The value of the prefix string is "na-".

The prefix string shall be included in all Media Size Self Describing Names that contain size dimensions that are to be interpreted as English units.  The prefix string must not be present if the size dimensions are in metric units.

5.1.2   size-name   This string provides a textual description of the media size.  It is normally derived from the Legacy or Alias name associated with the media size.  The size-name can consist of multiple words, with each word separated by a hyphen (0x2D).

5.1.3   short-dim and long-dim   These values define the media size.  The short-dim is always the smaller of the two dimensions.

For size dimensions measured in English units, the unit of measure is inches/1000 (.001 inches).

For size dimensions measured in Metric units, the unit of measure is millimeters/10 (.1 mm).

5.1.4   General

The Media Size Self Describing Name shall not contain any space characters (0x20).

While Media Size Self Describing Names are presented in this standard using lower case characters, other standards that use these names, MUST indicate the case sensitivity for their conformance.  Such other standards MAY:

- also require only lower case as in this standard

- allow lower, upper case, and mixed case to be used with the same meaning as the names in this standard, i.e., case insensitive matching

- require all uppercase letters to be used with the same meaning as the corresponding names in this standard.

Wherever possible, the Media Size Self Describing Name has been derived from the Legacy Name.  In many cases the 'prefix-size-name' portion is identical to the Legacy Name.  In the  remaining cases, the 'prefix' portion must be ignored to match the Legacy Name.

5.1.5   Example:  The letter size (8.5 inches by 11 inches) used in North America:

na-letter.8500-11000

5.1.6   Example:  The iso A4 size (210 mm by 297 mm) used in metric countries:

iso-a4.2100-29700

## Custom Media Size Self Describing Name Format

The Custom Media Size Self Describing Name format allows extensibility of the media size set without an update to this specification.  This feature is primarily intended for special media sizes that are used at a minimum number of locations.  The Media Size Self Describing Name format for custom sizes is structured similar to the format for the standardized sizes.

custom-media-size-self-describing-name =

[prefix] "custom" [ "-" size-name ] "." short-dim "-" long-dim

5.2.1   prefix   This string parameter must conform to all the requirements of section 3.1.1.

5.2.3   size-name   This string is optional and, if used, provides a textual description of the media size.  The size-name must conform to all the requirements of section 3.1.2.

5.2.4   short-dim and long-dim   These values must conform to all requirements of section 3.1.3.

5.2.5   Example:  A custom form measuring 6 inches by 14 inches known as "long and narrow".

na-custom-long-and-narrow.6000-14000   or   na-custom.6000-14000

5.2.6   Conventions for the Tables

The rest of this section contains the tables of Media Size Self Describing Names.  Within a table entries from different sources are grouped together.  The entries in these groups are arranged in order of increasing size of the smaller dimension.

Table 3 - North American Standard Sheet Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (inches / 1000) |
| --- | --- | --- | --- |
|  |  | index-3x5 | na-index-3x5.3000-5000 |
|  |  | index-4x6 | na-index-4x6.4000-6000 |
|  |  | a2 | na-a2.4375-5750 |
|  |  | index-5x8 | na-index-5x8.5000-8000 |
|  |  | 5x7 | na-5x7.5000-7000 |
| invoice | 2 | statement, mini | na-invoice.5500-8500 |
|  |  | index-4x6-ext | na-index-4x6-ext.6000-8000 |
|  |  | 7x9 | na-7x9.7000-9000 |
| executive | 2 |  | na-executive.7250-10500 |
|  |  | roc-16k | na-roc-16k.7750-10750 |
| na-8x10 | 2 | government-letter | na-govt-letter.8000-10000 |
| quarto | 2 |  | na-quarto.8500-10830 |
| na-letter | 1, 2 | letter, a, engineering-a | na-letter.8500-11000 |
|  |  | fanfold-European | na-fanfold-eur.8500-12000 |
|  |  | letter-plus | na-letter-plus.8500-12690 |
|  |  | foolscap | na-foolscap.8500-13000 |
| na-legal | 1, 2 | legal | na-legal.8500-14000 |
|  |  | super-a | na-super-a.8940-14000 |
|  |  | 9x11, letter-tab | na-9x11.9000-11000 |
| arch-a | 2 | architecture-a | na-arch-a.9000-12000 |
|  |  | letter-extra | na-letter-extra.9500-12000 |
|  |  | legal-extra | na-legal-extra.9500-15000 |
|  |  | 10x11 | na-10x11.10000-11000 |
|  |  | 10x13 | na-10x13.10000-13000 |
|  |  | 10x14 | na-10x14.10000-14000 |
|  |  | roc-8k | na-roc-8k.10750-15500 |
|  |  | 11x12 | na-11x12.11000-12000 |
|  |  | 11x15 | na-11x15.11000-15000 |
|  |  | edp | na-edp.11000-14000 |
|  |  | fanfold-us | na-fanfold-us.11000-14875 |
| ledger | 2 | b, engineering-b | na-ledger.11000-17000 |
|  |  | b-plus | na-b-plus.12000-19170 |
|  |  | european-edp | na-eur-edp.12000-14000 |
| arch-b | 2 | architecture-b, tabloid-extra | na-arch-b.12000-18000 |
|  |  | super-b | na-super-b.13000-19000 |
| c | 2 | engineering-c | na-c.17000-22000 |
| arch-c | 2 | architecture-c | na-arch-c.18000-24000 |
| d | 2 | engineering-d | na-d.22000-34000 |
| arch-d | 2 | architecture-d | na-arch-d.24000-36000 |
|  |  | e1 | na-e1.28000-40000 |
|  |  | wide-format | na-wide-format.30000-42000 |
| e | 2 | engineering-e | na-e.34000-44000 |
| arch-e | 2 | architecture-e | na-arch-e.36000-48000 |
|  |  | f, engineering-f | na-f.44000-68000 |

ISSUE 01:  The following Envelope Table and names are to be merged with the other tables and are to indicate only size, not type.

Table 4 - North American Standard Envelope Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (inches / 1000) |
| --- | --- | --- | --- |
|  |  | personal-envelope | na-personal-envelope.3625-6500 |
| monarch-envelope | 2 |  | na-monarch-envelope.3875-7500 |
| na-number-9-envelope | 1, 2 |  | na-9-envelope.3875-8875 |
| na-number-10-envelope | 1, 2 |  | na-10-envelope.4125-9500 |
|  |  | a2-envelope | na-a2-envelope.4375-5750 |
|  |  | number-11-envelope | na-11-envelope.4500-10375 |
|  |  | number-12-envelope | na-12-envelope.4750-11000 |
|  |  | number-14-envelope | na-14-envelope.5000-11500 |
| na-6x9-envelope | 1, 2 | 6x9-envelope | na-envelope.6000-9000 |
|  |  | c5-envelope | na-c5-envelope.6500-9500 |
| na-7x9-envelope | 1, 2 | 7x9-envelope | na-envelope.7000-9000 |
|  |  | letter-envelope | na-letter-envelope.8500-11000 |
| na-9x11-envelope | 1, 2 | 9x11-envelope | na-envelope.9000-11000 |
| na-9x12-envelope | 1, 2 | 9x12-envelope | na-envelope.9000-12000 |
| na-10x13-envelope | 1, 2 | 10x13-envelope | na-envelope.10000-13000 |
| na-10x14-envelope | 1, 2 | 10x14-envelope | na-envelope.10000-14000 |
| na-10x15-envelope | 1, 2 | 10x15-envelope | na-envelope.10000-15000 |

Table 5 - ISO Standard Sheet Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (mm / 10) |
| --- | --- | --- | --- |
| iso-a10 | 1, 2 | a10 | iso-a10.260-370 |
| iso-a9 | 1, 2 | a9 | iso-a9.370-520 |
| iso-a8 | 1, 2 | a8 | iso-a8.520-740 |
| iso-a7 | 1, 2 | a7 | iso-a7.740-1050 |
| iso-a6 | 1, 2 | a6 | iso-a6.1050-1480 |
| iso-a5 | 1, 2 | a5 | iso-a5.1480-2100 |
|  |  | a5-extra | iso-a5.1740-2350 |
| iso-a4 | 1, 2 | a4 | iso-a4.2100-2970 |
|  |  | a4-tab | iso-a4-tab.2250-2970 |
|  |  | a4-extra | iso-a4-extra.2355-3223 |
| iso-a3 | 1, 2 | a3 | iso-a3.2970-4200 |
| iso-a3-extra |  |  | iso-a3-extra.3220-4450 |
| iso-a2 | 1, 2 | a2 | iso-a2.4200-5940 |
| iso-a1 | 1, 2 | a1 | iso-a1.5940-8410 |
| iso-a0 | 1, 2 |  | iso-a0.8410-11890 |
|  |  | 2a0 | iso-2a0.11890-16820 |
|  |  | 4a0 | iso-4a0.16820-23780 |
| iso-b10 | 1, 2 | b10 | iso-b10.310-440 |
| iso-b9 | 1, 2 | b9 | iso-b9.440-620 |
| iso-b8 | 1, 2 | b8 | iso-b8.620-880 |
| iso-b7 | 1, 2 | b7 | iso-b7.880-1250 |
| iso-b6 | 1, 2 | b6 | iso-b6.1250-1760 |
| iso-b5 | 1, 2 | b5 | iso-b5.1760-2500 |
|  |  | b5-extra | iso-b5-extra.2010-2760 |
| iso-b4 | 1, 2 | b4 | iso-b4.2500-3530 |
| iso-b3 | 1, 2 | b3 | iso-b3.3530-5000 |
| iso-b2 | 1, 2 | b2 | iso-b2.5000-7070 |
| iso-b1 | 1, 2 | b1 | iso-b1.7070-10000 |
| iso-b0 | 1, 2 | b0 | iso-b0.10000-14140 |
| iso-c8 | 1 | c8 | iso-c8.570-810 |
| iso-c7 | 1 | c7 | iso-c7.810-1140 |
| iso-c6 | 1, 2 | c6 | iso-c6.1140-1620 |
| iso-c5 | 1, 2 | c5 | iso-c5.1620-2290 |
| iso-c4 | 1, 2 | c4 | iso-c4.2290-3240 |
| iso-c3 | 1, 2 | c3 | iso-c3.3240-4580 |
| iso-c2 | 1 | c2 | iso-c2.4580-6480 |
| iso-c1 | 1 | c1 | iso-c1.6480-9170 |
| iso-c0 | 1 | c0 | iso-c0.9170-12970 |
| iso-designated | 1, 2 | designated-long, dl | iso-dl.1100-2200 |
| iso-ra2 |  |  | iso-ra2.4300-6100 |
| iso-sra2 |  |  | iso-sra2.4500-6400 |
| iso-ra1 |  |  | iso-ra1.6100-8600 |
| iso-sra1 |  |  | iso-sra1.6400-9000 |
| iso-ra0 |  |  | iso-ra0.8600-12200 |
| iso-sra0 |  |  | iso-sra0.9000-12800 |

ISSUE 01:  The following Envelope Table and names are to be merged with the other tables and are to indicate only size, not type.

Table 6 - ISO Standard Envelope Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (mm / 10) |
| --- | --- | --- | --- |
|  |  | c10-envelope | iso-c10-envelope.280-400 |
|  |  | c9-envelope | iso-c9-envelope.400-570 |
|  |  | c8-envelope | iso-c8-envelope.570-810 |
|  |  | c7-envelope | iso-c7-envelope.810-1140 |
|  |  | c7/c6-envelope | iso-c7c6-envelope.810-1620 |
| iso-designated-long-envelope | 1, 2 | dl-envelope | iso-dl-envelope.1100-2200 |
|  |  | c6-envelope | iso-c6-envelope.1140-1620 |
|  |  | c6/c5-envelope | iso-c6c5-envelope.1140-2290 |
| iso-b6-envelope | 2 | b6-envelope | iso-b6-envelope.1250-1760 |
|  |  | b6/c4-envelope | iso-b6c4-envelope.1250-3240 |
| iso-c5-envelope | 1, 2 | c5-envelope | iso-c5-envelope.1620-2290 |
| iso-b5-envelope | 1, 2 | b5-envelope | iso-b5-envelope.1760-2500 |
| iso-c4-envelope | 1, 2 | c4-envelope | iso-c4-envelope.2290-3240 |
| iso-b4-envelope | 1, 2 | b4-envelope | iso-b4-envelope.2500-3530 |
| iso-c3-envelope | 2 | c3-envelope | iso-c3-envelope.3240-4580 |
|  |  | c2-envelope | iso-c2-envelope.4580-6480 |
|  |  | c1-envelope | iso-c1-envelope.6480-9170 |
|  |  | c0-envelope | iso-c0-envelope.9170-12970 |

Table 7 - Japanese Standard Sheet Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (mm / 10) |
| --- | --- | --- | --- |
| jis-b10 | 1, 2 |  | jis-b10.320-450 |
| jis-b9 | 1, 2 |  | jis-b9.450-640 |
| jis-b8 | 1, 2 |  | jis-b8.640-910 |
| jis-b7 | 1, 2 |  | jis-b7.910-1280 |
| jis-b6 | 1, 2 |  | jis-b6.1280-1820 |
| jis-b5 | 1, 2 |  | jis-b5.1820-2570 |
| jis-b4 | 1, 2 |  | jis-b4.2570-3640 |
| jis-b3 | 1, 2 |  | jis-b3.3640-5150 |
| jis-b2 | 1, 2 |  | jis-b2.5150-7280 |
| jis-b1 | 1, 2 |  | jis-b1.7280-10300 |
| jis-b0 | 1, 2 |  | jis-b0.10300-14560 |
|  |  | exec | jis-exec.2160-3300 |
|  |  | hagaki (postcard) | jpn-hagaki.1000-1480 |
|  |  | oufuku (postcard) | jpn-oufuku.1480-2000 |

ISSUE 01:  The following Envelope Table and names are to be merged with the other tables and are to indicate only size, not type.

Table 8 - Japanese Standard Envelope Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (mm / 10) |
| --- | --- | --- | --- |
|  |  | chou4-envelope | jpn-chou4-envelope.900-2050 |
|  |  | you4-envelope | jpn-you4-envelope.1050-2350 |
|  |  | envelope-2 | jpn-envelope-2.1111-1460 |
|  |  | chou3-envelope | jpn-chou3-envelope.1200-2350 |
|  |  | kaku3-envelope | jpn-kaku3-envelope.2160-2770 |
|  |  | kahu-envelope | jpn-kahu-envelope.2400-3221 |
|  |  | kaku2-envelope | jpn-kaku2-envelope.2400-3320 |

Table 9 - Chinese Standard Sheet Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (mm / 10) |
| --- | --- | --- | --- |
|  |  | prc-32k | prc-32k.970-1510 |
|  |  | prc-16k | prc-16k.1460-2150 |

ISSUE 01:  The following Envelope Table and names are to be merged with the other tables and are to indicate only size, not type.

Table 10 - Chinese Standard Envelope Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (mm / 10) |
| --- | --- | --- | --- |
|  |  | prc1-envelope | prc1-envelope.1020-1650 |
|  |  | prc2-envelope | prc2-envelope.1020-1760 |
|  |  | prc4-envelope | prc4-envelope.1100-2080 |
|  |  | prc5-envelope | prc5-envelope.1100-2200 |
|  |  | prc8-envelope | prc8-envelope.1200-3090 |
|  |  | prc6-envelope | prc6-envelope.1200-3200 |
|  |  | prc3-envelope | prc3-envelope.1250-1760 |
|  |  | prc7-envelope | prc7-envelope.1600-2300 |
|  |  | prc9-envelope | prc9-envelope.2290-3240 |
|  |  | prc10-envelope | prc10-envelope.3240-4580 |

Table 11 - Other Metric Standard Sheet Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (mm / 10) |
| --- | --- | --- | --- |
| folio | 2 |  | folio.2100-3300 |
|  |  | folio-sp | folio-sp.2150-3150 |

ISSUE 01:  The following Envelope Table and names are to be merged with the other tables and are to indicate only size, not type.

Table 12 - Other Metric Standard Envelope Media Sizes

| Legacy Name | Ref. | Alias (common name) | Self Describing Name (mm / 10) |
| --- | --- | --- | --- |
|  |  | italian-envelope | italian-envelope.1000-2300 |
|  |  | postfix-envelope | postfix-envelope.1140-2290 |
|  |  | invite-envelope | invite-envelope.2200-2200 |

# Conformance Requirements

The Media Type Names, Media Color Names, and Self Describing Media Size Names defined in this document are recommended for any future specifications that have a need for media type, media color, or media size definitions, respectively.  The proper procedure for including these names is to simply reference this specification as the definition and source of the media types, colors, or sizes, with the clause "or subsequent revisions".  In this manner, any updates to this document are automatically included in the referencing specification.

# IANA Considerations

Media size names were originally developed as a part of the ISO DPA specification and have not been registered with IANA.  The current revision of this specification, not an IANA registration, will define the official reference for media names.  IANA has a registry for Media Sizes started by the Internet FAX WG.  That registry is intended to be a small number of sizes in order to maximize interoperability for FAX.  On the other hand, the Media Size Names in this standard are intended to include as many sizes as are in common use somewhere.  Therefore, it seems more useful not to submit the Media Size Names to IANA for registration which would water down the sizes already registered for Internet FAX.

ISSUE 02:  Do we need this section ?  This appears to be an IETF remnant.  Any objections to not registering these names with IANA ?

# Internationalization Considerations

All standardized textual strings must be represented as US-ASCII character codes and local translations must never be performed.  Custom sizes, if limited to local use, may be represented using any desired character set.

ISSUE 03:  Do we need this section ?   Should UTF-8 be mentioned ?  If yes, then the ABNF needs to be enhanced for custom names.

# Security Considerations

This specification will have no impact on the security burden of or potential threats to the importing system.

# References

[DPA]

ISO/IEC 10175,  Document Printing Application,  June 1996.

[RFC1759]

Smith, R., Wright, F., Hastings, T., Zilles, S., Gyllenskog, J., “Printer MIB”, RFC 1759, March 1995.

[RFC2534]

Masinter, L., et al, “Media Features for Display, Print, and Fax”, RFC 2534, March 1999.

[RFC2911]

Hastings, T., Herriot, R., deBry, R., Isaacson, S., and P. Powell, “Internet Printing Protocol/1.1: Model and Semantics”, RFC 2911, September 2000.

[TIP/SI]

IEEE Std 1284.1-1997,  IEEE Standard for Information Technology, Transport Independent Printer/System Interface.

# Author’s Address

Ron Bergman

Hitachi Koki Imaging Solutions

1757 Tapo Canyon Road

Simi Valley, CA 93063-3394

Phone: 805 578 4421

Fax:   805 578 4005

e-mail: rbergma@hitachi-hkis.com

Tom Hastings

Xerox Corporation

737 Hawaii St.

El Segundo, CA 90245

Phone: 310 333-6413

Fax:  310 333-5514

e-mail: hastings@cp10.es.xerox.com

Additional contributors:

Harry Lewis  -  IBM Corporation

Jim Lo  -  Sun Microsystems

Roelof Hamberg  -  Oce

# Appendix A: Description of the IEEE Industry Standards and Technology (ISTO)

The IEEE-ISTO is a not-for-profit corporation offering industry groups an innovative and flexible operational forum and support services.  The IEEE-ISTO provides a forum not only to develop standards, but also to facilitate activities that support the implementation and acceptance of standards in the marketplace.  The organization is affiliated with the IEEE (http://www.ieee.org/) and the IEEE Standards Association (http://standards.ieee.org/).

For additional information regarding the IEEE-ISTO and its industry programs visit:

http://www.ieee-isto.org.

# Appendix B: Description of the IEEE-ISTO PWG

The Printer Working Group (or PWG) is a Program of the IEEE Industry Standards and Technology Organization (ISTO) with member organizations including printer manufacturers, print server developers, operating system providers, network operating systems providers, network connectivity vendors, and print management application developers.  The group is chartered to make printers and the applications and operating systems supporting them work together better.  All references to the PWG in this document implicitly mean “The Printer Working Group, a Program of the IEEE ISTO.” In order to meet this objective, the PWG will document the results of their work as open standards that define print related protocols, interfaces, procedures and conventions. Printer manufacturers and vendors of printer related software will benefit from the interoperability provided by voluntary conformance to these standards.

In general, a PWG standard is a specification that is stable, well understood, and is technically competent, has multiple, independent and interoperable implementations with substantial operational experience, and enjoys significant public support.

For additional information regarding the Printer Working Group visit:

http://www.pwg.org

# Appendix C: Change History

## Changes to D.03, February 22, 2001, to make D.04, March 21, 2001

The following changes were made:

- Added more Terminology

- Added Media Type Names

- Added Media Color Names

- Used ABNF to define the syntax for Media Size Self Describing Names