Syntax Rules and Guidelines for Group and Property Definitions

v04 RBL 20070726

First draft of a collection of rules, guidelines, and conventions to be used when defining groups, tables, and properties to be used for projector and display management.

# Scope

All property definitions in the Projector & Display Management Working Group.

# Terminology

For definitions of normative terms such as SHALL, SHOULD, MAY, and so forth, see RFC2119.

# Rules and Guidelines

## General

### Character Sets for Names

Be very careful with capitalization.  XML is very case-sensitive.

Names of groups, tables, properties, and enumerated values SHALL not contain blanks or any punctuation characters.  They SHALL contain only the characters A-Z, a-z, and 0-9.

### Normative Wording

Use SHALL, SHOULD, MAY, REQUIRED, RECOMMENDED, OPTIONAL, with the meanings specified in RFC2119.

## Group

A group MAY contain zero or more (scalar) properties that are not contained in tables.

A group MAY contain zero or one table (vector) of properties.  If a logical group needs more than one table, then break it into multiple groups, one group per table.

### Name

The name of a group SHALL not contain the word Group.  Similarly, the filename of the file containing the group definition SHOULD not contain the word Group.

The name of a group SHOULD be singular, not plural.  Examples: Fan not Fans, Interlock not Interlocks, Alert not Alerts, Capability not Capabilites, Setting not Settings.

### Contents

## Table

A table is a list of rows, where each row represents an instance of an object, such as an input connector or video mode.  Each row contains a set of properties.  The table MAY contain one or more properties.

Properties MAY also appear as scalars outside the context of a table.

### Name

The name of a table SHALL not contain the word Table.

The name of a table SHOULD be singular, not plural.

### Repetition

Tables are almost always variable length, min=0 and max=unbounded.  It is possible that some table might be fixed length, but we haven't seen one yet.

## Property

### Name

AbstractName SHOULD be something that can be turned into an SNMP or web service name with little effort.

- AbstractName SHALL begin with a capital letter.

- AbstractName SHOULD be intercapped (also called CamelCase) LikeThis if it contains more than one word.

- AbstractName SHALL contain no blanks, underscores, or hyphens.

SNMP requires that property names begin with lower case.  However, we will automatically add a prefix, such as "pdm" or similar, in front of all names, so that the lower case requirement will be met.  Therefore, AbstractName starts with a capital letter, A-Z.

Property names SHALL not contain the name of the group or table.  Example: just Description, not InterlockDescription.

### Definition

Please provide meaningful descriptions.  A description that simply repeats the name of the property is not informative.

Give a detailed definition of the property.  The definition MAY go on for many lines as needed to give the user a clear understanding of the semantic content of the property.

## Datatype

### Integer

Specify Range on integers.

### Integer Constant

### String

Specify Length on strings, min and max.

### StringConstant

### Enumerated Integer

Very common.  Please pay careful attention to the string values.

### Enumerated Values

- String values SHALL begin with a lower case letter;

- String values SHALL contain only letters and numbers, no blanks, underscores, or hyphens;

- String values SHOULD be intercapped (also called camelCase) likeThis if they contain more than one word.  The use of caps within the string enhances readability.

- Never use zero as an enum value.  If at all possible, 
1 = other
2 = unknown
if these values make sense in context.

### Enumerated String

### Counter

Counter is an integer that starts at zero and increases, never decreases.

A counter MAY be reset to zero by an operation, but it is not writable directly by the user.

If a counter is to be resettable, add a Reset property.  See the ResettableCounterExample XML file.

Counter SHALL be mutable.

Counter SHOULD be persistent.  Most interesting counters are persistent across power cycles.

Most interesting counters are available OffReadOnly so they can be examined offline.

Counter SHALL not specify a range.

Counter SHALL not specify a unit.  If it has a non-obvious unit, such as one that might differ between vendors, then add another property to describe that unit.

### Gauge

Gauge is a non-negative, mutable integer that MAY increase and decrease.  These may be useful for sizes, speeds, etc.

### Time

<Working on a proposal for time formats.>

## Property Modifiers

### Definition

### Default Value

Most properties require a default value.

- Exception: counters SHALL not have a default value.

If there is no sensible default value for a property, then usually "" (empty string) or 0 (zero) will suffice.

- Exception: if the property is mandatory, we have to do something about this.

- <Waiting for a proposal.>

- If the property is of type enumerated integer, then the default value SHALL be the string version of the value, not the integer version.

### Length

Specify the minimum length of the string.  The minimum SHALL be zero if the string is allowed to be empty.

LengthMax SHALL be greater than or equal to LengthMin.

If the string is fixed length, LengthMax SHALL = LengthMin.

### Range

### Unit

Unit of measurement, if applicable, e.g., centimeters, cubic feet per minute, hours.

- All measurements are integers; floating point representation is not available in management information.  Therefore, the unit of a measurement may need to be very small so that an integer representation is sufficiently accurate.  Examples: millimeters or micrometers rather than meters, microinches rather than inches, minutes rather than hours.

- Note that in many cases, if the unit of measurement is not absolutely fixed for all vendors, then we may need an additional data item to specify what the unit is.  Example: if we wish to permit vendors to choose between inches and meters for some measurement, then add a property, type enumerated integer, that specifies the interpretation of the numeric measurement.

- Some English/metric conversions will take place in the user interface according to the preferences of the local user.  It may not always be necessary to offer both representations in the management data.

### Localizable

If a string is vendor-supplied, then it SHOULD be localized.  Therefore it is mutable.

### Optionality

Description and Status SHOULD be mandatory unless there is a compelling reason not to.

### User Writable

Properties MAY be specified as user-writable.  This characteristic is a permission, not a requirement on implementations.  The SNMP terminology for this type of property is MAX-ACCESS.  In general, it is NOT REQUIRED that agents implement such properties as writable; it is conformant to implement such properties as read-only.

### Mutable

This characteristic is intended as a warning to applications that a property value should not be cached.

### Persistent

### Availability

The minimum availability of a data item directly affects the interoperability of the data between applications and target devices.

- Anything that we want to access when the device is in standby/powersave/sleep mode, SHALL be available at least OffReadOnly.

- If the property is a setting that we want to be able to change when the device is in standby, then it SHALL be available. OffReadWrite.

- On the other hand, some properties make sense only when the device is actually powered up, such as FanSpeed, then the availability SHALL be On.

Remember that the requirement here is only the minimum requirement.  Vendors MAY always implement more than is required.

### Relationship

If there is a relationship between two (or more) data items, please state it explicitly in the definition.  Example: if a failure counter increments every time some sensor goes out of range or some status goes to some value, please say that.

# Special Cases

## Status Properties

Status properties SHOULD always include 
1=other
2=unknown

If there is an "okay" or "good" status for a particular unit, use the value 
3=good.

The default value of a status property SHOULD be unknown(2).

## Description Properties

Properties named "Description" are almost always the vendor-supplied localizable description or caption string for a unit.

<Need to decide on conventions for lengths, mandatory-ness, and default values.>

## Alerts

<Need to define alert conditions for devices.>

## Counters

Counters are extremely useful to application software that wants to know if something changed recently.  An application can read a few counters, particularly error counters, power-on counters, configuration change counters, etc., quickly to assess any changes in the device.

A counter SHALL not specify an associated range of values.

## Resettable Counters

In general, counters may not be written directly by the user.  If it is desired permit a management client to reset a counter value to zero, then an additional Reset property should be included that is user writable.  Storing a specific value into the Reset property will result in setting the counter value to zero.

See the ResettableCounter.xml example. for sample property definitions.

## FRUs

When a Field Replaceable Unit is replaced, the agent SHOULD provide unambiguous information to management clients indicating the replacement.  The Replaceable FRU example in FruReset.xml illustrates a method to implement this change atomically.