Appendix D - Overall Printer Status Table

The following table defines a suggested relationship between various printer states and the variables Printer hrDeviceState, hrPrinterStatus, hrPrinterDetectedErrorState, prtAlertGroup, prtAlertCode,  and prtChannelStatus.  This table is the recommended implementation of these variables. It is provided to guide implementors of this MIB and users of the MIB by providing a sample set of states and the variable values that are expected to be produced as result of that state.

This information supplements that provided in Section 2.2.13.2 "Overall Printer Status". This is not an exhaustive list rather it is a guideline. More detailed status, if needed, should be obtained from the alert table and the sub-unit status variables.

|                                               |                               | Subunit Status

|           General Printer Status              |         prtAlertTable         | (PrtSubUnitStatusTC)

|-----------------------------------------------+-------------------------------+---------------------

|             |                 |               |               |               | prtInputStatus

|             |                 |               |               |               | (per input tray)

|             |                 |               |               |               | prtOutputStatus

|             |                 |               |               |               | (per output bin)

|             |                 |               |               |               | prtMarkerStatus

|             |                 |               |               |               | (per marker)

|             |                 |               |               |               | prtMediaPathStatus

|             |                 | hrPrinter     |               |               | (per path)

Printer            | hrDevice    | hrPrinter       | Detected      |               |               | prtChannelStatus

State              | Status      | Status          | ErrorState    | prtAlertGroup | prtAlertCode  | (per channel)

-------------------+-------------+-----------------+---------------+---------------+---------------+---------------------

Normal or idle     |             | idle (3)        | (0x0)         | (no binary    | (no binary    | all groups are:

(no binary alerts) | running (2) |                 |               |  alerts)      |  alerts)      |   Idle (0)

|             |                 |               |               |               |

Printing           | running (2) | printing (4)    | (0x0)         | (no binary    | (no binary    | Availability may be:

(no binary alerts) |             |                 |               |  alerts)      |  alerts)      |   Idle (0) or

|             |                 |               |               |               |   Active (4) or

|             |                 |               |               |               |   Busy (6)

|             |                 |               |               |               | depending on

|             |                 |               |               |               | subsystem state

|             |                 |               |               |               |

Off-line           | down (5)    | other (1)       | Offline (2)   | generalPrinter| subunitOffline| prtChannelStatus:

|             |                 |               | (5)           | (22)          |   Availability=

|             |                 |               |               |               |   Unavailable and

|             |                 |               |               |               |   OnRequest+

|             |                 |               |               |               |   Critical Alerts+

|             |                 |               |               |               |   Off-Line

|             |                 |               |               |               |   (1+16+32)

|             |                 |               |               |               |

Standby or Power   | running (2) | other (1)       | (0x0)         | generalPrinter| subunitPower  | prtChannelStatus:

Saver Mode         |             |                 |               | (5)           | Saver(23)     |   Availability=

|             |                 |               |               |               |   Available and

|             |                 |               |               |               |   Standby (2)

|             |                 |               |               |               |

Initial Power Up   |             |                 |               |               |               | all groups are:

(no binary alerts  | down (5)    | warmup (5)      | Offline (2)   | (no binary    | (no binary    |   Availability=

(while warming up) |             |                 |               |  alerts)      |  alerts)      |   Unknown+

|             |                 |               |               |               |   Transitioning

|             |                 |               |               |               |   (5+64)

Warming Up         | running (2) | warmup (5)      | (0x0)         | (no binary    | (no binary    | all groups are:

(no binary alerts) |             |                 |               |  alerts)      |  alerts)      |   Availability=

|             |                 |               |               |               |   Standby+

|             |                 |               |               |               |   Transitioning

|             |                 |               |               |               |   (2+64)

Critical Errors: the printer can not print

(the error may be current while printing)

Jam                | down (5)    | other (1)       | Jammed(4)     | appropriate   | jammed(8)     | PrtSubUnitStatusTC

|             |                 |               | group         |               |   Availability=

|             |                 |               |               |               |   Unavailable because

|             |                 |               |               |               |   broken+Critical

|             |                 |               |               |               |   Alerts (3+16)

|             |                 |               |               |               |

Cover/Door         | down (5)    | other (1)       | Door Open(8)  | cover(6)      | coverOpened(3)| prtCoverStatus:

Open (listed       |             |                 |               |               |               |    coverOpen (3)

in cover table)    |             |                 |               |               |               |

|             |                 |               |               |               |

Input Tray         | down (5)    | other (1)       | No Paper(64)  | input(8)      | subunitMissing| prtInputStatus:

Missing            |             |                 |               |               | (9)           |   Availability=

|             |                 |               |               |               |   Unavailable because

|             |                 |               |               |               |   broken+Critical

|             |                 |               |               |               |   Alerts (3+16)

|             |                 |               |               |               |

Input Tray         | down (5)    | other (1)       | No Paper(64)  | input(8)      | subunitEmpty  | prtInputStatus:

Empty              |             |                 |               |               | (13)          |   Availability=

|             |                 |               |               |               |   Unavailable because

|             |                 |               |               |               |   broken+Critical

|             |                 |               |               |               |   Alerts (3+16)

|             |                 |               |               |               |

Output Tray        | down (5)    | other (1)       | Service       | output(9)     | subunitMissing| prtOutputStatus:

Missing            |             |                 | Requested(1)  |               | (9)           |   Availability=

|             |                 | + Offline (2) |               |               |   Unavailable because

|             |                 |               |               |               |   broken+Critical

|             |                 |               |               |               |   Alerts (3+16)

|             |                 |               |               |               |

Output Tray        | down (5)    | other (1)       | Service       | output(9)     | subunitFull   | prtOutputStatus:

Full               |             |                 | Requested(1)  |               | (15)          |   Availability=

|             |                 | + Offline (2) |               |               |   Unavailable because

|             |                 |               |               |               |   broken+Critical

|             |                 |               |               |               |   Alerts (3+16)

|             |                 |               |               |               |

Marker Supply      | down (5)    | other (1)       | No Toner(16)  | markerSupplies| subunitMissing| prtMarkerStatus:

Missing (i.e.,     |             |                 |               | (11)          | (9)           |   Availability=

toner missing,    |             |                 |               |               |               |   Unavailable because

ink missing)      |             |                 |               |               |               |   broken+Critical

|             |                 |               |               |               |   Alerts (3+16)

|             |                 |               |               |               |

Marker Supply      | down (5)    | other (1)       | No Toner(16)  | markerSupplies| subunitEmpty  | prtMarkerStatus:

Empty (i.e.,       |             |                 |               | (11)          | (13)          |   Availability=

toner empty,      |             |                 |               |               |               |   Unavailable because

ink missing)      |             |                 |               |               |               |   broken+Critical

|             |                 |               |               |               |   Alerts (3+16)

|             |                 |               |               |               |

Non-Critical Errors: the printer can still print while condition exists

(the warning may be current while printing)

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitAlmost | prtInputStatus:

Low                |             | printing (4) or | Paper(128)    |               | Empty(12)     |   Non-Critical

|             | warmup (5)      |               |               |               |   Alerts (8)

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitAlmost | prtOutputStatus:

Almost Full        |             | printing (4) or | Requested(1)  |               | Full(14)      |   Non-Critical

|             | warmup (5)      |               |               |               |   Alerts (8)

|             |                 |               |               |               |

Marker Supply      | warning (3) | idle (3) or     | Low           | markerSupplies| subunitAlmost | prtMarkerStatus:

Almost Empty       |             | printing (4) or | Toner (32)    | (11)          | Empty(12)     |   Non-Critical

(i.e., toner low,  |             | warmup (5)      |               |               |               |   Alerts (8)

ink low)          |             |                 |               |               |               |

|             |                 |               |               |               |

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitMissing| prtInputStatus:

Missing (when n-1  |             | printing (4) or | Paper(128)    |               | (9)           |   Non-Critical

trays are missing  |             | warmup (5)      |               |               |               |   Alerts (8)

with linking)      |             |                 |               |               |               |

|             |                 |               |               |               |

Input Tray         | warning (3) | idle (3) or     | Low           | input(8)      | subunitEmpty  | prtInputStatus:

Empty (when n-1    |             | printing (4) or | Paper(128)    |               | (13)          |   Non-Critical

trays are empty    |             | warmup (5)      |               |               |               |   Alerts (8)

with linking)      |             |                 |               |               |               |

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitMissing| prtOutputStatus:

Missing (when n-1  |             | printing (4) or | Requested(1)  |               | (9)           |   Non-Critical

trays are missing  |             | warmup (5)      |               |               |               |   Alerts (8)

with linking)      |             |                 |               |               |               |

|             |                 |               |               |               |

Output Tray        | warning (3) | idle (3) or     | Service       | output(9)     | subunitFull   | prtOutputStatus:

Full (when n-1     |             | printing (4) or | Requested(1)  |               | (15)          |   Non-Critical

trays are full     |             | warmup (5)      |               |               |               |   Alerts (8)

with linking)      |             |                 |               |               |               |

The definition of PrtSubUnitStatusTC specifies that SubUnitStatus is an integer that is the

sum of 5 distinct values/states: Availability, Critical, Non-Critical, On-line and Transitioning.

Thus when a non-critical alert or alerts are present the values for Availability, On-Line and

Transitioning will be summed with the Non-Critical Alerts (8) value.