-- The Input Switching Group

--

-- The input switching group allows the administrator to set the

-- input subunit timeout for the printer and to control the automatic

-- input subunit switching by the printer when an input subunit becomes

-- empty.

--

-- This group is optional. However, to claim conformance to this group,

-- it is required to implement every object in the group.

prtInputMediaLoadTimeout OBJECT-TYPE

SYNTAX		Integer32

MAX-ACCESS	read-write

MIN-ACCESS	read-only

STATUS		current

DESCRIPTION

"When the printer is not able to print due to a subunit being

empty or the requested media must be manually loaded, the printer

will wait for the duration (in seconds) specified by this object.

Upon expiration of the timeout, the printer will take the action

specified by prtInputNextIndex.

The event which causes the printer to enter the waiting state

is product specific. If the printer is not waiting for manually

fed media, it may switch from an empty subunit to a different

subunit without waiting for the timeout to expire.

A value of (-1) implies 'other' or

'infinite' which translates to 'wait forever'. The action which

causes printing to continue is product specific.

A value of (-2) implies 'unknown'."

::= { prtInputEntry 24 }

prtInputNextIndex OBJECT-TYPE

SYNTAX		Integer32

MAX-ACCESS	read-write

MIN-ACCESS	read-only

STATUS		current

DESCRIPTION

"The value of prtInputIndex corresponding to the input subunit

which will be used when this input subunit is emptied and the

timeout specified by prtInputMediaLoadTimeout expires. A value

of zero(0) indicates that auto input switching will not occur

when this input subunit is emptied. If the timeout specified by

prtInputLoadMediaTimeout expires and this value is zero(0), the

job will be aborted. A value of (-1) means other.

The value (-2) means 'unknown' and specifically indicates that

an implementation specific method will determine the next input

subunit to use at the time this subunit is emptied and the

timeout expires. The value(-3) means input switching is not

supported for this subunit."

::= { prtInputEntry 26 }