Printer Management Interface

Management Object Specification

Version 0.7, April 27, 1994

By

Ronald L. Smith

Texas Instruments

email:	rlsmith@nb.ppd.ti.com
phone:	(817) 774-6151

F.D. Wright

Lexmark International

email:   don@lexmark.com

phone:    606-232-4808

Tom Hastings

Digital Equipment Corp.

email:	hastings@hannah.enet.dec.com

phone:	(508) 467-8299

This document specifies the semantics of the IETF Printer MIB and the DMTF Printer MIFin a manner that is independent of the syntax for MIB and MIF specifications. After this document is relatively stable, it will be converted into the appropriate MIB and MIF format specifications.

# Introduction

The Printer Management Interface (PMI) Object Specification specifies the semantics of the IETF Printer MIB and the DMTF Printer MIF in a manner that is independent of the syntax for MIB and MIF specifications.

<Model Description and Introduction goes here.  But I couldn't read Steve’s pmi_mod3 file.>

<Picture of printers, print systems, and Network Management Stations (NMSs) go here.>

Enumerations

This working group has defined several type of enumerations.  These enumerations differ in the method employed to control the addition of new enumerations.  Throughout this document, references to "enum(n)", where n can be 1, 2 or 3 can be found in the various tables.  The definitions of these types of enumerations are:

ENUM(1)	Additional enumerated values require a new RFC.

ENUM(2)	Additional enumerated values are registered after review by this working group

ENUM(3)	Additional enumerated values are registered without working group review.

Each enumeration is given a symbolic name and the values are listed at the end of this specification.

In IETF MIBs, the objects defined by this specification will have a prefix of prt and the name of the group (Input, Output, Interpreter, etc.  In DMTF MIBs, the same attributes will have ??? prefixes.  However, to save space in this specification and to be neutral between the IETF and the DMTF conventions, the prt and the group name prefix will not generally be given.  The prefix is given at the top of each MS-WORD table.

When a description of an object/attribute refers to another object/attribute, the name of that other object/attribute is given in boldface.  Such references to include neither the prt prefix nor the group name prefix, if the object/attribute is in the same group.  In the rare cases that a reference is made to an object in another group, both the prt prefix and the group name are included in the reference.

Printer Model

This working group has developed a model of a printer which is useful in understanding the classification of the various object.  Graphically this model is as shown below:

µ §

µ §

# Printer Component

To be supplied by DMTF to correspond to MIB II

DMTF MIF "Component ID: (From DMI Specification - Draft 4.2 - 12 January 1994)

1.	Manufacturer	String64	Name of the organization that produces this component.

2.	Product	String64	Name of this Printer

3.	Version	String64

4.	Serial Number	String64

5.	Installation	Date	Time and Date of the Last Installation

6.	Verify	Integer	Verification Level for this printer

"Asking for the 'Verify' attribute causes the component instrumentation code to perform checks to verify that the component is still in the system and working properly. It should return on of the following values:

Value	Meaning

0	An error occurred; Check status code

1	Component does not exist

2	Verify not supported

4	Component exists, functionally untested

5	Component exists, functionally unknown

6	Component exists, functionally no good

7	Component exists, functionally good

## System Group objects

This is  place holder to list the System Group objects from MIB-II RFC 1213 that shall be implemented when implementing the Printer MIF/MIF.......

## System Controller

This is a place holder to list the Host Resources objects from RFC 1514 that shall be implemented when implementing the Printer MIB/MIF.  Possible groups from which we may choose objects include: System, Storage, Device, Running Software, Running Software Performance, and Installed Software.......

## Interface Group objects

This is a place holder to list the Interface Group objects from RFC 1573 that shall be implemented when implementing the Printer MIB/MIF.......

# General Printer Group Objects

| prtGeneral Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ConfigChange | Read-Only | Counter32 |  | Counts configuration changes. |
| DefaultOrientation | Read-Write | Enum (1) | MediaOrientation | The current orientation default. This value applies to all input, all interpreters, etc. unless overridden by other control. |
| CurrentLocalization | Read-Write | Integer32 |  | Index into the localization table specifying the current language, country and character set for Description string values. |

Notes:

1.	Overall printer status can be determined using the following objects from the host MIB: (hrDevice Status, hrPrinterStatus, and hrPrinterDetectedErrorState.

| prtGeneral Mandatory prtSkinTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value used by the printer to identify this skin sub-unit. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new skin components to the printer), values are expected to remain stable across successive printer power cycles. |
| Description | Read-Only | String |  | The manufacturer provided skin component name in the localization specified by CurrentLocalization. This object should be in localization specified by CurrentLocalization. |
| Status | Read-Only | Enum(2) | SkinStatus | The status of this skin sub-unit. |

Notes:

| prtGeneral Mandatory prtLocalizationTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Language | Read-Only | String(2) |  | A two character language code from ISO 639. Examples: EN, FR, DE, ... |
| Country | Read-Only | String(2) |  | A two character country code from ISO 3166, a blank string (two space characters) shall indicate that the country is not defined. Examples: US, GB, CA, FR, DE, ... |
| CharacterSet | Read-Only | Integer32 |  | An index into the character set table identifying the character set used for this localization. |

Notes:

| prtGeneral Mandatory prtCharacterSetTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| CharacterSet | Read-Only | String |  | A character set name or identifier that identifies this character set. |

Notes:

| prtGeneral - Extended Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Reset | Write-Only | Enum(3) | ResetType | Writing causes a reset |

Notes:

## Input Group Objects - Mandatory

Input sub-units are managed as a tabular, indexed collection of possible devices capable of providing media for input to the printing process.  Input sub-units typically have a location, a type, an identifier, a set of constraints on possible media sizes and potentially other media characteristics, and may be capable of indicating current status or capacity.

| prtInput Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | The ID of the default input device, i.e., the value of the corresponding input device’s ID value. |

Notes:

| prtInput Mandatory prtInputTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value used by the printer to identify this input device. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new input devices to the printer), values are expected to remain stable across successive printer power cycles. |
| Type | Read-Only | Enum (2) | InputType | The type of technology (discriminated primarily according to feeder mechanism type) employed by the input device. Note, the Optional Input Class provides for a descriptor field to further qualify the other choice. |
| DimUnit | Read-Write | Enum (1) | PhyMediaUnit | The unit of measurement for use in calculating and relaying dimensional values for this input device. |
| MediaDimAcrossFeedDir | Read-Write | Integer32 |  | This object provides the value of the current dimensions supported by the input device for measurements taken ninety degrees relative to the feed direction associated with that device in input device dimensional units (DimUnit). The value (-1) means other and specifically indicates that the device places no restriction on this parameter. The value (-2) indicates unknown. |
| MediaDimFeedDir | Read-Write | Integer32 |  | The current dimensions supported by the input device for measurements taken parallel relative to the feed direction associated with that device in input device dimensional units (DimUnit). The value (-1) means other and specifically indicates that the device places no restriction on this parameter. The value (-2) indicates unknown. |
| CapacityUnit | Read-Write | Enum (1) | CapacityUnit | The unit of measurement for use in calculating and relaying capacity values for this input device. |
| MaxCapacity | Read-Write | Integer32 |  | The maximum capacity of the input sub-unit in input device capacity units (CapacityUnit). There is no convention associated with the media itself so this value reflects claimed capacity. This field is read-write in order to allow a management application to request the desired capacity input device. The value (-1) means other and specifically indicates that the device places no restrictions on this parameter. The value (-2) means unknown. |
| CurrentLevel | Read-Write | Integer32 |  | The current capacity of the input subunit in input device capacity units (CapacityUnit). This field is read-write to accommodate remote op-panel implementation. The value (-1) means other and specifically indicates that the device places no restrictions on this parameter. The value (-2) means unknown. |
| Status | Read-Only | Enum | EntStatus | The current status of this input device. |
| MediaName | Read-Write | String |  | The name of the current medium contained in this input sub-uni. Examples: “iso-a4-white”, “letter-white”, "Hillary's Pink Presidential Letterhead" |

Notes:

µ §

Input Group Objects - Optional

| prtInput - TypeExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Name | Read-Write | String |  | The name assigned to this input device. |
| VendorName | Read-Only | String |  | The vendor name of this input device. |
| Model | Read-Only | String |  | The model name of this input device. |
| Version | Read-Only | String |  | The version of this input device. |
| SerialNumber | Read-Only | String |  | The serial number assigned to this input device. |
| Description | Read-Only | String |  | A free-form text description of this input device in the localization specified by CurrentLocalization. |
| Security | Read-Write | Enum (2) | YesNo-OnOff | Indicates if this input device has some security associated with it. |

Notes:

Media Extended Group - OPTIONAL

The Media Extended Group supports identification of media installed or available for use on a printing device. Medium resources are identified by name, and include a collection of characteristic attributes that may further be used for selection and management of them. The Media Extended group is conceptually built in the form of an SNMP "AUGMENTS" construction off of the InputDevice table. In this manner, a minimally conforming implementation may choose to not support reporting of media resources if it cannot do so.

| prtInput - MediaExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MediaWeight | Read-Write | Integer32 | grams/meter2 | The weight of the medium in grams / per meter squared associated with this input device. The value (-1) means other and specifically indicates that the device places no restrictions on this parameter. The value (-2) means unknown. |
| MediaType | Read-Write | String |  | The type of medium associated with this input device. See Appendix for standardized names from ISO 10175 (DPA) & ISO 10180 (SPDL) |
| MediaColor | Read-Write | String |  | The color of the medium associated with this input device. See Appendix for standardized names from ISO 10175 (DPA) & ISO 10180 (SPDL) |
| MediaFormParts | Read-Write | Integer32 |  | The number of parts associated with the medium associated with this input device if the medium is a multi-part form. The value (-1) means other and specifically indicates that the device places no restrictions on this parameter. The value (-2) means unknown. |

Notes:

Output Group Objects - Mandatory

Output sub-units are managed as a tabular, indexed collection of possible devices capable of receiving media delivered from the printing process.  Output sub-units typically have a location, a type, an identifier, a set of constraints on possible media sizes and potentially other characteristics, and may be capable of indicating current status or capacity.

| prtOutput Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | The ID of the default output device, i.e., the value of the corresponding output device’s ID value. |

Notes:

| prtOutput Mandatory prtOutputTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value used by this printer to identify this output device. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new output devices to the printer), values are expected to remain stable across successive printer power cycles. |
| Type | Read-Only | Enum (2) | OutputType | The type of technology supported by this output device. |
| CapacityUnit | Read-Only | Enum (1) | CapacityUnit | The unit of measurement for use in calculating and relaying capacity values for this output device. |
| MaxCapacity | Read-Write | Integer32 |  | The maximum capacity of this output sub-unit in output device capacity units (CapacityUnit). There is no convention associated with the media itself so this value essentially reflects claimed capacity. This field is read-write in order to allow a management application to set the desired capacity of the output device. The value (-1) means other and specifically indicates that the device places no restrictions on this parameter. The value (-2) means unknown. |
| RemainingCapacity | Read-Write | Integer32 |  | The remaining capacity of the possible output sub-unit capacity in output device capacity units (CapacityUnit) of this output device.. |
| Status | Read-Only | Enum | EntStatus | The current status of this output device. |

Notes:

µ §

Output Group Objects - Optional

| prtOutput - TypeExtended Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Name | Read-Write | String |  | The name assigned to this output device. |
| VendorName | Read-Only | String |  | The vendor name of this output device. |
| Model | Read-Only | String |  | The name assigned to this output device. |
| Version | Read-Only | String |  | The version of this output device. |
| SerialNumber | Read-Only | String |  | The serial number assigned to this output device. |
| Description | Read-Only | String |  | A free-form text description of this output device in the localization specified by CurrentLocalization. |
| Security | Read-Write | Enum (2) | YesNo-OnOff | Indicates if this output device has some security associated with it. |

Notes:

| prtOutput - DimExtended Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DimUnit | Read-Only | Enum (1) | PhyMediaUnit | The unit of measurement for use in calculating and relaying dimensional values for this output device. |
| MaxDimAcrossFeedDir | Read-Write | Integer32 |  | The maximum dimensions supported by this output device for measurements taken ninety degrees relative to the feed direction associated with that device in output device dimensional units (DimUnit). |
| MaxDimFeedDir | Read-Write | Integer32 |  | The maximum dimensions supported by this output device for measurements taken parallel relative to the feed direction associated with that device in output device dimensional units (DimUnit). |
| MinDimAcrossFeedDir | Read-Write | Integer32 |  | The minimum dimensions supported by this output device for measurements taken ninety degrees relative to the feed direction associated with that device in output device dimensional units (DimUnit). |
| MinDimFeedDir | Read-Write | Integer32 |  | The minimum dimensions supported by this output device for measurements taken parallel relative to the feed direction associated with that device in output device dimensional units (DimUnit). |

Notes:

| prtOutput - Features Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| StackingOrder | Read-Write | Enum (1) | StackingOrder | The current state of the stacking order for the associated output device. “1 to n” means pages are output in the order in which they are acquired. “n to 1” means pages are output from last to first which implies that all pages must be acquired before they can be output. |
| PageDeliveryOrientation | Read-Write | Enum (1) | PageDeliveryOrientation | Indicates which reading surface will be "up" when pages are delivered to the associated output device. Values are Face-Up and Face-Down. (Note: interpretation of these values is in general context-dependent based on locale; presentation of these values to an end-user should be normalized to the expectations of the user). |
| Bursting | Read-Write | Enum (1) | YesNo-OnOff | By its presence, this object indicates that the output device supports bursting, and if so, whether the feature is enabled. Bursting is the process by which continuous media is separated into individual sheets, typically by bursting along pre-formed perforations. |
| Decollating | Read-Write | Enum (1) | YesNo-OnOff | By its presence, this object indicates that the output device supports decollating, and if so, whether the feature is enabled. Decollating is the process by which the individual parts within a multi-part form are separated and sorted into separate stacks for each part. |
| PageCollated | Read-Write | Enum (1) | YesNo-OnOff | By its presence, this object indicates that the output device supports page collation, and if so, whether the feature is enabled. |
| Job Collated | Read-Write | Enum (1) | YesNo-OnOff | By its presence, this object indicates that the output device supports job collation, and if so, whether the feature is enabled. |
| OffsetStacking | Read-Write | Enum (1) | YesNo-OnOff | By its presence, this object indicates that the output device supports offset stacking, and if so, whether the feature is enabled. |
| Slipsheet | Read-Write | Enum (1) | YesNo-OnOff | By its presence, this object indicates that the output device supports slipsheet insertion between sets, and if so, whether the feature is enabled. |

Notes:

## Output Device Associated Finishing

Finisher devices are typically complicated systems and can (or should be) dealt with largely isolated from the "output device."  Since there are so many configuration options for finisher devices (and some of them may indeed be integrated within the output device itself), the question is whether to deal with finishers as part of output device management, or whether to deal with them separately.  The merit of separating the two "types" of devices is that it enables greater modularity across the widest variety of interesting configurations, and that is the approach taken in this specification.

A common model for basic management of finisher devices is provided later in this proposal in the Finisher Class section.

| prtOutput - Associated Finisher Optional prtOutputAssocFinisher |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | The ID of the output device to which the finisher is connected. |
| Stitching | Read-Only | Enum(1) | YesNo-OnOff | By its presence, this object indicates that the output device supports stapling as a process accessible in the production path associated with it. The value returned by this object is the identifier for the associated finishing device; the value serves as the index for identifying the device in the prtFinisherTable. |
| Folding | Read-Only | Enum(1) | YesNo-OnOff | By its presence, this object indicates that the output device supports folding as a process accessible in the production path associated with it. The value returned by this object is the identifier for the associated finishing device; the value serves as the index for identifying the device in the prtFinisherTable. |
| Binding | Read-Only | Enum(1) | YesNo-OnOff | By its presence, this object indicates that the output device supports binding as a process accessible in the production path associated with it. The value returned by this object is the identifier for the associated finishing device; the value serves as the index for identifying the device in the prtFinisherTable. |
| Punching | Read-Only | Enum(1) | YesNo-OnOff | By its presence, this object indicates that the output device supports punching as a process accessible in the production path associated with it. The value returned by this object is the identifier for the associated finishing device; the value serves as the index for identifying the device in the prtFinisherTable. |
| Trimming | Read-Only | Enum(1) | YesNo-OnOff | By its presence, this object indicates that the output device supports trimming as a process accessible in the production path associated with it. The value returned by this object is the identifier for the associated finishing device; the value serves as the index for identifying the device in the prtFinisherTable. |
| Insertion | Read-Only | Enum(1) | YesNo-OnOff | By its presence, this object indicates that the output device supports inserting as a process accessible in the production path associated with it. The value returned by this object is the identifier for the associated finishing device; the value serves as the index for identifying the device in the prtFinisherTable. |

Notes:

Finisher Group Objects - Optional

The Finisher Group is an optional group, and is included in this specification for completeness due to its relationship to the Output Group.

In general, finishers are very complicated devices and their management is a topic suitable for considerable, additional investigation. The goal of this class is therefore not to standardize management of arbitrarily complex finishers. Rather, this class only standardizes basic management objects suitable for a very generic abstract finisher. The value of this, is that by defining this basic class we establish the architectural framework for a common approach which can then be augmented as required in a consistent manner by vendors and consortia.

Finishing Configuration

The finishing table maintains mappings between configurations of possibly zero or more finishers and zero or more output devices. Each output device may provide a value for the optional column "prtOutputAssociatedFinisher". The value in that column is an integer which is the "prtFinisherConfigurationId". The value for the "prtFinisherConfigurationId" is the index into the "prtFinisherTable".

| prtFinisher Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | The ID of the default finisher device, i.e., the value of the corresponding finisher device’s ID value. A value of 0 means that a finisher is not used in default configurations. |

Notes:

| prtFinisher Optional prtFinisherTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value used by the printer to identify this finisher device. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new finisher devices to the printer), values are expected to remain stable across successive printer power cycles. |
| Type | Read-Only | Enum(2) | FinisherType | The type of finishing performed by this finisher device |
| NamedFinishing | Read-Only | Enum(2) | NamedFinishing | The finishing operations available through this finishing device. |
| ConsumableUnit | Read-Only | Enum(1) | FinisherConsumableUnit | The consumable resource unit appropriate to this finisher device and its operation. |
| MaxConsumableCapacity | Read-Write | Integer32 |  | The maximum capacity of this finisher in consumable resource units of measure (ConsumableUnit). This value is particularly suited for providing a gauge that can reflect the maximum consumable resource capacity of this finisher. |
| CurrentConsumableCapacity | Read-Write | Integer32 |  | The current capacity of this finisher in consumable resource units of measure (ConsumableUnit). This value is particularly suited for providing a gauge that can reflect the current available consumable resource capacity of this finisher. |
| Status | Read-Only | Enum(1) | EntStatus | The current status of this finisher. |

Notes:

Marker Group Objects - Mandatory

| prtMarker Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | The ID of the default marker device, i.e., the value of the corresponding marker device’s ID value. |

Notes:

| prtMarker Mandatory prtMarkerTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value used by the printer to identify this marking device. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new marking devices to the printer), values are expected to remain stable across successive printer power cycles. |
| MarkTech | Read-Only | Enum (2) | MarkingTech | The type of marking technology used for this marking device |
| CounterUnit | Read-Only | Enum (1) | CounterUnit | The unit that will be used by the printer when reporting counter values for this marking device. The time units of measure are provided for a device like a strip recorder that does not or cannot track the physical dimensions of the media and does not use characters, lines or sheets. |
| LifeCount | Read-Only | Counter32 |  | The count of the number of units of measure counted during the life of printer using units of measure as specified by CounterUnit. |
| PowerOnCount | Read-Only | Counter32 |  | The count of the number of units of measure counted since the equipment was most recently powered on using units of measure as specified by CounterUnit. |
| ProcessColorants | Read-Only | Integer32 |  | The number of process colors supported by this marker. A process color of 1 implies monochrome. The value of this object and SpotColorants cannot both be 0. A value of (-1) implies 'other' or 'infinite' while a value of (-2) implies 'unknown'. |
| SpotColorants | Read-Only | Integer32 |  | The number of spot colors supported by this marker. The value of this object and ProcessColorants cannot both be 0. A value of (-1) implies 'other' or 'infinite' while a value of (-2) implies 'unknown' |
| AddressabilityUnit | Read-Only | Enum (1) | AddrUnit | The unit of measure of the denominator of Horizontal and Vertical addressability. |
| AddressabilityFeedDir | Read-Only | Integer32 |  | The addressability of the engine in the feed direction in units of measure specified by AddressabilityUnit. A value of (-1) implies 'other' or 'infinite' while a value of (-2) implies 'unknown'. |
| AddressabilityCrossFeedDir | Read-Only | Integer32 |  | The addressability of this marker in the cross feed direction in units of measure specified by AddressabilityUnit. A value of (-1) implies 'other' or 'infinite' while a value of (-2) implies 'unknown'. |
| NorthMargin | Read-Only | Integer32 |  | The margin from the leading edge of the medium in units identified by AddressabilityUnit. Printing within this margin (and all others) is guaranteed for all interpreters. The value (-2) means unknown. |
| SouthMargin | Read-Only | Integer32 |  | The margin from the trailing edge of the medium in units identified by AddressabilityUnit. Printing within this margin (and all others) is guaranteed for all interpreters. The value (-2) means unknown. |
| WestMargin | Read-Only | Integer32 |  | The margin from the left edge of the medium in units identified by AddressabilityUnit. Printing within this margin (and all others) is guaranteed for all interpreters. The value (-2) means unknown. |
| EastMargin | Read-Only | Integer32 |  | The margin from the right edge of the medium in units identified by AddressabilityUnit. Printing within this margin (and all others) is guaranteed for all interpreters. The value (-2) means unknown. |
| Status | Read-Only | Enum(1) | EntStatus | The current status of this marker device. |

## Printable Area Offsets and Extents

The printable area margins as listed above define an area of the print media which is guaranteed to be printable for all combinations of input, media paths, and interpreters for this marker.

Notes:

| prtMarkerSupplies Mandatory prtMarkerSuppliesTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MarkerSupplyID | Read-Only | Integer32 |  | A unique value used by the printer to identify this marker supply. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new marker supplies to the printer), values are expected to remain stable across successive printer power cycles. |
| MarkerID | Read-Only | Integer32 |  | The Marker ID of the marking device with which this marker supply entity is associated. |
| ColorantID | Read-Only | Integer32 |  | The Colorant ID of the colorant with which this marker supply entity is associated. This value shall be 0 if there is no colorant table. |
| Class | Read-Only | Enum (1) | MkrSupClass | Indicates whether this supply entity represents a supply container that is consumed or a receptacle that is filled. |
| Type | Read-Only | Enum(3) | MkrSupType | The type of this supply. |
| Description | Read-Only | String |  | The description of this supply container/receptacle in the localization specified by CurrentLocalization. |
| SupplyUnit | Read-Only | Enum(1) | MkrSupUnit | Unit of this marker supply container/receptacle. |
| MaxCapacity | Read-Only | Integer32 |  | The maximum capacity of this supply container/receptacle expressed in SupplyUnit. |
| Level | Read-Only | Integer32 |  | The current level if this supply is a container; the remaining space if this supply is a receptacle. |

Notes:

| prtMarkerColorant Optional prtMarkerColorantTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ColorantID | Read-Only | Integer32 |  | A unique value used by the printer to identify this colorant. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new colorants to the printer), values are expected to remain stable across successive printer power cycles. |
| MarkerID | Read-Only | Integer32 |  | The marker ID with which this colorant entry is associated. |
| Role | Read-Only | Enum (1) | ColorantRole | The role played by this colorant. |
| Value | Read-Only | String | ColorantValue | The name of the color of this colorant using standardized string names. Additional color string names may be added using ISO 9070 naming conventions. |
| Tonality | Read-Only | Integer32 |  | The distinct levels of tonality realizable by a marking device when using this colorant. This value does not include the number of levels of tonal difference that an interpreter can obtain by techniques such as half toning. This value must be at least 2. |

Notes:

Media Path Group Objects - Mandatory

| prtMediaPath Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | The ID of the default media path, i.e., the value of the corresponding media path’s ID value. |
| MaxSpeedPrintUnit | Read-Only | Enum (1) | SpeedPrintUnit | The unit of measure used in specifying the speed of all media paths in the printer, i.e., the numerator of the units of measure of the media path speed. |
| MaxSpeedTimeUnit | Read-Only | Enum (1) | SpeedTimeUnit | The unit of time used in specifying the speed of all media paths in the printer, i.e., the denominator of the units of measure of the media path speed. |
| MediaSizeUnit | Read-Only | Enum (1) | PhyMediaUnit | The units of measure of media size for use in calculating and relaying dimensional values for all media paths in the printer. |

Notes:

| prtMediaPath Mandatory prtMediaPathTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value used by the printer to identify this media path. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new media paths to the printer), values are expected to remain stable across successive printer power cycles. |
| MaxSpeed | Read-Only | Integer32 |  | The maximum printing speed of this media path expressed in media path speed units (units of MaxSpeedPrintUnit per MaxSpeedTimeUnit). A value of (-1) implies 'other', while a value of (-2) implies 'unknown'. |
| MaxMediaFeedDir | Read-Only | Integer32 |  | The maximum physical media size in the feed direction of this media path expressed in units of measure specified by MediaSizeUnit. A value of (-1) implies 'other' or 'infinite' while a value of -(-2) implies 'unknown'. A value of 0 implies unlimited. |
| MaxMediaAcrossFeedDir | Read-Only | Integer32 |  | The maximum physical media size across the feed direction of this media path expressed in units of measure specified by MediaSizeUnit. A value of (-1) implies 'other' or 'infinite' while a value of (-2) implies 'unknown'. |
| MinMediaFeedDir | Read-Only | Integer32 |  | The minimum physical media size in the feed direction of this media path expressed in units of measure specified by MediaSizeUnit. A value of (-1) implies 'other' while a value of (-2) implies 'unknown'. |
| MinMediaAcrossFeedDir | Read-Only | Integer32 |  | The minimum physical media size across the feed direction of this media path expressed in units of measure specified by MediaSizeUnit. A value of (-2) implies 'unknown'. |
| MediaPathType | Read-Only | Enum(2) | MediaPathType | The type of media path for this media path. |
| MediaPathDescription | Read-Only | String |  | The manufacturer-provided description of this media path in the localization specified by CurrentLocalization. |
| Status | Read-Only | Enum | EntStatus | The current status of this media path. |

Notes:

Channel Group Objects

Channels are sources of print data.  There are many kinds of channels; some of which are based on networks and others which are not.  For example, a channel can be a serial (or parallel) connection; it can be a service, such as the Unix Line Printer Daemon (LPD), offering itself over a network connection (interface); or it could be a disk drive into which a floppy disks with the print data is inserted.

| Channel example | Implementation |
| --- | --- |
| serial port channel | bi-directional data channel |
| parallel port channel | often uni-directional channel |
| IEEE 1284 port channel | bi-directional channel |
| SCSI port channel | bi-directional |
| Apple PAP channel | may be based on Local-, Ether- or TokenTalk |
| LPD Server channel | typically TCP/IP based, port 515 |
| Novell Remote Printer channel | typically SPX/IPX based |
| Novell Print Server channel | typically SPX/IPX based |
| port 9100 channel | HP and friends |
| Adobe AppSocket(9101) channel | a bi-directional extension of LPD |

It is easy to note that this is a mixed bag.  There are some physical connections over which no (or meager) protocols are run (e.g.  the serial or old parallel ports) and there are services which often have elaborate protocols that run over a number of protocol stacks.  In the end what is important is the delivery of print data through the channel.

Using the OSI seven layer model of for data interchange, the "channel" fits most accurately into the Application layer.  Although this is not entirely clean (especially for the serial and old-parallel ports) it is clear that the interpreters that are consuming the print data are running at an application level.

## The channel table and its underlying structure

The first five items in the Channel Table define the "channel" itself as seen at the OSI Application Layer (layer 7).  This channel depends on lower layers of the OSI protocol, most often layers 1-4, to provide the data that flows through the channel.  (Layer 1 is the physical connection; layer 2 is the data-link protocol, such as EtherNet; layer 3 is the inter-networking protocol, such as IP; and layer 4 is the transport protocol, such as TCP.) (Layers 5 (session) and 6(presentation) are most often irrelevant to the channel, but layer 6 could be used for character code conversion and/or decompression.  A printer may be connected to print data sources in more than one way which means that there may be multiple variants at each of layers 1-4.  Hence some way is needed to specify the "underlying" structure of each channel.

What is this underlying structure? A serial channel, has very little structure, except perhaps a protocol to allow binary data to be sent transparently over that channel along with control information.  A network connection will typically have one or more variants at each of levels 1-4 of the OSI model.  An example channel might have the Novell Remote Printer application protocol running on top of SPX (at layer 4) running on top of IPX (at layer 3) running on top of EtherNet (at layer 2) running over a large EtherNet cable (at layer 1).

In SNMP, there are groups which represent these various layers: the Interface Groups represent layers 1 and 2, including sub-layers.  The inter-networking groups, such as the IP group represent layer 3 and its associated addressing.  The Transmission groups, such as the TCP group and its associated connection table represent layer 4.  The IfTable in the Interfaces Group has a which represents the various possible interfaces up to the Interment Level (level 3).  There are no corresponding structures within SNMP that represent higher levels.  For this reason, for this standard, the underlying structure will be represented only by identifying the topmost interface that underlies each channel.

The prtChannelTable represents the set of input data sources which can provide print data to one or more of the interpreters available on a printer

| prtChannel Mandatory prtChannelTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value used by the printer to identify this data channel. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new data channels to the printer), values are expected to remain stable across successive printer power cycles. |
| CurrentJobCntlLangID | Read-Write | Integer32 |  | The value of this attribute is an Index that selects an entry in the prtInterpreterTable. This interpreter defines the syntax used for control functions, such as querying or changing environment variables and identifying job boundaries (e.g. PJL, PostScript, NPAP). |
| DefaultPageDescLangID | Read-Write | Integer32 |  | The value of this attribute is an Index that selects an entry in the prtInterpreterTable. This interpreter defines the default Page Description Language interpreter to be used for the print data unless the Control Language is used to select a specific interpreter (e.g., PCL, PostScript Language, auto-sense). |
| State | Read-Write | Enum(1) | ChannelState | The state of this print data channel. The value determines whether control information and/or print data is allowed through this channel. |
| Type | Read-Only | Enum(2) | ChannelType | The type of this print data channel. |
| IfIndex | Read-Write | Integer |  | The value of IfIndex (in the ifTable; see the interface section of MIB-2/RFC 1573) which corresponds to this channel. When more than one row of the ifTable is relevant, this is the index of the row representing the topmost layer in the if hierarchy. |
| Status | Read-Only | Enum(1) | EntStatus | The current status of the channel. |

Notes:

Console Group Objects

| prtOperatorConsole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ConsoleLocalization | Read-Write | Integer32 |  | An index into localization table giving the language, country, and character set to be used for the console. |
| NumberOfDisplayLines | Read-Only | Integer32 |  | The number of lines on printers physical display. |
| NumberOfDisplayChars | Read-Only | Integer32 |  | The number of characters per line displayed on the physical display. |
| ConsoleLock | Read-Write | Enum(2) | LockState | This object is used to lock or unlock the operatoon of the operators console. |

| prtOperatorConsole Mandatory prtDisplayBufferTable |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |  |
| LineText | Read-Write | String |  | This object is the content of a line in the logical display buffer of the operator’s console of the printer. When a write operation occurs, normally a critical message, to one of the LineText strings, the agent should make that line displayable if a physical display is present. The printer should not overwrite this line once written until a zero length string is written to unlock that line. Printer generated strings should be in the localization specified by ConsoleLocalization. Management Application generated strings should be localized by the Management Application. |  |

Notes:

| prtOperatorConsole Mandatory prtConsoleLightsTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| LightID | Read-Only | Integer32 |  | A unique value used by the printer to identify this light. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new lights to the printer), values are expected to remain stable across successive printer power cycles. |
| OnTime | Read-Only | Integer32 |  | The on time in milliseconds of blinking of this light; 0 indicates off always. |
| OffTime | Read-Only | Integer32 |  | The off time in milliseconds of blinking of this light; 0 indicates on always. |
| Color | Read-Only | Enum(2) | LightColor | The color of this light. |
| Description | Read-Only | String |  | The vendor description or label of this light in the localization specified by ConsoleLocalization. |

Notes:

Interpreter Group Objects - Mandatory

| prtInterpreter Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 | IntpLangFam | The ID of the default interpreter, i.e., the value of the corresponding interpreter’s ID, to be used in case no device specific method is used to specify which interpreter should be used. Priority on selecting the interpreter are 1) Commands in the data stream; 2) The default interpreter for the channel on which the data arrives; and lastly 3) This object. |

The prtInterpreterTable is a table representing the interpreters in the printer.  An entry shall be placed in the interpreter table for each interpreter on the printer.

| prtInterpreter Mandatory prtInterpreterTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | A unique value for each PDL or control language for which there exists an interpreter or emulator in the printer. The value is used to identify this interpreter. Although these values may change due to a major reconfiguration of the device (e.g. the addition of new interpreters to the printer), values are expected to remain stable across successive printer power cycles. |
| LangFamily | Read-Only | Enum(2) | IntpLangFam | The family name of a Page Description Language (PDL) or control language which this interpreter in the printer can interpret or emulate. This list of enumerations requires review before additional entries are made. |
| LangLevel | Read-Only | String |  | The level of the language which this interpreter is interpreting or emulating. This might contain a value like '5e' for an interpreter which is emulating level 5e of the PCL language. It might contain '2' for an interpreter which is emulating level 2 of the PostScript language. Similarly it might contain '2' for an interpreter which is emulating level 2 of the HPGL language. |
| LangVersion | Read-Only | String |  | The date code or version of the language which this interpreter is interpreting or emulating. |
| Description | Read-Only | String |  | A string to identify this interpreter in the localization specified by CurrentLocalization as opposed to the language which is being interpreted. It is anticipated that this string will allow manufacturers to unambiguously identify their interpreters. |
| Version | Read-Only | String |  | The date code, version number, or other product specific information tied to this interpreter. This value is associated with the interpreter, rather than with the version of the language which is being interpreted or emulated. |
| FeedAddressability | Read-Only | Integer32 |  | The maximum interpreter resolution in the feed direction in prtMarkerAddressabilityUnit (the same units of measure used by the object used to specify marker addressability) for this interpreter. If this interpreter has no limit on addressability the value is zero. The value (-1) means other and specifically indicates that the device places no restrictions on this parameter. The value (-2) means unknown. |
| CrossFeedAddressability | Read-Only | Integer32 |  | The maximum interpreter resolution in the across feed direction in prtMarkerAddressabilityUnit (the same units of measure used by the object used to specify marker addressability) for this interpreter. If this interpreter has no limit on addressability the value of this object is zero. The value (-1) means other and specifically indicates that the device places no restrictions on this parameter. The value (-2) means unknown. |
| DefaultCharSetIn | Read-Write | Integer32 |  | An index into the CharacterSetTable that identifies the default character set for data into this interpreter. |
| DefaultCharSetOut | Read-Write | Integer32 |  | An index into the CharacterSetTable that identifies the default character set for data coming from this interpreter through the printer's output channel |
| TwoWay | Read-Only | Enum(1) | YesNo-OnOff | Indicates whether or not this interpreter returns information back across the data channel to the host. |

Notes:

Alerts Group Object - Mandatory

The prtAlertTable lists all the critical and non-critical alerts currently active in the printer.  A critical alert is one that stops the printer from printing immediately and printing can not continue until the critical alert condition is eliminated.  Non-critical alerts are those items that do not stop printing but may at some future time.  The table contains information on the severity, component, detail location within the component, alert code and description of each critical alert that is currently active within the printer.

| prtAlert Mandatory prtAlertTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Index | Read-Only | Integer32 |  | The index value used to determine which alerts have been added or removed from the alert table. This in an incrementing integer starting from zero when the printer is powered on. When the printer adds an alert to the table, that alert is assigned the next higher integer value from the last item entered into the table. If the index value reaches its maximum value, the next item entered will cause the index value to roll over and start at zero again. The first event placed in the alert table after power up initialization of the printer will have an index value of 1. |
| SeverityLevel | Read-Only | Enum (1) | SeverityLevelAlertCode | The level of severity of this alert table entry. The printer determines the severity level assigned to each entry into the table. |
| TrainingLevel | Read-Only | Enum (2) | TrainingLevel | The level of training required to handle this alert. The training level is an enumeration that is determined and assigned by the printer manufacturer based on the information or the training required to handle this alert. The printer will break alerts into these different training levels. It is the responsibility of the management application in the system to determine how a particular alert is handled and how and to whom that alert is routed. |
| Group | Read-Only | Enum (1) | AlertGroupCode | The sub-unit within the printer model that this alert is related. Input, output, and markers are examples of printer model sub-units. |
| Group Index | Read-Only | Integer32 |  | An integer designation, assigned by the printer manufacturer, of the specific sub-unit within the group that caused this alert. The combination of the Group and the GroupIndex defines exactly which printer sub-unit caused the alert. Input #3, Output #2, and Marker #1 are examples. |
| Location | Read-Only | Integer32 |  | The sub-unit location that is defined by the printer manufacturer to further refine the location of this alert with the designated sub-unit. The location is used in conjunction with the Group and Index values. Example: There is an alert in Input #2 at location number 7. |
| Alert Code | Read-Only | Enum (2) |  | The code that describes the type of alert for this entry in the table. There are different codes for each sub-unit type. Jam, Media Low, Toner Out, and Output Full are examples of different codes for various sub-unit types. |
| Description | Read-Only | String |  | A description of this alert entry in the localization specified by CurrentLocalization. The description is provided by the printer to further elaborate on the enumerated alert or provide information in the case where the code is classified as “other” or “unknown”. The printer is required to return a description string but the string may be a null string. |

Notes:

Group Status

| EntStatus 1 Other 2 Unknown 3 Ready/Available 4 Critical State 5 Non-critical state 6 Both Critical and Non-critical state 7 On-request 8 Unavailable 9 Busy/Temp Unavailable |  |
| --- | --- |

PMI Enumerations - Units of measure

| AddressabililtyUnit 1 Other 2 Unknown 3 .0001 Inches 4 .001 millimeters | MediaUnit 1 Other 2 Unknown 3 .0001 Inches 4 .001 Millimeters 8 Sheets | CounterUnit 1 Other 2 Unknown 3 .0001 inches 4 .001 millimeters 5 Characters 6 Lines 7 Impressions 8 Sheets 9 Dot-Row 11 Hours 16 Feet 17 Meters |
| --- | --- | --- |
| SpeedPrintUnit 1 Other 2 Unknown 3 .0001 inches 4 .001 millimeters 5 Characters 6 Lines 7 Impressions 8 Sheets 9 Dot-Row | SpeedTimeUnit 1 Other 2 Unknown 11 Hours | FinisherConsumableUnit 1. Other 2. Unknown 3. .0001 inches 4. .001 millimeters 8. Sheets 12. .001 ounces 13. .1 grams 14. .01 fluid ounces 15. .1 milliliters 16. Feet 17. Meters 18. Each |
| MarkerSupplyUnit 1 Other 2 Unknown 3 .0001 inches 4 .001 millimeters 8 Sheets 12 .001 ounces 13 .1 grams 14 .01 fluid ounces 15 .1 milliliters | CapacityUnit 1 Other 2 Unknown 3 .0001 inches 4 .001 millimeters 8 Sheets 16 Feet 17 Meters |  |

PMI Enumerations - Device Types

| InputType 1 Other 2 Unknown 3 Sheet-feed-auto-removable-tray 4 sheet-feed-auto-non removable tray 5 Sheet Feed Manual 6 continuous-roll 7 continuous-fan-fold | OutputType 1 Other 2 Unknown 3 Removable Bin 4 UnRemovable Bin 5 Continuous Roll Device 6 Mail Box | MediaPathType 1 Other 2 Unknown 3 Long Edge Binding Duplex 4 Short Edge Binding Duplex 5 Either edge Duplex 6 Simplex |
| --- | --- | --- |
| FinisherType 1 Other 2 Unknown 3 Stitching 4 Binding 5 Trimming 6 Die-cutting 7 Punching 8 Perforating 9 Slitting 10 Inserting 11 Covers 12 Folding | SkinStatus 1 Other 2 Unknown 3 Door Open 4 Door Closed 5 Interlock Open 6 Interlock Closed | MarkerSupplyClass 1 Other 2 Unknown 3 Supply that is consumed 4 Receptacle that is filled |
| MarkingTechnology 1 Other 2 Unknown 3 Electophotographic:LED 4 Electrophotographic:Laser 5 Electrophotographic:Other 6 Impact:Moving_Head:Dot_Matrix:9pin 7 Impact:Moving_Head:Dot_Matrix:24pin 8 Impact:Moving_Head:Dot_Matrix:other 9 Impact:Moving_Head:Fully_Formed 10 Impact:band 11 Impact:other 12 Inkjet:Aqueous 13 Inkjet:Solid 14 Inkjet:Other 15 Pen 16 Thermal:Transfer 17 Thermal:Sensitive 18 Thermal:Diffusion 19 Thermal:Other 20 Electroerosion 21 Electrostatic 22 Photographic:Microfiche 23 Photographic:Imagesetter 24 Photographic:Other 25 Ion-deposition 26 E-beam 27 Typesetter | ChannelType 1. Other 2. Unknown 3. Serial Port 4. Parallel Port 5. IEEE1284 Port 6. SCSI Port 7. Apple PAP 8. LPD Server 9. Novell RPrinter 10. Novell Pserver 11. port 9100 12. Adobe AppSocket (9101) 13. FTP ("PUT" to printer) 14. TFTP (to printer) 15. DLC/LLC Port 16. IBM 3270 17. IBM 5250 18. Fax 19. IEEE1394 20. Transport 1 (port 35) 21. CPAP (port 170) 22. IPC (?) 23. OLE 24. Named Pipe | MarkerSupplyType 1 Other 2 Unknown 3 Toner 4 Waste Toner 5 Ink 6 Ink Cartridge 7 Ink Ribbon 8 Waste Ink 9 OPC (Organic Photo Conductor) 10 Developer 11 Fuser Oil 12 Wax, Solid 13 Wax, ribbon 14 Waste Wax |

IntpLangFam - Interpreter Language Family

| Value | Enum symbolic name | Description - becomes a MIB/MIF comment |
| --- | --- | --- |
| 1 | other(1) |  |
| 2 | unknown(2) |  |
| 3 | PCL(3) | Printer Control Language, or Printer Command Language Starting with version 5, HP-GL/2 is part of PCL. Hewlett-Packard Co. |
| 4 | HPGL(4) | Hewlett-Packard Graphics Language. Originally used to control pen movements on plotters.Includes many device specific commands. Hewlett-Packard Co. |
| 5 | PJL(5) | Peripheral Job Language. Appears in the data stream between data intended for a page description language. Hewlett-Packard Co. |
| 6 | PostScript(6) | Adobe Systems Incorporated Mountain View, CA |
| 7 | IPDS(7) | Intelligent Printer Data Stream IBM |
| 8 | PPDS(8) | IBM Personal Printer Data Stream. Originall called IBM ASCII, the name was changed to PPDS when the LaserPrinter was introduced in 1989. Lexmark International, Inc. |
| 9 | EscapeP(9) |  |
| 10 | Epson(10) |  |
| 11 | DDIF(11) | Digital Document Interchange Format Digital Equipment Corp. |
| 12 | Interpress(12) |  |
| 13 | ISO-6429(13) | ISO 6429. Control functions for Coded Character Sets (has ASCII control characters, plus additional controls for character imaging devices.) ISO Standard, Geneva, Switzerland |
| 14 | line-data(14) | Lines of data as separate ASCII or EBCDIC records and containing no control functions (no CR, LF, HT, FF, etc.)- traditional line printers |
| 15 | MODCA(15) |  |
| 16 | ReGIS(16) | Remote Graphics Instruction Set, DIgital Equipment Corp, Maynard MA |
| 17 | SCS(17) |  |
| 18 | SPDL(18) | ISO 10180 Standard Page Description Language ISO Standard, Geneva, Switzerland |
| 19 | Tek4014(19) |  |
| 20 | PDS(20) |  |
| 21 | IGP(21) |  |
| 22 | Code-V(22) | Magnum Code-V, Image and printer control language used to control impact/dot-matrix printers. QMS, Inc., Mobile AL |
| 23 | DSC-DSE(23) |  |
| 24 | WPS(24) | Windows Printing System, Resource based command/data stream used by Microsoft At Work Peripherals. Developed by the Microsoft Corporation. |
| 25 | LN03(25) | Early DEC-PPL3, Digital Equipment Corp. |
| 26 | CCITT(26) |  |
| 27 | QUIC(27) | QUIC (Quality Information Code), Page Description Language for laser printers. Included graphics, printer control capability and emulation of other well-known printer languages. QMS, Inc. Mobile, AL |
| 28 | CPAP(28). | Common Printer Access Protocol Digital Equipment Corp. |
| 29 | DEC-PPL(29) | Digital ANSI-Compliant Printing Protocol Digital Equipment Corp. |
| 30 | simple-text(30) | Character coded data, including NUL, CR , LF, HT, and FF control characters. See ISO 10175 Document Printing Application (DPA) ISO standard, Geneva, Switzerland |
| 31 | NPAP(31) | Network Printer Alliance Protocol |
| 32 | DOC(32) | Document Option Commands, Appears in the data stream between data intended for a page description language. QMS, Inc. Mobile, AL |
| 33 | IMPRESS(33) | imPRESS, Page description language originally developed for the ImageServer line of systems. A binary language providing representations for text, simple graphics (rules,lines,conic sections), and some large forms (simple bit-map and CCITT group 3/4 encoded).The language was intended to be sent over an 8-bit channel and supported early document preparation languages (e.g. TeX and TROFF). QMS, Inc. Mobile, AL |
| 34 | Pinwriter(34) | 24 wire dot matrix printer language for USA, Europe, and Asia except Japan. More widely used in Germany, and some Asian countries than in US. NEC |
| 35 | NPDL(35) | Page printer language for Japanese market. This is one of the majors in Japanese market. NEC |
| 36 | 201PL(36) | Serial printer language used in the Japanese market.This is one of the dominant in Japanese market. NEC |
| 37 | automatic | Automatic sensing of the interpreter language family by the printer examining the document content. Which actual interpreter language families are sensed depends on the printer implementation. |

PMI Enumerations - Misc

| MediaOrientation 1 Other 2 Unknown 3 Portrait 4 Reverse Portrait 5 Landscape 6 Reverse Landscape | MediaColor 1 Other 2 Unknown 3 White 4 Pink 5 Yellow 6 Buff 7 Goldenrod 8 Blue 9 Green 10 Transparent | LineStatus 1 Other 2 Unknown 3 On Line 4 Pending On Line 5 Off Line 6 Pending Off Line |
| --- | --- | --- |
| NamedFinshing 1 Other 2 Unknown 3 Staple 4 Staple Top Left 5 Staple Bottom Left 6 Stable Top Right 7 Staple Bottom Right 8 Saddle Stitch 9 Edge Stitch 10 Punch 11 Cover 12 Bind | ColorantValue - String Other Unknown White Red Green Blue Cyan Magenta Yellow Black Additional names using ISO 9070 naming convention. | LightColor 1. Other 2. Unknown 3. White 4. Red 5. Green 6. Blue 7. Cyan 8. Magenta 9. Yellow |
| YesNo-OnOff 1. Other 2. Unknown 3. Yes - On 4. No - Off | ResetType 3. Power Cycle Reset 4. Reset NVRAM 5. Reset to Factory Defaults | ColorantRole 1. Other 2. Unknown 3. Process 4. Spot |
| TrainingLevel 1 Other 2 Unknown 3 Untrained 4 Trained 5 Field Service 6 Management | ChannelState 1. Other 2. Unknown 3. Print and Control Data Accepted 4. No Data Accepted | PageDeliveryOrientation 1. Other 2. Unknown 3. Face-Up 4. Face-Down |
| StackingOrder 1 Other 2 Unknown 3 1 to n 4 n to 1 | LockState 1. Other 2. Unknown 3. Locked 4. Unlocked |  |

PMI Enumerations - Alert Codes

| SeverityLevelAlertCode 1 Other 2 Unknown 3 Critical 4 Warning |  | AlertGroupCode 1 Other 2 Unknown 3 Input 4 Output 5 Finisher 6 Media Path 7 General Printer 8 Marker 9 System Controller 10 Interface 11 Interpreter 12 Operator Console 13 Channel |
| --- | --- | --- |
| InputAlertCode 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Size Change 11 Media Weight Change 12 Media Type Change 13 Media Color Change 14 Media Form Parts Change 15 Media Supply Low 16 Media Supply Empty | OutputAlertCode 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Tray Almost Full 11 Media Tray Full | MarkerAlertCode 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Toner Empty 10 Ink Empty 11 Print Ribbon Empty 12 Toner Almost Empty 13 Ink Almost Empty 14 Print Ribbon Almost Empty 15 Waste Toner Receptacle Almost Full 16 Waste Ink Receptacle Almost Full 17 Waste Toner Receptacle Full 18 Waste Ink Receptacle Full 19 Fuser Under Temperature 20 Fuser Over Temperature 21 OPC Life Almost Over 22 OPC Life Over 23 Developer Almost Empty 24 Developer Empty |
| GeneralPrinterAlertCode 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Door Open 8 Door Closed 9 Power Up 10 Power Down | MediaPathDeviceAlertCode 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Tray Almost Full 11 Media Tray Full | FinisherAlertCode 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Supply Empty 11 Supply Low 12 Receptacle Almost Full 13 Receptacle Full |
| InterpreterAlertCode 1 Other 2 Unknown 3 Configuration Change 4 Memory Increase 5 Memory Decrease 6 Cartridge Added 7 Cartridge Deleted 8 Resource Added 9 Resource Deleted 10 Resource Unavailable | InterfaceAlertCode 1 Other 2 Unknown 3 Configuration Change | ChannelAlertCode 1 Other 2 Unknown 3 Configuration Change |

# Appendices

## Appendix A: Glossary of terms

Alert -- a reportable event for which there is an entry in the alert table

Bin -- an output sub-unit which may or may not be removable

Component -- a complete product, such as a printer. (DMTF usage)

Critical Alert -- an alert triggered by an event which leads to a state in which printing is no longer possible; the printer is stopped

Description -- information about the configuration and capabilities of the printer and its various sub-units

Group -- a collection of attributes or objects that represent a sub-unit of a component

Input -- a tray or bin from which instances of the media are obtained and fed into the Media Path

Localization -- the specification of human language, country, and character set needed to present information to people in their native languages.

Management Application (a.k.a. Manager) -- a program which queries and controls one or more managed nodes

Management Station -- a physical computer on which one or more management applications can run

Media Path -- the mechanisms that transport instances of the media from an input, through the marker, possibly through media buffers and duplexing pathways, out to the output with optional finishing applied. The inputs and outputs are not part of the Media Path.

Non-critical Alert -- an alert triggered by a reportable event which does not lead to a state in which printing is no longer possible; such an alert may lead to a state from which printing may no longer be possible in the future, such as the low toner state or the alert may be pure informational, such as a configuration change at the printer.

Output -- a bin or stacker which accepts instances of media that have been processed by a printer

Printer -- a physical device that takes media from an input source, produces marks on that media according to some page description or page control language and puts the result in some output destination, possibly with finishing applied.

Printing -- the entire process of producing a printed document from generation of the file to be printed, choosing printing properties, selection of a printer, routing, queuing, resource management, scheduling, and finally printing including notifying the user

Reportable event -- an event that is deemed of interest to a management station watching the printer

Status -- information regarding the current operating state of the printer and its various sub-units. This is an abstraction of the exact physical condition of the printer.

Sub-mechanism -- a distinguishable part of a sub-unit

Sub-unit -- a part of the printer which may be a physical part, such as an input source or a logical part such as an interpreter.

Tray -- an input sub-unit which is typically removable

Visible state -- that portion of the state of the printer that can be examined by a management application

## Appendix B - Media size names

This appendix lists the standardized media size names from ISO/IEC 10175 Document Printing Application (DPA).  Management application developers are encouraged to use these media sizes in order to display a media size name to the user, since the names are more likely to be familiar to them than the numerical sizes returned as the values of prtInputMediaDimAccrossFeedDir and prtInputMediaDimFeedDir.

| String name | Description |
| --- | --- |
| other |  |
| unknown |  |
| na-letter or letter | North American letter size: 8.5 by 11 inches |
| na-legal or legal | North American legal size: 8.5 by 14 inches |
| na-10x13-envelope | North American 10x13 size: 10 by 13 inches |
| na-9x12-envelope | North American 9x12 size: 9 by 12 inches |
| na-number-10-envelope | North American number 10 business envelope |
| na-7x9-envelope | North American 7x9 size: 7 by 9 inches |
| na-9x11-envelope | North American 9x11 size: 9 by 11 inches |
| na-10x14-envelope | North American 10x14 size: 10 by 14 inches |
| na-number-9-envelope | North American number 9 business envelope |
| na-6x9-envelope | North American 6x9 size: 6 by 9 inches |
| na-10x15-envelope | North American 10x15 size: 10 by 15 inches |
| a | engineering A size |
| b | engineering B size |
| c | engineering C size |
| d | engineering D size |
| e | engineering E size |
| iso-a0 | ISO A0 size: 841 mm by 1189 mm |
| iso-a1 | ISO A1 size: 594 mm by 841 mm |
| iso-a2 | ISO A2 size: 420 mm by 594 mm |
| iso-a3 | ISO A3 size: 297 mm by 420 mm |
| iso-a4 | ISO A4 size: 210 mm by 297 mm |
| iso-a5 | ISO A5 size: 148 mm by 210 mm |
| iso-a6 | ISO A6 size: 105 mm by 148 mm |
| iso-a7 | ISO A7 size: 74 mm by 105 mm |
| iso-a8 | ISO A8 size: 52 mm by 74 mm |
| iso-a9 | ISO A9 size: 37 mm by 52 mm |
| iso-a10 | ISO A10 size: 26 mm by 37 mm |
| iso-b0 | ISO B0 size: 1000 mm by 1414 mm |
| iso-b1 | ISO B1 size: 707 mm by 1000 mm |
| iso-b2 | ISO B2 size: 500 mm by 707 mm |
| iso-b3 | ISO B3 size: 353 mm by 500 mm |
| iso-b4 | ISO B4 size: 250 mm by 353 mm |
| iso-b5 | ISO B5 size: 176 mm by 250 mm |
| iso-b6 | ISO B6 size: 125 mm by 176 mm |
| iso-b7 | ISO B7 size: 88 mm by 125 mm |
| iso-b8 | ISO B8 size: 62 mm by 88 mm |
| iso-b9 | ISO B9 size: 44 mm by 62 mm |
| iso-b10 | ISO B10 size: 31 mm by 44 mm |
| iso-c3 | ISO C3 size: 229 mm by 324 mm |
| iso-c4 | ISO C4 size: 229 mm by 324 mm |
| iso-c5 | ISO C5 size: 162 mm by 229 mm |
| iso-c6 | ISO C6 size: 162 mm by 229 mm |
| iso-designated | ISO Designated Long size: 110 mm by 220 mm |
| jis-b0 | JIS B0 size |
| jis-b1 | JIS B1 size |
| jis-b2 | JIS B2 size |
| jis-b3 | JIS B3 size |
| jis-b4 | JIS B4 size |
| jis-b5 | JIS B5 size |
| jis-b6 | JIS B6 size |
| jis-b7 | JIS B7 size |
| jis-b8 | JIS B8 size |
| jis-b9 | JIS B9 size |
| jis-b10 | JIS B10 size |

# Appendix Y - Issues

This appendix lists the issues that have been raised during the meetings and in between meetings.  The issues listed under each MS-WORD table under NOTES: are additional issues which are not included here.

1) Is the media group optional?

- Leave MediaExtended as optional, media information in the InputTable will remain mandatory.

2) Will the MIB/MIF support multiple consoles?

- No, do with private extensions, a single "console" that contains the data for all the consoles, or some other means.

3) Is General Printer a table?

- No, support Virtual printers through some other means such as private extensions, etc.

4) How does "character set" tie into "channel"?

- There was no identified need to tie character set to channels.  Character Set is more closely tied to the interpreter or console and those objects are tied to localization and character set settings.

5) Static versus dynamic - how efficient will this be?

- Because there is no distinction between static and dynamic information in SNMP, the printer must implement all the information so the agent can report it.

6) The MIB/MIF consist of 58 objects - is that too many?

- OPEN --- Defer for final review.

7) Should we endorse some type of interlock method for all the writable objects?

- OPEN --- One of the reasons it remains open is that there is no defined method for this function for DMTF.

8) Should we use separate community names or additional index to support multiple printers being managed through a printer server?

- Use community names with predictable "numbering" for each device.  Randy Turner (QMS) will write up a proposal for inclusion in the RFC.

9) Should paper size be writable?

- Yes, to tell a printer that can't detect paper size what size paper is loaded.

10) Should units be writable?

- Yes, but manufacturers may choose to only implement R/O access.  Persistance across power cycles may vary  from manufacturer to manufacturer.

11) Should size units be only in millimeters?

- No, but we will reduce the number of units wherever possible.

12) How should out of range sets be handled - reject or snap to the nearest?

- Snap to the nearest integer and reject an invalid ENUM.  The data returned in the SNMP packet will indicate the value of the integer snapped to.

13) Are "Input - dimensions extended" mandatory?

- This will remain at best optional; possibly could be removed.

14) In what units is printable area reported?

- .0001 inches or .001 mm

15) Should media type be a string or an enum?

- String, recommend using ISO DPA, ISO SPDL names where possible.

16) How do we associate media with inputs, outputs with finishers, etc?

- Input: The only media that is described is the media in an input tray and it is described there.

- Outputs: OPEN - Harry Lewis to make a proposal

17) Do margins (or imagable area) need to be PDL specific?

- No, but this object may not be needed.

18) Does "other" have a meaning for objects which normally have an integer value?

- Yes - potential meanings are defined with the object where it applies.

19) Is "unknown" the same as "uninitialized" and if not how do you tell the difference?

- Possibly, if the object's value is "unknown" failing when attempting to write to it means the object is truly unknown.  If writing is successful, the object's value was just unitialized.

20) Should default interpreter be by channel or overall?

- OVERALL.  There is a hierarchy used to determine the interpreter:

1) If contained in the datastream, used that interpreter

2) Otherwise use the default for the channel

3) Otherwise use the default for the interpreters

4) Otherwise the default interpreter is undefined or manufacturer specific

21) How do we handle the "parallel in, serial out" channel?

- Ignore it

22) Is "console width"  "string width" or "screen width"?

- Closed by addressing issue #23.

23) Is "number of lines" and "number of characters" of the console mean physically or logical?

- They refer to the physical size of the console.

24) Which strings are localized and controlled by "current localization?"

- Spec has been updated to reflect which strings are localized by CurrentLocalization, which strings are localized by ConsoleLocalization and which strings are not localized.  String not specifically stated to be localized should not be localized as that data may be used to make decisions.

25) Do we need an "ID" object for each table "index?"

- OPEN

26) Are all consoles using the same language?

- We only support one console.

27) Should we enable or disable a channel as a whole or provide a way to selectively enable and disable "data" and "control" flows on a channel?

- No convincing argument for separation of control and data.  Therefore, the answer is no.

28) Is the consistent use of 1,2 (for enums) and -1, -2 (for integers) to represent "other" and "unknown" consistent with SNMP "tradition?"

- There is a tradition of having certain distinguished values but those values often vary from RFC to RFC.  Consistant of use of these values within the PRINTER MIB should be OK.

# Appendix Z - Change History

This appendix list the changes made to each version of the specification

## Changes from Version 0.6, 26-Apr, to Version 0.7, 27-apr

1.	Added console lock

2.	Changed printable area to margins and moved to marker

3.	Worked through all notes and removed after resolving the issues.

4.	Updated issues list based upon second day of meeting (April 27, 1994)

5. 	Added finisher consumables units

## Changes from Version 0.2, 25-Apr, to Version 0.6, 26-apr

1. 	Cleaned-up formating

2.	Updated issues list based upon first day of meeting in April.

3. 	Changed version number to allign with original PMI_OBJ series of documents.

4.	Hide Tom Hastings editorial comments on page 2.

5.	This version was only distributed at the April IETF/DMTF meeting.

## Changes from Version 0.1, 23-Apr, to Version 0.2, 25-apr

1.	Added Appendix Y, List of Issues from the ftp-boi.external.hp.com directory.

2.	Replaced Interpreter Language Family enum table with the contents of the Interpreter Spec, so that the comments are included as well for each enum value.  Now this specification will be the only copy of the enum values assigned.

## Changes to create Version 0.1, 23-Apr

1.	Started with Version 0.5 of the Object Summary that Don edited with agreements reached at the March Seattle meeting.

2.	Copied in the descriptions from the following specifications that the working group had reviewed: and had been posted in the ftp-boi.external.hp.com server:

| number | file name | date | groups |
| --- | --- | --- | --- |
| 1 | general/pmi_obj5.doc | 22-Apr | all |
| 2 | channel_class/channel3.mif | 19-Apr | Channel |
| 3 | channel2 mail | 28-Mar | Console |
| 4 | source_dest_class/pmi_inp_outp_V0.3.txt | 23-Feb | Input, Output, Finisher |
| 5 | interpreters_class/interpreters.05 | 21-Apr | Interpreter |
| 6 | localization/localization.objects | 16-Mar | General |
| 7 | printer_class/prt_mif4.doc | 21-Feb | Marker, MediaPath |
| 8 | general/pmi_modl.doc | 30-Mar | Alerts |
| 9 | general/glossary.txt | 29-Mar | Appendix |

My MS-WORD for Windows 2.0a could not read the pmi_mdl3.doc, 19-Apr.

3.	Changed the names of objects/attributes and enum symbols that had the word Units to Unit to be consistent with the rest that had just Unit  Also changed Dims to Dim to be consistent with the rest that had just Dim.

4.	Changed the terminology of the descriptions that used "units" to "unit of measure", so as to avoid confustion with hardware units and sub-units.

5.	Spell checked the entire document and created a custom dictionary, pmi_spec.dic.

6.	Changed the names of a few of the Language Families that had come from the ISO DPA, to agree with the names in the ISO DPA.  In half of the cases, the ISO DPA committee agreed to change their names.  The following changes were made in this specification:   1)  LinePrinter to line-data, 2) SimpleText to simple-text.

7.	Changed the Language Family enum values that had been added to the Object Summary V0.5, after the Seattle meeting to agree with the 21-Apr version of the Interpreter specification: interpreters.05.

8.	Added automatic as enum 37, since the committee had agree in Seattle and SnowBird to add automatic as a PDL for which the printer determines which interpreter to use by looking at the print data itself.

9.	Editorially changed the beginning of each object/attribute description to be a noun that is the value and start each with the definite article: "The" or "A".

10.	Removed the terms object in the object/attribute descriptions since IETF MIBs use the term objects, while the DMTF MIF use the term attribute.  Still need to change the title of the specification though, since it containes “Object”.