Summary overview for finisher variables

| Variable | Meaning |
| --- | --- |
| ComponentID group |  |
| Manufacturer name, Product name, Version string, serial number, last install time, verification level | same as printmib.30.1.1.10 finDeviceDescription either specific OID’s per attribute or |
| Controller group |  |
| status | as in finisher mib : printmib.30.1.1.9 finDeviceStatus |
| error condition | error codes: key values to be supplied by finisher devices |
| warning | warning codes: key values to be supplied by finisher devices |
| Counters |  |
| run hours since production start-up |  |
| number of operations since installation | e.g. number of cutter cuts, number of folder folds, amount of roll paper, … |
| number of operations since start-up |  |
| number of operations since consumable replacement | e.g. since knife change, since roll change .. |
| number of processed documents | amount of finisher input or output objects processed or produced by operations. Not supplies / consumables, but device input and output workflow. |
| number of processed stacks | different input / output levels could be specified in a table with info about object type. |
| number of processed rolls | idem |
| operation unit | unit of measure for operation e.g. unit of cut (1/6 inch), of paper length (meters, …), .. |
| operation size | amount of units per operation e.g. cut size in 1/6 inch, paper length in meters |
| rest of supply | printmib.31.1.1.8 finSupplyCurrentLevel, e.g. amount of paper remaining on roll in meters, roll diameter in mm, … |
| percent rest of supply | printmib.31.1.1.8 finSupplyCurrentLevel / printmib.31.1.1.7 finSupplyMaxCapacity * 10,000 e.g. percent of remaining paper on roll or stack |
| supply unit | printmib.31.1.1.6 finSupplyUnit e.g. roll diameter in mm, paper meters on roll |