Subj:		Five Approved Additional Objects for the Printer MIB

From:	Tom Hastings

Date:		3/1/96

File:		prt-add.doc, .ps

Version:	0.2

This paper contains the updated specifications of the 5 additional objects that the PWG reviewed at its Jan 24-25 meeting for adding to the Printer MIB when the Printer MIB progresses to draft status.  Many printers support these features today.  We also agreed to specify how phone numbers, fax numbers, and email addresses could be added to the existing prtGeneralCurrentOperator and prtGeneralServicePerson objects.

The PWG agreements were for a new optional AuxiliarySheet group, and addition to the optional ExtendedInput group.  However, in preparing all these additions to RFC 1759 and discussing it with several implementors, it seemed better to add the 3 new objects dealing with input tray switching and manual tray to a new optional group, rather than adding them to the current ExtendedInput group for three reasons:

- Low end implementations that have manual input trays and/or input tray switching are not likely to want to implement the ExtendedInput group (prtInputName, prtInputVendorName, prtInputModel, prtInputVersion, prtInputSerialNumber, prtInputDescription, prtInputSecurity)

- Current implementations of RFC 1759 that implement the optional ExtendedInput group (such as the 5si) will still be conforming to the draft Printer MIB, if we put the three new objects into a new optional group, say, InputSwitching group, instead of extending the ExtendedInput group.

- Since the three new objects have to be added to the end of the (Input) table, putting them in a new group, means that all the objects in each of the groups in the prtInputTable have adjacent OID arcs assigned, so that there are not gaps in any group.

So I took the liberty of showing the complete set of changes to the text of RFC 1759 for these objects as follows:

| Group | Table | Object |
| --- | --- | --- |
| AuxiliarySheet (new) | prtGeneralTable | prtGeneralStartupPage |
|  |  | prtGeneralBannerPage |
| InputSwitching (new) | prtInputTable | prtInputManualFeedTimeout |
|  |  | prtInputAutoSwitch |
|  |  | prtInputNextIndex |

In addition the minutes of the PWG January meeting indicated that prtInputManualFeedTimeout needed some clarification which I also added concerning its interaction with the other two objects that deal with switching.  Now a manual input tray time-out can either abort the job or switch to another tray, depending on the values set.  See the change history at the end.

Revision marks are used in the remainder of this document to indicate changes to be made to the Printer MIB when it is progressed from proposed status (RFC 1759) to draft status.

# Additional Textual Conventions

Add the following textual convention up front where the textual conventions are:

PrtSupplementaryPageContent ::= TEXTUAL-CONVENTION

STATUS     current

DESCRIPTION

"Supplementary pages are added by the printing system and

are not part of the actual print job. Examples include

error pages and banner pages. This textual convention is

used to specify the information content of these pages."

SYNTAX     INTEGER {

noPage(3),        -- no page will be produced

concise(4),       -- content of page is concise

verbose(5)        -- content of page is verbose

}

# Specify syntax for phone numbers, fax, email for ResponsibleParty group

Instead of adding new objects to the ResponsibleParty Group, add special syntax to indicate phone numbers, fax, and email that software (either the NMS or the agent) could use to notify the operator or service person.  The revision marks indicate the changes from RFC 1759, page 35:

-- The Responsible Party group

--

-- This group is optional.  However, to claim conformance to this

-- group, it is necessary to implement every object in the group.

prtGeneralCurrentOperator OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..127))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name of the current human operator responsible for

operating this printer. It is suggested that this string

include information that would enable other humans to reach

the operator, such as a phone number.

As a convention to facilitate automatic notification of the

operator by the agent or the network management station,

the phone number, fax number or email should be placed on a

separate line starting with ASCII LF (hex 0C) and the ASCII

text string (without the quotes): ‘phone: ‘, ‘fax: ‘, and

‘email: ‘, respectively.  Phone numbers may contain spaces

and parentheses, which shall be ignored, and all the digits

to reach the organization’s outside dial-up lines, if

required.  The ASCII comma (hex 23) is used to indicate a

half second pause."

::= { prtGeneralEntry 4 }

prtGeneralServicePerson OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..127))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name of the last human responsible for servicing

this printer. It is suggested that this string

include information that would enable other humans to reach

the service person, such as a phone number.  As a

convention to facilitate automatic notification of the

service person by the agent or the network management

station, the phone number, fax number or email should be

placed on a separate line starting with ASCII LF (hex 0C)

and the ASCII text string (without the quotes):

‘phone: ‘, ‘fax: ‘, and ‘email: ‘, respectively.  Phone

numbers may contain spaces and parentheses, which shall be

ignored and all the digits to reach the organization’s

outside dial-up lines, if required.  The ASCII comma

(hex 2C) is used to indicate a half second pause."

::= { prtGeneralEntry 5 }

# Put 2 objects into a new optional AuxiliarySheet group.

Put prtGeneralStartupPage and prtGeneralBannerPage into a new AuxiliarySheet group and add them to the end of the prtGeneralTable.

## Add the 2 objects to the end of prtGeneralTable (extending prtGeneralEntry)

Add prtGeneralStartupPage and prtGeneralBannerPage columns to the end of the prtGeneralTable (extending prtGeneralEntry) on page 28:

prtGeneralEntry OBJECT-TYPE

SYNTAX      PrtGeneralEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"An entry exists in this table for each

device entry in the hostmib device table who's type

is `printer'"

INDEX  { hrDeviceIndex }

::= { prtGeneralTable 1 }

PrtGeneralEntry ::= SEQUENCE {

-- Note that not all of the objects in this sequence are in the

-- general printer group.

prtGeneralConfigChanges         Counter32,

prtGeneralCurrentLocalization   Integer32,

prtGeneralReset                 INTEGER,

prtGeneralCurrentOperator       OCTET STRING,

prtGeneralServicePerson         OCTET STRING,

prtInputDefaultIndex            Integer32,

prtOutputDefaultIndex           Integer32,

prtMarkerDefaultIndex           Integer32,

prtMediaPathDefaultIndex        Integer32,

prtConsoleLocalization          Integer32,

prtConsoleNumberOfDisplayLines  Integer32,

prtConsoleNumberOfDisplayChars  Integer32,

prtConsoleDisable               INTEGER,

prtGeneralStartupPage           PresentOnOff,

prtGeneralBannerPage            PresentOnOff

}

## Put the 2 objects into a new optional AuxiliarySheet group

Add the specifications of a new optional AuxiliarySheet group to page 35, so that it is the last group specified as part of the prtGeneralTable, just before the start of the Input Group:

-- The Responsible Party group

--

-- This group is optional.  However, to claim conformance to this

-- group, it is necessary to implement every object in the group.

prtGeneralCurrentOperator OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..127))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name of the current human operator responsible for

operating this printer. It is suggested that this string

include information that would enable other humans to reach

the operator, such as a phone number."

::= { prtGeneralEntry 4 }

prtGeneralServicePerson OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..127))

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The name of the last human responsible for servicing

this printer. It is suggested that this string

include information that would enable other humans to reach

the service person, such as a phone number."

::= { prtGeneralEntry 5 }

-- The Auxiliary Sheet group

--

-- The auxiliary sheet group allows the administrator to control

-- the production of auxiliary sheets by the printer.

--

-- This group is optional.  However, to claim conformance to this

-- group, it is necessary to implement every object in the group.

prtGeneralStartupPage OBJECT-TYPE

SYNTAX          PresentOnOff

MAX-ACCESS      read-write

STATUS          current

DESCRIPTION

"Used to enable or disable printing a startup page.

If enabled, a startup page will be printed shortly

after power-up when the device is ready.

Typical startup pages include test patterns and/or

printer configuration information."

::= { prtGeneralEntry 16 }

prtGeneralBannerPage OBJECT-TYPE

SYNTAX            PresentOnOff

MAX-ACCESS        read-write

STATUS            current

DESCRIPTION

"Used to enable or disable printing banner pages at

the beginning of jobs. This is a master switch

which applies to all jobs, regardless of

interpreter."

::= { prtGeneralEntry 17 }

-- The Input Group

--

-- Input sub-units are managed as a tabular, indexed collection of

-- possible devices capable of providing media for input to the printing

-- process.  Input sub-units typically have a location, a type, an

-- identifier, a set of constraints on possible media sizes and

-- potentially other media characteristics, and may be capable of

-- indicating current status or capacity.

--

-- Implementation of every object in this group is mandatory.

prtInput   OBJECT IDENTIFIER ::= { printmib 8 }

## Add the new prtAuxiliarySheetGroup to the list of optional groups

Add prtAuxiliarySheetGroup to the list of optional groups on page 93:

-- the prtResponsiblePartyGroup, prtExtendedInputGroup,

-- prtInputMediaGroup, prtExtendedOutputGroup,

-- prtOutputDimensionsGroup, prtOutputFeaturesGroup,

-- prtMarkerSuppliesGroup, prtMarkerColorantGroup,

-- prtAlertTimeGroup, and the prtAuxiliarySheetGroup

-- are completely optional.

::= { prtMIBConformance 1 }

## Specify the conformance macro for the prtAuxiliarySheetGroup

Add the specification of the new prtAuxiliarySheetGroup to page 97 where it becomes the new last group to have its conformance specified:

prtAlertTimeGroup OBJECT-GROUP

OBJECTS { prtAlertTime }

STATUS  current

DESCRIPTION

"The alert time group."

::= { prtMIBGroups 18 }

prtAuxiliarySheetGroup OBJECT-GROUP

OBJECTS { prtGeneralStartupPage, prtGeneralBannerPage }

STATUS  current

DESCRIPTION

"The auxiliary sheet group."

::= { prtMIBGroups 19 }

END

# Put 3 objects into a new optional InputSwitching group

Put prtInputManualFeedTimeout, prtInputAutoSwitch, and prtInputNextIndex into a new prtInputSwitchingGroup and add them to the end of the prtInputTable.

## Add the 3 objects to the end of prtInputTable (extending prtInputEntry)

Add prtInputManualFeedTimeout, prtInputAutoSwitch, and prtInputNextIndex columns to the end of the prtInputTable (extending prtInputEntry) on pages 36-37:

prtInputTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtInputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A table of the devices capable of providing media for input

to the printing process."

::= { prtInput 2 }

prtInputEntry OBJECT-TYPE

SYNTAX     PrtInputEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Attributes of a device capable of providing media for input

to the printing process.

Entries may exist in the table for each device

index whose device type is `printer'."

INDEX  { hrDeviceIndex, prtInputIndex }

::= { prtInputTable 1 }

PrtInputEntry ::= SEQUENCE {

prtInputIndex                     Integer32,

prtInputType                      INTEGER,

prtInputDimUnit                   MediaUnit,

prtInputMediaDimFeedDirDeclared   Integer32,

prtInputMediaDimXFeedDirDeclared  Integer32,

prtInputMediaDimFeedDirChosen     Integer32,

prtInputMediaDimXFeedDirChosen    Integer32,

prtInputCapacityUnit              CapacityUnit,

prtInputMaxCapacity               Integer32,

prtInputCurrentLevel              Integer32,

prtInputStatus                    SubUnitStatus,

prtInputMediaName                 OCTET STRING,

prtInputName                      OCTET STRING,

prtInputVendorName                OCTET STRING,

prtInputModel                     OCTET STRING,

prtInputVersion                   OCTET STRING,

prtInputSerialNumber              OCTET STRING,

prtInputDescription               OCTET STRING,

prtInputSecurity                  PresentOnOff,

prtInputMediaWeight               Integer32,

prtInputMediaType                 OCTET STRING,

prtInputMediaColor                OCTET STRING,

prtInputMediaFormParts            Integer32,

prtInputManualFeedTimeout         Integer32,

prtInputAutoSwitch                PresentOnOff,

prtInputNextIndex                 Integer32

}

## Put the 3 objects into a new optional prtInputSwitchingGroup

Add the specifications of a new optional InputSwitching group to page 44, so that it is the last group specified as part of the prtInputTable, just before the start of the Output Group:

prtInputMediaFormParts OBJECT-TYPE

SYNTAX     Integer32

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The number of parts associated with the medium

associated with this input sub-unit if the medium is a

multi-part form.  The value (-1) means other and

specifically indicates that the device places no

restrictions on this parameter.  The value (-2) means

unknown."

::= { prtInputEntry 23 }

-- The Input Switching group

--

-- The input switching group allows the administrator to set the manual

-- input subunit timeout for the printer and to control the automatic

-- input subunit switching by the printer when an input subunit becomes

-- empty.

--

-- This group is optional.  However, to claim conformance to this

-- group, it is necessary to implement every object in the group.

prtInputManualFeedTimeout OBJECT-TYPE

SYNTAX            Integer32

MAX-ACCESS        read-write

STATUS            current

DESCRIPTION

"The duration in seconds after which the printer

shall either:

(a) switch to another input subunit,

if the value of prtInputNextIndex is non-zero and

prtInputAutoSwitch is on(3)

or

(b) abort any job waiting for manually fed input,

if the value of prtInputNextIndex is zero or

prtInputAutoSwitch is: off(4) or notPresent(5).

The event which causes the printer to enter the

waiting state is product specific.

A value of (-1) implies 'other' or 'infinite'

or this input subunit doesn’t support manual feed

while a value of (-2) implies 'unknown'."

::= { prtInputEntry 24 }

prtInputAutoSwitch OBJECT-TYPE

SYNTAX            PresentOnOff

MAX-ACCESS        read-write

STATUS            current

DESCRIPTION

"Indicates the state of the auto input switching

feature. The value notPresent(5) indicates

the feature is not currently supported. Exact

behavior of this feature is product specific."

::= { prtInputEntry 25 }

prtInputNextIndex OBJECT-TYPE

SYNTAX          Integer32

MAX-ACCESS      read-write

STATUS          current

DESCRIPTION

"The value of prtInputIndex corresponding to the

input subunit which will be used when this input

subunit is emptied. A value of zero(0) indicates

that auto input switching will not occur when

this input subunit is emptied."

::= { prtInputEntry 26 }

-- The Output Group

--

-- Output sub-units are managed as a tabular, indexed collection of

-- possible devices capable of receiving media delivered from the

-- printing process.  Output sub-units typically have a location,

-- a type, an identifier, a set of constraints on possible media

-- sizes and potentially other characteristics, and may be capable

-- of indicating current status or capacity.

--

-- Implementation of every object in this group is mandatory.

prtOutput  OBJECT IDENTIFIER ::= { printmib 9 }

prtOutputDefaultIndex OBJECT-TYPE

SYNTAX     Integer32

MAX-ACCESS read-write

Smith, Wright, Hastings, Zilles & Gyllenskog                   [Page 44]

## Add the new group prtInputSwitchingGroup to the list of optional groups

Add prtInputSwitchingGroup to the list of optional groups on page 93:

-- the prtResponsiblePartyGroup, prtExtendedInputGroup,

-- prtInputMediaGroup, prtExtendedOutputGroup,

-- prtOutputDimensionsGroup, prtOutputFeaturesGroup,

-- prtMarkerSuppliesGroup, prtMarkerColorantGroup,

-- prtAlertTimeGroup, prtAuxiliarySheetGroup, and

-- the prtInputSwitchingGroup are completely optional.

::= { prtMIBConformance 1 }

## Specify the conformance macro for the prtInputSwitchingGroup

Add the specification of the new prtAuxiliarySheetGroup to page 97 where it becomes the new last group to have its conformance specified:

prtAlertTimeGroup OBJECT-GROUP

OBJECTS { prtAlertTime }

STATUS  current

DESCRIPTION

"The alert time group."

::= { prtMIBGroups 18 }

prtAuxiliarySheetGroup OBJECT-GROUP

OBJECTS { prtGeneralStartupPage, prtGeneralBannerPage }

STATUS  current

DESCRIPTION

"The auxiliary sheet group."

::= { prtMIBGroups 19 }

prtInputSwitchingGroup OBJECT-GROUP

OBJECTS { prtInputManualFeedTimeout, prtInputAutoSwitch,

and prtInputNextIndex }

STATUS  current

DESCRIPTION

"The input switching group."

::= { prtMIBGroups 20 }

END

# Change History from version 0.1 (1/19/96) to make 0.2 (1/25/96)

1.	Added special syntax for phone numbers, fax numbers, and email numbers to prtGeneralCurrentOperator and prtGeneralServicePerson objects, instead of new objects.

2.	Removed the proposed prtGeneralErrorPage, since error pages are controlled by the print job submission protocol on a per job basis.

3.	Removed the proposed prtOutputAutoSwitch and prtOutputNextIndex objects, since most printer don’t do auto switching of the output bins.

4.	Removed the proposed prtInterpreterDefaultName interpreter object, since PostScript doesn’t have a setable default font, and PCL needs more.

5.	Removed the proposed prtInterpreterErrorPageType and prtInterpreterBannerPageType since current practice is not sufficient to justify adding them.

6.	Put the prtInputManualFeedTimeout, prtInputAutoSwitch, and prtInputNextIndex the deal with manual trays and input tray switching into a new prtInputSwitchingGroup, instead of adding them to the existing optional prtExtendedInputGroup, in order to be upwards compatible with RFC 1759 and allow implementation of the three new objects without requiring implementation of the other objects in the prtExtendedInputGroup.

7.	Clarified that a manual input feed can use the switch mechanism when the time-out occurs or abort, depending on the value of prtInputAutoSwitch and prtInputNextIndex.

8.	Clarified that a prtInputManualFeedTimeout value of other(-1) is the value to be used for input subunits that don’t support manual feed.